3GPP TSG-RAN WG4 Meeting #114bis                                      R4-2504283

Wuhan, China, April 7th – April 11th, 2025

Source: 	Huawei, HiSilicon

Title: 	On signalling aspect of fragmented carriers

Agenda Item:	7.5.2

Document for:	Discussion

Introduction

The SI of NR FR1 DL Fragmented Carriers was approved at RAN#104. During previous RAN4 meetings, the signalling aspect was discussed but there are still open issues to be decided. For this meeting, we provided the initial performance evaluation in [6]. In this document, we would like to discuss the signalling aspect accordingly.

Discussion

Triggering condition

At previous meetings, the triggering condition to enable “One RF Rx chain” was discussed. To our understanding, whether a triggering condition is needed is dependent on the expected RF performance of “One RF Rx chain”. If the RF performance is comparable with existing CA RF requirements, a specific triggering condition is not necessary.

Observation 1: If the RF performance of “One RF Rx chain” is comparable with existing CA RF requirements, a specific triggering condition is not necessary.

In this meeting we provided some RF performance evaluation results in [6]. Based on the evaluation results, we have the observation that the relaxation required for “One RF Rx chain” is <10dB for REFSENS and <2dB for ACS/IBB. This amount of relaxation is at the same level of many MSD exceptions of normal CA configurations.

Observation 2: Current RF performance evaluation shows the relaxation required for “One RF Rx chain is at the same level of many MSD exceptions of existing CA configurations.

Based on the performance evaluation, we think it’s possible that network handles the “One RF Rx chain” in the same way of normal CA configurations, i.e. no special triggering condition is needed. This approach could lead to fast deployment of this new feature once the UE could support “One RF Rx chain” on certain band. We think it could be considered as the baseline solution.

Proposal 1: The “One RF Rx chain” could be handled in the same way of normal CA configurations without triggering condition. This approach could be considered as the baseline solution.

Although the evaluation shows the RF performance of “One RF Rx chain” is promising within the scope of this WI, some companies expressed the concern that there might be some unexpected interference in the real network. As an enhancement, the UE could optionally monitor the signal quality and provide more information to help the network configuration, which could be considered as a potential “triggering condition”.

Proposal 2: As an enhancement, the UE could optionally provide extra information to help the network configuration.

In previous meetings, some companies proposed to measure and report the absolute power level of in-gap interference. We feel whether the UE could work well with single Rx RF Chain is not only dependent on the absolute power level of interference, but also dependent on the power level of wanted carrier, position of the interference(s), the UE’s RF design and baseband design etc. Only providing the absolute power level of in-gap interference could barely help the network to improve the configuration. More information related to the UE’s real capability would be more helpful.

Another method mentioned by companies was that UE could provide a single bit indicator to tell the network whether the “One Rx RF chain” is the preferred configuration in the corresponding band. With this approach, the UE is possible to take multiple factors into consideration and make a more accurate decision. The remaining issue for this approach is what it means exactly when UE sends “1” by the indicator. What level of performance could be expected, and how robust is the performance needs to be answered. This needs to be aligned between the UE and the network.

Proposal 3: A few options for the extra triggering condition could be further investigated.

Fallback scheme

The fallback scheme was also discussed in previous meetings. From CA configuration point of view, the two fallback options are:

- CA with Single Rx RF Chain --> Non CA. E.g, CA_nX(2A)-nYA --> CA_nXA-nYA

- CA with Single Rx RF Chain --> CA with Separate Rx RF Chain. E.g, CA_nX(2A)-nYA --> CA_nX(2A)

The option a) is a simple solution from both UE aspect and Network aspect. A UE supporting CA with Single Rx RF chain will undoubtedly support the non-CA configuration on the same band. From Network perspective, if the performance of a component carrier is not meeting the expectation, it’s also natural choice to deactivate/release this component carrier.

Observation 3: CA with Single Rx RF Chain falling back to Non-CA on the same band is a simple solution from both UE aspect and Network aspect. No extra specification impact is needed.

Proposal 4: The option of “CA with Single Rx RF Chain falling back to Non-CA” on the same band is considered as a baseline solution.

The option b), on the other hand will require proper coordination between the UE and the Network. The switching from Single Rx RF Chain to Separate Rx RF Chain requires additional RF resource, which is dependent on UE’s hardware ability. For example, the initial CA configuration is CA_nX(2A)-nY(2A) with Single Rx RF Chain on both bands (Conf. 1 in Table 2.2-1). The UE might be able to fall back to Separate Rx RF Chain on only one band due to the hardware limitation, which means the possible fallback configurations are Conf. 2 or Conf. 3 in the same table, but the Conf. 4 couldn’t be supported by the UE. This needs to be clearly communicated to the Network.

Table 2.2-1 The possible fallback configurations of CA_nX(2A)-nY(2A)

|  | nX(2A) | nY(2A) | Total Rx Chain | Support status |
| --- | --- | --- | --- | --- |
| Conf. 1 | Single | Single | 2 | √ |
| Conf. 2 | Single | Separate | 3 | √ |
| Conf. 3 | Separate | Single | 3 | √ |
| Conf. 4 | Separate | Separate | 4 | X |

From Network perspective, the option b) also requires more complex implementation and the signalling adaptation. The Network needs to first decide the better configuration between the Non-CA and CA with Separate Rx RF Chain. To enable the switching between Single Rx RF Chain and Separate Rx RF Chain, a new signalling scheme is also required to explicitly indicating the RF architecture.

Observation 4: CA with Single Rx RF Chain falling back to CA with Separate Rx RF Chain requires UE fallback capability reporting, new CA configuration signalling with indication of RF architecture and Network implementation update.

Proposal 5: The option of “CA with Single Rx RF Chain falling back to CA with Separate Rx RF Chain on the same band” could be considered as an optional enhancement. To enable this option, the dedicated UE fallback capability reporting and new CA configuration signalling with indication of RF architecture needs to be specified.

UE capability reporting

Based on the RF performance evaluation, the UE’s Rx requirements are different for Single Rx RF Chain architecture and Separate Rx RF Chain architecture. Considering some higher order CA configurations could only be supported with Single Rx RF Chain, the conformance testing needs to follow the updated RF requirements accordingly. A new UE capability signalling is needed to indicate whether Single Rx RF Chain is supported for a specific CA configuration at least for the testing purpose.

Proposal 6: A new UE capability signalling is needed to indicate whether Single Rx RF Chain is supported for a specific CA configuration at least for the testing purpose.

As mentioned in proposal 5, a dedicated UE capability signalling for the fallback scheme also needs to be considered. Considering the example in Table 2.2-1, the UE supports the CA configuration CA_nX(2A)-nY(2A) with Conf.1/2/3 but doesn’t support the same CA configuration with Conf.4. The clearest way to communicate this to the network would be to report the CA configuration with different RF architectures separately.

Proposal 7: In UE capability signalling, the same CA configuration with Single Rx RF Chain and with Separate Rx RF Chain are reported separately.

To easily distinguish the different RF architecture, a new notation for CA configuration as suggested by [4] and [5] could be also considered. The letter ‘F’ is suggested in both contributions, but we would like to suggest using a different letter since ‘F’ is being used in TS 38.101-2 and TS 36.101 to represent a contiguous CA bandwidth class. Taking the FR2 bandwidth classes ‘R2~R12’ as a reference, maybe we could use a similar way to represent the CA with Single Rx RF Chain, such as ‘Z2~Z6’, where ‘Z2’ means 2 non-contiguous CCs received by a single Rx RF Chain.

Proposal 8: Using a new notation for CA configurations with Single Rx RF Chain, such as ‘Z2’~’Z6’, where ‘Z2’ means 2 non-contiguous CCs received by a single Rx RF Chain.

Conclusion

In this contribution we provide some consideration on the signalling aspect of fragmented carriers. According to the analysis, we have the following proposals:

Observation 1: If the RF performance of “One RF Rx chain” is comparable with existing CA RF requirements, a specific triggering condition is not necessary.

Observation 2: Current RF performance evaluation shows the relaxation required for “One RF Rx chain is at the same level of many MSD exceptions of existing CA configurations.

Proposal 1: The “One RF Rx chain” could be handled in the same way of normal CA configurations without triggering condition. This approach could be considered as the baseline solution.

Proposal 2: As an enhancement, the UE could optionally provide extra information to help the network configuration.

Proposal 3: A few options for the extra triggering condition could be further investigated.

Observation 3: CA with Single Rx RF Chain falling back to Non-CA on the same band is a simple solution from both UE aspect and Network aspect. No extra specification impact is needed.

Proposal 4: The option of “CA with Single Rx RF Chain falling back to Non-CA” on the same band is considered as a baseline solution.

Observation 4: CA with Single Rx RF Chain falling back to CA with Separate Rx RF Chain requires UE fallback capability reporting, new CA configuration signalling with indication of RF architecture and Network implementation update.

Proposal 5: The option of “CA with Single Rx RF Chain falling back to CA with Separate Rx RF Chain on the same band” could be considered as an optional enhancement. To enable this option, the dedicated UE fallback capability reporting and new CA configuration signalling with indication of RF architecture needs to be specified.

Proposal 6: A new UE capability signalling is needed to indicate whether Single Rx RF Chain is supported for a specific CA configuration at least for the testing purpose.

Proposal 7: In UE capability signalling, the same CA configuration with Single Rx RF Chain and with Separate Rx RF Chain are reported separately.

Proposal 8: Using a new notation for CA configurations with Single Rx RF Chain, such as ‘Z2’~’Z6’, where ‘Z2’ means 2 non-contiguous CCs received by a single Rx RF Chain.

- References

- RP-241669, “New SID: Study on NR FR1 DL Fragmented Carriers”, MediaTek Inc, RAN#104 Meeting, June 2024

- R4-2420519, “WF on NR FR1 DL fragmented carriers study”, MediaTek Inc, RAN#113 Meeting, November 2024

- R4-2418146, “Topic summary for [113][124] FS_NR_FR1_DL_Frag_Carrier”, MediaTek, RAN4#113 Meeting, November 2024

- R4-2418531, “On methods for reducing the number of UE Rx chains for fragmented carriers”, Apple, RAN4#113 meeting, November 2024

- R4-2419454, “Discussion on Fragmented Carrier reduction of Rx RF chains”, Nokia, RAN4#113 meeting, November 2024

- R4-2504284, “On RF requirements of fragmented carriers”, Huawei, HiSilicon, RAN4#114bis meeting, April 2025