3GPP TSG-RAN WG4 Meeting #114bis                                    R4-2504285

Wuhan, Hubei, China, 07th – 11th April, 2025

Source:	Samsung

Title:              General discussion on RRM requirements for LB CA switching

Agenda item:			7.7.3

Document for:	Discussion

Introduction

In last meeting, RAN4 had some initial discussions on RRM requirements for LB CA switching. In this meeting, we will give some further discussions for some open issues captured in the agreed WF [1].

2 Discussion

2.1 Potential RRM requirement impact

2.1.1 General interpretations

From our understanding, the following interpretation regarding switching pattern can be given.

- (A) periodic pattern configured by RRC with at least the following parameters: periodicity, start slot/offset, and active period, see Fig.1. The principle of the pattern is very similar as cell DTX/DRX but with more restricted considerations, and it can be designed as a common/separate pattern depending on RAN1/RAN2 discussion. Specifically,

  - UE behaviour in PCell non-active periods: Rx reception on SDL carrier 2 and no Tx transmission/Rx reception on FDD carrier 1

  - UE behaviour in SCell non-active periods: Tx transmission/Rx reception on FDD carrier 1 and no Rx reception on SDL carrier 2

  - UE behaviour in PCell/SCell active periods: Depends on RF switching period location discussions and RAN1 switching pattern design

Besides, considering only a semi-static switching pattern based on RRC configuration is supported at current stage, one of the possible implementations for the switching pattern is: Switching pattern (s) is configured by UE specific RRC signalling and the corresponding configurations are explicitly signalled to the switching-capable UEs with the SwitchingPeriodLocation related signalling. As a baseline, the patterns are activated/deactivated by RRC signalling.

Fig.1 Interpretation: Common Switching pattern and one example of UE-NW interaction

Observation 1:One interpretation can be given for the switching pattern:

- A periodic pattern configured by RRC with the following parameters: periodicity, start slot/offset, active period.

Under the interpretation, the time mask requirement is discussed in RF session, and the following tentative agreement regarding switching period was achieved

< R4-2503016 WF on low-low band switching > Tentative Agreement: - Define switching periods between FDD low band and SDL low band with the following values as a UE capability to accommodate different UE implementations - 35us, 140us - FFS on 13us without LO switching - FFS on whether to capture 13us in the LS

Observation 2: As per RF guidance, the switching periods between FDD and SDL band are defined as [13 us], 35us and 140us.

At the same time, from our understanding, since UE RRM requirements are defined based on RSs-based measurement mainly, and from NW perspective, e.g., the cell-specific and periodic SSB transmission status of the serving cell and/or neighbor cell (based on the same DU assumption) can be known. In this sense, in order to avoid huge performance degradation, in theory, the RAN1/RAN2 switching pattern configuration has to satisfy UEs’ QoS, HARQ and RRM requirements and should guarantee the available measurement resources are enough during the pattern active periods, based on such resources, UE can to perform measurement and/or evaluation on PCC/SCC. From this, from our understanding,

- In order to ensure the normal RRM DL/UL operations will not be impacted, RAN4 to discuss the switching pattern configuration based on the number of SSB bursts, considering the requirements specified by the RAN4. Send LS to RAN1 capturing such information if needed.

Proposal 1: In order to ensure the normal RRM DL/UL operations will not be impacted, the available measurement resources should be enough during the pattern active periods

- RAN4 to discuss the switching pattern configuration based on the number of SSB bursts, considering the requirements specified by the RAN4.

Since the existing RRM measurements are related to DRX cycle and SSB period/SMTC period, MG, scaling factors, etc.. At least the following two issues will occur:

- Mis-alignment between cell switching pattern and UE DRX cycle.

Case 1: Part of the OFF duration of UE DRX is overlapped with the active period of the switching pattern.

Case 2: The ON duration of UE DRX is overlapped with the non-active period of the switching pattern.

- Critical signal/channel transmission periodicity occasions are not end in the active period of the switching pattern

Observation 3: Considering the existing RRM measurements are related to DRX cycle and SSB period/SMTC period, at least the following two issues will occur:

- Mis-alignment between cell switching pattern and UE DRX cycle

- Critical signal/channel transmission periodicity occasions are not end in the active period of the switching pattern

Proposal 2: RAN4 to discuss how to define RRM requirements considering switching pattern cycle, DRX cycle and RSs periods.

2.1.2 Interruption requirement

From our understanding, the interruption requirement discussion is needed. First, DL interruptions can be caused since carrier switching is happened. Second, the FDD UL interruptions can also be caused due to the UE architecture that PLL/LO sharing or the UL and DL is close in frequency proximity. Regarding how to define the interruption requirement, slot level or symbol level, we prefer to consider symbol level, since NW may not have precise timing information on the occasion of carrier switching, then the whole slot may have to be dropped if the former is considered. In this sense, the PUCCH transmission on UL carrier (especially with A/N) may be impacted. At the same time, the UE capability regarding application of interruptions due to carrier switching between two low bands should be defined, to indicate e.g., whether interruptions are allowed or not, during switching between two low bands.

Proposal 3:

- RAN4 to define symbol level interruption requirement for carrier switching in R19 LB CA

- UE capability regarding application of interruptions due to switching between two low bands should be defined

    - Impact on SCell (SSB based SCell operation)

As we discussed in Section 2.1.1, in the current spec., the activated SCell measurements including L1 measurement (e.g., L1-RSRP, BFD, CBD) and L3 measurement (e.g., inter-f measurement w/o gap) are related to DRX operation and RSs periodicity mainly. Although switching pattern will be defined, we do not think there is a need to strictly align UE DRX pattern with SDL SCell switching pattern. In this sense, there are two Alts:

- UE to define the measurement requirements in conjunction with SDL SCell switching pattern cycle, DRX cycle and RSs periods.

  - As a baseline, UE will not to measure RS occasions overlapping with switching pattern non-active periods

- To specify the priority for SDL SCell switching pattern and UE DRX pattern, and the SCell measurement requirements are defined based on the pattern priority

Proposal 4: Regarding SDL SCC related RRM requirements (SSB based activated SDL SCell operation), RAN4 can to discuss the following measurements requirements: L1-RSRP, BFD, CBD, and L3 inter-frequency measurements with/without gap

Proposal 5: To define SDL activated SCell L3 measurement requirement if SSB based SCell operation is used, two Alts can be considered:

- Alt 1. UE to define the measurement requirements in conjunction with SDL SCell switching pattern cycle, DRX cycle and RSs periods.

  - As a baseline, UE will not to measure RS occasions overlapping with switching pattern non-active periods.

- Alt 2. To specify priorities for SDL SCell switching pattern and UE DRX pattern, and the SCell measurement requirements are defined based on the pattern priorities.

  - UE may follow one of the patterns.

Besides, since some RS resouces for measurement transmitted on SDL/FDD will be punctured due to switching, suggest RAN4 to discuss where/how UE to receive the first available SSB resources to measure, i.e., Tfirst_SSB.

Proposal 6: RAN4 to discuss where/how UE to receive the first available SSB resources to measure, i.e., Tfirst_SSB.

    - Impact on SCell (SSB-less based SCell operation)

Based on the agreement on RTD between the two bands at UE (option 1 & option 2< CP for 15kHz) achieved in last meeting and feasibility analyzes for SSB-less SCell operation for inter-band CA for FR1 and co-located sync. cells in Rel-18 NES. From our understanding, it is acceptable to consider SSB-less based SCell activation delay requirement in LB CA switching scenario, and the target SDL SCell can obtain timing/frequency/AGC information from another inter-band FR1 serving cell (e.g., the n5 PCell or another active sync. co-located FR1 reference cell with SSB transmission within every 160 ms if it exists). Besides, based on Rel-18 NES discussion, no matter by using temporary RS or A-TRS, the SDL SCell activation procedure can be shorten, the PCell can be switched back quickly, the sync maintanence and UEs’ QoS requirements won’t be impacted significantly caused by switching.

Furthermore, since the SSB is transmitted with a relatively longer periodicity than the temporary RS, and according to RAN1 working assumption, the interval between two TRS bursts < 160 ms, from our understanding, if the configured switching pattern accommodates with SSB resources transmission characteristics, the TRS/A-TRS resources transmission can be naturally involved, and the SDL SCell can be activated based on TRS/A-TRS resources measured during SCell switching pattern active periods. Of course, the pre-condition to meet the above analyzes is that the UE need to support SSB-less related capabilities.

Proposal 7: For UE supporting SSB-less SCell operation, it is acceptable to consider SSB-less based SCell activation delay requirement in LB CA switching scenario

- The SDL SCell can obtain timing/ AGC information from another active sync. co-located FR1 reference cell, e.g., the serving cell PCell or another reference cell with SSB transmission within every 160 ms if it exists

Proposal 8: The existing Rel-18 NES SSB-less based SCell activation delay requirement based on TRS and A-TRS can be used as baseline for R19 LB CA.

    - Impact on PCell

From our understanding, as per WID guidance, RAN1 should strive to minimize specifications impact, while the delay of carrier switching would result in additional processing time for a UL transmission associated with a PDCCH or PDSCH similar as BWP switching. For example, a PDCCH schedules a PUSCH, a PDCCH indicates SRS transmission, a RAR schedules a PUSCH transmission, and a PUCCH with HARQ-ACK for a PDSCH or PDCCH reception. From this,

Proposal 9: Whether to/how to define potential requirement impacts to the PCell including scheduling restriction and/or measurement delay extension depending on RAN1 switching pattern design

3 Conclusion

In this contribution, we provided our viewpoints on RRM requirements for LB CA switching. The following observations and proposals are obtained:

Observation 1:One interpretation can be given for the switching pattern:

- A periodic pattern configured by RRC with the following parameters: periodicity, start slot/offset, active period.

Observation 2: As per RF guidance, the switching periods between FDD and SDL band are defined as [13 us], 35us and 140us.

Observation 3: Considering the existing RRM measurements are related to DRX cycle and SSB period/SMTC period, at least the following two issues will occur:

- Mis-alignment between cell switching pattern and UE DRX cycle

- Critical signal/channel transmission periodicity occasions are not end in the active period of the switching pattern

Proposal 1: In order to ensure the normal RRM DL/UL operations will not be impacted, the available measurement resources should be enough during the pattern active periods

- RAN4 to discuss the switching pattern configuration based on the number of SSB bursts, considering the requirements specified by the RAN4.

Proposal 2: RAN4 to discuss how to define RRM requirements considering switching pattern cycle, DRX cycle and RSs periods.

Proposal 3:

- RAN4 to define symbol level interruption requirement for carrier switching in R19 LB CA

- UE capability regarding application of interruptions due to switching between two low bands should be defined

Proposal 4: Regarding SDL SCC related RRM requirements (SSB based activated SDL SCell operation), RAN4 can to discuss the following measurements requirements: L1-RSRP, BFD, CBD, and L3 inter-frequency measurements with/without gap

Proposal 5: To define SDL activated SCell L3 measurement requirement if SSB based SCell operation is used, two Alts can be considered:

- Alt 1. UE to define the measurement requirements in conjunction with SDL SCell switching pattern cycle, DRX cycle and RSs periods.

  - As a baseline, UE will not to measure RS occasions overlapping with switching pattern non-active periods.

- Alt 2. To specify priorities for SDL SCell switching pattern and UE DRX pattern, and the SCell measurement requirements are defined based on the pattern priorities.

  - UE may follow one of the patterns.

Proposal 6: RAN4 to discuss where/how UE to receive the first available SSB resources to measure, i.e., Tfirst_SSB.

Proposal7: For UE supporting SSB-less SCell operation, it is acceptable to consider SSB-less based SCell activation delay requirement in LB CA switching scenario

- The SDL SCell can obtain timing/ AGC information from another active sync. co-located FR1 reference cell, e.g., the serving cell PCell or another reference cell with SSB transmission within every 160 ms if it exists

Proposal 8: The existing Rel-18 NES SSB-less based SCell activation delay requirement based on TRS and A-TRS can be used as baseline for R19 LB CA.

Proposal 9: Whether to/how to define potential requirement impacts to the PCell including scheduling restriction and/or measurement delay extension depending on RAN1 switching pattern design

4 References

[1] R4-2502588 “WF on RRM requirements for NR_LBCA_Sw”, Apple