3GPP TSG-RAN WG4 Meeting #114bis 	R4-2504301

Wuhan, China, 7th – 11st April, 2025

Source: 	Huawei, HiSilicon

Title: 	On modification of existing BS TX requirements for SBFD

Agenda Item:	7.23.2.2

Document for:	Approval

1 Introduction

During RAN4#113, BS RF requirements for SBFD have been discussed. Following conclusions for existing TX requirements [6], we would like to share our views regarding the open issues.

2 Discussion

2.1 Unwanted emissions

Per WF in [6], the following agreement was reached.

Agreement:

- FFS whether/how to introduce the additional ACLR and/or OBUE requirements for FR1 WA BS.

Whether to introduce the additional requirements, i.e. whether to consider the SBFD-SBFD co-existence in Rel-19, which is discussed in [10]. There is clear request from operators and companies. This paper mainly focusses on how to define the additional unwanted emission requirements.

Option 1: Reuse the co-existence limit for protection of BS UL.

The discussion can be found in our previous paper [5], the following observations are made

Observation 1: For protection of SBFD UL, the limit -49 dBm/MHz for protection of BS UL can be reused.

Observation 2: available guard band for different operators’ spectrum cases are different, which depends on the spectrum allocation: number of operations in one band and bandwidth for each operator.

Observation 3: The ratio of transition guard/sub frequency band is higher than that of ΔfOBUE, which means it would not impose additional implementation difficulty for taking the advantage of per operator filter.

Observation 4: with larger frequency offset than ΔfOBUE, the required rejection for protection of SBFD UL is achievable for the analog filter.

Option 2: RAN4 co-ex simulation

Still based on RAN4 co-ex simulation methodology, we use the same simulation assumptions as in Rel-18 SBFD SI and focus on SBFD-SBFD deployment. The only difference is we traverse the ACIR in order to get a desired BS-BS ACIR level.

Following figure demonstrates how UL throughput loss @5%-tile, i.e. cell edge, could be improved with an increasing ACIR.

The grid shift is set to 10%. Around 38 dB improvement on ACIR (comparing to the existing 42.5dB ACIR determined by 45dB ACLR and 46dB ACS) can be enough to guarantee <3% UL throughput loss.

The grid shift is set to 20%. Around 30 dB improvement on ACIR can be enough to guarantee <3% UL throughput loss.

The grid shift is set to 50%. Around 17 dB improvement on ACIR can be enough to guarantee <3% UL throughput loss.

As discussed in [7], it is proposed to adopt 20% grid shift to be used for both TX and RX requirements definition for adjacent channel co-existence cases. Around 30 dB improvement are needed, which results a protection level to -45 dBm/MHz (-15 dBm/MHz -30 dB).

Proposal 1: For protection of SBFD UL, it is proposed to define the limit [-45 ~ -49 dBm/MHz] as the additional emission mask requirements.

Proposal 2: For protection of SBFD UL, text proposal for the additional emission mask requirement is proposed as below

==== Text proposal for SBFD additional OBUE requirement for TS 38.104====

6.6.4.2.5.8	Additional emissions limits for SBFD-capable BS

In regions where SBFD-capable BSs are operating in adjacent channel. For protection of SBFD UL sub-band in adjacent channel, emissions shall not exceed the maximum levels specified in table 6.6.4.2.5.8-1. f is the separation between the SBFD DL edge frequency and the nominal -3 dB point of the measuring filter closest to the edge.

Table 6.6.4.2.5.8-1: additional OBUE requirement for protection of SBFD UL

| Declared frequency offset of measurement filter -3dB point, f | Minimum requirement | Measurement bandwidth |
| --- | --- | --- |
| f | [-45~-49] dBm | 1MHz |

==== End of Text proposal ====

2.2 Text Proposals for TX

At last meeting in [9], it was agreed to use a separate chapter for requirements for SBFD-capable BS. Some baseline proposals are agreed in [7]. It includes the BS type 1-H, 1-O and 2-O, so both conducted requirements and radiated requirements should be defined. To keep the same levels for SBFD requirements, the following clauses skeleton are proposed for TX requirements.

Proposal 3: the following clauses skeleton are proposed for TX requirements.

=================== End of Text Proposal ===================

12	Conducted transmitter characteristics for SBFD capable BS

12.1	General

12.2	Base station output power for SBFD

12.3	Output power dynamics for SBFD

12.4	Transmit ON/OFF power for SBFD

12.5	Transmitted signal quality for SBFD

12.6	Unwanted emissions for SBFD

12.7	Transmitter intermodulation for SBFD

13	Radiated transmitter characteristics for SBFD capable BS

13.1	General

13.2	Radiated transmit power for SBFD

13.3	OTA base station output power for SBFD

13.4	OTA output power dynamics for SBFD

13.5	OTA transmit ON/OFF power

13.6	OTA transmitted signal quality for SBFD

13.7	OTA unwanted emissions for SBFD

13.8	OTA transmitter intermodulation

=================== End of Text Proposal ===================

Conclusion

In this contribution we discussed on the BS TX requirement for SBFD.

For unwanted emission, this paper provide the analysis on how to define the additional emission requirements with two options.

Option 1: Reuse the co-existence limit for protection of BS UL.

Option 2: RAN4 co-ex simulation

We have the following observations and proposals:

Observation 1: For protection of SBFD UL, the limit -49 dBm/MHz for protection of BS UL can be reused.

Observation 2: available guard band for different operators’ spectrum cases are different, which depends on the spectrum allocation: number of operations in one band and bandwidth for each operator.

Observation 3: The ratio of transition guard/sub frequency band is higher than that of ΔfOBUE, which means it would not impose additional implementation difficulty for taking the advantage of per operator filter.

Observation 4: with larger frequency offset than ΔfOBUE, the required rejection for protection of SBFD UL is achievable for the analog filter.

Proposal 1: For protection of SBFD UL, it is proposed to define the limit -49 dBm/MHz as the additional emission mask requirements.

Proposal 2: For protection of SBFD UL, text proposal for the additional emission mask requirement is proposed as below

==== Text proposal for SBFD additional OBUE requirement for TS 38.104====

6.6.4.2.5.8	Additional emissions limits for SBFD-capable BS

In regions where SBFD-capable BSs are operating in adjacent channel. For protection of SBFD UL sub-band in adjacent channel, emissions shall not exceed the maximum levels specified in table 6.6.4.2.5.8-1. f is the separation between the SBFD DL edge frequency and the nominal -3 dB point of the measuring filter closest to the edge.

Table 6.6.4.2.5.8-1: additional OBUE requirement for protection of SBFD UL

| Declared frequency offset of measurement filter -3dB point, f | Minimum requirement | Measurement bandwidth |
| --- | --- | --- |
| f | [-45~-49] dBm | 1MHz |

==== End of Text proposal ====

Proposal 3: the clauses skeleton in clause 2.2 are proposed for TX requirements.

References

- R4-2406107, “Way Forward for [110bis][313] NR_duplex_evo”, Samsung, 3GPP TSG-RAN4 Meeting #110bis, Changsha, China, April 15-19, 2024.

- R4-2409958	Way Forward for [111][313] NR_duplex_evo	Samsung

- R4-2413515, Way Forward for [112][308] NR_duplex_evo_BSRF, Huawei, HiSilicon

- R4-2416585, Way Forward for [112bis][307] NR_duplex_evo_BSRF, Huawei, HiSilicon

- R4-2419244, On modification of existing BS TX requirements for SBFD, Huawei, HiSilicon

- R4-2419908, Way Forward for [113][307] NR_duplex_evo_BSRF, Huawei

- R4-2501856, On modification of existing RX RF requirements for SBFD

- R4-2502380, Way Forward for [114][307] NR_duplex_evo_BSRF, Huawei

- R4-2502310, Way Forward for [114][306] NR_duplex_evo_General, Samsung

- R4-2504300, Deployment scenarios for FR1 SBFD macro BS, Huawei, HiSilicon, Samsung, Qualcomm, Verizon, Vodafone, China Telecom, China Unicom