3GPP TSG-RAN WG4 Meeting #114bis	R4-2504315

# Wuhan, CN, 07 - 11 Apr, 2025

Title: 	Discussion on less than 5MHz for NR NTN

Source: 	Huawei,HiSilicon

Agenda item:	7.10.4.2

Document for:	Discussion

Background

During RAN#106 meeting, Revised WID [1] was approved. In this contribution, we share our view on less than 5MHz for NR NTN.

Discussion

UE demodulation requirements

In Rel-18 NR FR1 less than 5MHz WI, there is only punctured PDCCH cases and PBCH cases are defined for TN UE in TS 38.101-4. However, there is only additional PDSCH requirements are defined for NTN UE in TS 38.101-5 assuming that NTN UE also need to pass TN test cases. Therefore, we think there is no any specification changes in TS 38.101-5.

Note: For UE supporting NR NTN access (nonTerrestrialNetwork-r17), the requirements in TS38.101-4 also applies to UE according to applicability rules in TS38.101-4 Clause 5.1, 6.1, 7.1 and 8.1

No specification changes in TS 38.101-5.

SAN demodulation requirements

In Rel-18 NR FR1 less than 5MHz WI, there is only PUCCH format 2 cases with applicability rules are defined for TN UE in TS 38.104. For NTN SAN, we think the same cases with same applicability rules can be defined with same configurations, except that change propagation conditions to NTN-TDLA100-200 Low.

Only define PUCCH format 2 cases for SAN.

Add same applicability rule in TS 38.181.

Proposals

In this contribution, we share our view on less than 5MHz for NR NTN. Our observations and proposals are:

- No specification changes in TS 38.101-5.

Only define PUCCH format 2 cases for SAN with the following simulation assumption.

| Parameter | Values |
| --- | --- |
| Modulation order | QPSK |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | enabled |
| First PRB after frequency hopping | 3 |
| Number of PRBs | 9 |
| Number of symbols | 2 |
| The number of UCI information bits | 22 |
| First symbol | 12 |
| DM-RS sequence generation | NID0=0 |
| Antenna | 1x1, 1x2 |
| Test metric | BLER |

Add following applicability rule in TS 38.181.

For a BS declaring to support less than 5MHz (see D.7 in table 4.6-1) a UCI BLER performance test with Format 2 PUCCH shall be tested in 3 MHz. The UCI BLER performance tests on 5MHz PUCCH Format 2 to be skipped if applicable.

Reference

RP-242901, Revised WID on Enhanced requirements and conductive test methodology for NR NTN and IoT NTN, RAN#106, Apple, Thales, Inmarsat, Viasat, Globalstar