3GPP TSG-RAN WG4 Meeting #114bis	  R4-2504321

Wuhan, China, April 7 – 11, 2025

Agenda Item:	7.20.2

Source:	Huawei, HiSilicon

Title:	Discussion on testing issues for CSI compression two-sided models

Document for:	Discussion and Decision

- Introduction

According to WF in RAN4 #114 [1], we discuss the remaining issues related to CSI compression two-sided models.

- Feasibility verification of potential testing Option 3

RAN4 #112 WF [2]: Option 3 next steps - Check on performance alignment -> see simulation results from contributing companies - repeat simulations until good alignment is achieved - move to next step after alignment - Share models(encoder or decoder or both)/datasets (training/testing/inference) - framework to share data among companies(dataset/model) to be discussed - Select one or more decoder for further analysis - selection criteria to be discussed - Each company brings results for training of “own encoder” with selected decoder(s) - performance alignment to be checked/discussed - using “own” data or data shared by other companies - Conclude on overall feasibility of Option 3 - consider the conditions under which Option 3 is feasible if found feasible - FFS on what the feasibility criteria Note: some steps could be reused for Option 4, FFS on what can be reused

According to the previous agreement of RAN4, companies provide simulation results under a common model structure using their own training dataset. It is expected that after a round of parameters refinement, good alignment can be achieved. Currently, the intermediate KPI SGCS is collected and the criteria of performance alignment have not been determined yet. Since the primary goal during SI phase is to reach a consensus on whether we can achieve agreement regarding the complete model, encompassing both its structure and parameters, using the margin of SGCS as a criterion of procedure feasibility verification should suffice.

Proposal 1: Use SGCS as a key metric for verifying the feasibility of procedures related to the standardization of full models.

If we do not consider eventual throughput alignment, it seems rather challenging to determine the specific value of the SGCS margin at which performance can be considered convergent, and thus the feasibility of the procedure can be claimed. Maybe we could define multiple degrees of performance convergence based on SGCS margin. For SGCS margin definition, we suggest to follow RAN4 legacy principle. For example, take the average of SGCS values, or use the SGCS value corresponding to the 90% of the CDF curve.

Proposal 2: Define SGCS margin by considering the following options.

- Option 1: Average of SGCS

- Option 2: SGCS @90% of CDF

Proposal 3: Define multiple degrees of performance convergence based on SGCS margin:

- SGCS margin > 0.1 indicates low - degree convergence.

- SGCS margin within [0.05, 0.1] indicates medium - degree convergence.

- SGCS margin < 0.05 indicates high - degree of convergence.

RAN4 #114 WF [1]: How to continue with track 1 until RAN4#114bis Companies should provide quantized and unquantized models according to the advice above, in particular Samsung, CATT and Huawei. - Each company is encouraged to train own encoder using the fixed decoder and their own dataset, and then test with every companies datasets, in order to check for consistency in results

Currently, we are at the feasibility verification step 4. Two tracks have been identified for this step. In track 1, each company generates its own training dataset, which means the datasets are not aligned among companies. In track 2, the training dataset is generated by mixing the datasets from different companies, which means the datasets are aligned among companies.

Assuming an encoder with the agreed model structure, we provide simulation results both in track 1 and track 2. In track 1, with fixed decoder from the selected Samsung and CATT, we use our own dataset to train the encoder. For testing, we tried the dataset from individual companies as well as the mixed testing dataset.

Proposal 4: SGCS results for Track 1 with encoder model structure under current assumption are provided in Table 1.

Table 1. SGCS results for Track 1 with encoder model structure under current assumption

|  | Track 1: Training @ Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Testing @ Huawei, Hisilicon |  | Testing @CATT |  | Testing @QC |  | Testing @Vivo |  | Testing @Mixed |  |
|  | Unquantized | Quantized | Unquantized | Quantized | Unquantized | Quantized | Unquantized | Quantized | Unquantized | Quantized |
| Huawei, Hisilicon decoder | 0.6843 | 0.66 | - | - | - | - | - | - | - | - |
| Samsung decoder fixed | 0.5779 | 0.5235 | 0.4505 | 0.4722 | 0.5059 | 0.4881 | 0.617 | 0.6191 | 0.6477 | 0.6364 |
| CATT decoder fixed | 0.5458 | 0.4707 | 0. 4281 | 0.3891 | 0.4645 | 0.3906 | 0.6148 | 0.5964 | 0.6329 | 0.6004 |

In Track 1, when using the mixed dataset as the testing dataset, we achieve relatively higher SGCS performance compared to using individual companies’ datasets. Even so, when employing the mixed testing dataset, we can observe approximately a 0.05 loss in SGCS performance compared to the original model pair trained with our own dataset. This performance degradation likely stems from disparities in the distribution of training data.

Observation 1: Around 0.05 SGCS performance loss can be observed in Track 1 under mixed testing dataset.

Observation 2: Around 0.1 SGCS performance loss can be observed in Track 1 under companies’ individual dataset.

In track 2, we use the mixed dataset to train the model pair. Then we tried both the mixed training dataset and our own training dataset to train the encoder against the selected decoder from Samsung according to last meeting WF.

Proposal 5: SGCS results for Track 2 with encoder model structure under current assumption are provided in Table 1.

Table 2. SGCS results for Track 2 with encoder model structure under current assumption

|  | Training @ mixed, testing @ mixed |  | Training @ Huawei, Hisilicon, testing @ mixed |  |
| --- | --- | --- | --- | --- |
|  | Unquantized | Quantized | Unquantized | Quantized |
| Huawei, Hisilicon decoder | 0.76 (Joint training) | 0.71 (Joint training) | - | - |
| Samsung decoder fixed | 0.7556 (encoder retraining) | 0.7029 (encoder retraining) | 0.7085 (encoder retraining) | 0.6673 (encoder retraining) |

Observation 3: In track 2, nearly no SGCS performance loss with fixed selected decoder using mixed dataset to retrain the encoder, assuming the encoder model structure is aligned between training and retraining.

Observation 4: In track 2, around 0.05 SGCS performance loss is observed with fixed selected decoder using our own dataset to retrain the encoder.

It is observed that when a decoder is selected and a private encoder is trained by using the mixed dataset, there’s less performance degradation compared to joint training.

Proposal 6: Use track 2 to move forward by both considering encoder retraining under mixed dataset and companies own dataset.

- Consideration on low-complexity encoder

It is agreed in R18 38.843 that the assumption of encoder should consider the complexity limitation at UE.

Table 3: Complexity of CNN (current assumption in RAN4) and R16 Type-II

| CNN-based encoder (current assumption in RAN4) | 47 MFLOPs |
| --- | --- |
| R16 Type-II | 0.37 MFLOPs |

However, the complexity of agreed CNN model structure at the feasibility study stage is much higher than that of R16 eType-II, as shown in Table 3.

RAN4 #113 WF [2]: Issue 1-2: Option 3 next step - decoder(s) selection Agreement: choose the decoders with Low, medium, high SGCS among the decoders submitted by companies Continue the feasibility study on the two tracks below: Track 1: Select 3 decoders at RAN4#113. Companies train own encoders (with own Eigenvector dataset) and check performance against decoders (with at least own test dataset) - The 3 decoders are selected based on low, medium, high mean SGCS out of available decoders - At least an encoder with the agreed model structure should be considered when making own encoder. Optionally companies can try own encoder with their preferred structure. Companies should report the structure if different. Track 2: Create a mixed dataset at RAN4#113. By RAN4#114, companies train decoders based on the mixed dataset. At RAN4#114, one or more decoder selected and after RAN4#114, companies develop encoders and check encoder performance with own dataset checked against decoder(s). At least an encoder with the agreed model structure should be considered when making own encoder. Optionally companies can try own encoder with their preferred structure. Companies should report the structure if different.

According to RAN4 #113 WF, companies can try and report own encoder with other structures rather than the one under current assumption. We provide both the CNN-based and transformer-based encoders with much lower complexity. The model structures and detailed descriptions are shown in the following.

Figure 1-1. Model structure of CNN-based encoder with 4.14MFLOPs

Table 1-2. Description of CNN-based encoder with 4.14MFLOPs

| Model type | CNN (Resnet like design) |
| --- | --- |
| FLOPs | 4.14M |
| Model size | 92Kbyte |
| Num of Resnet blocks | 2 |
| Layer after resnet blocks | 2 layers (one FC layer, one Conv layer) |
| Layer before resnet blocks | 1 layer (one conv layer) |
| Layer within a resnet block | 2 Conv layer and direct combination |
| CNN kernel parameters | - dilation =1 (which is the default parameter) - padding type = zero padding (which is the default parameter) - The value of groups =1 (which is default parameter) |
| Kernel/Channel size within a resnet block | (3,3,16) |
| Size of message (feature map) passed across layers | (13,32) |

Figure 2-1. Model structure of Transformer-based encoder with 3.63MFLOPs

Table 2-2. Description of Transformer-based encoder with 3.63MFLOPs

| Model type | Transformer |
| --- | --- |
| FLOPs | 3.63M |
| Model size | 1.6Mbyte |
| d_model | 64 |
| d_feedforward | 256 |
| N_head | 8 |
| d_head | 64 |
| N_TF | 4 |

With fixed decoder, we retrain the lower-complexity CNN and transformer encoder using mixed dataset. The performance results are as follows.

Proposal 7: SGCS results for lower-complexity CNN and transformer encoder are provided in Table 3.

Table 3. SGCS results for lower-complexity CNN and transformer encoder retraining

|  | Training/Testing @ mixed, retraining |  |
| --- | --- | --- |
|  | Unquantized | Quantized |
| Assumed CNN-based encoder (94M FLOPs) Joint training, no retraining | 0.76 | 0.71 |
| CNN-based encoder (4.14M FLOPs) Encoder retraining | 0.73 | 0.68 |
| Transformer-based encoder (3.14M FLOPs) Encoder retraining | 0.64 | 0.62 |
| Transformer-based encoder (3.14M FLOPs) Joint training, no retraining | 0.72 | 0.68 |

Observation 5: Under the same backbone, 0.03 SGCS performance loss is observed for encoder retraining with a lower-complexity CNN-based model structure.

Observation 6: Under a different backbone from current CNN-based assumption, around 0.1 SGCS is observed for encoder retraining with a lower-complexity Transformer-based model structure.

If a lower - complexity encoder is not captured at the initial stage of deriving the test decoder, companies will experience an SGCS performance decline when they retrain the encoder with reduced complexity, even when using the same training dataset. In contrast, when a lower - complexity encoder is employed to derive the test decoder, there’s almost no observable performance degradation compared to using a significantly higher - complexity encoder.

Proposal 5: Use lower complexity encoder model structure for testing/reference model structure alignment if AI-CSI turns to WI.

Observation 1: Around 0.05 SGCS performance loss can be observed in Track 1 under mixed testing dataset.

Observation 2: Around 0.1 SGCS performance loss can be observed in Track 1 under companies’ individual dataset.

- Conclusions

According to the discussion, following proposals and observations are provided:

Proposal 1: Use SGCS as a key metric for verifying the feasibility of procedures related to the standardization of full models.

Proposal 2: Define SGCS margin by considering the following options.

- Option 1: Average of SGCS

- Option 2: SGCS @90% of CDF

Proposal 3: Define multiple degrees of performance convergence based on SGCS margin:

- SGCS margin > 0.1 indicates low - degree convergence.

- SGCS margin within [0.05, 0.1] indicates medium - degree convergence.

- SGCS margin < 0.05 indicates high - degree of convergence.

Proposal 4: SGCS results for Track 1 with encoder model structure under current assumption are provided in Table 1.

- Table 1. SGCS results for Track 1 with encoder model structure under current assumption

|  | Track 1: Training @ Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Testing @ Huawei, Hisilicon |  | Testing @CATT |  | Testing @QC |  | Testing @Vivo |  | Testing @Mixed |  |
|  | Unquantized | Quantized | Unquantized | Quantized | Unquantized | Quantized | Unquantized | Quantized | Unquantized | Quantized |
| Huawei, Hisilicon decoder | 0.6843 | 0.66 | - | - | - | - | - | - | - | - |
| Samsung decoder fixed | 0.5779 | 0.5235 | 0.4505 | 0.4722 | 0.5059 | 0.4881 | 0.617 | 0.6191 | 0.6477 | 0.6364 |
| CATT decoder fixed | 0.5458 | 0.4707 | 0. 4281 | 0.3891 | 0.4645 | 0.3906 | 0.6148 | 0.5964 | 0.6329 | 0.6004 |

Proposal 5: SGCS results for Track 2 with encoder model structure under current assumption are provided in Table 1.

Table 2. SGCS results for Track 2 with encoder model structure under current assumption

|  | Training @ mixed, testing @ mixed |  | Training @ Huawei, Hisilicon, testing @ mixed |  |
| --- | --- | --- | --- | --- |
|  | Unquantized | Quantized | Unquantized | Quantized |
| Huawei, Hisilicon decoder | 0.76 (Joint training) | 0.71 (Joint training) | - | - |
| Samsung decoder fixed | 0.7556 (encoder retraining) | 0.7029 (encoder retraining) | 0.7085 (encoder retraining) | 0.6673 (encoder retraining) |

Observation 3: In track 2, nearly no SGCS performance loss with fixed selected decoder using mixed dataset to retrain the encoder, assuming the encoder model structure is aligned between training and retraining.

Observation 4: In track 2, around 0.05 SGCS performance loss is observed with fixed selected decoder using our own dataset to retrain the encoder.

Proposal 6: Use track 2 to move forward by both considering encoder retraining under mixed dataset and companies own dataset.

Proposal 7: SGCS results for lower-complexity CNN and transformer encoder are provided in Table 3.

Table 3. SGCS results for lower-complexity CNN and transformer encoder retraining

|  | Training/Testing @ mixed, retraining |  |
| --- | --- | --- |
|  | Unquantized | Quantized |
| Assumed CNN-based encoder (94M FLOPs) Joint training, no retraining | 0.76 | 0.71 |
| CNN-based encoder (4.14M FLOPs) Encoder retraining | 0.73 | 0.68 |
| Transformer-based encoder (3.14M FLOPs) Encoder retraining | 0.64 | 0.62 |
| Transformer-based encoder (3.14M FLOPs) Joint training, no retraining | 0.72 | 0.68 |

Observation 5: Under the same backbone, 0.03 SGCS performance loss is observed for encoder retraining with a lower-complexity CNN-based model structure.

Observation 6: Under a different backbone from current CNN-based assumption, around 0.1 SGCS is observed for encoder retraining with a lower-complexity Transformer-based model structure.

Proposal 8: Use lower complexity encoder model structure for testing/reference model structure alignment if AI-CSI turns to WI.

Observation 7: Around 0.05 SGCS performance loss can be observed in Track 1 under mixed testing dataset.

Observation 8: Around 0.1 SGCS performance loss can be observed in Track 1 under companies’ individual dataset.

References

- R4-2503953, “WF on AI/ML air interface study”, 3GPP RAN4 #114, Athens, Greece, 17th ‒ 24th February, 2025.

- R4-2410570, “WF on AI/ML”, 3GPP RAN4 #112, May 20th – 24th, 2024.

- R4-2420513, “WF on AI/ML air interface part 1”, 3GPP RAN4 #113, Orlando, USA 18th ‒ 22nd November, 2024.