3GPP TSG-RAN WG4 Meeting #114bis	R4-2504329

Wuhan, China, 7th – 11th April, 2025

Agenda Item:	7.1.3

Source:	Ericsson

Title:	On UE demodulation and CSI reporting requirements for 6Rx

Document for:	Discussion

# 1	Introduction

Following the agreements made in the RF session below, the following UE demodulation and CSI reporting requirements will be defined but applied to handheld UE and FWA devices separately.

Issue 2-1-1: Way forward for 6-layers support for handheld UE and FWA devices Way Forward: - RAN4 Chair to take note of the following in the meeting report: - “RAN4 unable to conclude on a common set of simulation assumptions to determine feasibility of 6 layer for handheld UE In Rel-19” - 6-layer performance requirements will be only defined for FWA. - 4-layer performance requirements will be defined and will apply to FWA and handheld UE devices. - The fact that 6-layer is only supported for FWA shall be explicitly captured at least in the RAN4 specifications.

In this paper, we present our view on UE demodulation and CSI reporting requirements for enhanced 6Rx in FR1. Further RF enhancements are being discussed, while in this paper, we will provide our views and which parameters to propose in this first meeting discussing the UE performance part. In RP-240828 [1], the objective for UE demodulation with 6Rx has been stated as follow.

### 4.2 Objective of Performance part WI NOTE: Leave empty if the WI proposal does not contain a RAN performance part. The objectives of performance part for UE RF enhancements for NR FR1 and FR2 Phase 4 include: 6Rx for handheld and FWA UE - Specify the performance requirements to enable 6Rx on higher frequency bands (>2.5GHz) targeting at support of smartphone for NR FR1 single carrier scenario - Investigate and if necessary, specify RLM test cases to support 6Rx - Specify UE PDSCH demodulation performance and CSI requirements to support 6 Rx - Support 4 MIMO layers at least, and follow the conclusion in core part for whether to support 6 MIMO layers - Specify the SDR requirements to support 6Rx

# 2	Discussion

Following the guidance in WID [1], we divided our discussion into three different subclauses to further discuss the parameter for PDSCH, CSI reporting and SDR requirements.

## 2.1	PDSCH demodulation requirements

As RAN4 has already defined PDSCH demodulation requirements for 8Rx in Rel-18, we are of the opinion that most of the agreements and conclusions can be reused to 6Rx demodulation, CSI reporting and SDR requirements. Based on the instruction in the WID [1], the 6Rx requirements is for higher frequency bands (>2.5GHz) targeting NR FR1 single carrier scenario. Therefore, we propose not to define CA requirements for 6Rx.

Proposal 1: Not to define CA requirements for 6Rx

For the MIMO layer, we propose to define requirements for both 4 layers and 6 layers. 4-layer cases are applied to both handheld UEs and FWA. 6-layer case will only be applied to FWA. Additional applicability rule can be discussed in a later stage.

Proposal 2: Define PDSCH test cases and requirements for both 4 layers and 6 layers.

Accordingly, we propose the following antenna configurations for PDSCH requirements to consider both low and medium B antenna correlation to better fit different UE types.

Proposal 3: Consider following antenna configurations:

- 4T6R ULA low (for FWA) and ULA medium B (for handheld)

- 8T6R ULA low (for FWA)

For the MCS configuration, we propose to reuse the agreements for 8Rx and consider MCS17 for rank 4 tests, MCS13 for rank 6 tests.

Proposal 4:

- Rank 4: MCS17

- Rank 6: MCS13

For the propagation channel, we propose to consider TDLC300-100 on top of reusing the TDLA30-10 model since we should cover the mobility characteristic of handheld UEs.

Proposal 5: Consider both TDLC300-100 and TDLA30-10

- TDLC300-100 for handheld case

- TDLA30-10 for both handheld and FWA cases

For other parameters we propose to reuse the agreements for 8Rx PDSCH requirements.

Proposal 6: Reuse other parameter configurations for Rel-18 8Rx

Based on previous experience on 8Rx WI in Rel-18, there could be different UE receiver types when receiving antenna is over 4, e.g. simplified receiver (3Rx + 3Rx), baseline receiver (6Rx). Thus, we propose to evaluate the performance for different UE receiver types and decide further whether to consider more than one set of requirements.

Proposal 7: Evaluate the performance of different UE receiver types if any

## 2.2	CSI reporting requirements

Similar to Rel-18 8Rx, it is necessary to define CQI reporting requirements for both handheld UEs and FWA UEs with 6Rx. Following main assumptions are proposed as follows:

- Rank: 4

- Propagation channel:  AWGN

- Antenna: 4T6R with static channel specified in Annex B.1

Reuse other parameter assumptions for 8Rx CSI reporting requirements.

In general, we propose to define one case each for FDD and TDD with above assumptions and apply it to both handheld UEs and FWA UEs, which means there is no need to define additional applicability rule for 6Rx CQI reporting requirements.

Proposal 8: Consider following parameter assumptions:

- Rank: 4

- Propagation channel:  AWGN

- Antenna: 4x6 with static channel specified in Annex B.1

Proposal 9: RAN4 to only define one CQI reporting requirement applying to both handheld UEs and FWA UEs without additional applicability rule.

## 2.3	SDR requirements

The purpose of the SDR requirements is to verify that Layer 1 and Layer 2 correctly process in a sustained manner the received packets corresponding to the maximum data rate indicated by UE capabilities. Following the same approach of Rel-18 8Rx, we propose to evaluate 6Rx SDR performance in the first stage with both 4 and 6 MIMO layers.

SDR requirements can be defined for 6 MIMO layers, and the MCS index table should be extended accordingly.

Proposal 10: Define SDR requirements and extend the MCS index table to 6 MIMO layers, considering

- Update Table 5.5A-5 to support ‘Maximum number of PDSCH MIMO layers = 6’

  - RAN4 to study the performance of 6 MIMO layers with different maximum modulation format

- Update Table 5.5A-6 (1024QAM) to support 6Rx.

  - RAN4 to study the Maximum number of PDSCH MIMO layers

  - RAN4 to study the maximum MCS for scaling factor 1

## 2.4	Applicability

In order to apply defined requirements to handheld UEs and FWA separately, basically to apply 4-MIMO layer tests to handheld UEs and apply 6-MIMO layer tests to FWA, we propose to consider adding 6-layer support information into the existing UE capability ‘maxNumberMIMO-LayersPDSCH’. Currently, UE can report only 2, 4 and 8 for this capability. With reporting ‘6’ for this capability, it is 6-layer requirements can be applied.

Proposal 11: Add ‘6’ into the UE capability ‘maxNumberMIMO-LayerPDSCH’

## 2.5	Specification structure

As for the specification structure, we propose to add a new clause to include requirements for 6Rx just like what RAN4 did for 8Rx in Rel-18.

Proposal 12: Add a new clause to capture 6Rx requirements for both FDD and TDD

The following is one example of such new clause for FDD: 5.2.5 6RX requirements 5.2.5.1 FDD 5.2.5.1.1 Minimum requirements for PDSCH Mapping Type A The performance requirements are specified in Table 5.2.5.1.1-3 - Table 5.2.5.1.1-7, with the addition of test parameters in Table 5.2.5.1.1-2 and the downlink physical channel setup according to Annex C.3.1. The test purposes are specified in Table 5.2.5.1.1-1. Table 5.2.5.1.1-1: Tests purpose Purpose; Test index. Verify the PDSCH mapping Type A normal performance under 6 receive antenna conditions and with different channel models, MCSs and number of MIMO layers; 1-1, 2-1, 3-1, 4-1, 5-1. Table 5.2.5.1.1-2: Test parameters Parameter; Unit; Value. Duplex mode; FDD. Active DL BWP index; 1. PDSCH configuration; Mapping type; Type A. k0; 0. Starting symbol (S); 2. Length (L); 12. PDSCH aggregation factor; 1. PRB bundling type; Static. PRB bundling size; 2. Resource allocation type; Type 0. RBG size; Config2. VRB-to-PRB mapping type; Non-interleaved. VRB-to-PRB mapping interleaver bundle size; N/A. PDSCH DMRS configuration; DMRS Type; Type 1. Number of additional DMRS; 1. Maximum number of OFDM symbols for DL front loaded DMRS; 1 for rank <= 4 2 for rank > 4. Codebook configuration; CodebookType; typeI-SinglePanel. CodebookMode; 1. (CodebookConfig-N1,CodebookConfig-N2); (2,1) for 4Tx (4,1) for 8Tx. (CodebookConfig-O1,CodebookConfig-O2); (4,1). CSI-RS for tracking; First OFDM symbol in the PRB used for CSI-RS; l0 = 5 for CSI-RS resource 1 and 3 l0 = 9 for CSI-RS resource 2 and 4. Number of HARQ Processes; 4. The number of slots between PDSCH and corresponding HARQ-ACK information; 2. Table 5.2.5.1.1-3: Minimum performance for Rank 4 Test num.; Reference channel; Bandwidth (MHz) / Subcarrier spacing (kHz); Modulation format and code rate; Propagation condition; Correlation matrix and antenna configuration; Reference value. Fraction of maximum throughput (%); SNR (dB). Table 5.2.5.1.1-4: Minimum performance for Rank 6 Test num.; Reference channel; Bandwidth (MHz) / Subcarrier spacing (kHz); Modulation format and code rate; Propagation condition; Correlation matrix and antenna configuration; Reference value. Fraction of maximum throughput (%); SNR (dB). …

# 3	Summary

In this contribution, we shared our views for the demodulation requirement discussion for 6Rx covering both handheld UEs and FWA. Proposals are summarized as follows:

Proposal 1: Not to define CA requirements for 6Rx

Proposal 2: Define PDSCH test cases and requirements for both 4 layers and 6 layers.

Proposal 3: Consider following antenna configurations:

- 4T6R ULA low (for FWA) and ULA medium B (for handheld)

- 8T6R ULA low (for FWA)

Proposal 4:

- Rank 4: MCS17

- Rank 6: MCS13

Proposal 5: Consider both TDLC300-100 and TDLA30-10

- TDLC300-100 for handheld case

- TDLA30-10 for both handheld and FWA cases

Proposal 6: Reuse other parameter configurations for Rel-18 8Rx

Proposal 7: Evaluate the performance of different UE receiver types if any

Proposal8 : Consider following parameter assumptions:

- Rank: 4

- Propagation channel:  AWGN

- Antenna: 4x6 with static channel specified in Annex B.1

Proposal 9: RAN4 to only define one CQI reporting requirement applying to both handheld UEs and FWA UEs without additional applicability rule.

Proposal 10: Define SDR requirements and extend the MCS index table to 6 MIMO layers, considering

- Update Table 5.5A-5 to support ‘Maximum number of PDSCH MIMO layers = 6’

  - RAN4 to study the performance of 6 MIMO layers with different maximum modulation format

- Update Table 5.5A-6 (1024QAM) to support 6Rx.

  - RAN4 to study the Maximum number of PDSCH MIMO layers

  - RAN4 to study the maximum MCS for scaling factor 1

Proposal 11: Add ‘6’ into the UE capability ‘maxNumberMIMO-LayerPDSCH’

Proposal 12: Add a new clause to capture 6Rx requirements for both FDD and TDD

# References

[1]	RP-240828: New WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4, RAN4 chair (Huawei), RAN #103, Maastricht, Netherlands

# Annex

Parameter configurations and example test cases for information:

Table 5.2.5.1.1-2: Test parameters Parameter; Unit; Value. Duplex mode; FDD. Active DL BWP index; 1. PDSCH configuration; Mapping type; Type A. k0; 0. Starting symbol (S); 2. Length (L); 12. PDSCH aggregation factor; 1. PRB bundling type; Static. PRB bundling size; 2. Resource allocation type; Type 0. RBG size; Config2. VRB-to-PRB mapping type; Non-interleaved. VRB-to-PRB mapping interleaver bundle size; N/A. PDSCH DMRS configuration; DMRS Type; Type 1. Number of additional DMRS; 1. Maximum number of OFDM symbols for DL front loaded DMRS; 1 for rank <= 4 2 for rank > 4. Codebook configuration; CodebookType; typeI-SinglePanel. CodebookMode; 1. (CodebookConfig-N1,CodebookConfig-N2); (2,1) for 4Tx (4,1) for 8Tx. (CodebookConfig-O1,CodebookConfig-O2); (4,1). CSI-RS for tracking; First OFDM symbol in the PRB used for CSI-RS; l0 = 5 for CSI-RS resource 1 and 3 l0 = 9 for CSI-RS resource 2 and 4. Number of HARQ Processes; 4. The number of slots between PDSCH and corresponding HARQ-ACK information; 2. Table 5.2.5.1.1-3: Minimum performance for Rank 4 Test num.; Reference channel; Bandwidth (MHz) / Subcarrier spacing (kHz); Modulation format and code rate; Propagation condition; Correlation matrix and antenna configuration; Reference value. Fraction of maximum throughput (%); SNR (dB). 1-1; xxx; 10 / 15; MCS17; TDLA30-10; 4x6, ULA medium B; 70; xxx. 1-2; xxx; 10 / 15; MCS17; TDLC300-100; 4x6, ULA medium B; 70; xxx. 1-3; xxx; 10 / 15; MCS17; TDLA30-10; 4x6, ULA Low; 70; xxx. Table 5.2.5.1.1-4: Minimum performance for Rank 6 Test num.; Reference channel; Bandwidth (MHz) / Subcarrier spacing (kHz); Modulation format and code rate; Propagation condition; Correlation matrix and antenna configuration; Reference value. Fraction of maximum throughput (%); SNR (dB). 2-1; xxx; 10 / 15; MCS13; TDLA30-10; 8x6, ULA Low; 70; xxx.

Draft LS to RAN2 to for adding information in maxNumberMIMO-LayerPDSCH:

3GPP TSG-RAN WG4 Meeting #xxx                                                                         R4-250xxxx

xxx, xxx, xx – xx xxxxxx, 2025

Title:	LS on UE Capabilities for maximum number of layers

Response to:

Release:	Release 19

Work Item:	NR_ENDC_RF_Ph4-Perf

Source:	RAN4

To:	RAN2

Cc:

Contact Person:

#### Name:	xxx

#### E-mail Address:	xxx

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments: -

# 1. Overall Description:

RAN4 respectfully asks RAN2 to add ‘6’ into the existing UE capability: ‘maxNumberMIMO-LayerPDSCH’ so that UE can report its support on 6-layer downlink transmission.

# 2. Actions:

To RAN2 group:

RAN4 respectfully asks RAN2 to take above into account.

# 3. Date of Next RAN4 Meetings:

RAN4#115	from 2025-05-19	to 2025-05-23		Malta, MT

RAN4#116	from 2025-08-25	to 2025-08-29		Bangalore, ID