3GPP TSG-RAN WG4 Meeting # 114-bis	R4-2504335

# Wuhan, China, 7th - 11th April, 2025

Title:	Discussion on IoT-NTN HPUE Tx requirements and SAR solution

Source:	Samsung

Agenda item:	7.10.2.3.1

Document for:	Approval

1	Introduction

In last #114 meeting, the WF[1] had progressed on determine ACLR, SEM, etc. And also the SAR solutions submitted by different companies had been presented and thoroughly discussed during the ad-hoc.

In this paper, we will continue to discuss the remaining open issues for Tx requirements of IoT-NTN and also the potential SAR solution other than P-MPR.

2	P-MPR

In [1], there was one open issue about the P-MPR for NB based IoT-NTN as follows

Issue 1-1-7: P-MPR definition for NB-Iot based Iot-NTN. - Proposals - For NB-Iot based Iot-NTN, discuss the proposal, confirm it is applicable for all the power classes, and discuss whether it can be defined from Rel-18. - Discuss whether P-MPR should be applied to handheld UEs only and whether this needs to be specified or not. - WF - Further discuss P-MPR definition

In our view, in previous TS 38.101-1, the definition for P-MPR is given as shown below, where it does not mention anything about the handheld or non-handheld.

P-MPRc is the power management maximum power reduction for a) ensuring compliance with applicable electromagnetic energy absorption requirements and addressing unwanted emissions / self desense requirements in case of simultaneous transmissions on multiple RAT(s) for scenarios not in scope of 3GPP RAN specifications; b) ensuring compliance with applicable electromagnetic energy absorption requirements in case of proximity detection is used to address such requirements that require a lower maximum output power. The UE shall apply P-MPRc for serving cell c only for the above cases. For UE conducted conformance testing P-MPRc shall be 0 dB NOTE 1: P-MPRc was introduced in the PCMAX,f,c equation such that the UE can report to the gNB the available maximum output transmit power. This information can be used by the gNB for scheduling decisions. NOTE 2: P-MPRc may impact the maximum uplink performance for the selected UL transmission path.

Observation 1: In TS 38.101-1, the P-MPR did not mention anything about handheld or non-handheld.

Besides, the EMF exposure regulation not only applies to the handheld device, but also to other portable devices or any devices designed to be used in proximity to human body. Having said that, it means non-handheld UE may face less challenges than handheld devices, the P-MPR can still be beneficial to the devices for the same EMF exposure requirements.

Observation 2: Both handheld and non-handheld devices have to comply with EMF exopsure (SAR) requirements. Non-handheld devices may have less challenges than handheld devices, but any solution, including P-MPR, can be beneficial for them for the same purpose.

Considering the observations above, we would like to suggest to not explicitly mention the P-MPR or any other SAR solutions if agreed can only be applied to handheld devices in the spec.

Proposal 1: To not explicitly limit the P-MPR or any other agreed SAR solutions in the spec for both NR and IoT-NTN, due to the EMF exposure (SAR) requirements is applicable to all types of devices, though the challenges can be different.

Proposal 2: To re-use the P-MPR definition in TS 38.101-1.

3	MOP tolerance

In last meeting, there’re two open issues related to the MOP tolerance for IoT-NTN, which are shown below:

Issue 1-1-8: The Basic MOP tolerance (Including eMTC and NB-Iot based) - Proposals - For PC2: - Option 1: +2 / -3dB - Option 2: +2 / -2dB - For PC1: - Option 1: +2 / -3dB - WF FFS Issue 1-1-9: On relaxing the upper limitation of nominal MOP or MOP tolerance for NTN HD-FDD operation. - Proposals - Option 1: Relaxation of TX MOP tolerance upper limitation for NTN HD-FDD EUTRA band; Class 2 (dBm); Tolerance (dB); Class 3 (dBm); Tolerance (dB). 256; 26; +2x, -2; 23; +2x, -2. 255; 26; +2x, -2; 23; +2x, -2. NOTE X: The upper power tolerance limit relaxation and related requirements in HD-FDD operation are specified in sub-clause [xxx].. - Option 2: Additional NW signal to control relaxation of TX MOP upper limitation for NTN HD-FDD. EUTRA band; Class 2 (dBm); Tolerance (dB); Class 3 (dBm); Tolerance (dB). 256; 26x; +2, -2; 23x; +2, -2. 255; 26x; +2, -2; 23x; +2, -2. NOTE X: The power limit relaxation and related requirements in HD-FDD operation are specified in sub-clause [xxx].. - Option 3: New sub power classes (e.g., PC2NTN-HD-FDD) to relax the TX output power upper limitation. - Option 4: Other ways are not excluded - FD-FDD can also be considered. - WF - FFS.

The argument is that the HD-FDD can have less insertion loss which would results in higher output power. And the proposals were to either relax the upper limit of the tolerance or to change the nominal power (e.g. to introduce a new power class).

Firstly, in our view, relaxing the upper limit of the tolerance seems to be one way forward, but it is not a good practice. The tolerance was designed to tolerate the performance fluctuation during mass production, but here the situation is the ’nominal’ power is increased. In our view, relaxing the upper tolerance may accomodate such increase of the output power from HD-FDD IoT devices, however it contradicts to the original meaning of the tolerance, which may cause confusions and inconsistency with current spec.

Secondly, to change the nominal power of the existing power class or introducing new power class may not fully align with the current scope of this WI. We are not fully confident nor convinced to agree on such approach to solve this issue.

Lastly, introducing a new signaling indication for such power relaxation seems redundant to us. We are not sure how would NW utilize such signaling and improve the performance in real network, if it’s only for compliance, we don’t think having additional signaling is worth to be implemented.

Proposal 2: To re-use +2/-3 for PC2 and PC1 for IoT-NTN MOP tolerance.

Proposal 3: To not introduce new power classes other than stated in the WID.

4	SAR solutions

In #114 meeting, the SAR solutions from different companies are thoroughly discussed, and the WF[1] is provided as follows:

SAR Related issues Issue 2-1-1: Whether and how to assume some scenarios like satellite orbits and traffic model etc. for the study of SAR issue. - Proposals - Option 1: No need. - [E.g. The scheme has to be general enough to cover various scenarios;] - Option 2: Needed. - [E.g. If typical scenarios do not have an issue may be the discussion is not needed.] - Option 3: Others WF (Ad-hoc agreement): - Option 1 Issue 2-1-3: Whether and how to introduce duty cycle related method. - Proposals (Not necessarily mutually exclusive): - Scheme 1: (vivo) Consider to introduce a scheme that UE voluntarily dropping uplink transmissions by self-assessment to control the SAR. This may be simple enough, can be controlled by network and still useful. - Scheme 2: (oppo) UE indicate whether the following Tx time duration should be counted into the UL duty cycle statistics in addition to current maxUplinkdutycycle capability. How the information is reported is up to RAN2. - Scheme 3: (Samsung) To use beam hopping information as the UE uplink duty ratio in time domain if duty cycle-based SAR solution is to be specified. And to discuss following impacts to existing duty cycle solution, considering our target is for UE to maintian its power class Scheme 3-1: UE to fall back power classes based on information above ( legacy duty cycle solution, not quite aligned with purpose to transmit as much and as long as possible) Scheme 3-2: UE to fall back several dB but not entire power class, based on information above ( technically UE can reach more power than Scheme 3-1) Scheme 3-3: UE to not fall back any power, but to monitor SAR by itself. ( In this case, UE may fail SAR compliance.) - Scheme 3-3a: UE to enter scheme like a ’DTx’, or Tx UL Gap for a certain duration. - Scheme 3-3b: UE to give up a certain ratio of uplink opportunities for a certain duration to recover its margin for SAR compliance. (Like above scheme 1 from vivo) - Scheme 3-3c: UE to fall back to lower power class for a certain duration to recover its margin for SAR compliance. - Scheme 4: (Ericsson, Sony) Both P-MPR and “duty-cycle” based method could be considered, but only if the “duty-cycle” based method is mandatorily accompanied with the ΔPPowerClass reporting, triggered upon uplink duty cycle exceedance or upon return to the power class after the duty cycle exceedance (already introduced in Rel-18 for TN). The same reporting mechanism shall also include the reporting of the actual P-MPR value. (Ericsson, Sony) If both the TN “duty-cycle” based method and the P-MPR method accompanied with the ΔPPowerClass reporting are considered for NTN UEs, “Single Entry PHR MAC CE” could be reused with the following reporting details: (Ericsson, Sony) - If the P-bit is set to ‘0’, power-class fallback reporting is used due to e.g. a duty cycle exceedance: - DPC = ‘00’: the criteria to report DPC is not met - DPC = ‘01’: ΔPPowerClass = 0 dB - DPC = ‘10’: ΔPPowerClass = 3 dB - DPC = ‘11’: ΔPPowerClass = 6 dB - If the P-bit is set to ‘1’, P-MPR method is used due to e.g. a duty cycle exceedance (where ΔPPowerClass is not actually applied in PCMAX,f,c): - DPC = ‘00’: P-MPR is applied due to a proximity sensor or in case of simultaneous transmissions on multiple RAT(s) for scenarios not in scope of 3GPP RAN specifications - DPC = ‘01’: 0 dB back-off when P-MPRc = 0dB - DPC = ‘10’: 3 dB back-off when 0 < P-MPRc ≤ 3 dB - DPC = ‘11’: 6 dB back-off when 3 < P-MPRc ≤ 6 dB - WF - Further discuss the proposed schemes related to duty cycle related SAR scheme. - Other schemes or no duty cycle related scheme are not precluded.

From the discussion, we have the following observations:

Observation 3-1: Legacy dutycycle method does not fit the purpose of keeping as much power as possible for NTN scenario, as legacy solution requires UE to fall back 3/6 dBs, which is the power of entire power class.

Observation 3-2: In Rel-19, NTN would not introduce any beam service ratio in time domain in NTN phase3 WI in parallel.

Based on these observations above, we believe that to let UE to drop certain ratio of uplink opportunities or to let UE enter a UL gap period to recover its SAR margin can be the potential solution, however they also have some disadvantages.

Observation 4-1: To let UE drop certain ratio of uplink opportunities will face the same challenge as other duty cycle method, because there is no NW uplink ratio in time domain in a FDD NTN band. It means if we follow this way, there is no way we could specify or to guide UE behaviors, like how much ratio of uplink should UE drop, and for how long.

Observation 4-2: To let UE go into a Tx UL Gap also faces same challenges as above.

In our view, if there’s no common design that can be specified in details, it means it can both guide the UE to act accordingly and to fulfill the SAR regulations, we’d rather to not specify anything when it’s not mature. P-MPR has served as a legacy method that leaves spaces for UE to implement to deal with SAR regulations, and we can live with that.

Proposal 4: To not define any duty cycle based SAR solution, and to use the baseline P-MPR method, considering following:

- 1) P-MPR had been working well and leave space for implementation;

- 2) without uplink duty ratio from NW side, current solutions on the table (including legacy tdd duty cycle solution) cannot be optimized and clear enough for UE implementation;

- 3) There are many non-3GPP technologies already implemented in actual SAR compliance, and it helped to maintain power level well enough.

5	Conclusion

For IoT-NTN HPUE Tx requirements and SAR solutions, we propose the following:

Observation 1: In TS 38.101-1, the P-MPR did not mention anything about handheld or non-handheld.

Observation 2: Both handheld and non-handheld devices have to comply with EMF exopsure (SAR) requirements. Non-handheld devices may have less challenges than handheld devices, but any solution, including P-MPR, can be beneficial for them for the same purpose.

Proposal 1: To not explicitly limit the P-MPR or any other agreed SAR solutions in the spec for both NR and IoT-NTN, due to the EMF exposure (SAR) requirements is applicable to all types of devices, though the challenges can be different.

Proposal 2: To re-use the P-MPR definition in TS 38.101-1.

Proposal 2: To re-use +2/-3 for PC2 and PC1 for IoT-NTN MOP tolerance.

Proposal 3: To not introduce new power classes other than stated in the WID.

Observation 3-1: Legacy dutycycle method does not fit the purpose of keeping as much power as possible for NTN scenario, as legacy solution requires UE to fall back 3/6 dBs, which is the power of entire power class.

Observation 3-2: In Rel-19, NTN would not introduce any beam service ratio in time domain in NTN phase3 WI in parallel.

Observation 4-1: To let UE drop certain ratio of uplink opportunities will face the same challenge as other duty cycle method, because there is no NW uplink ratio in time domain in a FDD NTN band. It means if we follow this way, there is no way we could specify or to guide UE behaviors, like how much ratio of uplink should UE drop, and for how long.

Observation 4-2: To let UE go into a Tx UL Gap also faces same challenges as above.

Proposal 4: To not define any duty cycle based SAR solution, and to use the baseline P-MPR method, considering following:

- 1) P-MPR had been working well and leave space for implementation;

- 2) without uplink duty ratio from NW side, current solutions on the table (including legacy tdd duty cycle solution) cannot be optimized and clear enough for UE implementation;

- 3) There are many non-3GPP technologies already implemented in actual SAR compliance, and it helped to maintain power level well enough.

6	Reference

[1] R4-2503006, WF on HPUE requirement for IoT-NTN, vivo