3GPP TSG-RAN WG4 Meeting #114bis	R4-2504345

Wuhan, Hubei, China, 7 April – 11 April 2025

Agenda Item:	7.10.4.2

Source:	Ericsson

Title:	UE demodulation requirements with time varying Doppler shift and propagation delay model

Document for:	Discussion

# 1	Introduction

RAN4#114 agreed with the way forward on the TE-emulated channel modeling for NGSO NTN testing [1]. In the WF, RAN4 agreed with the satellite propagator model to derive the time-varying Doppler shift and propagation delay.

According to the WID [2], one of the objectives in the performance part is to define UE demodulation requirements with the time-varying Doppler shift and propagation delay model discussed so far in this WI.

NTN testing for NGSO - Specify UE demodulation performance requirement(s) of PDSCH for NGSO for NR-NTN and IoT-NTN in the FR1-NTN bands and the corresponding LTE bands based on the above channel model - Minimize the number of demodulation performance requirements - Specify the applicability of the tests under the TE-emulated channel model

This contribution focuses on the UE demodulation performance requirements with the time varying Doppler shift and propagation delay.

# 2	NGSO NTN test environment

Figure 1	Assumed test environment for NGSO NTN UE testing.

Figure 1 illustrates an overview of the NGSO NTN test environment. During the test, DUT (UE) estimates the Doppler shift and propagation delay based on the satellite location/velocity (ephemeris) information and UE’s location information at a given time. In the DUT, the UE location information is pre-configured or provided via AT command, as specified in TS 38.508 (See C.1). The estimated Doppler shift and propagation delay are used to perform the pre-compensation before the baseband processing in the DUT.

Figure 2 shows an example of Doppler shift and propagation delay using LEO-600 maximum positive Doppler test parameters where the initial evaluation angle is 30 degrees.

Figure 2	Doppler shift and propagation delay for LEO-600 maximum positive Doppler scenario.

# 3	UE demodulation requirements

## 3.1	Doppler shift and propagation delay estimation error

As we discussed so far, it is assumed UE can acquire the satellite ephemeris information every 10.24 seconds for UE demodulation requirements. Although the ephemeris information is provided every 10.24 seconds, the applied Doppler shift and propagation delay is changed every slot. Therefore, UE need to continue to update the Doppler shift and propagation delay until it receives the new ephemeris information. RAN4#114 agreed that it is assumed that UE use Keplerian model or Equation of motion to derive the satellite location/velocity information [1].

Consider Keplerian model and Equation of Motion models in UE side as baseline assumption for RAN4 requirement - Note: There is no limitation for UE implementation to estimate the doppler shift and propagation delay based on the signalled ephemeris information

In this contribution, we have investigated the Doppler shift and propagation delay estimation error if UE does NOT use the satellite propagator model, that is,

- UE receives the satellite ephemeris information every 10.24 seconds.

- UE estimates the Doppler shift and propagation delay from the ephemeris information.

- UE does not run the satellite propagator model.

- UE updates the Doppler shift and propagation delay every slot by interpolating until UE receives the new ephemeris information.

Figure 3 and Figure 4 show the interpolated Doppler shift and propagation delays at UE, respectively, with LEO-600 maximum positive Doppler scenario.

These figures show three interpolation methods: linear (1st order) interpolation, quadratic (2nd order) interpolation, and cubic spline interpolation. The lower figures in Figure 3 and Figure 4 show the difference (mismatch) of Doppler shift and propagation delay between TE and UE. Note it is assumed the TE updates the satellite position/velocity every 1.00 second by running the satellite propagation model, and then calculates Doppler shift and propagation delay. Between the satellite updates, the TE generates the Doppler shift and propagation delay with the cubic spline interpolation. The lower right figures highlight the difference among three interpolation methods.

Figure 3	Doppler shift estimation in UE (LEO-600 maximum positive Doppler scenario).

Figure 4		Propagation delay estimation in UE (LEO-600 maximum positive Doppler scenario).

It is observed from the figures that the errors with linear interpolation are ~70 Hz for Doppler shift and ~3.5 us for propagation delay. We think it is NOT negligible considering the Tx frequency error of 0.1ppm (=200Hz @ 2GHz) and the cyclic prefix of 4.7us for SCS=15kHz.

On the other hand, if we assume UE uses the quadratic interpolation or cubic spline interpolation, the mismatch from TE is very small, that is, ~2Hz/0.024us for the quadradic interpolation and <<1Hz/0.01us for the cubic spline interpolation methods.

Since RAN4 agreed that UE also uses the satellite propagator model, e.g., Keplerian model, to estimate the satellite position/velocity vectors, we expect the estimated Doppler shift and propagation delay are well aligned with those applied by TE.

Observation 1: The mismatch of Doppler shift and propagation delay estimation between TE and UE could be less than 2Hz and 0.024us, respectively, if UE receives the ephemeris information every 10.24 seconds.

## 3.2	NR NTN UE demodulation requirements

In Rel-17, RAN4 defined four new PDSCH demodulation requirements for FR1-NTN as shown in Table 1.

Table 1	PDSCH test cases for FR1-NTN in TS 38.101-5 Table 8.2.1.2.2.1.1-3.

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | R.PDSCH.1-1.1 FDD | 10 / 15 | QPSK, 0.30 | NTN-TDLA100-200 | 1x2, ULA Low | 70 | 0.3 |
| 1-2 | R.PDSCH.1-2.1 FDD | 10 / 15 | 16QAM, 0.48 | NTN-TDLC5-200 | 1x2, ULA Low | 70 | 7.6 |
| 1-3 | R.PDSCH.1-1.1 FDD | 10 / 15 | QPSK, 0.30 | NTN-TDLC5-200 | 1x2, ULA Low | 70 | -0.4 |
| 1-4 | R.PDSCH.1-1.1 FDD(1) | 10 / 15 | QPSK, 0.30 | NTN-TDLA100-200 | 1x2, ULA Low | 70 | 1.1 |
| Note1: The Maximum throughput is based on the HARQ processes with HARQ feedback enabled. |  |  |  |  |  |  |  |

As we discussed in 3.1, as far as the TE provides the satellite ephemeris information every 10.24 seconds and generates the Doppler shift and propagation delay based on the signalled ephemeris information, the estimation errors of Doppler shift and propagation delay can be less than 2Hz and 0.024us, respectively. We think these values are small enough compared with the UE Tx frequency error requirements of 0.1ppm (=200Hz@2GHz) and cyclic prefix of 0.47us (SCS=15kHz). Moreover, when RAN4 define the PDSCH demodulation requirements, RAN4 first average companies’ impairment results and then add the additional margin 0.5 to 1.0dB according to the modulation order. Therefore, we propose to keep the same requirements without extra margin even if the time-varying Doppler shift and propagation delay model is applied.

Proposal 1: Keep the same NR NTN PDSCH requirements (SNR) even if the time-varying Doppler shift and propagation delay model is applied.

Regarding the channel model applicability, we think the new model should apply to at least tests 1-1 and 1-2 above. We may not need to apply to tests 1-3 and 1-4, because the tests 1-3 and 1-4 are derived from test 1-1, where test 1-3 configures 32 HARQ processes and 1-4 applies HARQ retransmission disabling.

Proposal 2: Apply the time-varying Doppler shift and propagation delay model with LEO-600 30 degrees elevation angle condition to TS 38.101-5 Clause 8.2.1.2.2.1 Test 1-1 and Test 1-2.

In the existing PDSCH demodulation requirements, it is configured cellSpecificKoffset-r17 (Koffset) is set to 8 slots. According to RAN1 agreements, Koffset should be larger than the maximum round-trip time (RTT). In our understanding, Koffset=8 is set based on the assumption the one-way delay is 2 slots (2ms).

| cellSpecificKoffset-r17 (Note 1) | Slots/15kHz | 8 |
| --- | --- | --- |

If we apply the time-varying propagation delay model with LEO-600 30 degrees evaluation angle condition, the one-way delay varies from 2.0 to 3.6ms. Although Koffset=8 may not be suitable when the propagation delay from the satellite to UE is 3.6ms, we propose to keep Koffset=8, because the parameter Koffset specify the slot offset for the HARQ-ACK feedback transmission, and it is independent of propagation delay estimation.

Proposal 3: RAN4 keep cellSpecificKoffset-r17 = 8 even if the time-varying propagation delay model is used.

## 3.3	IoT NTN UE demodulation requirements

In Rel-18, RAN4 has defined two NB-IoT NPDSCH demodulation and three Cat-M1 PDSCH demodulation requirements for IoT-NTN as shown in Table 2 and Table 3, respectively.

Table 2	NPDSCH test case for IoT NTN NB-IoT (in TS 36.102 Table 8.3.1.1.1.1-2)

| Test number | Bandwidth | Carrier Type | Reference Channel | Repetition number | Propagation condition | Number of NRS ports | Antenna Configuration | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |
| 1 | 200kHz | Anchor | R.NB.1 FDD | 32 | NTN-TDLC5-200 | 1 | 1x1 | 70% | -4.7 | NB1, NB2 |
| 2 | 200kHz | Non-anchor | R.NB.2 FDD | 128 | NTN-TDLA100-10 | 1 | 1x1 | 70% | -10.6 | NB1, NB2 |

Table 3	PDSCH test case for IoT NTN Cat-M1 (in TS 36.102 Table 8.2.1.1.1.1-2)

| Test number | Bandwidth and MCS | Reference Channel | OCNG Pattern | Propagation Condition | Correlation Matrix and Antenna Configuration | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |
| 1 | 1.4MHz 16QAM 1/2 | R.1 FDD | OP.1 FDD | NTN-TDLC5-30 | 1x1 | 70 | 10.4 | M1 |
| 2 | 1.4MHz QPSK 1/3 | R.2 FDD | OP.1 FDD | NTN-TDLA100-200 | 1x1 | 70 | -4.2 | M1 |
| 3 | 1.4MHz QPSK 1/10 | R.3 FDD | OP.1 FDD | NTN-TDLA100-10 | 1x1 | 70 | -11.5 | M1 |

One of the differences from NR NTN PDSCH demodulation requirements is the NB-IoT/Cat-M1 UE demodulation requirements assume the PDSCH/NPDSCH are transmitted with repetitions considering the coverage enhancement, and this feature makes the test time longer.

The latest RAN5 conformance test specification has defined the minimum test time for PDSCH for UE Category NB1 in TS 38.521-4 Table G.3.5-1. According to the requirements, NB-IoT test 1 requires 243,360 subframes (=244 sec) and test 2 requires 602,496 subframes (~603 sec). As shown in 3.1, the maximum simulation time for LEO-600 30 degrees elevation angle condition is around 250 seconds. This means test 1 can be covered by LEO-600 model. However, since the test time of 244 seconds is the minimum test time, we may need to consider LEO-1200 model because the test time may exceed 250 seconds.

For test 2, since the minimum test time is 603 seconds, both the LEO-600 and LEO-1200 models are not sufficient to cover the test time for test 2. As we discussed in RAN4#114, we think it is enough to apply the time-varying Doppler shift and propagation delay model to test 1 only, because test 2 assumes deep coverage scenario and it could be the GSO satellite scenario.

Proposal 4: Apply the time-varying Doppler shift and propagation delay model with LEO-600 30 degrees elevation angle condition to TS 36.102 Clause 8.3.1.1 Test 1. If the LEO-600 model is not enough to cover the test time of Test 1, RAN4 should consider using the LEO-1200 model.

Regarding Cat-M1, although RAN5 has not specified the minimum test time, we propose to apply the time-varying Doppler shift and propagation delay model to tests 1 and 2 only, because test 3 targes Cat-M1 CE Mode B, which is operated in deep coverage area.

Proposal 5: Apply the time-varying Doppler shift and propagation delay model with LEO-600 30 degrees elevation angle condition to TS 36.102 Clause 8.2.1.1 Test 1 and Test 2. If the LEO-600 model is not enough to cover the test time of Tests 1/2, RAN4 should consider using the LEO-1200 model.

Regarding the requirements, same as NR NTN UE demodulation requirements, we think the existing requirements can be reused without adding extra margin.

Proposal 6: Keep the same IoT NTN UE demodulation requirements (SNR) even if the time-varying Doppler shift and propagation delay model is applied.

## 3.4 Impact to the specification

According to our proposal, RAN4 apply the time-varying Doppler shift and propagation delay model for selected UE demodulation requirements by reusing the existing requirements. For this purpose, RAN4 only need to add a new applicability description which test cases use the time-varying Doppler shift and propagation delay model.

Proposal 7: Add new applicability description which test cases use the time-varying Doppler shift and propagation delay model in Rel-19 TS 36.102 and TS 38.101-5.

# 4	Summary

Observations:

Observation 1: The mismatch of Doppler shift and propagation delay estimation between TE and UE could be less than 2Hz and 0.024us, respectively, if UE receives the ephemeris information every 10.24 seconds.

Proposals:

Proposal 1: Keep the same NR NTN PDSCH requirements (SNR) even if the time-varying Doppler shift and propagation delay model is applied.

Proposal 2: Apply the time-varying Doppler shift and propagation delay model with LEO-600 30 degrees elevation angle condition to TS 38.101-5 Clause 8.2.1.2.2.1 Test 1-1 and Test 1-2.

Proposal 3: RAN4 keep cellSpecificKoffset-r17 = 8 even if the time-varying propagation delay model is used.

Proposal 4: Apply the time-varying Doppler shift and propagation delay model with LEO-600 30 degrees elevation angle condition to TS 36.102 Clause 8.3.1.1 Test 1. If the LEO-600 model is not enough to cover the test time of Test 1, RAN4 should consider using the LEO-1200 model.

Proposal 5: Apply the time-varying Doppler shift and propagation delay model with LEO-600 30 degrees elevation angle condition to TS 36.102 Clause 8.2.1.1 Test 1 and Test 2. If the LEO-600 model is not enough to cover the test time of Tests 1/2, RAN4 should consider using the LEO-1200 model.

Proposal 6: Keep the same IoT NTN UE demodulation requirements (SNR) even if the time-varying Doppler shift and propagation delay model is applied.

Proposal 7: Add new applicability description which test cases use the time-varying Doppler shift and propagation delay model in Rel-19 TS 36.102 and TS 38.101-5.

# References

- R4-2502314, “Way Forward for [114][325] NTN_testing_NGSO_channel_model”, Samsung.

- RP-240857, “New WID: Enhanced requirements and test methodology for NR and IoT NTN”, RAN4 chair.

# Appendix

## A.1	Minimum Test time for PDSCH (TS 38.521-5)

Table G.1.5-1: Minimum Test time for PDSCH demodulation (TS38.521-5)

| Test number | Reference Channel | Propagation condition | Minimum number of active subframes (MNAS) | MNAS to MNS Scaling factor (Note 3) | Minimum Number of Subframes (MNS) after rounding up to nearest thousand MNS= |
| --- | --- | --- | --- | --- | --- |
| 1-1 | R.PDSCH.1-1.1 FDD | NTN-TDLA100-200 | 30000 (Note 1) | 1.0526 | 32000 |
| 1-2 | R.PDSCH.1-2.1 FDD | NTN-TDLC5-200 | 72000 (Note 1) | 1.0526 | 76000 |
| 1-3 | R.PDSCH.1-1.1 FDD | NTN-TDLC5-200 | 80000 (Note 1) | 1.0526 | 85000 |
| 1-4 | R.PDSCH.1-1.1 FDD | NTN-TDLA100-200 | 30000 (Note 1) | 1.0526 | 32000 |
| Note 1: MNAS determined by simulations. Note 2: For cases where MNAS is not determined by simulations, use same MNAS as the similar case simulated (same doppler speed) Note 3: MNS/MNAS ratio decided by scheduling pattern and is ratio of all slots to DL slots. |  |  |  |  |  |

## A.2	Minimum Test time for NPDSCH (TS 36.521-4)

Table G.3.5-1: Minimum Test time for PDSCH for UE Category NB1

| Test No | Demodulation scenario (info only) | MNAS (Simulation) | Min Gap between consecutive NPDCCH taking into account RTT (ms) | Actual Gap between consecutive NPDCCH taking into account RTT (ms) | MNSF (Min No Sub frames) HD-FDD (Note2) |
| --- | --- | --- | --- | --- | --- |
| 8.3.1.1.1 Test 1 | R.NB.1 FDD (200kHz, QPSK,1/2) (1x1) NTN TDLC5-200 | 845 (Note 1) | 241 | 288 | 243360 |
| 8.3.1.1.1 Test 2 | R.NB.2 FDD (200kHz, QPSK,1/3) (1x1) NTN TDLA100-10 | 523 (Note 1) | 881 | 1152 | 602496 |
| 8.3.1.1.2 Test 1 | R.NB.6 FDD (200kHz, QPSK,1/2) (1x1) EPA5 | 683 (Note 1) | 489 | 576 | 393408 |
| 8.3.1.1.2 Test 2 | R.NB.6-1 FDD (200kHz, QPSK,1/2) (1x1) ETU1 | 440 (Note 1) | 1982 | 2304 | 1013760 |
| Note1: Minimum test time has been derived by simulating the test scenario taking into account the minimum scheduling gap needed between successive NPDCCH/NPDSCH Note2: For MNSF calculations, with the current assumption of SIB2/SIB31 scheduling every 640ms with max 8 subframes and max 2 repetition per SI window, 16 additional subframes are needed every 640ms to avoid collisions with NPDSCH. |  |  |  |  |  |