3GPP TSG-RAN WG4 Meeting #114bis	R4-2504347

Wuhan, Hubei, China, 7 April – 11 April 2025

Agenda Item:	7.3.4

Source:	Ericsson

Title:	UE demodulation requirements for CA including 3MHz CBW

Document for:	Discussion

# 1	Introduction

RAN#107 completed the WI NR channel BW less than 5MHz for FR1 Phase 2 and approved the WI summary [1]. RAN#107 also approved the revised WID [2] where TS 38.101-4 was included as an impacted specification, which means RAN4 will discuss the necessary UE demodulation and CSI reporting requirements. This contribution discusses the initial view on the UE demodulation performance requirements for CA including 3MHz CBW.

# 2	Discussion

According to the objective specified in WID, this WI targets to specify the UE capability to allow CA/DC of 3MHz or 5MHz CBW in one band (e.g., n100) and 5MHz or 10MHz CBW in the other bands (e.g., n101) [2].

- Define common co-located and non-co-located inter-band NR CA/DC UE RF requirements with 3MHz CBW in the one band and 5MHz or 10MHz CBW in the other band - Example band combination: CA/DC of 3MHz or 5MHz in band n100 and 5MHz or 10MHz in band n101 - NOTE: Consideration of 5MHz channel bandwidth in the above “3MHz or 5MHz” is specific to band n100 and does not apply to other bands.

RAN4#114 has approved the Big CR for less than 5MHz UE RF requirements [3], where TS38.101-1 specifies the new CA/DC combination of n100 and n101 as follows:

Table 1	NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands) ([3] Table 5.5A.3.1-1o).

| NR CA configuration | Uplink CA configuration or single uplink carrier10 | NR Band | Channel bandwidth (MHz) (NOTE 3) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n100A-n101A | CA_n100A-n101A | n100 | 3, 5 | 0 |
|  |  | n101 | 5, 10 |  |

Observation 1:	RAN4 has defined CA/DC n100+n101 with 3/5MHz CBW for n100 and 5/10MHz CBW for n101.

## 2.1	PDSCH demodulation requirements

TS 38.101-4 defines PDSCH CA demodulation requirements, and the requirements are designed to cover all the possible CA combination scenario supported in the UE RF requirements specifications, TS38.101-1/2/3. With the assumption that UE receives PDSCH for each component carrier (CC) in CA/DC, RAN4 specifies the PDSCH demodulation requirements per CC. For example, for FR1 FDD SCS=15kHz, RAN4 has defined PDSCH demodulation requirements with 5, 10, 15, 20, 25, 30, 35, 40, 45, and 50MHz.

In Rel-18, for the WI less than 5MHz, RAN4 concluded no new PDSCH demodulation requirements for 3MHz CBW, because it is assumed the UE supporting ‘Dedicated Spectrum of Less than 5MHz (support-3MHz-ChannelBW-r18)’ should support other band with 10MHz CBW. For Rel-19 WI Less than 5MHz CA, on the other hand, RAN4 has defined the CA combination of 3MHz CBW in n100 and 5/10MHz CBW as shown in Table 1. This means RAN4 need to define new PDSCH demodulation requirements covering the CA combination of 3MHz + 5/10MHz, and it could be a reason the revised WID included TS 38.101-4 as an impacted specification [2].

Since RAN4 has already defined PDSCH demodulation requirements for FDD SCS=15kHz 5MHz/10MHz CBW, we only need to define the PDSCH demodulation requirements for FDD SCS=15kHz 3MHz. Since Rel-18 defined the PDCCH/PBCH demodulation requirements for 3MHz for 2Rx/4Rx, we propose to define PDSCH requirements for 2Rx and 4Rx.

RAN4 has defined PDSCH CA demodulation requirements assuming several scenarios including high speed train (HST). We understand n100 3MHz CBW targets the railway communication system. However, to ensure the minimum requirements, we propose to define normal scenario only, that is, define 3MHz requirements only in TS 38.101-4 5.2A.2.

Proposal 1:	RAN4 define the 2Rx/4Rx PDSCH demodulation requirements with FDD SCS=15kHz 3MHz in TS 38.101-4 5.2A.2.1.

Based on the existing FRC defined in TS 38.101-4 Table A.3.2.1.1-9, we propose to use the simulation parameters shown in Table 2.

Table 2	Simulation parameters for PDSCH demodulation for 3MHz CC

| Band width | SCS | MCS | TBS | Propagation condition | Antenna configuration |
| --- | --- | --- | --- | --- | --- |
| 3MHz (12PRB) | FDD 15kHz | 64QAM, 0.48 | 6016 | TDLA30-10 | 2x2, ULA Low |
| 3MHz (12PRB) | FDD 15kHz | 64QAM, 0.48 | 6016 | TDLA30-10 | 2x4, ULA Low |

Proposal 2:	Use the simulation assumption shown in Table 2, i.e., FDD 15kHz, 12PRB, MCS19 Rank 2, TDLA30-10, 2x2/2x4 ULA Low.

According to TS 38.101-4 Table 5.2-1, PDCCH is configured with AL8, which corresponds to 48PRB (= 2 OFDM symbols x 24 PRBs). Since the maximum channel bandwidth of 3MHz CC is 12 PRB, it is not possible to configure AL8 even if 3 OFDM symbols are configured in CORESET. Therefore, we propose to configure AL4 for PDCCH with 2 OFDM symbols for 3MHz CC.

Proposal 3:	Configure AL4 for PDCCH for 3MHz carrier.

## 2.2	PDCCH/PBCH demodulation requirements

In Rel-18 less than 5MHz WI, RAN4 defined PBCH/PDCCH demodulation requirements with puncturing to fit to 3MHz CBW according to the RAN1 specification. In Rel-19, since RAN1 has not enhanced PBCH/PDCCH due to CA/DC operation, we don’t think it is necessary to add new PBCH/PDCCH demodulation requirements.

Proposal 4:	No new PDCCH/PBCH demodulation requirements for WI NR channel BW less than 5MHz for FR1 Phase 2.

## 2.3 CQI reporting requirements

TS 38.101-4 also specifies the CQI reporting requirements in static condition for CA scenario. Since the static CA CQI requirements are designed to apply to any channel bandwidth, it is applicable for even for 3MHz CBW. For example, the measurement channel configuration in Table 6.2A.3.1.1-1 and sub-band size configurations in Table 6.2A.3.1.1-2 consider any channel bandwidth specified in TS 38.101-1 Table 5.3.2. Therefore, we don’t see any reason to exclude 3MHz CBW from the CQI test in static condition.

Observation 2:	The existing CA CQI reporting requirements in static condition specified in TS 38.101-4 6.2A is applicable for any channel bandwidth.

Proposal 5:	Existing CA CQI reporting requirements specified in TS 38.101-4 6.2A.3.1.1 is applicable for CA 3MHz + 5/10MHz.

Like the PDSCH CA demodulation part, since the existing CA CQI requirements configure AL8 for PDCCH (See TS 38.101-4 Table 6.1.2-1), we need to add the configuration for 3MHz, e.g., AL4. Moreover, in Table 6.2A.3.1.1-2, the CQI reporting sub-band size does not include 3MHz. According to TS 38.214 5.2.1.4, if the UE is configured with a CSI Reporting Setting for a bandwidth part with fewer than 24 PRBs, the CSI reporting setting is expected to have a wideband frequency granularity. If we look TS 38.214 Table 5.2.1.4-2, RAN1 has not specified the table corresponding to 3MHz (=12PRB), which means we should not configure subbandSize.

Proposal 6:	For CA CQI reporting requirements, add the configuration parameters specific to 3MHz CBW, that is, AL4 for PDCCH in Table 6.2A.3.1.1-1, and no subbandSize configuration in Table 6.2A.3.1.1-2.

# 3	Summary

Observations:

Observation 1:	RAN4 has defined CA/DC n100+n101 with 3/5MHz CBW for n100 and 5/10MHz CBW for n101.

Observation 2:	The existing CA CQI reporting requirements in static condition specified in TS 38.101-4 6.2A is applicable for any channel bandwidth.

Proposals:

Proposal 1:	RAN4 define the 2Rx/4Rx PDSCH demodulation requirements with FDD SCS=15kHz 3MHz in TS 38.101-4 5.2A.2.1.

Proposal 2:	Use the simulation assumption shown in Table 2, i.e., FDD 15kHz, 12PRB, MCS19 Rank 2, TDLA30-10, 2x2/2x4 ULA Low.

Proposal 3:	Configure AL4 for PDCCH for 3MHz carrier.

Proposal 4:	No new PDCCH/PBCH demodulation requirements for WI NR channel BW less than 5MHz for FR1 Phase 2.

Proposal 5:	Existing CA CQI reporting requirements specified in TS 38.101-4 6.2A.3.1.1 is applicable for CA 3MHz + 5/10MHz.

Proposal 6:	For CA CQI reporting requirements, add the configuration parameters specific to 3MHz CBW, that is, AL4 for PDCCH in Table 6.2A.3.1.1-1, and no subbandSize configuration in Table 6.2A.3.1.1-2.

# References

- RP-250464, “WI summary: NR channel BW less than 5MHz for FR1 Phase 2”, Intel Corporation.

- RP-250781, “Revised WID: NR channel BW less than 5MHz for FR1 Phase 2”, Intel Corporation.

- R4-2502262, “Big CR for less than 5MHz UE RF requirements in Rel-19”, Intel Corporation.