3GPP TSG-RAN WG4 Meeting#114bis                                R4-2504354

Wuhan, China, 07th – 11th April, 2025

Agenda item:	7.24.2.2

Source: 	ZTE Corporation, Sanechips

Title: 	Discussion on RF requirement of Ambient IoT device

Document for:	Approval

Introduction

In the RAN#106 meeting, New WID on Rel-19 Ambient IoT [7] was approved and it’s expected to start the normative work from the Feb, 2025 meeting. In this contribution, we would like to share some further views based on the SI outcome and scope reached during RAN#106 meeting.

Issue 3-1-1: General Agreement: - Only define radiated RF requirements for device 1 in Rel-19 WI. The details will be further discussed. - Topic 3-2: Tx Issue 3-2-1: Transmit output power Agreement: - Measurement configuration including input CW power and potential other conditions need to be defined. Issue 3-2-2: Modulation quality requirements Agreement: - Further discuss whether and how to define the modulation signal quality requirements Issue 3-2-3: Occupied bandwidth Agreement: - Further discuss whether to define the occupied bandwidth for device 1. Issue 3-2-4: Emission requirements Agreement: - Consider one composite emission requirements covering the emission frequency range of SEM and spurious emission as starting point. - Further discuss whether to define TRP or EIRP requirement for unwanted emission. Issue 3-2-5: Other requirements Agreement: - Do not define output power dynamics and intermodulation requirements for device 1. Issue 3-2-6: Other requirements (frequency error) Agreement: - Do not define frequency error requirements. - Small frequency shift error will be discussed in modulation quality requirements. Issue 3-2-7: Other requirements (Transmit ON/OFF time mask) Agreement: - Do not define transmit ON/OFF time mask for device 1 - Topic 3-3: Rx Issue 3-3-1: Reference sensitivity Agreement: - For R2D reference sensitivity, consider following options: - Option 1: define reference sensitivity using legacy approach, e.g. PREFSENSdBm=−174dBm+10×log10BW+NF+IM+SNR - Option 2: define reference sensitivity based on link budget and/or physical limits of the receiver circuitry. - Other options are not precluded. Issue 3-3-2: Maximum input power Agreement: - Further discuss the maximum input power for A-IOT device for R2D signal. Issue 3-3-3: Interference rejection Agreement: - Do not define interference rejection requirements.

Discussion

- 2.1. RF requirement for Ambient IoT device

- In the following section, we would like to share some initial understandings on the RF requirements for A-IoT device.

- Table 1. The initial analysis for RF requirement of A-IoT device

| Tx part |  |
| --- | --- |
| Maximum output power/Output to input power gain within A-IoT carrier | For Device 1, from our understanding, it is highly dependent on maximum input power of CW signal and its backscattering gain according to its input CW signal e.g.-6dBi. Proposal 1: propose to discuss the maximum input power for CW signal and backscattering gain for device. |
| EVM requirement | EVM for backscattering OOK signal is not needed since OOK signal is not mapped by the legacy constellation, instead it’s reflected in the envelope level as following. The power stability or power accuracy for OOK ON signal and OOK OFF signal is important and also power difference between OOK ON and OOK OFF is essential to ensure the tag OOK detection performance. In addition, as pointed out in [3], RFID uses ASK modulation with PIE data encoding. The ASK modulation mask was defined as shown in Figure 2.4-2 below. The pulsewidth (PW), Modulation Depth ((A–B)/A), RF Envelope Ripple (M), RF envelope Rise time (tr,10–90), and RF envelope fall time (tf,10–90) are the major requirements. Figure 2-4-2: Interrogator-to-Tag RF envelope For A-IoT device EVM requirement, we propose to define the EVM requirement similar as RFID with OOK signal considered. For BPSK based D2R design,the legacy EVM requirement need also to be revisited. |
| OBW | The legacy OBW requirement could be used as starting point. |
| ACLR | This depends on further coexistence study. Considering the regional RFID requirement at 900MHz, both ACLR and SEM requirement are specified. |
| SEM | This depends on the outcome of coexistence study and regulatory requirement as part of input. Considering the regional RFID requirement at 900MHz, both ACLR and SEM requirement are specified. ETSI EN 302 208 V3.4.1 (2023-12) 4.3.5.3 low freq (2W power) |
| Transmitter spurious emission requirement | The legacy transmitter spurious emission requirement could be used as starting point. |
| Transmitter intermodulation requirements | No need to define the Tx intermodulation requirement. |
| Rx part |  |
| REFSENS | The REFSENS requirement might be not based on the throughput metric and it should be dependent on the miss detection ratio and false alarm detection ratio instead if without any HARQ-ACK feedback. e.g. -26dBm for RF energy harvesting, -36dBm for OOK detection. |
| Maximum input power | At least the following aspect needs to be considered: - Maximum input power for R2D signal reception e.g. with OOK signal detection with measurement metric as miss detection ration and false alarm detection ratio; - Maximum CW input power for D2R reflection. |

Conclusions

In this contribution, we shared some further views on the RF requirement for Ambient IoT device and proposals are made as following:

- Proposal 1: propose to discuss the maximum input power for CW signal and backscattering gain for device.

- Proposal 2: For A-IoT device EVM requirement, we propose to define the EVM requirement similar as RFID with OOK signal considered. For BPSK based D2R design,the legacy EVM requirement need also to be revisited.

- Proposal 3: The legacy OBW requirement could be used as starting point unless some OBW issues identified by the simulation.

- Proposal 4: for device emission mask, propose to consider ETSI EN 302 208 V3.4.1 (2023-12) 4.3.5.3 at low freq (2W power) as starting point.

References

- RP-240826,Revised SID: Study on solutions for Ambient IoT (Internet of Things) in NR (CMCC), Approved.

- R4-2410597, WF on impacts of A-IoT on RF requirements, Approved.

- R4-2414305,WF on RF requirements for A-IoT, Approved.

- R4-2414304, WF on Co-existence study for ambient IoT, Approved

- ISO/IEC 18046-3 Information technology – Radio frequency identification device performance test methods – Part 3: Test methods for tag performance

- R4-2502235,On the RF requirements for Ambient IoT Device, Huawei, Noted.

- R4-2502858, WF on requirements for A-IoT device, CMCC, approved.

- R4-2502859, WF on requirements for A-IoT BS and CW, Huawei, approved.