3GPP TSG-RAN WG4 Meeting#114bis                                R4-2504356

Wuhan, China, 07th – 11th April, 2025

Agenda item:	7.12.1.1

Source: 	ZTE Corporation, Sanechips

Title: 	Discussion on RF requirement for Inter-band CA for ATG UE in Rel-19

Document for:	Approval

Introduction

In the RAN#103 meeting, new WID on Enhancements for Air-to-ground network for NR has been approved and it’s expected to start the normative work from this April meeting. In this contribution, we would like to share some views on the inter-band CA related issues for ATG UE.

Topic #2: R19 UE RF for inter-band CA Issue 2-1: Assumed parameters when supporting n39 UL for CA_n3A-n39A with common antenna. Agreement: - Companies are encouraged to provide MSD values based on the following parameters. Parameters; Values. Antenna Isolation; 20dB. The rejection for band n3 Rx filter at 1885~1915MHz; [35dB]. n39 Tx filter rejection at 1805~1830MHz; [50dB]. - Above parameters are only applicable for ATG UE with dedicated filter design Issue 2-2: MSD for CA_n3A-n39A with n39 UL Agreement: - Companies are encouraged to provide MSD values based on the following Tables. - MSD analysis based on separated antenna are not precluded. - All of these MSD values are only applicable to ATG UE. - Further study on how to capture these MSD values to in the specification. UL band; DL band; UL Fc; UL BW; SCS of UL band; UL RB Allocation; DL Fc; DL BW; MSD; Cross-band Interference source. (MHz); (MHz); (kHz); LCRB; (MHz); (MHz); (dB). n39; n3; 1900; 30; 15; 160 (RBstart=0); 1817.5; 25; MSD Table X; >ACLR2. Table X CA_n3-n39 with band n39 UL MSD requirements for different output powers Output power; MSD (Based on Antenna Architecture with common antenna). 23dBm; TBD. 24dBm; TBD. …; …. 39dBm; TBD. 40 dBm; TBD. Issue 2-3: Assumed parameters for CA_n3A-n39A with n3 UL WF: - Use the following parameters in the approved Tdoc R4-2410641 with 20dB antenna isolation as baseline to analysis the MSD value for n39 DL. - Other parameters are not precluded. Parameters; Value. Antenna isolation; [20dB]. Tx attenuation at 1880~1920MHz for band n3 duplexer; [30dB]. Rx attenuation at 1710~1785MHz for band n3+n39 Rx filter; [40dB]. Issue 2-4: MSD for CA_n3A-n39A with n3 UL WF: - Companies are encouraged to provide MSD values based on the following Tables. - Other MSD test configurations are not precluded. - All of these MSD values are only applicable to ATG UE. - Further study on how to capture these MSD values to in the specification. UL band; DL band; UL Fc; UL BW; SCS of UL band; UL RB Allocation; DL Fc; DL BW; MSD; Cross-band Interference source. (MHz); (MHz); (kHz); LCRB; (MHz); (MHz); (dB). n3; n39; [1722.5]; [25]; [15]; [160 (RBstart=0)]; [1900]; [30]; MSD Table Y; [>ACLR2]. Table Y CA_n3-n39 with band n3 UL MSD requirements for different output powers Output power; MSD (Based on Antenna Architecture with common antenna). 23dBm; TBD. 24dBm; TBD. …; …. 39dBm; TBD. 40 dBm; TBD. Issue 2-5: whether preclude Case 4. Issue 1-1-2: UE antenna type for inter-band CA Agreement: - Refine the agreement from last meeting as: - For intra-band contiguous CA, same antenna type should be applied on a band. - For inter-band CA, either different or same antenna type can be applied on different bands. - For inter-band CA, consider the following cases: - 1. ATG UE with omnidirectional antennas on the bands of PCC and SCC. - 2. ATG UE with an omnidirectional antenna on the band of PCC and an antenna array on the band of SCC. - 3. ATG UE with an antenna array on the band of PCC and an omnidirectional antenna on the band of SCC. - [4. ATG UE with an antenna array on the bands of PCC and SCC, and one common Rx beam assumed.] - 5. ATG UE with an antenna array on the bands of PCC and SCC, and two simultaneous separate Rx beams assumed. Further check which understanding is correct: - Understanding #1: Case 4 is excluded in RF session. - Understanding #2: Case 4 is not explicitly discussed in RF session, since no case 4 specific RF requirement. Case 4 UE implementation is possible.. Agreement: - ATG UE with an antenna array on the bands of PCC and SCC, and one common Rx beam is possible for specific band combination. - Antenna array type for inter-band CA is highly depending on frequency range of operating bands and UE implementation. - Option 1: Case 4 UE implementation is possible for CA_n3-n39 that frequency ranges between two bands are close to each other. - Option 2: Case 4 UE implementation is possible for CA_n3-n39 that DL frequency separation between two bands are less than [X=FFS, e.g, 500 MHz].

Discussion

- 2.1. RF requirements for Inter-band CA with band 39 UL transmission

In this section, we provide some further analysis for DL CA_n3_n39 with UL band 39 transmission. Based on the reported UE capability for ATG UE ranging from 23dBm to 40dBm, the detailed MSD analysis are provided as following in Table 1 and Table 2.

Table 1. MSD analysis for ATG UE supporting inter-band DL CA with band 39 UL transmission as 23dBm

|  | parameters | n3 main path | n3 diversity path |
| --- | --- | --- | --- |
| transmit power for n39, dBm |  | 23 | 23 |
| Tx band n39 BW(MHz, Lcrb@15kHz) | 30 |  |  |
| RFFE loss, dB | 4 |  |  |
| Diplexer isolation at n3 uplink freq，dB | 0 |  |  |
| antenna isolation, dB | 20 |  |  |
| n3 received signal at ANT port, dBm |  | 23 | 3 |
| n3 filter rejection at 1885~1915MHz, dB | 50 |  |  |
| signal After n3 filter, dBm |  | -31 | -51 |
| Typical receiver IIP2, dB | 50 |  |  |
| TX IM2 noise level refer to RX LNA input, dBm |  | -112 | -152 |
| front-end loss | 4 |  |  |
| TX IM2 noise level at ANT port, dBm |  | -108 | -148 |
| noise figure dB | 12 |  |  |
| Thermal noise at RX ant port(dBm/Hz) | -162 |  |  |
| Rx band n3 BW(MHz, NRB) | 4.5 |  |  |
| Thermal noise, dBm |  | -95.47 | -95.47 |
| ACLR1, dB | 30 |  |  |
| ACLR2, dB | 43 |  |  |
| n39 PA leakage PSD at PA output port at 2nd adjacent channel, dBm/MHz |  | -34.77 |  |
| n39 PA noise PSD at PA output port, dBm/RxBW |  | -28.24 |  |
| n39 Tx filter rejection at 1805~1880MHz, dB | 50 |  |  |
| n39 PA noise power at Rx n3 ant port at 1825~1830MHz, dBm |  | -78.24 | -98.24 |
| Total noise level at ANT port |  | -78.15 | -93.63 |
| SNR requirement for QPSK | -1 |  |  |
| REFSENSE (referred to antenna)(5MHz BW) |  | -79.15 | -94.63 |
| Implementation Margin, dB | 2.5 |  |  |
| combined REFSENS(5MHz BW), dBm | -92.25 |  |  |
| MSD | 4.75 |  |  |

Table 2. MSD analysis for ATG UE supporting inter-band DL CA with band 39 UL transmission as 40dBm

|  | parameters | n3 main path | n3 diversity path |
| --- | --- | --- | --- |
| transmit power for n39, dBm |  | 40 | 40 |
| Tx band n39 BW(MHz, Lcrb@15kHz) | 30 |  |  |
| RFFE loss, dB | 4 |  |  |
| Diplexer isolation at n3 uplink freq，dB | 0 |  |  |
| antenna isolation, dB | 20 |  |  |
| n3 received signal at ANT port, dBm |  | 40 | 20 |
| n3 filter rejection at 1885~1915MHz, dB | 50 |  |  |
| signal After n3 filter, dBm |  | -14 | -51 |
| Typical receiver IIP2, dB | 50 |  |  |
| TX IM2 noise level refer to RX LNA input, dBm |  | -78 | -152 |
| front-end loss | 4 |  |  |
| TX IM2 noise level at ANT port, dBm |  | -74 | -148 |
| noise figure dB | 12 |  |  |
| Thermal noise at RX ant port(dBm/Hz) | -162 |  |  |
| Rx band n3 BW(MHz, NRB) | 4.5 |  |  |
| Thermal noise, dBm |  | -95.47 | -95.47 |
| ACLR1, dB | 30 |  |  |
| ACLR2, dB | 43 |  |  |
| n39 PA leakage PSD at PA output port at 2nd adjacent channel, dBm/MHz |  | -17.77 |  |
| n39 PA noise PSD at PA output port, dBm/RxBW |  | -11.24 |  |
| n39 Tx filter rejection at 1805~1880MHz, dB | 50 |  |  |
| n39 PA noise power at Rx n3 ant port at 1825~1830MHz, dBm |  | -61.24 | -81.24 |
| Total noise level at ANT port |  | -61.01 | -81.08 |
| SNR requirement for QPSK | -1 |  |  |
| REFSENSE (referred to antenna)(5MHz BW) |  | -62.01 | -82.08 |
| Implementation Margin, dB | 2.5 |  |  |
| combined REFSENS(5MHz BW), dBm | -79.62 |  |  |
| MSD | 17.38 |  |  |

Based on the above analysis, MSD for 23dBm to 40dBm transmission power would be ranging from 4.75dB to 17.38dB which will bring the significant performance degradation of band n3 DL reception.

Proposal 1: for the inter-band CA CA_n3-n39 with uplink transmission as n39, MSD requirement could defined as following:

| CA_n3-n39 with UL n39 | MSD |
| --- | --- |
| 23 | 4.76dB |
| 24 | 5.15dB |
| 25 | 5.59dB |
| 26 | 6.09dB |
| 27 | 6.64dB |
| 28 | 7.24dB |
| 29 | 7.90dB |
| 30 | 8.60dB |
| 31 | 9.36dB |
| 32 | 10.15dB |
| 33 | 10.97dB |
| 34 | 11.83dB |
| 35 | 12.71dB |
| 36 | 13.62dB |
| 37 | 14.54dB |
| 38 | 15.48dB |
| 39 | 16.43dB |
| 40 | 17.38dB |

- 2.2. RF requirements for Inter-band CA with band n3 UL transmission

For CA_n3-n39 with uplink transmission as n3, MSD requirement would be provided later which depends on filtering feasibility study.

- 2.3. Common UE antenna type for inter-band CA

Regarding the Rx antenna design for inter-band CA_n3-n39, we prefer to agree with option 1. Option 2 is kind of up-scoping from our understanding and we also don’t see the market demand and feasibility study for this freq gap limitation for this topic yet.

Proposal 2: for Rx antenna gain for CA_n3-n39, propose to go with option 1 to confirm the this specific band combination.

  - Option 1: Case 4 UE implementation is possible for CA_n3-n39 that frequency ranges between two bands are close to each other.

Conclusions

In this contribution, we shared some initial views on the RF requirements for ATG UE supporting inter-band DL CA in Rel-19 and proposals are made as following:

Observation 1:MSD for 23dBm to 40dBm transmission power would be ranging from 4.75dB to 17.38dB which will bring the significant performance degradation of band n3 DL reception.

Proposal 1: for CA_n3-n39 with uplink transmission as n39, MSD requirement could defined as following:

| CA_n3-n39 with UL n39 | MSD |
| --- | --- |
| 23 | 4.76dB |
| 24 | 5.15dB |
| 25 | 5.59dB |
| 26 | 6.09dB |
| 27 | 6.64dB |
| 28 | 7.24dB |
| 29 | 7.90dB |
| 30 | 8.60dB |
| 31 | 9.36dB |
| 32 | 10.15dB |
| 33 | 10.97dB |
| 34 | 11.83dB |
| 35 | 12.71dB |
| 36 | 13.62dB |
| 37 | 14.54dB |
| 38 | 15.48dB |
| 39 | 16.43dB |
| 40 | 17.38dB |

Proposal 2: for Rx antenna gain for CA_n3-n39, propose to go with option 1 to confirm the this specific band combination.

  - Option 1: Case 4 UE implementation is possible for CA_n3-n39 that frequency ranges between two bands are close to each other.

References

- RP-240839,New WID on Enhancements for Air-to-ground network for NR, Approved.

- R4-2321919,Big CR for TS 38.101-1 for NR ATG, Agreed.

- R4-2403864, CR for 38101-1 to update ATG related signaling name, Agreed

- R4-2406594, WF on Rel-19 ATG UE requirements, Approved.

- R4-2410687,WF on Rel-19 ATG UE RF requirements, Approved.

- TR 38.718-02-01

- R4-2417121, WF on UE RF for ATG enhancement, CMCC, approved.

- R4-2502952,WF on UE RF requirements for ATG, CMCC, approved.