3GPP TSG-RAN WG4 Meeting#114bis                                R4-2504367

Wuhan, China, 07th – 11th April, 2025

Agenda item:	7.13.4.2

Source: 	ZTE Corporation, Sanechips

Title: 	Discussion on Rel-20 BS RF enhancement

Document for:	Information

Introduction

In the RAN#107 meeting, there was some initial discussion on Rel-20 BS RF enhancement and the relevant issues has been initially discussed and summarized in moderator’ summary [1]. In this contribution, we would like to share further views on each objective to clarify it in details.

Discussion

- 1) Workable and testable co-location requirements for NR BS operating in FR1 high band(s)

  - Necessity of this objective depends on the progress in Rel-19 and the group will check it in September 2025.

    - Both intra-band co-location and inter-band colocation should be considered.

  - NOTE: refinement of wording on this bullet is needed.

- 2) Additional requirement set(s) for AAS BS with more OTA requirements than the existing BS type 1-H

  - Applicable specifications: AAS BS, NR BS

  - Sub-objective 1: Specify the applicable conducted reference points of BS type 1-H for hybrid beamforming BS architecture in FR1

    - New Antenna modelling for HBF BS to be captured in TR38.803

  - Sub-objective 2: additional radiated requirement for BS type 1-H e.g.EVM, ACLR, spurious emission, ACS, co-located blocking requirement;

- 3) OTA requirements enhancement for receiver in-band blocking for FR1 high band(s)

  - Re-evaluate assumptions used for the derivation of existing levels and establish new requirement levels, if needed.

    - Start with SI phase with normative WI depends on the outcome of SI.

- 4) NB-IoT introduction to AAS BS specifications

  - NB-IoT in-band, NB-IoT guard band, and standalone NB-IoT

    - NR BS type 1-H/1-O with NB-IoT in-band (TS38.104/ TS38.141-1/2)

    - AAS MSR (TS37.105/ TS37.145-1/2) to capture the NB-IoT in-band, NB-IoT guard band, and standalone NB-IoT

    - Preclude the AAS BS type 1-H supporting standalone NB-IoT only;

  - Define TX/RX (core, perf) requirements for introduction of operation to AAS BS specifications.

- 5) OBUE requirements structure simplification

  - All network nodes specifications to be addressed,

  - Conducted and OTA requirements,

  - FFS on whether to include this objective in Rel-20 WI, RAN task, or as maintenance

Conclusions

In this contribution, we shared some initial views on Rel-20 BS RF enhancement based on RAN#107 moderator’s summary and the outcome of adhoc discussion.

References

- Moderator's summary for RAN4 led REL-20 topics [RP-250817]