3GPP TSG-RAN WG4 Meeting#114                               R4-2504371

Wuhan, China, 07th – 11th April, 2025

Agenda item:	7.23.2.3

Source: 	ZTE Corporation

Title: 	Discussion on modification of existing Rx requirements for FR1 and FR2-1 for SBFD BS

Document for:	Approval

Introduction

In the RAN#102 meeting, new WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD) has been approved. It’s expected to start the RAN4 meeting from April meeting. Since we have already reached good progress on the SBFD BS RF impacts in SI phase and outcome was captured in TR 38.858. In this contribution, we would like to share some further views on these remaining open issues.

Issue 1: Joint measurement for normal DL symbols/slots and SBFD symbols/slots WF: - Separate testing for SBFD slot and non-SBFD slot, with the consideration to try to adopt slot-level for the test granularity. Issue 2: In-band blocking and additional emission WF: - For FR2-1 WA SBFD BS, FFS re-use the current FR2-1 WA BS in-band blocking requirements. - For FR1 WA BS, define the in-band blocking requirement, FFS on following options. Other options are not precluded. - Option 1: Define the requirement by selecting one of the following grid-shift values. Additional emission issue is not included in this release. If regulators make a decision to require SBFD on adjacent operators in a band, a future WI would address necessary emissions and blocking requirements for the SBFD-SBFD adjacent operator scenario. - Option A: 20% - Option B: 50% - Option 2: Define general requirements for SBFD-TDD adjacent operator scenario. Define an additional in-band blocking requirement and additional emission to address SBFD-SBFD adjacent operator scenario. - Option 3: Define in-band blocking requirements based on co-existence simulation results with grid-shift 5%. - Option 4: Define in-band blocking requirements based on co-existence simulation results with grid-shift 100%. - For FR1 MR SBFD BS, FFS. Issue 3: Dynamic range requirement WF: - The dynamic range requirements of FR1 can reuse existing requirement. - For FR2, no dynamic range requirement is needed. Issue 4: ACS requirement WF: - FFS on the following proposals. - Proposal 1: Prioritize discussing the scenarios under non-colocation case first. - Proposal 2: The same definition of ACS could be reused for SBFD, and the interference level of [9] dB offset should be further checked. - Proposal 3: The definition of ACS is reused for SBFD. The interference level keeps 9 dB offset to the general in-band blocking interference level defined for SBFD. - Proposal 4: It is needed to determine whether interference signal level of BS to BS can be defined as ACS interference signal level. - Proposal 5: The requirement level of ACS should be derived from the BS-BS co-existence simulation results obtained in SI, instead of keeping 9 dB offset to the general in-band blocking interference level defined for SBFD. - Proposal 6: Derive ACS requirement level based on the simulation results using 5% GS in SI. - Proposal 7: The OTA sensitivity degradation should be taken into account for the baseline REFSENS for OTA ACS requirement and should not be taken into account when testing conducted ACS. Issue 5: Necessity of New RX intermodulation requirement with 1 interfering signal WF: - FFS on the following options. - Option 1: IMD requirement for single interfering signal scenario is not needed. Such a requirement is implicitly captured by the SBFD RX blocking requirement. - Option 2: RAN4 to consider adjusting current RX IMD requirement with interfering signals mean power levels and frequencies. Issue 6: Impact on existing RX intermodulation requirement with 2 interfering signals WF: - FFS on the following options. - Option 1: - for the receiver intermodulation requirements, BS2BS CLI should be taken into account for power level for interference signal. - for receiver intermodulation requirements, consider IMD between CW/NBB/general intermodulation interfering signal intermodulate with SBFD DL transmission with some performance degradation on SBFD receiver. - Option 2: new IMD requirement with 2 interfering signal scenario is not needed. - Option 3: RAN4 to consider new RX intermodulation requirement with 2 interfering signals. RAN4 to consider the three possibilities for RX IMD and choose the most challenging one to cover others. - Option 4: The scenario for 2 input signals can be regarded as the worst case when setting requirement on receiver linearity. For RX IMD, the mean power of both CW and modulated interfering signals is [the final agreed in-band blocking level] – 9 dB.

Discussion

## 2.1. Rx requirements

### 2.1.1. Receiver intermodulation

Agreement: - FFS on the following options: - Option 1: IMD requirement for single interfering signal scenario is not needed. Such a requirement is implicitly captured by the SBFD RX blocking requirement. - Option 2: RAN4 to define new RX intermodulation scenario with self-interference and 1 interfering signal.

Similar as receiver dynamic range requirement, for Rx IMD requirement, the BS2BS CLI should be also taken into account when specifying the power level for the interference signals.

For the legacy/normal slot/symbols, the existing receiver intermodulation requirement in section 7.7.2 of TS 38.104 are still applicable.

For the SBFD slot/symbols, there might be some self interference and additional Rx intermodulation caused by downlink part especially when considering with CW signal or NBB or general Rx intermodulation signals configured during the Rx IMD test, therefore it’s suggested to have further study for it.

Figure 2.1.1-1: Example of Rx intermodulation requirement for SBFD BS

- Proposal 1: for the receiver intermodulation requirements, BS2BS CLI should be taken into account for power level for interference signal.

- Proposal 2: for receiver intermodulation requirements, consider IMD between CW/NBB/general intermodulation interfering signal intermodulate with SBFD DL transmission with some performance degradation on SBFD receiver as shown in Figure 2.2.3-1.

### 2.1.2. IBB requirements

- During the last RAN4 meeting, there were some discussions on IBB evaluation to figure out the appropriate requirement definition. In the following section, we provide some initial simulation results according to the simulation assumption captured in TR 38.858.

- FR1:

Figure 2.1.2-1. the evaluation results for Scenario 1

Figure 2.1.2-2. the evaluation results for Scenario 3

Figure 2.1.2-3. the evaluation results for Scenario 5

Considering the 99% CDF to define the IBB interfering power level, the blocking level over antenna elements or over antenna array is collected as following. It could be observed that the received power level for Scenario 1, namely FR1 Urban macro scenario are much higher than the existing IBB requirement -43dBm which would be quite challenging for SBFD BS to handle the receiver dynamic range together with the REFSENS requirements.

In addition, according to the feasibility study for FR1 WA SBFD BS, some analog filter after the LNA need to be implemented to guarantee the required dynamic range and maximum input power for LNA is more than 10dB which will not been blocked somehow. Based on the analog filtering analysis, then in-band blocking power level for FR1 WA BS could be set as -15dBm.

For Scenario 3 indoor scenario, the received power is still close to the existing requirement and therefore it should be okay to reuse the existing requirements without any feasibility issue or additional blocking issues.

| Scenario | Grid shift | Per antenna elements [dBm] | Per antenna array [dBm] | Per sub-array [dBm] |
| --- | --- | --- | --- | --- |
| 1 | 100% | -25.3301 | -19.7623 | TBD |
|  | 10% | -1.5067 | 3.4934 | TBD |
|  | 20% | -7.4879 | -1.6402 | TBD |
|  | 50% | -17.4928 | -13.0954 | TBD |
| 3 | NA | -42.4091 | -34.4724 | TBD |
| 5 | 100% | -38.9889 | -35.7181 | TBD |
|  | 10% | -15.4155 | -11.2276 | TBD |
|  | 20% | -22.3199 | -18.4471 | TBD |
|  | 50% | -32.3749 | -29.5694 | TBD |

- Observation 1: for Scenario 1 FR1 Urban macro scenario, the received power level are much higher than the existing IBB requirement -43dBm which would be quite challenging for SBFD BS to handle the receiver dynamic range together with the REFSENS requirements.

- In addition, as mentioned above, the analog filtering after the LNA is needed for FR1 WA SBFD BS, with the initial analysis, the achievable in-band blocking would be similar as the legacy OOBB power level.

- Proposal 3: for FR1 WA SBFD BS, propose the in-band blocking level as -15dBm.

- Observation 2: for Scenario 3 FR1 indoor scenario, the received power is still close to the existing requirement and therefore it should be okay to reuse the existing requirements without any feasibility issue or additional blocking issues.

- In addition, as mentioned above, the analog filtering after the LNA is needed for FR1 WA SBFD BS, with the initial analysis, the achievable in-band blocking would be similar as the legacy OOBB power level.

- Observation 3: for Scenario 5 FR1 micro scenario, the received power under the 100% grid shift is still close to the existing requirement, however for the other grid shift assumption, the received power level (10% grid shift) are much higher than the existing IBB requirement -38dBm which would be quite challenging for SBFD BS to handle the receiver dynamic range together with the REFSENS requirements.

- Proposal 4: for FR1 MR SBFD, propose to define the in-band blocking requirement as -38dBm under the assumption of without any analog filtering design after the LNA of receiver chain which is close to 100% grid shift in the practical deployment.

- FR2-1:

Figure 2.1.4-3a. the evaluation results for Scenario 6

Figure 2.1.4-3b. the evaluation results for Scenario 6

Figure 2.1.4-4a. the evaluation results for Scenario 9

Figure 2.1.4-4b. the evaluation results for Scenario 9

Similar as FR1, considering the 99% CDF to define the IBB interfering power level, the blocking level over antenna array and relative power difference between wanted signal and IBB signal is collected as following. It could be observed that the relative power difference between wanted signal and IBB signal in Scenario 6 and Scenario 9 is still close to the existing IBB requirement if without considering 6dB performance degradation of wanted signal. In other words, for Scenario 6 and 9, the received power is still close to the existing requirement and therefore it should be okay to reuse the existing requirements without any feasibility issue or additional blocking issues.

| Scenario | Grid shift | Per antenna array [dBm] | Relative power difference [dBm] |
| --- | --- | --- | --- |
| 6 | 100% | -48.3600 | 25.6302 |
|  | 10% | -36.5674 | 37.2361 |
|  | 20% | -41.2973 | 32.3168 |
|  | 50% | -45.7134 | 27.8767 |
| 9 | N/A | -56.5661 | 16.8709 |

- Observation 4: for Scenario 6 FR2 urban macro scenario and Scenario 9 FR2 indoor scenario, the received power is close to the existing requirement and therefore it should be okay to reuse the existing requirements without any feasibility issue or additional blocking issues.

Conclusions

In this contribution, we want to share some further views on RF requirements for SBFD BS and proposals/observations are made as following:

- Proposal 1: for the receiver intermodulation requirements, BS2BS CLI should be taken into account for power level for interference signal.

- Proposal 2: for receiver intermodulation requirements, consider IMD between CW/NBB/general intermodulation interfering signal intermodulate with SBFD DL transmission with some performance degradation on SBFD receiver as shown in Figure 2.2.3-1.

- Observation 1: for Scenario 1 FR1 Urban macro scenario, the received power level are much higher than the existing IBB requirement -43dBm which would be quite challenging for SBFD BS to handle the receiver dynamic range together with the REFSENS requirements.

- Proposal 3: for FR1 WA SBFD BS, propose the in-band blocking level as -15dBm.

- Observation 2: for Scenario 3 FR1 indoor scenario, the received power is still close to the existing requirement and therefore it should be okay to reuse the existing requirements without any feasibility issue or additional blocking issues.

- Observation 3: for Scenario 5 FR1 micro scenario, the received power under the 100% grid shift is still close to the existing requirement, however for the other grid shift assumption, the received power level (10% grid shift) are much higher than the existing IBB requirement -38dBm which would be quite challenging for SBFD BS to handle the receiver dynamic range together with the REFSENS requirements.

- Proposal 4: for FR1 MR SBFD, propose to define the in-band blocking requirement as -38dBm under the assumption of without any analog filtering design after the LNA of receiver chain which is close to 100% grid shift in the practical deployment.

- Observation 4: for Scenario 6 FR2 urban macro scenario and Scenario 9 FR2 indoor scenario, the received power is close to the existing requirement and therefore it should be okay to reuse the existing requirements without any feasibility issue or additional blocking issues.

References

- RP-234035,New WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD), Huawei, Samsung, Approved.

- TR 38.858

- R4-2321091, TP to TR 38.858 Impact on BS RF requirements, ZTE, Agreed

- R4-2409958, Way forward for [111][313] NR_duplex_evo, Approved.

- R4-2413515, Way Forward for [112][308] NR_duplex_evo_BSRF, Approved

- R4-2502310	Way Forward for [114][306] NR_duplex_evo_General, approved.

- R4-2502380	Way Forward for [114][307] NR_duplex_evo_BSRF, approved.