3GPP TSG-RAN WG4 Meeting #114bis	R4-2504379

Wuhan, China, 7th – 11th April, 2025

Agenda Item:	7.10.2.2.2

Source:	MediaTek Inc.

Title:	Discussion on NR-NTN HPUE RX RSD

Document for:	Approval

- Introduction

In RAN#103 plenary meeting, the WID on enhanced requirements and test methodology for NR and IoT NTN was approved in [1]. One of the main objectives is to enable PC2/PC1.5 high power UE (HPUE) with defining RF requirements targeting at support of NR-NTN UEs for FR1 single carrier scenario.

In this contribution, our evaluation about HPUE RX reference sensitivity degradation (RSD) requirements for NR-NTN FR1 bands is provided for discussion.

- Discussion

2.1		High power UE (HPUE) for NTN

Some of the main Rel-19 WID [1] objectives for enhanced requirements and test methodology for NR and IoT NTN are shown below.

- Conduct the coexistence analysis based on TR 38.863 for the above example bands and corresponding power classes - Evaluate the impact on the uplink performance of TN (Terrestrial network) - Example bands include bands n256 and 256, n255 and 255 - PC2, PC1.5 (for NR-NTN only) and PC1 will be evaluated for ACLR and ACS - For NR-NTN and IoT-NTN: coexistence studies shall be carried out under the same conditions, including geographical separation between NTN UE and Terrestrial Networks (TN), as the co-existence studies in Rel-17 and Rel-18 - Specify high power UE (HPUE) for NR-NTN (Non-Terrestrial Networks) and IoT-NTN (NB-IoT and eMTC based NTN) in FR1-NTN bands and the corresponding LTE NTN bands for the single uplink (UL) carrier scenario - Support UE with power class 2 (PC2, 26 dBm) that applies to both handheld and non-handheld based on the co-existence study. - Support other power classes based on the co-existence and feasibility study - Non-handheld UE is prioritized - Specify RF requirements for all considered power classes for NR-NTN and IoT-NTN in FR1-NTN bands and the corresponding LTE NTN bands - Necessary Tx requirements, at least including - Investigate and if necessary, specify the solution to address potential SAR (Specific Absorption Rate restriction) requirements for handheld UE - Specify MPR and A-MPR for the example bands considering the regulation for the corresponding bands, if necessary - Necessary Rx requirements, at least including - Specify the RSD requirements on the example bands, if necessary

2.2		Previous Meeting Agreements

Based on the WID’s objective, it indicates that it is necessary to evaluate whether RX RSD requirements would be needed when TX HPUE is enabled.  In RAN4#113 and RAN4#114 meetings, the agreed WF [2][3] provided the guidance for following works. The partial contents are listed below for reference.

RAN4#113 Issue 3-1-2: RSD Requirements for PC2 on n254 Agreement: For PC2 on n254: [0] dB RSD for PC2 NR-NTN for 1Tx and 2Tx Issue 3-1-3: RSD requirements for PC2 on n255 Way forward: For PC2 on n255: Further discuss the follow assumptions - Option 1: Duplex isolation as 50~52; PA interference as -107 dBm/Hz (P2/R4-2419073) - Option 2: Duplex isolation as 51; PA interference as -117 dBm/Hz (O1/R4-2419540) Issue 3-1-4: RSD requirements for PC2 on n256 Way forward: For PC2 on n256: [0] dB RSD for PC2 NR-NTN for 1Tx and 2Tx as starting point - To be confirmed by further discuss the following assumptions: - Option 1: Duplex isolation as 50~52; PA interference as -117.8 dBm/Hz - Option 2: Duplex isolation as 51; PA interference as -124 dBm/Hz Issue 3-2: RSD requirements for PC1.5 - Proposals - Option 1: For NR NTN PC 1.5 handheld HPUE, the RSD value depends on the architecture of HPUE (P1/R4-2417815) - Option 2: For PC1.5, 2Tx RSD are the same for handheld and non-handheld, while 1Tx RSD is likely to consider non-handheld only (P3/R4-2418201) - Option 3: no RSD is needed for non-handheld UE including PC 1 and PC 1.5. (P3/R4-2419016) - Option 4: [2.2] dB RSD for PC1.5 with 2Tx architecture for n256. Issue 3-3: RSD requirements for PC1 - Proposals - Option 1: For PC1 1Tx for NR-NTN, which is only considered feasible for non-handheld, the RSD is proposed to be 0dB (P4a/R4-2418201) - Option 1-1: FFS for PC1 with 2 or 3 Tx cases (P4b/R4-2418201) - Option 2 no RSD is needed for non-handheld UE including PC 1 and PC 1.5. (P3/R4-2419016) Issue 3-4: Others general proposals w.r.t RSD Way forward: - Encourage companies to investigate and discuss the impact of Rx non-linearity in RSD evaluations.

RAN4#114 Issue 3-1-1: RSD requirements for PC2 on n256 Agreement: - For PC2 on n256: 0dB RSD for PC2 NR-NTN for 1Tx and 2Tx. Issue 3-1-2: RSD requirements for PC2 on n254 Agreement: - For PC2 on n254: 0dB RSD for PC2 NR-NTN for 1Tx and 2Tx. Issue 3-1-3: RSD requirements for PC2 on n255 - Proposals - Option 1: Duplex isolation as 51, and PA noise as -117 dBm/Hz (MediaTek) Operating Band n255; 5 MHz (dB); 10 MHz (dB); 15 MHz (dB); 20 MHz (dB). 1Tx; [0.4]; [0.4]; [0.5]; [0.5]. 2Tx; [0.8]; [0.8]; [0.9]; [0.9]. - Option 2: Discuss if only default Tx-Rx frequency separation is used. (Huawei) - PA noise as -117 dBm/Hz. - Option 3: Assuming IIP2 54dBm (Qualcomm) Operating Band n255; 5 MHz (dB); 10 MHz (dB); 15 MHz (dB); 20 MHz (dB). 1Tx; 1.3; 1.3; 1.3; 1.3. 2Tx; 2.4; 2.4; 2.4; 2.4. Way forward: - Continue align the assumptions and further discuss above options. Issue 3-2: RSD requirements for PC1.5 - Proposals - Option 1: For PC1.5, 2Tx RSD are the same for handheld and non-handheld, while 1Tx RSD is likely to consider non-handheld only if defined. (vivo) - Option 2: Regarding NR NTN HPUE operated in PC1.5, discuss whether the RX RSD values as shown in tables below could be used for bands n256, n255 and n254. (MediaTek) Table 4 RSD from PC3 to PC1.5 for FDD bands for NR NTN UE not supporting Tx Diversity Operating Band; 5 MHz (dB); 10 MHz (dB); 15 MHz (dB); 20 MHz (dB). n256; [1.8]; [1.8]; [1.8]; [1.8]. n255; [2.4]; [2.4]; [2.5]; [2.5]. n254; [1.8]; [1.8]; [1.8]. Table 5 RSD from PC3 to PC1.5 for FDD bands for NR NTN UE supporting Tx Diversity Operating Band; 5 MHz (dB); 10 MHz (dB); 15 MHz (dB); 20 MHz (dB). n256; [2.2]; [2.2]; [2.2]; [2.2]. n255; [3.2]; [3.2]; [3.3]; [3.3]. n254; [2.2]; [2.2]; [2.2]. - Option 3: For the NR-NTN, as the starting point consider not to specify RSD requirements for PC1.5 and PC1. (Ericsson) Way forward: - Further discuss above options. Issue 3-3: RSD requirements for PC1 - Proposals - Option 1: For PC1 1Tx for NR-NTN, which is only considered feasible for non-handheld, the RSD is proposed to be 0dB. And FFS PC1 2Tx or 3Tx. (vivo, Samsung) - Option 2: Regarding NR NTN HPUE operated in PC1, discuss whether the RX RSD values as shown in tables below could be used for band n256. Table 6 RSD from PC3 to PC1 for FDD bands for NR NTN UE not supporting Tx Diversity Operating Band; 5 MHz (dB); 10 MHz (dB); 15 MHz (dB); 20 MHz (dB). n256; [3]; [3]; [3]; [3]. Table 7 RSD from PC3 to PC1 for FDD bands for NR NTN UE supporting Tx Diversity Operating Band; 5 MHz (dB); 10 MHz (dB); 15 MHz (dB); 20 MHz (dB). n256; [3.7]; [3.7]; [3.7]; [3.7]. - Option 3: For the NR-NTN, as the starting point consider not to specify RSD requirements for PC1.5 and PC1. (Ericsson) Way forward: - Further discuss above options.

2.3		UE RF requirements

2.3.1	NR-NTN PC2 HPUE RX RSD for n255

According to our evaluation, because of high TX-RX duplex spacing frequency, no RX RSD is needed for NR NTN bands n256 and n254 when UE enables PC2 TX. Regarding NR NTN band n255, the RX RSD is needed when UE enables PC2 TX.

Proposal 1: Regarding NR-NTN HPUE operated in PC2, discuss whether the RX RSD values as shown in tables below could be used for band n255.

Table 1: RSD from PC3 to PC2(1Tx) for NR-NTN UE

| Operating Band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) |
| --- | --- | --- | --- | --- |
| n255 | 0.4 | 0.4 | 0.5 | 0.5 |

Table 2: RSD from PC3 to PC2(2Tx) for NR-NTN UE

| Operating Band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) |
| --- | --- | --- | --- | --- |
| n255 | 0.8 | 0.8 | 0.9 | 0.9 |

2.3.2	NR-NTN PC1.5 HPUE RX RSD for n256, n255 and n254

Proposal 2: Regarding NR-NTN HPUE operated in PC1.5(2Tx), discuss whether the RX RSD values as shown in tables below could be used for bands n256, n255 and n254.

Table 3: RSD from PC3 to PC1.5(2Tx) for NR-NTN UE

| Operating Band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) |
| --- | --- | --- | --- | --- |
| n256 | 2.2 | 2.2 | 2.2 | 2.2 |
| n255 | 3.2 | 3.2 | 3.3 | 3.3 |
| n254 | 2.2 | 2.2 | 2.2 |  |

2.3.3	NR-NTN PC1 HPUE RX RSD for n256, n255 and n254

Proposal 3: Regarding NR-NTN HPUE operated in PC1(1Tx), discuss whether the RX RSD values as shown in tables below could be used for bands n256, n255 and n254.

Table 4: RSD from PC3 to PC1(1Tx) for NR-NTN UE

| Operating Band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) |
| --- | --- | --- | --- | --- |
| n256 | [3] | [3] | [3] | [3] |
| n254 | [3] | [3] | [3] |  |

3 Conclusion

In this paper, our views on NR-NTN HPUE RX RSD are provided.

Proposal 1: Regarding NR-NTN HPUE operated in PC2, discuss whether the RX RSD values as shown in tables below could be used for band n255.

Table 1: RSD from PC3 to PC2(1Tx) for NR-NTN UE

| Operating Band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) |
| --- | --- | --- | --- | --- |
| n255 | 0.4 | 0.4 | 0.5 | 0.5 |

Table 2: RSD from PC3 to PC2(2Tx) for NR-NTN UE

| Operating Band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) |
| --- | --- | --- | --- | --- |
| n255 | 0.8 | 0.8 | 0.9 | 0.9 |

Proposal 2: Regarding NR-NTN HPUE operated in PC1.5(2Tx), discuss whether the RX RSD values as shown in tables below could be used for bands n256, n255 and n254.

Table 3: RSD from PC3 to PC1.5(2Tx) for NR-NTN UE

| Operating Band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) |
| --- | --- | --- | --- | --- |
| n256 | 2.2 | 2.2 | 2.2 | 2.2 |
| n255 | 3.2 | 3.2 | 3.3 | 3.3 |
| n254 | 2.2 | 2.2 | 2.2 |  |

Proposal 3: Regarding NR-NTN HPUE operated in PC1(1Tx), discuss whether the RX RSD values as shown in tables below could be used for bands n256, n255 and n254.

Table 4: RSD from PC3 to PC1(1Tx) for NR-NTN UE

| Operating Band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) |
| --- | --- | --- | --- | --- |
| n256 | [3] | [3] | [3] | [3] |
| n254 | [3] | [3] | [3] |  |

4 Reference

- RP-240857, “New WID: Enhanced requirements and test methodology for NR and IoT NTN”, Huawei

- R4-2420403, “WF on NR NTN HPUE”, Samsung

- R4-2502954, “WF on HPUE requirements for NR-NTN”, Samsung