3GPP TSG-RAN WG4 Meeting #114-bis	R4-2504380

# Wuhan, China, April 7th – 11st 2025

Source:	Ericsson

Title:	Discussion on general aspects of NR SCM for demodulation

Agenda item:	7.16.1

Document for:	Discussion

# Introduction

In RAN4#114 meeting, companies have some discussions on model generalization and TR content, but no agreement on it. As the deadline of the study is close, a clearer guidance for model generalization and TR construction is needed. In this contribution, we would bring up some thinkings on these issues.

# Discussion

#### Capturing of Channel Model Description

Way Forward:

(Keep Prior Agreement) High level descriptions of channel models in Chapter 5, comparison using cases and metrics in Chapter 6, alignment in chapter 7, with detailed description in Annex [B]

Note: Further details for each section to be clarified with contributions and pCRs at future meetings.

#### Capturing of Alignment and Comparison in the TR

Way Forward:

Company views on how to capture alignment and comparison in the TR to be brought to RAN4#114

The content of the TR was discussed in RAN4#114 meeting. One question is the relationship between alignment (chapter 7) and comparison (chapter 6), but it seems companies have different views on this. In recent meetings, comparison and alignment are normally mixed together which means companies deliver simulation results for both comparison and alignment. It is true that the performance gap can be seen during the comparison, but alignment in our view should include “companies’ preference” based on the discussion to avoid too much effort on corner cases.

During previous meetings, more new models and test cases proposed by companies which is very good for model improvement, but it also brings up the issue that companies have to take more effort to implement new models and simulations. Companies don’t have time to check the simulation results and to find the reason behind the performance gap. In that case, no conclusion on the model alignment and comparison has been achieved. The study progress would be seriously delayed if we don’t have clear way of working to handle this situation.

In our view, there are at least two ways to do comparison and alignment:

- WoW 1: Alignment at the first. This way would need companies to devote effort at the first to implement all candidate models and deliver corresponding simulation results, then start to compare pros and cons for alignable models. It would be better to find out the reason of unalignable models/cases. The absolute performance value might not be so important in alignment stage, and this could be considered in comparison stage.

Based on the motivation of this study and current status, this WoW could be preferred with prioritized models/cases to reduce effort.

- WoW 2: Comparison at the first. It would need companies not only focus on alignablity but also on other aspects, such as practicability and testability etc. After the comparison, companies should choose at least one preferred models for further alignment. The risk of this WoW is that companies might not have full and clear view on pros/cons of different models, so the decision making will not be solid.

Currently, we have two basic approaches to derive SCM model including CDL extension and TDL extension. CDL extension models we have now include 38.827 model, model partially using 38.827 model, model partially using 38.901 and 38.827 models. The methodology of these models are same but using different parameter values more or less from 38.827 models. TDL extension models we have now include R4-2415382 model and R4-2419895 model. The two models are also similar which use Tx-Rx beam steering on top of existing TDL model and correlation matrices, but the former model use all taps per channel for one beam direction (channel taps per beam are independent) while the latter one use partial taps from same channel for one beam direction.

From the alignment complexity perspective, there would be no difference among models per extension approach. Thus, companies could only choose one model per extension to get alignment at the first to check the alignability, especially for CDL extension. Then do comparison on other aspects to make final conclusion. As for the chosen model for alignment, we prefer to use 38.827 based model (maybe start from no AAV, then AAV no full connected) with simpler test cases (maybe start from lower rank case, then higher rank) because almost all companies have implemented it and might be easier to do debugging.

Mixing comparison and alignment discussion together would slow the study progress.

Proposal 1	RAN4 to keep same way of working for whole discussion that get alignment for test cases with preferred model at the first and then compare pros/cons between TDL and CDL approaches.

Since the current structure of TR have comparison in Chapter 6 and alignment in Chapter 7 which looks more like WoW 2, but we might let Chapter 6 to capture results for all candidate models and test cases no matter how many companies deliver corresponding results while we can just capture alignment results for “most companies preferred” models/cases. This could be helpful to get the logic of conclusion in Chapter 8.

Proposal 2	In Chapter 6 for comparison between TDL and CDL methodologies, consider only capture results of alignable models. The average performance among companies could be used.

Proposal 3	In Chapter 7 for alignment, only results of “most companies preferred” models after comparison can be captured.

The scope of WID include FR1 and FR2, SU and MU scenario. But the current progress is just focus on FR1 and SU. It could be possible to narrow down the scope to FR1 only because FR2 scenario would be more complex. The study on FR2 part could be considered in the next release.

Proposal 4	Consider SCM model study of FR2 in the next release.

# Conclusions

In the previous sections we made the following observations:

Observation 1	Mixing comparison and alignment discussion together would slow the study progress.

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN4 to keep same way of working for whole discussion that get alignment for test cases with preferred model at the first and then compare pros/cons between TDL and CDL approaches.

Proposal 2	In Chapter 6 for comparison between TDL and CDL methodologies, consider only capture results of alignable models. The average performance among companies could be used.

Proposal 3	In Chapter 7 for alignment, only results of “most companies preferred” models after comparison can be captured.

Proposal 4	Consider SCM model study of FR2 in the next release.

# References

[1] 	R4-2419782, Way Forward for [113][320] FS_NR_demod_SCM, Nokia