3GPP TSG-RAN WG4 Meeting #114bis	R4-2504424

Wuhan, China, 7th – 11st, April 2025

Source:	Ericsson

Title:	Discussion on reply LS on measurement quantities for IoT NTN

Agenda item:	8.2

Document for:	Discussion

# 1	Introduction

In RAN1 #120 meeting, a LS was sent to RAN4 [1]:

… # 1. Overall Description: RAN1 discussed whether to support measurement quantities (i.e., DL RS TX power, Received Interference Power, and Thermal Noise Power) for IoT NTN measurement, and agreed to consult RAN4 based on the following agreement: Agreement Send an LS to ask RAN4 the following: - Whether any of the following measurement quantities, described in clause 5.2 of TS 36.214, are applicable for NB-IoT-based NTN and/or eMTC-based NTN: - DL RS TX power - Received Interference Power - Thermal Noise Power - For the measurement quantitie(s) that are applicable, if any, whether reference point(s) need to be defined.. Question1: Whether any of the following measurement quantities, described in clause 5.2 of TS 36.214, are applicable for NB-IoT-based NTN and/or eMTC-based NTN: - DL RS TX power - Received Interference Power - Thermal Noise Power Question2: For the measurement quantitie(s) that are applicable, if any, whether reference point(s) need to be defined? …

In this paper we present our views on the questions in LS.

# 2	Discussion

In 36.214, the clause 5.2 is quoted below:

### 5.2.1	DL RS TX power

| Definition | Downlink reference signal transmit power is determined for a considered cell as the linear average over the power contributions (in [W]) of the resource elements that carry cell-specific reference signals which are transmitted by the eNode B within its operating system bandwidth. For DL RS TX power determination the cell-specific reference signals R0 and if available R1 according TS 36.211 [3] can be used. The reference point for the DL RS TX power measurement shall be the TX antenna connector. |
| --- | --- |

### 5.2.2	Received Interference Power

| Definition | The uplink received interference power, including thermal noise, within one physical resource block's bandwidth of resource elements as defined in TS 36.211 [3]. The reported value shall contain a set of Received Interference Powers of physical resource blocks as defined in TS 36.211 [3]. The reference point for the measurement shall be the RX antenna connector. In case of receiver diversity, the reported value shall be linear average of the power in the diversity branches. |
| --- | --- |

### 5.2.3	Thermal noise power

| Definition | The uplink thermal noise power within the UL system bandwidth consisting of resource blocks as defined in [3]. It is defined as (No x W), where No denotes the white noise power spectral density on the uplink carrier frequency and denotes the UL system bandwidth. The measurement is optionally reported together with the Received Interference Power measurement, it shall be determined over the same time period as the Received Interference Power measurement, The reference point for the measurement shall be the RX antenna connector. In case of receiver diversity, the reported value shall be linear average of the power in the diversity branches. |
| --- | --- |

The antenna connector is defined as conducted reference point for LTE BS in clause 6.1 and 7.1 in TS 36.104. There is also the E-UTRA AAS BS defined for E-UTRA and this is specified in 37.105. The reference point for hybrid AAS is defined in clause 4.3 in 37.105 as TAB (Transceiver array boundary connector) connector and also RIB (radiated interference boundary ). For OTA AAS BS, only the RIB reference point is defined.  However, there is no modification of the reference point for E-UTRA hybrid or OTA AAS BS in 36.214. LTE is a mature technology deployed in network since Rel-8 and it seems there is no need to patch the TN side for reference point.

Observation-1: No specification update for 36.214 to include the hybrid AAS or OTA AAS BS for E-UTRA.

Observation-2: No need to patch the reference point for E-UTRA.

For IoT NTN, the BS type 1-H/1-O are introduced and therefore, the reference point is only defined as either TAB connector or RIB in clause 4.3.1 in TS 36.108. If following the TN on the reference point definition in 36.214, there is no need to define this. However, NTN is new feature introduced in Rel-17, it seems there is no harm in updating the reference point for NTN. We slightly prefer defining such reference point as the SAN node may include multiple entities. And response is proposed below:

Question1: The measurement quantities still apply to the NTN SAN

Question 2: there is a need to update the reference point in TR 36.214 for NTN SAN with below wording:

“The reference point for the measurement shall be the TAB connector for SAN type 1-H and RIB for SAN type 1-O.”

Proposal-1: update the reference point of clause 5.2 for the reference point definition for NTN only.

# 3 	Summary

We present our view on the reply of LS[1]:

Question1: The measurement quantities still apply to the NTN SAN

Question 2: there is a need to update the reference point in TR 36.214 for NTN SAN with below wording:

“The reference point for the measurement shall be the TAB connector for SAN type 1-H and RIB for SAN type 1-O.”

# References

[1] R1-2501512, LS on measurement quantities for IoT NTN, Vivo