3GPP TSG-RAN WG4 Meeting #114bis		R4-2504454

Wuhan, Hubei, China, 07th – 11th April, 2024

Agenda item:	7.23.3.1

Source:	Apple

Title:   	On UE RRM requirements for UE-to-UE CLI handling

Document for:	Discussion

# 1	Introduction

In the latest R19 SBFD WI [1], there is RAN4 objective on UE CLI measurement and reporting, as shown below:

- L1 based UE-to-UE CLI measurement and reporting based on existing CSI framework, including [RAN1, RAN2, RAN4] - Periodic, semi-persistent, or aperiodic measurement resource (set), i.e., SRS-RSRP resource or CLI-RSSI resource, and configuration/determination of ‘typeD’ QCL assumptions for the CLI measurement resource - At least aperiodic reporting - New report quantities, e.g., L1-SRS-RSRP, L1-CLI-RSSI and/or measurement resource indices - UCI bits generation - Priority rules for multiple CSI reporting - CLI measurement accuracy requirement - Notes: Following are assumed for L1 based UE-to-UE CLI measurement and reporting - The measurement resources for SRS-RSRP are based on the existing legacy RS patterns. No specification impact for SRS configuration information exchange between gNBs - Periodic and semi-persistent reporting can be considered in addition to aperiodic reporting - For the new reporting quantities, at least wideband reporting is supported - The existing CSI processing unit, CPU occupation rule and timeline for L1 beam reporting are reused for L1 UE-to-UE CLI measurement and reporting as starting point

At the last RAN4 meeting, a WF was agreed [2]. In this contribution, we continue to share our views on the open issues.

# 2	Discussion

Time offset between DL timing and SRS arrival timing

In response to RAN4’s inquiry, RAN1 provided their views in [3] as copied below:

# 1 Overall description When discussing L1 UE-to-UE CLI measurement reporting, RAN1 has not made a distinction between the two application scenarios mentioned in the RAN4 LS (intra-cell vs. inter-cell UE-to-UE L1 CLI measurement). L1 UE-to-UE CLI measurement and reporting specified by RAN1 can be applied for both application scenarios. For the case of intra-cell L1 SRS-RSRP measurement and reporting, a victim UE may be configured with a set of SRS-RSRP measurement resources that overlap with the resources used by the aggressor UE(s) to transmit SRS since the gNB has knowledge the aggressor UE’s SRS configurations. However, for inter-cell L1 SRS-RSRP measurement, SRS resource set configurations may need to be exchanged among gNBs. But specification impact of this in Xn/F1 has been excluded according the following note from the WID - The measurement resources for SRS-RSRP are based on the existing legacy RS patterns. No specification impact for SRS configuration information exchange between gNBs From RAN1 perspective, intra-cell UE-to-UE L1 CLI measurement scenario can be prioritized for the requirements of L1 SRS-RSRP measurement.

Given the following points from the RAN1 reply LS:

- For intra-cell L1 SRS-RSRP measurement and reporting, a victim UE may be configured with a set of SRS-RSRP measurement resources that overlap with the resources used by the aggressor UE(s) to transmit SRS since the gNB has knowledge the aggressor UE’s SRS configurations. However, for inter-cell L1 SRS-RSRP measurement, SRS resource set configurations may need to be exchanged among gNBs. But specification impact of this in Xn/F1 has been excluded.

- From RAN1 perspective, intra-cell UE-to-UE L1 CLI measurement scenario can be prioritized for the requirements of L1 SRS-RSRP measurement.

We think it is reasonable that RAN4 specify requirements for intra-cell case only. As a result, the cell phase synchronization error of 3us is not considered in the time offset between DL reference timing in the serving cell and SRS arrival time. More specifically, the time offset is no larger than Terror_SRS_RSRP, where Terror_SRS_RSRP is TC × NTA_offset + 1.67us for FR1 or TC × NTA_offset + 0.67us for FR2.

Proposal 1: Per RAN1 LS that from RAN1 perspective, intra-cell UE-to-UE L1 CLI measurement scenario can be prioritized for the requirements of L1 SRS-RSRP measurement, it is proposed that RAN4 focuses on requirements for Intra-cell UE-to-UE L1 CLI measurement scenario.

Measurement resources

Regarding measurement resources, the following agreements were captured in the WF [2]:

#### Issue 1-1-3: Measurement resources - Agreements - Define measurement requirements assuming wideband reporting for L1-SRS-RSRP and L1-CLI-RSSI, and do not need to consider subband reporting. FFS if UE is able to measure both DL subbands simultaneously in case of DUD and how such measurement capability affects measurements period and accuracy requirements. - RAN4 to define requirements for L1-CLI-RSRI and L1-SRS-RSRP measurement for semi-static, aperiodic and periodic resource types.

As the L1-SRS-RSRP measurement is performed within UL subband, and there is only one UL subband in a channel, the FFS part “FFS if UE is able to measure both DL subbands simultaneously in case of DUD and how such measurement capability affects measurements period and accuracy requirements.” concerns only L1-CLI-RSSI in CLI measurement method #1.

Meanwhile, RAN1 has made the following agreement concerning L1-CLI-RSSI measurement:

CSI report - For CSI processing timeline in SBFD symbols to process the CSI-RS across two DL subbands, up to UE capability to indicate one of the following: - CSI processing timeline and CPU counting are the same as legacy. - Scale the CSI processing timeline Z/Z’ by factor of 2 and keep the same CPU counting as legacy.

Also, in the SBFD WID [1], there is a note concerning L1 based UE-to-UE CLI measurement and reporting, as shown below:

- Notes: Following are assumed for L1 based UE-to-UE CLI measurement and reporting - The measurement resources for SRS-RSRP are based on the existing legacy RS patterns. No specification impact for SRS configuration information exchange between gNBs - Periodic and semi-persistent reporting can be considered in addition to aperiodic reporting - For the new reporting quantities, at least wideband reporting is supported - The existing CSI processing unit, CPU occupation rule and timeline for L1 beam reporting are reused for L1 UE-to-UE CLI measurement and reporting as starting point

Therefore, for a UE that is unable to measure both DL subbands simultaneously in case of DUD subband configuration, we can assume that the L1-CLI-RSSI measurement timeline follows the CSI processing timeline. In other words, the measurement period for such a UE is doubled so the UE can combine the two RSSI measurements, one for each DL subband in one measurement period, and report. How to combine the two subband RSSI measurement can be left to UE implementation as long as the measurement accuracy requirement is met.

Proposal 2: For a UE that is unable to measure L1-CLI-RSSI over both DL subbands simultaneously in case of DUD subband configuration, it is proposed to double the measurement period so the UE can combine the two RSSI measurements, one for each DL subband in one measurement period, and report. How to combine the two subband RSSI measurement can be left to UE implementation as long as the measurement accuracy requirement is met.

Similarly, regarding the following open issue in the WF [2]:

#### Issue 2-1-1b: CSI-RS L1 measurement – puncturing in frequency domain - Agreements - Further discuss measurement period requirements for CSI-RS based L1 measurement in SBFD symbols with DUD case - Option 1: existing measurement period for CSI-RS based L1 measurement apply, with the assumption that UE measures both DL subbands on each CSI-RS resource occasion. - Option 2: doubling existing measurement period, with the assumption that UE measures one DL subband on each CSI-RS resource occasion. - Option 3: Consider the UE measurement capability - FFS whether and how to account for the UE capability on CSI processing time.

We can also follow the RAN1 agreement.

Proposal 3: For CSI-RS based L1 measurement in SBFD symbols with DUD case, it is proposed to adopt the following two options, subject to UE capability:

- Option 1: existing measurement period for CSI-RS based L1 measurement apply, with the assumption that UE measures both DL subbands on each CSI-RS resource occasion.

- Option 2: doubling existing measurement period, with the assumption that UE measures one DL subband on each CSI-RS resource occasion.

Reporting range and granularity for L1 SRS-RSRP and L1 CLI-RSSI

In the recent LS from RAN1 [4], the agreement on reporting range and granularity for L1 SRS-RSRP and L1 CLI-RSSI are shared.

# 1 Overall description The following working assumption was made in RAN1 on L1-based CLI reporting since RAN1 specifications must capture the format and bitwidths of the CLI reports. Working assumption For L1 based CLI measurement and reporting, the following is adopted - For L1 SRS-RSRP, a 7-bit value in the quantization range [-140, -44] dBm with 1dB step size, and the differential L1 SRS-RSRP is quantized to a 4-bit value. The differential L1 SRS-RSRP value is computed with 2 dB step size with a reference to the largest measured L1 SRS-RSRP value. - For L1 CLI-RSSI, a 7-bit value in the quantization range [-100, -25] dBm with 1dB step size, and the differential L1 CLI RSSI is quantized to a 4-bit value. The differential L1 CLI-RSSI value is computed with 2 dB step size with a reference to the largest measured L1 CLI-RSSI value.. RAN1 assumes that the reporting range and granularity for L3 SRS-RSRP measurement in Table 10.1.22.1.2-1, TS38.133 can be reused for L1 SRS-RSRP. In addition, the reporting range and granularity for L3 CLI-RSSI measurement in Table 10.1.22.2.2-1 of TS38.133 can be reused for L1 CLI-RSSI. For a CLI report, RAN1 agreed that the absolute value of the most interfered (largest) measured L1-SRS-RSRP or L1-CLI-RSSI is reported. RAN1 also agreed to use differential reporting for L1 SRS-RSRP and L1 CLI-RSSI when the number of reported measurement resources is larger than one, in the same manner as for existing L1-RSRP/SINR reporting. RAN1 would like to ask RAN4 whether the reporting range and granularity for L1 SRS-RSRP and L1 CLI-RSSI are reasonable. RAN1 also would like to check with RAN4 whether there is any issue with the range and resolution for differential reporting of L1 SRS-RSRP and L1 CLI-RSSI. # 2 Actions To RAN4 ACTION: RAN1 respectfully asks RAN4 to provide feedback to following two questions - Is the reporting range and granularity for L1 SRS-RSRP and L1 CLI-RSSI in the RAN1 working assumption reasonable from RAN4 perspective? - Is the range and resolution for differential reporting of L1 SRS-RSRP and CLI-RSSI reporting reasonable from RAN4 perspective?

In our understanding, the R16 L3 CLI measurements are for inter-cell UE-to-UE CLI scenarios whereas the current L1 CLI measurements are mainly for or at least prioritized for intra-cell UE-to-UE CLI scenarios, according to the RAN1 LS in [3]. As such, the distance between aggressor UEs and victim UEs may be different in the two scenarios and thus the measured L1 SRS-RSRP and CLI-RSSI may have a different distribution than the measured L3 SRS-RSRP and CLI-RSSI. It is a bit unclear if the R16 L3 SRS-RSRP or CLI-RSSI range can be readily reused. Regarding granularity and resolution for differential reporting, it can be decided once the reporting range and the number of bits used to represent the reporting range are decided.

Proposal 4: It is proposed to investigate if the R16 L3 SRS-RSRP or CLI-RSSI range can be reused for L1 SRS-RSRP or CLI-RSSI, considering R16 L3 CLI measurements are for inter-cell UE-to-UE CLI scenarios whereas L1 CLI measurements are mainly for or at least prioritized for intra-cell UE-to-UE CLI scenarios.

# 3	Conclusions

In this contribution, we make the following proposals.

Proposal 1: Per RAN1 LS that from RAN1 perspective, intra-cell UE-to-UE L1 CLI measurement scenario can be prioritized for the requirements of L1 SRS-RSRP measurement, it is proposed that RAN4 focuses on requirements for Intra-cell UE-to-UE L1 CLI measurement scenario.

Proposal 2: For a UE that is unable to measure L1-CLI-RSSI over both DL subbands simultaneously in case of DUD subband configuration, it is proposed to double the measurement period so the UE can combine the two RSSI measurements, one for each DL subband in one measurement period, and report. How to combine the two subband RSSI measurement can be left to UE implementation as long as the measurement accuracy requirement is met.

Proposal 3: For CSI-RS based L1 measurement in SBFD symbols with DUD case, it is proposed to adopt the following two options, subject to UE capability:

- Option 1: existing measurement period for CSI-RS based L1 measurement apply, with the assumption that UE measures both DL subbands on each CSI-RS resource occasion.

- Option 2: doubling existing measurement period, with the assumption that UE measures one DL subband on each CSI-RS resource occasion.

Proposal 4: It is proposed to investigate if the R16 L3 SRS-RSRP or CLI-RSSI range can be reused for L1 SRS-RSRP or CLI-RSSI, considering R16 L3 CLI measurements are for inter-cell UE-to-UE CLI scenarios whereas L1 CLI measurements are mainly for or at least prioritized for intra-cell UE-to-UE CLI scenarios.

# 4	References

RP-241614, "Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD)," Huawei

RP-2502699, "WF on RRM requirements for NR_duplex_evo," Huawei, HiSilicon

R1-2410890, "Reply LS on the scenarios of UE-to-UE CLI measurement report in SBFD operation," RAN1

R1-2501620, "LS on L1 based UE-to-UE CLI measurement and reporting," RAN1