3GPP TSG-RAN WG4 Meeting #114bis	R4-2504468

Wuhan, China, April 7th – April 11th, 2025

Source:	Nokia

Title:	Discussion on LP-WUS core requirements

Agenda item:	7.26.4.2

Document for:	Discussion

Introduction

This contribution discusses about the remaining issues related to LP-WUS/WUR. Simulation paper is in R4-2504467, and the simulation results with the corresponding assumptions are in R4-2504466 .

Discussion

MR wake up (RAN1 LS also considered)

RAN1 sent the following LS to RAN4:

In RAN1#118bis, it was agreed to support 3 candidate values for the wake-up delay capability a UE reports for LP-WUS operation in IDLE/INACTIVE mode. In addition, the following agreement was made in RAN1#120: Agreement For UE capability report on the wake-up delay: - Alt 1: For the 3 candidate values for the wake-up delay capability report, support {[70ms], [500ms] and [900ms]}. - The reported values assume SSB periodicity of 20ms, where [70ms] assumes [3] SSBs needed for synchronization, [500ms] and [900ms] assume [5] SSBs needed for synchronization. - FFS: translation of wake-up delay for different SSB periodicities - FFS: different sets of 3 candidate value for different SSB periodicities Note that the definition of wake-up delay is the minimum gap time between LP-WUS reception and MR to start PDCCH monitoring. RAN1 respectfully asks RAN4 to check and confirm the number of SSBs currently assumed by RAN1 for MR synchronization for SSB periodicity of 20ms. RAN1 also respectfully asks RAN4 to confirm if it is reasonable to assume the same number of SSBs for MR synchronization for different SSB periodicities.

Issue 1-2-6: On MR wake up delay related requirements for LP-WUR

RAN1 is currently discussing about wake-up time related to MR sync. Wake-up delay is defined as the minimum gap time between UE LP-WUS reception and when the UE shall be able to start PDCCH monitoring for paging monitoring with the MR. RAN1 is suggesting that there are three values that UE may report for wake-up time. Based on either 70ms, 500ms or 900ms, the network configures the paging to be transmitted according to when UE listens to PDCCH on relation to LO. It is worth noting that when the network transmits paging is up to the network configuration and if this does not match the UE wake up time, then the UE can fall back to legacy paging monitoring.

RAN1 has been discussing the wake up as follows:

| Scenario (RAN1) | Wake up part of the delay (RAN1) | Sync part of the delay (RAN4) In good radio conditions | Total wake up time |
| --- | --- | --- | --- |
| Light sleep | 20ms | 3 x 20ms SSB periodicity = 60ms | 70ms |
| Deep sleep | 400ms | 5x 20ms SSB periodicity = 100ms | 500ms |
| Ultra deep sleep | 800ms | 5x 20ms SSB periodicity = 100ms | 900ms |

Currently in RAN4 requirements. If the target cell is an unknown intra-frequency cell and the target cell Es/Iot≥-2 dB, then Tsearch = Trs ms. Therefore 1 sample for MR in very good radio conditions should be enough. In RAN1 discussions the assumption has been that the UE can reuse the timing from LR, so one sample should be enough.

1 SSB samples is enough for sync in good radio conditions where LR is assumed to be used.

After triggering MR wake up, the MR should follow SSB periodicity at least for first set of measurements. This guarantees that the MR is able to perform a quick cell reselection if needed.

Confirm that the following wake up times with sync times are ok for RAN4:

| Scenario (RAN1) | Wake up part of the delay (RAN1) | Sync part of the delay (RAN4) | Total wake up time |
| --- | --- | --- | --- |
| Light sleep | 20ms | 1 x 20ms SSB periodicity | 40ms |
| Deep sleep | 400ms | 1 x 20ms SSB periodicity | 420ms |
| Ultra deep sleep | 800ms | 1 x 20ms SSB periodicity | 820ms |

After MR wake up, MR follows SSB periodicity at least for first set of measurements

High priority frequency layer

Issue 1-1-10: Considerations on higher priority frequency layer

When configured, UE should always perform measurement on higher priority carriers. Higher priority carriers are important for the network to manage UE neighbouring cell measurement behaviour and to guarantee service across the cell during when UE is mobile. Currently the MR relaxation factor is 16 and if UE should measure carriers every 60s, the time would not be enough to perform neighbouring cell measurement and evaluation according to 4.2.2.4-1, and if cell detection is added the full round takes around 286s.

High priority carriers cannot be relaxed (e.g. in relaxation in case#3)

Similarly when the MR is fully offloaded, the UE shall perform higher priority measurements in order not to break mobility. Relaxing higher priority carriers and relying solely on LR measurements will cause the UE not to measure the neighbouring cells for a long period of time. When we consider the current idle-mode measurement requirements, it is highly likely that when the UE exits LP-WUS relaxation, MR neighbouring cells start from the cell detection. This means that when the network configures thresholds for offloading or relaxation, it needs to take account also the long neighbouring cell measurement time after the LR has decided to exit LP-WUS mode.

Relaxing high priority carriers may break idle-mode mobility.

Long measurement time after offloading / relaxation exit will impact the network threshold configurations, hence reducing the usefulness of the feature.

High priority carriers are not relaxed in relaxation case#1 case#3 due to long idle-mode detection and measurement time after the relaxation.

Accuracy requirements

Issue 1-2-1: Accuracy requirements (LR)

In the previous meetings LR accuracy requirements were discussed. We still support defining clear accuracy requirements for LR. The tradition of keeping having accuracy requirement in the clause 10 only for reported measurements seems to be the main argument. We are not against of using section 10 for defining accuracy requirements, but we are also ok for defining them in a core section.

Define accuracy requirements in the core section.

Issue 1-2-8-2: Accuracy for relaxed MR neighbour cell measurement

Issue 1-2-8-1: Accuracy for normal or relaxed MR serving cell measurement

For MR, we support to re-use the legacy section 10 accuracy requirements for relaxed MR neighbouring and serving cell measurements. The requirements should be the same regardless of the scaling factor. Some companies also mention they are open to modify the existing accuracy requirements. In this case we are open to improve the legacy accuracy requirements for RRC_IDLE – INACTIVE mode.

Use legacy accuracy requirements for relaxed MR neighbouring and cell measurements.

  - If companies want to change the legacy accuracy requirements, improving them can be discussed.

Support of FR2 scenario

Issue 1-1-11: RRM requirements for FR2#

Currently there are various open issues around the LP-WUS framework. The current WID has support for FR2, but when considering the RAN4 workload, the maturity of LP-SS design, and the complexity of defining FR2 requirements wit beam sweeping. RAN4 can prioritise the FR1 requirement work over FR2-1 and FR2-2 requirements.

Proposal 6: RAN4 to prioritize defining RRM requirements for FR1 in this WI.

Selection evaluation requirements

Issue 1-2-7: On LP-WUR cell selection evaluation requirements for case 1

This issue is about whether to have LR cell selection evaluation criteria or not. LR is expected to be used in cell centre. Compared to MR cell centre observation, the LR cell centre may be different because of LR measurement accuracy. If the criteria is not defined for LR this can be handled by deployment by setting the LR exit thresholds so that MR is performing the evaluation.

It is not mandatory to define LR based cell selection criterion evaluation.

Measurement metrics

Issue 1-1-12: On LR measurement metrics LP-RSRP and LP-RSRQ

Agreement For LP-RSSI definition for LP-RSRQ, the following is agreed - Option 1: LP-RSSI is the linear average of total received power in ON and OFF LP-SS OOK symbols. ### RAN2 agreement: Procedure and configuration of LP-WUS in RRC_IDLE/INACTIVE - Baseline: The network does not need to be aware of whether the UE is monitoring LP-WUS or not in RRC_IDLE/INACTIVE - Separate entry/exit thresholds can be configured for OFDM-based and OOK-based WUR if a cell supports both types of LRs. Signalling details are FFS. - Working assumption (can revisit if R1/R4 reached different conclusions): If the entry/exit conditions are configured, besides MR-based thresholds, LP-WUS monitoring entry condition can also include LR-based thresholds. - The metrics for serving cell quality measured by MR/LR for entry condition includes (LP-)RSRP and optional (LP-)RSRQ. The metrics for serving cell quality measured by LR for exit condition includes (LP-)RSRP and optional (LP-)RSRQ.

There is an open issue, which we understand is about whether to define RSRQ measurement metrics. From the network point of view, both of them should be supported, and the network may configure LP-RSRP and LP-RSRQ thresholds with different thresholds. We can see above that RAN1 has been defining RSRQ already, and what we think RAN2 means with “optional” is that the signalling is supported but for the network the parameter is optional to configure. If the parameter is configured then the UE shall use it.

Optional LP-RSRQ in RAN2 agreement refers to network configuration, not that the support of it is optional.

Define measurement metrics for LP-RSRP and LP-RSRQ

MR evaluation requirements

Issue 1-2-5-2: On MR evaluation requirements

In the last meeting, RAN4 agreed that

According to agreements  in previous RAN4 meeting, it is up to UE implementation when and how to turn on the LR for serving cell measurements. This is ok behaviour from UE power saving perspective as it allows UE to have control over it. Furthermore, it was agreed that when both MR and LR threshold are configured, MR or LR evaluates whether its own entry/exit threshold is satisfied separately. One thing to clarify here is that when MR and LR thresholds are both configured, the UE shall enter relaxations / serving cell offloading only when both of the criteria are met, when configured. We believe this is the thinking from companies but we also think it is good to clarify as the

If both MR and LR entry thresholds are configured, the UE shall evaluate both thresholds before MR relaxations or MR offloading requirements apply.

Joint evaluation of thresholds

Issue 1-1-7: On jointly consideration on Criteria (entry/exit conditions) for fully offloading/MR RRM measurement relaxation and LP-WUS monitoring

WUS monitoring and MR measurement relaxation have different triggering requirements. The threshold to enter LP-WUS monitoring is intended to determine and ensure that UE uses LP-WUS only when reception quality is sufficient to ensure paging performance (no degradation). The threshold for MR relaxation determines when the MR measurements can be relaxed (i.e. performed less frequently) or offloaded to LR.

The main goal of LP-WUS entry/exit monitoring is to allow reduction of UE's power consumption while maintain paging performance by allowing UE to stop monitoring the traditional paging occasion (PO) while monitoring LP-WUS instead. MR relaxation, on the other hand, is focused on reducing the power consumed during the serving and neighbouring cell measurements. These are two separate mechanisms designed to optimize different aspects of power consumption, and their thresholds should be independently tuned to achieve the best results.

Network may want to configure LP-WUS entry and exit based on RSRP, while the network configures MR relaxation thresholds based on RSRQ. For instance, when UE is sectorized cell and approaches  a sector boundary, the RSRP measurements may not reflect the proximity of another cell, hence ned to do re-selection, while RSRQ values could indicate this. When considering the case 1 with RSRP, the UE could end up being out of service after crossing a sector border.

LP-WUS entry / exit and MR measurement relaxation are independent procedures from system perspective. Joint threshold evaluation may occur depending on network configuration but RAN4 should not define only requirements for this case.

LP-WUS monitoring and MR relaxation/offloading requirements shall be independent from each other.

Issue 1-2-2: Periodicity for LP-WUR RRM requirements - Proposals - P1: Regardless of whether LP-SS is configured or not, periodicity for SSB based LP-WUR measurement delay requirements is based on LO periodicity. (LG Apple CMCC oppo) - P2: For periodicity for SSB based LP-WUR measurement delay requirements, when LP-SS is configured, the periodicity for SSB based LP-WUR measurement delay requirements can be max { LO periodicity, LP-SS periodicity}. (CATT) - P3: For periodicity for SSB based LP-WUR measurement delay requirements, postpone the discussion for the case where LP-SS periodicity > LO periodicity until RAN1 has more agreements for LP-SS periodicity. (Huawei) - P4: For periodicity for SSB based LP-WUR measurement delay requirements, no requirement is defined if LO periodicity is smaller than LP-SS periodicity.(Ericsson) Background:

The RAN1 design is such that the LO periodicity is not configurable by the network, but the LO is coming once per DRX cycle. Therefore we think that for SSB it’s better to use DRX rather than LO. Currently in idle-mode it is specified that the UE shall filter the SS-RSRP and SS-RSRQ measurements of the serving cell using at least 2 measurements. For SSB based LR, similar can be assumed. Within the set of measurements used for the filtering, at least two measurements shall be spaced by DRX_IDLE cycle/2.

The measurement periodicity for SSB based LR should be at least DRX_IDLE cycle/2.

LR Measurement filtering

Issue 1-2-4-1: On LR measurement filtering requirement

Currently in idle-mode it is specified that the UE shall filter the SS-RSRP and SS-RSRQ measurements of the serving cell using at least 2 measurements. To avoid fast fading impact, the LR measurements should be at least two samples. Also our simulations seems to indicate that two samples is enough for reasonable LR accuracy.

In the last meeting it was agreed that LR evaluation requirement is not differentiated on entry or exit; or on different cases. The evaluation period can be defined as [2] measurement periods.

Proposal 12: Define evaluation requirements for the entry and the exit condition for all cases based on evaluation period which includes [2] measurement periods. Each measurement is based on filtering of at least 2 samples.

Idle-mode measurements and LR

Issue 1-5-1: LR based RRM with EMR in IDLE/inactive mode

In the previous meetings, companies have brought up issues related to early measurement reporting. The rel-18 work on fast CA/DC setup expanded the idle-mode procedures to cover reselection measurements also. RAN4 should discuss whether the relaxation factor applies to all EMR or whether the validity of the measurement results can be guaranteed when the UE has relaxing MR measurements.

RAN4 to discuss the UE’s behaviour of MR relaxation with EMR impact

Some companies have already proposed that the UE shall keep the MR measurements on while there is EMR timer running, this should also work when UE is in idle-mode less than the duration of the timer.

The configuration for both EMR and LP-WUS is coming from system information, which means that the UE may not be configured by the same cell as the UE left. Therefore, if the UE is configured simultaneously with EMR and LP-WUS the UE should not use LR.

Discuss about two alternatives to handle the situation

  - When T331 running UE shall keep the MR ON for EMR measurement regardless of the MR offloading condition is met or not.

  - UE shall not enter relaxation / offloading if configured simultaneously with EMR.

LP-WUR in connected mode

Issue 1-4-1: LP-WUR at CONNECTED mode

Currently it is not clear if RAN4 should be defining any requirements for the connected mode. For the scenario where UE MR is part of MR it is highly likely that the receiver is also capable of performing SSB measurements. Therefore, there may not be need to add any special handing for connected mode in this scenario, at least in Rel-19 scope.

Prioritise idle-mode requirements over connected mode requirements.

Lower / Upper bound on LP-SS periodicity

Issue 1-2-2-1: Lower bound on LP-SS measurement periodicity

This issue is about mandating lower bound to LP-SS. So far RAN1 has agreed LP-SS periodicity of 320ms. From the network point of view this means that the network does not need to transmit LP-SS more than every 320ms. Before RAN1 agrees other periodicities, the LP-SS periodicity assumption should be 320ms. As far as the network is concerned, when UEs with LP-SS are configured in the network, the LP-SS is only transmitted every 320ms if no further agreements made in RAN1.

RAN1 Agreement For LP-SS periodicity, support at least 320ms - FFS: 80ms,160ms, 640ms,1280ms, 2560ms, 5120ms, 10240ms

No need for lower bound in requirements use the agreed 320ms periodicity.

Issue 1-2-2-2: Upper bound on SSB-based LP-WUR measurement periodicity

Similarly to lower bound, there is no need to restrict any configurations on upper bound, the requirements should assume reasonable configuration from the network side.

Assume that the network configuration is reasonable. No need to define upper bound for SSB-based LP-WUR measurement periodicity

Issues pending RAN2

The following issues can wait for further input from RAN2:

  - Issue 1-1-1: Cases/states to be considered for RRM relaxation and serving cell measurement offloading

  - Issue 1-1-5: Criteria (entry/exit conditions) for fully offloading case and MR RRM measurement relaxation

  - Issue 1-1-6: Criteria (entry/exit conditions) for LP-WUS monitoring

  - Issue 1-1-7: On jointly consideration on Criteria (entry/exit conditions) for fully offloading/MR RRM measurement relaxation and LP-WUS monitoring

RAN2 is still working on entry / exit  and offloading / relaxation criteria. RAN4 can prioritise other issues than

  - Cases/states to be considered for RRM relaxation and serving cell measurement offloading

  - Criteria (entry/exit conditions) for fully offloading case and MR RRM measurement relaxation

  - Criteria (entry/exit conditions) for LP-WUS monitoring

  - On jointly consideration on Criteria (entry/exit conditions) for fully offloading/MR RRM measurement relaxation and LP-WUS monitoring

Conclusion

The following Observations and Proposals were made:

Proposal 1: 1 SSB samples is enough for sync in good radio conditions where LR is assumed to be used.

Proposal 2: Confirm that the following wake up times with sync times are ok for RAN4:

Proposal 3: After MR wake up, MR follows SSB periodicity at least for first set of measurements

Observation 1: High priority carriers cannot be relaxed (e.g. in relaxation in case#3)

Observation 2: Relaxing high priority carriers may break idle-mode mobility.

Observation 3: Long measurement time after offloading / relaxation exit will impact the network threshold configurations, hence reducing the usefulness of the feature.

Proposal 4: High priority carriers are not relaxed in relaxation case#1 case#3 due to long idle-mode detection and measurement time after the relaxation.

Proposal 5: Define accuracy requirements in the core section.

Proposal 6: Use legacy accuracy requirements for relaxed MR neighbouring and cell measurements.

	If companies want to change the legacy accuracy requirements, improving them can be discussed.

Proposal 7: It is not mandatory to define LR based cell selection criterion evaluation.

Observation 4: Optional LP-RSRQ in RAN2 agreement refers to network configuration, not that the support of it is optional.

Proposal 8: Define measurement metrics for LP-RSRP and LP-RSRQ

Proposal 9: If both MR and LR entry thresholds are configured, the UE shall evaluate both thresholds before MR relaxations or MR offloading requirements apply.

Observation 5: LP-WUS entry / exit and MR measurement relaxation are independent procedures from system perspective. Joint threshold evaluation may occur depending on network configuration but RAN4 should not define only requirements for this case.

Proposal 10: LP-WUS monitoring and MR relaxation/offloading requirements shall be independent from each other.

Proposal 11: The measurement periodicity for SSB based LR should be at least DRX_IDLE cycle/2.

Proposal 12: RAN4 to discuss the UE’s behaviour of MR relaxation with EMR impact

Proposal 13: Discuss about two alternatives to handle the situation

	When T331 running UE shall keep the MR ON for EMR measurement regardless of the MR offloading condition is met or not.

	UE shall not enter relaxation / offloading if configured simultaneously with EMR.

Proposal 14: Prioritise idle-mode requirements over connected mode requirements.

Proposal 15: No need for lower bound in requirements use the agreed 320ms periodicity.

Proposal 16: Assume that the network configuration is reasonable. No need to define upper bound for SSB-based LP-WUR measurement periodicity

Proposal 17: RAN2 is still working on entry / exit  and offloading / relaxation criteria. RAN4 can prioritise other issues than

	Cases/states to be considered for RRM relaxation and serving cell measurement offloading

- Agreements in RAN4#114

Issue 1-1-3-2: On RRM requirements to be specified for SSB-based LP-WUR based only on LP-SS with and without overlaid OFDM sequence Agreement: - Define the requirements for OFDM-based LP-WUR serving cell measurement based only on LP-SS - Reuse the same requirement for LP-SS OOK based Issue 1-1-9: LP-WUR status at legacy case (not at LP-WUS monitoring case/fully offloading(case 1) case/RRM relaxation (case 3) case) Agreement: At legacy state, when and how to turn on LR for serving cell measurement is up to UE implementation. Issue 1-2-3: On requirements for entry/exit criteria(threshold) evaluation for WUS paging monitoring/Fully Offloading (Case 1)/MR RRM relaxation (Case 3) Agreement: When both MR and LR threshold are configured, MR or LR evaluates whether its own entry/exit threshold is satisfied separately. MR and LR evaluation requirements are defined separately. Issue 1-2-4-2-2: On single or multiple LR evaluation requirements Agreement: Define a single LR evaluation requirement for both LR entry and exit threshold evaluation for LP-WUS monitoring, case 1 and case 3, i.e., LR evaluation requirement is not differentiated on entry or exit; or on different cases. Note: RAN2’s discussion on timer will not impact on RAN4’s evaluation requirement. Issue 1-2-4-2-3: On how to define LR evaluation requirements Agreement: LR evaluation duration is [x1 samples]*LP-SS (for OOK LR) or [y1 samples] *LO (for SSB LR), assuming x or y samples are used to satisfy accuracy requirement and x1 > x and y1>y. Number x and y are defined as measurement requirement as [x samples]*LP-SS (for OOK LR) or [y samples] *LO (for SSB LR) Issue 1-3-1: MR RRM relaxation for serving cell/neighbour cell for case 3 Agreement: - MR RRM relaxation for serving cell/neighbour cell: scaling factor is 16.