3GPP TSG-RAN WG4 Meeting #114bis	R4-2504470

Wuhan, China, April 7th – April 11th, 2025

Source:	Nokia

Title:	Discussion on CSI-RS based L1 measurements

Agenda item:	7.27.2.2

Document for:	Discussion

Introduction

This contribution discusses about the open issues related to CSI-RS measurements and CSI-RS acquisition before and after the cell switch.

Discussion

Known cell definition in CSI-RS based L1 measurement

The following aims to summarise the previous agreements about the known cell condition:

RRM requirements for CSI-RS based L1-RSRP measurement only apply for known target cells For CSI-RS based L1-RSRP measurement, the cell is considered as known if the following conditions are met: - The UE has performed L3 measurement on the target cell during the last 5 seconds, and - The SSB from the target cell configured for L3[/L1] measurement remains detectable according to the cell identification requirements in clause 9.2. - The CSI-RS from the target cell configured for L1 measurement remains measurable - [SINR ≥ -3dB]

Currently one of the open issues related to know condition is the side condition. Our understanding is this will also impact which accuracy requirements apply on CSI-RS measurements (i.e., -6 or -3 requirements). As the CSI-RS measurements are expected to be the same as are eventually continued for CSI-RS measurements for beam management at the target cell, it makes the most sense to use the same side condition as is used for the serving cell CSI-RS measurements.

Use legacy side condition of SINR= -3dB

Measurement procedure

- Agree the following measurement procedure for CSI-RS based LTM candidate cell measurements: - Step 1: SSB based L3 measurements and reports at least before LTM configuration. - Step 2: For FR2-1, SSB-based L1 measurements and reports on the same candidate cell - Further discuss whether Step 2 can be skipped under certain conditions or based on UE capability or based on network RRC configuration. FFS the details. - Note: Step 2 is removed for FR1. - Step 3: CSI-RS based L1 measurements on the same candidate cell - Depending on whether step 2 is skipped, the requirements in step 3 may be different. - Note: The CSI-RS in Step 3 should be QCL’ed with the SSB in Step 1 or 2. - [Note: For FR1, the CSI-RS in Step 3 should be QCL’ed with the SSB in Step 1.] Issue 4-2-1: Whether to keep or remove SSB based L1 measurement for FR2-1 in step 2 - Option A: Whether step 2 can be skipped or not can be based on UE capability reporting. (Apple, CATT, Nokia, Ericsson, Qualcomm, CMCC - compromise, ZTE, China Telecom, vivo, Xiaomi, OPPO) - Option B: Always keep step 2 for FR2-1 (MTK, HW) - Option C: Remove L1 measurement for FR2-1 in the 2nd bullet. (CMCC, OPPO)

In the last meeting, the basic measurement procedure for CSI-RS L1 measurement on candidate cells was discussed and the following steps are currently considered:

- Step 1: UE performs SSB based L3 measurement and acquires the SSB index information.

  - The beam assumptions of this procedure: gNB TX wide SSB beam, UE RX wide SSB beam

- Step 2: For FR2-1 UE performs SSB based L1-RSRP measurement with beam sweeping on the candidate cell to refine the SSB based RX beam.

  - Option 1: UE performs SSB based beam sweeping, and in CSI-RS requirements beam sweeping is omitted.

  - Option 2: UE does not perform SSB based beam sweeping, and in CSI-RS requirements beam sweeping is enabled.

- Step 3 (gNB TX narrow CSI-RS beam, UE RX fine SSB beam): UE performs measurements on the configured CSI-RS resources of candidate cell where each CSI-RS resource is QCL-typeD with SSB for L1-RSRP measurement in step 2.

  - Option 1: UE does not perform CSI-RS beam sweeping because of Step 2 option 1. Beam sweeping is omitted.

  - Option 2: UE performs CSI-RS measurement directly based on L3 SSB measurements, and performs beam sweeping on CSI-RS

For Step 2 and Step 3 option 1, the UE uses Rel-18 SSB beam sweeping and the CSI-RS beam sweeping can be omitted from the requirements

For Step 2 and Step 3 option 2, the UE needs to support CSI-RS based beam sweeping for CSI-RS that is measured after L3 SSB based measurements.

We understand that using SSB L1 measurements for beam sweeping is more legacy implementation friendly while the CSI-RS beam sweeping is more advanced mechanism. From the requirements point of view, it is not a problem to define requirements for both cases. For UE supporting CSI-RS beam sweeping capability, step 2 can be skipped. For UEs that do not support that, will perform L1 SSB based beam sweeping for FR2-1.

Define the following measurement procedure for rel-19 LTM

  - Step 1: UE performs SSB based L3 measurements and reports at least before LTM configuration.

  - Step 2: For FR2-1, [For UE not supporting CSI-RS based beam sweeping], UE performs SSB-based L1 measurements and reports the measurement results on the same candidate cell. Otherwise, skip step 2.

  - Step 3: For FR1 and FR2-1

    - [For UE supporting CSI-RS based beam sweeping], UE performs CSI-RS based L1 measurements on the same candidate cell with beam sweeping

    - [For UE not supporting CSI-RS based beam sweeping], perform CSI-RS based L1 measurements on the same candidate cell without beam sweeping

Definition of intra-frequency/inter-frequency for CSI-RS based L1 measurement

Candidate options: - Option 1: (Apple, ZTE, MTK, CMCC, Huawei, Ericsson) A measurement is defined as a CSI-RS based intra-frequency measurement provided that: - the SCS of the CSI-RS resource of the neighbor cell configured for L1 measurement is the same as the SCS of active DL BWP, and - the CP type of the CSI-RS resource of neighbor cell configured for L1 measurement is the same as the CP type of active DL BWP, and - It is applied for SCS = 60KHz - the center frequency of the CSI-RS resource of the neighbour cell configured for L1 measurement is the same as the centre frequency of any of the CSI-RS resources within DL BWP (if configured). - Option 2: (Qualcomm, MTK, Nokia, E///, vivo, China Telecom, HW, ZTE, CATT) A measurement is defined as a CSI-RS based intra-frequency L1 measurement provided that: - the SCS of the CSI-RS resource of the neighbour cell configured for L1 measurement is the same as the SCS of active DL BWP, and - the CP type of the CSI-RS resource of neighbour cell configured for L1 measurement is the same as the CP type of active DL BWP, and - the CSI-RS resource of the neighbour cell configured for L1 measurement is included within the active DL BWP. - at least 48 RBs of the CSI-RS resource, needs to be confined within the active DL BWP.

The main aspect here is that we should define intra-frequency CSI-RS resources in such a way that they can be measured without a measurement gap, and there is no need for a definition of inter-frequency without gap scenario.

In release-18 RAN4 decided to define requirements for inter-frequency without gap with the same requirements as intra-frequency. From the network point of view it would be better to address this type of scenario within intra-frequency requirements as it is generally understood that intra-frequency measurements are gapless measurements.

Regarding the center frequency limitation in the requirements, we don’t see a technical reason to consider this as if we define that at least 48 PRBs of the CSI-RS resource are within active BWP, that should be enough configuration limitation to allow UE to measure the CSI-RS resources without gap and to fulfil RAN4 requirements which are assuming 48 PRBs.

If some companies want to have the center-frequency limitation in their implementations, they can indicate to the network a capability where they say that they are not able to measure if the CSI-RS centre frequencies are not aligned. This way the network can decide not to configure any CSI-RS for these UEs.

Do not consider the center frequency as a part of intra-frequency definition

For intra-frequency, at least 48 PRBs of the neighbouring cell CSI-RS resource needs to be confined within the active DL BWP

RTD assumption and CSI-RS with gaps

Candidate options: - Option 1: CMCC, ZTE, CATT, China Telecom - Support L1 measurement with gap for CSI-RS based L1 measurement on neighbour cell, as optional UE feature. - Define requirements for RTD > CP, as optional UE feature. FFS details - Option 2: E///, HW, CATT, Apple - compromise, CMCC - compromise - Support L1 measurement with gap for CSI-RS based L1 measurement on neighbour cell, as optional UE feature. - No Ran4 requirements for RTD > CP. - Option 3: - Not Support L1 measurement with gap for CSI-RS based L1 measurement on neighbour cell. - Define requirements for support RTD > CP, as optional UE feature. FFS details - Option 4: QC, Apple, vivo, Xiaomi, Samsung, MTK, OPPO, Nokia - compromise, E/// - compromise - Not Support L1 measurement with gap for CSI-RS based L1 measurement on neighbour cell. - No Ran4 requirements for RTD > CP. Agreement: - Conclude this issue in the next meeting.

CSI-RS measurements would need to share measurement gaps with SSB-based measurements. When considering the use case for CSI-RS measurements with gap, it is very difficult to find a scenario where there would be any justification for the added complexity and that would overcome the negative effects of the CSI-RS within the gap.

As we are all aware, the network does not have visibility to RTD conditions on the UE side. However, in the last meeting we agreed to compromise not to define RTD > CP requirements. When considering the time we have to define the rest of the core requirements, we should focus on the scenarios where RTD < CP and CSI-RS measurements are performed without gaps.

The following is agreed together:

  - Not Support L1 measurement with gap for CSI-RS based L1 measurement on neighbour cell.

  - No Ran4 requirements for RTD > CP.

#### Issue 4-9: Measurement gap related Candidate options: < omitted >

As discussed in the above issue, we prefer not to mix CSI-RS measurement gaps with SSB based measurement gaps. The issue about RTD assumption and MG should be discussed first before agreeing on measurement gaps. If measurement gaps are to be supported, the minimum condition should be that they do not impact legacy SSB based L1 or L3 measurements.

If gap-based CSI-RS measurements are supported, define the gap-based CSI-RS measurement which do not impact the legacy SSB based L1-/L3- measurements.

Definition of “inside/outside active BWP”

Candidate options: - Option 1: (Apple) The CSI-RS of candidate cell should be considered inside the active BWP if all the resources are confined within the BWP. Otherwise, it is outside of active BWP. - Option 2: (CATT) RAN4 to firstly align the understanding of ‘the CSI-RS of candidate cell outside active BWP’ among companies. It is easier to define what ‘the CSI-RS of candidate cell within active BWP’ is, and all cases except for ‘within’ are considered as ‘the CSI-RS of candidate cell outside active BWP’. - Option 3: (vivo) From RRM requirement perspective, RAN4 further discuss which option is taken as the clarification for ‘the CSI-RS of candidate cell outside active BWP’ in R19 LTM: Option 1: a CSI-RS resource is regarded as outside active BWP if the frequency domain occupied RBs of the CSI-RS include at least one RB that is outside any active BWP. Option 2: a CSI-RS resource is regarded as outside active BWP if the frequency domain occupied RBs of the CSI-RS are all outside active BWP(s) of the UE. Option 3: a CSI-RS resource is regarded as outside active BWP if the frequency domain occupied RBs of the CSI-RS include no more than 48 RBs within any active BWP.

There may not be a need to define what inside or outside of the active BWP if we define that for intra-frequency least 48 PRBs are within active BWP. If something needs clarification, we propose to describe it this way:

Fully within active BWP means that all the resources are confined within the active BWP.

  - Partially within active BWP means that some of the resources may be outside of the active BWP but at least 48 PRBs are within the active BWP

  - Outside of the active BWP means that less than 48 PRBs are within the active BWP.

Measurement on CSI-RS with repetition ON/OFF

Candidate options: - Option 1: (CATT, CT, vivo) In FR2, RAN4 will not define the requirements for CSI-RS resources with repetition ON for L1 measurement on neighbor cell. In FR2, RAN4 will define the requirements for the following case: CSI-RS resources is configured with repetition OFF, UE perform CSI-RS based L1-RSRP measurement without RX beam sweeping after UE has performed L1-RSRP measurement on the associated SSB. - Option 2: (Huawei) Not to support CSI-RS resources with repetition ON in FR2 for CSI-RS based L1 RSRP measurement on candidate cell. - Option 3: (Nokia) Wait until RAN1 has agreed to support repetition “ON” configuration before agreeing to define requirements. As a baseline, define requirements for the case where the repetition is ‘OFF’. Revisit the agreement if RAN1 decides to support “ON” configuration. - Option 4: (Ericsson, Apple, Nokia) RAN4 to wait for RAN1 conclusion on support of CSI-RS with repetition ON.

As can be seen from the RAN1 WF, there is still discussion ongoing whether to support the repetition “ON” configuration. To include this scenario, we think it’s quite important to also have configuration for it. If RAN1 agrees to have no configuration, then RAN4 doesn’t need to define requirements.

A CSI-RS resource set configured with higher layer repetition is used for beam management, where the value of the repetition can be set to ‘ON’ or ‘OFF’. A CSI-RS resource set with repetition set to ‘ON’ is used for the UE to evaluate quality across different DL Rx beams, allowing the UE to determine a fine DL Rx beam and a corresponding UL Tx beam, conditional to UE supporting beam correspondence. The procedure is triggered by the network using dynamic CSI-RSs (semi-persistent or aperiodic), where only a few selected DL Tx beams are used for repeated CSI-RS transmissions with the same DL Tx beam. According to our understanding, semi-persistent and/or aperiodic CSI-RSs support would be needed. As these decisions are still pending in RAN1, RAN4 may start from defining requirements with the assumption that the repetition is set to ‘OFF’.

Although it is possible to configure a CSI-RS resource set with repetition set to 'ON' using periodic CSI-RSs, this may be feasible only for limited scenarios (e.g., with a small number of CSI-RS-based gNB beams), and it’s currently not clear even how many candidate cells UE is measuring before cell switch or how CSI-RS beam selection is made. For instance, if TCI state activation is used, or L3 SSB based selection is used using CSI-RS that is QCL’ed with SSB.  Based on the cell selection, we would also need to consider whether UE has performed L3 and L1 measurements, and what has been the Rx assumption.

If UE measures neighbouring cell CSI-RS with repetition “ON”, it needs to be discussed whether the measurements are applicable after the UE has entered the cell switch interruption. It hardly makes sense to define requirements for early CSI-acquisition procedure if the UE needs to repeat the procedure after the interruption.

Wait until RAN1 has agreed to support repetition “ON” configuration before agreeing to define requirements. As a baseline, define requirements for the case where the repetition is ‘OFF’. Revisit the agreement if RAN1 decides to support “ON” configuration.

CSI-RS resource type

#### Issue 4-8: Candidate options: - Option 1: (Apple, OPPO, Nokia, vivo) - RAN4 prioritize CSI-RS based L1 measurement requirement for periodic CSI-RS and FFS on aperiodic and semi-persistent CSI-RS pending RAN1 decision. - Option 2: (CMCC, Ericsson) Semi-persistent CSI-RS resource for LTM candidate cells is supported. - Option 3: (CATT) RAN4 to define CSI-RS based L1 measurement for periodic and semi-persistent CSI-RS.

RAN1 agreed to support both periodic and semi-persistent CSI-RS resource configuration and at least for non-event triggered reporting towards serving cell. RAN4 should therefore define requirements for both.

RAN4 to define CSI-RS based L1 measurement for periodic and semi-persistent CSI-RS.

Definition of frequency layer and CSI-RS timing configuration

Candidate options: - Option 1: (Apple) - Definition of frequency layer can be considered after intra-frequency/inter-frequency layer definition and measurement type are concluded. - Option 2: (MTK) - If RAN4 concludes not to define CSI-RS based L1 measurement requirements for both RTD>CP case and CSI-RS outside active BWP, it is not necessary to define “frequency layer”. - Option 3: (Huawei) - If the center frequency, SCS and CP length of the to-be-measured L1 CSI-RS resources are the same, they are regarded as the same CSI-RS frequency layer. - Option 4: (Nokia) Not to define new terminology of “frequency layer” for CSI-RS based L1-RSRP measurement. Discuss directly the intra/inter-frequency CSI-RS scenarios where CSI-RS resources have similar properties (SCS, CP type, periodicity) - Option 5: (Ericsson) Two CSI Resource are of same frequency layer if they have same SCS, CP type, BW, center frequency, and the CSI-RS occasions are fully/partially overlapping in time domain.

This issue can resolved once other issues have been progressing. If RAN4 concludes not to define CSI-RS based L1 measurement requirements for both RTD>CP case and CSI-RS outside active BWP, it is not necessary to define “frequency layer”.

Definition of frequency layer can be considered after intra-frequency/inter-frequency definition and measurement type are concluded.

CSI-Acquisition before and after the cell switch

Candidate options: - Option 1: (ZTE) Wait for more RAN1 progress to start the discussion on CSI acquisition in RAN4. - Option 2: (Nokia) CSI-acquisition may start before the cell switch (e.g. early CSI-acquisition case) or it may start from the cell switch command (e.g. After CSC CSI-acquisition). RAN4 can start to discuss if and how CSI-acquisition is performed during and after the cell switch. - Option 3: (Qualcomm) For the baseline CSI acquisition (DL CSI acquisition after LTM CSC MAC-CE reception), RAN4 to not define requirements. For the optional CSI acquisition (DL CSI acquisition before LTM CSC MAC-CE reception), RAN4 to discuss whether and what requirements to define after the following aspects for CSI-RS based L1-RSRP measurement from the candidate cell are concluded: • Pre-requisite conditions, maximum RTD value, gap-less vs. gap-based, etc.

According to the latest RAN4 agreements, the CSI-acquisition may start before or after the cell switch. Depending on which time the UE will start the CSI-Acquisition after the cell switch command will impact the time the UE has to receive the reference signal and process it. If the time when UE is able to start the CSI-acquisition is not precisely defined, the network is not able to transmit the CSI-RS within the cell switch delay so that the UE would have time to report the cell switch on time.

Discuss what is the point in time during the cell switch delay UE starts CSI-acquisition.

CSI-RS accuracy requirements

Candidate options: - Option 1: RAN4 to discuss the CSI-RS accuracy requirements during performance part and CSI-RS based L1-RSRP accuracy of the serving cell can be used as baseline. (Apple, ZTE)

We think as a baseline, we should not consider the CSI-RS for mobility accuracy requirements. RAN4 can reuse the legacy serving cell CSI-RS accuracy requirements.

RAN4 to discuss the CSI-RS accuracy requirements during performance part and CSI-RS based L1-RSRP accuracy of the serving cell can be used as baseline

TCI state activation and CSI-RS measurements

The CSI-RS measurements may follow similar behaviour as the legacy LTM SSB based measurements with regards to TCI state activation. After TCI state activation command, CSI-RS measurement(s) for the cells whose TCI state(s) are in the active TCI state list shall be prioritized

After TCI state activation command, CSI-RS measurement(s) for the cells whose TCI state(s) are in the active TCI state list shall be prioritized

UE capability to measure number of CSI-RSs

Similar to legacy SSB based measurements in rel18. If the number of resources/cells, including the number of resources/cells configured for CSI-RS based L1-RSRP measurement exceeds the UE capability, it is up to UE implementation on how to choose cells/CSI-RS to measure. Generally, the network does not configure resources so that they exceed UE capability as it’s not a very good practice.

If the number of resources/cells, including the number of resources/cells configured for CSI-RS based L1-RSRP measurement exceeds the UE capability, it is up to UE implementation on how to choose cells/CSI-RS to measure

Number of CSI-RS resources

In the legacy L3 based CSI-RS requirements, during each measurement period, the UE shall be capable of performing CSI-RSRP, CSI-RSRQ measurements for at least:

-	For FR1 32 CSI-RSs with different CSI-RS index and/or PCI on the intra-frequency layer, and

-	For FR2 32 CSI-RSs with different CSI-RS index and/or PCI on the intra-frequency layer

Discuss how many measurements CSI-RS measurements UE is capable of performing during each measurement period.

Conclusion

The following Observations and Proposals were made:

Proposal 1: Use legacy side condition of SINR= -3dB

Proposal 2: Define the following measurement procedure for rel-19 LTM

  - Step 1: UE performs SSB based L3 measurements and reports at least before LTM configuration.

  - Step 2: For FR2-1, [For UE not supporting CSI-RS based beam sweeping], UE performs SSB-based L1 measurements and reports the measurement results on the same candidate cell. Otherwise, skip step 2.

  - Step 3: For FR1 and FR2-1

    - [For UE supporting CSI-RS based beam sweeping], UE performs CSI-RS based L1 measurements on the same candidate cell with beam sweeping

    - [For UE not supporting CSI-RS based beam sweeping], perform CSI-RS based L1 measurements on the same candidate cell without beam sweeping

Proposal 3: Do not consider the center frequency as a part of intra-frequency definition

Proposal 4: For intra-frequency, at least 48 PRBs of the neighbouring cell CSI-RS resource needs to be confined within the active DL BWP

Proposal 5: The following is agreed together:

- Not Support L1 measurement with gap for CSI-RS based L1 measurement on neighbour cell.

- No Ran4 requirements for RTD > CP.

Proposal 6: If gap-based CSI-RS measurements are supported, define the gap-based CSI-RS measurement which do not impact the legacy SSB based L1-/L3- measurements.

Proposal 7: Fully within active BWP means that all the resources are confined within the active BWP.

  - Partially within active BWP means that some of the resources may be outside of the active BWP but at least 48 PRBs are within the active BWP

  - Outside of the active BWP means that less than 48 PRBs are within the active BWP.

Proposal 8: Wait until RAN1 has agreed to support repetition “ON” configuration before agreeing to define requirements. As a baseline, define requirements for the case where the repetition is ‘OFF’. Revisit the agreement if RAN1 decides to support “ON” configuration.

Proposal 9: RAN4 to define CSI-RS based L1 measurement for periodic and semi-persistent CSI-RS.

Proposal 10: Definition of frequency layer can be considered after intra-frequency/inter-frequency definition and measurement type are concluded.

Proposal 11: Discuss what is the point in time during the cell switch delay UE starts CSI-acquisition.

Proposal 12: RAN4 to discuss the CSI-RS accuracy requirements during performance part and CSI-RS based L1-RSRP accuracy of the serving cell can be used as baseline

Proposal 13: After TCI state activation command, CSI-RS measurement(s) for the cells whose TCI state(s) are in the active TCI state list shall be prioritized

Proposal 14: If the number of resources/cells, including the number of resources/cells configured for CSI-RS based L1-RSRP measurement exceeds the UE capability, it is up to UE implementation on how to choose cells/CSI-RS to measure

Proposal 15: Discuss how many measurements CSI-RS measurements UE is capable of performing during each measurement period.