3GPP TSG-RAN WG4 Meeting #114bis	R4-2504471

Wuhan, China, April 7th – April 11th, 2025

Source:	Nokia

Title:	LS reply to RAN2 on granularity definition of pdcch-RACH-AffectedBandsList-r18

Agenda item:	8.2	R18 related (NR_Mob_enh2)

Document for:	Discussion

Introduction

This paper discusses about the LS received from RAN2 (R2-2501392) and proposes LS reply in the annex.

Discussion

RAN4 received an LS with the following core text, the full LS text is in the appendix.

- Understanding 1: The source band of the band pair represents the source band (or PCell) on which the PDCCH order is sent to the UE to perform PDCCH-ordered early RACH to the target cell/band. Regarding the interruption band(s), there are two further understandings, denoted by 1a and 1b: - Understanding 1a: The value “interruption/noInterruption” means that interruption is needed or not for all serving bands of the current band combination. - Understanding 1b: The value “interruption/noInterruption” means that interruption is needed or not for the source band (the same band in which the PDCCH order is sent) of the current band combination. For other serving bands, noInterruption can be always assumed. - Understanding 2: The source band of the band pair represents the specific source band that have interruption or not (depends on the value “interruption” or “noInterruption”) in the BC when UE performs PDCCH-ordered early RACH toward a target cell on target band.

Our understanding is that the question is whether the UE causes interruption only on source band where the PDCCH ordered RACH was transmitted, or for all the bands. The following figure illustrates the situation:

Figure 1: Interruption pdcch-RACH-AffectedBandsList-r18

To discuss further the issue, RAN4 can discuss what is the Tx assumption on LTM PRACH transmission, and whether UE supporting LTM early PRACH should be capable of two simultaneous Tx when PDCCH ordered RACH is transmitted.

Discuss the LTM UE assumptions related to PRACH transmission.

From our point of view, the RAN4 requirements are less ambiguous than the current capability text. If we interpret RAN4 requirements and consider the option 1b,  the value “interruption” means that interruption is allowed for source serving band which active serving cell’s DL slot(s) overlapped with RACH occasion. Similarly, the value “No Interruption” means that UE RACH transmission does not cause interruption for any of the bands indicated.

Discuss whether the interruption is only on source band or all of the bands

Currently RAN2 is only considering the RACH transmission interruption. However, RAN4 defines also more interruption around the PRACH transmission. It would be good to make RAN2 aware that the interruption caused by PRACH transmission may be much more insignificant compared to the other components when the PRACH bandwidth is outside of the active UL BWP.

Clarify the interruption in LS reply when PRACH bandwidth is outside active UL BWP

Finally, we propose a reply LS to be sent to RAN2.

Use the reply LS in the annex to reply RAN2

Conclusion

The following Observations and Proposals were made:

Proposal 1: Discuss the LTM UE assumptions related to PRACH transmission.

Proposal 2: Discuss whether the interruption is only on source band or all of the bands

Proposal 3: Clarify the interruption in LS reply when PRACH bandwidth is outside active UL BWP

Proposal 4: Use reply LS in the annex to reply RAN2

LS Reply to RAN2

3GPP TSG RAN WG4 Meeting #114bis		R2-25xxxxx

Wuhan, China, 07th – 11th April. 2025

Title:	LS reply to RAN2 on granularity definition of pdcch-RACH-AffectedBandsList-r18

Response to:

Release:	Rel-18

Work Item:	NR_Mob_enh2-Core

Source:	RAN4

To:	RAN2

Cc:

Contact person:	Jani-Pekka Kainulainen

jani.pekka.kainulainen@nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

- Overall description

RAN4 would like to thank RAN2 for the LS R2-2501392:

The UE capability pdcch-RACH-AffectedBandsList-r18 (FG39-4) is defined in RAN4 with the granularity as “Per band pair per band combination (between the target band for RACH transmission and band under UE’s current band combo)” For each reported capability, there are two possible values: interruption or noInterruption. Regarding the band pair (one source band +one target band) of the BC, RAN2 understands that the target band refers to the target band towards which the UE performs PDCCH-ordered early RACH. However, the definition of the source band of the band pair remains unclear in RAN2. - Understanding 1: The source band of the band pair represents the source band (or PCell) on which the PDCCH order is sent to the UE to perform PDCCH-ordered early RACH to the target cell/band. Regarding the interruption band(s), there are two further understandings, denoted by 1a and 1b: - Understanding 1a: The value “interruption/noInterruption” means that interruption is needed or not for all serving bands of the current band combination. - Understanding 1b: The value “interruption/noInterruption” means that interruption is needed or not for the source band (the same band in which the PDCCH order is sent) of the current band combination. For other serving bands, noInterruption can be always assumed. - Understanding 2: The source band of the band pair represents the specific source band that have interruption or not (depends on the value “interruption” or “noInterruption”) in the BC when UE performs PDCCH-ordered early RACH toward a target cell on target band. Please note that in Understanding 1, the UE cannot indicate which specific source band of the BC has an interruption or not (either all bands in current BC in 1a or the same source band that receives the PDCCH order in 1b). In Understanding 2, the information about which source band receives the PDCCH order is not provided.

RAN4 understanding of the issue is that the UE is allowed interruption on source serving band(s) DL slots overlapping with RACH transmission occasion that have active serving cell(s).

RAN4 38.133 8.2.2.2.20 states:

##### 8.2.2.2.20 Interruptions due to PDCCH ordered RACH on target LTM cell When PRACH transmission is triggered by PDCCH order on target LTM cell, - During RACH transmission, the UE is allowed an interruption on activated serving cell’s DL slot(s) overlapped with RACH occasion depending on UE capability pdcch-RACH-AffectedBandsList-r18. - Before and after PRACH transmission, UE is allowed an interruption on any activated serving cell as follows: - of up to the duration shown in table 8.2.2.2.20-1, if PRACH bandwidth is outside active UL BWP but within one of configured UL BWPs of any active serving cell, or - of up to ceil(Y/NR Slot length)+1 slots, where Yms as reported in pdcch-RACH-SwitchingTimeList-r18, if PRACH bandwidth is outside any of the configured UL BWPs of any active serving cell Table 8.2.2.2.20-1: Interruption duration due to PDCCH ordered RACH on target LTM cell when PRACH bandwidth is outside active UL BWP but within one of configured UL BWPs of any active serving cell NR Slot length (ms) of victim cell; Interruption length X (slots). - 0; - 1; - 2. - 1; - 0.5; - 2. - 2; - 0.25; - 4. - 3; - 0.125; - 6.

Therefore,

The value “interruption” means that interruption is allowed for source serving band which active serving cell’s DL slot(s) overlapped with RACH occasion

The value “No Interruption” means that UE RACH transmission does not cause interruption for any of the bands indicated.

RAN4 would respectfully request RAN2 to take into consideration the interruption before and after PRACH transmission specified in 8.2.2.20.

# 2	Actions

To RAN4

ACTION: 	RAN4 respectfully requests RAN4 to take the above into consideration.

# 3	Dates of next TSG RAN WG 4 meetings

TSG RAN WG5 Meeting #115		19th May – 23rd May 2025                       St Julian, Malta

TSG RAN WG5 Meeting #116		25th August – 29rd August 2025              Bangalore, India