3GPP TSG-RAN WG4 Meeting #114bis	R4-2504503

Wuhan, China, April 7th – April 11th, 2025

Source:	Nokia

Title:	Simulation results for CJT RRM requirements

Agenda item:	7.22.3.2

Document for:	Discussion

Introduction

Simulation results based on the assumptions described in [1] and some updated assumptions based on our companion paper in [2].

Discussion

Description of scenarios

- Scenario 1: TRP1 and TRP2 transmitting only TRS

- Scenario 2: Data on all the OFDM symbols, except for the symbols carrying TRS.

- Scenario 3: Data on all the OFDM symbols; except for the resource elements carrying TRS.

Figure 1. Illustraion of the Scenario 1: TRP1 and TRP2 transmitting only TRS.

Figure 2. Illustraion of the Scenario 2: Data on all the OFDM symbols, except for the symbols carrying TRS.

Figure 3. Illustraion of the Scenario 3: Data on all the OFDM symbols; except for the resource elements carrying TRS.

CJT simulation results

Carrier frequency offset (CFO) requirements

Table 1. Simulation assumptions for the CFO reporting simulations.

| Parameter | Value |
| --- | --- |
| Waveform | OFDM |
| Carrier Frequency | 3.5GHz |
| SCS | 15kHz, 30kHz |
| Channel Model | TDLC300-10 |
| TRP number | 2 |
| UE antenna configuration | 1x2 |
| UE speed | 3km/h |
| CSI-RS Bandwidth | 48 RB |
| CSI-RS configuration | 4 TRSs per TRP |
| CSI-RS periodicity | 40, 20 slots |
| SNR | -3dB, 0dB, 3dB |
| Frequency offset between TRPs | 100ppb |
| AFO | 200ppb |

Carrier frequency offset estimation error values in Hz are shown in Table 1.

Scenario 1: TRP1 and TRP2 transmitting only TRS

A comparison for the carrier frequency offset computation without averaging and with averaging of 3 and 5 samples measurements are shown in parts per billion (ppb) with respect to the carrier frequency of 3.5 GHz are shown in Table 2.

Table 2: Carrier frequency offset estimation error (in ppb) under the assumptions of Scenario 1.

| Test type | Quantization Levels | FO (ppm) | SCS | SNR | No averaging |  |  | 3 samples average |  |  | 5 samples average |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | P5 | P50 | P95 | P5 | P50 | P95 | P5 | P50 | P95 |
| Freq offset | 32 | 0.1 | 15 | -3 | 11.29 | 11.29 | 33.87 | 11.29 | 11.29 | 11.29 | 11.29 | 11.29 | 11.29 |
| Freq offset | 32 | 0.1 | 15 | 0 | 11.29 | 11.29 | 11.29 | 11.29 | 11.29 | 11.29 | 11.29 | 11.29 | 11.29 |
| Freq offset | 32 | 0.1 | 15 | 3 | 11.29 | 11.29 | 11.29 | 11.29 | 11.29 | 11.29 | 11.29 | 11.29 | 11.29 |
| Freq offset | 32 | 0.1 | 30 | -3 | 11.29 | 11.29 | 33.87 | 11.29 | 11.29 | 11.29 | 11.29 | 11.29 | 11.29 |
| Freq offset | 32 | 0.1 | 30 | 0 | 11.29 | 11.29 | 33.87 | 11.29 | 11.29 | 11.29 | 11.29 | 11.29 | 11.29 |
| Freq offset | 32 | 0.1 | 30 | 3 | 11.29 | 11.29 | 11.29 | 11.29 | 11.29 | 11.29 | 11.29 | 11.29 | 11.29 |
| Freq offset | 128 | 0.1 | 15 | -3 | 2.76 | 8.27 | 13.78 | 8.27 | 8.27 | 8.27 | 8.27 | 8.27 | 8.27 |
| Freq offset | 128 | 0.1 | 15 | 0 | 2.76 | 8.27 | 13.78 | 8.27 | 8.27 | 8.27 | 8.27 | 8.27 | 8.27 |
| Freq offset | 128 | 0.1 | 15 | 3 | 2.76 | 8.27 | 8.27 | 8.27 | 8.27 | 8.27 | 8.27 | 8.27 | 8.27 |
| Freq offset | 128 | 0.1 | 30 | -3 | -8.27 | 8.27 | 19.29 | 2.76 | 8.27 | 8.27 | 2.76 | 8.27 | 8.27 |
| Freq offset | 128 | 0.1 | 30 | 0 | -2.76 | 8.27 | 13.78 | 2.76 | 8.27 | 8.27 | 2.76 | 8.27 | 8.27 |
| Freq offset | 128 | 0.1 | 30 | 3 | 2.76 | 8.27 | 13.78 | 2.76 | 8.27 | 8.27 | 8.27 | 8.27 | 8.27 |
| Freq offset | 256 | 0.1 | 15 | -3 | -1.37 | 6.86 | 12.35 | 4.12 | 6.86 | 6.86 | 4.12 | 6.86 | 6.86 |
| Freq offset | 256 | 0.1 | 15 | 0 | 1.37 | 6.86 | 9.61 | 4.12 | 6.86 | 6.86 | 4.12 | 6.86 | 6.86 |
| Freq offset | 256 | 0.1 | 15 | 3 | 4.12 | 6.86 | 9.61 | 4.12 | 6.86 | 6.86 | 5.49 | 6.86 | 6.86 |
| Freq offset | 256 | 0.1 | 30 | -3 | -6.86 | 4.12 | 17.84 | 4.12 | 4.12 | 6.86 | 4.12 | 4.12 | 6.86 |
| Freq offset | 256 | 0.1 | 30 | 0 | -4.12 | 4.12 | 15.10 | 4.12 | 4.12 | 6.86 | 4.12 | 4.12 | 6.86 |
| Freq offset | 256 | 0.1 | 30 | 3 | -1.37 | 4.12 | 9.61 | 4.12 | 4.12 | 6.86 | 4.12 | 4.12 | 6.86 |

Table 3: Carrier frequency offset estimation error (in ppb) comparison for the 3 scenarios (without averaging).

| Test type | Quantization Levels | FO (ppm) | SCS | SNR | Scenario 1 |  |  | Scenario 2 |  |  | Scenario 3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | P5 | P50 | P95 | P5 | P50 | P95 | P5 | P50 | P95 |
| Freq offset | 256 | 0.1 | 15 | -3 | -1.37 | 6.86 | 12.35 | -32.94 | 6.86 | 42.55 | -34.31 | 6.86 | 42.55 |
| Freq offset | 256 | 0.1 | 15 | 0 | 1.37 | 6.86 | 9.61 | -20.59 | 6.86 | 31.57 | -20.59 | 6.86 | 31.57 |
| Freq offset | 256 | 0.1 | 15 | 3 | 4.12 | 6.86 | 9.61 | -12.35 | 6.86 | 23.33 | -12.35 | 6.86 | 23.33 |
| Freq offset | 256 | 0.1 | 30 | -3 | -6.86 | 4.12 | 17.84 | -67.25 | 4.12 | 75.49 | -67.25 | 6.86 | 75.49 |
| Freq offset | 256 | 0.1 | 30 | 0 | -4.12 | 4.12 | 15.10 | -39.80 | 4.12 | 50.78 | -45.29 | 6.86 | 53.53 |
| Freq offset | 256 | 0.1 | 30 | 3 | -1.37 | 4.12 | 9.61 | -27.45 | 6.86 | 37.06 | -28.82 | 4.12 | 39.80 |

Time offset (TO) requirements

Table 4. Simulation assumptions for the TO reporting simulations.

| Parameter | Value |
| --- | --- |
| Waveform | OFDM |
| Carrier Frequency | 3.5GHz |
| SCS | 15kHz, 30kHz |
| Channel Model | TDLC300-10 |
| TRP number | 2 |
| UE antenna configuration | 1x2 |
| UE speed | 3km/h |
| CSI-RS Bandwidth | 48 RB |
| CSI-RS configuration | 4 TRSs per TRP |
| CSI-RS periodicity | 40 slots for 30kHz; 20 slots for 15kHz |
| CSI-RS offset | Slot 10 and 11 for 15kHz , slot 20 and 21 for 30KHz |
| SNR | -3dB, 0dB, 3dB |
| Time offset between TRPs | CP/2 |
| AD | CP |

Time offset estimation error values as a percentage of the cyclic prefix (% of CP) are shown in Table 2.

Scenario 1: TRP1 and TRP2 transmitting only TRS

Table 5: Time offset estimation error (% of CP) under the assumptions of Scenario 1.

| Test type | Quantization Levels | TO (% CP) | SCS | SNR | No averaging |  |  | 3 samples average |  |  | 5 samples average |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | P5 | P50 | P95 | P5 | P50 | P95 | P5 | P50 | P95 |
| Time offset | 32 | 50 | 15 | -3 | 1.61 % | 1.61 % | 1.61 % | 1.61 % | 1.61 % | 1.61 % | 1.61 % | 1.61 % | 1.61 % |
| Time offset | 32 | 50 | 15 | 0 | 1.61 % | 1.61 % | 1.61 % | 1.61 % | 1.61 % | 1.61 % | 1.61 % | 1.61 % | 1.61 % |
| Time offset | 32 | 50 | 15 | 3 | 1.61 % | 1.61 % | 1.61 % | 1.61 % | 1.61 % | 1.61 % | 1.61 % | 1.61 % | 1.61 % |
| Time offset | 32 | 50 | 30 | -3 | 1.61 % | 1.61 % | 1.61 % | 1.61 % | 1.61 % | 1.61 % | 1.61 % | 1.61 % | 1.61 % |
| Time offset | 32 | 50 | 30 | 0 | 1.61 % | 1.61 % | 1.61 % | 1.61 % | 1.61 % | 1.61 % | 1.61 % | 1.61 % | 1.61 % |
| Time offset | 32 | 50 | 30 | 3 | 1.61 % | 1.61 % | 1.61 % | 1.61 % | 1.61 % | 1.61 % | 1.61 % | 1.61 % | 1.61 % |
| Time offset | 128 | 50 | 15 | -3 | -1.18 % | 0.39 % | 1.97 % | 0.39 % | 0.39 % | 0.39 % | 0.39 % | 0.39 % | 0.39 % |
| Time offset | 128 | 50 | 15 | 0 | 0.39 % | 0.39 % | 0.39 % | 0.39 % | 0.39 % | 0.39 % | 0.39 % | 0.39 % | 0.39 % |
| Time offset | 128 | 50 | 15 | 3 | 0.39 % | 0.39 % | 0.39 % | 0.39 % | 0.39 % | 0.39 % | 0.39 % | 0.39 % | 0.39 % |
| Time offset | 128 | 50 | 30 | -3 | -1.18 % | 0.39 % | 1.97 % | 0.39 % | 0.39 % | 0.39 % | 0.39 % | 0.39 % | 0.39 % |
| Time offset | 128 | 50 | 30 | 0 | 0.39 % | 0.39 % | 0.39 % | 0.39 % | 0.39 % | 0.39 % | 0.39 % | 0.39 % | 0.39 % |
| Time offset | 128 | 50 | 30 | 3 | 0.39 % | 0.39 % | 0.39 % | 0.39 % | 0.39 % | 0.39 % | 0.39 % | 0.39 % | 0.39 % |
| Time offset | 256 | 50 | 15 | -3 | -0.98 % | 0.20 % | 1.76 % | 0.20 % | 0.20 % | 0.20 % | 0.20 % | 0.20 % | 0.20 % |
| Time offset | 256 | 50 | 15 | 0 | 0.20 % | 0.20 % | 0.20 % | 0.20 % | 0.20 % | 0.20 % | 0.20 % | 0.20 % | 0.20 % |
| Time offset | 256 | 50 | 15 | 3 | 0.20 % | 0.20 % | 0.20 % | 0.20 % | 0.20 % | 0.20 % | 0.20 % | 0.20 % | 0.20 % |
| Time offset | 256 | 50 | 30 | -3 | -0.98 % | 0.20 % | 1.76 % | 0.20 % | 0.20 % | 0.20 % | 0.20 % | 0.20 % | 0.20 % |
| Time offset | 256 | 50 | 30 | 0 | 0.20 % | 0.20 % | 0.20 % | 0.20 % | 0.20 % | 0.20 % | 0.20 % | 0.20 % | 0.20 % |
| Time offset | 256 | 50 | 30 | 3 | 0.20 % | 0.20 % | 0.20 % | 0.20 % | 0.20 % | 0.20 % | 0.20 % | 0.20 % | 0.20 % |

Table 6: Time offset estimation error (% of CP) comparison for the 3 scenarios (without averaging).

| Test type | Quantization Levels | TO (% CP) | SCS | SNR | Scenario 1 |  |  | Scenario 2 |  |  | Scenario 3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | P5 | P50 | P95 | P5 | P50 | P95 | P5 | P50 | P95 |
| Time offset | 256 | 50 | 15 | -3 | -0.98 % | 0.20 % | 1.76 % | -0.98 % | 0.20 % | 2.94 % | -0.98 % | 0.20 % | 2.94 % |
| Time offset | 256 | 50 | 15 | 0 | 0.20 % | 0.20 % | 0.20 % | -0.98 % | 0.20 % | 2.94 % | -0.98 % | 0.20 % | 2.94 % |
| Time offset | 256 | 50 | 15 | 3 | 0.20 % | 0.20 % | 0.20 % | -0.98 % | 0.20 % | 1.76 % | -0.98 % | 0.20 % | 1.76 % |
| Time offset | 256 | 50 | 30 | -3 | -0.98 % | 0.20 % | 1.76 % | -0.98 % | 0.20 % | 2.94 % | -0.98 % | 0.20 % | 2.94 % |
| Time offset | 256 | 50 | 30 | 0 | 0.20 % | 0.20 % | 0.20 % | -0.98 % | 0.20 % | 2.94 % | -0.98 % | 0.20 % | 2.94 % |
| Time offset | 256 | 50 | 30 | 3 | 0.20 % | 0.20 % | 0.20 % | -0.98 % | 0.20 % | 1.76 % | -0.98 % | 0.20 % | 2.94 % |

Phase offset (PO) requirements

Table 7. Simulation assumptions for the PO reporting simulations.

| Parameter | Value |
| --- | --- |
| Waveform | OFDM |
| Carrier Frequency | 3.5GHz |
| SCS | 15kHz |
| Channel Model | TDLA30-10 |
| TRP number | 2 |
| UE antenna configuration | 1x2 |
| UE speed | 3km/h |
| CSI-RS Bandwidth | 48 RB |
| CSI-RS configuration | Single port CSI-RS (row 2) |
| CSI-RS periodicity | 40 slots |
| SNR | -3dB, 0dB, 3dB |
| Phase offset between TRPs | 180 degrees |

Phase offset estimation error values as degrees from 0 to 180 are shown in Table 3.

- Scenario 1: TRP1 and TRP2 transmitting only TRS

- Scenario 2: Data on all the OFDM symbols, except for the symbols carrying TRS.

- Scenario 3: Data on all the OFDM symbols; except for the resource elements carrying TRS.

Table 8: Phase offset estimation error 90th percentile in degrees from 0 to 180.

| Estimation and quantization error in degrees from 0 to 180 |  |  | Scenario 1 |  |  | Scenario 2 |  |  | Scenario 3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Quant. Levels | SCS | SNR | No averaging | 3 samples | 5 samples | No averaging | 3 samples | 5 samples | No averaging | 3 samples | 5 samples |
| 32 | 15 | -3 | 0 | 0 | 0 | 67.5 | 45 | 33.75 | 67.5 | 50.625 | 33.75 |
| 32 | 15 | 0 | 0 | 0 | 0 | 33.75 | 33.75 | 22.5 | 33.75 | 33.75 | 22.75 |
| 32 | 15 | 3 | 0 | 0 | 0 | 22.5 | 22.5 | 11.25 | 22.5 | 22.75 | 11.25 |

- References

- R4-2502596, WF on RRM requirements for NR_MIMO_Ph5_Part2, MediaTek

- R4-2504501, Other RRM requirements for MIMO ph5, Nokia