3GPP TSG-RAN WG4 Meeting #114bis	R4-2504508

Wuhan, China, April 7th – April 11th, 2025

Source:	Nokia

Title:	Discussion on XR ph3 RRM requirements

Agenda item:	7.28.2

Document for:	Discussion

Introduction

In this paper we present Nokia’s views on open issues after RAN4 #114 on RRM requirements for XR enhancements [1]. The main contributions of this paper are listed in section 2.1. Section 2.2 depicts items regarding scenarios for measurement skipping. Section 2.3 deals with impact to L1 measurement requirements. Supported types of measurement gaps are discussed in section 2.4, whilst the impact due to measurement skipping for XR is analysed in section 2.5, the conditions for measurement skipping in section 2.6 and the time offset for measurement skipping in section 2.7. UAI is discussed in section 2.8, semi-static schemes in section 2.9 and the skipping indication for multiple CCs in section 2.10. Corresponding observations and proposals are made.

Discussion

Discussion Highlights

Scenarios for measurement skipping

In the last RAN4 meeting the following was agreed regarding deployment scenarios [2]:

## Issue 1-1: Deployment scenarios < Agreement>: - Deprioritize FR2-2 for enabling transmission/reception in gaps/restrictions that are caused by RRM measurements <Way forward>: - Further discussion needed for - NR-DC

The discussion during RAN4 #114 on NR-DC support was based on various contributions summarized in [3].

- Issue 1-1: Deployment scenarios - Background information: In RAN4 #113 the following agreement regarding deployment scenarios was reached R4-2420103: < Agreement>: - EN-DC and NE-DC for DCI-based solution - Is NOT supported for measurement gap skipping by the WID <Way forward>: - Further discussion needed for - FR2-2 - NR-DC. - Proposals related to dual connectivity - Options that don’t allow NR-DC - Option 1 (Huawei, Mediatek): Deprioritize NR-DC for enabling transmission/reception in gaps/restrictions that are caused by RRM measurements - Option 2 (ZTE): Not support for DCI-based measurement gap skipping for NR-DC. - Options supporting NR-DC - Option 3 (Nokia): RAN4 to consider for XR enhancements only NR-DC scenario with MCG and SCG in different FRs using per-FR gaps. - Option 4 (Interdigital): If NR-DC gap skipping is to be supported, semi-static indication/UE assistance combined with DCI dynamic indications solutions are required. - Ask other working groups - Option 3 (vivo): Send an LS to RAN2 and RAN1 to ask if NR-DC scenario is supported for gap cancellation from their perspective. - Recommended WF - Proposals on NR-DC - Please discuss the proposals

We observe from the discussion during RAN4 #114 that the significance of the NR-DC scenario support is limited. No operator support is mentioned. Views of companies diverge and furthermore RAN1 has not discussed this scenario. In addition, dual connectivity scenarios that may have similar or even higher commercial relevance such as EN-DC or NE-DC were already excluded. In our view, measurement gap skipping in all possible NR-DC scenarios will severely impact RRM requirements as pointed out in our RAN4 #114 contribution [4], Thus, we propose to not consider measurement gap skipping for NR-DC scenario in the Rel-19 WI.

NR-DC is not supported for measurement gap skipping for XR enhancements.

Impact to L1 requirements

In the last RAN4 meeting, the following was agreed regarding L1, LTM and L3 without gaps [2]:

## Issue 1-3: L3 Measurements without gaps with scheduling restrictions ## Issue 1-4: L1 measurements with scheduling restrictions and procedures ## Issue 1-5: L1/L2 triggered mobility measurements and procedures <Agreement> - During the skipped measurement gap occasion, the UE is not required to perform measurements (i.e. L3 measurements and L1 measurements), and the UE shall not cause scheduling restrictions. - Outside a gap occasion, skipping of L3 Measurements without gaps with scheduling restrictions is not supported. - Outside a gap occasion, skipping of L1 Measurements with scheduling restrictions is not supported.

L1 measurements are based on measurement samples for a particular SSB index of a SSB burst. In case SSB periodicity is equal to SMTC periodicity and equal to MGRP, there is a full overlap of L1 measurements with the MG. For instance, if SSB periodicity = 20ms, SMTC = 20ms and MGRP = 20ms, all L1 measurements fully overlap with the MG. In existing RAN4 requirements there are no requirements for this scenario in FR1. One example is the definition of the L1/L3 sharing factor P in L1-RSRP requirements, with is defined when some but not all occasions are overlapping with SSB.

Existing L1/L3 sharing factor P is not defined for the case where gaps are overlapping with all SSB occasions.

Therefore, we consider that the scenario where all SSBs for L1 measurements overlap with gaps is not typical for XR enhancements and hence no RRM requirements should be defined for it.

RAN4 not to define requirements for measurement gap skipping for the case of measurement gap fully overlapped with L1 measurements.

Types of measurement gaps

The following issue has been open now for several meetings regarding types of measurement gaps [2]:

## Issue 2-1: Types of measurement gaps <Way forward>: - In this issue we discuss the types of measurement gaps to be considered in this WID. In order to help the discussion, we reuse the definition of Type-1 and Type-2 measurement gaps as agreed in Rel-18 NR_MG-enh2: - Type-1 MG: Gap(s) configured via GapConfig without suffix - Type-2 MG: Gap(s) configured via GapConfig-r17 without preConfigInd-r17 or ncsgInd-r17 - Rel-18 Concurrent Gap: gap with assigned priority which can be also NCSG or Pre-MG. - Companies may bring proposals regarding the priorities of each gap type for RAN4 to work based on the following table Gap types; First priority; Second priority; Don’t include. Type 1. Type 2. NCSG. MUSIM. Positioning. Rel 18 concurrent gap. PreConfigured.

The discussion during RAN4 #114 on support of measurement gap types was based on various contributions summarized in [3].

- Company proposals on the priorities for RAN4 to work are summarized using based on the following table Gap types; First priority; Second priority; Shall not be cancelled. Type 1; Moderator: Type 1 is omitted since it is already agreed. Type 2; Apple; Ericsson. NCSG; Apple; Ericsson. MUSIM; Apple; Ericsson. Positioning; Ericsson, Apple. Rel 18 concurrent gap; Apple. PreConfigured; Apple; Ericsson.

We observe from the discussion during RAN4 #114 that companies have different views here. As baseline Type 1 MG was already agreed with first priority. This proceeding should be maintained in RAN4. Though, investigation in selected Rel-17 MG types should be done as well with second priority.

NCSG may be in use for a CA capable UE receiver with idle receive chain and thus significantly reduce the interruption time compared to MG (e.g. from MGL=6ms to 2*VIL=2ms) and therefore is a good candidate for use in combination with XR services. Therefore, we consider that single NCSG (Rel-17) can be added as second priority for defining RRM requirements for measurement gap skipping.

RAN4 to consider the following gap types for measurement gap skipping:

| Gap types | First priority | Second priority | Don’t include |
| --- | --- | --- | --- |
| Type 1 | Yes |  |  |
| Type 2 |  |  | Don’t include |
| NCSG |  | Yes, Rel-17 NCSG |  |
| MUSIM |  |  | Don’t include |
| Positioning |  |  | Don’t include |
| Rel 18 concurrent gap |  |  | Don’t include |
| PreConfigured |  |  | Don’t include |

XR skipping requirements impact

In the last RAN4 meeting the following was agreed regarding how to extend measurement delay due to measurement skipping [2]:

## Issue 3-1: Principles for measurement delay calculation <Agreement>: - Follow NR-U-like approach for designing the formulas for measurement delay extension. ## Issue 3-2: Formulas for measurement delay extension <Way foward>: The purpose of the options provided below is for guidance and not to preclude any other options. - Option 1: RAN4 to define measurement delay extension due to measurement skipping for inter-frequency measurements with gap as - Max(200 ms × CSSFinter, Ceil( 8 * Kgap) × Max(MGRP , SMTC period) × CSSFinter + Nskip × Max(MGRP , SMTC period)), - where Nskip is the number of skipped MG occasions in the measurement period. - Use this formula as example and follow similar approach for the other cases. - Option 2: RAN4 to define measurement delay extension due to measurement skipping for inter-frequency measurements with gap as - Max(200 ms, Ceil(( 8 + Nskip ) * Kgap) ´ Max(MGRP , SMTC period)) ´ CSSFinter - where Nskip is the number of skipped MG occasions in the measurement period. - Use this formula as example and follow similar approach for the other cases.

In our view, both options are appropriate during XR operation, but it will be clearer if we start discussing the formulas for measurement delay calculation.

The Rel-18 requirements of measurement period with measurement gaps in FR1 are referred as one example for the analysis of the measurement delay extension. The requirements without DRX on Table 9.3.5-1 are represented by the formula:

- Max(200 ms, Ceil(8 * Kgap) ´ Max(MGRP , SMTC period)) ´ CSSFinter

Where

- 200 ms is the lower bound for the measurement delay calculation

- 8 is the maximum number of samples for inter-frequency measurements

- MGRP is the gap periodicity

- SMTC period is the period for the SMTC window

- CSSF is the carrier specific scaling factor.

- Kgap is used to take into account the overlap of concurrent gaps

Considering a simple example of a single carrier measurement (CSSF=1), without concurrent gaps (Kgap=1), and MGRP=SMTC period, we can simplify the formula above as

- Max(200 ms, 8 ´ MGRP)

So if we skip N samples on a measurement cycle, we could extend the formula as:

- Max(200 ms, (8 + N) ´ MGRP)

That would still keep the 200 ms as lower bound, and in some cases the measurement delay is not extended. If we follow that logic, we can extend the measurement delay formula as done in NR-U requirements:

- Max(200 ms, Ceil((8 + Nskip) * Kgap) ´ Max(MGRP , SMTC period)) ´ CSSFinter

One problem of this simple extension arises if we consider multiple frequency layers with CSSFinter > 1. In that case, every skipped sample would result in an extension of CSSFinter ´ Max(MGRP,SMTC) according to the round robin approach. Therefore, we propose one alternative solution that allows for extending the measurement delay just by a single SMTC/MRGP

- Max(200 ms ´ CSSFinter, Ceil(8 * Kgap) ´ Max(MGRP , SMTC period) ´ CSSFinter + Nskip ´ Max(MGRP , SMTC period))

Instead of the round robin approach for applying the CSSF between different frequency layers, this stops the round robin at the skipped MG occasion and continues it at the next available MG occasion. Thus, the overall delay can be remarkably reduced.

Table 1 Comparison of measurement delay calculation for Option 1 and Option 2 in Issue 3-2

|  |  |  | Total delay (ms) |  |  | Delay extension (ms) |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| SMTC=MRGP | CSSF | Nskip | Legacy (no skipping) | Option 1 | Option 2 | Option 1 | Option 2 |
| 40 | 1 | 1 | 320 | 360 | 360 | 40 | 40 |
| 40 | 1 | 3 | 320 | 440 | 440 | 120 | 120 |
| 40 | 1 | 5 | 320 | 520 | 520 | 200 | 200 |
| 40 | 3 | 1 | 960 | 1000 | 1080 | 40 | 120 |
| 40 | 3 | 3 | 960 | 1080 | 1320 | 120 | 360 |
| 40 | 3 | 5 | 960 | 1160 | 1560 | 200 | 600 |
| 40 | 5 | 1 | 1600 | 1640 | 1800 | 40 | 200 |
| 40 | 5 | 3 | 1600 | 1720 | 2200 | 120 | 600 |
| 40 | 5 | 5 | 1600 | 1800 | 2600 | 200 | 1000 |

Table 1 shows the comparison between the formulas in Option 1 and Option 2 in Issue 3-2. Option 1 is in line with the proposed formula in this document, while option 2 is the plain adaptation of the NR-U formula. As it can be observed in the table, the formulas result in the same measurement delay when CSSF=1, however there is a significant difference int eh measurement delay extension for CSSF=3 and CSSF=5.  When CSSF =3 and Nskip=5, the measurement delay extension is 200 ms for Option 1, whereas it is 600 ms for option 2. When CSSF=5 and Nskip=5 the measurement delay extension is 200 ms for Option 1, whereas it Is 1000 ms for Option 2.

The measurement delay of option 2 is 44,4 % larger than the delay for Option 1 when CSSF=5 and Nskip=5.

RAN4 to define measurement delay extension due to measurement skipping for inter-frequency measurements with gap as

Max(200 ms ´ CSSFinter, Ceil(8  * Kgap) ´ Max(MGRP , SMTC period) ´ CSSFinter + Nskip ´ Max(MGRP , SMTC period)), where Nskip is the number of  skipped MG occasions in the measurement period.

  - Use this formula as example and follow similar approach for the other cases.

Additionally, we also have to consider the case where multiple frequency layers are measured. When multiple layers are configured, the current RAN4 assumption is that the UE is using a single searcher for measurements with gaps, and a single frequency layer is measured at every given measurement occasion. In some network deployments, the importance of certain frequencies is not equal in relation to others. Therefore, it is desirable that the measurement skipping is not affecting equally the measurement delay for all the frequency layers. This can be achieved by the UE prioritizing higher priority frequency layers to be measured first in case a measurement gap is skipped, while low priority gaps may be measured on remaining gap occasions and hence experience longer measurement delays.

RAN4 to define RRM requirements such that the network can configure which frequency layers are measured with higher priority.

One example is if one frequency layer is used for mobility, while another is more often used for additional carriers in carrier aggregation. Figure 1 shows one example where one carrier is assigned a high priority. In this example the skipping of few SMTCs is compensated by measuring more often for the high priority carrier, while the low priority carrier has its delay extended.

Figure 1 – Example assigning priority to measure one frequency layer, where measurement delay is not extended to that layer, while it is extended to the low priority layer

When that approach is used, we can consider the high priority carriers to have unchanged measurement delay as long as the number of skipped measurement occasions in the measurement cycle is Nskip < 8 ´ (CSSF - Nhigh) for inter-frequency measurements and Nskip < 5 ´ (CSSF - Nhigh) for intra-frequency measurements.

Examples if Nskip and the resulting measurement skipping ratio are shown on Table 1 for different values of CSSF and high priority layers. In some examples with small number of Nhigh the maximum skipping ratio that can be achieved without impact to high priority layers can get between 50% to 80%.

- It is possible to increase skipping ratio to up to 50% for 2 layers and 80% for 5 layers without impacting the measurement delay of 1 frequency layer.

High priority layers do not have measurement delay extended if number of skipping occasions in a measurement cycle is

  - Nskip ≤8 ´ (CSSF - Nhigh) for inter-frequency measurements and Nskip ≤ 5 ´ (CSSF - Nhigh) for intra-frequency measurements

  - where Nhigh is the number of high priority layers.

Table 2 Calculation of maximum skipping ratio without high priority delay extension

| Number of inter-freq samples (per frequency layer) | CSSF | Nhigh | Maximum Nskip | Maximum skipping ratio % |
| --- | --- | --- | --- | --- |
| 8 | 2 | 1 | 8 | 50 |
| 8 | 3 | 1 | 16 | 66,7 |
| 8 | 3 | 2 | 8 | 33,3 |
| 8 | 4 | 1 | 24 | 75 |
| 8 | 4 | 2 | 16 | 50 |
| 8 | 4 | 3 | 8 | 25 |
| 8 | 5 | 1 | 32 | 80 |
| 8 | 5 | 2 | 24 | 60 |
| 8 | 5 | 3 | 16 | 40 |
| 8 | 5 | 4 | 8 | 20 |

Conditions for measurement skipping

In the last RAN4 meeting the following was agreed regarding how to extend measurement delay due to measurement skipping [2]:

## Issue 3-3: Conditions for measurement skipping <Way forward>: - Option 1: conditions are needed - Option 1a: For measurement gap skipping, the max number of gaps skipped during the measurement period should be limited/specified. - Option 1b: The information related to the maximum number of measurement gap occasions allowed for skipping/reallocated to XR over a time period (mentioned in RAN1 LS on UAI) or the maximum time between any consecutive samples can be defined in TS 38.133 - Option 1c: Maximum acceptable time separation between two measurement occasions available for RRM measurements - Option 2: No conditions are needed for measurement skipping regarding maximum number of occasions or MG configuration.

In this issue few companies proposed that a limitation condition where measurement skipping may be used. These included maximum number of occasions to be skipped or measurement gap configurations.

Regarding Option 1 of issue 3-3, we understand that some companies have concerns on potential impacts if measurement skipping exceeds a certain limit. However, it is not very clear how to define a limit to measurement gap skipping that is general enough to account for all possible scenarios.

In our companion paper we provided analysis of the need for measurement skipping and mobility impacts [7]. The results in our paper show that it can be beneficial in some situations to increase the measurement gap skipping to about 60%. Additionally, at least in one scenario we could observe that 75% skipping could be used without significant performance impact.

- Our simulations results show benefit of using about 60% skipping ratio.

In at least some scenarios skipping ratio can be increased to about 75% without significant mobility degradation.

In other scenarios the UE will be performing mobility mostly on intra-frequency without gaps. In that case, if gaps are used for inter-frequency there should be no mobility performance impact of skipping inter frequency measurements.

Therefore, we think that it is natural that in the network deployment there is a limitation on the maximum number of skipping. However, this is an optimization that can be determined case by case, for example considering the typical mobility of XR UEs in a given location, the ISD, or layers to monitor. If we define a limit in the specification, we will simply limit available options on the deployment and the benefit of the feature will be impacted.

When considering Option 3, it is not clear what is the relationship between the measurement gap length and the impact to measurement skipping. We understand that the measurement skipping will have a direct impact on measurement delay requirements, however those requirements do not depend on the measurement gap length.

- Limiting the maximum number of occasions that can be skipped in the specification will have an impact on the network deployment flexibility.

No conditions are needed for measurement skipping regarding maximum number of skipped occasions per measurement period or related to MG configuration.

Time offset for measurement skipping processing

In the last RAN4 meeting the following was left open regarding time offset for measurement gap skipping [2]:

## Issue 4-1: Time offset requirement for measurement gap skipping <Way forward>: - Option 1: 3 ms optional capability only apply when UE is only configured with NR measurement. - Option 2: No need to define additional condition for the optional capability of 3 ms ## Issue 4-2: Time offset requirement for cancelling skip indication <Way forward>: - Option 1: The support of Option 2 as agreed by RAN1 is conditioned upon the arrival of such DCI at least X ms before the MG occasion, where - X=5ms as baseline and X=3ms as optional UE capability. - [RAN1 agreement] Option 2: For explicit indication by DCI, bit value equal to “0” means UE behaviour for the corresponding gap/restriction occasion is as per legacy behaviour.

Regarding the FFS point of the optional capability of 3 ms, there were a company that wanted to check whether conditions for the 3 ms to apply are needed. In our view we already agreed that the UE is required to measure SRVCC is not supported by RAN4. That was done in order to easy the implementation, in case of 2G-3G measurements, and should suffice in guaranteeing the 3 ms condition.

No condition is needed for the optional capability of 3 ms.

As for the time offset for cancelling indication, we think that the same time offset can be applied to both the skipping and overwriting the skip indication. Therefore, we agree with the proposal originally brought by Qualcomm in [2]:

The support of Option 2 as agreed by RAN1 is conditioned upon the arrival of such DCI at least X ms before the MG occasion, where

  - X=5ms as baseline and X=3ms as optional UE capability.

Need/feasibility of UE assistance information

In the last RAN4 meeting the following was agreed regarding UE assistance information for measurement skipping [2]:

## Issue 5-1: General on UAI <Agreement>: - Agree to define UAI based on the following assumptions, and further discuss the content of UAI - RAN4 will not specify any explicit or implicit requirement or expectation on gNB behaviour in response to any XR UAI (from RAN4 perspective) - Not define UE behaviour and the corresponding requirements based on UAI - Supporting of UAI is optional for UE with optional UE capability. <Way forward>: - Proposals related to measurement occasions - Option 1: Do not define UAI related to measurement occasions. - Option 2: The UE indicates a pattern of unused gaps - Option 3: The UE indicates a ratio/number of occasions that can be skipped without measurement delay extension - Option 4: The UE indicates NW a status whether it can tolerate measurement cancellation. - Option 5: One of options 2, 3, or 4 but the information is reported as UE capability. - Proposals related to channel conditions - Option 1: Provide information on channel conditions - Option 2: do not provide information on channel conditions

As included in our previous discussion papers, we believe it is very hard for the UE to provide accurate and reliable information that can help measurement skipping. More details can be found at [3]. But since many companies have very strong desire to define this type of UE reporting, we attempt to cover missing details that are not covered in the open options.

Regarding “Option 2: UE Indicated a pattern of unused gaps.”, we think this proposal is not in line with the objectives of the WID. This option puts implicit restrictions on which gaps can be skipped, but the pattern of skipped gaps depends on the XR application. So placing those restrictions will limit the XR user experience. Additionally, once the UE skips a measurement gap, it should reschedule its measurements, so unused gaps should be used for avoiding measurement delay extension.

Considering the observation regarding unused gap pattern, we believe that the most benefit can be achieved with the UE indicating the ratio of gaps that can be skipped without measurement delay as in Option 3. As some companies have pointed out, the number of measurement samples the UE needs for a measurement depends on the experienced SNR

- The number of samples needed to complete a measurement depends on the UE experience SNR.

The experienced SNR is different for each frequency layer.

Considering the observation above, if a single report is given covering multiple layers the tendency will be that the reported value takes the worst case among the frequency layers. Additionally, if the network has the information of how much skipping each layer can handle, the network can also reconfigure the measurement objects to minimize measurement skipping impact. Furthermore, if concurrent gaps are agreed, the network cannot know how much can be skipped for each of the concurrent gaps. Therefore, if UAI is to be defined, it would be more useful if the report is done per measurement object.

If single UAI report is done, the network cannot distinguish the desired skipping ratio for each of the configured measurement gaps.

Define UAI reporting where the UE indicates a ratio of occasions that can be skipped without measurement delay extension per measurement object.

Semi-static schemes

The following issue is open for discussion regarding semi-static skipping schemes [2]:

## Issue 6-1: Semi-static schemes <Way forward>: - Further discussion based on recommended options - Option 1: Do not consider semi-static solutions - Option 2: RAN4 to further evaluate semi-static skipping indication in addition to DCI-based solution. - Option 2a: RRC pattern-based - Option 2b: MAC CE bitmap for skipping indication - Option 2c: RRC configuration patterns with MAC CE activation - Option 3: Network indicates by RRC which gaps can be skipped via DCI.

During RAN1#120, the following was agreed:

Confirm the Working assumption in RAN1#118 for supporting Alt. 1-1: Working Assumption For solutions based on triggering/enabling by network signaling to enable Tx/Rx in gaps/restrictions that are caused by RRM measurements select the following option: - Alt. 1: Dynamic indication to enable Tx/Rx in particular gap/restriction that are caused by RRM measurements. - Alt 1-1: Explicit indication by DCI to skip a particular gap/restriction; - Indication is included as part of scheduling DCI: - Bit-field size is one bit; - The bit in the DCI is used to indicate whether to skip the first gap/restriction occasion after a minimum time offset required between the last symbol of the PDCCH carrying the DCI format and the start of corresponding skipped gap/restriction occasion indicated by the DCI..

That means, RAN1 already confirmed the working assumption regarding Alt. 1-1, and there is no need for specifying duplicated solutions for XR.

RAN1 already confirmed working assumption regarding Alt 1-1.

In our view, measurement gap skipping should be fully under network control. The network has all information available on mobility performance of the UE as well as on the scheduled data traffic on DL and UL. Hence Alt 1-1 selected by RAN1 based on DCI scheduling is appropriate for this.

- The network has information on mobility performance of the UE as well as on the scheduled data traffic on DL and UL.

Additionally, many companies had concerns regarding Alt 1-1 because of the potential short time for reaction between DCI and measurement gap to be skipped. We already addressed in the last RAN4 meeting when we agreed with a time offset for measurement skipping of 5 ms as baseline.

- RAN4 already agreed on 5 ms time offset for measurement skipping as a baseline.

Overall, the performance of semi-static schemes for XR is judged inferior versus the DCI based scheme, as shown in our simulation paper  in RAN4 #113 R4-2418890.As shown in Figure 4 and 5 of R4-2418890, the capacity gain of the semi static scheme (Alt-3) in relation to measurements with gaps is very minor in comparison to the dynamic (DCI-based) scheme. This is also reasoned by the fact that Alt-3 is highly depending on prediction (of configured gaps subject to be skipped) and hence generates useless skipping, whilst XR traffic is highly dynamic, has jitter and hence cannot be matched with a predicted traffic pattern.

- Our simulations showed for the same skipping ratio only 25% capacity gain for semi-static scheme (Alt 3), whereas 50% to 75% capacity gain was achieved for dynamic schemes (Alt 1-1)

Another problem related to Alt-3 is that it relies on good traffic pattern prediction before the network has enough information to configure the skipping pattern. Figure 4 shows one example of what happens during the call start of a XR traffic stream. If Alt 3 approach is used as in Figure 4 a), the network needs a training phase in order to determine the XR traffic pattern. Once the prediction is ready at time t1, the network can determine the skipping pattern to be used that matches the XR traffic, and send an RRC command configuring the UE to use that pattern at t2. Therefore, before t2, the UE would experience worst performance related to the XR traffic. On the other hand, Figure 4 b) shows the example of using dynamic approach, as decided by RAN1. When using this approach, the traffic prediction training is not a mandatory step, and the network can start skipping measurement gaps from the start of the XR traffic in t0.

- Semi-static schemes are dependent on accurate XR traffic prediction, which may depend on a training stage at XR traffic start, and delay the start of using skipping for XR traffic.

a)

b)

Figure 4 Comparison of XR traffic quality variation over time when using a) semi-static approach (Alt 3) and b) dynamic approach as the decided by RAN1 working assumption (Alt 1-1)

Next, when using semi-static schemes, the network cannot quickly react to changes in channel condition. Consider the example where a UE is configured with a semi-static pattern with 50% skipping and approaches the cell edge. In that situation, once the network notices that the UE is moving toward cell edge, it would need to send RRC reconfiguration to change the pattern. Only after that the UE could perform measurements with short delay. In that case the Alt 1-1 can in a very dynamic way adapt the skipping ratio and prioritize mobility measurements.

- Semi-static pattern cannot adapt quickly to channel conditions (e.g. decrease skipping ratio as response to reaching cell edge).

Furthermore, RAN1 has already made a down-selection between DCI and RRC schemes

- RAN1 already made down selection between DCI and RRC schemes

Follow RAN1 agreement and consider DCI scheme only.

Skipping indication on frequency domain

The following issue is open for discussion regarding skipping indication on frequency domain [2]:

## Issue 6-2: Skipping indication on frequency domain <Way forward>: - Proposals - Proposal 1: For the gap/restriction skipping impact in frequency domain, - For the gap skipping, depend on whether the UE capable of per-FR gap - If capable of per-FR gap, the skipping is only valid for the gap instance in the same FR with scheduled XR transmission. - Otherwise, no need to distinguish different FR. - For the scheduling restriction skipping, reuse the principle defined in existing scheduling restriction to decide whether the intra-band/inter-band/same FR MO(s) would be skipped. - Proposal 2: - RAN4 to investigate scheduling restrictions for CA if DCI skipping indication is sent on one of the CCs. - RAN4 to discuss and clarify whether UE can conduct reception/transmission also on other cells that fall within the applicable range of carriers during the skipped gap/restriction, in addition to the cell to which the skipping was indicated. - Proposal 3 (Huawei): Depending on whether UE is configured with per-FR gap, if UE is configured with per-FR gap, the MG skipping only applies to the same FR and has no impacts on UE behavior on the other FR. If UE is configured with per-UE gap, the MG skipping applies to both FR. - Proposal 4 (vivo): RAN4 agrees to introduce RRC signaling of which gap or both gaps when per-FR gap is configured are to be skipped via further DCI indication.

RAN1 made following conclusion in RAN1#120 regarding skipping on frequency domain.

Conclusion It is RAN1 understanding that if measurement gap/restriction is indicated to be skipped by explicit indication by DCI, the TX/RX is enabled on all the serving cells in the applicable range of the skipped measurement gap/restriction. - Note: Applicable range of measurement gap/restriction is based on RAN4 specifications

Considering the RAN1 agreement, it is important that RAN4 captures that agreement, and no further discussion is needed on Issue 6-2.

Capture in RAN4 specifications following RAN1 agreement:

  - It is RAN1 understanding that if measurement gap/restriction is indicated to be skipped by explicit indication by DCI, the TX/RX is enabled on all the serving cells in the applicable range of the skipped measurement gap/restriction.

  - Note: Applicable range of measurement gap/restriction is based on RAN4 specifications.

Conclusion

The paper has presented Nokia’s views on RRM requirements for measurement skipping for XR. As part of this discussion, the following Observations and Proposals were made:

Proposal 1: NR-DC is not supported for measurement gap skipping for XR enhancements.

Observation 1: Existing L1/L3 sharing factor P is not defined for the case where gaps are overlapping with all SSB occasions.

Proposal 2: RAN4 not to define requirements for measurement gap skipping for the case of measurement gap fully overlapped with L1 measurements.

Proposal 3: RAN4 to consider the following gap types for measurement gap skipping:

| Gap types | First priority | Second priority | Don’t include |
| --- | --- | --- | --- |
| Type 1 | Yes |  |  |
| Type 2 |  |  | Don’t include |
| NCSG |  | Yes, Rel-17 NCSG |  |
| MUSIM |  |  | Don’t include |
| Positioning |  |  | Don’t include |
| Rel 18 concurrent gap |  |  | Don’t include |
| PreConfigured |  |  | Don’t include |

Observation 2: The measurement delay of option 2 is 44,4 % larger than the delay for Option 1 when CSSF=5 and Nskip=5.

Proposal 4: RAN4 to define measurement delay extension due to measurement skipping for inter-frequency measurements with gap as

a.	Max(200 ms ´ CSSFinter, Ceil( 8  * Kgap) ´ Max(MGRP , SMTC period) ´ CSSFinter + Nskip ´ Max(MGRP , SMTC period)), where Nskip is the number of  skipped MG occasions in the measurement period.

b.	Use this formula as example and follow similar approach for the other cases.

Proposal 5: RAN4 to define RRM requirements such that the network can configure which frequency layers are measured with higher priority.

Observation 1: It is possible to increase skipping ratio to up to 50% for 2 layers and 80% for 5 layers without impacting the measurement delay of 1 frequency layer.

Proposal 6: High priority layers do not have measurement delay extended if number of skipping occasions in a measurement cycle is

a.	Nskip ≤8 ´ (CSSF - Nhigh) for inter-frequency measurements and Nskip ≤ 5 ´ (CSSF - Nhigh) for intra-frequency measurements

b.	where Nhigh is the number of high priority layers.

Observation 2: Our simulations results show benefit of using about 60% skipping ratio.

Observation 3: In at least some scenarios skipping ratio can be increased to about 75% without significant mobility degradation.

Observation 4: Limiting the maximum number of occasions that can be skipped in the specification will have an impact on the network deployment flexibility.

Proposal 7: No conditions are needed for measurement skipping regarding maximum number of skipped occasions per measurement period or related to MG configuration.

Proposal 8: No condition is needed for the optional capability of 3 ms.

Proposal 9: The support of Option 2 as agreed by RAN1 is conditioned upon the arrival of such DCI at least X ms before the MG occasion, where

a.	X=5ms as baseline and X=3ms as optional UE capability.

Observation 5: The number of samples needed to complete a measurement depends on the UE experience SNR.

Observation 6: The experienced SNR is different for each frequency layer.

Observation 7: If single UAI report is done, the network cannot distinguish the desired skipping ratio for each of the configured measurement gaps.

Proposal 10: Define UAI reporting where the UE indicates a ratio of occasions that can be skipped without measurement delay extension per measurement object.

Observation 8: RAN1 already confirmed working assumption regarding Alt 1-1.

Observation 9: The network has information on mobility performance of the UE as well as on the scheduled data traffic on DL and UL.

Observation 10: RAN4 already agreed on 5 ms time offset for measurement skipping as a baseline.

Observation 11: Our simulations showed for the same skipping ratio only 25% capacity gain for semi-static scheme (Alt 3), whereas 50% to 75% capacity gain was achieved for dynamic schemes (Alt 1-1)

Observation 12: Semi-static schemes are dependent on accurate XR traffic prediction, which may depend on a training stage at XR traffic start, and delay the start of using skipping for XR traffic.

Observation 13: Semi-static pattern cannot adapt quickly to channel conditions (e.g. decrease skipping ratio as response to reaching cell edge).

Observation 14: RAN1 already made down selection between DCI and RRC schemes

Proposal 11: Follow RAN1 agreement and consider DCI scheme only.

Proposal 12: Capture in RAN4 specifications following RAN1 agreement:

a.	It is RAN1 understanding that if measurement gap/restriction is indicated to be skipped by explicit indication by DCI, the TX/RX is enabled on all the serving cells in the applicable range of the skipped measurement gap/restriction.

b.	Note: Applicable range of measurement gap/restriction is based on RAN4 specifications

- References

- RP-250107, Revised WID: XR (eXtended Reality) for NR Phase 3, Nokia, Qualcomm

- R4-2502700, WF on RRM requirements for NR_XR_Ph3, Nokia

- R4-2500542, Topic summary for [113][228] NR_XR_Ph3, Moderator (Nokia)

- R4-2502241, General aspects for XR enhancements, Nokia

- R4-2502229, RRM requirements for XR enhancements, Qualcomm Incorporated

- R4-2502150, Discussion on XR ph3 RRM requirements, Nokia

- R4-2504505, Analysis of skip ratio on XR capacity and mobility performance, Nokia