3GPP TSG-RAN WG4 Meeting # 114-bis	R4-2504512

# Wuhan, China, 7th – 11th February, 2025

Source:	LG Electronics

Title:	Discussion on Power domain enhancements for single carrier

Agenda item:	7.1.1.2.1

Document for:	Discussion

Introduction

In 3GPP RAN4 #114, WF[1] was approved. In this paper, we provide our views on Power domain enhancements for single carrier.

Sub-topic 1-1: Applicable requirements - Issue 1-1-1: Boundary to apply SE - Options - Option 1: The SE limits apply for the frequency ranges that are more than UE CBW+5MHz from the edge of the extended UE CBW - Option 2: Boundary between NR out of band and general spurious emission domain on power domain enhancements for single carrier context is defined as Extended UE CBW + 5 MHz - WF - option 1, to meet the ITU recommendation - Issue 1-1-2: Integral region basis for OOBE - Options - Option 1: The OOBE requirements are based on UE CBW - Option 2: The OOBE requirements are based on extended UE CBW OP 1. OP 2. - WF - option 1 - Issue 1-1-3: A-MPR handling - Proposals - Option 1: - If A-MPR is specified for a given RB allocation X, then the UE performs max(MPRenh(X), A-MPR(X)) - If A-MPR is not specified for a given RB allocation, then the UE performs max(MPR(X), A-MPR(X)) - Option 2: - For a given additional emissions requirement and corresponding A-MPR table, if it can be confirmed that A-MPR is specified for all RB allocations X for which A-MPR(X)≥minX∈SMPR(X) where S is the set of all possible RB allocations for the given carrier, then the UE performs max(MPRenh(X), A-MPR(X)) - Option 3: Only apply the power domain enhancement scheme to channels which are not subject to A-MPR. In many cases A-MPR is defined for the channel located closest to the additional emission requirement while channels with certain guard gap default to MPR. Those channels could benefit from the power enhancement scheme while minimising the risk for implementation. - Others - WF - FFS, considering that additional emission in most cases are applied on one side of the UE CBW - Issue 1-1-4: How to derive the extended IBE - Options - Option 1: Fixed IBE used for extended CBW at each side of UE CBW, the value is derived from the outmost allocated RB of UE CBW (illustrated in the figure below) - For UE CBW with full RB allocation, the IBE of the extended part could be simplified with ΔRB =1 or ΔRB =0. FFS the specific offset value. - IBE is not used in the GB - Option 2: The extended IBE requirements between the edge of the configured UE CBW and the edge of the virtual CBW should be an extension of the IBE requirement of the configured UE CBW by replacing NRB and LCRB with NRB,V and LCRB,V as below functions (Moderator: NRB,V and LCRB,V could be replaced by other terms, but the concept is similar) max-25-10⋅log10(NRB,V/LCRB,V), 20⋅log10EVM-3-5⋅(ΔRB-1)/LCRB,V, -57dBm+10log10SCS/15kHz-PRB - Option 3: Others - WF - FFS Extended IBE can be used in the original GB of UE CBW - FFS the extended IBE in the GB of extended CBW - Issue 1-1-5: Modulation order for the extended IBE - Proposals - For the IBE derivation for the extended CBW, use the general part of legacy IBE requirements and fix the EVM parameter to the one for BPSK. Table. Comparison of emission level @ the first 1MHz adjacent to the UE CBW UE CBW (MHz). 10; 15; 20; 25; 30; 35; 40; 45; 50; 60; 70; 80; 90; 100. SEM (dBm/30kHz); -18.2; -20.0; -21.2; -22.2; -23.0; -23.6; -24.2; -24.8; -24.0. IBE QPSK 26dBm (dBm/30kHz); -16.7; -18.7; -20.0; -21.0; -21.8; -22.5; -23.2; -23.7; -24.1; -25.0; -25.7; -26.3; -26.8; -27.2. IBE 16QAM 26dBm (dBm/30kHz); -19.6; -21.6; -22.9; -24.0; -24.8; -25.5; -26.1; -26.6; -27.1; -28.0; -28.6; -29.2; -29.7; -30.2. IBE BPSK 26dBm (dBm/30kHz); -12.0; -14.0; -15.3; -16.4; -17.2; -17.8; -18.5; -19.0; -19.5; -20.3; -21.0; -21.6; -22.1; -22.6. - WF - FFS whether the extended IBE with high order EVM could have impact on MPR for new inner RB allocation Sub-topic 1-2: Approaches to enable MPR reduction for both scenario 1 and scenario2 Scenarios discussed previous RAN4 meetings: - Scenario 1-1: Scenario with no adjacent in-band/out-of-band co-existence issue (single operator) - Scenario 1-2: Scenario with no adjacent in-band/out-of-band co-existence issue (adjacent operators) - Scenario 2: Narrower UE channel BW within wider BS bandwidth Sub-topic description - Issue 1-2-1: Width/RB numbers of extended CBW at each side of UE CBW - Options - Option 1: Fixed 0.5* UE CBW is considered - Symmetrical extension is 0.5 * UE CBW on each side. - Asymmetric extension is 0.5 * UE CBW on one side. - Option 2: ≤ 0.5* UE CBW, namely flexible size at each side pending on deployment scenario, and offset of the edges between UE CBW and BS CBW, etc. - Option 3: In scenario 2 no limitations are needed on width/RB numbers of extended CBW at each side of UE CBW except that Extended UE BW is limited to BS CBW. Scenario 1 needs more discussion. - Option 4: The CBW extension shall be aligned with existing channel bandwidth sizes. The extension should always result into a total number of RBs (“original UE channel RBs” + “RB extension”) being an element of Table 5.3.2-1 from TS 38.101-1. - Option 5: Extension is performed by 1/6 of the UE channel bandwidth on each side, totaling 4/3 of the original UE channel bandwidth - WF - FFS in next meeting - The max size of the extension at each side of UE CBW - Whether extended CBW is fixed or could be flexible - Issue 1-2-2: How to get the width/RB numbers of extended CBW at each side of UE CBW - Options - Option 1: Fixed value, i.e., 0.5* UE CBW is stipulated in the spec - Option 2: Flexible value signalled by the NW, the extended CBW at each side ≤ 0.5* UE CBW but not necessarily the one in Table 5.3.2-1 in TS 38.101-1 - Option 2a: Flexible value signalled by the NW, the extended CBW at each side should be the CBW specified in Table 5.3.2-1 in TS 38.101-1 - Option 3: Based on SIB1 cell specific CBW - WF - FFS in next meeting - Issue 1-2-3: Guard band for extended CBW - Options - Option 1: if extended CBW at each side is from CBW in Table 5.3.2-1 in TS 38.101-1, the RB number is the maximum transmission bandwidth configuration corresponding to the subcarrier spacing of the component carrier and the extended CBW, GB is determined accordingly - Option 2: guard band for extended CBW is derived from the total RBs including the extended ones, which is larger than the larger guard band between: - The guard band of the CBW which is immediately lower than the bandwidth corresponding to the total number of RBs (UE RBs + extended RB). - The guard band of the CBW which is immediately higher than the bandwidth corresponding to the total number of RB (UE RBs + extended RB) - Option 3: For fixed 0.5* UE CBW case, The GB of the original CBW should be retained, and the ‘GB’ of the extension part should only be retained on the side that is far away from the original CBW. - ‘GB’ of the extension part = [(0.5* BWChannel x 1000 (kHz) – ceil(NRB/2) x SCS x 12) -SCS/2], BWChannel and NRB correspond to the original UE CBW. - Others - WF - FFS in next meeting - Issue 1-2-4: Approaches of deriving the new inner RB region with RBs in extended CBW - Options - Option 1: Fixed value case with extended CBW - If both sides of the UE extend by 0.5 times the UE CBW, then the entire RB allocation inside the original UE CBW is inner region. If only the higher side of the UE extends by 0.5 times the UE CBW, then the inner RB region should be re-defined as: RBStart_Low ≤ RBStart ≤ NRB – LCRB, and LCRB ≤ ceil(2/3 *NRB) If only the lower side of the UE extends by 0.5 times the UE CBW, then the inner RB region should be re-defined as: RBStart ≤ RBStart,High, and LCRB ≤ ceil(2/3 *NRB) - Option 2: Flexible value case with extended CBW at each side - Variety expressions from companies, but essentially are based on the new NRB to derive the new inner region, e.g. - N'RB=Nextended_low+NRB+Nextended_high - Option 3: Using cell specific CBW to derive the new inner region - WF - FFS in next meeting Sub-topic 1-3: Signaling - Issue 1-3-1: NW signaling - Proposals - Proposal 1: (NTT DOCOMO) - BS indication should be designed to allow selection of which ACLR/SEM/spurious emission can be relaxed to fully enable this feature. - Proposal 2: (MediaTek) - Define RRC signalling from network to UE to permit UE to apply “shifted OOBE + MPR improvement”, in order to allow mobile operators the choice of whether to allow it or not. - Proposal 3: (vivo) - Based on the fixed extension ratio, i.e., 0.5* UE CBW on each side, the following signaling mechanism could be considered: - It is up to the NW to judge the gap width on both sides of the UE. - When the gap on one side is ≥ 0.5* UE CBW and the UE supports [MPR improvement with BW extension-r19], the NW sends the signaling (e.g., BWExtension -LowerSide-Rel19, or BWExtension-HigherSide-Rel19) to indicate the extension on the corresponding side. - When the gap on both sides is ≥ 0.5* UE CBW and the UE supports [MPR improvement with BW extension-r19], the NW sends two signaling (e.g., BWExtension -LowerSide-Rel19 + BWExtension-HigherSide-Rel19) or one combined signaling (e.g., BWExtension -BothSides-Rel19) to indicate the extension on both sides. - Proposal 4: (Skyworks) - Proposal on RB extension: - Bandwidth extensions are signalled by the BS to the UE with the lower side and higher side RB extensions expressed as: RBextL and RBextH - Maximum RBextL and RBextH is NRB_UE/2, and thus for the BS, this is NRB/2 of the largest supported UE CBW. - Inner and outer allocation can be redefined by the UE, using the existing inner/outer allocation equations where: - NRB is replaced by NRB+RBextL+RBextH - RBstart is replaced by RBstart+RBextL - With 273RB for 100MHz CBW, the maximum RB extension value is 137, thus can be signalled with 8 bits, corresponding to 16bit total for the lower and higher side RB extensions. For 15kHz SCS cases, 50MHz has NRB/2=135, thus the same number of bits. - Note that for the 100MHz case in 30kHz SCS and 50MHz in 15kHz SCS, the RB extension would correspond to BS bandwidth cases with DL CA or multiple channels (for example band n41 100+90MHz case in the US, or 100+60MHz case in China) - several unused codes can be used as “short-cut” signalling: for example, all 1s would mean that all allocations can be turned to inner. - Others - WF - FFS whether NW signalling need to consider different requirements or just a function enabler without considering specific requirements - Issue 1-3-2: UE capability - Proposals - Proposal 1: - If the UE reports the capability supporting MPR reduction with extended CBW, e.g. [MPR improvement with BW extension-r19], the NW decides whether to allow such feature with specific signaling. - Proposal 2: If any reduction of MPR would be specified in the end, it should be an optional feature for UE with per-band capability (Sony) - Proposal 3: Define separate UE capabilities for scenario 1 and scenario 2 (Nokia) - Proposal 4: (Skyworks) - A UE capability is needed to inform the BS that the UE can reassign its outer/edge allocations to inner. Two levels of UE capability may be considered: - One where the UE is able to reassign outer/edge to inner, based on 1RB granularity of signalled RB extension. - One where the UE is only able to turn all or none of its outer/edge RBs into inner (this may be a valid option for RedCap UEs). - Others - WF - MPR reduction with extended CBW is an optional feature with UE capability - FFS details of UE capability

Discussion

Approaches to enable MPR reduction for both scenario 1 and scenario 2

Network indication method can be one of simple method without additional simulation and used to cover outer RB allocation to inner RB allocation while including the above aspects in WF. So far as we have discussed with extended UE CHBW. In this discussion, unified solution converting outer RB allocation to inner RB allocation is coming. Because MPR value in the spec was made under the condiction that there is adjacent allocated channel. At this aspect, if network indicate some information (e.g. NRB of extended UE CHBW and RBextension) to UE. UE can calculate the new MPR value with converting outer RB allocation to inner RB allocation.

Proposal 1: NRB applied with new approach can be defined as following:

NRB_extension=RBleft_extension+ RBright_extension +NRB

NRB_extension is calculated as follow NRB_extension=RBleft_extension+ RBright_extension +NRB. Then, if the network indicates the NRB_extension and RBleft_extension values to the UE, the UE can apply the MPR value of the new region based on the expended UE CHBW. The outer RB allocation can be changed to an inner RB allocation based on the network indication.

Figure2. Operation based on BS indication

We consider network indication when converting outer RB allocation to inner allocation as below

RBStart,Low = max(1, floor(LCRB/2))

RBStart,High = NRB_extension – RBStart,Low – LCRB

The RB allocation is an Inner RB allocation if the following conditions are met

RBStart,Low  ≤  RBStart + RBleft_extension  ≤  RBStart,High, and

LCRB  ≤  ceil(NRB_extension /2)

Proposal 2: Consider network indication when converting outer RB allocation to inner allocation as below;

RBStart,Low = max(1, floor(LCRB/2))

RBStart,High = NRB_extension – RBStart,Low – LCRB

The RB allocation is an Inner RB allocation if the following conditions are met

RBStart,Low  ≤  RBStart + RBleft_extension  ≤  RBStart,High, and

LCRB  ≤  ceil(NRB_extension/2)

Inband emission requirement

Until last meeting, RAN4 agree that IBE should be used between edges of UE CBW and extended UE CBW. For the application of IBE in the range between edges of UE CHBW and extended UE CHBW. IBE can be specified as following figure 4. IBE of extended UE CBW can be applied guard RB of original CBW. Because there are no active RBs in the extension region. and also virtual CBW will be defined. That’s why IBE of extended CBW can applied to guard RB of UE CBW.

Figure4. IBE requirement applied with new approach[1]

Equation of general requirement for in-band emissions is limited according to UE CHBW and allocated RB.

IBE requirement for extended UE CHBW can be changed from NRB to NRB_extension(NRB_extension=RBleft_extension+ RBright_extension +NRB). if IBE of extended UE CHBW was specified with above method. There is no need to define the IBW with extension region separately.

Proposal3 : Extended IBE requirement could be applied in the original GB of UE CBW

Width/RB numbers of extended CBW at each side of UE CBW

Actually,when half of NRB on each side was added, the existing outer RB allocation of UE CHBW totally comes into the inner. However, there is no harm when the extension is extended beyond that. So. It is possible to freely decide the width of extended UE CBW on the network.

Proposal 4: Width of extended UE CBW could be freely decided based on the network.

- 2. Conclusion

In this contribution, we want to share some views on Power domain enhancements for single carrier and proposals are made as following:

Proposal 1: NRB applied with new approach can be defined as following:

NRB_extension=RBleft_extension+ RBright_extension +NRB

Proposal 2: Consider network indication when converting outer RB allocation to inner allocation as below;

RBStart,Low = max(1, floor(LCRB/2))

RBStart,High = NRB_extension – RBStart,Low – LCRB

The RB allocation is an Inner RB allocation if the following conditions are met

RBStart,Low  ≤  RBStart + RBleft_extension  ≤  RBStart,High, and

LCRB  ≤  ceil(NRB_extension/2)

Proposal3 : Extended IBE requirement could be applied in the original GB of UE CBW

Proposal4: Width of extended UE CBW could be freely decided based on the network.

- 3. Reference

[1] R4-2503013, “WF on Power domain enhancements”