3GPP TSG-RAN WG4 Meeting # 114-bis	R4-2504516

# Wuhan, China, 7th – 11th April, 2025

Source:	LG Electronics

Title:	Discussion on RF requirement for A-IoT device 1

Agenda item:	7.24.2.2

Document for:	Discussion

Introduction

In 3GPP RAN4 #114, WF[1] was approved. In this paper, we provide our views on RF requirement for device 1.

Issue 3-2-1: Transmit output power Agreement: - Measurement configuration including input CW power and potential other conditions need to be defined. Issue 3-3-2: Maximum input power Agreement: - Further discuss the maximum input power for A-IOT device for R2D signal.

Discussion

Backscattering power

A-IoT device 1 is simple structure and there is no amplification function in device 1. It will be expected to specify simply requirement related to output power. So, Until SI period, RAN4 agree with it need to define backscattering power or backscattering loss. Backscattering power can be calculated with link budget.

Figure1. Backscattering power and Backscattering loss

Through above figure, we can see the what is the variale parameter for determining backscattering power or loss. This backscattiner power depend on the path loss in air. So it necessary to fix the distance for calculating path loss. Need to discuss whether it specify specific value or equation format for backscattering power. And the equation is naturally coming according to link budget as follow.

CBackscattering power=ACW EIRP-Ploss air+M-Ploss iot device+Antenna Gain

Or 
Backscaterring power= CW Received power-Ploss_iot_device +Antenna Gain

In the above equation, A(CW EIRP) means CW power from CW node and Ploss air means path loss on air. The CW signal passes through the path loss in the air and enters the A-IoT device. At this time, the loss that occurs when experiencing the inside of the A-IoT device is expressed as Ploss_iot_device , and M is defined as implementation margin. Ploss_iot_device  mean loss in the A-IoT device. Regarding the loss in A-IoT device(Ploss_iot_device ), it is specified according to what A-IoT device structure will design. Actually it is totally depending on backscatter part. CW is not related to operation kind of modulation. It is just carrier for carrying for signal. So it is belong to backscattering modulator. So it is expected to define as following.

Backscattering loss=2*Ploss backscattering modulator

Specific value can be specified according to backscattering modulator design. And at last meeting, diverse method to define output was suggested. The efficiency is one of way to define output power of A-IoT device. It can be defined as following

Backscattering efficiency=Refelction powerInput power×100(%)

Specific value can be specified with A-IoT device design.

Proposal1: Transmitted output power of A-IoT Device 1 can be defined by one of the following three formulas:

Option1 Backscattering power

Backscattering power= CW Received power-Ploss_iot_device +Antenna Gain

Option2 Backscattering loss

Ploss_iot_device =2*Ploss backscattering modulator

Option3 Backscattering efficiency

Backscattering efficiency=Refelction powerInput power×100(%)

Maximum input level

The device cannot control power. If large power enter in device, device can be damaged such as diode or transistor saturation and breakdown. That’s why the maximum input level may need to be specified. Generally most of block consist of device is passive element. But such as RF-ED receiver would be consist of active element such as transistor. It should be demaged from the large power. It make degrade the performance of RF-ED. RAN4 need to have further discuss.

Proposal 2: RAN4 need to discuss maximum input power considering RF-ED receiver impact

- 2. Conclusion

In this contribution, we want to share some views on Ambient IoT device1 for RF requirements and proposals are made as following:

Proposal 1: Transmitted output power of A-IoT Device 1 can be defined by one of the following three formulas:

Option1 Backscattering power

Backscattering power= CW Received power-Ploss_iot_device +Antenna Gain

Option2 Backscattering loss

Ploss_iot_device =2*Ploss backscattering modulator

Option3 Backscattering efficiency

Backscattering efficiency=Refelction powerInput power×100(%)

Proposal 2: RAN4 need to discuss maximum input power considering RF-ED receiver impact

- 3. Reference

[1] R4-2502858, “WF on requirements for system parameters and A-IOT device”