3GPP TSG-RAN WG4 Meeting # 114-bis	R4-2504517

# Wuhan, China, 7th – 11th April, 2025

Source:	LG Electronics

Title:	Discussion on RF requirement for CW

Agenda item:	7.24.2.3

Document for:	Discussion

Introduction

In 3GPP RAN4 #114, WF[1] was approved. In this paper, we provide our views on RF requirement for CW.

[#114 meeting WF in RAN4] CW RF Requirement; Potentially needed or not. Output power; YES. ON/OFF power; TBD. Frequency error; YES. Phase noise; TBD for outside topology. Unwanted emission; YES. Spurious emissions; YES. Intermodulation; TBD.

Discussion

Output power

Scenario in Rel-19 is assumed with D1T1-B. it is A-IoT BS and A-IoT device is in indoor condition and NR gNB and NR UE is in outdoor condition. Figure1 is represented as following.

Figure1. D1T1-B scenario

As above, the CW node exists outside the BS. And this CW is a newly defined value as a node that did not exist before. The following should be considered when defining the output power of this value.

Looking at the A-IoT operating principle below, the power output of the A-IoT device is affected by the CW. Therefore, the power of the CW directly affects the output power of the A-IoT device, so it becomes an important parameter.

Figure2. CW operation principle in A-IoT device

CW mean carrier wave, this signal role is carring for signal from A-IoT transceiver and module. So this is related to output power of device. When considering the CW Transmitted output power, it is highly likely that the reference sensitivity of the BS will also be taken into account. This is because device1 is a passive component and cannot generate power on its own. Therefore, since it is a structure that reflects the power coming in from the outside as it is, it must reach reader where the BS can recover the signal by reflecting it with the power of the CW. The parameters shown in the figure below can be calculated as the path loss parameters to be considered.

Calculations can be made under the following conditions:

1. Indoor environment (indoor, 50m distance)

2. Device receiving sensitivity: -40dBm ~-30dBm

3. High receiving sensitivity (D): -110dBm

4. High output power: 33dBm

5. Example band: 900MHz (n8, FR1)

6. Benefit antenna gain: 5dBi

7. Imported antenna gain: 0dBi

ITU indoor path loss formula is following as:

PL=Ploss air=20logf+20logd+32.4

According to the above equation, when it is 50m, PL=20 log(0.9)+20 log(50)+32.4=65.46dB. If we substitute this calculated value into the following equation,

CTransmitted output power= ACW EIRP-Ploss air+M1-Ploss_iot_device +Antenna Gain

Ploss_iot_device =2*PBackscattering modulator

M1 is the value due to fading including implementation margin, penetration loss, etc. Consider M1 =-5dB. Ploss_iot_device  is the loss experienced inside the device and can be expressed as reflection loss. The reason Ploss_iot_device =2*PBackscattering modulator is because it experiences PBackscattering modulator twice, once when entering and once when exiting. Let's assume Ploss_iot_device =7dB(PBackscattering modulator=3.5dB). Then, the transmitted output power of the device can be calculated as follows. The values ​​assumed here can change.

CTransmitted output power of A-IoT=A-65.46dBm-5dB-7dB+0dB=A-77.46dBm

Then, the transmitted output power emitted from this device must go back to the BS and have a value greater than the reference sensitivity.

BS Received power=A(CW transmitted output power)-77.46dBm-Ploss air=A-77.46dBm-65.46dB=A-142.92dBm≥-110dBm

Then, A(CW transmitted output power)≥32.92dBm is obtained. That is, under the above conditions, the required CW transmitted output power will be a value greater than or equal to 32.92dBm. Therefore, the CW transmitted output power can be defined as follows, for example CW transmitted output power can be defined as follow.

CW transmitted output power, PCW

| Condition | PCW(Transmitted output power) |
| --- | --- |
| Indoor(10-50m) | ≤33dBm |

Proposal 1: CW output power should be ≤ 33dBm

ON/OFF requirement

CW is required when device backscatter, ON/OFF requirement of CW is needed for aligning with D2R transmission in time aspect. If CW always turn on, it effect on BS operation as interference. it will be beneficial to define ON/OFF requirement. This will also minimize the interference in D2R receiving of reader.

- Proposal 2:It is necessary to specify ON/OFF TX requirement.

- 2. Conclusion

In this contribution, we want to share some views on Ambient IoT CW for RF requirements and proposals are made as following:

Proposal 1: CW output power should be ≤33dBm

- Proposal 2: It is necessary to specify ON/OFF TX requirement.

- 3. Reference

[1] R4-2502859, “WF on requirements for A-IoT BS and CW”