3GPP TSG-RAN WG4 Meeting #114bis	R4-2504518

Wuhan, China, April 7th – April 11th, 2025

Source:	Nokia

Title:	Discussion on performance requirements for Fast SCell activation with Rel-18 EMR

Agenda item:	7.17.5

Document for:	Discussion

Introduction

This contribution discusses the performance part for fast Scell activation with EMR.

Discussion

Performance part test case

To recap some legacy from Rel-18 performance part, our view was that the Rel-16 test case did not sufficiently cover the impact of validityDuration, because validityDuration applies even if T331 is still running. In Rel-16, the UE is expected to report measurement results if T331 is running. If the duration of T331 is longer than validityDuration, the UE would be allowed to report results from the time before validityDuration, which is not really correct UE behaviour according to Rel-18. Therefore, in Rel-18 we proposed to consider adding a test case, where the UE reports valid measurements from the duration of validityDuration, similar to the test cases that have been defined for cell reselection measurement based early reporting.

However, we understand that the validityDuration may have been challenging to test. Now when considering the rel-19 test case, to avoid any similar issues, we can focus on the test case with successful SCell activation and test the time duration associated with it. This makes the test case more meaningful for the Rel-19 feature and also adds some test coverage on Rel-18 feature. We would also like to see all the meaningful scenarios to be tested. We also understand that there are quite many combinations so we can discuss how to best test the functionality while not having too many test cases.

Test the scenario where measIdleValidityDuration-r18 is configured with successful SCell activation with EMR.

Test scenario where measReselectionValidityDuration-r18 is configured

When the measIdleValidityDuration-r18 is not configured, the test is quite similar to legacy EMR test case. Similar principle than EMR can be used, and Successful SCell activation is tested in addition to legacy EMR.

Test the scenario when measIdleValidityDuration-r18 is configured while T331 is running with SCell activation.

As both FR1 and FR2 are supported, RAN4 can discuss whether all test cases are done for FR1 and FR2 or whether some of the cases can be consolidated to only be done for FR1 or FR2.

Discuss whether all test cases are done for both FR1 and FR2.

In addition to previous, multiple activation options are supported. Discuss if all of them are tested with all of the combinations or if it is enough to test a subset.

Discuss which scenarios to apply on each test case:

  - Direct SCell Activation at SCell addition

  - MAC CE based Single SCell Activation

  - MAC CE based PUCCH SCell activation

Conclusion

The following Observations and Proposals were made:

Proposal 1: Test the scenario where measIdleValidityDuration-r18 is configured with successful SCell activation with EMR.

Proposal 2: Test scenario where measReselectionValidityDuration-r18 is configured

Proposal 3: Test the scenario when measIdleValidityDuration-r18 is configured while T331 is running with SCell activation.

Proposal 4: Discuss whether all test cases are done for both FR1 and FR2.

Proposal 5: Discuss which scenarios to apply on each test case:

	Direct SCell Activation at SCell addition

	MAC CE based Single SCell Activation

	MAC CE based PUCCH SCell activation