3GPP TSG-RAN WG4 Meeting #114bis            	R4-2504534

Wuhan, China, 7th-11th, April, 2024

Agenda item:	7.18.3

Source:	Samsung

Title:	View on the remaining issue for BS demodulation requirement with MMSE-IRC receiver

Document for:	Discussion

- Introduction

In the last RAN4 meeting, the remaining issue of Rel-19 WI on demodulation performance requirements enhancement with MMSE-IRC receiver was further discussed. The related agreements are captured in the WF [1] as following

- Interference modelling - Number of interferers for 2Rx - For HetNet scenario, introduce BS MMSE-IRC requirements with 2Rx - For the number of interferers for 2Rx under HetNet, further decide between the following: - Option 1: 2 interferers - Option 2: 1 interferer - For HomNet scenario, further decide among the following - Option 1: 2 interferers - Option 2: 1 interferer - Option 3: Not to consider 2Rx requirements for HomNet scenario - Companies are encouraged to bring results for all options above and this issue will be concluded in the next meeting - Test parameters and simulation assumptions - Channel Bandwidth and waveform - For CP-OFDM - For 15kHz SCS, cover 5MHz, 20MHz - For 30kHz SCS, cover 10MHz, 100MHz - Not to cover DFT-s-OFDM - MCS - MCS 16 for 2Rx (if introduced), 4Rx and 8Rx - Propagation condition for the serving cell - Select between the following options - Option 1: TDLA30-10 for HetNet and TDLB100-400 for HomNet - Option 2: TDLC300-100 for all cases - Option 3: TDLA30-10 for HetNet - Companies are encouraged to bring results for all options above and this issue will be concluded in the next meeting - DMRS configuration- whether to include DMRS configuration type 2 - Not to consider DMRS configuration type 2 - Test requirement definition - Use the SNR based requirement definition - Manufacturer declarations and applicability rules - Option 1: Introduce new manufactory declaration for BS MMSE-IRC receiver - Option 2: Do not need to introduce a new Manufacturer Declaration on MMSE-IRC - Applicability - Candidate options on test requirement applicability for different BS classes and test scenarios: - Option 1: HomNet requirements are not applicable for Local Area BS and HetNet requirements are not applicable for Wide and Medium Area BS - Other options are not precluded - Based on test requirement applicability, further discuss whether to consider additional applicability rule on how the test is performed: - Option 1: If a BS declared to support Wide Area BS, Medium Range BS and pass the tests for HomNet (DIP Set1), the test for HetNet (DIP Set 2) can be skipped - Option 2: If a BS declared to support HomNet or HetNet and pass the tests for HomNet, can also consider the test for HetNet as passed - Option 3: Do not define additional applicability rule for homogenous and heterogenous network scenarios

In this contribution, the view on the remaining issue of test setup for BS demodulation with MMSE-IRC were provided.

# 2	General

## 2.1	Test scenario

Manufacture declaration and applicability rule

For test scenario, both HomNet and HetNet scenarios were agreed to cover the performance requirement of MMSE-IRC receiver. Regarding the applicability for requirement in different scenarios, it will be further discussed.

In LTE, for requirement in HomNet scenario was not appliable for Local Area BS and Home BS, since the HomNet is mainly targeting for macro cell only as following table

Table 1: Enhanced performance requirement type A for PUSCH, 1.4 MHz Channel Bandwidth

| Number of TX antennas (Note 1) | Number of RX antennas (Note 1) | Propagation conditions and correlation matrix (Annex B) (Note 2) |  |  | DIP set | FRC (Annex A) | Fraction of maximum throughput | SINR [dB] (Note 3) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Tested signal | Interferer 1 | Interferer 2 |  |  |  |  |
| 1 | 2 | EPA 5 Low | ETU 5 Low | N/A | Set 2 | A12-1 | 70% | -4.8 |
|  |  | EVA 70 Low | ETU 70 Low | N/A | Set 1* | A12-1 | 70% | -2.0 |
|  | 4 | EPA 5 Low | ETU 5 Low | ETU 5 Low | Set 2 | A13-1 | 70% | -4.1 |
|  |  | EVA 70 Low | ETU 70 Low | ETU 70 Low | Set 1* | A13-1 | 70% | -0.1 |
|  | 8 | EPA 5 Low | ETU 5 Low | ETU 5 Low | Set 2 | A4-3 | 70% | -4.7 |
|  |  | EVA 70 Low | ETU 70 Low | ETU 70 Low | Set 1* | A4-3 | 70% | 0.0 |
| Note*: Not applicable for Local Area BS and Home BS. |  |  |  |  |  |  |  |  |

For NR, only three categories of BS class were defined as wide area base station, medium Range base station and local area range base station in current specification. Therefore, we think the similar approach can be applied for NR to differentiate the different BS classes.

Observation 1:  The requirement in HomNet (DIP set 1) is not applicable for local Area BS and Home BS in LTE system.

Observation 2:  Only three categories of BS class were defined in NR system as wide area base station, medium Range base station and local area range base station.

Regarding the Home BS scenario, since it is not introduced in current specification, we do not need to mention it in current release. If home BS was introduced into future release, the applicable of requirement can be updated.

For HetNet scenario, it should be not applied for wide and medium area BS,

Proposal 1: Introduce the applicability rule for these two requirements testing. The following test applicability rule can be considered as

  - HomNet requirements are not appliable for local area BS, and HetNet requirements are not appliable for Wide and Medium Area BS

  - Note: If Home BS is introduced into future release, the appliable of requirement can be updated as

HomNet requirements are not appliable for local area BS and Home BS.

Meanwhile, from the baseband processing perspective of MMSE-IRC, there is no difference foreseen. Since the test applicability for different HomNet and HetNet requirements will be considered, which means if BS is declared to support wide area BS, Medium Area BS, pass the tests of HomNet (DIP Set 1), the test for HetNet (DIP Set 2) can be skipped. Therefore, based on the declaration of BS categories, in our understanding, the BS should only to test the appliable requirement. Then, the additional applicability rule for homogenous and heterogenous network scenarios may not need.

Proposal 2: If test requirement applicability rule for different BS classes and test scenarios was introduced, the additional applicability rule for homogenous and heterogenous network scenario is not needed.

Regarding manufacturer declaration for BS with MMSR-IRC receiver, although the MMSE-IRC receiver algorithm may be implemented from Rel-15 for BS vendors, there is no interference model and related requirement defined.

Since it was specified in Rel-19 for advanced receiver to handle the interference, meanwhile, the baseline assumption for interference covariance estimation is per RB based on DMRS symbol, therefore, it was better to introduce the new manufactory declaration for BS MMSE-IRC receiver.

Therefore, the following manufacturer declaration for BS MMSE-IRC receiver can be considered

Table 2: manufactory declaration for BS with MMSE-IRC receiver

| Declaration identifier | Declaration | Description | Applicability |  |
| --- | --- | --- | --- | --- |
|  |  |  | BS type 1-C | BS type 1-H |
| D.xxx | MMSE-IRC receiver | Declaration of BS supporting MMSE-IRC receiver. | x | x |

Proposal 3: Introduce new manufactory declaration for BS with MMSE-IRC receiver as following

| Declaration identifier | Declaration | Description | Applicability |  |
| --- | --- | --- | --- | --- |
|  |  |  | BS type 1-C | BS type 1-H |
| D.xxx | MMSE-IRC receiver | Declaration of BS supporting MMSE-IRC receiver. | x | x |

## 2.2	Interference modeling

Number of Interferers

Regarding number of interferer UE modeling, it was agreed to check the performance with 1 interferer and 2 interferers for 2Rx. Based on our initial simulation, the gain can be achieved by MMSE-IRC receiver compared with MMSE receiver for both HomNet and HetNet scenario for both 1 interferer and 2 interferes. Therefore, either 1 interferer and 2 interferers can be considered for both HomNet and HetNet scenarios. We slightly prefer to consider 1 interferer.

For different number of interferers applied for different scenario, since the test applicability rule for different scenario was under discussion. Therefore, it may require the new test applicability rule introduced to differentiate different number of interferers for different scenarios. To simplify the test applicability, we slightly prefer to apply the number of interferers for both HomNet and HetNet scenarios. We are also open to further discuss whether it was necessary to differentiate it, considering the gain can be achieved for both 1 interferer and 2 interferers under two scenarios.

Observation 3: The gain can be achieved with MMSE-IRC receiver compared with MMSE receiver for both 1 interferer and 2 interferers modeling, under the interference profile for both HomNet and HetNet scenarios.

Proposal 4:  Either 1 interferer and 2 interferers can be considered for 2Rx to specify the PUSCH requirement with MMSE-IRC receiver. Considering 1 interferer for moving forward under two sceanrios.

# 3	Test parameters and simulation assumption

In this section, the remaining simulation assumptions are provided for evaluation.

Propagation condition

In the last meeting, different channel condition combination is considered for specifying the PUSCH requirement with MMSE-IRC. In Rel-15, several channel models are specified to cover different delay spread and doppler spread, including TDLA30-10, TDLB400-100 and TDLC300-100, and different MCS will be applied for different channel, where MCS 20 with TDLA30-10, MCS 16 with TDLC300-100, and MCS 2 with TDLB100-400.

Observation 4: In Rel-15, several channel models are specified to cover different delay spread and doppler spread, including TDLA30-10, TDLB400-100 and TDLC300-100, and different MCS will be applied for different channel, where MCS 20 with TDLA30-10, MCS 16 with TDLC300-100, and MCS 2 with TDLB100-400

In the last meeting, MCS 16 was agreed for all the Rx configuration. To align the existing cases, our preference is to consider TDLC300-100 for all cases with HetNet and HomNet Scenario. Considering the HetNet and HomNet scenario are different scenarios, to consider the test coverage of MMSE-IRC and, we are ok to define the sperate channel models for different scenario. To check the per-PRB interference covariance estimation, it is better to choose the channel with high delay spread for the interferers. Meanwhile, based on our simulation results, the achieved SNR will be high for TDLB100-400 under MCS 16, considering the doppler is high. Therefore, we would like to prefer with option 3: TDA30-10 for HetNet, and TDLC300-100 for HomNet scenario, as compromise

Proposal 5: RAN4 considers the following channel model for targeting cell to specify the PUSCH requirement with MMSE-IRC receiver

  - Targeting cell

    - Option 2: TDLC300-100 for all cases

    - Option 3: TDLA30-10 for HetNet and TDLC300-100 for HomNet as compromise

# 4	Conclusion

In this contribution, the view on the remaining issues of BS demodulation requirement with MMSE-IRC receiver was provided.

Observation 1:  The requirement in HomNet (DIP set 1) is not applicable for local Area BS and Home BS in LTE system.

Observation 2:  Only three categories of BS class were defined in NR system as wide area base station, medium Range base station and local area range base station.

Proposal 1: Introduce the applicability rule for these two requirements testing. The following test applicability rule can be considered as

  - HomNet requirements are not appliable for local area BS, and HetNet requirements are not appliable for Wide and Medium Area BS

  - Note: If Home BS is introduced into future release, the appliable of requirement can be updated as

HomNet requirements are not appliable for local area BS and Home BS.

Proposal 2: If test requirement applicability rule for different BS classes and test scenarios was introduced, the additional applicability rule for homogenous and heterogenous network scenario is not needed.

Proposal 3: Introduce new manufactory declaration for BS with MMSE-IRC receiver as following

| Declaration identifier | Declaration | Description | Applicability |  |
| --- | --- | --- | --- | --- |
|  |  |  | BS type 1-C | BS type 1-H |
| D.xxx | MMSE-IRC receiver | Declaration of BS supporting MMSE-IRC receiver. | x | x |

Observation 3: The gain can be achieved with MMSE-IRC receiver compared with MMSE receiver for both 1 interferer and 2 interferers modeling, under the interference profile for both HomNet and HetNet scenarios.

Proposal 4:  Either 1 interferer and 2 interferers can be considered for 2Rx to specify the PUSCH requirement with MMSE-IRC receiver. Considering 1 interferer for moving forward under two sceanrios.

Observation 4: In Rel-15, several channel models are specified to cover different delay spread and doppler spread, including TDLA30-10, TDLB400-100 and TDLC300-100, and different MCS will be applied for different channel, where MCS 20 with TDLA30-10, MCS 16 with TDLC300-100, and MCS 2 with TDLB100-400

Proposal 5: RAN4 considers the following channel model for targeting cell to specify the PUSCH requirement with MMSE-IRC receiver

  - Targeting cell

    - Option 2: TDLC300-100 for all cases

    - Option 3: TDLA30-10 for HetNet and TDLC300-100 for HomNet as compromise

# References

R4-2502272, WF on [114][323] NR_demod_Ph5_Part1_General_BS