3GPP TSG-RAN WG4 Meeting #114bis	R4-2504541

Wuhan, China, April 7 – 11, 2025

Source:	Nokia

Title:	CSI reporting requirement and testing framework for CSI prediction

Agenda item:	7.19.2

Document for:	Discussion

Introduction

This paper discusses the CSI reporting requirement framework for CSI prediction. The testability and interoperability issues for CSI prediction has been discussed, and the agreements from RAN4#113 are summarize in WF [1]. In this paper, we address the challenges related to testability and LCM requirements for AI/ML-based CSI prediction use cases.

Discussion

On Rel-19 AIML-based CSI reporting

Training and test data consistency

The prediction accuracy of an AIML-based algorithm can be enhanced if the training and test data are closely matched. The use of AI/ML requires careful consideration of data consistency to ensure accurate and reliable results. In RAN4#114, the consistency between training and test data has been discussed [2].

Issue 1-3: Training and test data consistency Agreement: - further discuss the following options - Option 1: Consistency between training data and test data should be ensured - RAN4 to study how to ensure consistency between training data(simulation) and test data (conformance test) and whether anything special should be done during the test - Option 2: Consistency will be implicit as test emulated channel will be very similar to the one used in RAN4 simulations

Related to the discussion on consistency in RAN4, we observe that the agreements in RAN1#118bis have been made on the potential NW-side conditions to ensure consistency between and training and inference. Also, the RAN1#120 meeting has discussed the resource configuration for data collection for training using the UE-side model.

(RAN1#118bis) Agreement For consistency between training and inference, study to identify which potential NW-side additional conditions, if any, may impact on UE assumption for CSI prediction using UE-sided model, resulting non-negligible degradation on model generalization performance. Note: Companies are encouraged to provide generalization performance evaluation on the impact of NW-side additional conditions, if considered. Agreement For generalization evaluation to identify potential NW-side additional conditions to ensure consistency between training and inference, consider one or more of the following aspects, if evaluated: - [Optional evaluation] Various tilt angle (e.g., [102] degrees, [110] degrees) - [Optional evaluation] Various TXRU mapping - With TXRU virtualization (e.g., for 32 port CSI-RS, [N1, N2, P] = [2, 8, 2] (baseline), [4,4,2] (optional) with (8x8x2) and (12 x 8 x 2) antenna elements) - Without TXRU virtualization (e.g., for 32 port CSI-RS, [N1, N2, P] = [2, 8, 2] (baseline), [4,4,2] (optional) with (2x8x2) and (4 x 4 x 2) antenna elements, respectively) - Other configurations are not precluded Note: for other evaluation assumptions, reuse baseline of TR 38.843 Note: report how to map antenna port to antenna elements Note: report the backbone/structure of AI/ML model used (RAN1#120) Agreement For CSI prediction using UE-side model, for data collection for training, - For resource configuration, - For CSI-RS resource type for CMR, periodic and semi-persistent are supported. - FFS on support of aperiodic CSI-RS resource - FFS on whether to separately configure CSI-RS resource(s) used for measurements for model input and CSI-RS resource(s) used for measurements for ground-truth CSI - FFS whether/how to enhance CSI-RS resource configuration to reduce CSI-RS signalling overhead. - FFS on CSI report configuration Conclusion 4-2 For CSI prediction using UE-sided model, RAN1 has no consensus on specifying consistency of training/inference due to NW-side additional conditions. Note: The reason for no consensus is that some companies think the observation is good enough to conclude negligible performance loss while other companies think there is non-negligible performance degradation.

While RAN1 has considered to evaluate the different tilt angles and TXRU mappings to identify a data inconsistency problem stemming from NW-side additional conditions, RAN1 has no consensus on specifying consistency of training/inference due to NW-side additional conditions. In RAN4 perspective, specifying the requirements dedicated to the consistency of training and test data can yield an unnecessarily high complexity with any additional conditions. Moreover, the benefits of specifying consistency requirements are unclear in that the consistency can be achieved through the proper selection of the test configuration and parameters. Therefore, the performance gain of a separate requirement on data consistency can be marginal. Thus, Hence, RAN4 must support ‘Option 2’ in that data consistency between training and test data is achievable without a dedicated requirement.

RAN4 to support Option 2 in that data consistency between training and test data is achievable without a dedicated requirement.

LCM requirements

The testing goals were discussed and agreed upon in RAN4#112, including the use of LCM to verify the minimum performance of AI/ML functionality/feature.

Agreement: - The testing goal is to verify whether the minimum performance of AI/ML functionality/feature can be achieved - FFS on whether and how many different test configurations/parameters are used for tests. - LCM would be tested

Additionally, the scope of the LCM discussion was addressed in RAN4#113.

Agreement: - The discussion should consider use-cases specific aspects Agreement: - Whether transition from AI to non-AI would be UE initiated or network initiated - This will be a decision for other groups - RAN4 to assess what RRM requirements are needed for each case.

Moreover, for localized models, RAN1#116 has made the following agreements on testing.

Agreement For the evaluation of AI/ML-based CSI prediction using localized models in Release 19, consider the following options as a starting point to model the spatial correlation in the dataset for a local region: - Option 1: The dataset is derived from UEs dropped within the local region, with spatial consistency modelling as per TR 38.901. - E.g., Dropped in a specific cell or within a specific boundary. - Option 2: By using a scenario/configuration specific to the local region. - E.g., Indoor-outdoor ratio, LOS-NLOS ratio, TXRU mapping, etc.

Since CSI prediction relies on a UE-side model, localized models deployed in the field can influence feature performance based on their specific characteristics. Consequently, monitoring these localized models becomes imperative as part of LCM for the deployed model. By associating each model’s ID with tracked performance metrics from the training and deployment phases, it is essential that RAN4 considers LCM actions based on the localization of the models in a given region. This includes studying model-ID-based LCM requirements for the functionality assigned to the UE.

RAN4 to study the model-ID-based LCM requirements if localized models are considered for CSI prediction.

Also, RAN2 has made the agreements related to LCM procedures.

Agreements on procedures from #127: - Step 1: Network sends UECapabilityEnqiry message to initiate the procedure to a UE reporting its AI/ML supported functionalities. - Step 2: UE sends UECapablityInformation message to network, containing supported functionalities at the UE side. - “Step 3”: Following configurations are provided from NW to UE: - 1) UE is allowed to do UAI reporting via OtherConfig. - 2) Network may provide NW-side additional condition. FFS on the RRC signalling and whether it is mandatory or optional. - 3) FFS on configuration (e.g. inference configuration) of supported functionalities. FFS on the content of configuration. - UE decides the applicable functionalities based on NW-side additional conditions (if provided), UE-side additional conditions (internally known by UE) and model availability in device. FFS whether other configuration can considered by UE (e.g. inference configuration). FFS how the applicable functionality is decided if NW-side additional condition is not provided in step 3. - “Step 4”: UE reports applicable functionality in the following scenarios: - 1) Upon being configured to provide applicable functionality and upon change of applicable functionality via UAI - 2) As response to NW-side additional condition requesting applicable functionality reporting in step 3, FFS other network configuration (e.g. inference configuration), FFS via UAI or RRCReconfigurationComplete, etc - Step 5: - 1) Network configures inference configuration to UE after applicable functionality reporting, if inference configuration based on supported functionality is not provided in Step 3 (i.e. inference configuration is provided in Step 5). - 2) If inference configuration based on supported functionality is provided in Step 3, it is up to network implementation whether to provide an updated configuration or not. - The applicable functionality may be activated by receiving its inference configuration when it is provided in Step 5. FFS the initial activation state. FFS on initial state of applicable functionality if inference configuration of supported functionality is provided in Step 3. FFS on additional L1/L2 signaling for activation/deactivation. FFS if multiple applicable functionalities can be activated at the same time. FFS what is the granularity of functionality

Additionally, the agreement related LCM procedures in RAN4#111 are as follows:

Agreement: RAN 4 to consider defining RRM requirements for LCM procedures based on the specific use cases (beam management and positioning) and candidate are: - Identification - Selection - Activation - Deactivation - Switching - Fallback to non-AI operation - Performance monitoring - Others are not precluded

RAN4 needs to discuss how to manage dynamic and non-static conditions for CSI prediction. Initially, RAN4 can assume the LCM procedures defined in RAN2. For instance, a delay requirement may be needed in the procedure due to the potential processing time of the UE to determine failure or performance degradation. Therefore, RAN4 should also investigate how the RAN2 approach impacts RAN4 requirements.

RAN4 to study delay requirements in Steps 4 and 5 of the LCM procedure based on RAN2’s 5-step approach.

The LCM operations including activation, deactivation, and fallback can be different behaviors depending on CSI report configurations. Currently, AIML beam managements (BM) take into account the different CSI report configurations such as periodic, semi-periodic, or aperiodic CSI reporting. The outcome and principle of the LCM and performance monitoring in the AIML BM will be similarly applicable to the AIML CSI prediction. Therefore, similar to the BM cases, RAN4 should consider that the delay components of LCM operations can vary depending on the specific CSI report configurations.

RAN4 to discuss whether the delay components of LCM operations can vary depending on the specific CSI report configurations.

Conclusion

In this contribution, we have discussed the RRM impact of AI/ML-based CSI prediction. The proposals are presented as follows.

- RAN4 to support Option 2 in that data consistency between training and test data is achievable without a dedicated requirement.

RAN4 to study the model-ID-based LCM requirements if localized models are considered for CSI prediction.

RAN4 to study delay requirements in Steps 4 and 5 of the LCM procedure based on RAN2’s 5-step approach.

RAN4 to discuss whether the delay components of LCM operations can vary depending on the specific CSI report configurations.

- References

- R4-2420334, “WF on AI/ML air interface part 1,” RAN4#113, Orlando, USA.

- R4-2502856, “WF on the requirements for AI/ML air interface,” RAN4#114, Athens, Greece.