3GPP TSG-RAN WG4 Meeting #114-bis									                      R4-2504582

Wuhan, China, April 7 – 11, 2025

Agenda item:			7.10.3.1

Source:	Ericsson

Title:	On system parameters for less than 5 MHz CBW for NTN

Document for:	Approval

# Introduction

In RAN#106, a revised WID has been approved for “Enhanced requirements and conductive test methodology for NR NTN and IoT NTN” WI [1]. One of the three working areas of the WI is to specify less than 5 MHz for NTN with the following objectives:

- Specify the system parameters for channel bandwidth of less than 5 MHz for NR-NTN (NR based Non-Terrestrial Networks) in FR1-NTN bands

  - Consider the NR-NTN bands n255, n256, and n254

  - Add less than 5MHz channel bandwidth

    - 3MHz with the RAN1 design in Rel-18

  - Investigate and if necessary, specify the additional synchronization (sync) raster(s) for the additional channel bandwidth

    - Leverage the work on less than 5MHz for TN

  - Note: no RAN1 impact is expected

- Specify the necessary SAN RF core requirements for NR-NTN in FR1-NTN bands

- Specify the necessary NR-NTN UE RF core requirements for less than 5 MHz for NR-NTN in FR1-NTN bands

  - Focus on power class 3 (PC3) for Tx RF requirements

- Specify the necessary RRM core requirements for less than 5 MHz for NR-NTN in FR1-NTN bands

- Specify the necessary signaling to support the above objectives

In this contribution, we focus on the remaining issues for the system parameters for channel bandwidths of less than 5 MHz for NR-NTN in FR1-NTN bands from the approved WF [2], and we provide our view on them.

# Discussion

In the approved WF [2], all the discussed issues in the previous meeting have been resolved and no open issue has been captured. However, there was an open issue listed in a topic summary which hasn’t been discussed in the previous meeting:

Issue 1-3: The clarification on the design of SS/PBCH, RACH preamble formats and CORESET#0 to support 3MHz Channel bandwidth for NTN operation

- Option 1: (Huawei, Ericsson)

  - The following system design should be considered for 3MHz CBW for NTN bands:

    - 12 PRB SS/PBCH obtained by puncturing a legacy 20 PRBs SS/PBCH.

    - Short RACH preamble formats with 15kHz SCS, and long PRACH formats with 1.25kHz SCS.

    - 15 PRB CORESET#0 obtained by puncturing a legacy 24 PRBs CORESET#0.

- Option 2: (CATT)

  - For NR-NTN UE supporting 3MHz channel bandwidth in the FR1-NTN bands, the reception of 12 RB PBCH based on RB-level puncturing should be supported. 12 PRB SS/PBCH obtained by puncturing a legacy 20 PRBs SS/PBCH.

- Recommended WF

  - TBD

For NTN, SS/PBCH for 3MHz CBW should follow the same puncturing as defined in RAN1 for TN, i.e. the SSB of 12 PRBs (punctured from 20 PRBs).

According to TS38.306, UE indicating support3MHz-ChannelBW-Symmetric-r18 or support3MHz-ChannelBW-Asymmetric-r18 UE capabilities, also indicates the support for short RACH preamble formats with 15kHz SCS, and long PRACH formats with 1.25kHz SCS.

For TN 3 MHz channel bandwidth, CORESET#0 size is defined to be 15 PRBs, which was derived from puncturing the original CORESET#0 consisting of 24 PRBs. 12 PRB CORESET#0 was defined for one specific sync raster frequency for n100, and in that case a new CORESET#0 size consisting of 12 PRBs was added (i.e., no puncturing in this case, but new size). Its usage was not precluded for the main case of 3MHz CBW where transmissions can span up to the maximum transmission bandwidth, but if in such a case transmissions can span up to 15 PRBs then is preferred to use a 15 PRBs CORESET#0 (due to a link budget).

Even though 3MHz CBW has been agreed as an optional feature in the previous meeting, the same design for SS/PBCH, RACH preamble formats and CORESET#0 should be followed as for the TN general case of 3 MHz CBW.

Proposal 1: 3MHz CBW for NTN shall follow the same design as for TN for a general case of 3 MHz CBW:

- 12 PRB SS/PBCH obtained by puncturing a legacy 20 PRBs SS/PBCH.

- Short RACH preamble formats with 15kHz SCS, and long PRACH formats with 1.25kHz SCS.

- 15 PRB CORESET#0 obtained by puncturing a legacy 24 PRBs CORESET#0.

# Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1: 3MHz CBW for NTN shall follow the same design as for TN for a general case of 3 MHz CBW:

- 12 PRB SS/PBCH obtained by puncturing a legacy 20 PRBs SS/PBCH.

- Short RACH preamble formats with 15kHz SCS, and long PRACH formats with 1.25kHz SCS.

- 15 PRB CORESET#0 obtained by puncturing a legacy 24 PRBs CORESET#0.

# References

- RP-242901, “Revised WID on Enhanced requirements and conductive test methodology for NR NTN and IoT NTN”, RAN#106.

- R4-2502282, “WF for [113][311] NR_IoT_NTN_less_than_5MHz_UERF”, Xiaomi, RAN4#114.