# 3GPP TSG-RAN WG4 Meeting #114bis	R4-2504588

# Wuhan, Hubei, China, 07th – 11th April, 2025

Source: 	Huawei, HiSilicon

Title: 		OTA requirements consideration for network-nodes specifications for 7MHz channel introduction

Agenda Item:	7.8.3

Document for:	Discussion

# Introduction

WF on requirements for 7MHz channel bandwidth was agreed during RAN4#114 meeting in [2]. One open issue was captured there:

Open issue: FFS if BS OTA requirements should also be covered within this WI.

In this contribution, we provide feedback on the consideration of OTA requirements for network-nodes specifications for the introduction of 7 MHz channel bandwidth.

# Discussion

During RAN#107 meeting, WID update was Approved in [1]. It has removed band n26 and n5 from the WID name. Still, objectives are covering (UE RF) band specific core requirements to introduce 7 MHz for NR band n5, n26. 7 MHz channel bandwidth for other bands can be introduced via the appropriate basket WI. Therefore, it would be pragmatic to cover all band-agnostic requirement already in this WI – more specifically to cover both conducted and OTA requirements for BS nodes.

When it comes to the argument that bands n26 and n5 are low frequency bands (800-900 MHz) which do not require AAS BS architecture, it shall be noted that during Rel-19 we have already discussed possibility of restricting AAS BS specification to higher frequency bands. It was concluded among BS vendors that such restriction is not agreeable, as AAS products are expected to be developed during 5G-Advanced phase for below 1 GHz bands.

Furthermore, we have provided Draft CRs to the following BS specifications, covering OTA requirements – the workload was not exhaustive:

- Draft CR to TS 37.105: 7MHz channel bandwidth introduction [3]

- Draft CR to TS 37.145-1: 7MHz channel bandwidth introduction [4]

- Draft CR to TS 37.145-2: 7MHz channel bandwidth introduction [5]

- Draft CR to TS 38.141-2: 7MHz channel bandwidth introduction [6]

Therefore, the following is proposed:

Proposal 1: Introduce complementary OTA requirements for all applicable network-nodes specifications under 7 MHz channel bandwidth WI.

Proposal 2: Introduce OTA requirements for the following specifications:

- Draft CR to TS 37.105: 7MHz channel bandwidth introduction

- Draft CR to TS 37.145-1: 7MHz channel bandwidth introduction

- Draft CR to TS 37.145-2: 7MHz channel bandwidth introduction

- Draft CR to TS 38.141-2: 7MHz channel bandwidth introduction

Related Draft CR were submitted in [3-6].

# Conclusions

Based on the discussion above, the following proposals were formulated:

Proposal 1: Introduce complementary OTA requirements for all applicable NR network-nodes specifications under 7 MHz channel bandwidth WI.

Proposal 2: Introduce OTA requirements for the following specifications:

- Draft CR to TS 37.105: 7MHz channel bandwidth introduction

- Draft CR to TS 37.145-1: 7MHz channel bandwidth introduction

- Draft CR to TS 37.145-2: 7MHz channel bandwidth introduction

- Draft CR to TS 38.141-2: 7MHz channel bandwidth introduction

Related WID objectives can be updated during RAN#108. Suggested modifications were captured for reference below:

# 4 Objective ### 4.1 Objective of SI or Core part WI or Testing part WI The core part of this WI includes: - Specify the generic core requirements to support 7 MHz NR FR1 channel bandwidth [RAN4] - Introduce the channel bandwidth 7MHz - Only include 15 kHz SCS - Specify the spectrum utilization - Specify UE RF requirements for UL and DL - Specify BS RF requirements (both conducted and OTA) for UL and DL - Specify the band specific core requirements to introduce 7 MHz for NR band n5, n26. [RAN4] - BS and UE Channel bandwidth per operating band - UE reference sensitivity - When needed: - MPR (relative bandwidth criteria) - Additional Maximum Power Reduction (A-MPR) - Enable, if feasible, optional UE support of 7 MHz channel bandwidth from Rel-15 onwards in a release independent manner. [RAN2, RAN4] - No impact to 38.307 is expected. - Once RAN4 has introduced the new channel bandwidth, RAN2 will introduce the necessary signalling for this channel bandwidth. [RAN2] Note: This should be similar to the signalling for 35 and 45 MHz channel bandwidths. This signalling should take into account that also additional bands (not considered by this WI) may consider this new channel bandwidth in the future. RAN2 will be informed by RAN4 via LS. No additional RAN2 TUs will be requested for this objective. - Once RAN4 has introduced the new channel bandwidth, RAN3 will introduce the necessary signalling for this channel bandwidth [RAN3] Note: This should be similar to the signalling for 35 and 45 MHz channel bandwidths. This signalling should take into account that also additional bands (not considered by this WI) may consider this new channel bandwidth in the future. RAN3 will be informed by RAN4 via LS. No additional RAN3 TUs will be requested for this objective. NOTE: Once this WI is completed, the 7 MHz bandwidth can be introduced also for other bands (not addressed in this WI) via the appropriate basket WI.

Impacted existing TS/TR. TS/TR No.; Description of change; Target completion plenary#; Remarks. 37.104; RAN#109; Core part. 37.105; RAN#109; Core part. 37.113; RAN#109; Perf. part. 37.141; RAN#109; Perf. part. 37.145-1; RAN#109; Perf. part. 37.145-2; RAN#109; Perf. part. 38.101-1; NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone; RAN#109; Core part. 38.101-4; Add UE demodulation performance requirements; RAN#109; Perf. part. 38.104; NR; Base Station (BS) radio transmission and reception; RAN#109; Core part. 38.104; Add BS demodulation performance requirements if needed; RAN#109; Perf. part. 38.106; RAN#109; Core part. 38.113; RAN#109; Perf. part. 38.115-1; RAN#109; Perf part. 38.115-2; RAN#109; Perf part. 38.133; Add RRM core requirements if needed; RAN#109; Core part. 38.133; Add RRM performance requirements if needed; RAN#109; Perf. part. 38.141-1; Add BS conformance tests including RF and demodulation part; RAN#109; Perf part. 38.141-2; Add BS conformance tests including RF and demodulation part; RAN#109; Perf part. 38.174; RAN#109; Core part. 38.176-1; RAN#109; Perf part. 38.176-2; RAN#109; Perf part. 38.306; NR; User Equipment (UE) radio access capabilities; RAN#109; Core part. 38.307; Requirements on User Equipments (UEs) supporting a release-independent frequency band; RAN#109; Perf part. 38.331; NR; Radio Resource Control (RRC) protocol specification; RAN#109; Core part. 38.423; NG-RAN; Xn Application Protocol (XnAP); RAN#109; Core part.

# References

[1] 	RP-250373	Revised WID on NR FR1 7 MHz Channel Bandwidth, RAN#107

[2]		R4-2502870	WF on requirements for 7MHz channel bandwidth, RAN4#114

[3]		R4-2504589 	Draft CR to TS 37.105: 7MHz channel bandwidth introduction

[4]		R4-2504590 	Draft CR to TS 37.145-1: 7MHz channel bandwidth introduction

[5]		R4-2504591 	Draft CR to TS 37.145-2: 7MHz channel bandwidth introduction

[6]		R4-2504592 	Draft CR to TS 38.141-2: 7MHz channel bandwidth introduction