# 3GPP TSG-RAN WG4 Meeting #114bis	R4-2504594

# Wuhan, Hubei, China, 07th – 11th April, 2025

Source: 	Huawei, HiSilicon

Title: 		Further discussion on CLTA related matters

Agenda Item:	7.8.3

Document for:	Discussion

# Introduction

In this contribution, we provide further inputs on the CLTA related matters, including BS-BS isolation, intra-system TX IMD, beam steering, etc.

# Discussion

## Beam steering

When co-location requirements were derived it was mainly based on investigating column AAS systems co-located with passive BS antennas or other column AAS antennas. Some work was done investigating the coupling between multi-column antennas but at the time the single column was regarded as the worst case.

With a single column antenna there is no horizontal beam steering possible as such it was not considered. However recent results indicate that the coupling between an aggressor AAS with horizontal beam steering and a co-located victim varies with the direction of the beam.

This seems a likely result (although the exact effect is FFS), so how will this affect the co-location requirement.

The core requirement assumes that requirements apply under all operation conditions as such it seems the core covers the case where the aggressor beam is steered.

Proposal 1: All beam steering cases are covered by the core requirement as such no change is required in the core.

In the conformance requirements the transmitter configurations for the test are listed in the initial conditions, as it was not seen as important at the time the direction of the beam is not specified (it was assumed all directions would be equal). This is clearly an area which may need clarification.

Observation 1: Currently the direction of the beam during test is not specified.

Whilst it is accepted that the beam steering direction affects the level of the wanted signal coupled into a co-located antenna it is not clear that this directly affects any of the specific co-location requirements.

### Transmitter spurious emissions - co-location with other base stations

This requirement measures the power out of the CLTA in other co-located bands. It is measuring the noise/spurious power output of the DUT in the victim band. It is only the wanted signal that is beam formed the noise/spurious from the aggressor BS is unlikely to be beam formed.

Observation 2: Only the wanted signal is beam formed, noise and spurious emissions in other bands will not be beam formed.

There is an ongoing discussion about correlation and frequency shift from the wanted signal, this should be considered however in this case the requirement is very low, it can be assumed it is outside the channel filters as such correlation is very unlikely.

Observation 3: Discussion on correlation wrt frequency should be considered but is unlikely to change observation 2.

Proposal 2: Co-location Transmitter spurious emissions are not affected by DUT beam direction.

### Transmitter intermodulation

The co-location aspect of Transmitter intermodulation requirement involves how the interferer is injected into the DUT. The emission requirements are TRP and as such unaffected by the direction of the DUT beam. What we need to consider is:

- Does the direction of the DUT beam affect its IMD products?

- If the co-located system should be considered an AAS

  - If so, how is beam steering dealt with.

On the 1st point, TX IMD is generally a product of a reverse interferer from the antenna getting into the output of the non-linear PA and mixing with wanted signal from the PA. In conducted requirement the reverse interferer signal is considered to be -30dBc.

In an AAS any active beam steering will likely be before the PA as such the path between the aggressor and the PA non-linearity is fixed and not affected by any beam steering in the DUT. The coupling between the interferer antenna and the PA output is therefore fixed, each PA in the array therefore will output a constant IMD power irrespective of the wanted signal beam direction.

Observation 4: The path between the interferer and the PA output is fixed and not affected by DUT beam steering.

Potentially IMD products will be affected by the phase of the wanted signal hence the radiation pattern of the IMD products may vary depending on the wanted signal beam direction, however as the requirement is TRP this will not change the results.

Observation 5: The IMD beam pattern may be affected by DUT beam direction but as the requirement is TRP the result is not affected.

Proposal 3: TX IMD is not affected by DUT beam direction.

Secondly if the interferer is an AAS then does this affect the TX IMD?

If we consider a simple analysis assuming we have a beam formed interferer and FSPL dominate the coupling.

For sake of the simplistic approach the element gain in the direction of the DUT is -25dB, the column separation for the aggressor is 0.5λ, and frequency 5GHz and the aggressor power is 40dBm.

The maximum array gain occurs when the phase of the signals in the direction of the DUT aligns.

So, the 1st column is fed with 34dBm and is 10cm away, the FSPL is:

20*log10λ4πd=20*log103e85e9*4π*0.1=-26.4dBm

The 2nd column also fed with 34dBm and is 10cm plus half a wavelength (0.5*3e85e9=3cm) the FSPL is:

20*log10λ4πd=20*log103e85e9*4π*0.13=-28.7dBm

And so on for each of the columns.

Assuming these all add in the direction of the victim (unlikely but worst case) the total interferer is -13.9dBm.

Now if all the power had been placed in a single column next to the DUT:

The level of the interferer is -11.4dBm.

Clearly the single column is the worst case, as the number of columns increases the difference gets larger.

Figure 2-1. Power at DUT for different number of column co-located antenna

In the example it is assumed that all the signals in the direction of the DUT are in phase which is the case when a beam is formed. This in itself is unlikely but is also a worst case.

Observation 6: Using a simplistic worst-case analysis the single column co-located aggressor is a worst case radiating more power in the direction of the DUT than the equivalent horizontal array even if beam steering is assumed in the aggressor.

Proposal 4: The single column co-located aggressor is a worst case.

Taking proposal 2 and 3 we can assume the existing test scenario is a worst case. Potentially interference from a co-located AAS with active beam steering may be lower.

### Tx OFF requirements

Tx OFF requirements measure the power in a co-located victim when the transmitter is in its OFF state. By definition, as the TX is OFF there is no wanted signal and hence nothing to beamform. The output of the TX is the OFF-state noise from the PA and will be uncorrelated so no beam will form. As such beam forming is not an issue for this requirement.

Observation 7: As here is no wanted signal for the Tx OFF requirement there is no beam and hence no beam forming.

Proposal 5: Beam forming direction is not an issue for the Tx OFF requirement.

### Co-location blocking

Co-location blocking has similarities to the Tx IMD case. However, in this case rather than consider the signal coupled to the output of the PA we consider the signal coupled to the input of the RX/LNA. In this case we need to consider the beam steering of the DUT Rx and the co-located aggressor Tx.

As with the Tx, loss between the LNA and the antenna is designed to a minimum as such beam forming is generally done after the LNA (in FR1 in the BB). The LNA input therefore will have signals coupled to it via the element/sub-array which is not subject to any form of beam steering.

Observation 8: The signal coupled to the RX/LNA input is not subject to Rx beam steering.

The analysis of the co-located aggressor and the difference between a single column and an AAS horizontal array is the same as for the TX IMD case with the same conclusion, that the single column is the worst case.

Proposal 6: Co-location blocking levels coupled to the Rx are not affected by the DUT Rx beam direction and the single column co-located aggressor are the worst case. Beam steering does not affect the co-location blocking test.

## TX IMD requirement: intra-system requirement

WF in [1] has captured the following agreement on the TX IMD intra-system requirement:

## Sub-topic 1-4: TX IMD co-location Way forward: Companies are encouraged to provide inputs on the handling of the intra-system TX IMD requirements.

The discussion related to the intra-system IMD requirement was triggered by a comment asking for clarification on its applicability, when the general TX IMD requirement was proposed to be limited up to 4.9GHz frequency limit in [2].

Referring to the TS 38.141-1, transmitter intermodulation intra-system minimum requirements apply to the BS type 1-H, and are not covered in the TS 38.141-2 for BS type 1-O. As BS-BS coupling analysis as well as CLTA discussion in general is by definition related to the OTA requirements, there is simple answer to the question on the intra-system requirement handling: it shall not be addressed in this WI.

With this, it shall be clarified that previous proposals (refer to Proposal 1 in [2]: “Proposal 1: Remove TX IMD CLTA test for frequency bands above 4.9 GHz.”) shall have been clarified to be applicable to the General requirement only, for the BS type 1-O.

Proposal 7: TX IMD intra-system requirement is applicable to BS type 1-H, and therefore not affected by the discussion in this WI.

## BS-BS isolation

WF in [1] has agreed to collect more BS-BS isolation data, with additional clarifications on the applicable scenarios, distinguishing simulations results and measurements.

For now, no additional data was made available. Therefore, the following set (from the previous WF) is the current baseline subject to further inputs.

Proposal 8: Use the following data as baseline (subject to further inputs) for the BS-BS isolation conclusions derivation.

- No beam steering consideration; same band; same BS type; A-to-A (unless otherwise stated):

|  | 2.6 GHz | 3.5 GHz | 4.9 GHz | 7 GHz |
| --- | --- | --- | --- | --- |
| Isolation in vertical domain (dB) | -54 | -52 | -56 |  |
| Isolation in horizontal domain (dB) | -31 | -34 ÷ -46 | -46 |  |
| - S: simulations - M: measurements - A-to-A: array to array coupling - A-to-SA: array to sub-array coupling |  |  |  |  |

- Beam steering consideration; same band; same BS type; A-to-A (unless otherwise stated):

|  | 2.6 GHz | 3.5 GHz | 4.9 GHz | 6.7 GHz --> 7 GHz |
| --- | --- | --- | --- | --- |
| Isolation in vertical domain (dB) |  |  |  |  |
| Isolation in horizontal domain (dB) |  | A-to-SA: -55±20 dB |  | -77±25; A-to-SA: -70±20 |

## WID objectives

Referring to the WID [3], the following objective are related to the CLTA:

- Core: Investigate whether the BS/IAB OTA co-location reference antenna definition need be improved for FR1, and if feasible, update the definition. - Perf: Investigate whether the existing test approach can be improved, and if feasible, improve test methods for BS/IAB OTA co-location requirements and tests for AAS-based test specifications for FR1.

During some of the recent offline discussions, some companies were suggesting that the current Rel-19 WI shall be rather treated as a SI phase before the Rel-20 WI and related normative work for co-location requirements improvements.

At the same time, there was only one concept identified as potential candidate for the CLTA alternative test method, i.e., horn antenna. Still, there are mane unanswered questions and there is lack of confidence if the horn antenna can reach normative work within Rel-19.

If we would strictly follow WID objectives extracted above, the following can be summarized as of today:

Core: The BS/IAB OTA co-location reference antenna definition could be improved for high frequency bands of FR1, but there are no firm proposals available to potentially update the CRTA definition. CLRA remains as the default test setup for co-location requirements. There are 3 remaining RAN4 meetings for completion of Rel-19 Core work.

Perf: Following Core above, CLTA remains as the default test setup for co-location requirements. There are 6 remaining RAN4 meetings for completion of Rel-19 Perf work.

Proposal 9: Considering timeline and progress so far, refrain from further WID objectives modifications. Aim to conclude the ongoing open issues.

# Conclusions

Based on the discussion above, the following proposals were formulated:

Proposal 1: All beam steering cases are covered by the core requirement as such no change is required in the core.

Proposal 2: Co-location Transmitter spurious emissions (co-location with other base stations) are no affected by DUT beam direction.

Proposal 3: Transmitter intermodulation requirement is not affected by DUT beam direction.

Proposal 4: The single column co-located aggressor for the Transmitter intermodulation requirement is confirmed as a worst case.

Proposal 5: Beam forming direction is not an issue for the Tx OFF requirement.

Proposal 6: Co-location blocking levels coupled to the Rx are not affected by the DUT Rx beam direction and the single column co-located aggressor is confirmed as the worst case. Beam steering does not affect the co-location blocking test.

Proposal 7: TX IMD intra-system requirement is applicable to BS type 1-H, and therefore not affected by the discussion in this WI.

Proposal 8: Use the following data as baseline (subject to further inputs) for the BS-BS isolation conclusions derivation.

- No beam steering consideration; same band; same BS type; A-to-A (unless otherwise stated):

|  | 2.6 GHz | 3.5 GHz | 4.9 GHz | 7 GHz |
| --- | --- | --- | --- | --- |
| Isolation in vertical domain (dB) | -54 | -52 | -56 |  |
| Isolation in horizontal domain (dB) | -31 | -34 ÷ -46 | -46 |  |
| - S: simulations - M: measurements - A-to-A: array to array coupling - A-to-SA: array to sub-array coupling |  |  |  |  |

- Beam steering consideration; same band; same BS type; A-to-A (unless otherwise stated):

|  | 2.6 GHz | 3.5 GHz | 4.9 GHz | 6.7 GHz --> 7 GHz |
| --- | --- | --- | --- | --- |
| Isolation in vertical domain (dB) |  |  |  |  |
| Isolation in horizontal domain (dB) |  | A-to-SA: -55±20 dB |  | -77±25; A-to-SA: -70±20 |

Proposal 9: Considering timeline and progress so far, refrain from further WID objectives modifications. Aim to conclude the ongoing open issues.

# References

[1] 	R4-2502279	Way Forward for [114][303] NR_BS_RF_Part2_CLTA, RAN4#114

[2]		R4-2502135	Analysis of BS-to-BS coupling values for the TX IMD requirement optimisation, Huawei, RAN4#114

[3]		RP-243270	NR base station (BS) RF requirement evolution for FR1/FR2 and testing, WID, RAN#106