3GPP TSG-RAN-WG4 Meeting #114bis 	R4-2504612

# Wuhan, China, 7th   – 11th April, 2025

# Title:	TP for TR38.746 for adding HPUE for DL CA_n41-n66-n77 with UL CA_n41-n77

# Source:	T-Mobile USA, Nokia

# Agenda Item:	6.8.3

# Document for:	Approval

- Introduction

This contribution provides a text proposal for TR38.746[1] adding the following HPUE NRCA combinations with UL CA_n41A-n77A.

- CA_n41A-n66A-n77A

This TP is voiding clause 6.11 because it is a duplicate of 6.10

- Reference

[1] 3GPP TR38.746: “High power UE (power class 1.5 or 2) for NR inter-band Carrier Aggregation (CA) / Dual Connectivity (DC) with/without Supplementary Uplink (SUL) with high power on TDD band(s); (Release 19)”: V0.2.0

[2] 3GPP TS38.101-1: “User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone (Release 19)”: V19.0.0

[3] 3GPP TR38.718-03-01: “Rel-18 NR Inter-band Carrier Aggregation/Dual Connectivity for3 bands DL with x bands UL (x=1,2) (Release 18)”: V18.0.0

[4] 3GPP TR38.716-02-00: “NR inter-band Carrier Aggregation/Dual connectivity for 2 bands DL with x bands UL (x=1, 2) (Release 16)”: V16.0.0

# Text Proposal

# -- Start of TP –

-- Unaffected parts omitted –

## 6.10	CA_n41-n66-n77

### 6.10.1	Operating bands for CA and Configurations

Table 6.10.1-1:  CA band combination of band n25+n41+n77

| NR CA Band combination | NR Band |
| --- | --- |
| CA_n25-n41-n77 | n25, n41, n77 |

Table 6.10.1-1:  NR CA configurations and bandwidth combinations sets defined for inter-band CA (three bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier6 | NR Band | Channel bandwidth (MHz) (NOTE 3) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n41A-n66A-n77A | n417,9 n777,9 CA_n41A-n66A7,13,14 CA_n41A-n77A7,9,13,14 CA_n66A-n77A7,13,14 | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 1 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n41 | n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |

### 6.10.2	Maximum output power for inter-band CA

No changes needed to the 3Tx tables in 38.101-1. Changes for UL CA_n41A-n77A PC1.5 are captured in 6.9.2.

Table 6.10.2-1: UE Power Class for 3Tx inter-band CA with UL MIMO

| NR UL CA Configuration | Class 1.5 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n41A-n66A | 293 | +2/-3 | 262 | +2/-3 | 23 | +2/-3 |
| CA_n66A-n77A | 293 | +2/-3 | 262 | +2/-3 | 23 | +2/-3 |
| NOTE 1: (Void in this TR) NOTE 2: The UE supports PC3 in FDD band and PC3 or PC2 with UL MIMO in TDD band. NOTE 3: The UE supports PC3 in FDD band and PC1.5 with UL MIMO in TDD band. |  |  |  |  |  |  |

Table 6.10.2-2: UE Power Class for 3Tx inter-band CA with TxD

| NR UL CA Configuration | Class 1.5 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n41A-n66A | 293 | +2/-3 | 262 | +2/-3 | 23 | +2/-3 |
| CA_n66A-n77A | 293 | +2/-3 | 262 | +2/-3 | 23 | +2/-3 |
| NOTE 1: (Void in this TR) NOTE 2: The UE supports PC3 in FDD band and PC2 with Tx Diversity in TDD band. NOTE 3: The UE supports PC3 in FDD band and PC1.5 with Tx Diversity in TDD band. |  |  |  |  |  |  |

### 6.10.3	MSD scenario studies

3BDL/2BUL configurations only have MSD requirements for intermodulation.

Table 6.10.3-1 and 6.10.3-2 summarizes frequency ranges where harmonic mixing, cross band leakage, IMD interferences occur for DL CA_n41-n66-n77.

Table 6.10.3-1: Intermodulation MSD scenarios for DL CA_n41A-n66A-n77A with total power class 2

| Uplink combination | IMD | Aggressor Tx | Victim Rx | Whether 2Tx requirements exists |
| --- | --- | --- | --- | --- |
| CA_n41A-n66A | IMD3 | 2*f n41 – f n66 | n77 | Yes |
| CA_n41A-n77A | IMD4 | 2*fn77-2*fn41 | n66 | Yes |
| CA_n66A-n77A | IMD5 | 4*f n66 – f n41 | n41 | Yes (But not testable for US n77 configurations) |

Table 6.10.3-2: Intermodulation MSD scenarios for DL CA_n411A-n66A-n77A with total power class 1.5

| Uplink combination | IMD | Aggressor Tx | Victim Rx | Whether 2Tx requirements exists |
| --- | --- | --- | --- | --- |
| CA_n41A-n66A | IMD3 | 2*f n41 – f n66 | n77 | No |
| CA_n41A-n77A | IMD4 | 2*fn77-2*fn41 | n66 | Yes, but it is a copy and paste error |
| CA_n66A-n77A | IMD5 | 4*f n66 – f n41 | n41 | No |

### 6.10.4	REFSENS requirements

#### 6.10.4.1	REFSENS requirements for total power class 2

- The MSD due to intermodulation PC2 for UL CA_n41A-n66A is already specified in 38.101-1.

- The MSD due to intermodulation PC2 for UL CA_n66A-n77A is already specified in 38.101-1.

- The MSD due to intermodulation PC2 for UL CA_n66A-n77A is already specified in 38.101-1.

#### 6.10.4.2	REFSENS requirements for total power class 1.5

For UL CA_n41A-n66A PC1.5, since the n66 power remains PC3, the n41 power increases by 4.7 dB compared to UL CA_n41A-n66A PC2.

For the UL CA_n41A-n66A IMD3, since only the n41 power is changing by 4.7 dB, the interferer increases by X=2*4.7 = 9.4 dB, so the MSD increases from 25.0 dB to 34.4 dB, using the following equation:

New_MSD

For the UL CA_n66A-n77A IMD5, since only the n41 power is changing by 4.7 dB, the interferer increases by X=1*4.7 = 4.7 dB, so the MSD increases from 18.7 dB to 23.4 dB, using the equation above.

For the CA_n41A-n77A PC1.5 IMD4, the power increases by 3 dB, so the interferer increases by X=4*3 = 12 dB, so the MSD increases from 20.5 dB for PC2 to 32.5 dB, using the equation above. This MSD analysis covers UL CA_n41A-n77A PC1.5 for both 3Tx and 2Tx PC1.5

The MSD due to IMDs for PC1.5 3Tx are specified in the following table.

Table 6.10.4.2-1: 3DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC1.5 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band |  | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n41-n66-n77 | n41 |  | 2600 | 5 | 25 | 2600 | N/A | TDD | N/A |
|  | n66 | 1730 |  | 5 | 25 | 2130 | N/A | FDD | N/A |
|  | n77 | N/A |  | 10 | N/A | 3470 | 34.4 | TDD | IMD31,2 |
|  | n41 | N/A |  | 5 | N/A | 2670 | 23.4 | TDD | IMD55 |
|  | n66 | 1715 |  | 5 | 25 | 2115 | N/A | FDD | N/A |
|  | n77 | 4190 |  | 10 | 50 | 4190 | N/A | TDD | N/A |
|  | n41 | 2640 |  | 5 | 25 | 2640 | N/A | TDD | N/A |
|  | n66 | N/A |  | 5 | N/A | 2160 | 32 | FDD | IMD4 |
|  | n77 | 3720 |  | 10 | 50 | 3720 | N/A | TDD | N/A |
| NOTE 1: This band is subject to IMD5 also which MSD is not specified. NOTE 2: This band is subject to IMD4 also which MSD is not specified. NOTE 3: The requirements only apply for UEs supporting inter-band carrier aggregation with simultaneous Rx/Tx capability. Simultaneous Rx/Tx capability does not apply for UEs supporting band n78 with a n77 implementation. NOTE 4: This band is subject to IMD3 also which MSD is not specified. NOTE 5: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. NOTE 6: Both of the transmitters shall be set min(+23 dBm, PCMAX_L,f,c) as defined in clause 6.2A.4 |  |  |  |  |  |  |  |  |  |  |

## 6.11	Void

-- End of TP –