3GPP TSG-RAN WG4 Meeting # 114bis										                 R4-2504923

Wuhan, China, 07 – 11 April 2025

Agenda item:			7.28.3

Source:	Moderator (Nokia)

Title:	Ad-hoc minutes for NR_XR_Ph3

Document for:	Information

# Introduction

The following issues are proposed to be prioritized for discussion

- Issue 1-1: Deployment scenarios

- Issue 1-4: L1 measurements with scheduling restrictions and procedures

- Issue 1-5: L1/L2 triggered mobility measurements and procedures

- Issue 3-1: Principles for measurement delay calculation

- Issue 3-2: Formulas for measurement delay extension without DRX

- Issue 3-4: Conditions for measurement skipping

- Issue 3-5: Measurement gap configurations relevant to skipping

- Issue 2-1: General approach for defining requirements for measurements with gaps

- Issue 2-2: Types of measurement gaps

- Issue 5-1: General on UAI

- Issue 6-1: Semi-static schemes

- Issue 4-1: Time offset requirement for measurement gap skipping

- Issue 4-2: Time offset requirement for cancelling skip indication

- Issue 6-2: Skipping indication on frequency domain

Issues for email discussion

- Issue 7-1: Update work split

- Issue 7-2: Comments on CRs

# Agreed workplan (from R4-2414027 for information)

| RAN4 meeting | Tasks |
| --- | --- |
| RAN4 #112 - August 2024 | - Approval of workplan - Initial discussion on scenarios for measurement skipping based on RAN1 conclusions. |
| RAN4 #112bis - October2024 | - Conclusions on scenarios for measurement skipping based on RAN1 conclusions. - Discussion on the need for UE assistance information regarding measurements occasions needed. |
| RAN4 #113 - November 2024 | - Discussion on RRM requirements with measurement skipping. - Discussion on the necessary UE assistance information. |
| RAN4 #114 - February 2025 | - Conclusion on RRM requirements with measurement skipping. - Discussion on the need for UE assistance information regarding measurement skipping. - Agreement of CR work split. |
| RAN4 #114bis - April 2025 | - Conclusion on the necessary UE assistance information. - First Draft CRs for RRM core requirements. - Draft CRs for 38.133 expected. - First big Draft CR. |
| RAN4 #115 - May 2025 | - Discussion on remaining open issues for RRM core |
| RAN4 #116 - August 2025 | - Discussion on remaining open issues for RRM core - Revision of CRs for RRM core requirements. - Planning of RRM performance. - Initial discussion on RRM test cases. |

# Ad hoc minutes

#### Issue 1-1: Deployment scenarios

- Background information: In RAN4 #112 bis the following agreement regarding deployment scenarios was reached R4-2416864:

< Agreement>: Frequency ranges - Agree on FR1 and FR2-1 - Scenarios - Start the discussion for - NR-SA - Carrier aggregation

- Background information: In RAN4 #113 the following agreement regarding deployment scenarios was reached R4-2420103:

< Agreement>: - EN-DC and NE-DC for DCI-based solution - Is NOT supported for measurement gap skipping by the WID <Way forward>: - Further discussion needed for - FR2-2 - NR-DC

- Background information: In RAN4 #114 the following agreement regarding deployment scenarios was reached R4-2502700:

< Agreement>: - Deprioritize FR2-2 for enabling transmission/reception in gaps/restrictions that are caused by RRM measurements <Way forward>: - Further discussion needed for - NR-DC

- Proposals form companies

  - Proposals not supporting NR-DC

    - Proposal 1 (Meta) Deprioritize NR-DC scenarios.

    - Proposal 2 (Nokia): NR-DC is not supported for measurement gap skipping for XR enhancements.

    - Proposal 3 (MediaTek): RAN4 should de-prioritize NR-DC.

    - Proposal 4 (Samsung): Deprioritize NR-DC for enabling transmission/reception in gaps/restrictions that are caused by RRM measurements.

    - Proposal 5 (ZTE): The gap/restriction skipping and the support of NR-DC are two separate objectives in the WID and assigned to different groups. Since same challenge as EN/NE-DC, apply aligned principle to NR-DC and EN/NE-DC, i.e. not support DCI-based measurement gap skipping for NR-DC.

  - Proposals supporting NR-DC

    - Proposal 6 (Interdigital): If Inter-band NR-DC gap skipping is to be supported, then the rule of per band applicability for DCI gap skipping order is required and sufficient.

  - Proposals where other working groups are asked for decision

    - Proposal 7 (vivo): Send an LS to RAN2 and RAN1 to ask if NR-DC scenario is supported for gap cancellation from their perspective.

- Merged proposals from Moderator

  - Option 1: NR-DC is not supported

  - Option 2: NR-DC is de-prioritized

  - Option 3: Inter-band NR-DC is supported and per-band applicability for DCI skipping order is required and sufficient.

  - Option 4: Send LS to RAN1 and RAN2

- Recommended WF

  - Majority of the companies support deprioritizing or precluding NR-DC. Please check if Option 1 or Option 2 can be accepted.

- Comments from companies

  - QC: We prefer Option 2 since we don’t know if RAN2 can decide on this.

  - Vivo: ok to deprioritize

  - Apple: ok to deprioritize. Need to mention that 5 ms is not for NR-DC. Ok not to close the door for RAN2.

  - QC: One way is to specify requirements for the applicable cases. But don’t need to mention that feature is not applicable for NR-DC.

  - Nokia: Preferred to say explicitly not supported. Should align with EN-DC and NE-DC, and it should be clear that the feature is not applicable and additional statement is needed. Otherwise we keep open and it is unclear if this can be used. Majority of companies also don’t see this is an important scenario for measurement gap skipping. This can lead to additional issues that cannot be dealt with with our workplan timeline. RAN1 didn’t discuss it.

  - Samsung: Agree with not support. Not leave open to discussion. MO and scheduler would need ot be discussed if deprioritized

  - Ericsson: Need to agree together with statement of the reason. Most likely lack of support from other groups. It would be helpful to not come back to this issue. If we comme back we need to know the reason. Prefer option 1. Better ot state  RAN4 will not dedfine requirements in Rel 19 unless RAN3 support is in place.

  - Mediatek: State support or not is out of RAN4 reach. This can deprioritized, and on the applicability of the requirement we state that this doesn’t apply to NR-DC

  - Huawei: Option 2 means not have RAN4 requirements. We can discuss details on CR drafting. Maybe RAN3 is not only missing piece.

  - Interdigital: Ok with option 1. That means CRs have to state requirements applicability. And it will be clear that NR-DC is not there

  - QC: From RAN4 point of view we can support if RAN2 can support. Ok not specify requirements. We can add in the agreement that NR-DC is supported if we get input fomr other groups.

  - Ericsson Need to state applicability, but rather that the requirements apply for NR-SA

  - ZTE: In previous agreement EN-DC and NE-DC is not supported. Why no requirements for NR-DC, what is the difference?

  - Nokia: already discussed EN-DC and NE-DC, we wonder why to make special agreement for NR-DC. If NRDC is only deprioritized the workplan is hard to meet

  - Interdigital:

- Conclusion in Ad hoc session:

  - Agreement:

    - Skipping requirements applies for NR-SA

#### Issue 1-4: L1 measurements with scheduling restrictions and procedures

- Background information: In RAN4 #114 the following agreement regarding deployment scenarios was reached R4-2502700:

## Issue 1-3: L3 Measurements without gaps with scheduling restrictions ## Issue 1-4: L1 measurements with scheduling restrictions and procedures ## Issue 1-5: L1/L2 triggered mobility measurements and procedures <Agreement> - During the skipped measurement gap occasion, the UE is not required to perform measurements (i.e. L3 measurements and L1 measurements), and the UE shall not cause scheduling restrictions. - Outside a gap occasion, skipping of L3 Measurements without gaps with scheduling restrictions is not supported. - Outside a gap occasion, skipping of L1 Measurements with scheduling restrictions is not supported.

- Proposals from companies

  - Proposal 1 (Nokia): RAN4 not to define requirements for measurement gap skipping for the case of measurement gap fully overlapped with L1 measurements.

- Recommended WF

  - Please discuss Proposal 1.

- Comments from companies

  - QC: Agree on the last meeting that we don’t repurpose the gpas for other measurements. Wording confusing.

  - Huawei: In current spec there is no requirements already. Current working looks like network is not allocwe dot skip gap

  - ZTE: Similar view as QC. In legacy no requirements.

  - Mediatek: Not clear why to exclude.

  - Vivo: We don’t have this case in legacy as commented by other companies. L! measurements from LTM, with gap need ot be considered.

  - Nokia: Our understanding is about SMTC availability, in this case there is no

  - Moderator: RAN4 understands that no requirement impact discussion on L1 is needed.

- Conclusion in Ad hoc session:

  - No further discussion is needed

#### Issue 1-5: L1/L2 triggered mobility measurements and procedures

- Background information: In RAN4 #114 the following agreement regarding deployment scenarios was reached R4-2502700:

## Issue 1-3: L3 Measurements without gaps with scheduling restrictions ## Issue 1-4: L1 measurements with scheduling restrictions and procedures ## Issue 1-5: L1/L2 triggered mobility measurements and procedures <Agreement> - During the skipped measurement gap occasion, the UE is not required to perform measurements (i.e. L3 measurements and L1 measurements), and the UE shall not cause scheduling restrictions. - Outside a gap occasion, skipping of L3 Measurements without gaps with scheduling restrictions is not supported. - Outside a gap occasion, skipping of L1 Measurements with scheduling restrictions is not supported.

- Proposals from companies

  - Proposal 1 (vivo): Requirements in clause 9.15 NR inter-frequency L1 measurement are updated for gap skipping.

- Recommended WF

  - Discuss the proposal.

- Comments from companies

  - MTK: need to understand what the proposal is about. The gap does not differentiate between L3 and L1, so the UE cannot differentiate. So the proposal makes sense.

- Conclusion in Ad hoc session:

  - Agreement: Requirements in clause 9.15 NR inter-frequency L1 measurement with gaps are updated for gap skipping.

#### Issue 3-1: Principles for measurement delay calculation

- Proposals

  - Proposal 1 (Apple): as minimum requirements, measurement period is extended by number of occasions which are cancelled due to XR traffic.

  - Proposal 2 (Qualcomm): For DCI based dynamic indication, RAN4 to extend the L3 delays by adding the number of MGs deactivated during the L3 period to the number of measurement samples.

- Recommended WF

  - Discuss the proposals.

- Comments from companies

  - Discussion based on Issue 3-2

- Conclusion in Ad hoc session:

  - None

#### Issue 3-2: Formulas for measurement delay extension without DRX

- Proposals

  - Option 1 (Nokia, vivo, Interdigital): RAN4 to define measurement delay extension due to measurement skipping for inter-frequency measurements with gap as

    - Max(200 ms × CSSFinter, Ceil( 8  * Kgap) × Max(MGRP , SMTC period) × CSSFinter + Nskip × Max(MGRP , SMTC period)),

    - where Nskip is the number of  skipped MG occasions in the measurement period.

    - Use this formula as example and follow similar approach for the other cases.

  - Option 2b (MediaTek, ZTE, Ericsson, Huawei, CMCC): RAN4 to define measurement delay extension due to measurement skipping for inter-frequency measurements with gap as

    - Max(200 ms, Ceil(( 8 + Nskip ) * Kgap)  Max(MGRP , SMTC period))  CSSFinter

    - where Nskip is the number of  skipped MG occasions in the measurement period.

    - Use this formula as example and follow similar approach for the other cases.

  - Option 2a (Samsung): Take the formula: Max(200 ms, Ceil(( 8 + Nskip ) * Kgap) ´ Max(MGRP , SMTC period)) ´ CSSFinter as starting point to define the measurement delay extension due to measurement skipping for inter-frequency measurements with gap

    - FFS: The maximum number of skipped MG occasions in the measurement period to limit Nskip

    - FFS: CSSFinter

- Recommended WF

  - Please discuss based on Option 1, 2, and 2b

- Comments from companies

  - None

- Conclusion in Ad hoc session:

  - None

#### Issue 3-4: Conditions for measurement skipping

- Proposals from companies

  - Proposals for no conditions

    - Proposal 1 (Meta): no conditions are needed for measurement skipping regarding maximum number of occasions or MG configuration.

    - Proposal 2 (Nokia): No conditions are needed for measurement skipping regarding maximum number of skipped occasions per measurement period or related to MG configuration

    - Proposal 3 (ZTE): No conditions are needed for measurement skipping regarding maximum number of occasions or MG configuration.

    - Proposal 4 (Interdigital): No conditions are needed for measurement skipping regarding maximum number of occasions or MG configuration.

  - Proposals on max number of occasions

    - Proposal 5 (MediaTek): RAN4 shall discuss and define the max number of occasions cancelled during a measurement period.

    - Proposal 6 (Samsung, vivo): For measurement gap skipping, the max number of gaps skipped during the measurement period should be limited/specified

    - Proposal 7 (Apple): max number of occasions cancelled during a measurement period shall be defined.

    - Proposal 8 (Ericsson): The maximum number of extensions are

|  | PSSS/SSS detection | SSB index detection | Measurement period |
| --- | --- | --- | --- |
| No DRX or max(DRX cycle, SMTC period, MGRP)≤40 ms | FR1: 12 FR2: 1.5×Mpss/sss_sync | FR1: 5 FR2: 1.5×MSSB_index | FR1: 12 FR2: 1.5×Mmeas_period |
| DRX cycle ≤ 320 ms | FR1: 8 FR2: Mpss/sss_sync | FR1: 3 FR2: MSSB_index | FR1: 8 FR2: Mmeas_period |
| DRX cycle > 320 ms | FR1: 5 FR2: 0.5×Mpss/sss_sync | FR1: 2 FR2: 0.5× MSSB_index | FR1: 5 FR2: 0.5× Mmeas_period |

    - Upon exceeding XPSS/SSS,gaps,max the UE is not required to meet the corresponding PSS/SSS detection requirement.

    - Upon exceeding Xind,gaps,max the UE shall restart the time index detection.

    - Upon exceeding Xmeas,max the UE shall restart the measurement.

  - Proposals on maximum separation between two occasions shall be

    - Proposal 9 (Ericsson): Maximum acceptable time separation between two measurement occasions available for RRM measurements may need to be defined, e.g.:

      - Any two closest SMTC occasions available at the UE for the measurement shall be separated by no more than the maximum time requirement for the cell to remain known.

- Proposals merged by the Moderator

  - Option 1: No conditions are defined

  - Option 2: Define the maximum number of gaps skipped during the measurement period.

  - Option 3: Define a maximum separation between two non-skipped gaps for measurement

- Recommended WF

  - Discussion on the conditions is needed. If the group agrees that conditions are necessary, we can further discuss the limits taking Proposal 8 as an example.

- Comments from companies

  - Not discussed

- Conclusion in Ad hoc session:

  - None

#### Issue 3-5: Measurement gap configurations relevant to skipping

- Options

  - Option 1 (Ericsson): RAN4 to discuss MG configurations relevant for MG cancellation, e.g.,

    - minimum MGL, e.g., MGL≥3 ms,

    - maximum MGRP, e.g., MGRP≤320 ms to avoid too long delays.

- Recommended WF

  - Please discuss if there is need for limiting which MG configurations can be skipped.

#### Issue 2-1: General approach for defining requirements for measurements with gaps

- Proposals

  - Proposal 1 (MediaTek): RAN4 shall identify what issues should be addressed to enable gap skipping to other measurement gap types, which are Type-2, NCSG, Con-current GAP, and Pre-MG.

- Recommended WF

  - Please discuss the proposal. We can use the open points for the discussion on the next issue 2-2.

- Comments from companies

  - Discussed based on Issue 2-2.

- Conclusion in Ad hoc session:

#### Issue 2-2: Types of measurement gaps

- In this issue we discuss the types of measurement gaps to be considered in this WID. In order to help the discussion, we reuse the definition of Type-1 and Type-2 measurement gaps as agreed in Rel-18 NR_MG-enh2:

- Type-1 MG: Gap(s) configured via GapConfig without suffix - Type-2 MG: Gap(s) configured via GapConfig-r17 without preConfigInd-r17 or ncsgInd-r17 - Rel-18 Concurrent Gap: gap with assigned priority which can be also NCSG or Pre-MG

- Company proposals on the priorities for RAN4 to work are summarized using based on the following table

| Gap types | First priority | Second priority | Shall not be cancelled |
| --- | --- | --- | --- |
| Type 1 | Moderator: Type 1 is omitted since it is already agreed |  |  |
| Type 2 | CMCC, Apple | vivo (one MG only, per FR or per-UE) | Meta, Nokia |
| NCSG | CMCC, Apple | Nokia, Ericsson | Meta, vivo |
| MUSIM |  | Apple | Meta, Nokia, Ericsson, vivo |
| Positioning |  |  | Meta, Nokia, Ericsson, Apple, vivo |
| Rel 18 concurrent gap | CMCC, Apple |  | Meta, Nokia, vivo |
| PreConfigured | CMCC, Apple | Ericsson, vivo? | Meta, Nokia |

- Recommended WF

  - Recommended to further progress on other issues before coming back to gap types.

- Comments from companies

  - Moderator: Discuss for each gap type considering Proposal 1 in Issue 2-1

    - Proposal 1 (MediaTek): RAN4 shall identify what issues should be addressed to enable gap skipping to other measurement gap types, which are Type-2, NCSG, Con-current GAP, and Pre-MG.

  - Moderator: Start discussion with MUSIM or positioning.

  - Huawei: We didn’t way either column because there was no technical reason for excluding certain types of gaps

  - Vivo: We already have agreement for baseline type 1. For the others discussion needed. For MUSIM, this is for another network, if this is cancelled, the UE needs more coordination for another SIM. We don’t see that this feature combination is likely.

  - Nokia: Prefer to exclude MUSIM and positioning. As there might be different demands. Should be excluded. If the UE is configured with those, they are not subject to skipping

  - QC: Can preclude MUSIM and positioning. Even if gNB knows the configuration, MUSIM is for connectivity for other network. To maintain the connectivity to other network this would be critical.

  - Ericsson: Both scenarios are not typical. Procedure for positioning is also very different. Configurations come form core network. More work would be needed. Positioning can be also used for emergency. We don’t have positioning requirements for DRX.

  - Moderator: Comments about Type 2 Rel 17 Concurrent gap.

    - In favour of requirements Apple and CMCC, QC, Mediatek, Huawei

    - Not in favour of supporting: Nokia, vivo

  - Vivo: we support Type 2 only if there is no concurrent gaps

  - Nokia: Type 2 means gaps are used in concurrent mode. Implicitly define all the requirements for concurrent gaps. Not primary use case for XR

  - Apple: The proposal is both gNB and UE know clearly the final pattern after gap dropping rules.

  - QC: We already have rules for resolving concurrency. Once we receive the DCI skip, it applies for the gap that survived the dropping rules.

  - MTK: Similar to Apple and QC. Afte the calculation of dropping rules, we can apply DCI skipping.

  - Moderator:

  - Ericsson: Possible solution is that the companies bring the solutions on the next meetings. If not agreeable in the next meeting, the gap type is not part of Rel 19

- Conclusion in Ad hoc session:

  - Agreement: Gaps which shall not be cancelled

    - MUSIM

    - Positioning

  - Tentative agreement for other gap types:

    - The companies bring the issues and solutions on the RAN4#115. If the solution is not agreeable in RAN4#115, the gap type is not part of Rel-19

#### Issue 5-1: General on UAI

- Agreement from RAN4 #114

Issue 5-1: General on UAI <Agreement>: - Agree to define UAI based on the following assumptions, and further discuss the content of UAI - RAN4 will not specify any explicit or implicit requirement or expectation on gNB behaviour in response to any XR UAI (from RAN4 perspective) - Not define UE behaviour and the corresponding requirements based on UAI - Supporting of UAI is optional for UE with optional UE capability. <Way forward>: - Proposals related to measurement occasions - Option 1: Do not define UAI related to measurement occasions. - Option 2: The UE indicates a pattern of unused gaps - Option 3: The UE indicates a ratio/number of occasions that can be skipped without measurement delay extension - Option 4: The UE indicates NW a status whether it can tolerate measurement cancellation. - Option 5: One of options 2, 3, or 4 but the information is reported as UE capability. - Proposals related to channel conditions - Option 1: Provide information on channel conditions - Option 2: do not provide information on channel conditions

- Proposals from the companies

  - Proposals not supporting defining UAI

    - Proposal 1 (Interdigital): Do not define UAI related to measurement occasions.

    - Proposal 2 (Interdigital): Do not provide information on channel conditions

    - Proposal 3 (Ericsson): The information related to the maximum number of measurement gap occasions allowed for skipping/reallocated to XR over a time period (mentioned in RAN1 LS on UAI) or the maximum time between any consecutive samples can be defined in TS 38.133, without the need to signal it in UAI.

    - Proposal 4 (Ericsson): No time-domain patterns are provided in UAI, e.g., because of scheduling impact and the potential ambiguity in patterns indicated by UEs and their interpretation by the network, when the UE is configured with DRX, since the measurement occasions and RAN4 measurement period depend on the DRX cycle, and the UE is not required to perform measurements more frequently than once per DRX cycle, which should also be followed for XR.

  - Proposals on measurement occasions

    - Proposal 5 (Meta): Regarding UAI, support Option 5 in the WF.

    - Proposal 6 (Nokia): Define UAI reporting where the UE indicates a ratio of occasions that can be skipped without measurement delay extension per measurement object.

    - Proposal 7 (ZTE): Based on the guidance of UAI approved in last meeting, the following types of UAI are helpful to NW decision:

      - Option 2: The UE indicates a pattern of unused gaps

      - Option 3: The UE indicates a ratio/number of occasions that can be skipped without measurement delay extension

      - Option 4: The UE indicates NW a status whether it can tolerate measurement cancellation.

    - Proposal 8 (vivo): The patterns of gaps occasions not used for measurement within a time period should be provided as UE assistance information.

    - Proposal 9 (vivo): The maximum interval between two measurement occasions available for RRM measurements can be defined as UAI.

    - Proposal 10 (vivo): RAN4 to further study how UE assistance information/UE capability is reported when DRX is configured.

    - Proposal 11 (Huawei): Introduce UAI on number of MG occasions that can be skipped within measure period without measurement delay extension.

      - Note: It applies to both DCI-based solution and semi-static solution if introduced.

    - Proposal 12 (Huawei): Introduce UAI maximum interval between available measurement occasions for delay extension. UE is allowed to restart measurement upon exceeding the interval instead of only extending the delay.

    - Proposal 13 (CMCC): it is proposed that the content of UE assistance information could be the number and/or location of measurement gap occasions not used for measurement.

    - Proposal 14 (Apple): support option 4 for simplicity. Open for option 2 and 3.

  - Proposals regarding UE capabilities

    - Proposal 15 (vivo): The information about gaps not used for measurement are reported as UE capability rather than UE assistance information.

  - Other proposals

    - Proposal 16 (vivo): Send an LS to RAN2 and RAN1 on RAN4 agreements about UE assistance information.

- Proposals merged by the Moderator

  - Options related to measurement occasions

    - Option 1: Do not define UAI related to measurement occasions.

    - Option 2: The UE indicates a pattern of unused gaps

    - Option 3: The UE indicates a ratio/number of occasions that can be skipped without measurement delay extension

      - Option 3a: The UE indicates a ratio/number of occasions that can be skipped per measurement object

    - Option 4: The UE indicates NW a status whether it can tolerate measurement cancellation.

    - Option 5: One of options 2, 3, or 4 but the information is reported as UE capability.

    - Option 6: The UE indicates the maximum interval between two available measurement occasions

    - Option 7: The information about gaps not used for measurement is reported as UE capability rather than UE assistance information.

  - Proposals related to channel conditions

    - Option 1: Provide information on channel conditions

    - Option 2: do not provide information on channel conditions

- Recommended WF

  - Discussion based on the options merged by the Moderator.

    - If we agree on UAI reporting, please consider also Proposal 10 on the relationship to DRX configuration and Proposal 16 on sending an LS to RAN2 and RAN1.

  - Moderator: Discuss first on channel conditions

  - Huawei: There is already existing tools that the network can use for this. So we need to focus on information that is not available already.

  - Nokia: Could be good to study a bit more. We know that measurement tasks are depending on this. It is known by UE but not network.

  - vivo: Related to SCell measure. This is up to UE implementation. Not sure if all UEs stops measurements based on this threshold. Othe rpoint is that this is about coverage. If UE towards cell edge, than RSRP becomes worst and the UE starts measurement of neighbours. This can be very dynamic reporting.

  - CMCC: According to RAN1 LS< RAN4 is only asked only about measurement occasion. So RAN4 is not asked bout this topic.

  - Moderator: Focus on Options related to measurement occasions

    - Option 1: Do not define UAI related to measurement occasions.

    - Option 2 (vivo): The UE indicates a pattern of unused gaps

    - Option 3 (Huawei, vivo, QC, Apple): The UE indicates a ratio/number of occasions that can be skipped

      - Option 3a (Nokia) : The UE indicates a ratio/number of occasions that can be skipped per measurement object

      - Option 3b (,) : The UE indicates a ratio/number of occasions that can be skipped per measurement gap

    - Option 4: The UE indicates NW a status whether it can tolerate measurement cancellation.

    - Option 5: One of options 2, 3, or 4 but the information is reported as UE capability.

    - Option 6 (QC): The UE indicates the maximum interval between two available measurement occasions

    - Option 7: The information about gaps not used for measurement is reported as UE capability rather than UE assistance information.

  - Huawei: Our current preference is compromise Option 3 or 3a. Based on different carriers. From network point of view I have the confidence to use that feature.

  - QC: Option 3 is the best among those options. It gives network an idea on system level what is the preference for most of the UEs, and better decision on how many gaps can be cancelled. We can also consider Option 6

  - Vivo: We support option 2. This guarantees the mobility performance. For option 3 we are ok.

  - Ericsson: need to understand the impact on BS. How does this impact the agreement from last meeting?

  - Mediatek: This doesn’t impact the requirements. The requirements still apply if the ratio is exceeded.

  - CMCC: We support option 3. We think that there is no impact on BS behaviour.

  - Ericsson: how does this number can be different than the maximum number of occasions in the requirements

  - Interdigital: Based on the assumptions agreed, we propose not to have UAI, but since companies are concerned on how many gaps can be skipped can be fixed in the spec specifying that maximum number of consecutive skipped samples is no longer than X.

  - Huawei: The compromise solution was that UAI is not impacting delay and BS expectation. In real deployment the UE can have some tolerance. We can work on wording

  - Interdigital: details on the report are still missing.

- Comments from companies

- Conclusion in Ad hoc session:

  - Agreement:

    - Do not provide information on channel conditions

  - Options for further discussion:

    - Option 1: The UE indicates a maximum ratio/number of occasions that can be cancelled

    - Option 2: The UE indicates the expected number of unused gaps without measurement gap skipping in legacy requirements

#### Issue 6-1: Semi-static schemes

- Proposals from companies

  - General proposals

    - Proposal (Huawei) RAN4 to conclude whether to introduce semi-static indication by RAN4#114bis.

  - Proposals not supporting semi-static schemes

    - Proposal 2 (Nokia): Follow RAN1 working assumption and consider DCI only.

    - Proposal 4 (Samsung): Do not consider semi-static solutions.

    - Proposal 8 (Ericsson): RAN4 does not need to further discuss other options for MG cancellation control such as semi-static schemes.

    - Proposal 13 (Interdigital): Do not consider semi-static solutions

  - Proposals supporting semi-static schemes

    - Proposal 1 (Qualcomm): In addition to the dynamic indication scheme to deactivate MGs as agreed by RAN1, support semi-static schemes such as RRC and MAC-CE based signaling as well.

    - Proposal 3 (MediaTek): RAN4 also suggest RAN1/2 to introduce the semi-static approach that NW indicate which occasions to be skipped using RRC and/or MAC-CE, thus UE can have a stable measurement pattern/period.

    - Proposal 5 (vivo): RAN4 agrees to introduce RRC signaling indication for gap skipping and restriction skipping if supported.

    - Proposal 6 (vivo): RAN4 agrees that for DCI-based triggering scheme, the gaps to be skipped during a time period is indicated by RRC in advance.

    - Proposal 7 (vivo): RAN4 LS to RAN1 and RAN2 the agreements on introduction of RRC-based triggering scheme and early indication for DCI-based triggering scheme, and additionally on only gap skipping is considered in the WI.

    - Proposal 10 (Huawei): RAN4 to consider following approach to move forward:

      - RAN4 to introduce semi-static indication to skip measurement gap.

      - RAN4 discuss whether UE is allowed to only support semi-static indication.

    - Proposal 11 (CMCC): for solutions to enable transmission/reception in gaps/restrictions, it is proposed to introduce semi-static schemes.

    - Proposal 12 (Apple): In addition to DCI-based MG skipping control, RRC-based MG skipping control should also be supported, especially for DC scenario.

- Proposals merged by the Moderator

  - Further discussion based on recommended options

    - Option 1: Do not consider semi-static solutions

    - Option 2: RAN4 to further evaluate semi-static skipping indication in addition to DCI-based solution.

      - Option 2a: RRC pattern-based

      - Option 2b: MAC-CE bitmap for skipping indication

      - Option 2c: RRC configuration patterns with MAC-CE based activation

    - Option 3: Network indicates by RRC which gaps can be skipped via DCI.

      - Option 3a: UE is allowed to only support semi-static indication.

- Recommended WF

  - Discuss the proposals.

- Comments from companies

  - Not discussed

- Conclusion in Ad hoc session:

  - None

#### Issue 4-1: Time offset requirement for measurement gap skipping

- Background information: Agreement from RAN4 #113 (R4-2420103)

## Issue 4-1: Time offset requirement for measurement gap skipping <Agreement>: - Baseline feature 5 ms - Optional capability 3 ms, FFS if any condition is needed

- Proposals

  - Option 1 (Huawei): 3 ms optional capability only apply when UE is only configured with NR measurement.

  - Option 2 (Nokia, ZTE, CMCC): No need to define additional condition for the optional capability of 3 ms

- Recommended WF

  - Discuss the options.

- Comments from companies

  - Not discussed

- Conclusion in Ad hoc session:

  - None

#### Issue 4-2: Time offset requirement for cancelling skip indication

- Background information: RAN1 agreement

A component of an FG can have Option 1 as a first value and Option 2 as a second value: - Option 1: For explicit indication by DCI, bit value equal to “0” means UE ignores the indication of this field in the DCI. - Option 2: For explicit indication by DCI, bit value equal to “0” means UE behaviour for the corresponding gap/restriction occasion is as per legacy behaviour. Note: UE indicates only one value of this component.

- Proposals

  - Proposal 1 (Qualcomm, Nokia): The support of Option 2 as agreed by RAN1 is conditioned upon the arrival of such DCI at least X ms before the MG occasion, where

    - X=5ms as baseline and X=3ms as optional UE capability.

    - [RAN1 agreement] Option 2: For explicit indication by DCI, bit value equal to “0” means UE behaviour for the corresponding gap/restriction occasion is as per legacy behaviour.

- Recommended WF

  - Please consider if Proposal 1 can be agreed.

- Comments from companies

  - Not discussed

- Conclusion in Ad hoc session:

  - None

#### Issue 6-2: Skipping indication on frequency domain

- Proposals

  - Option 1: Follow RAN1 agreements

    - Option 1a (Nokia): Capture in RAN4 specifications following RAN1 agreement:

      - It is RAN1 understanding that if measurement gap/restriction is indicated to be skipped by explicit indication by DCI, the TX/RX is enabled on all the serving cells in the applicable range of the skipped measurement gap/restriction.

      - Note: Applicable range of measurement gap/restriction is based on RAN4 specifications

    - Option 1b (vivo): RAN4 doesn’t need to discuss the gap skipping in frequency domain as it is already clear based on RAN1 agreements.

  - Option 2: Further RAN4 discussion needed

    - Option 2a (Qualcomm) If the UE indicates the MG to be per-FR, the MG skipping indication is only valid for the gap instance in the same FR with scheduled XR transmission, otherwise, the indication applies to all the carriers.

    - Option 2b (MediaTek): When a UE is configured with Per-FR gap, the MG skipping applies only to the FR associated with that Per-FR gap, otherwise, when Per-UR gap is configured, the MG skipping applies to both FRs.

    - Option 2c (ZTE): For the gap/restriction skipping impact in frequency domain,

      - For the gap skipping, depend on whether the UE capable of per-FR gap

        - If capable of per-FR gap, the skipping is only valid for the gap instance in the same FR with scheduled XR transmission.

        - Otherwise, no need to distinguish different FR.

      - For the scheduling restriction skipping, reuse the principle defined in existing scheduling restriction to decide whether the intra-band/inter-band/same FR MO(s) would be skipped.

    - Option 2d (Huawei, Interdigital): Depending on whether UE is configured with per-FR gap, if UE is configured with per-FR gap, the MG skipping only applies to the same FR and has no impacts on UE behavior on the other FR. If UE is configured with per-UE gap, the MG skipping applies to both FR.

- Recommended WF

  - Issue has been agreed in RAN1. Please discuss if Options 1/1a/1b are ok or if further discussion is needed.

- Comments from companies

  - Not discussed

- Conclusion in Ad hoc session:

  - None

#### Issue 7-1: Update work split

- Background information: In RAN4 #114 the following worksplit and list of affected clauses was agreed R4-2502700:

| Clause | Topic | Volunteer |
| --- | --- | --- |
| 8.xr (new) Or 9.1 | Time offset for DCI based measurement skipping | Nokia |
| 9.2.6 | Intra-freq measurements with gaps | ZTE |
| 9.3.4 | Inter-freq measurements with gaps | Ericsson |
| 9.4.2 | Inter-RAT measurements E-UTRAN FDD measurements | Huawei |
| 9.4.3 | Inter-RAT measurements E-UTRAN TDD measurements | Apple |
| NOTE: The full list of clauses with potential impact due to XR skipping is found in the Ad-hoc minutes R4-2502580 |  |  |

.

- Recommended WF

  - Check if L1 impact is foreseen based on agreement for Issue 1-4.

  - Check if impact on TS 38.133, clause 9.15 can be agreed based on agreement for Issue 1-5.

- Comments from companies

  - Not discussed

- Conclusion in Ad hoc session:

  - None

- Moderator compromise suggestion

  - Include in the work split

    - 9.15.5 Inter-frequency SSB based L1-RSRP Reporting, vivo

#### Issue 7-2: Comments on CRs

- List of CRs for commenting

  - R4-2503921, Ericsson, draftCR38133 Inter-frequency measurement requirements with XR

  - R4-2503575, Apple, Inter-RAT measurements E-UTRAN TDD measurements

  - R4-2504029, vivo, Draft CR on NR inter-frequency L1 measurement for XR

  - R4-2504257, ZTE Corporation, Sanechips, Draft CR on measurement gap skipping in intra-frequency measurement with gaps for R19 XR

  - R4-2504509, Nokia, Draft CR38.133 Introduction of measurement gap cancellation

- Recommended WF

  - Collect comments

  - To be updated based on agreements during RAN4 #114bis

- Comments from companies by email

| T-doc number | Company comments |
| --- | --- |
| R4-2503921 Ericsson | draftCR38133 Inter-frequency measurement requirements with XR <Nokia> - Change 1 overlaps with changes on R4-2504509. We suggest to keep the discussion as part of R4-2504509. - Change 2 - The formulas related to measurement skipping pend decision on Issues 3-2, 3-3, and 3-4. - The conditions on max skipping are pending decision on Issue 3-5. - The Notes limiting requirements for PC3 are pending decision on Issue 1-2. - We propose no to use wording like “XR operation”. - For text related to not agreed Issues we propose that the text (formulas, notes, conditions) are removed and editors note is included. |
| R4-2503575 Apple | Inter-RAT measurements E-UTRAN TDD measurements <Nokia> Clause 9.4.3.2 – The conditions on max skipping are pending decision on Issue 3-5. We propose that the text with conditions is removed and editors note is included <Ericsson> In the last meeting we agreed to follow the NR-U approach. The number of skipped occasions should be a separate parameter and not mixed into the Kgap factor. |
| R4-2504029 Vivo | Draft CR on NR inter-frequency L1 measurement for XR <Nokia> Proposed change is not part of the original work split. Pending discussion from Issue 1-5. The conditions on max skipping are pending decision on Issue 3-5. We propose that the text with conditions is removed and editor’s note is included <Ericsson> The CR is not needed |
| R4-2504257 ZTE Corporation, Sanechips | Draft CR on measurement gap skipping in intra-frequency measurement with gaps for R19 XR <Nokia> The formulas related to measurement skipping pend decision on Issues 3-2, 3-3, and 3-4. For text related to not agreed Issues we propose that the text (formulas, notes, conditions) are removed and editor’s note is included. <Ericsson> Table titles should indicate the XR feature; not all UE power classes are relevant; K-factor is not relevant and should not be included in the new tables Maximum numbers for skipped occasions should also be defined. The maximum time separation between any closest available MG occasions needs to be added. |
| R4-2504509 Nokia | Draft CR38.133 Introduction of measurement gap cancellation <Ericsson> 8.x is not needed - can be stated as an applicability condition simply in 9.1.y. Also, we normally do not write to what scenarios the requirements do not apply but rather the opposite - for what the requirements apply (this relates to NR SA, SRVCC, etc. – it would be sufficient to state the sections for the relevant measurements). Reference to RAN1 spec is needed. |

- Moderator suggestion:

  - All CRs are postponed, and the companies are encouraged to use consider the comments when drafting the CRs for RAN4 #115.