3GPP TSG-RAN WG4 Meeting#114bis	R4-2505131

# Wuhan, China, 07 – 11 April 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-2 | CR | - | rev | - | Current version: | 15.26.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draft CR to 38.101-2 EESS (Rel-15) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon, China Unicom, China Southern Power Grid, Ericsson, NTT DOCOMO, INC, Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_mmWave_protect-Core |  |  |  |  |  | Date: |  |  | 2025-04-08 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In RAN4#114, the draftCR R4-2502851 was endorsed and CR R4-2501408 was agreed to capture the UE RF requirements for Rel-19 EESS, introducing NS_205/CA_NS_205 and updating NS_202/CA_NS_202 with additional spectrum emission -5dBm/200MHz, and the corresponding A-MPR values. In RAN4#113, it is mandatory to report modifiedMPRbehaviour bit for NS_205 and the updated NS_202 from Rel-15 for new UEs. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add bits for A-MPR requirements for NS_205 and updated NS_202 in Table H.1-1 [There may be some misunderstandings among companies, and would be refreshed in the next meeting.] |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The A-MPR requirements corresponding to NS_205 and updated NS_202 cannot be supported in Rel-15. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex H |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-2 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< start of changes >

3.2	Symbols

For the purposes of the present document, the following symbols apply:

∆EIRPBC	The beam correspondence tolerance, where ∆EIRPBC = EIRP2 – EIRP1

ΔFGlobal	Granularity of the global frequency raster

ΔFRaster	Band dependent channel raster granularity

ΔfOOB	Δ Frequency of Out Of Band emission

ΔRB	The starting frequency offset between the allocated RB and the measured non-allocated RB

ΔRIB	Allowed reference sensitivity relaxation due to support for inter-band CA operation

ΔRIBC	Allowed reference sensitivity relaxation due to support for intra-band contiguous CA operation

ΔRIBNC	Allowed reference sensitivity relaxation due to support for intra-band non-contiguous CA operation

ΔMBP,n	Allowed relaxation to minimum peak EIRP and reference sensitivity due to support for multi-band operation, per supported band in a combination.

ΔMBS,n	Allowed relaxation to EIRP spherical coverage and EIS spherical coverage due to support for multi-band operation, per supported band in a combination

∑MBP	Total allowed relaxation to minimum peak EIRP and reference sensitivity due to support for multi-band operation, for all supported bands in a combination

∑MBS	Total allowed relaxation to each, EIRP spherical coverage and EIS spherical coverage due to support for multi-band operation, for all supported bands in a combination

BWChannel	Channel bandwidth

BWChannel_CA	Aggregated channel bandwidth, expressed in MHz

BWGB	max( BWGB,Channel(k) )

BWGB,Channel(k)	Minimum guard band defined in clause 5.3A.2 of carrier k

BWinterferer	Bandwidth of the interferer

BWintraCA	Aggregated channel bandwidth for intraband CA with or without contiguous RBs allocation, expressed in MHz. BWintraCA = BWChannel_CA in case of contiguous RBs allocation.  BWintraCA is the frequency separation between the lower edge of the lowest allocated RB and the higher edge of the highest allocated RB in case of non-contiguous RBs allocation.

Ceil(x)	Rounding upwards; ceil(x) is the smallest integer such that ceil(x) ≥ x

EIRP1	The measured total EIRP based on the beam the UE chooses autonomously (corresponding beam) to transmit in the direction of the incoming DL signal, which is based on beam correspondence without relying on UL beam sweeping

EIRP2	The measured total EIRP based on the beam yielding highest EIRP in a given direction, which is based on beam correspondence with relying on UL beam sweeping

EIRPmax	The applicable maximum EIRP as specified in clause 6.2.1

Floor(x)	Rounding downwards; floor(x) is the greatest integer such that floor(x) ≤ x

F_center	The center frequency of an allocated block of PRBs

FC	Center frequency of a carrier for a numerology defined by the RF reference frequency on the channel raster mapped to the carrier according to sub-clause 5.4.2.2

FC,block, high	Fc of the highest transmitted/received carrier in a sub-block.

FC,block, low	Fc of the lowest transmitted/received carrier in a sub-block.

FC, low	The Fc of the lowest carrier, expressed in MHz.

FC, high	The Fc of the highest carrier, expressed in MHz.

FDL_low	The lowest frequency of the downlink operating band

FDL_high	The highest frequency of the downlink operating band

Fedge,block,low	The lower sub-block edge, where Fedge,block,low = FC,block,low - Foffset, low.

Fedge,block,high	The upper sub-block edge, where Fedge,block,high = FC,block,high + Foffset, high.

Fedge, low	The lower edge of Aggregated Channel Bandwidth, expressed in MHz. Fedge, low = FC, low - Foffset, low.

Fedge, high	The upper edge of Aggregated Channel Bandwidth, expressed in MHz. Fedge, high = FC, high + Foffset, high.

FInterferer	Frequency of the interferer

FInterferer (offset)	Frequency offset of the interferer (between the center frequency of the interferer and the carrier frequency of the carrier measured)

FIoffset	Frequency offset of the interferer (between the center frequency of the interferer and the closest edge of the carrier measured)

Floor(x)	Rounding downwards; floor(x) is the greatest integer such that floor(x) ≤ x

Foffset, low	Frequency offset from FC, low to the lower UE RF Bandwidth edge, or from FC,block, low to the lower sub-block edge

Foffset, high	Frequency offset from FC, high to the upper UE RF Bandwidth edge, or from FC,block, high to the upper sub-block edge

FOOB	The boundary between the NR out of band emission and spurious emission domains

FREF	RF reference frequency

FREF-Offs	Offset used for calculating FREF

FUL_low	The lowest frequency of the uplink operating band

FUL_high	The highest frequency of the uplink operating band

FUL_Meas	The sub-carrier frequency for which the equalizer coefficient is evaluated

GBChannel	Minimum guard band defined in clause 5.3.3, expressed in kHz

LCRB	Transmission bandwidth which represents the length of a contiguous resource block allocation expressed in units of resources blocks

LCRB,Max	Maximum number of RB for a given Channel bandwidth and sub-carrier spacing

Max()	The largest of given numbers

Min()	The smallest of given numbers

MPRf,c	Maximum output power reduction for carrier f of serving cell c

MPRnarrow	Maximum output power reduction due to narrow PRB allocation

MPRWT	Maximum power reduction due to modulation orders, transmit bandwidth configurations, waveform types

nPRB	Physical resource block number

NRACLR	NR ACLR

NRB	Transmission bandwidth configuration, expressed in units of resource blocks

NRB,low	Transmission bandwidth configurations according to Table 5.3.2-1 for the lowest assigned component carrier in clause 5.3A.1

NRB,high	Transmission bandwidth configurations according to Table 5.3.2-1 for the highest assigned component carrier in clause 5.3A.1

NREF	NR Absolute Radio Frequency Channel Number (NR-ARFCN)

NREF-Offs	Offset used for calculating NREF

PCMAX	The configured maximum UE output power

PCMAX, f, c	The configured maximum UE output power for carrier f of serving cell c

Pint	The intermediate power point as defined in table 6.3.4.2-2

PInterferer	Modulated mean power of the interferer

Pmax	The maximum UE output power as specified in clause 6.2.1

Pmin	The minimum UE output power as specified in clause 6.3.1

P-MPRf,c	The Power Management UE Maximum Power Reduction for carrier f of serving cell c

PPowerClass	Nominal UE power class (i.e., no tolerance) as specified in clause 6.2.1

PRB	The transmitted power per allocated RB, measured in dBm

PTMAX,f,c	The measured total radiated power for carrier f of serving cell c

PUMAX	The measured configured maximum UE output power

Pw	Power of a wanted DL signal

RBstart	Indicates the lowest RB index of transmitted resource blocks

SCSlow	SCS for the lowest assigned component carrier in clause 5.3A.1, expressed in kHz

SCShigh	SCS for the highest assigned component carrier in clause 5.3A.1, expressed in kHz

SSREF	SS block reference frequency position

T(∆P)	The tolerance T(∆P) for applicable values of ∆P (values in dB)

TRPmax	The maximum TRP for the UE power class as specified in clause 6.2.1

< Next changes >

6.2.3	UE maximum output power with additional requirements

6.2.3.1	General

Additional emission requirements can be signalled by the network. Each additional emission requirement is associated with a unique network signalling (NS) value indicated in RRC signalling by an NR frequency band number of the applicable operating band and an associated value in the field additionalSpectrumEmission. Throughout this specification, the notion of indication or signalling of an NS value refers to the corresponding indication of an NR frequency band number of the applicable operating band (the IE field freqBandIndicatorNR) and an associated value of additionalSpectrumEmission in the relevant RRC information elements.

To meet these additional requirements, additional maximum power reduction (A-MPR) is allowed for the maximum output power as specified in clause 6.2.1. Unless stated otherwise, an A-MPR of 0 dB shall be used.

Table 6.2.3.1-1 specifies the additional requirements with their associated network signalling values and the allowed A-MPR and applicable operating band(s) for each NS value. The mapping of NR frequency band numbers and values of the additionalSpectrumEmission to network signalling labels is specified in Table 6.2.3.1-2. Unless otherwise stated, the allowed total back off is maximum of A-MPR and MPR specified in clause 6.2.2.

Table 6.2.3.1-1: Additional maximum power reduction (A-MPR)

| Network Signalling label | Requirements (clause) | NR Band | Channel bandwidth (MHz) | Resources Blocks (NRB) | A-MPR (dB) |
| --- | --- | --- | --- | --- | --- |
| NS_200 |  |  |  |  | N/A |
| NS_201 (NOTE 1) | 6.5.3.2.2 | n258 |  |  | 6.2.3.2 |
| NS_202 | 6.5.3.2.3 | n257, n258 | 50, 100, 200, 400 | Table 5.3.2-1 | 6.2.3.3 |
| NS_203 | 6.5.3.2.4 | n258 | 50, 100, 200, 400 | Table 5.3.2-1 | 6.2.3.4 |
| NS_205 | 6.5.3.2.6 | n257, n258 | 50, 100, 200, 400 | Table 5.3.2-1 | 6.2.3.6 |
| NOTE 1: NS_201 is obsolete, the associated additional spurious emission requirements are not applicable. |  |  |  |  |  |

Table 6.2.3.1-2: Mapping of Network Signalling label

| NR Band | Value of additionalSpectrumEmission (NOTE 1) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| n257 | NS_200 | NS_202 | NS_205 |  |  |  |  |  |
| n258 | NS_200 | NS_201 (NOTE 2) | NS_202 | NS_203 | NS_205 |  |  |  |
| n259 | NS_200 |  |  |  |  |  |  |  |
| n260 | NS_200 |  |  |  |  |  |  |  |
| n261 | NS_200 |  |  |  |  |  |  |  |
| NOTE 1: additionalSpectrumEmission corresponds to an information element of the same name defined in sub-clause 6.3.2 of TS 38.331 [13]. NOTE 2: NS_201 is obsolete, the associated additional spurious emission requirements are not applicable. |  |  |  |  |  |  |  |  |

6.2.3.2	Void

6.2.3.2.1	Void

Table 6.2.3.2.1-1: (Void)

6.2.3.2.2	Void

Table 6.2.3.2.2-1: (Void)

6.2.3.2.3	Void

Table 6.2.3.2.3-1: (Void)

6.2.3.2.4	Void

6.2.3.3	A-MPR for NS_202

6.2.3.3.1	A-MPR for NS_202 for power class 1

For power class 1, A-MPR for NS_202 shall be 11.0 dB.

6.2.3.3.2	A-MPR for NS_202 for power class 2

For power class 2, A-MPR for NS_202 specified in clause 6.2.3.3.3 applies.

6.2.3.3.3	A-MPR for NS_202 for power class 3

For power class 3, A-MPR for NS_202 shall be 1.0 dB if the corresponding bit of modifedMPR-Behavior for bands n257, n258 as defined in Annex H is set to 0 otherwise 2.0 dB.

6.2.3.3.4	A-MPR for NS_202 for power class 4

For power class 4, A-MPR for NS_202 specified in clause 6.2.3.3.3 applies.

6.2.3.4	A-MPR for NS_203

6.2.3.4.1	A-MPR for NS_203 for power class 1

For power class 1, A-MPR for NS_203 shall be 3.0 dB if Offset frequency < BWchannel, 0.0 dB otherwise. 
The Offset frequency is defined as the frequency from 24.25 GHz to the lower edge of the channel bandwidth.

6.2.3.4.2	A-MPR for NS_203 for power class 2

For power class 2, A-MPR for NS_203 specified in subclause 6.2.3.4.3 applies.

6.2.3.4.3	A-MPR for NS_203 for power class 3

For power class 3, A-MPR for NS_203 shall be 0 dB.

6.2.3.4.4	A-MPR for NS_203 for power class 4

For power class 4, A-MPR for NS_203 specified in subclause 6.2.3.4.3 applies.

6.2.3.5	[Reserved]

6.2.3.6	A-MPR for NS_205

6.2.3.6.1	A-MPR for NS_205 for power class 1

For power class 1, A-MPR for NS_205 shall be 7.0 dB if offset frequency < BWchannel, 6.0 dB otherwise.

Offset frequency is defined as the frequency from the upper edge of the protected frequency range i.e. 24 GHz to the lower edge of the channel bandwidth.

6.2.3.6.2	A-MPR for NS_205 for power class 2

For power class 2, A-MPR for NS_205 specified in clause 6.2.3.6.3 applies.

6.2.3.6.3	A-MPR for NS_205 for power class 3

For power class 3, A-MPR for NS_205 shall be 2.0 dB if offset frequency < BWchannel, 0.0 dB otherwise.

Offset frequency is defined as the frequency from the upper edge of the protected frequency range i.e. 24 GHz to the lower edge of the channel bandwidth.

6.2.3.6.4	A-MPR for NS_205 for power class 4

For power class 4, A-MPR for NS_205 specified in clause 6.2.3.6.3 applies.

< Next changes >

6.2A.3	UE maximum output power with additional requirements for CA

6.2A.3.1	General

Additional emission requirements can be signalled by the network with network signalling value indicated by the field additionalSpectrumEmission. To meet these additional requirements, additional maximum power reduction (A-MPR) is allowed for the maximum output power as specified in clause 6.2A.1. Unless stated otherwise, an A-MPR of 0 dB shall be used. Unless otherwise stated, the allowed total back off is maximum of A-MPR and MPR specified in clause 6.2A.2.

For intra-band contiguous aggregation with the UE configured for transmissions on two serving cells, the maximum output power reduction specified in Table 6.2A.3.1-1 is allowed for all serving cells of the applicable uplink contiguous CA configurations.

Table 6.2A.3.1-1 specifies the additional requirements and allowed A-MPR with corresponding network signalling label and operating band. The mapping between network signalling labels and the additionalSpectrumEmission IE defined in TS 38.331 [13] is specified in Table 6.2A.3.1-2. Unless otherwise stated, the allowed total back off is maximum of A-MPR and MPR specified in clause 6.2A.2.

Table 6.2A.3.1-1: Additional maximum power reduction (A-MPR)

| Network Signalling value | Requirements (clause) | NR Band | Channel bandwidth (MHz) | Resources Blocks (NRB) | A-MPR (dB) |
| --- | --- | --- | --- | --- | --- |
| CA_NS_200 |  |  |  |  | N/A |
| CA_NS_201 | 6.5A.3.2.2 | n258 |  |  | 6.2A.3.2 |
| CA_NS_202 | 6.5A.3.2.3 | n257, n258 |  |  | 6.2A.3.3 |
| CA_NS_203 | 6.5A.3.2.4 | n258 |  |  | 6.2A.3.4 |
| CA_NS_205 | 6.5A.3.2.6 | n257, n258 |  |  | 6.2A.3.6 |
| NOTE: CA_NS_201 is obsolete, the associated additional spurious emission requirements are not applicable. |  |  |  |  |  |

Table 6.2A.3.1-2: Value of additionalSpectrumEmission

| NR Band | Value of additionalSpectrumEmission / NS number |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| n257 | CA_NS_200 | CA_NS_202 | CA_NS_205 |  |  |  |  |  |
| n258 | CA_NS_200 | CA_NS_201 | CA_NS_202 | CA_NS_203 | CA_NS_205 |  |  |  |
| n259 | CA_NS_200 |  |  |  |  |  |  |  |
| n260 | CA_NS_200 |  |  |  |  |  |  |  |
| n261 | CA_NS_200 |  |  |  |  |  |  |  |
| NOTE 1: additionalSpectrumEmission corresponds to an information element of the same name defined in clause 6.3.2 of TS 38.331 [13]. NOTE 2: CA_NS_201 is obsolete, the associated additional spurious emission requirements are not applicable. |  |  |  |  |  |  |  |  |

6.2A.3.2	Void

6.2A.3.2.1	Void

Table 6.2A.3.2.1-1: (Void)

6.2A.3.2.2	Void

Table 6.2A.3.2.2-1: (Void)

6.2A.3.2.3	Void

Table 6.2A.3.2.3-1: Void

6.2A.3.2.4	Void

6.2A.3.3	A-MPR for CA_NS_202

6.2A.3.3.1	A-MPR for CA_NS_202 for power class 1

If the corresponding bit of modifedMPR-Behavior for bands n257, n258 as defined in Annex H is set to 0:

- For intra-band contiguous and non-contiguous UL CA, A-MPR for CA_NS_202 shall be 11.0 dB.

If the corresponding bit of modifedMPR-Behavior for bands n257, n258 as defined in Annex H is set to 1:

- For intra-band contiguous UL CA, A-MPR for CA_NS_202 shall be 11.0 dB.

- For intra-band contiguous UL CA with non-contiguous RB allocation or non-contiguous UL CA, A-MPR for CA_NS_202 shall be [13.0 dB] if offset frequency < BWintraCA, 11.0 dB otherwise.

Offset frequency is defined as the frequency from the upper edge of the protected frequency range i.e. 24 GHz to the lower edge of the aggregated channel bandwidth of the configured UL CA.

6.2A.3.3.2	A-MPR for CA_NS_202 for power class 2

For UL CA, A-MPR for CA_NS_202 specified in sub-clause 6.2A.3.3.3 applies.

6.2A.3.3.3	A-MPR for CA_NS_202 for power class 3

If the corresponding bit of modifedMPR-Behavior for bands n257, n258 as defined in Annex H is set to 0:

- For intra-band contiguous and non-contiguous CA, A-MPR for CA_NS_202 shall be 2.0 dB.

If the corresponding bit of modifedMPR-Behavior for bands n257, n258 as defined in Annex H is set to 1:

- For intra-band contiguous UL CA, A-MPR for CA_NS_202 shall be [5.0] dB if offset frequency < BWintraCA, 2.0 dB otherwise.

- For intra-band contiguous UL CA with non-contiguous RB allocation or non-contiguous UL CA, A-MPR for CA_NS_202 shall be [8.0 dB] if offset frequency < BWintraCA, 2.0 dB otherwise.

Offset frequency is defined as the frequency from the upper edge of the protected frequency range i.e. 24 GHz to the lower edge of the aggregated channel bandwidth of the configured UL CA.

6.2A.3.3.4	A-MPR for CA_NS_202 for power class 4

For UL CA, A-MPR for CA_NS_202 specified in sub-clause 6.2A.3.3.3 applies.

#### 6.2A.3.4	A-MPR for CA_NS_203

6.2A.3.4.1	A-MPR for CA_NS_203 for power class 1

For intra-band contiguous CA, A-MPR for CA_NS_203 shall be 6.5 dB, if Offset frequency < BWChannel_CA of the UL CA configuration, 0.0 dB, otherwise
The Offset frequency is defined as the frequency from 24.25 GHz to the lower edge of the lowest CC among the configured UL CA.

6.2A.3.4.2	A-MPR for CA_NS_203 for power class 2

For intra-band contiguous CA, AMPR specified in clause 6.2A.3.4.3 applies.

6.2A.3.4.3	A-MPR for CA_NS_203 for power class 3

For intra-band contiguous CA, A-MPR for CA_NS_203 shall be 2.5 dB, if Offset frequency < BWChannel_CA of the UL CA configuration, 0.0 dB otherwise. 
The Offset frequency is defined as the frequency from 24.25 GHz to to the lower edge of the lowest CC among the configured UL CA.

6.2A.3.4.4	A-MPR for CA_NS_203 for power class 4

For intra-band contiguous CA, AMPR specified in clause 6.2A.3.4.3 applies.

#### 6.2A.3.5	[Reserved]

6.2A.3.6	A-MPR for CA_NS_205

6.2A.3.6.1	A-MPR for CA_NS_205 for power class 1

For intra-band contiguous UL CA with single contiguous RB allocation, A-MPR for CA_NS_205 shall be [9.0 dB] if offset frequency < BWintraCA, 6.0 dB otherwise.

For intra-band contiguous UL CA with non-contiguous RB allocation or non-contiguous UL CA, A-MPR for CA_NS_205 shall be [13.0 dB] if offset frequency < BWintraCA, 6.0 dB otherwise.

Offset frequency is defined as the frequency from the upper edge of the protected frequency range i.e. 24 GHz to the lower edge of the aggregated channel bandwidth of the configured UL CA.

6.2A.3.6.2	A-MPR for CA_NS_205 for power class 2

For UL CA, A-MPR for CA_NS_205 specified in sub-clause 6.2A.3.6.3 applies.

6.2A.3.6.3	A-MPR for CA_NS_205 for power class 3

For intra-band contiguous UL CA with single contiguous RB allocation, A-MPR for CA_NS_205 shall be [5.0 dB] if offset frequency < BWintraCA, 0.0 dB otherwise.

For intra-band contiguous UL CA with non-contiguous RB allocation or non-contiguous UL CA, A-MPR for CA_NS_205 shall be [8.0 dB] if offset frequency < BWintraCA, 0.0 dB otherwise.

Offset frequency is defined as the frequency from the upper edge of the protected frequency range i.e. 24 GHz to the lower edge of the aggregated carrier bandwidth of the configured UL CA.

6.2A.3.6.4	A-MPR for CA_NS_205 for power class 4

For UL CA, A-MPR for CA_NS_205 specified in sub-clause 6.2A.3.6.3 applies.

< Next changes >

6.5.3.2	Additional spurious emissions

6.5.3.2.1	General

These requirements are specified in terms of an additional spectrum emission requirement. Additional spurious

emission requirements are signalled by the network to indicate that the UE shall meet an additional requirement for

a specific deployment scenario as part of the cell handover/broadcast message.

6.5.3.2.2	Void

Table 6.5.3.2.2-1: (Void)

6.5.3.2.3	Additional spurious emission requirements for NS_202

When "NS_202" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5.3.2.3-1.

Table 6.5.3.2.3-1: Additional requirements (NS_202)

| Frequency Range | Maximum Level | Measurement bandwidth |
| --- | --- | --- |
| 7.25 GHz ≤ f ≤ 2nd harmonic of the upper frequency edge of the UL operating band | -10 dBm | 100 MHz |
| 23.6 GHz f 24.0 GHz | -5 dBm | 200 MHz |
| NOTE 1: This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3-1 from the edge of the channel bandwidth. The protection of frequency range 23600 - 24000 MHz is meant for protection of satellite passive services. |  |  |

6.5.3.2.4	Additional spurious emission requirements for NS_203

When "NS_203" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5.3.2.4-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3-1 from the edge of the channel bandwidth.

Table 6.5.3.2.4-1: Additional requirements (NS_203)

| Frequency band (GHz) | Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
| 23.6 f 24.0 | +1 | 200 MHz |

6.5.3.2.5	[Reserved]

6.5.3.2.6	Additional spurious emission requirements for NS_205

When "NS_205" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5.3.2.6-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3-1 from the edge of the channel bandwidth.

Table 6.5.3.2.6-1: Additional requirements (NS_205)

| Frequency band (GHz) | Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
| 23.6 f 24.0 | -5 | 200 MHz |

< Next changes >

6.5A.3.2	Additional spurious emissions

6.5A.3.2.1	General

These requirements are specified in terms of an additional spectrum emission requirement. Additional spurious emission requirements are signalled by the network to indicate that the UE shall meet an additional requirement for a specific deployment scenario as part of the cell handover/broadcast message.

6.5A.3.2.2	Void

6.5A.3.2.3	Additional spurious emission requirements for CA_NS_202

When "CA_NS_202" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5.3.2.3-1.

6.5A.3.2.4	Additional spurious emission requirements for CA_NS_203

When "CA_NS_203" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5.3.2.4-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) as defined in clause 6.5A.3.

6.5A.3.2.5	[Reserved]6.5A.3.2.6	Additional spurious emission requirements for CA_NS_205

When "CA_NS_205" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5.3.2.6-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) as defined in section 6.5A.3.

< Next changes >

Annex H (Normative):
Modified MPR behavior

H.1	Indication of modified MPR behavior

This annex contains the definitions of the bits in the field modifiedMPR-Behavior indicated per supported NR band in the IE RF-Parameters [13] by a UE supporting an MPR or A-MPR modified in a given version of this specification. A modified MPR or A-MPR behaviour can apply to a supported NR band in stand-alone operation (including CA and NN-DC operation) or in non-standalone operation with the said NR band as part of an EN-DC or NE-DC band combination. Moreover, the bits in the field can explicitly indicate NS value(s) supported by a UE.

NOTE 1:	In the present release, the modifiedMPR-Behavior is indicated [13] by an 8-bit bitmap per supported NR band.

Table H.1-1: Definitions of the bits in the field modifiedMPRbehavior

| NR Band | Index of field (bit number) | Definition (description of the supported functionality if indicator set to one) | Notes |
| --- | --- | --- | --- |
| n257 | 0 (leftmost bit) | - FR2 power class 3 MPR as defined in clause 6.2.2.3 of 38.101-2 v16.2.0 | - This bit may be set to 1 by a UE supporting n257 |
|  | 1 | - NS_202 as defined in clause 6.5.3.2.3 or both NS_202 and CA_NS_202 as defined in clause 6.5A.3.2.3 of 38.101-2 v16.27.0 | - This bit shall be set to 1 by a UE supporting n257 or both n257 and CA_n257 |
|  | 2 | - NS_205 as defined in clause 6.5.3.2.6 or both NS_205 and CA_NS_205 as defined in clause 6.5A.3.2.6 of 38.101-2 v16.27.0 | - This bit shall be set to 1 by a UE supporting n257 or both n257 and CA_n257 |
| n258 | 0 (leftmost bit) | - FR2 power class 3 MPR as defined in clause 6.2.2.3 of 38.101-2 v16.2.0 | - This bit may be set to 1 by a UE supporting n258 |
|  | 1 | Void |  |
|  | 2 | - NS_203 as defined in clause 6.5.3.2.4 or both NS_203 and CA_NS_203 as defined in clause 6.5A.3.2.4 of 38.101-2 v15.12.0 | - This bit shall be set to 1 by a UE supporting n258 or both n258 and CA_n258 |
|  | 3 | - NS_202 as defined in clause 6.5.3.2.3 or both NS_202 and CA_NS_202 as defined in clause 6.5A.3.2.3 of 38.101-2 v16.27.0 | - This bit shall be set to 1 by a UE supporting n258 or both n258 and CA_n258 |
|  | 4 | - NS_205 as defined in clause 6.5.3.2.6 or both NS_205 and CA_NS_205 as defined in clause 6.5A.3.2.6 of 38.101-2 v16.27.0 | - This bit shall be set to 1 by a UE supporting n258 or both n258 and CA_n258 |
| n260 | 0 (leftmost bit) | - FR2 power class 3 MPR as defined in clause 6.2.2.3 of 38.101-2 v16.2.0 | - This bit may be set to 1 by a UE supporting n260 |
| n261 | 0 (leftmost bit) | - FR2 power class 3 MPR as defined in clause 6.2.2.3 of 38.101-2 v16.2.0 | - This bit may be set to 1 by a UE supporting n261 |

< end of changes >