| 3GPP TR 38.768 V0.0.3 (2025-04) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; NR; Low NR Band Carrier Aggregation via Switching (Release 19) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet https://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

1	Scope	6

2	References	6

3	Definitions of terms, symbols and abbreviations	7

3.1	Terms	7

3.2	Symbols	7

3.3	Abbreviations	7

4	Low NR band aggregation scenarios	7

5	Switching time mask	8

5.1	Switching periods	8

5.2	Switching period location	8

5.3	Time mask for switching	8

6	RRM aspects	9

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# 1	Scope

This Technical Report captures the outcomes from the Work Item on NR low band carrier aggregation via switching.  The scope of the Work Item is to introduce physical layer procedures and requirements to enable low band carrier aggregation via switching according to the following objectives:

-	Specify UE requirements, including at least switching gap (if needed), and corresponding physical layer procedures to allow switching between {case 1, case 2} [RAN4, RAN1]

-	Case 1: Tx/Rx on FDD carrier 1 and no Rx on SDL carrier 2

-	Case 2: Rx on SDL carrier 2 and no Tx/Rx on FDD carrier 1

-	RAN1 to specify only a semi-static switching pattern based on RRC configuration, liaising with RAN2 and RAN4 as necessary

-	Specify the switching delay and time mask for carrier switching [RAN4]

-	Specify necessary RRM requirements [RAN4]

-	Define the corresponding UE capabilities [RAN4, RAN2, RAN1]

-	Consider the following deployment constraints:

-	The carrier frequency for all cases is <1 GHz

-	Co-located and synchronized network deployment for both carriers

-	Both carriers are in a single TAG

-	SCS 15KHz on both carriers

Note 1: Specify requirements for the feature with the following example band combination: CA_n5A-n29A in this WI, with additional band combinations to be handled via the basket work item approach

Note 2: Strive to minimize the RAN1 impact

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 38.101-1: "NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone".

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

Definition format (Normal)

<defined term>: <definition>.

example: text used to clarify abstract rules by applying them literally.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

Symbol format (EW)

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

Abbreviation format (EW)

<ABBREVIATION>	<Expansion>

# 4	Low NR band aggregation scenarios

<Editor’s note: capture operator-requested scenarios in this clause>

Operator interest for aggregating low band spectrum has always been high during the entire history of the carrier aggregation feature, spanning LTE and NR specifications.  Considering the practicalities of handset RF front end architectures available in the market today, the operators have requested 3GPP to consider specifying a solution based on a switching scheme which can potentially accommodate the related limitations.

The following observations motivate the effort undertaken by this Work Item:

-	The amount of mid-band spectrum an operator holds is typically 10-20 times greater than their low-band spectrum holdings.

-	Mid-band spectrum is more effective closer to cell sites, while low-band spectrum propagates farther, making it more useful at greater distances from sites.

-	Low-band spectrum carries significant traffic volumes in urban (indoor) and rural areas, leading to congestion that severely degrades customer experience.

-	This issue is exacerbated by the fact that low-band SDL bands, which reach most poor coverage areas, cannot be paired with an UL, rendering them useless.

-	Utilization of SDL bands via Low-Low band CA, where one low band serves as an SDL, could potentially solve this problem. However, this solution doesn't exist due to OEM challenges in supporting it.

In the context of the scope of this Work Item, Table 4-1 below illustrates the applicable operator-provided scenarios.

Table 4-1: Operator-provided scenarios for NR low band aggregation via switching

| Configuration | Illustration | Notes |
| --- | --- | --- |
| CA_n5A-n29A |  | (1), (2) |
| CA_n12A-n29A |  | (3) |
| CA_n14A-n29A |  |  |
| CA_n28A-n67A |  | (4), (5) |
| CA_n29A-n71A |  | (6), (7) |
| NOTE 1: CA_n5A-n29A is already specified in TS38.101-1 [1] based on Rel-15 NR CA framework NOTE 2: Fractional bandwidth of a single antenna to support this combination is 22% and represents a practical implementation challenge NOTE 3: There is no incumbent narrowband service in the band gap between n29 and n12 DL (728~729 MHz) in Canada NOTE 4: In Europe band n28 spectrum is restricted to 703-733 MHz UL and 758-788 MHz DL NOTE 5: 3GPP will introduce requirements for band n28 assuming full band duplexer architecture in Rel-19 NOTE 6: CA_n29A-n71A is already specified in TS38.101-1 [1] based on Rel-15 NR CA framework NOTE 7: Fractional bandwidth of a single antenna to support this combination is 16.5% and represents a practical implementation challenge |  |  |

# 5	Switching time mask

<Editor’s note: capture outcome of the discussion on the switching time mask in this clause>

## 5.1	Switching periods

To accommodate different UE implementations, RAN4 has reached the agreement in RAN4#114 on the applicable switching periods of 35μs and 140μs for switching between Case 1 and Case 2 as an optional UE capability.

## 5.2	Switching period location

<TBD>

## 5.3	Time mask for switching

<TBD>

# 6	RRM aspects

<Editor’s note: capture RRM aspects in this clause>

Annex <F> (informative):
Change history

Use style "Heading 8" in TSs and "Heading 9" in TRs. Do not use "informative" in the title in TRs.

This is the last annex for TS/TSs which details the change history using the following table.
This table is to be used for recording progress during the WG drafting process till TSG approval of this TS/TR.
For TRs under change control, use one line per approved Change Request
Date: use format YYYY-MM
CR: four digits, leading zeros as necessary
Rev: blank, or number (max two digits)
Cat: use one of the letters A, B, C, D, F
Subject/Comment: for TSs under change control, include full text of the subject field of the Change Request cover
New vers: use format [n]n.[n]n.[n]n

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2025-02 | RAN4#114 | R4-2502875 |  |  |  | Initial skeleton | 0.0.2 |
| 2025-04 | RAN4#114bis | R4-2505146 |  |  |  | Implemented text proposals from the following: R4-2503477 TP to TR 38.768 on switching periods R4-2505147 TP to TR38.768 on operator provided scenarios for low band aggregation via switching | 0.0.3 |