3GPP TSG-RAN WG4 Meeting #114-bis		R4-2504767

Wuhan, Hubei, China, 07th – 11th April, 2025

Title:	LS on the transient period for SBFD operation

Response to:	N/A

Release:	Rel-19

Work Item:	NR_duplex_evo-Core

Source:	RAN4

To:	RAN1

Cc:

Contact person:	Wang, He (Jackson)

h0809.wang@samsung.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

## 1	Overall description

On RAN4 requirements for SBFD transient period at gNB, RAN4 has made the following agreements:

Agreement: - Transient period length between SBFD and non-SBFD: - The existing TDD BS transmitter transient period reused, i.e., 10us for FR1 and 3us for FR2-1. Agreement: - No need to specify transmitter transient period requirement between different SBFD configurations. Agreement: - RAN4 shall not define an SBFD-architecture dependent requirement. - Transient period between SBFD and non-SBFD is applicable to all four cases: Frequency regions for transition; BS transmitter behavior during transition. Case-A (SBFD symbol to DL symbol); SBFD UL subband and guardband(s); BS transmitter OFF-to-ON. Case-B (SBFD symbol to UL symbol); SBFD DL subband; BS transmitter ON-to-OFF. Case-C (DL symbol to SBFD symbol); SBFD UL subband and guardband(s); BS transmitter ON-to-OFF. Case-D (UL symbol to SBFD symbol); SBFD DL subband; BS transmitter OFF-to-ON.

Particularly for the location of transient period, RAN4 has identified four cases of transitions to/from SBFD symbols as illustrated in Figure 1:

Figure 1: Illustration of four cases of transitions to/from SBFD symbols

And the following agreement is achieved on the location of transient period for the above four cases:

Agreement: - RAN4 reach the conclusion of the location of transient periods: - Case-A: in TA_offset in SBFD symbol(s); - Case-D: in TA_offset in non-SBFD UL symbol(s). Agreement: - For Case-B, guard symbol(s) should be reserved before UL symbols - The transient period can be reserved only in the DL part(s) of SBFD symbol(s) or in the whole channel bandwidth, up to gNB implementation. Agreement: - For the location of transient periods Case-C, - Both Option 1 (guard symbols reserved in non-SBFD DL symbol(s)) and Option 2 (blanked SBFD symbols in the beginning of SBFD UL transmission) shall be allowed, up to gNB implementation.

RAN4 respectfully asks RAN1 take the above RAN4 agreement into account, and provide further information on the location of transient period if any.

## 2	Actions

To RAN1

ACTION: RAN4 respectfully asks RAN1 take the above RAN4 agreement into account, and provide further information on the location of transient period if any.

## 3	Dates of next TSG RAN WG1 meetings

RAN4#115		19th – 23rd May 2025	 	Malta, MT

RAN4#116 		25th – 29th Aug. 2025	 	Bangalore, India