3GPP TSG-RAN WG4 Meeting #114bis		  R4-2504962

Wuhan, China, 7th – 11th April, 2025

Title:	LS on Rx BSF optimization for NR RRM Ph5

Response to:

Release:	Rel-19

Work Item:	NR_RRM_Ph5_Core

Source:	RAN4

To:	RAN2

Cc:

Contact person:	Han Li, lihan2@cictmobile.com

Send any reply LS to:  3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

1	Overall description

For NR RRM Phase 5 WI, RAN4 would like to update RAN2 the progress of the discussion on the solution for FR2-1 L3 measurement delay reduction by optimizing Rx BSF (beam sweeping factor). Following agreements have been reached, which need RAN2 signaling design.

Agreement in RAN4#113: - RAN4 to introduce serving cell’s signal quality (i.e., based on RSRP and/or RSRQ threshold(s)) based triggering condition to activate/deactivate multi-Rx for L3 measurement. Agreement in RAN4#114: - Adopt FBS triggering condition of serving cell is worse than a threshold, including RSRP, RSRQ threshold. Agreement in RAN4#114bis: - For the baseline scenario agreed in RAN4 #112, limit the serving cell to a FR2 serving cell. Agreement in RAN4#112: - Baseline: L3 delay enhancements in Rel-19 by optimizing Rx BSF for UE supporting multi-Rx simultaneous reception are applicable provided that: - the target carrier(s) to be measured: only one carrier in the single FR2-1 band is configured for L3 SSB measurement and - UE serving carrier(s): UE is configured with single carrier on FR2-1 band, i.e. FR2-1 PCell without CA/DC. Note: Target and serving carrier frequency can be the same or different. Agreement in RAN4#112bis: - RAN4 agree that the criteria/threshold(s) are configured by RRC signaling.

Meanwhile, RAN4 is still discussing the de-activation condition and re-triggering mechanism. If any demands on additional RAN2 design are confirmed, RAN4 will provide corresponding information in the future LS. RAN4 respectfully requests RAN2 to take above agreements into account in their work.

2	Actions

To RAN2:

ACTION: 	RAN4 respectfully asks RAN2 to take the above agreements into account in their future work.

3	Dates of next TSG RAN WG4 meetings

TSG RAN WG4 Meeting #115			19th – 23rd May, 2025			Malta

TSG RAN WG4 Meeting #116			25th – 29th August, 2025			Bangalore, India