3GPP TSG-RAN WG4 Meeting #114bis	R4-2505161

Wuhan, China, 7th– 11th, Apr, 2025

Title:	LS to RAN2 on Type 4 for non-collocated deployment

Response to:

Release:	Rel-19

Work Item:	NonCol_intraB_ENDC_NR_CA_Ph2-Core

Source:	TSG RAN WG4

To:	TSG RAN WG2

Cc:

Contact Person:

Name:  Yuanyuan Zhang

E-mail Address:  tina55.zhang@samsung.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

- Overall Description:

RAN4 discussed the applicable UE requirements, new UE capability and new BS signalling for Type 4a/4b1 in previous meetings for which the following agreements were reached. RAN4 respectfully ask RAN2 to devise necessary UE capabilities and BS signalling accordingly.

UE capability Type

- For intra-band non-contiguous NR-CA, Type 4b is considered, Type 4a is not considered

- For inter-band non-contiguous EN-DC with overlapping or partially overlapping bands, both Type 4a and Type 4b are considered

UE requirements:

- Reuse the power imbalance of Type 2

- Reuse the MRTD and MTTD requirements of Type 2

UE capability:

- Define one capability for NR CA and one capability for EN-DC

- For EN-DC, Type 4a and Type 4b capability can be differentiated by legacy capability, such as MIMO-CapabilityDL-r10 on E-UTRA carrier

- If UE reports capability for Type 4a/4b, Type 2 capability shall be deemed as supported by default, and UE still needs to report Type 2 capability if UE supports it in order to keep backward compatibility (i.e., for legacy BS)

BS signalling:

- When maxMIMO-layers > 4, Type 1(collocated) requirements apply. No new BS signaling is needed

- When maxMIMO-layers=2, UE follow Rel-18 specification. No new BS signaling is needed

- When maxMIMO-layers=4, new BS signalling(s) is needed for NR-CA and EN-DC, and the details are up to RAN2

NOTE: maxMIMO-layers intends for the legacy BS signaling

NOTE 1: Type4a/4b is mentioned in the WID RP-242083.

2. Actions:

To RAN2:

ACTION: RAN4 respectfully ask RAN2 to devise necessary UE capabilities and BS signaling.

3. Date of Next TSG WG RAN4 Meetings:

TSG-RAN WG4 Meeting #115	18 - 24 May, 2025		                   Malta

TSG-RAN WG4 Meeting #116	25 – 29 Aug, 2025		                  India