3GPP TSG-RAN WG4 Meeting #115	R4-25xxxxx

St Julian’s, Malta, 19th – 25th May 2025

Third Generation Partnership Project (3GPP™)

DRAFT Meeting Report
for
TSG RAN WG4
meeting: 114-bis

Wuhan, China, 07/04/2025 to 11/04/2025

Report generated on Wednesday, 2025-04-02 09:12  UTC

Contents:

1	Opening of the meeting	8

2	Meeting agenda, arrangement, and meeting report	8

3	RAN4 Chair Elections	8

4	Incoming LS	8

5	Rel-19 maintenance for LTE and NR and Rel-18 TEI18	11

5.1	Moderator summary and conclusions (for Agenda 5)	11

5.1.1	Main session	11

5.1.2	RRM session	11

5.1.3	BDaT sessoin	11

5.2	Rel-19 spectrum related WI maintenance	12

5.2.1	Introduction of the 1.4 GHz Band	12

5.2.2	Introduction of LTE FDD band in 1800–1830 MHz for Canada	12

5.2.3	Introduction of NR bands n87 and n88	12

5.2.4	Introduction of NR band n68	12

5.2.5	Introduction of additional operating NR bands for HAPS (High Altitude Platform Station)	13

5.2.6	Introduction of Power Class 2 and UE 40MHz Channel Bandwidth in NR band n28	13

5.2.7	Others	13

5.3	Rel-18 TEI	14

5.3.1	UE RF related topics	14

5.3.2	RRM related topics	17

5.3.3	BS RF, demodulation performance and other topics	18

6	Rel-19 on-going spectrum related work items for NR and LTE	18

6.1	Moderator summary and conclusions (for Agenda 6)	18

6.2	Rel-19 DC of x LTE band(s), y NR band(s) (x<=6) and single or two NR SUL bands	20

6.2.1	Rapporteur input (WID/TR/big CR)	20

6.2.2	UE RF requirements for EN-DC and NE-DC of 2 DL with 2 UL (DC_R19_1BLTE_1BNR_2DL2UL)	21

6.2.3	UE RF requirements for EN-DC and NE-DC of 2 LTE and 1 NR, or of 1 LTE and 2 NR (DC_R19_xBLTE_yBNR_3DL2UL)	22

6.2.4	UE RF requirements for EN-DC and NE-DC of x LTE and y NR with total z DL bands and q UL bands (DC_R19_xBLTE_yBNR_zDLqUL)	25

6.2.5	UE RF requirements for EN-DC and NE-DC with one SUL and two SULs (DC_R19_LTE_NR_SUL_combos)	26

6.3	Rel-19 NR CA/DC for x bands DL with y bands UL (x<7, y<3) and SUL/CA band combinations with a single SUL or two SUL cells	26

6.3.1	Rapporteur input (WID/TR/big CR)	26

6.3.2	UE RF requirements for NR intra-band CA combinations for x CC DL/y CC UL (NR_CA_R19_Intra with/without UL-MIMO)	29

6.3.3	UE RF requirements for NR inter-band CA/DC configurations including inter band CA for 2 DL with up to 2UL (NR_CADC_R19_2BDL_xBUL)	29

6.3.4	UE RF requirements for NR inter-band CA/DC configurations including inter band CA for 3 DL with x UL (NR_CADC_R19_3BDL_xBUL)	34

6.3.5	UE RF requirements for NR inter-band CA/DC configurations including inter band CA for y DL with x UL (NR_CADC_R19_yBDL_xBUL)	38

6.3.6	UE RF requirements for SUL and CA band combinations with SULs (NR_SUL_combos_R19)	39

6.4	Rel-19 LTE-Advanced Carrier Aggregation for x bands (1<=x<= 6) DL with y bands (y=1, 2) UL	40

6.4.1	Rapporteur input (WID/TR/big CR)	40

6.4.2	UE RF requirements	40

6.5	Rel-19 HPUE for NR FR1 TDD/FDD single band	40

6.5.1	Rapporteur input (WID/TR/big CR)	40

6.5.2	HPUE in a single TDD band	41

6.5.3	HPUE in a single FDD band	41

6.6	Rel-19 HPUE in a single LTE band	41

6.6.1	Rapporteur input (WID/TR/big CR)	41

6.6.2	UE RF requirements	41

6.7	Rel-19 HPUE for DC combinations of LTE band(s) and NR band(s)	41

6.7.1	Rapporteur input (WID/TR/big CR)	41

6.7.2	UE RF requirements	42

6.8	Rel-19 HPUE for NR intra-band CA and inter-band CA/DC with/without NR SUL	42

6.8.1	Rapporteur input (WID/TR/big CR)	42

6.8.2	UE RF requirements for intra-band CA	43

6.8.3	UE RF requirements for inter-band CA/DC with high power on TDD band(s)	43

6.8.4	UE RF requirements for inter-band CA/DC with high power on FDD band(s)	44

6.8.5	UE RF requirements for inter-band CA/DC with high power on both FDD and TDD bands	45

6.9	Rel-19 Additional NR bands for NR features	45

6.9.1	Rapporteur input (WID/TR/big CR)	45

6.9.2	UE RF requirements for UL-MIMO in a single band	46

6.9.3	UE RF requirements for 4Rx and 8Rx	46

6.10	Rel-19 downlink interruption for NR and EN-DC band combinations at dynamic Tx Switching in Uplink	46

6.10.1	Rapporteur input (WID/TR/big CR)	46

6.10.2	UE RF requirements	46

6.11	Simultaneous Rx/Tx band combinations for NR CA/DC, NR SUL and LTE/NR DC in Rel-19	46

6.11.1	Rapporteur input (WID/TR/big CR)	46

6.11.2	UE RF requirements	46

6.12	Adding channel bandwidth(s) support to existing NR bands and CA/ENDC combinations in REL-19	47

6.12.1	Rapporteur input (WID/TR/big CR)	47

6.12.2	UE RF requirements	47

6.13	Introduction of NR-NTN S-band (MSS band 2000-2020 MHz UL and 2180-2200 MHz DL)	48

6.13.1	System parameters and UE RF requirements	48

6.13.2	SAN RF core requirements	49

6.13.3	RRM core requirements	49

6.14	Introduction of IoT-NTN S-band (MSS band 2000-2020 MHz UL and 2180-2200 MHz DL)	50

6.14.1	System parameters and UE RF requirements	50

6.14.2	SAN RF core requirements	50

6.14.3	RRM core requirements	50

6.15	Introduction of new NR NTN bands to support the Extended L-band (UL 1668-1675MHz, DL 1518-1525MHz) and the combined MSS L-band and Extended L-band ranges	51

6.15.1	System parameters and UE RF requirements	51

6.15.2	SAN RF core requirements	52

6.15.3	RRM core requirements	52

6.16	New bands for LTE based 5G terrestrial broadcast for early deployments	52

6.16.1	General aspects and work plan	52

6.16.2	Band and system parameters	52

6.16.3	UE RF requirements	52

6.16.4	BS RF core requirements	53

6.17	mmWave in NR: UE spurious emissions and EESS (Earth Exploration Satellite Service) protection	53

6.17.1	General aspects	53

6.17.2	UE RF requirements	53

6.17.3	BS RF requirements and conformance testing	56

7	Rel-19 on-going non-spectrum related work/study items	56

7.1	UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4	56

7.1.1	UE RF requirements	56

7.1.1.1	High power UE (HPUE) for CA in terrestrial network (TN)	56

7.1.1.1.1	General aspects (incl. duty cycle solutions, MSD rules)	56

7.1.1.1.2	Intra-band contiguous and non-contiguous UL CA with PC1.5	59

7.1.1.1.3	Inter-band UL NR-CA/EN-DC with 2 bands and 2Tx and/or 3Tx	61

7.1.1.1.4	Increasing UE transmission high power limit	61

7.1.1.2	Power domain enhancement for NR single carrier and NR intra-band UL CA for PC2 and PC3	62

7.1.1.2.1	Power domain enhancements for single carrier	62

7.1.1.2.2	MPR applicability for FR1 intra-band UL CA	64

7.1.1.2.3	MPR applicability for FR2	65

7.1.1.3	6Rx UE	65

7.1.1.3.1	Reference sensitivity requirements	65

7.1.1.3.2	MIMO layer evaluation for 6Rx UE	65

7.1.1.3.3	SRS antenna switching requirements	67

7.1.1.3.4	SRS IL imbalance	67

7.1.2	RRM core and performance requirements for 6Rx	69

7.1.3	UE demodulation performance requirements for 6Rx	69

7.1.4	Moderator summary and conclusions	70

7.2	NR sidelink Intra-band Carrier Aggregation in ITS band	71

7.2.1	UE RF requirements for intra-band non-contiguous CA	71

7.2.2	UE RF requirements for intra-band contiguous CA	72

7.2.3	Moderator summary and conclusions	72

7.3	NR channel BW less than 5MHz for FR1 Phase 2	72

7.3.1	UE RF requirements maintenance for inter-band NR CA/DC with 3MHz CBW	72

7.3.2	RRM core requirements maintenance	73

7.3.3	RRM performance requirements	73

7.3.4	UE demodulation performance requirements	75

7.3.5	Moderator summary and conclusions	76

7.4	Support of intra-band non-collocated EN-DC/NR-CA deployment Phase2: new receiver type(s)	76

7.4.1	UE RF requirements	76

7.4.1.1	UE RF requirements for Type 4a/4b capable FWA UE for EN-DC/NR-CA	76

7.4.1.2	UE Capability/UE behavior and network signaling for Type 4 EN-DC/NR-CA	77

7.4.1.3	Other aspects (incl. clarification of contiguous LTE CCs)	78

7.4.2	RRM core requirements	78

7.4.3	Moderator summary and conclusions	80

7.5	Study on NR FR1 DL Fragmented Carriers	80

7.5.1	General aspects	80

7.5.2	Methods for reducing the number of UE Rx chains	81

7.5.3	Impacts on UE RF requirements and DL performance	83

7.5.4	Moderator summary and conclusions	85

7.6	NR power class 2 RedCap (Reduced Capability) UE in FR1	85

7.6.1	UE RF requirements	85

7.6.2	Moderator summary and conclusions	86

7.7	Low band carrier aggregation via switching	87

7.7.1	General aspects	87

7.7.2	UE RF requirements and UE capability	87

7.7.3	RRM core requirements	89

7.7.4	Moderator summary and conclusions	90

7.8	NR FR1 7 MHz Channel Bandwidth	91

7.8.1	General aspects	91

7.8.2	UE RF requirements and system parameters	91

7.8.3	BS RF core requirements	93

7.8.4	Moderator summary and conclusions	95

7.9	New LTE band for 5G broadcast for region 3 utilizing a geosynchronous satellite	95

7.9.1	General aspects and work plan	95

7.9.2	Band definition and system parameters	95

7.9.3	UE RF core requirements	96

7.9.4	SAN RF core requirements	96

7.9.5	Moderator summary and conclusions	96

7.10	Enhanced requirements and conductive test methodology for NR NTN and IoT NTN	96

7.10.1	General aspects	96

7.10.2	UE RF requirements for NTN HPUE	97

7.10.2.1	Coexistence study for example bands	97

7.10.2.2	NR-NTN HPUE RF requirements	97

7.10.2.2.1	Tx requirements	97

7.10.2.2.2	Rx requirements	99

7.10.2.3	IoT-NTN HPUE RF requirements	100

7.10.2.3.1	Tx requirements	100

7.10.2.3.2	Rx requirements	101

7.10.3	Less than 5MHz for NTN	102

7.10.3.1	System parameters	102

7.10.3.2	UE RF requirements	102

7.10.3.3	SAN RF core and conformance requirements	104

7.10.3.4	RRM core requirements	105

7.10.3.5	RRM performance requirements	106

7.10.3.6	Demodulation performance requirements	107

7.10.4	NTN testing for NGSO	108

7.10.4.1	Channel modeling	108

7.10.4.2	Demodulation performance requirements	109

7.10.5	Moderator summary and conclusions	110

7.11	Introduction of Ku Band for NR NTN	111

7.11.1	General aspects	111

7.11.2	Coexistence study based on ITU regulations	111

7.11.3	System parameters	112

7.11.4	UE RF requirements	114

7.11.5	SAN RF core requirements	115

7.11.6	RRM core requirements	116

7.11.7	Moderator summary and conclusions	117

7.12	Enhancements for Air-to-ground network for NR	118

7.12.1	UE RF requirements	118

7.12.1.1	Intra-band contiguous and inter-band CA	118

7.12.1.2	UL MIMO	118

7.12.1.3	DL MIMO	119

7.12.1.4	Others	119

7.12.2	BS RF requirements	119

7.12.3	BS RF conformance requirements	120

7.12.4	RRM core requirements for CA	121

7.12.5	RRM performance requirements	124

7.12.6	Demodulation performance requirements	124

7.12.7	Moderator summary and conclusions	125

7.13	NR base station (BS) RF requirement evolution for FR1/FR2 and testing	126

7.13.1	General aspects	126

7.13.2	Expected EIRP mask for upper 6GHz	126

7.13.3	OTA test enhancement	128

7.13.4	Transmitter co-existence spurious emission requirements	129

7.13.5	BS conformance testing	130

7.13.5.1	U6GHz BS EEIRP mask	130

7.13.5.2	Identification and reduction of BS OTA test scope	131

7.13.5.3	Simplification of BS TRP test methods	132

7.13.6	Moderator summary and conclusions	132

7.14	TRP (Total Radiated Power), TRS (Total Radiated Sensitivity) and MIMO OTA (Over the Air) testing enhancement Phase 3	133

7.14.1	General aspects	133

7.14.2	Core requirements	133

7.14.2.1	Test methodology for FR1 non-RedCap headworn XR devices	133

7.14.2.2	Test methodology and radiated performance metric for FR1 NTN devices	134

7.14.2.3	FR1 dynamic MIMO OTA test methodology	136

7.14.3	Performance requirements	137

7.14.3.1	TRP TRS requirements	137

7.14.3.2	MIMO OTA requirements	137

7.14.4	Moderator summary and conclusions	137

7.15	Study on NR FR2 OTA (Over the Air) testing enhancement Phase 3	138

7.15.1	General aspects	138

7.15.2	RF testing methodology for FR2 non-handheld UE that can transmit simultaneously with multi-panel	138

7.15.3	Moderator summary and conclusions	139

7.16	Study on spatial channel model for demodulation performance requirements	139

7.16.1	General aspects and work plan	139

7.16.2	Spatial channel modelling methodology	140

7.16.3	Moderator summary and conclusions	144

7.17	NR Radio Resource Management (RRM) Phase 5	144

7.17.1	General aspects	144

7.17.2	RRM core requirements of FR2-1 SSB based L3 measurement delay reduction for connected mode	145

7.17.2.1	FR2-1 L3 measurement delay by optimizing Rx beam sweeping factor	145

7.17.2.2	FR2-1 L3 measurement delay by optimizing CSSF outside gap in CA/DC	148

7.17.3	RRM core requirements of Fast SCell activation for UE supporting Rel-18 EMR	150

7.17.4	RRM performance requirements of FR2-1 SSB based L3 measurement delay reduction for connected mode	151

7.17.5	RRM performance requirements of Fast SCell activation for UE supporting Rel-18 EMR	152

7.17.6	Moderator summary and conclusions	153

7.18	NR demodulation performance Phase 5	153

7.18.1	General aspects	153

7.18.2	UE demodulation performance requirements for 8Rx with MMSE-IRC	154

7.18.3	BS demodulation performance requirements for MMSE-IRC	156

7.18.4	Moderator summary and conclusions	159

7.19	Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface	159

7.19.1	General aspects	159

7.19.2	CSI reporting requirement and testing framework for CSI prediction	160

7.19.3	RRM core requirement and testing framework for beam management	162

7.19.4	RRM core requirement and testing framework for Positioning accuracy enhancement	164

7.19.5	Moderator summary and conclusions	165

7.20	Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface Phase 2	166

7.20.1	General aspects	166

7.20.2	Testing issues for CSI compression two-sided models	166

7.20.3	Moderator summary and conclusions	168

7.21	Study on AI (Artificial Intelligence)/ML (Machine Learning) for mobility in NR	168

7.21.1	General aspects	168

7.21.2	Study of impacts on RAN4 requirements	169

7.21.2.1	Study of RAN4 impacts for RRM measurement prediction	169

7.21.2.2	Study of RAN4 impacts for measurement event prediction	171

7.21.3	Study of testability and interoperability	173

7.21.4	Moderator summary and conclusions	174

7.22	NR MIMO Phase 5	174

7.22.1	General aspects	174

7.22.2	UE RF requirements	175

7.22.3	RRM core requirements	176

7.22.3.1	Enhancement for UE-initiated/event-driven beam management	176

7.22.3.2	Other RRM requirements	177

7.22.4	Moderator summary and conclusions	179

7.23	Evolution of NR duplex operation: Sub-band full duplex (SBFD)	179

7.23.1	General aspects (including RAN4 aspects for SBFD system parameters)	179

7.23.2	BS RF requirements	181

7.23.2.1	Potentially new requirements for SBFD operation for FR1 and FR2-1	182

7.23.2.2	Modification of existing Tx requirements for FR1 and FR2-1	183

7.23.2.3	Modification of existing Rx requirements for FR1 and FR2-1	184

7.23.3	RRM core requirements	185

7.23.3.1	RRM requirements for UE-to-UE CLI handling	185

7.23.3.2	RRM impacts for SBFD operation	187

7.23.4	Moderator summary and conclusions	188

7.24	Solutions for Ambient IoT (Internet of Things) in NR	189

7.24.1	General aspects	189

7.24.2	RF requirements for A-IoT	190

7.24.2.1	RF requirements for Type 1-C Ambient-IoT BS	190

7.24.2.2	RF requirements for device 1	191

7.24.2.3	RF requirements for CW	193

7.24.3	RRM core requirements	194

7.24.4	OTA test method for A-IoT device 1	195

7.24.5	Moderator summary and conclusions	196

7.25	Enhancements of network energy savings for NR	197

7.25.1	General aspects	197

7.25.2	RRM core requirements	197

7.25.2.1	On-demand SSB SCell operation	197

7.25.2.2	Adaptation of common signal/channel transmission	199

7.25.2.3	On-demand SIB1	201

7.25.3	Moderator summary and conclusions	202

7.26	Low-power wake-up signal and receiver for NR (LP-WUS/WUR)	202

7.26.1	General aspects and RF TR (for email approval)	202

7.26.2	UE RF requirements for LP-WUS/WUR	203

7.26.2.1	System parameters	203

7.26.2.2	Rx requirements of REFSENS, ASCS and ACS	204

7.26.2.3	Rx requirements of IBB, OBB, intermodulation, spurious emissions and others	205

7.26.2.4	Testability for UE RF requirements	206

7.26.3	BS RF requirements for LP-WUS/WUR	207

7.26.4	RRM core requirements for LP-WUS/WUR	208

7.26.4.1	Simulation assumptions and results	208

7.26.4.2	RRM core requirements	210

7.26.5	Moderator summary and conclusions	212

7.27	NR mobility enhancements Phase 4	213

7.27.1	General aspects	213

7.27.2	RRM core requirements	213

7.27.2.1	Event triggered L1 measurement reporting	213

7.27.2.2	CSI-RS based L1 measurement	214

7.27.2.3	Conditional Intra-CU LTM	216

7.27.3	Moderator summary and conclusions	217

7.28	XR for NR Phase 3	217

7.28.1	General aspects	217

7.28.2	RRM core requirements	218

7.28.3	Moderator summary and conclusions	220

7.29	Non-Terrestrial Networks (NTN) for NR Phase 3	220

7.29.1	General aspects	220

7.29.2	UE RF requirements	221

7.29.2.1	RedCap UE RF requirements	221

7.29.2.2	Other requirements	222

7.29.3	SAN RF requirements	222

7.29.4	RRM core requirements	223

7.29.4.1	(e)RedCap RRM requirements	223

7.29.4.2	Other RRM requirements	224

7.29.5	Moderator summary and conclusions	226

7.30	Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3	226

7.30.1	General aspects	226

7.30.2	RF core requirements	227

7.30.3	RRM core requirements	228

7.30.4	Moderator summary and conclusions	228

7.31	Introduction of IoT-NTN TDD mode	229

7.31.1	General aspects	229

7.31.2	Band and system parameters (DL and UL channelization, channel bandwidth)	229

7.31.3	SAN RF requirements	229

7.31.4	UE RF requirements	229

7.31.5	RRM core requirements	230

7.31.6	Moderator summary and conclusions	231

8	Liaison output to other groups and related issues	231

8.1	R19 related (LS which cannot be submitted to any dedicated agenda under AI 7)	231

8.2	R18 related	231

8.3	R17, R15 and R16 related	233

8.4	Moderator summary and conclusions	234

9	RAN task and other topics	234

9.1	Framework simplification for co-location/co-existence requirements (RP-243288)	234

9.2	Principle of selected specification series in case of spectrum sharing by multiple RATs (related to approval of RP-250611)	237

10	New or revised WID/SID	237

11	Any other business	238

12	Close of the meeting	259

## 1	Opening of the meeting

The Chair Xizeng Dai (Huawei) opened the meeting at RAN4#114-bis on April7, 2025, at 09:00.

xxx presented the welcome speech.

Intellectual Property Rights Declaration Policy

The attention of the delegates to the meeting of this Technical Specification Group was drawn to the fact that 3GPP Individual Members have the obligation under the IPR Policies of their respective Organizational Partners to inform their respective Organizational Partners of Essential IPRs they become aware of.

The delegates were asked to take note that they were thereby invited:

-	to investigate whether their organization or any other organization owns IPRs which were, or were likely to become Essential in respect of the work of 3GPP.

-	to notify their respective Organizational Partners of all potential IPRs, e.g., for ETSI, by means of the IPR Information Statement and the Licensing declaration forms.

Statement regarding competition law

The attention of the delegates to the meeting was drawn to the fact that 3GPP activities were subject to all applicable antitrust and competition laws and that compliance with said laws was therefore required by any participant of the meeting, including the Chair and Vice-Chairs and were invited to seek any clarification needed with their legal counsel. The leadership would conduct the present meeting with impartiality and in the interests of 3GPP. Delegates were reminded that timely submission of work items in advance of TSG/WG meetings was important to allow for full and fair consideration of such matters.

Meeting arrangements

The meeting was conducted in three parallel sessions: Main session, RRM session, and BS RF Test Demod session. The Main session was chaired by RAN4 Chair Xizeng Dai (Huawei), RRM session was chaired by RAN4 Vice Chair Shan Yang (China Telecom), and BS RF Test Demod session was chaired by RAN4 Vice Chair Gene Fong (Qualcomm). The sessions were further broken down into separate GTW sessions (separate meeting rooms in F2F meeting). Webinar sessions were made available for online particpants.

Note: One or two additional offline(s) / adhoc session(s) may be scheduled according to RAN conclusion. Total three parallel GTW sessions would be scheduled. Plus, any additonal Offline(s) / ad hoc sesion(s) = ad hoc room or breakout room in F2F meeting.

Check-in for Registered Delegates

The attention of the delegates to this meeting was drawn to the fact that it is not permitted to check in other delegates on their behalf. In the even of technical difficulties preventing check in, delegates are encouraged to contact in person MCC.

Ordinary E-meeting participation

Attendance at ordinary e-meetings now counts towards accrual and maintenance of voting rights.

-	A delegate is deemed to have attended a given meeting if they confirm their participation by check in. If a delegate does not check in during the meeting, it shall be assumed that the individual did not attend.

Face-to-Face meeting with one-way remote participation (going forward there is no longer two-way remote)

When it is a face-to-face (ordinary) meeting with one-way remote participation.

Note: This meeting MCC was asked by MHPG to exceptionally have a 2-way remote access, . e.g. allowing comments/presentations remotely, but in no case the remote users are allowed to object to contributions or participate in show of hands.

-	In a meeting designated as face to face (ordinary), those participating remotely are not to be counted toward quorum or attendance, and are not allowed to vote

F2F network usage conditions

The PCG has laid down the following network usage conditions as provided below:

Users shall not use the network to engage in illegal activities. This includes activities such as copyright violation, hacking, espionage or any other activity that may be prohibited by local laws.

Users shall not engage in non-work related activities that consume excessive bandwidth or cause significant degradation of the performance of the network.

Since the network is a shared resource, users should exercise some basic etiquette when using the 3GPP network at a meeting. It is understood that high bandwidth applications such as downloading large files or video streaming might be required for business purposes, but delegates should be strongly discouraged in performing these activities for personal use. Downloading a movie or doing something in an interactive environment for personal use essentially wastes bandwidth that others need to make the meeting effective. The meeting Chair should remind end users that the network is a shared resource; the more one user grabs, the less there is for another. Email and its attachments already take up significant bandwidth (certain email programs are not very bandwidth efficient). In case of need the chair can ask the delegates to restrict IT usage to things that are essential for the meeting itself.

1. DON’T place your WiFi device in ad-hoc mode

2. DON’T set up a personal hotspot in the meeting room

3. DO try 802.11a if your WiFi device supports it

4. DON’T manually allocate an IP address

5. DON’T be a bandwidth hog by streaming video, playing online games, or downloading huge files

6. DON’T use packet probing software which clogs the local network (e.g., packet sniffers or port scanners)

Recording of RAN4 Meeting

Recording of the GoToWebinar sessions of the present meeting is strictly prohibited. No individual or entity – including the speakers and/or the authors – may electronically record any portion of the meeting without prior written consent of the Chair and all the RAN4 meeting participants.

Snapshot of contributions type areas submitted in 3GU before the start of the meeting: Total: 1592

Figure 1: Breakdown of contributions type areas for RAN4#114-bis pre-meeting

There are 12 formal CRs submitted this meeting that are only for endorsement. All formal CRs need to be resubmitted to RAN4#115 for agreement. The list of formal CRs is provided below.

| TDoc | Title | Source | AI | Rel | Spec | Ver | Related WIs | CR | CR CAT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| R4-2503577 | [37.941] Incorrect MU data for n104 band | ROHDE & SCHWARZ | 7.13.2 | Rel-17 | 37.941 | 17.5.0 | NR_BS_RF_req_evo-Core | 0058 | F |
| R4-2503588 | [37.941] Incorrect MU data for n104 band | ROHDE & SCHWARZ | 7.13.2 | Rel-18 | 37.941 | 18.2.0 | NR_BS_RF_req_evo-Core | 0059 | A |
| R4-2503589 | [38.141-2] Incorrect MU data for n104 band | ROHDE & SCHWARZ | 7.13.2 | Rel-17 | 38.141-2 | 17.17.0 | NR_BS_RF_req_evo-Perf | 0632 | F |
| R4-2503593 | [38.141-2] Incorrect MU data for n104 band | ROHDE & SCHWARZ | 7.13.2 | Rel-18 | 38.141-2 | 18.9.0 | NR_BS_RF_req_evo-Perf | 0633 | A |
| R4-2503601 | [Not inc C1-1] PWS specific MU data to be included for n104 band | ROHDE & SCHWARZ | 7.13.2 | Rel-19 | 38.908 | 19.0.0 | NR_BS_RF_req_evo-Core | 0001 | F |
| R4-2503602 | [C1-1 focus] Spec An MU to be updated for n104 band | ROHDE & SCHWARZ | 7.13.2 | Rel-19 | 38.908 | 19.0.0 | NR_BS_RF_req_evo-Core | 0002 | F |
| R4-2503810 | CR to TS38.161 for an alternate TRS and TRP test procedure for XR devices | Apple | 7.14.2.1 | Rel-19 | 38.161 | 18.3.0 | TRP_TRS_MIMO_OTA_Ph3-Core | 0024 | B |
| R4-2503985 | TS 38.101-1 draft big CR for NR_bands_xFeature_R19-Core | ZTE Corporation | 6.9.1 | Rel-19 | 38.101-1 | 19.1.0 | NR_bands_xFeature_R19-Core | 2726 | B |
| R4-2504182 | Draft CR for split measurement grids method for TRP/TRS measurements | Nokia, Keysight Technologies UK Ltd | 7.14.2.2 | Rel-18 | 38.870 | 18.4.0 | TRP_TRS_MIMO_OTA_Ph3-Core | 0020 | B |
| R4-2504323 | CR on introduction of RF requirements for FR2 PC8 in TS 38.101-2 [FR2_PC8] | Huawei, HiSilicon, CTC, CEPRI, OPPO, vivo, Xiaomi, CATT, Spreadtrum, Google, Honor, TD Tech, StarPoint, SRTC | 5.3.1 | Rel-18 | 38.101-2 | 18.9.0 | TEI18 | 0790 | B |
| R4-2504324 | CR on introduction of RRM requirements for FR2 PC8 in TS 38.133 [FR2_PC8] | Huawei, HiSilicon, CTC, CEPRI, OPPO, vivo, Xiaomi, CATT, Spreadtrum, Google, Honor, TD Tech, StarPoint, SRTC | 5.3.1 | Rel-18 | 38.133 | 18.9.0 | TEI18 | 5447 | B |
| R4-2504325 | CR on release independence for FR2 PC8 in TS 38.307 [FR2_PC8] | Huawei, HiSilicon, CTC, CEPRI, OPPO, vivo, Xiaomi, CATT, Spreadtrum, Google, Honor, TD Tech, StarPoint, SRTC | 5.3.1 | Rel-18 | 38.307 | 18.3.0 | TEI18 | 0182 | B |

## 2	Meeting agenda, arrangement, and meeting report

R4-2503100	RAN4#114 Meeting Report

Type: report		For: Approval
					Source: ETSI MCC

Abstract:

[RAN4#114-bis][100] Main Session

Decision: 		The document was not treated.

R4-2503101	Agenda for RAN4#114-bis

Type: agenda		For: Approval
					Source: RAN4 Chair (Huawei)

Abstract:

[RAN4#114-bis][100] Main Session

Decision: 		The document was not treated.

R4-2503102	RAN4#114-bis Meeting Arrangements and Guidelines

Type: other		For: Approval
					Source: RAN4 Chair (Huawei)

Abstract:

[RAN4#114-bis][100] Main Session

Decision: 		The document was not treated.

## 3	RAN4 Chair Elections

NOTE:	RAN4#114-bis will elect 1 new RAN4 Chair

-	Details will be announced during RAN4#114-bis

-	Electronic voting tool: Electronic voting tool: guidance see https://help.3gpp.org/index.php?title=3GPP_voting_tool

-	Candidatures are requested under: https://portal.3gpp.org/VotingTool/ (EOL login required).

-	3GPP working procedures: see https://www.3gpp.org/ftp/Information/Working_Procedures

-	Remember to consult the voting list and to check in before voting.

The elections of the new RAN4 leadership team (1 Chair) during RAN4#114-bis

-	was led by RAN4 current Chair, Dr. Xizeng Dai, Huawei (CCSA)

-	who referred to the latest changes of the 3GPP Working Procedures

Candidates for RAN4 Chair position:

-	Mr. Gene FONG, Qualcomm Incorporated (ATIS), CorpOPD: ATIS

-	Dr. Yang TANG Apple Inc (ATIS), CorpOPD: ATIS

On April 08 (Tuesday) morning, RAN4 current Chair: Dr. Xizeng Dai, Huawei (CCSA)

-	asked TSG RAN4 participants whether there is any other candidate for the RAN4 Chair position

-	apart from the two indicated above.

On April 08 (Tuesday) 09:00~13:45 (GMT+8, Beijing time), the first ballot. The announcement of results will be held at 14:00 in main session

-	the RAN4 Chair position was carried out with the following result:

-	Number of Individual Members with the right to vote: xxx.

-	xx votes in person, xx votes by proxy and xx abstentions

-	so, the quorum (xx%) was reached and the following result was achieved:

-	Mr. Gene FONG, Qualcomm Incorporated (ATIS), CorpOPD: ATIS

-	received xxx votes (xxx%).

-	Dr. Yang TANG Apple Inc (ATIS), CorpOPD: ATIS

-	received xxx votes (xxx%).

## 4	Incoming LS

R4-2503103	LS on Rel-18 RAN1 UE features list for NR after RAN1#120

Type: LS in		For: Information
					Original outgoing LS: R1-2501390, to RAN2, cc RAN4
					Source: RAN1

Decision: 		The document was not treated.

R4-2503104	LS on Rel-19 RAN1 UE features list for NR after RAN1#120

Type: LS in		For: Information
					Original outgoing LS: R1-2501399, to RAN2, cc RAN4
					Source: RAN1

Decision: 		The document was not treated.

R4-2503105	LS on measurement quantities for IoT NTN

Type: LS in		For: Information
					Original outgoing LS: R1-2501512, to RAN4, cc -
					Source: RAN1

Decision: 		The document was not treated.

R4-2503106	Reply LS on CSI-RS measurement with SBFD operation

Type: LS in		For: Information
					Original outgoing LS: R1-2501560, to RAN4, cc RAN2
					Source: RAN1

Decision: 		The document was not treated.

R4-2503107	Reply LS on Ku band numerology

Type: LS in		For: Information
					Original outgoing LS: R1-2501609, to RAN4, cc RAN, RAN2
					Source: RAN1

Decision: 		The document was not treated.

R4-2503108	LS on L1 based UE-to-UE CLI measurement and reporting

Type: LS in		For: Information
					Original outgoing LS: R1-2501620, to RAN4, cc -
					Source: RAN1

Decision: 		The document was not treated.

R4-2503109	LS on TDM DMRS for 3.75kHz SCS OCC

Type: LS in		For: Information
					Original outgoing LS: R1-2501622, to RAN4, cc -
					Source: RAN1

Decision: 		The document was not treated.

R4-2503110	LS on the wake-up delay for LP-WUS operation in IDLE/INACTIVE mode

Type: LS in		For: Information
					Original outgoing LS: R1-2501624, to RAN4, cc -
					Source: RAN1

Decision: 		The document was not treated.

R4-2503111	LS on time location of on-demand SSB for SCell

Type: LS in		For: Information
					Original outgoing LS: R1-2501633, to RAN4, cc RAN2
					Source: RAN1

Decision: 		The document was not treated.

R4-2503112	LS on Rel-19 higher layers parameters list Post RAN1#120

Type: LS in		For: Information
					Original outgoing LS: R1-2501644, to RAN2, RAN3, cc RAN4
					Source: RAN1

Decision: 		The document was not treated.

R4-2503113	Reply LS on AdditionalSpectrumEmission

Type: LS in		For: Information
					Original outgoing LS: R2-2501383, to RAN4, cc -
					Source: RAN2

Decision: 		The document was not treated.

R4-2503114	Reply LS on LP-WUS subgrouping

Type: LS in		For: Information
					Original outgoing LS: R2-2501388, to SA2, RAN3, CT1, cc RAN1, RAN4
					Source: RAN2

Decision: 		The document was not treated.

R4-2503115	LS to RAN4 on granularity definition of pdcch-RACH-AffectedBandsList-r18

Type: LS in		For: Information
					Original outgoing LS: R2-2501392, to RAN4, cc -
					Source: RAN2

Decision: 		The document was not treated.

R4-2503116	Reply LS on LP-WUS operation in IDLE/INACTIVE mode

Type: LS in		For: Information
					Original outgoing LS: R2-2501394, to RAN1, RAN4, cc -
					Source: RAN2

Decision: 		The document was not treated.

R4-2503117	Reply LS on SSB adaptation

Type: LS in		For: Information
					Original outgoing LS: R2-2501484, to RAN4, cc RAN1
					Source: RAN2

Decision: 		The document was not treated.

R4-2503118	Reply LS on Ku band numerology

Type: LS in		For: Information
					Original outgoing LS: R2-2501530, to RAN4, cc RAN, RAN1
					Source: RAN2

Decision: 		The document was not treated.

R4-2503119	Response LS on simultaneous operation between GNSS and NR NTN

Type: LS in		For: Information
					Original outgoing LS: R2-2501568, to RAN4, cc RAN1
					Source: RAN2

Decision: 		The document was not treated.

R4-2503120	Reply LS on UE capability for FDD-FDD inter-band CA simultaneous Rx/Tx

Type: LS in		For: Information
					Original outgoing LS: R2-2501572, to RAN4, cc RAN1
					Source: RAN2

Decision: 		The document was not treated.

R4-2503121	Reply LS on L3 measurement based LTM support over F1

Type: LS in		For: Information
					Original outgoing LS: R3-250879, to RAN2, cc RAN, RAN4
					Source: RAN3

Decision: 		The document was not treated.

R4-2503122	LS Reply on Parameters for 14800 to 15350 MHz of terrestrial component of IMT for sharing and compatibility studies in preparation for WRC-27

Type: LS in		For: Information
					Original outgoing LS: RP-250086, to ITU-R WP 5D, cc RAN4
					Source: RAN

Decision: 		The document was not treated.

R4-2503123	Reply LS on L3 measurement based LTM support over F1

Type: LS in		For: Information
					Original outgoing LS: RP-250790, to RAN3, cc RAN2,RAN4
					Source: RAN

Decision: 		The document was not treated.

## 5	Rel-19 maintenance for LTE and NR and Rel-18 TEI18

### 5.1	Moderator summary and conclusions (for Agenda 5)

#### 5.1.1	Main session

R4-2504659	Topic summary for [114bis][101] R19_UERF_maintenance_TEI

Type: other		For: Information
					Source: Moderator(Huawei)

Abstract:

Summary for AI 5.2.1, 5.2.2, 5.2.3, 5.2.4, 5.2.6, 5.2.7, 5.3.1

Decision: 		The document was not treated.

#### 5.1.2	RRM session

#### 5.1.3	BDaT sessoin

R4-2504626	Topic summary for [114bis][301] BSRF_Maintenance_TEI

Type: other		For: Information
					Source: Moderator (Ericsson)

Abstract:

[114bis] BDaT Session AI 5.3.3 (BSRF related). MCC: The session chair marked this as withdrawn.

Decision: 		The document was withdrawn.

R4-2504644	Topic summary for [114bis][319] Demod_Maintenance_TEI

Type: other		For: Information
					Source: Moderator (Nokia)

Abstract:

[114bis] BDaT Session AI 5.3.3 (demod related). MCC: The session chair marked this as withdrawn.

Decision: 		The document was withdrawn.

R4-2504652	Topic summary for [114bis][327] OTA_Maintenance_TEI

Type: other		For: Information
					Source: Moderator (Keysight)

Abstract:

[114bis] BDaT Session AI 5.3.3 (OTA related). MCC: The session chair marked this as withdrawn.

Decision: 		The document was withdrawn.

### 5.2	Rel-19 spectrum related WI maintenance

#### 5.2.1	Introduction of the 1.4 GHz Band

R4-2503490	draftCR to TS 38.104 on sync raster correction for band n110

Type: draftCR		For: Endorsement
					38.104 v19.0.0	  CR-  rev  Cat: F (Rel-19)

					Source: CATT

Decision: 		The document was not treated.

R4-2503491	draftCR to TS 38.101-1 on sync raster correction for band n110

Type: draftCR		For: Endorsement
					38.101-1 v19.1.0	  CR-  rev  Cat: F (Rel-19)

					Source: CATT

Decision: 		The document was not treated.

#### 5.2.2	Introduction of LTE FDD band in 1800–1830 MHz for Canada

#### 5.2.3	Introduction of NR bands n87 and n88

#### 5.2.4	Introduction of NR band n68

R4-2503513	Draft CR to TS 38.101-1 - Correction of A-MPR tables for band n68

Type: draftCR		For: Endorsement
					38.101-1 v19.1.0	  CR-  rev  Cat:  (Rel-19)

					Source: Qualcomm Technologies

Abstract:

Corrections and clarifications for A-MPR tables for band n68

Decision: 		The document was not treated.

#### 5.2.5	Introduction of additional operating NR bands for HAPS (High Altitude Platform Station)

#### 5.2.6	Introduction of Power Class 2 and UE 40MHz Channel Bandwidth in NR band n28

#### 5.2.7	Others

R4-2503162	Draft CR to TS 38.922 on corrections and clarifications on IMT parameters

Type: draftCR		For: Endorsement
					38.922 v19.0.0	  CR-  rev  Cat: F (Rel-19)

					Source: Nokia

Abstract:

1) Remove the verbal form ‘shall’ from the TR and align the wordings for the BS blocking response in the three frequency ranges

2) Clarify only out-of-band blocking radiated requirements in listed in Table 6.3.2.3-1.

Decision: 		The document was not treated.

R4-2503480	CR to TR 38.922 on power dynamic range

Type: draftCR		For: Endorsement
					38.922 v19.0.0	  CR-  rev  Cat: F (Rel-19)

					Source: CATT

Decision: 		The document was not treated.

R4-2504265	Draft CR to TR 38.922 Removal of references to internal TRs and Tdocs

Type: draftCR		For: Endorsement
					38.922 v19.0.0	  CR-  rev  Cat: D (Rel-19)

					Source: Ericsson

Abstract:

This CR removes the references to internal TRs and TDOCs based on RAN information.

Decision: 		The document was not treated.

R4-2504266	Draft CR to TR 38.922 Addition of AAS BS spectral mask and spurious emissions for 14800 to 15350 MHz frequency range

Type: draftCR		For: Endorsement
					38.922 v19.0.0	  CR-  rev  Cat: F (Rel-19)

					Source: Ericsson

Abstract:

This CR adds the AAS BS spectral mask and spurious emissions for 14800 to 15350 MHz frequency range based on the last meeting's agreements.

Decision: 		The document was not treated.

### 5.3	Rel-18 TEI

#### 5.3.1	UE RF related topics

R4-2503175	On the proposed FR2 PC8 (new power class) UE

Type: other		For: Approval
					Source: Qualcomm Incorporated

Abstract:

The PC8 proposal could benefit from improvement in spherical coverage compared to PC7 (RedCap)

Decision: 		The document was not treated.

R4-2503272	(TEI18) DraftCR on 38.101-1 clarifications on special sync rasters for n100 [Sync_raster_3MHz_n100]

Type: draftCR		For: Endorsement
					38.101-1 v18.9.0	  CR-  rev  Cat: F (Rel-18)

					Source: Huawei Technologies France

Decision: 		The document was not treated.

R4-2503273	(TEI18) DraftCR on 38.101-1 clarifications on special sync rasters for n100 [Sync_raster_3MHz_n100]

Type: draftCR		For: Endorsement
					38.101-1 v19.1.0	  CR-  rev  Cat: A (Rel-19)

					Source: Huawei Technologies France

Decision: 		The document was not treated.

R4-2503562	n1 NS_05 and NS_05U PC2 A-MPR proposal

Type: other		For: Approval
					Source: Qualcomm Incorporated

Abstract:

Vote for Gene!

Decision: 		The document was not treated.

R4-2503717	(NR_newRAT-Core) Further study on NR A-MPR reevaluation scenarios on n1 for Japan

Type: discussion		For: (not specified)
					Source: KDDI Corporation

Decision: 		The document was not treated.

R4-2503826	Draft CR to 38.101-5 on UE channel bandwidth for FR2-NTN

Type: draftCR		For: Endorsement
					38.101-5 v18.9.0	  CR-  rev  Cat: F (Rel-18)

					Source: vivo

Decision: 		The document was not treated.

R4-2504078	Introducing Tx switching for 3Tx UEs in Rel-19 TEI

Type: discussion		For: Discussion
					Source: MediaTek Inc., Orange, T-Mobile USA, Vodafone, Verizon

Decision: 		The document was not treated.

R4-2504079	CR 38.101-1 on time mask of Tx switching for 3Tx UEs [3TxSwitching]

Type: draftCR		For: Endorsement
					38.101-1 v18.9.0	  CR-  rev  Cat: B (Rel-18)

					Source: MediaTek inc.

Decision: 		The document was not treated.

R4-2504164	(NR_MC_enh) R18 correction of switchig time between NUL and SUL carriers

Type: draftCR		For: Endorsement
					38.101-1 v18.9.0	  CR-  rev  Cat: F (Rel-18)

					Source: OPPO

Decision: 		The document was not treated.

R4-2504165	(NR_MC_enh) R19 correction of switchig time between NUL and SUL carriers

Type: draftCR		For: Endorsement
					38.101-1 v19.1.0	  CR-  rev  Cat: A (Rel-19)

					Source: OPPO

Decision: 		The document was not treated.

R4-2504166	R16 n1 NS_05 PC2 AMPR

Type: other		For: Approval
					Source: OPPO

Decision: 		The document was not treated.

R4-2504167	R16 n1 NS_05U PC2 and PC3 AMPR

Type: other		For: Approval
					Source: OPPO

Decision: 		The document was not treated.

R4-2504180	Discussion on introducing Tx switching for 4Tx FWA

Type: discussion		For: Discussion
					Source: Huawei,HiSilicon

Decision: 		The document was not treated.

R4-2504184	On the options proposed for PC8

Type: other		For: Approval
					Source: Ericsson

Abstract:

In this contribution we discuss the options for PC8 in the proposed WF in R4-2502994. It is proposed that PC8 is not introduced.

Decision: 		The document was not treated.

R4-2504322	Discussion on introduction of FR2 PC8

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon, CTC, CEPRI, OPPO, vivo, Xiaomi, CATT, Spreadtrum, Google, Honor, TD Tech, StarPoint, SRTC

Decision: 		The document was not treated.

R4-2504323	CR on introduction of RF requirements for FR2 PC8 in TS 38.101-2 [FR2_PC8]

Type: CR		For: Endorsement
					38.101-2 v18.9.0	  CR-0790  rev  Cat: B (Rel-18)

					Source: Huawei, HiSilicon, CTC, CEPRI, OPPO, vivo, Xiaomi, CATT, Spreadtrum, Google, Honor, TD Tech, StarPoint, SRTC

Abstract:

All CRs are only for endorsement at RAN4#114-bis and need to be resubmitted at RAN4#115 for agreement.

Decision: 		The document was not treated.

R4-2504324	CR on introduction of RRM requirements for FR2 PC8 in TS 38.133 [FR2_PC8]

Type: CR		For: Endorsement
					38.133 v18.9.0	  CR-5447  rev  Cat: B (Rel-18)

					Source: Huawei, HiSilicon, CTC, CEPRI, OPPO, vivo, Xiaomi, CATT, Spreadtrum, Google, Honor, TD Tech, StarPoint, SRTC

Abstract:

All CRs are only for endorsement at RAN4#114-bis and need to be resubmitted at RAN4#115 for agreement. Parsing Failure: Datas could not be saved in database

Decision: 		The document was not treated.

R4-2504325	CR on release independence for FR2 PC8 in TS 38.307 [FR2_PC8]

Type: CR		For: Endorsement
					38.307 v18.3.0	  CR-0182  rev  Cat: B (Rel-18)

					Source: Huawei, HiSilicon, CTC, CEPRI, OPPO, vivo, Xiaomi, CATT, Spreadtrum, Google, Honor, TD Tech, StarPoint, SRTC

Abstract:

All CRs are only for endorsement at RAN4#114-bis and need to be resubmitted at RAN4#115 for agreement.

Decision: 		The document was not treated.

R4-2504326	LS on introduction of FR2 PC8

Type: LS out		For: Approval
					to RAN2
					Source: Huawei, HiSilicon

Decision: 		The document was not treated.

R4-2504558	(PHS_System_NRonly) On the requirements of modifiedMPR-Behavior

Type: other		For: Approval
					Source: Huawei, HiSilicon

Decision: 		The document was not treated.

R4-2504569	A-MPR regions for NS_05/NS_05U and optionality in earlier releases by modified MPR behaviour

Type: other		For: Approval
					Source: Ericsson

Abstract:

In this contribution we discuss the updated A-MPR tables, propose changes to the regions and the support of the updated A-MPR in earlier releases.

Decision: 		The document was not treated.

R4-2504570	Draft CR to 38.101-1: Modification of additional requirements indicated by NS_05/NS_05U

Type: draftCR		For: Endorsement
					38.101-1 v16.23.0	  CR-  rev  Cat: C (Rel-16)

					Source: Ericsson, Samsung

Abstract:

CR to introduce a modified MPR bit for the new NS_05/NS_05U in Japan.

Decision: 		The document was not treated.

R4-2504571	Draft CR to 38.101-1: Modification of additional requirements indicated by NS_05/NS_05U

Type: draftCR		For: Endorsement
					38.101-1 v19.1.0	  CR-  rev  Cat: A (Rel-19)

					Source: Ericsson, Samsung

Abstract:

Draft CR to introduce modified emissions limits indicated by NS_05, remove PHS requirements and introduce a modified MPR bit for the new NS_05/NS_05U in Japan.

Decision: 		The document was not treated.

R4-2504609	3TX chain TX switching requirements

Type: other		For: Approval
					Source: Qualcomm Incorporated

Abstract:

Vote for Gene!

Decision: 		The document was not treated.

#### 5.3.2	RRM related topics

R4-2504080	CR 38.133 on interruption of Tx switching for 3Tx UEs [3TxSwitching]

Type: draftCR		For: Endorsement
					38.133 v18.9.0	  CR-  rev  Cat: B (Rel-18)

					Source: MediaTek inc.

Decision: 		The document was not treated.

#### 5.3.3	BS RF, demodulation performance and other topics

## 6	Rel-19 on-going spectrum related work items for NR and LTE

### 6.1	Moderator summary and conclusions (for Agenda 6)

R4-2504642	Topic summary for [114bis][317] 5G_Broadcast_bands

Type: other		For: Information
					Source: Moderator (EBU)

Abstract:

[114bis] BDaT Session AI 6.21.1, 6.21.2, 6.21.3, 6.21.4

Decision: 		The document was not treated.

R4-2504660	Topic summary for [114bis][102] NR_Baskets_Part_1

Type: other		For: Information
					Source: Moderator(Nokia)

Abstract:

Summary for AI 6.2

Decision: 		The document was not treated.

R4-2504661	Topic summary for [114bis][103] NR_Baskets_Part_2

Type: other		For: Information
					Source: Moderator(Ericsson)

Abstract:

Summary for AI 6.3

Decision: 		The document was not treated.

R4-2504662	Topic summary for [114bis][104] LTE_Baskets

Type: other		For: Information
					Source: Moderator(Huawei)

Abstract:

Summary for AI 6.4

Decision: 		The document was not treated.

R4-2504663	Topic summary for [114bis][105] HPUE_NR_bands

Type: other		For: Information
					Source: Moderator(CMCC)

Abstract:

Summary for AI 6.5

Decision: 		The document was not treated.

R4-2504664	Topic summary for [114bis][106] LTE_NR_HPUE_FWVM

Type: other		For: Information
					Source: Moderator(Nokia)

Abstract:

Summary for AI 6.6. MCC: The moderator (Nokia) stated this should be withdrawn.

Decision: 		The document was withdrawn.

R4-2504665	Topic summary for [114bis][107] HPUE_Basket_EN-DC

Type: other		For: Information
					Source: Moderator(China Unicom)

Abstract:

Summary for AI 6.7

Decision: 		The document was not treated.

R4-2504666	Topic summary for [114bis][108] HPUE_Basket_CADC_SUL

Type: other		For: Information
					Source: Moderator(China Telecom)

Abstract:

Summary for AI 6.8

Decision: 		The document was not treated.

R4-2504667	Topic summary for [114bis][109] LTE_NR_Other_basket

Type: other		For: Information
					Source: Moderator(Huawei)

Abstract:

Summary for AI 6.9, 6.10, 6.11, 6.12

Decision: 		The document was not treated.

R4-2504668	Topic summary for [114bis][110] NR_IoT_NTN_Bands

Type: other		For: Information
					Source: Moderator(Aalyria)

Abstract:

Summary for AI 6.13, 6.14, 6.15

Decision: 		The document was not treated.

R4-2504669	Topic summary for [114bis][111] NR_mmWave_protect

Type: other		For: Information
					Source: Moderator(China Unicom)

Abstract:

Summary for AI 6.17

Decision: 		The document was not treated.

### 6.2	Rel-19 DC of x LTE band(s), y NR band(s) (x<=6) and single or two NR SUL bands

#### 6.2.1	Rapporteur input (WID/TR/big CR)

R4-2503135	TR37.719-21-11 v0.5.0 for DC_R19_xBLTE_yBNR_3DL2UL

Type: draft TR		For: Agreement
					37.719-21-11 v0.5.0	  CR-  rev  Cat:  (Rel-19)

					Source: LG Electronics Deutschland

Abstract:

TR37.719-21-11 v0.5.0 for DC_R19_xBLTE_yBNR_3DL2UL for email agreement

Decision: 		The document was not treated.

R4-2504094	TR 37.719-11-11 v0.4.0 Rel-19 Dual Connectivity of EN-DC and NE-DC configurations consisting of 2 different bands downlink (DL) with 2 different bands uplink (UL) (1 LTE band and 1 NR band)

Type: draft TR		For: Agreement
					37.719-11-11 v0.4.0	  CR-  rev  Cat:  (Rel-19)

					Source: CHTTL

Decision: 		The document was not treated.

R4-2504097	draft BigCR for DC_R19_xBLTE_yBNR (OBJ-1 DC_R19_1BLTE_1BNR_2DL2UL)

Type: draftCR		For: Endorsement
					38.101-3 v19.1.0	  CR-  rev  Cat: B (Rel-19)

					Source: CHTTL

Decision: 		The document was not treated.

R4-2504122	Big draftCR for DC_R19_xBLTE_yBNR (OBJ-3 DC_R19_xBLTE_yBNR_zDLqUL)

Type: draftCR		For: Endorsement
					38.101-3 v19.1.0	  CR-  rev  Cat: B (Rel-19)

					Source: Nokia

Abstract:

To capture agreed combinations at RAN4#114bis

Decision: 		The document was not treated.

R4-2504123	Big draftCR for DC_R19_xBLTE_yBNR

Type: draftCR		For: Endorsement
					38.101-3 v19.1.0	  CR-  rev  Cat: B (Rel-19)

					Source: Nokia, CHTTL, LG Electronics, Samsung

Abstract:

To capture agreed combinations at RAN4#114bis - draftCR since this is a bis meeting

Decision: 		The document was not treated.

#### 6.2.2	UE RF requirements for EN-DC and NE-DC of 2 DL with 2 UL (DC_R19_1BLTE_1BNR_2DL2UL)

R4-2503241	DC_(n)3CA MSD evaluation

Type: other		For: Approval
					38.101-3 v	  CR-  rev  Cat:  (Rel-19)

					Source: Skyworks Solutions Inc.

Abstract:

1UL MSD is evaluated for DL DC_(n)3CA

Decision: 		The document was not treated.

R4-2503242	Clarifications on valid 1UL E-UTRA configuration for EN-DC intra-band contiguous MSD

Type: other		For: Approval
					38.101-3 v	  CR-  rev  Cat:  (Rel-19)

					Source: Skyworks Solutions Inc.

Abstract:

This document proposes to send an LS to RAN clarifying that the E-UTRA "N/A" UL configurations specified by RAN4 for EN-DC intra-band contiguous MSD test points are intended to be verified using the carrier agnostic approach.

Decision: 		The document was not treated.

R4-2503367	TP for TR 37.719-11-11 DC_(n)3

Type: pCR		For: Approval
					37.719-11-11 v0.3.0	  CR-  rev  Cat:  (Rel-19)

					Source: Samsung, KT, [Murata, Skyworks, Apple, Qualcomm]

Decision: 		The document was not treated.

R4-2503682	Draft CR for TS 38.101-3 to introduce DC_3C_n71A

Type: draftCR		For: Endorsement
					38.101-3 v19.1.0	  CR-  rev  Cat:  (Rel-19)

					Source: Huawei, HiSilicon

Decision: 		The document was not treated.

R4-2503683	TP for TR 37.719-11-11 on introduction of DC_28A_n71A

Type: pCR		For: Approval
					37.719-11-11 v0.3.0	  CR-  rev  Cat:  (Rel-19)

					Source: Huawei, HiSilicon

Decision: 		The document was not treated.

R4-2503684	TP for TR 37.719-11-11 on introduction of DC_8A_n71A

Type: pCR		For: Approval
					37.719-11-11 v0.3.0	  CR-  rev  Cat:  (Rel-19)

					Source: Huawei, HiSilicon

Decision: 		The document was not treated.

R4-2503724	DC_(n)3CA configuration and MSD

Type: discussion		For: Discussion
					Source: Murata Manufacturing Co Ltd.

Decision: 		The document was not treated.

R4-2504140	TP to TR 37.719-11-11 addition of DC_40A_n28A

Type: pCR		For: Approval
					37.719-11-11 v0.3.0	  CR-  rev  Cat:  (Rel-19)

					Source: Nokia, Telefonica

Decision: 		The document was not treated.

#### 6.2.3	UE RF requirements for EN-DC and NE-DC of 2 LTE and 1 NR, or of 1 LTE and 2 NR (DC_R19_xBLTE_yBNR_3DL2UL)

R4-2503155	draftCR to 38.101-3 to include EN-DC band combinations (DC_R19_xBLTE_yBNR_3DL2UL)

Type: draftCR		For: Endorsement
					38.101-3 v19.1.0	  CR-  rev  Cat: B (Rel-19)

					Source: Huawei, Hisilicon

Decision: 		The document was not treated.

R4-2503368	Draft CR for TS 38.101-3 to add 3-band higher order ENDC configurations

Type: draftCR		For: Endorsement
					38.101-3 v19.1.0	  CR-  rev  Cat: B (Rel-19)

					Source: Samsung, KT

Decision: 		The document was not treated.

R4-2503676	TP for TR 37.719-21-11 on introduction of DC_1A_n40A-n71A

Type: pCR		For: Approval
					37.719-21-11 v0.4.0	  CR-  rev  Cat:  (Rel-19)

					Source: Huawei, HiSilicon

Decision: 		The document was not treated.

R4-2503677	TP for TR 37.719-21-11 on introduction of DC_3A_n40A-n71A DC_3C_n40A-n71A

Type: pCR		For: Approval
					37.719-21-11 v0.4.0	  CR-  rev  Cat:  (Rel-19)

					Source: Huawei, HiSilicon

Decision: 		The document was not treated.

R4-2503678	TP for TR 37.719-21-11 on introduction of DC_1A_n71A-n77A

Type: pCR		For: Approval
					37.719-21-11 v0.4.0	  CR-  rev  Cat:  (Rel-19)

					Source: Huawei, HiSilicon

Decision: 		The document was not treated.

R4-2503679	TP for TR 37.719-21-11 on introduction of DC_3A_n71A-n77A DC_3C_n71A-n77A

Type: pCR		For: Approval
					37.719-21-11 v0.4.0	  CR-  rev  Cat:  (Rel-19)

					Source: Huawei, HiSilicon

Decision: 		The document was not treated.

R4-2503681	Draft CR for TS 38.101-3 to introduce DC_1A-3C_n71A

Type: draftCR		For: Endorsement
					38.101-3 v19.1.0	  CR-  rev  Cat:  (Rel-19)

					Source: Huawei, HiSilicon

Decision: 		The document was not treated.

R4-2503685	TP for TR 37.719-21-11 on introduction of DC_8A-28A_n71A

Type: pCR		For: Approval
					37.719-21-11 v0.4.0	  CR-  rev  Cat:  (Rel-19)

					Source: Huawei, HiSilicon

Decision: 		The document was not treated.

R4-2504141	TP to TR 37.719-21-11 addition of DC_1A-40A_n28A

Type: pCR		For: Approval
					37.719-21-11 v0.4.0	  CR-  rev  Cat:  (Rel-19)

					Source: Nokia, Telefonica

Decision: 		The document was not treated.

R4-2504142	TP to TR 37.719-21-11 addition of DC_20A-38A_n28A

Type: pCR		For: Approval
					37.719-21-11 v0.4.0	  CR-  rev  Cat:  (Rel-19)

					Source: Nokia, Telefonica

Decision: 		The document was not treated.

R4-2504143	TP to TR 37.719-21-11 addition of DC_20A-40A_n28A

Type: pCR		For: Approval
					37.719-21-11 v0.4.0	  CR-  rev  Cat:  (Rel-19)

					Source: Nokia, Telefonica

Decision: 		The document was not treated.

R4-2504144	TP to TR 37.719-21-11 addition of DC_28A-40A_n1A

Type: pCR		For: Approval
					37.719-21-11 v0.4.0	  CR-  rev  Cat:  (Rel-19)

					Source: Nokia, Telefonica

Decision: 		The document was not treated.

R4-2504145	TP to TR 37.719-21-11 addition of DC_3A-38A_n1A

Type: pCR		For: Approval
					37.719-21-11 v0.4.0	  CR-  rev  Cat:  (Rel-19)

					Source: Nokia, Telefonica

Decision: 		The document was not treated.

R4-2504146	TP to TR 37.719-21-11 addition of DC_3A-40A_n28A

Type: pCR		For: Approval
					37.719-21-11 v0.4.0	  CR-  rev  Cat:  (Rel-19)

					Source: Nokia, Telefonica

Decision: 		The document was not treated.

R4-2504147	TP to TR 37.719-21-11 addition of DC_8A-28A_n1A

Type: pCR		For: Approval
					37.719-21-11 v0.4.0	  CR-  rev  Cat:  (Rel-19)

					Source: Nokia, Telefonica

Decision: 		The document was not treated.

R4-2504148	TP to TR 37.719-21-11 addition of DC_8A-38A_n28A

Type: pCR		For: Approval
					37.719-21-11 v0.4.0	  CR-  rev  Cat:  (Rel-19)

					Source: Nokia, Telefonica

Decision: 		The document was not treated.

R4-2504149	TP to TR 37.719-21-11 addition of DC_8A-40A_n28A

Type: pCR		For: Approval
					37.719-21-11 v0.4.0	  CR-  rev  Cat:  (Rel-19)

					Source: Nokia, Telefonica

Decision: 		The document was not treated.

R4-2504419	TP to TR 37.719-21-11 addition of DC_8A-38A_n78A

Type: pCR		For: Approval
					37.719-21-11 v0.4.0	  CR-  rev  Cat:  (Rel-19)

					Source: Nokia, Telefonica

Decision: 		The document was not treated.

R4-2504420	TP to TR 37.719-21-11 addition of DC_38A-40A_n1A

Type: pCR		For: Approval
					37.719-21-11 v0.4.0	  CR-  rev  Cat:  (Rel-19)

					Source: Nokia, Telefonica

Decision: 		The document was not treated.

R4-2504421	TP to TR 37.719-21-11 addition of DC_38A-40A_n28A

Type: pCR		For: Approval
					37.719-21-11 v0.4.0	  CR-  rev  Cat:  (Rel-19)

					Source: Nokia, Telefonica

Decision: 		The document was not treated.

R4-2504489	Draft CR on TS 38.101-3 to correct EN-DC configurations for DC_1-28_n77 and DC_8_n28-n77

Type: draftCR		For: Endorsement
					38.101-3 v19.1.0	  CR-  rev  Cat: F (Rel-19)

					Source: ZTE Corporation, Sanechips

Abstract:

Objective: DC_R19_xBLTE_yBNR_3DL2UL

Decision: 		The document was not treated.

#### 6.2.4	UE RF requirements for EN-DC and NE-DC of x LTE and y NR with total z DL bands and q UL bands (DC_R19_xBLTE_yBNR_zDLqUL)

R4-2503154	draftCR to 38.101-3 to include EN-DC band combinations(DC_R19_xBLTE_yBNR_zDLqUL)

Type: draftCR		For: Endorsement
					38.101-3 v19.1.0	  CR-  rev  Cat: B (Rel-19)

					Source: Huawei, Hisilicon

Decision: 		The document was not treated.

R4-2503369	Draft CR for TS 38.101-3 to add 4-band 5-band higher oder ENDC configurations

Type: draftCR		For: Endorsement
					38.101-3 v19.1.0	  CR-  rev  Cat: B (Rel-19)

					Source: Samsung, KT

Decision: 		The document was not treated.

R4-2503680	Draft CR for TS 38.101-3 to introduce DC_1A-3A_n40A-n71A/DC_1A-3C_n40A-n71A/DC_1A-3A_n71A-n77A/DC_1A-3C_n71A-n77A

Type: draftCR		For: Endorsement
					38.101-3 v19.1.0	  CR-  rev  Cat:  (Rel-19)

					Source: Huawei, HiSilicon

Decision: 		The document was not treated.

R4-2504490	Draft CR on TS 38.101-3 to add missing EN-DC configurations for delta TIB and RIB

Type: draftCR		For: Endorsement
					38.101-3 v19.1.0	  CR-  rev  Cat: F (Rel-19)

					Source: ZTE Corporation, Sanechips

Abstract:

Objective: DC_R19_xBLTE_yBNR_zDLqUL

Decision: 		The document was not treated.

R4-2504491	Draft CR on TS 38.101-3 to introduce new BC for DC_3-20-28_n7

Type: draftCR		For: Endorsement
					38.101-3 v19.1.0	  CR-  rev  Cat: B (Rel-19)

					Source: ZTE Corporation, Sanechips

Abstract:

Objective: DC_R19_xBLTE_yBNR_zDLqUL

Decision: 		The document was not treated.

#### 6.2.5	UE RF requirements for EN-DC and NE-DC with one SUL and two SULs (DC_R19_LTE_NR_SUL_combos)

### 6.3	Rel-19 NR CA/DC for x bands DL with y bands UL (x<7, y<3) and SUL/CA band combinations with a single SUL or two SUL cells

#### 6.3.1	Rapporteur input (WID/TR/big CR)

R4-2503674	Draft Big CR on Introduction of completed SUL band combinations into TS 38.101-1

Type: draftCR		For: Endorsement
					38.101-1 v19.1.0	  CR-  rev  Cat:  (Rel-19)

					Source: Huawei, HiSilicon

Decision: 		The document was not treated.

R4-2503675	TR for 38.719-00-00 v0.2.0

Type: draft TR		For: Agreement
					38.719-00-00 v0.2.0	  CR-  rev  Cat:  (Rel-19)

					Source: Huawei, HiSilicon

Decision: 		The document was not treated.

R4-2503733	draft big CR 38.101-1  for NR_CADC_R19_2BDL_xBUL

Type: draftCR		For: Endorsement
					38.101-1 v19.1.0	  CR-  rev  Cat: B (Rel-19)

					Source: ZTE Corporation, Sanechips

Decision: 		The document was not treated.

R4-2503734	draft big CR 38.101-2 for NR_CADC_R19_2BDL_xBUL

Type: draftCR		For: Endorsement
					38.101-2 v19.0.0	  CR-  rev  Cat: B (Rel-19)

					Source: ZTE Corporation, Sanechips

Decision: 		The document was not treated.

R4-2503955	draft big CR 38.101-1 new combinations Rel-19 NR Intra-band

Type: draftCR		For: Endorsement
					38.101-1 v19.1.0	  CR-  rev  Cat: B (Rel-19)

					Source: Ericsson

Abstract:

draft big CR 38.101-1 new combinations Rel-19 NR Intra-band

Decision: 		The document was not treated.

R4-2503956	draft big CR 38.101-2 new combinations Rel-19 NR Intra-band

Type: draftCR		For: Endorsement
					38.101-2 v19.0.0	  CR-  rev  Cat: B (Rel-19)

					Source: Ericsson

Abstract:

draft big CR 38.101-2 new combinations Rel-19 NR Intra-band

Decision: 		The document was not treated.

R4-2503957	Rel-19 NR intra-band CA combinations

Type: draft TR		For: Agreement
					38.719-01-01 v0.5.0	  CR-  rev  Cat:  (Rel-19)

					Source: Ericsson

Abstract:

TR 38.719-01-01 v0.5.0 NR_CA_R19_Intra

Decision: 		The document was not treated.

R4-2503981	TR38.719-02-01 v0.5.0_NR_CADC_R19_2BDL_xBUL

Type: draft TR		For: Agreement
					38.719-02-01 v0.5.0	  CR-  rev  Cat:  (Rel-19)

					Source: ZTE Corporation

Decision: 		The document was not treated.

R4-2503982	draft big CR for NR_CADC_SUL_R19(OBJ-2 NR_CADC_R19_2BDL_xBUL)

Type: draftCR		For: Endorsement
					38.101-3 v19.1.0	  CR-  rev  Cat: B (Rel-19)

					Source: ZTE Corporation

Decision: 		The document was not treated.

R4-2504269	draft big CR 38.101-1 for NR_CADC_SUL_R19(OBJ-4 NR_CADC_R19_yBDL_xBUL)

Type: draftCR		For: Endorsement
					38.101-1 v19.1.0	  CR-  rev  Cat: B (Rel-19)

					Source: Ericsson

Abstract:

draft big CR 38.101-1 for NR_CADC_SUL_R19(OBJ-4 NR_CADC_R19_yBDL_xBUL)

Decision: 		The document was not treated.

R4-2504270	draft big CR 38.101-3 for NR_CADC_SUL_R19(OBJ-4 NR_CADC_R19_yBDL_xBUL)

Type: draftCR		For: Endorsement
					38.101-3 v19.1.0	  CR-  rev  Cat: B (Rel-19)

					Source: Ericsson

Abstract:

draft big CR 38.101-3 for NR_CADC_SUL_R19(OBJ-4 NR_CADC_R19_yBDL_xBUL)

Decision: 		The document was not treated.

R4-2504486	TR 38.719-03-01 v0.5.0 on Rel-19 NR Inter-band CA DC configurations including inter band CA for 3 different bands DL with x different bands UL (x=1,2)

Type: draft TR		For: Agreement
					38.719-03-01 v0.5.0	  CR-  rev  Cat:  (Rel-19)

					Source: ZTE Corporation

Abstract:

Objective: NR_CADC_R19_3BDL_xBUL

Decision: 		The document was not treated.

R4-2504487	Draft big CR for NR_CADC_SUL_R19 (OBJ-3 NR_CADC_R19_3BDL_xBUL) into TS 38.101-1

Type: draftCR		For: Endorsement
					38.101-1 v19.1.0	  CR-  rev  Cat: B (Rel-19)

					Source: ZTE Corporation

Abstract:

Objective: NR_CADC_R19_3BDL_xBUL

Decision: 		The document was not treated.

R4-2504488	Draft big CR for NR_CADC_SUL_R19 (OBJ-3 NR_CADC_R19_3BDL_xBUL) into TS 38.101-3

Type: draftCR		For: Endorsement
					38.101-3 v19.1.0	  CR-  rev  Cat: B (Rel-19)

					Source: ZTE Corporation

Abstract:

Objective: NR_CADC_R19_3BDL_xBUL

Decision: 		The document was not treated.

#### 6.3.2	UE RF requirements for NR intra-band CA combinations for x CC DL/y CC UL (NR_CA_R19_Intra with/without UL-MIMO)

R4-2503243	Further MSD clean-up for CA intra-band non-contiguous

Type: other		For: Approval
					Source: Skyworks Solutions Inc.

Decision: 		The document was not treated.

R4-2504520	On (in)valid intra-band non-contiguous UL CA configurations

Type: other		For: Approval
					38.101-1 v	  CR-  rev  Cat:  (Rel-19)

					Source: Skyworks Solutions Inc.

Abstract:

This contribution flags intra-band non-contiguous UL CA configurations that are either invalid and should be removed or may be considered valid but for which specifications are incomplete, in which case numerous changes are needed since Rel-17.

Decision: 		The document was withdrawn.

#### 6.3.3	UE RF requirements for NR inter-band CA/DC configurations including inter band CA for 2 DL with up to 2UL (NR_CADC_R19_2BDL_xBUL)

R4-2503210	Draft CR for TS38.101-1 to add BCS4 and 5 for CA_n77-n79

Type: draftCR		For: Endorsement
					38.101-1 v19.1.0	  CR-  rev  Cat: B (Rel-19)

					Source: SoftBank Corp.

Abstract:

to add BCS4 and 5 for CA_n77-n79

Decision: 		The document was not treated.

R4-2503220	Draft CR for TR 38.101-1 include CA_n20-n41

Type: draftCR		For: Endorsement
					38.101-1 v19.1.0	  CR-  rev  Cat:  (Rel-19)

					Source: Huawei, HiSilicon

Decision: 		The document was not treated.

R4-2503240	Cross-band isolation exception cases and corrections

Type: other		For: Approval
					38.101-1 v	  CR-  rev  Cat:  (Rel-19)

					Source: Skyworks Solutions Inc.

Abstract:

This document flags cross-band isolation MSD requirements that deviate from the agreed PRD rules and proposes 1) new MSD test points for invalid requirements, 2) MSD test point for missing requirements, 3) amendments to PRD rules for the 2nd MSD test poin

Decision: 		The document was not treated.

R4-2503339	Reply LS on UE capability for FDD-FDD inter-band CA simultaneous Rx/Tx

Type: LS out		For: Approval
					to RAN2, cc RAN1
					Source: MediaTek Korea Inc.

Abstract:

RAN4 kindly reply on the RAN2 questions and ask to take above feedback information into account for RAN2 decision to define UE capability if necessary

Decision: 		The document was not treated.

R4-2503482	draft reply LS to RAN2 on UE capability for FDD-FDD inter-band CA simultaneous Rx/Tx

Type: LS out		For: Approval
					to RAN2, cc RAN1
					Source: CATT

Decision: 		The document was not treated.

R4-2503686	TP for TR 38.719-02-01 on introduction of CA_n40A-n71A

Type: pCR		For: Approval
					38.719-02-01 v0.4.0	  CR-  rev  Cat:  (Rel-19)

					Source: Huawei, HiSilicon

Decision: 		The document was not treated.

R4-2503687	TP for TR 38.719-02-01 on introduction of CA_n28A-n71A

Type: pCR		For: Approval
					38.719-02-01 v0.4.0	  CR-  rev  Cat:  (Rel-19)

					Source: Huawei, HiSilicon

Decision: 		The document was not treated.

R4-2503692	Discussion and replied LS on UL CA_n5-n8

Type: other		For: Approval
					Source: Huawei, HiSilicon

Decision: 		The document was not treated.

R4-2503812	Views on RAN2 reply LS on UE capability for FDD-FDD inter-band CA simultaneous Rx-Tx

Type: other		For: Approval
					Source: Apple

Abstract:

MCC: R4-2503812 is moved from AI 8.1 to AI 6.3.3 and treated in [103].

Decision: 		The document was not treated.

R4-2503972	Clarification on ULCA_n41C IMD MSD test point

Type: other		For: Approval
					Source: ZTE Corporation, Sanechips

Decision: 		The document was not treated.

R4-2503973	Reply LS on UE capability for FDD-FDD inter-band CA simultaneous RxTx

Type: LS out		For: Approval
					to RAN2, cc RAN1
					Source: ZTE Corporation, Sanechips

Decision: 		The document was not treated.

R4-2503975	TP for TR38.719-02-01_CA_n40A-n41C

Type: pCR		For: Approval
					38.719-02-01 v0.4.0	  CR-  rev  Cat:  (Rel-19)

					Source: ZTE Corporation, Sanechips

Decision: 		The document was not treated.

R4-2503976	TP for TR38.719-02-01_CA_n41C-n79A

Type: pCR		For: Approval
					38.719-02-01 v0.4.0	  CR-  rev  Cat:  (Rel-19)

					Source: ZTE Corporation, CMCC

Decision: 		The document was not treated.

R4-2503977	draft CR to TS38.101-1_CA_n41A-n104C and CA_n41C-n104A

Type: draftCR		For: Endorsement
					38.101-1 v19.1.0	  CR-  rev  Cat: B (Rel-19)

					Source: ZTE Corporation, Sanechips

Decision: 		The document was not treated.

R4-2504082	Draft CR for TS 38.101-3 to introduce DC band combinations consist of two bands

Type: draftCR		For: Endorsement
					38.101-3 v19.1.0	  CR-  rev  Cat: B (Rel-19)

					Source: Huawei, HiSilicon

Decision: 		The document was not treated.

R4-2504116	CA_n5A-n8A capabilities

Type: other		For: Approval
					Source: Qualcomm France

Abstract:

UE capabilities and other issues for CA_n5A-n8A are discussed in this contribution.

Decision: 		The document was not treated.

R4-2504117	Reply LS on UE capability for FDD-FDD inter-band CA simultaneous Rx/Tx

Type: LS out		For: Approval
					to RAN2, cc RAN1
					Source: Qualcomm France

Decision: 		The document was not treated.

R4-2504124	On LS response to RAN1 on UE capability for non-simultaneous RxTx FDD-FDD inter-band CA

Type: discussion		For: Approval
					Source: Nokia

Decision: 		The document was not treated.

R4-2504133	TP to TR 38.719-02-01 CA_n2-n5

Type: pCR		For: Approval
					38.719-02-01 v0.4.0	  CR-  rev  Cat:  (Rel-19)

					Source: Nokia, Verizon

Decision: 		The document was not treated.

R4-2504134	TP to TR 38.719-02-01 CA_n2-n48

Type: pCR		For: Approval
					38.719-02-01 v0.4.0	  CR-  rev  Cat:  (Rel-19)

					Source: Nokia, Verizon

Decision: 		The document was not treated.

R4-2504135	TP to TR 38.719-02-01 CA_n2-n77

Type: pCR		For: Approval
					38.719-02-01 v0.4.0	  CR-  rev  Cat:  (Rel-19)

					Source: Nokia, Verizon

Decision: 		The document was not treated.

R4-2504136	TP to TR 38.719-02-01 CA_n5-n48

Type: pCR		For: Approval
					38.719-02-01 v0.4.0	  CR-  rev  Cat:  (Rel-19)

					Source: Nokia, Verizon

Decision: 		The document was not treated.

R4-2504137	TP to TR 38.719-02-01 CA_n5-n77

Type: pCR		For: Approval
					38.719-02-01 v0.4.0	  CR-  rev  Cat:  (Rel-19)

					Source: Nokia, Verizon

Decision: 		The document was not treated.

R4-2504138	TP to TR 38.719-02-01 CA_n48-n66

Type: pCR		For: Approval
					38.719-02-01 v0.4.0	  CR-  rev  Cat:  (Rel-19)

					Source: Nokia, Verizon

Decision: 		The document was not treated.

R4-2504139	TP to TR 38.719-02-01 CA_n66-n77

Type: pCR		For: Approval
					38.719-02-01 v0.4.0	  CR-  rev  Cat:  (Rel-19)

					Source: Nokia, Verizon

Decision: 		The document was not treated.

R4-2504412	Draft CR for TS 38.101-1 to add bandwidth combination set 4 and 5 for inter-band CA combinations (two bands)

Type: draftCR		For: Endorsement
					38.101-1 v19.1.0	  CR-  rev  Cat: B (Rel-19)

					Source: Samsung, Boost Mobile Network

Decision: 		The document was not treated.

R4-2504492	TP for TR 38.719-02-01 to correct CA_n20-n77

Type: pCR		For: Approval
					38.719-02-01 v0.5.0	  CR-  rev  Cat:  (Rel-19)

					Source: ZTE Corporation, Sanechips

Abstract:

Objective:

NR_CADC_R19_2BDL_xBUL

Decision: 		The document was not treated.

R4-2504493	Draft CR on TS 38.101-1 to correct inter-band CA configurations for two bands combinations

Type: draftCR		For: Endorsement
					38.101-1 v19.1.0	  CR-  rev  Cat: F (Rel-19)

					Source: ZTE Corporation, Sanechips

Abstract:

Objective:

NR_CADC_R19_2BDL_xBUL

Decision: 		The document was not treated.

R4-2504574	draft CR for TS38.101-1 to add new NR DL CA n78(2A)-n79A with UL n78(2A)

Type: draftCR		For: Endorsement
					38.101-1 v19.1.0	  CR-  rev  Cat: B (Rel-19)

					Source: NTT DOCOMO, INC.

Decision: 		The document was not treated.

R4-2504625	IMD range definition for inter-band MSD due to one band contiguous CA UL configuration

Type: other		For: Approval
					Source: Skyworks Solutions Inc.

Abstract:

A way forward [2] captured this MSD test points, accounting for other companies’ contributions. Additionally, the way forward encouraged companies to provide clarification on the source of IMD for such MSD test points. In this contribution, we provide our

Decision: 		The document was not treated.

#### 6.3.4	UE RF requirements for NR inter-band CA/DC configurations including inter band CA for 3 DL with x UL (NR_CADC_R19_3BDL_xBUL)

R4-2503211	TP for TR38.719-03-01 addition of UL CA_n77(2A) to CA_n1-n3-n77

Type: pCR		For: Approval
					38.719-03-01 v0.4.0	  CR-  rev  Cat:  (Rel-19)

					Source: SoftBank Corp.

Abstract:

addition of UL CA_n77(2A) to CA_n1-n3-n77

Decision: 		The document was not treated.

R4-2503212	TP for TR38.719-03-01 addition of CA_n3-n78-n79

Type: pCR		For: Approval
					38.719-03-01 v0.4.0	  CR-  rev  Cat:  (Rel-19)

					Source: SoftBank Corp., LG Electronics

Abstract:

addition of CA_n3-n78-n79

Decision: 		The document was not treated.

R4-2503213	Draft CR for TS38.101-1 to add BCS4 and 5 for 3CC NRCA combinations

Type: draftCR		For: Endorsement
					38.101-1 v19.1.0	  CR-  rev  Cat: B (Rel-19)

					Source: SoftBank Corp.

Abstract:

to add BCS4 and 5 to CA_n1-n3n-n77, CA_n1-n3-n79, CA_n1-n28-n77, CA_n1-n28-n79, CA_n3-n28-n79 and CA_n28-n77-n79

Decision: 		The document was not treated.

R4-2503221	Draft CR for TS 38101-1 include BCS 4 and 5 for CA configurations of 3BDL

Type: draftCR		For: Endorsement
					38.101-1 v19.1.0	  CR-  rev  Cat:  (Rel-19)

					Source: Huawei, HiSilicon,MTN

Decision: 		The document was not treated.

R4-2503222	draft TP to TR 38.719-03-01 include CA_n3-n78-n79

Type: pCR		For: Approval
					38.719-03-01 v0.4.0	  CR-  rev  Cat:  (Rel-19)

					Source: Huawei, HiSilicon

Decision: 		The document was not treated.

R4-2503223	Draft CR for TS 38101-1 include BCS 4 and 5 for CA configurations of 3BDL

Type: draftCR		For: Endorsement
					38.101-1 v19.1.0	  CR-  rev  Cat:  (Rel-19)

					Source: Huawei, HiSilicon

Decision: 		The document was not treated.

R4-2503688	TP for TR 38.719-03-01 on introduction of CA_n28A-n40A-n71A

Type: pCR		For: Approval
					38.719-03-01 v0.4.0	  CR-  rev  Cat:  (Rel-19)

					Source: Huawei, HiSilicon

Decision: 		The document was not treated.

R4-2503689	TP for TR 38.719-03-01 on introduction of CA_n28A-n71A-n77A CA_n28A-n71A-n77(2A)

Type: pCR		For: Approval
					38.719-03-01 v0.4.0	  CR-  rev  Cat:  (Rel-19)

					Source: Huawei, HiSilicon

Decision: 		The document was not treated.

R4-2503690	TP for TR 38.719-03-01 on introduction of CA_n40A-n71A-n77A CA_n40A-n71A-n77(2A)

Type: pCR		For: Approval
					38.719-03-01 v0.4.0	  CR-  rev  Cat:  (Rel-19)

					Source: Huawei, HiSilicon

Decision: 		The document was not treated.

R4-2503780	Draft CR for TS38.101-1 Rel-19 for adding new NRCA combinations for FR1 for 3DL

Type: draftCR		For: Endorsement
					38.101-1 v19.1.0	  CR-  rev  Cat: B (Rel-19)

					Source: KDDI Corporation (TTC)

Decision: 		The document was not treated.

R4-2503978	TP for TR38.719-03-01_3DL_2UL CA_n3A-n34A-n41A

Type: pCR		For: Approval
					38.719-03-01 v0.4.0	  CR-  rev  Cat:  (Rel-19)

					Source: ZTE Corporation, Sanechips

Decision: 		The document was not treated.

R4-2503979	TP for TR38.719-03-01_3DL_2UL CA_n3A-n34A-n79A

Type: pCR		For: Approval
					38.719-03-01 v0.4.0	  CR-  rev  Cat:  (Rel-19)

					Source: ZTE Corporation, Sanechips

Decision: 		The document was not treated.

R4-2504068	draft TP to TR 38.719-03-01 include CA_n8-n78-n79

Type: pCR		For: Approval
					38.719-03-01 v0.4.0	  CR-  rev  Cat:  (Rel-19)

					Source: Huawei, HiSilicon

Decision: 		The document was not treated.

R4-2504083	Draft CR for TS 38.101-3 to introduce DC band combinations consist of three bands

Type: draftCR		For: Endorsement
					38.101-3 v19.1.0	  CR-  rev  Cat: B (Rel-19)

					Source: Huawei, HiSilicon

Decision: 		The document was not treated.

R4-2504271	TP for 38.719-03-01 to add  UL CA_n2A-n77C and  CA_n5A-n77C for CA_n2-n5-n77

Type: pCR		For: Approval
					38.719-03-01 v0.4.0	  CR-  rev  Cat:  (Rel-19)

					Source: Ericsson, Verizon

Abstract:

TP for 38.719-03-01 to add  UL CA_n2A-n77C and  CA_n5A-n77C for CA_n2-n5-n77

Decision: 		The document was not treated.

R4-2504272	TP for 38.719-03-01 to add  UL CA_n2A-n77C and  CA_n66A-n77C for CA_n2-n66-n77

Type: pCR		For: Approval
					38.719-03-01 v0.4.0	  CR-  rev  Cat:  (Rel-19)

					Source: Ericsson, Verizon

Abstract:

TP for 38.719-03-01 to add  UL CA_n2A-n77C and  CA_n66A-n77C for CA_n2-n66-n77

Decision: 		The document was not treated.

R4-2504273	TP for 38.719-03-01 to add  UL CA_n2A-n77C for CA_n2A-n48A-n77C

Type: pCR		For: Approval
					38.719-03-01 v0.4.0	  CR-  rev  Cat:  (Rel-19)

					Source: Ericsson, Verizon

Abstract:

TP for 38.719-03-01 to add  UL CA_n2A-n77C for CA_n2A-n48A-n77C

Decision: 		The document was not treated.

R4-2504274	TP for 38.719-03-01 to add  UL CA_n5A-n77C and  CA_n66A-n77C for CA_n5-n66-n77

Type: pCR		For: Approval
					38.719-03-01 v0.4.0	  CR-  rev  Cat:  (Rel-19)

					Source: Ericsson, Verizon

Abstract:

TP for 38.719-03-01 to add  UL CA_n5A-n77C and  CA_n66A-n77C for CA_n5-n66-n77

Decision: 		The document was not treated.

R4-2504275	TP for 38.719-03-01 to add  UL CA_n5A-n77C for CA_n5-n48-n77

Type: pCR		For: Approval
					38.719-03-01 v0.4.0	  CR-  rev  Cat:  (Rel-19)

					Source: Ericsson, Verizon

Abstract:

TP for 38.719-03-01 to add  UL CA_n5A-n77C for CA_n5-n48-n77

Decision: 		The document was not treated.

R4-2504276	TP for 38.719-03-01 to add  UL CA_n66A-n77C for CA_n48-n66-n77

Type: pCR		For: Approval
					38.719-03-01 v0.4.0	  CR-  rev  Cat:  (Rel-19)

					Source: Ericsson, Verizon

Abstract:

TP for 38.719-03-01 to add  UL CA_n66A-n77C for CA_n48-n66-n77

Decision: 		The document was not treated.

R4-2504411	Draft CR for TS 38.101-1 to add bandwidth combination set 4 and 5 for inter-band CA combinations (three bands)

Type: draftCR		For: Endorsement
					38.101-1 v19.1.0	  CR-  rev  Cat: B (Rel-19)

					Source: Samsung, Boost Mobile Network

Decision: 		The document was not treated.

R4-2504494	Draft CR on TS 38.101-1 to correct inter-band CA configurations for three bands combinations

Type: draftCR		For: Endorsement
					38.101-1 v19.1.0	  CR-  rev  Cat: F (Rel-19)

					Source: ZTE Corporation, Sanechips

Abstract:

Objective:

NR_CADC_R19_3BDL_xBUL

Decision: 		The document was not treated.

R4-2504495	Draft CR on TS 38.101-3 to introduce new BC for CA_n8--n41-n258

Type: draftCR		For: Endorsement
					38.101-3 v19.1.0	  CR-  rev  Cat: B (Rel-19)

					Source: ZTE Corporation, Sanechips

Abstract:

Objective:

NR_CADC_R19_3BDL_xBUL

Decision: 		The document was not treated.

R4-2504575	draft CR for TS38.101-1 to add new NRCA n1A-n78(2A)-n79A and n28A-n78(2A)-n79A

Type: draftCR		For: Endorsement
					38.101-1 v19.1.0	  CR-  rev  Cat: B (Rel-19)

					Source: NTT DOCOMO, INC.

Decision: 		The document was not treated.

#### 6.3.5	UE RF requirements for NR inter-band CA/DC configurations including inter band CA for y DL with x UL (NR_CADC_R19_yBDL_xBUL)

R4-2503691	Draft CR for TS 38.101-1 to introduce CA_n28A-n40A-n71A-n77A

Type: draftCR		For: Endorsement
					38.101-1 v19.1.0	  CR-  rev  Cat:  (Rel-19)

					Source: Huawei, HiSilicon

Decision: 		The document was not treated.

R4-2504084	Draft CR for TS 38.101-3 to introduce DC band combinations consist of more than three bands

Type: draftCR		For: Endorsement
					38.101-3 v19.1.0	  CR-  rev  Cat: B (Rel-19)

					Source: Huawei, HiSilicon

Decision: 		The document was not treated.

R4-2504422	Draft CR 38.101-1 to add new inter-band combinations of bands n1, n3, n7, n20, and n78

Type: draftCR		For: Endorsement
					38.101-1 v19.1.0	  CR-  rev  Cat: B (Rel-19)

					Source: Nokia, BT plc

Decision: 		The document was not treated.

R4-2504496	Draft CR on TS 38.101-1 to correct inter-band CA configurations for four bands combinations

Type: draftCR		For: Endorsement
					38.101-1 v19.1.0	  CR-  rev  Cat: F (Rel-19)

					Source: ZTE Corporation, Sanechips

Abstract:

Objective:

NR_CADC_R19_yBDL_xBUL

Decision: 		The document was not treated.

#### 6.3.6	UE RF requirements for SUL and CA band combinations with SULs (NR_SUL_combos_R19)

R4-2503156	draftCR to 38.101-1 to include SUL band combinations(NR_CADC_SUL_R19)

Type: draftCR		For: Endorsement
					38.101-1 v19.1.0	  CR-  rev  Cat: B (Rel-19)

					Source: Huawei, Hisilicon

Decision: 		The document was not treated.

R4-2504497	Draft CR on TS 38.101-1 to correct operating bands for SUL band combinations

Type: draftCR		For: Endorsement
					38.101-1 v19.1.0	  CR-  rev  Cat: F (Rel-19)

					Source: ZTE Corporation, Sanechips

Abstract:

Objective:

NR_SUL_combos_R19

Decision: 		The document was not treated.

### 6.4	Rel-19 LTE-Advanced Carrier Aggregation for x bands (1<=x<= 6) DL with y bands (y=1, 2) UL

#### 6.4.1	Rapporteur input (WID/TR/big CR)

R4-2503280	TR 36.719-01-01 LTE-A CA for x-DL y-UL

Type: draft TR		For: Agreement
					36.719-01-01 v0.4.0	  CR-  rev  Cat:  (Rel-19)

					Source: Huawei Technologies France

Decision: 		The document was not treated.

R4-2503281	draftBig CR on Introduction of completed R19 x(1<=x<=6) DL y(y<=2) UL CA band combinations to TS 36.101

Type: draftCR		For: Endorsement
					36.101 v19.1.0	  CR-  rev  Cat: B (Rel-19)

					Source: Huawei Technologies France

Decision: 		The document was not treated.

R4-2503282	draftBig CR on Introduction of completed R19 x(1<=x<=6) DL y(y<=2) UL CA band combinations to TS 36.101

Type: draftCR		For: Endorsement
					36.101 v19.1.0	  CR-  rev  Cat: B (Rel-19)

					Source: Huawei Technologies France

Decision: 		The document was not treated.

R4-2503283	Revised WID Rel-19 LTE-A CA for x(1<=x<=6) DL y(y<=2)

Type: WID revised		For: Endorsement
					Source: Huawei Technologies France

Decision: 		The document was not treated.

#### 6.4.2	UE RF requirements

R4-2503284	draftCR on Introduction of new LTE band combinations LTE_CA_R19_xBDL_yBUL

Type: draftCR		For: Endorsement
					36.101 v19.1.0	  CR-  rev  Cat: B (Rel-19)

					Source: Huawei Technologies France

Decision: 		The document was not treated.

### 6.5	Rel-19 HPUE for NR FR1 TDD/FDD single band

#### 6.5.1	Rapporteur input (WID/TR/big CR)

R4-2504095	TR 38.796 v0.4.0 HPUE_NR_FR1_bands_R19-FDD

Type: draft TR		For: Agreement
					38.796 v0.4.0	  CR-  rev  Cat:  (Rel-19)

					Source: China Unicom

Decision: 		The document was not treated.

#### 6.5.2	HPUE in a single TDD band

#### 6.5.3	HPUE in a single FDD band

R4-2504185	Specification of PC2 and NS values for n5 and n26

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

In this discussion we discuss the requirements for n5 and n26 and consider NS values indicating protection of Public Safety  for n5.

Decision: 		The document was not treated.

R4-2504186	Draft CR to 38.101-1: Introduction of requirements for PC2 operation in band n26

Type: draftCR		For: Endorsement
					38.101-1 v19.1.0	  CR-  rev  Cat: B (Rel-19)

					Source: Ericsson

Abstract:

Draft CR to introduce requirements for PC2 operation in band n26

Decision: 		The document was not treated.

R4-2504555	On HPUE PC2 requirements

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon

Decision: 		The document was not treated.

### 6.6	Rel-19 HPUE in a single LTE band

#### 6.6.1	Rapporteur input (WID/TR/big CR)

#### 6.6.2	UE RF requirements

### 6.7	Rel-19 HPUE for DC combinations of LTE band(s) and NR band(s)

#### 6.7.1	Rapporteur input (WID/TR/big CR)

R4-2504103	TR 37.898 v0.4.0 HPUE_DC_LTE_NR_R19

Type: draft TR		For: Agreement
					37.898 v0.4.0	  CR-  rev  Cat:  (Rel-19)

					Source: China Unicom, Ericsson

Decision: 		The document was not treated.

R4-2504106	Draft Big CR on Rel-19 HPUE for DC combinations of LTE band(s) and NR band(s)

Type: draftCR		For: Endorsement
					38.101-3 v19.1.0	  CR-  rev  Cat: B (Rel-19)

					Source: China Unicom, Ericsson

Decision: 		The document was not treated.

#### 6.7.2	UE RF requirements

R4-2503214	Draft CR for TS38.101-3 addition of n77(3A) to DC_1_n77, DC_3_n77 and DC_8_n77

Type: draftCR		For: Endorsement
					38.101-3 v19.1.0	  CR-  rev  Cat: B (Rel-19)

					Source: SoftBank Corp.

Abstract:

addition of n77(3A) to DC_1_n77, DC_3_n77 and DC_8_n77

Decision: 		The document was not treated.

R4-2503980	Draft CR for 38.101-3 to remove redundant DC with TxD table

Type: draftCR		For: Endorsement
					38.101-3 v19.1.0	  CR-  rev  Cat: B (Rel-19)

					Source: ZTE Corporation, Sanechips, T-Mobile USA, Samsung, Qualcomm

Decision: 		The document was not treated.

### 6.8	Rel-19 HPUE for NR intra-band CA and inter-band CA/DC with/without NR SUL

#### 6.8.1	Rapporteur input (WID/TR/big CR)

R4-2503259	Revised WID for HPUE_NR_CADC_SUL_R19

Type: WID revised		For: Endorsement
					Source: China Telecom, China Unicom

Abstract:

for email approval

Decision: 		The document was not treated.

R4-2503260	DraftBig CR to 38.101-1 new combinations for HPUE_NR_CADC_SUL_R19

Type: draftCR		For: Endorsement
					38.101-1 v19.1.0	  CR-  rev  Cat: B (Rel-19)

					Source: China Telecom, China Unicom

Abstract:

for email approval

Decision: 		The document was not treated.

R4-2503261	TR 38.746 v0.4.0  HPUE_NR_CADC_SUL_R19-TDD

Type: draft TR		For: Agreement
					38.746 v0.4.0	  CR-  rev  Cat:  (Rel-19)

					Source: China Telecom

Abstract:

for email approval

Decision: 		The document was not treated.

R4-2504098	TR 38.750 v0.5.0 HPUE_NR_CADC_SUL_R19-FDD

Type: draft TR		For: Agreement
					38.750 v0.5.0	  CR-  rev  Cat:  (Rel-19)

					Source: China Unicom

Decision: 		The document was not treated.

R4-2504099	draft objective BigCR for HPUE_NR_CADC_SUL_R19

Type: draftCR		For: Endorsement
					38.101-1 v19.1.0	  CR-  rev  Cat: B (Rel-19)

					Source: China Unicom

Decision: 		The document was not treated.

#### 6.8.2	UE RF requirements for intra-band CA

#### 6.8.3	UE RF requirements for inter-band CA/DC with high power on TDD band(s)

R4-2503263	Draft CR for TS38.101-1 Rel-19 for adding new HP-NRCA combinations for for 4 DL

Type: draftCR		For: Endorsement
					38.101-1 v19.1.0	  CR-  rev  Cat:  (Rel-19)

					Source: SoftBank Corp.

Decision: 		The document was not treated.

R4-2503264	Draft CR for TS38.101-1 Rel-19 to add PC2 ULCA_n77(2A) for CA_n41A-n77(3A)

Type: draftCR		For: Endorsement
					38.101-1 v19.1.0	  CR-  rev  Cat:  (Rel-19)

					Source: SoftBank Corp.

Decision: 		The document was not treated.

R4-2504132	Draft CR 38.101-1 to add band combinations of bands n25, n41, n66, n71 and n77 with HPUE on TDD bands

Type: draftCR		For: Endorsement
					38.101-1 v19.1.0	  CR-  rev  Cat: B (Rel-19)

					Source: Nokia, T-Mobile USA

Decision: 		The document was not treated.

R4-2504576	draft CR for TS38.101-1 to add new NRCA with high power UL n78 or n79

Type: draftCR		For: Endorsement
					38.101-1 v19.1.0	  CR-  rev  Cat: B (Rel-19)

					Source: NTT DOCOMO, INC.

Decision: 		The document was not treated.

R4-2504611	TP for TR38.746 for adding HPUE for DL CA_n25-n41-n77 with UL CA_n41-n77

Type: pCR		For: Approval
					38.746 v0.3.0	  CR-  rev  Cat:  (Rel-19)

					Source: T-Mobile USA, Nokia

Decision: 		The document was not treated.

R4-2504612	TP for TR38.746 for adding HPUE for DL CA_n41-n66-n77 with UL CA_n41-n77

Type: pCR		For: Approval
					38.746 v0.3.0	  CR-  rev  Cat:  (Rel-19)

					Source: T-Mobile USA, Nokia

Decision: 		The document was not treated.

R4-2504613	TP for TR38.746 for adding HPUE for DL_CA_n41-n71-n77 with UL CA_n41-n77

Type: pCR		For: Approval
					38.746 v0.3.0	  CR-  rev  Cat:  (Rel-19)

					Source: T-Mobile USA, Nokia

Decision: 		The document was not treated.

#### 6.8.4	UE RF requirements for inter-band CA/DC with high power on FDD band(s)

R4-2503370	TP for TR 38.750 CA_n2-n48 with PC2 FDD single UL

Type: pCR		For: Approval
					38.750 v0.4.0	  CR-  rev  Cat:  (Rel-19)

					Source: Samsung, Verizon

Decision: 		The document was not treated.

R4-2503371	TP for TR 38.750 CA_n48-n66 with PC2 FDD single UL

Type: pCR		For: Approval
					38.750 v0.4.0	  CR-  rev  Cat:  (Rel-19)

					Source: Samsung, Verizon

Decision: 		The document was not treated.

R4-2504131	Draft CR 38.101-1 to add band combinations of bands n25, n41, n66, n71 and n77 with UL PC2 on FDD bands

Type: draftCR		For: Endorsement
					38.101-1 v19.1.0	  CR-  rev  Cat: B (Rel-19)

					Source: Nokia, T-Mobile USA

Decision: 		The document was not treated.

#### 6.8.5	UE RF requirements for inter-band CA/DC with high power on both FDD and TDD bands

R4-2504130	Draft CR 38.101-1 to add 4-band combinations of bands n25, n41, n66, n71 and n77 with UL mixed TDD-FDD PC2

Type: draftCR		For: Endorsement
					38.101-1 v19.1.0	  CR-  rev  Cat: B (Rel-19)

					Source: Nokia, T-Mobile USA

Decision: 		The document was not treated.

### 6.9	Rel-19 Additional NR bands for NR features

#### 6.9.1	Rapporteur input (WID/TR/big CR)

R4-2503983	Revised WID: Additional NR bands for NR features in Rel-19

Type: WID revised		For: Endorsement
					Source: ZTE Corporation, Huawei

Decision: 		The document was not treated.

R4-2503984	TS 38.101-1 draft big CR to include 4Rx and 8Rx

Type: draftCR		For: Endorsement
					38.101-1 v19.1.0	  CR-  rev  Cat: B (Rel-19)

					Source: ZTE Corporation

Decision: 		The document was not treated.

R4-2503985	TS 38.101-1 draft big CR for NR_bands_xFeature_R19-Core

Type: CR		For: Endorsement
					38.101-1 v19.1.0	  CR-2726  rev  Cat: B (Rel-19)

					Source: ZTE Corporation

Decision: 		The document was not treated.

#### 6.9.2	UE RF requirements for UL-MIMO in a single band

#### 6.9.3	UE RF requirements for 4Rx and 8Rx

### 6.10	Rel-19 downlink interruption for NR and EN-DC band combinations at dynamic Tx Switching in Uplink

#### 6.10.1	Rapporteur input (WID/TR/big CR)

R4-2503781	TR 37.887 0.0.5 Rel-19 downlink interruption for NR and EN-DC band combinations at dynamic Tx Switching in Uplink

Type: draft TR		For: Agreement
					37.887 v0.0.5	  CR-  rev  Cat:  (Rel-19)

					Source: China Telecom

Abstract:

For email approval

Decision: 		The document was not treated.

#### 6.10.2	UE RF requirements

R4-2503782	TP to 37.887: DL interruption clarification for CA_n3-n5-n78 for 3 bands dynamic Tx switching

Type: pCR		For: Approval
					37.887 v0.0.4	  CR-  rev  Cat:  (Rel-19)

					Source: China Telecom

Decision: 		The document was not treated.

### 6.11	Simultaneous Rx/Tx band combinations for NR CA/DC, NR SUL and LTE/NR DC in Rel-19

#### 6.11.1	Rapporteur input (WID/TR/big CR)

R4-2503143	TR 38.793 v0.1.0 Rel-19 Requirements for simultaneous RxTx band combinations for NR CADC, NR SUL and LTENR DC

Type: draft TR		For: Agreement
					38.793 v0.1.0	  CR-  rev  Cat:  (Rel-19)

					Source: Huawei, Hisilicon

Decision: 		The document was not treated.

#### 6.11.2	UE RF requirements

R4-2503144	draftCR to 38.101-1 Simultaneous Rx-Tx

Type: draftCR		For: Endorsement
					38.101-1 v19.1.0	  CR-  rev  Cat: B (Rel-19)

					Source: Huawei, Hisilicon

Decision: 		The document was not treated.

R4-2503145	Discussion on simultaneous Rx-Tx

Type: other		For: Approval
					Source: Huawei, Hisilicon

Decision: 		The document was not treated.

R4-2503190	On simultaneous RxTx for CA_n40A-n41A

Type: discussion		For: Discussion
					Source: Apple

Decision: 		The document was not treated.

R4-2504126	On further improvements for simultaneous Rx-Tx notes

Type: discussion		For: Approval
					Source: Nokia

Decision: 		The document was not treated.

### 6.12	Adding channel bandwidth(s) support to existing NR bands and CA/ENDC combinations in REL-19

#### 6.12.1	Rapporteur input (WID/TR/big CR)

#### 6.12.2	UE RF requirements

R4-2503163	TP to TR 37.862: Introduction of 3MHz channel BW for n5 and n12

Type: pCR		For: Approval
					37.862 v0.1.0	  CR-  rev  Cat:  (Rel-19)

					Source: Nokia, AT&T

Abstract:

This contribution provides a text proposal to include 3MHz channel BW for n5 and n12 into TR 37.862.

Decision: 		The document was not treated.

R4-2503193	n48: Adding channels of 50MHz and beyond

Type: discussion		For: Discussion
					Source: Apple

Decision: 		The document was not treated.

R4-2503563	On adding n48 NS_27 A-MPR for CBW 50MHz and higher

Type: other		For: Approval
					Source: Qualcomm Incorporated

Abstract:

Vote for Gene!

Decision: 		The document was not treated.

### 6.13	Introduction of NR-NTN S-band (MSS band 2000-2020 MHz UL and 2180-2200 MHz DL)

#### 6.13.1	System parameters and UE RF requirements

R4-2503176	Further input on UE power back-off while operating in 2000-2020 MHz UL range of the NTN band n252

Type: discussion		For: Discussion
					Source: Apple

Decision: 		The document was not treated.

R4-2503177	Introduction of the NTN S-band to TS 38.101-5

Type: draftCR		For: Endorsement
					38.101-5 v19.0.0	  CR-  rev  Cat: B (Rel-19)

					Source: Apple

Decision: 		The document was not treated.

R4-2503440	Discussion on UE RF requirements for the NTN new S-band

Type: other		For: Approval
					Source: CATT

Decision: 		The document was not treated.

R4-2503441	DraftCR for TS 38.108, Introduction on system parameters for SAN supporting New S band

Type: draftCR		For: Endorsement
					38.108 v19.0.0	  CR-  rev  Cat: B (Rel-19)

					Source: CATT

Decision: 		The document was not treated.

R4-2503565	Views on open issues of n252/B252 UE RF requirements

Type: discussion		For: Decision
					Source: EchoStar, Boost Mobile Network

Decision: 		The document was not treated.

R4-2503566	DraftCR to TR 38.863 to capture agreement for n252 introduction

Type: draftCR		For: Endorsement
					38.863 v18.5.0	  CR-  rev  Cat: B (Rel-19)

					Source: EchoStar, Boost Mobile Network

Decision: 		The document was not treated.

R4-2503697	Discussion on the NTN S band definition

Type: other		For: Approval
					Source: Huawei, HiSilicon

Decision: 		The document was not treated.

R4-2503735	draftCR to TS38.108 Introduction of NR-NTN S-band

Type: draftCR		For: Endorsement
					38.108 v19.0.0	  CR-  rev  Cat: B (Rel-19)

					Source: ZTE Corporation, Sanechips

Decision: 		The document was not treated.

R4-2504387	Discussion on NR-NTN n252 UE RF requirements

Type: discussion		For: Approval
					38.101-5 v	  CR-  rev  Cat:  (Rel-19)

					Source: MediaTek (Hefei) Inc.

Decision: 		The document was not treated.

R4-2504544	A-MPR and OOB blocking for n252

Type: other		For: Approval
					Source: Qualcomm Inc.

Decision: 		The document was not treated.

#### 6.13.2	SAN RF core requirements

R4-2503439	Discussion on SAN RF requirements for the NTN new S-band

Type: other		For: Approval
					Source: CATT

Decision: 		The document was not treated.

R4-2504623	Draft CR to TS 38.108: Clause 5.2 Operating bands

Type: draftCR		For: Endorsement
					38.108 v19.0.0	  CR-  rev  Cat: B (Rel-19)

					Source: THALES

Abstract:

Add n252 band definition as per NR_NTN_Sband WID, RP-241885.

Decision: 		The document was not treated.

#### 6.13.3	RRM core requirements

R4-2503800	Draft CR to TS 38.133 Introduction of a new NTN S-band

Type: draftCR		For: Endorsement
					38.133 v19.0.0	  CR-  rev  Cat: B (Rel-19)

					Source: ZTECorporation,Sanechips

Decision: 		The document was not treated.

### 6.14	Introduction of IoT-NTN S-band (MSS band 2000-2020 MHz UL and 2180-2200 MHz DL)

#### 6.14.1	System parameters and UE RF requirements

R4-2503443	DraftCR for TS 36.108, Introduction on system parameter for SAN supporting New S band

Type: draftCR		For: Endorsement
					36.108 v18.8.0	  CR-  rev  Cat: B (Rel-19)

					Source: CATT

Decision: 		The document was not treated.

R4-2504391	Discussion on IoT-NTN B252 UE RF requirements

Type: discussion		For: Approval
					Source: MediaTek (Hefei) Inc.

Decision: 		The document was not treated.

R4-2504522	DraftCR to TR 36.764 to capture agreement for B252 introduction

Type: draftCR		For: Endorsement
					36.764 v18.3.0	  CR-  rev  Cat: B (Rel-19)

					Source: Boost Mobile Network

Decision: 		The document was not treated.

R4-2504545	UE-to-UE coexistence and OOB blocking for B252

Type: other		For: Approval
					Source: Qualcomm Inc.

Decision: 		The document was not treated.

#### 6.14.2	SAN RF core requirements

R4-2504624	Draft CR to TS 36.108: Clause 5.2 Operating bands

Type: draftCR		For: Endorsement
					36.108 v18.8.0	  CR-  rev  Cat: B (Rel-19)

					Source: THALES

Abstract:

Add 252 band definition as per IoT_NTN_FDD_S_band WID, RP-242062.

Decision: 		The document was not treated.

#### 6.14.3	RRM core requirements

R4-2503802	Draft CR to TS 36.133 Introduction of a new IoT NTN S-band

Type: draftCR		For: Endorsement
					36.133 v19.0.0	  CR-  rev  Cat: B (Rel-19)

					Source: ZTECorporation,Sanechips

Decision: 		The document was not treated.

### 6.15	Introduction of new NR NTN bands to support the Extended L-band (UL 1668-1675MHz, DL 1518-1525MHz) and the combined MSS L-band and Extended L-band ranges

R4-2503548	Open items on NR_IoT_NTN_Bands

Type: discussion		For: Discussion
					Source: Aalyria

Decision: 		The document was withdrawn.

#### 6.15.1	System parameters and UE RF requirements

R4-2503178	Further considerations on the emission requirements for the UE operating in 1626.5-1660.5MHz and 1668-1675MHz UL frequency ranges

Type: discussion		For: Decision
					Source: Apple

Decision: 		The document was not treated.

R4-2503179	Introduction of the NTN L-bands to TS 38.101-5

Type: draftCR		For: Endorsement
					38.101-5 v19.0.0	  CR-  rev  Cat: B (Rel-19)

					Source: Apple

Decision: 		The document was not treated.

R4-2503442	DraftCR for TS 38.108, Introduction on system parameter for SAN supporting combined L band

Type: draftCR		For: Endorsement
					38.108 v19.0.0	  CR-  rev  Cat: B (Rel-19)

					Source: CATT

Decision: 		The document was not treated.

R4-2503538	Possible approach on L-band NS value for RA protection

Type: discussion		For: Discussion
					Source: Aalyria

Decision: 		The document was withdrawn.

R4-2503709	Capturing of L-band Regulatory Requirements

Type: discussion		For: Discussion
					Source: ViaSat Satellite Holdings Ltd, Inmarsat

Decision: 		The document was not treated.

R4-2503736	draftCR to TS38.108 Introduction of NR-NTN combined L-band

Type: draftCR		For: Endorsement
					38.108 v19.0.0	  CR-  rev  Cat: B (Rel-19)

					Source: ZTE Corporation, Sanechips

Decision: 		The document was not treated.

#### 6.15.2	SAN RF core requirements

R4-2504620	Draft CR to TS 38.108: Clause 5.2 Operating bands

Type: draftCR		For: Endorsement
					38.108 v19.0.0	  CR-  rev  Cat: B (Rel-19)

					Source: THALES

Abstract:

Add n253, n251 and n250 band definition as per NR_NTN_combinedLband WID, RP-241856.

Decision: 		The document was not treated.

#### 6.15.3	RRM core requirements

R4-2503801	Draft CR to TS 38.133 Introduction of a new NTN L-band

Type: draftCR		For: Endorsement
					38.133 v19.0.0	  CR-  rev  Cat: B (Rel-19)

					Source: ZTECorporation,Sanechips

Decision: 		The document was not treated.

### 6.16	New bands for LTE based 5G terrestrial broadcast for early deployments

#### 6.16.1	General aspects and work plan

#### 6.16.2	Band and system parameters

R4-2503953	New bands for LTE based 5G broadcast - band definition

Type: other		For: Approval
					Source: Ericsson

Abstract:

This contribution discusses the new bands introduction for LTE based 5G broadcast

Decision: 		The document was not treated.

R4-2504034	Introduction of two new separate bands for LTE based 5G terrestrial broadcast for early deployments

Type: discussion		For: Approval
					Source: EBU

Decision: 		The document was not treated.

#### 6.16.3	UE RF requirements

R4-2504044	CR to TS 36.101: New bands for LTE based 5G terrestrial broadcast for early deployments, Rel-19

Type: draftCR		For: Endorsement
					36.101 v19.1.0	  CR-  rev  Cat: B (Rel-19)

					Source: EBU, BNE

Decision: 		The document was not treated.

#### 6.16.4	BS RF core requirements

R4-2504045	CR to TS 36.104 : New bands for LTE based 5G terrestrial broadcast for early deployments, Rel-19

Type: draftCR		For: Endorsement
					36.104 v19.0.0	  CR-  rev  Cat:  (Rel-19)

					Source: EBU, BNE

Decision: 		The document was not treated.

### 6.17	mmWave in NR: UE spurious emissions and EESS (Earth Exploration Satellite Service) protection

#### 6.17.1	General aspects

R4-2503146	TR 38.749 v0.2.0 Protection of Earth Exploration Satellite Service (EESS)

Type: draft TR		For: Agreement
					38.749 v0.2.0	  CR-  rev  Cat:  (Rel-19)

					Source: Huawei, Hisilicon

Decision: 		The document was not treated.

#### 6.17.2	UE RF requirements

R4-2503147	TP to 38.749: UE requirements for Rel-19 EESS

Type: pCR		For: Approval
					38.749 v0.1.0	  CR-  rev  Cat:  (Rel-19)

					Source: Huawei, Hisilicon, China Southern Power Grid

Decision: 		The document was not treated.

R4-2503148	draft CR to 38.101-2 EESS (Rel-15)

Type: draftCR		For: Endorsement
					38.101-2 v15.26.0	  CR-  rev  Cat: C (Rel-15)

					Source: Huawei, Hisilicon, China Unicom, China Southern Power Grid

Decision: 		The document was not treated.

R4-2503149	draft CR to 38.101-2 EESS (Rel-16)

Type: draftCR		For: Endorsement
					38.101-2 v16.21.0	  CR-  rev  Cat: A (Rel-16)

					Source: Huawei, Hisilicon, China Unicom, China Southern Power Grid

Decision: 		The document was not treated.

R4-2503150	draft CR to 38.101-2 EESS (Rel-17)

Type: draftCR		For: Endorsement
					38.101-2 v17.17.0	  CR-  rev  Cat: A (Rel-17)

					Source: Huawei, Hisilicon, China Unicom, China Southern Power Grid

Decision: 		The document was not treated.

R4-2503151	draft CR to 38.101-2 EESS (Rel-18)

Type: draftCR		For: Endorsement
					38.101-2 v18.9.0	  CR-  rev  Cat: A (Rel-18)

					Source: Huawei, Hisilicon, China Unicom, China Southern Power Grid

Decision: 		The document was not treated.

R4-2503946	EESS protection - Remaining UE RF issues

Type: other		For: Approval
					Source: Ericsson

Abstract:

This contribution discusses the remaining open issues related to EESS protection WI

Decision: 		The document was not treated.

R4-2503947	Draft CR to TS 38101-2 - Rel-15 EESS protection - UE RF impacts

Type: draftCR		For: Endorsement
					38.101-2 v15.26.0	  CR-  rev  Cat: B (Rel-15)

					Source: Ericsson

Abstract:

This is draft CR to TS 38.101-2 Rel-15 porposing how to implement EESS protection in Release earlier to Rel-19

Decision: 		The document was not treated.

R4-2503949	EESS protection - TP to TR 38749 - UE aspects

Type: pCR		For: Approval
					38.749 v0.1.0	  CR-  rev  Cat:  (Rel-19)

					Source: Ericsson

Abstract:

This TP to TR 38.749 proposes text to fix some mistakes and better clarify text related to the UE clause

Decision: 		The document was not treated.

R4-2503974	TP for TR38.749 UE RF requirements (Clause 8)

Type: pCR		For: Approval
					38.749 v0.1.0	  CR-  rev  Cat:  (Rel-19)

					Source: ZTE Corporation, Sanechips

Decision: 		The document was not treated.

R4-2504159	draft CR to add new requirements for EESS protection from Rel-15

Type: draftCR		For: Endorsement
					38.101-2 v15.26.0	  CR-  rev  Cat: C (Rel-15)

					Source: NTT DOCOMO, INC.

Abstract:

This draft CR is to introduce new emission requirements for EESS protection from n257 and n258 into the Rel-15 spec.

Decision: 		The document was not treated.

R4-2504160	draft CR to add new requirements for EESS protection from Rel-15

Type: draftCR		For: Endorsement
					38.101-2 v16.21.0	  CR-  rev  Cat: A (Rel-16)

					Source: NTT DOCOMO, INC.

Abstract:

This draft CR is to introduce new emission requirements for EESS protection from n257 and n258 into the Rel-16 spec.

Decision: 		The document was not treated.

R4-2504161	draft CR to add new requirements for EESS protection from Rel-15

Type: draftCR		For: Endorsement
					38.101-2 v17.17.0	  CR-  rev  Cat: A (Rel-17)

					Source: NTT DOCOMO, INC.

Abstract:

This draft CR is to introduce new emission requirements for EESS protection from n257 and n258 into the Rel-17 spec.

Decision: 		The document was not treated.

R4-2504162	draft CR to add new requirements for EESS protection from Rel-15

Type: draftCR		For: Endorsement
					38.101-2 v18.9.0	  CR-  rev  Cat: A (Rel-18)

					Source: NTT DOCOMO, INC.

Abstract:

This draft CR is to introduce new emission requirements for EESS protection from n257 and n258 into the Rel-18 spec.

Decision: 		The document was not treated.

R4-2504482	24 GHz EESS Band Phase 2 Compliance

Type: draftCR		For: Endorsement
					38.101-2 v15.26.0	  CR-  rev  Cat:  (Rel-15)

					Source: Qualcomm Incorporated

Abstract:

Proposal for EESS band protection with new emission requirements

Decision: 		The document was not treated.

#### 6.17.3	BS RF requirements and conformance testing

R4-2503164	TP to TR 38.749: BS RF requirements for Protection of EESS

Type: pCR		For: Approval
					38.749 v0.1.0	  CR-  rev  Cat:  (Rel-19)

					Source: Nokia

Abstract:

This contribution provides a text proposal to update the BS RF requirements clause in TR 38.749 with the RAN4 agreements in previous meetings.

Decision: 		The document was not treated.

R4-2503948	EESS protection - TP to TR 38749 - BS aspects

Type: pCR		For: Approval
					38.749 v0.1.0	  CR-  rev  Cat:  (Rel-19)

					Source: Ericsson

Abstract:

This TP to TR 38.749 proposes text to fix some mistakes and better clarify text related to the BS clause

Decision: 		The document was not treated.

## 7	Rel-19 on-going non-spectrum related work/study items

### 7.1	UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4

#### 7.1.1	UE RF requirements

##### 7.1.1.1	High power UE (HPUE) for CA in terrestrial network (TN)

###### 7.1.1.1.1	General aspects (incl. duty cycle solutions, MSD rules)

R4-2503153	Discussion on HPUE MSD

Type: other		For: Approval
					Source: Huawei, Hisilicon

Decision: 		The document was not treated.

R4-2503169	Simplification of HPUE 2UL DeltaMSD

Type: other		For: Approval
					Source: Skyworks Solutions Inc.

Abstract:

In this contribution, we provide our analysis for all 2UL HPUE cases (and sub-cases) including those with NRU in at least one band. Then further simplification compared to [1] is proposed for DeltaMSD for two-band and three-band DL combinations but exclud

Decision: 		The document was not treated.

R4-2503196	On Rel-19 HPUE CA DC MSD consideration

Type: other		For: Approval
					Source: Apple

Decision: 		The document was not treated.

R4-2503215	Further discussion on HPUE MSD rules

Type: discussion		For: Discussion
					Source: Murata Manufacturing Co Ltd.

Decision: 		The document was not treated.

R4-2503225	On 1UL/1CC HPUE MSD rules

Type: other		For: Approval
					Source: Nokia, Samsung

Decision: 		The document was not treated.

R4-2503238	On 1UL HPUE MSD simplifications

Type: other		For: Discussion
					38.101-1 v	  CR-  rev  Cat:  (Rel-19)

					Source: Skyworks Solutions Inc.

Abstract:

This document captures our views/observations on simplifying the 1UL HPUE MSD requirements for Rel-19.

Decision: 		The document was not treated.

R4-2503239	Unified HPUE 1UL2UL MSD simplification proposals for Rel-19

Type: other		For: Approval
					38.101-1 v	  CR-  rev  Cat:  (Rel-19)

					Source: Skyworks Solutions Inc., Samsung

Abstract:

This document proposes a single LUT set to simplify both 1UL and 2UL MSD requirements. Other remaining aspects are proposed for consideration into the core requirements of TS 38.101-1.

Decision: 		The document was not treated.

R4-2503361	Discussion on HPUE intra-band/inter-band CA/DC with 2Tx/3Tx

Type: other		For: Approval
					Source: MediaTek Korea Inc.

Abstract:

In this paper, we we share our view on the HP CA/DC UE related Configured Tx power requirements and SAR compliance solution in Rel-19.

Decision: 		The document was not treated.

R4-2503362	Discussion on MSD simplification of HPUE CA/DC UE

Type: other		For: Approval
					Source: MediaTek Korea Inc.

Abstract:

In this paper, we we share our view on the MSD simplification methodology according to 1UL and 2UL interference sources in Rel-19.

Decision: 		The document was not treated.

R4-2503365	Consideration on Rel-19 and Rel-20 HPUE band combination work

Type: discussion		For: Discussion
					Source: Samsung

Decision: 		The document was not treated.

R4-2503590	draft CR on duty cycle SAR solution for inter-band UL CA with 3Tx

Type: draftCR		For: Endorsement
					38.101-1 v19.1.0	  CR-  rev  Cat: B (Rel-19)

					Source: LG Electronics

Abstract:

It is a draft CR of duty cycle solution for inter-band UL CA with 3Tx.

Decision: 		The document was not treated.

R4-2503592	draft CR on duty cycle SAR solution for inter-band EN-DC with 3Tx

Type: draftCR		For: Endorsement
					38.101-1 v19.1.0	  CR-  rev  Cat: B (Rel-19)

					Source: LG Electronics

Abstract:

It is a draft CR of duty cycle solution for inter-band EN-DC with 3Tx

Decision: 		The document was not treated.

R4-2503839	Discussion on HPUE MSD simplification

Type: other		For: Approval
					Source: vivo

Decision: 		The document was not treated.

R4-2503969	Consideration on R19 HPUE MSD rule

Type: other		For: Approval
					Source: ZTE Corporation, Sanechips

Decision: 		The document was not treated.

R4-2504118	MSD beyond PC3

Type: other		For: Approval
					Source: Qualcomm France

Abstract:

Considerations and proposals for HPUE MSD are provided in this contribution.

Decision: 		The document was not treated.

R4-2504187	On the duty-cycle solutions for SAR compliance

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

In this contibution we discuss alternative solutions to duty-cycle repoting

Decision: 		The document was not treated.

R4-2504614	MSD for 3Tx UL CA combinations with 2Tx FDD + 1Tx TDD

Type: discussion		For: Approval
					Source: T-Mobile USA

Abstract:

MCC: R4-2504614 was moved from AI 6.8.5 to AI 7.1.1.1.1 and treat it in [113].

Decision: 		The document was not treated.

###### 7.1.1.1.2	Intra-band contiguous and non-contiguous UL CA with PC1.5

R4-2503197	On PC1.5 intra-band UL CA MPR

Type: other		For: Approval
					Source: Apple

Decision: 		The document was not treated.

R4-2503279	MPR evaluations for contiguous intra-band CA with NC allocation for PC1.5 TxDiv

Type: discussion		For: (not specified)
					Source: Huawei Technologies France

Decision: 		The document was not treated.

R4-2503511	MPR and A-MPR for intra-band contiguous and non-contiguous UL CA for PC1.5

Type: other		For: Approval
					38.101-1 v	  CR-  rev  Cat:  (Rel-19)

					Source: Qualcomm Technologies

Abstract:

In this paper we present MPR and A-MPR values for contiguous and non-contiguous CA

Decision: 		The document was not treated.

R4-2504170	R19 MPR for PC1.5 contig CA

Type: other		For: Approval
					Source: OPPO

Decision: 		The document was not treated.

R4-2504171	R19 MPR for PC1.5 NCCA

Type: other		For: Approval
					Source: OPPO

Decision: 		The document was not treated.

R4-2504188	Applicability of MPR for contiguous and non-contiguous intra-band UL CA with PC1.5

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

In this contribution we discuss the MPR applicability for uequal PSD and for use in the field

Decision: 		The document was not treated.

R4-2504241	HPUE MPR for intra-band UL CA

Type: discussion		For: Approval
					Source: LG Electronics Finland

Abstract:

On requirements for PC1.5 intra-band contiguous and non-contiguous UL CA

Decision: 		The document was not treated.

R4-2504258	Draft CR on -30dBm and -13dBm MPR for PC1.5 NCCA

Type: draftCR		For: Endorsement
					38.101-1 v19.1.0	  CR-  rev  Cat: B (Rel-19)

					Source: LG Electronics Finland

Abstract:

Add MPR requirements to meet -30dBm and -13dBm SEM for PC1.5 NCCA

Decision: 		The document was not treated.

R4-2504282	draft CR on MPR for PC1.5 intra-band contiguous CA

Type: draftCR		For: Endorsement
					38.101-1 v19.1.0	  CR-  rev  Cat: B (Rel-19)

					Source: LG Electronics Finland

Abstract:

Add PC1.5 intra-band contiguous CA, MPR requirements for handheld and FWA

Decision: 		The document was not treated.

R4-2504294	draft CR on Pcmax,c for PC1.5 intra-band NC CA

Type: draftCR		For: Endorsement
					38.101-1 v19.1.0	  CR-  rev  Cat: B (Rel-19)

					Source: LG Electronics Finland

Abstract:

For intra-band UL NC CA with PC1.5 dualPA-architecture the upper bound of Pcmax,c is 26dBm per CC and the upper bound of Pcmax is 29dBm.

Decision: 		The document was not treated.

R4-2504386	Results for PC1.5 contiguous ULCA with comparison to PC2 and 6dB PSD imbalance

Type: other		For: Approval
					Source: Skyworks Solutions Inc.

Abstract:

At the time of R4#114 we did not have measurements for PC2 2Tx nor PC1.5 with 6dB PSD differences, so we were not able to comment on the proposed values in the way forward as we usually rely on relative back-off values with direct comparisons between PC1.

Decision: 		The document was not treated.

###### 7.1.1.1.3	Inter-band UL NR-CA/EN-DC with 2 bands and 2Tx and/or 3Tx

R4-2503174	dCR to 38.101-1, to remove redundancies in 2Tx/3Tx notes

Type: draftCR		For: Endorsement
					38.101-1 v19.1.0	  CR-  rev  Cat:  (Rel-19)

					Source: Qualcomm, Nokia, Verizon, T-Mobile

Abstract:

Proposal for streamlining of  notes sprinkled across sections 5 and 6 of 38.101-1 pertaining to the Tx chain makeup of the UL band(s).

Decision: 		The document was not treated.

R4-2503249	Discussion on HPUE inter-band UL CA and EN-DC

Type: discussion		For: Discussion
					Source: SK Telecom

Decision: 		The document was not treated.

R4-2503970	Draft CR to TS 38.307 Release independence for R19 3Tx band combination

Type: draftCR		For: Endorsement
					38.307 v19.0.0	  CR-  rev  Cat: B (Rel-19)

					Source: ZTE Corporation, Sanechips

Decision: 		The document was not treated.

R4-2504127	On future HPUE Basket Arrangement

Type: discussion		For: Approval
					Source: Nokia

Decision: 		The document was not treated.

###### 7.1.1.1.4	Increasing UE transmission high power limit

R4-2503968	Draft Rel-19 CR for 38.101-3 on increasing higher power limit for EN-DC with 2Tx/3Tx

Type: draftCR		For: Endorsement
					38.101-3 v19.1.0	  CR-  rev  Cat: B (Rel-19)

					Source: ZTE Corporation, Samsung, CHTTL

Decision: 		The document was not treated.

##### 7.1.1.2	Power domain enhancement for NR single carrier and NR intra-band UL CA for PC2 and PC3

###### 7.1.1.2.1	Power domain enhancements for single carrier

R4-2503189	On Rel-19 power domain enhancement for FR1

Type: discussion		For: Discussion
					Source: Apple

Decision: 		The document was not treated.

R4-2503224	On single carrier MPR Enhancement

Type: other		For: Approval
					Source: Nokia

Decision: 		The document was not treated.

R4-2503237	Flexible RB extension for improved MPR when NW BW is larger than UE CBW

Type: other		For: Approval
					Source: Skyworks Solutions Inc.

Abstract:

In this contribution, we further explain the concept of a flexible extended bandwidth based on RB extension in fractions of the UE NRB, and define the associated guard bands and extended bandwidth in relation to the permitted emission.

Decision: 		The document was not treated.

R4-2503492	Further discussion on power domain enhancements for single carrier

Type: other		For: Discussion
					Source: CATT

Decision: 		The document was not treated.

R4-2503512	Power boosting and MPR reduction

Type: other		For: Approval
					38.101-1 v	  CR-  rev  Cat:  (Rel-19)

					Source: Qualcomm Technologies

Abstract:

We present further views on the open issues in this topic

Decision: 		The document was not treated.

R4-2503737	Discussion on power domain enhancements for single carrier

Type: other		For: Approval
					Source: ZTE Corporation

Decision: 		The document was not treated.

R4-2503837	Discussion on UE RF power enhancement for NR single carrier

Type: other		For: Approval
					Source: vivo

Decision: 		The document was not treated.

R4-2503838	draftCR to TS 38.101-1 on  MPR reduction for single carrier

Type: draftCR		For: Endorsement
					38.101-1 v19.1.0	  CR-  rev  Cat: B (Rel-19)

					Source: vivo

Decision: 		The document was not treated.

R4-2503938	Views on Power domain enhancements for single carrier

Type: discussion		For: Discussion
					Source: China Telecom

Decision: 		The document was not treated.

R4-2504062	Further Views on MPR Reduction

Type: discussion		For: Approval
					Source: Sony

Decision: 		The document was not treated.

R4-2504085	On power domain enhancements for single carrier

Type: other		For: Approval
					Source: Huawei, HiSilicon

Decision: 		The document was not treated.

R4-2504086	Draft CR for TS 38.101-1 to introduce MPR reduction for single carrier

Type: draftCR		For: Endorsement
					38.101-1 v19.1.0	  CR-  rev  Cat: B (Rel-19)

					Source: Huawei, HiSilicon

Decision: 		The document was not treated.

R4-2504152	Discussion on MPR reduction for single carrier

Type: discussion		For: Discussion
					Source: OPPO

Decision: 		The document was not treated.

R4-2504189	Power domain enhancement for UE-specific CHBWs within a single carrier

Type: other		For: Approval
					Source: Ericsson

Abstract:

In this contribution we propose power enhancement for a narrower UE CHBW configured within a wider carrier

Decision: 		The document was not treated.

R4-2504240	Discussion on power domain enhancement for NR single carrier

Type: discussion		For: Approval
					Source: MediaTek (Wuhan) Inc.

Decision: 		The document was not treated.

R4-2504262	Power domain enhancements for single carrier

Type: discussion		For: Discussion
					Source: Beijing Xiaomi Mobile Software

Decision: 		The document was not treated.

R4-2504373	Discussion on Power domain enhancements for single carrier

Type: discussion		For: Discussion
					Source: LG Electronics UK

Decision: 		The document was withdrawn.

R4-2504512	Discussion on Power domain enhancements for single carrier

Type: discussion		For: Discussion
					Source: LG Electronics UK

Decision: 		The document was not treated.

R4-2504543	Handling A-MPR with Bandwidth Extension

Type: discussion		For: Approval
					Source: Lenovo

Decision: 		The document was not treated.

###### 7.1.1.2.2	MPR applicability for FR1 intra-band UL CA

R4-2503493	Further discussion on MPR applicability for FR1 intra-band UL CA

Type: other		For: Discussion
					Source: CATT

Decision: 		The document was not treated.

R4-2503738	DraftCR to 38.101-1 MPR applicability for FR1 intra-band contiguous UL CA

Type: draftCR		For: Endorsement
					38.101-1 v19.1.0	  CR-  rev  Cat: B (Rel-19)

					Source: ZTE Corporation, Sanechips

Decision: 		The document was not treated.

R4-2504560	Bandwidth Extension for Intra-Band UL CA with a Single CC

Type: discussion		For: Approval
					Source: Lenvovo

Decision: 		The document was withdrawn.

R4-2504564	Bandwidth Extension for Intra-Band UL CA with a Single CC

Type: discussion		For: Approval
					Source: Lenovo

Decision: 		The document was not treated.

###### 7.1.1.2.3	MPR applicability for FR2

R4-2503647	LS on introduction of new UE capabilities for FR2 MPR improvement

Type: LS out		For: Approval
					to RAN2
					Source: Samsung

Decision: 		The document was not treated.

R4-2503992	draft CR to TS 38.101-2 for FR2 MPR improvements

Type: draftCR		For: Endorsement
					38.101-2 v19.0.0	  CR-  rev  Cat: B (Rel-19)

					Source: NTT DOCOMO, INC.

Decision: 		The document was not treated.

##### 7.1.1.3	6Rx UE

###### 7.1.1.3.1	Reference sensitivity requirements

R4-2503965	On release independent for 6Rx RF requirement

Type: other		For: Approval
					Source: ZTE Corporation, Sanechips

Decision: 		The document was not treated.

R4-2503966	Draft CR to TS38.101-1: Introduction of ?TRxSRS requirements for 6Rx

Type: draftCR		For: Endorsement
					38.101-1 v19.1.0	  CR-  rev  Cat: B (Rel-19)

					Source: ZTE Corporation, Sanechips

Decision: 		The document was not treated.

R4-2504087	Draft CR for TS 38.101-1 to introduce 6Rx RF requirements

Type: draftCR		For: Endorsement
					38.101-1 v19.1.0	  CR-  rev  Cat: B (Rel-19)

					Source: Huawei, HiSilicon

Decision: 		The document was not treated.

R4-2504372	Discussion on the usage of the terminology “FWA form factor” and its application for 6Rx, 4Rx and 8Rx requirements

Type: discussion		For: Discussion
					Source: Anritsu Limited

Decision: 		The document was not treated.

###### 7.1.1.3.2	MIMO layer evaluation for 6Rx UE

R4-2503158	Discussion on 6 Layer MIMO Feasibility

Type: discussion		For: Discussion
					Source: Nokia

Decision: 		The document was not treated.

R4-2503207	Discussion on remaining issues on 6 MIMO layer evaluation for 6Rx UE

Type: discussion		For: Discussion
					Source: China Telecom

Decision: 		The document was not treated.

R4-2503524	On Core Requirements for MIMO Layer Support for 6Rx UE

Type: discussion		For: Discussion
					Source: Apple

Decision: 		The document was not treated.

R4-2503655	Views on MIMO layer evaluation for 6Rx UE

Type: discussion		For: Discussion
					Source: Samsung

Decision: 		The document was not treated.

R4-2504035	Discussion on the support of MIMO layers for 6Rx UE

Type: other		For: Approval
					Source: Ericsson

Abstract:

This paper will discuss the objectives specified in the WI with a focus on the 6Rx MIMO layers.

Decision: 		The document was not treated.

R4-2504088	Remaining issues for MIMO layers for 6Rx

Type: other		For: Approval
					Source: Huawei, HiSilicon

Decision: 		The document was not treated.

R4-2504178	draft LS on iIntroduction of supporting 6 layers capability for FWA device

Type: LS out		For: Approval
					to RAN2
					Source: Huawei, HiSilicon

Decision: 		The document was not treated.

R4-2504348	Discussion on UE capability for 6 MIMO layer support

Type: discussion		For: Discussion
					Source: MediaTek inc.

Decision: 		The document was not treated.

R4-2504561	Discussions on signaling requirements for 6Rx devices

Type: discussion		For: Discussion
					Source: QUALCOMM Europe Inc. - Spain

Decision: 		The document was not treated.

R4-2504563	[Draft] LS on introducing capability signaling for 6Rx devices

Type: LS out		For: Approval
					to RAN2, cc RAN1, RAN5
					Source: QUALCOMM Europe Inc. - Spain

Decision: 		The document was not treated.

###### 7.1.1.3.3	SRS antenna switching requirements

R4-2503820	Further discussion on SRS antenna switching requirements for 6Rx

Type: other		For: Approval
					Source: vivo

Decision: 		The document was not treated.

R4-2503821	Draft CR to 38.101-1 on SRS antenna switching requirements for 6Rx

Type: draftCR		For: Endorsement
					38.101-1 v19.1.0	  CR-  rev  Cat:  (Rel-19)

					Source: vivo

Decision: 		The document was not treated.

R4-2503822	LS on indication of support t3r6 SRS antenna switching capability

Type: LS out		For: Approval
					to RAN2, cc RAN1
					Source: vivo

Decision: 		The document was not treated.

###### 7.1.1.3.4	SRS IL imbalance

R4-2503195	On SRS Antenna Switching IL Imbalance Solution for 6Rx

Type: discussion		For: Discussion
					Source: Apple

Decision: 		The document was not treated.

R4-2503333	Discussion on SRS IL imbalance for 6Rx UE

Type: discussion		For: Discussion
					Source: Xiaomi

Decision: 		The document was not treated.

R4-2503846	Discussion on SRS IL imbalance reporting

Type: other		For: Approval
					Source: vivo

Decision: 		The document was not treated.

R4-2503939	Further study on SRS IL imbalance

Type: discussion		For: Discussion
					Source: China Telecom

Decision: 		The document was not treated.

R4-2503967	Views on 6Rx SRS antenna IL

Type: other		For: Approval
					Source: ZTE Corporation, Sanechips

Decision: 		The document was not treated.

R4-2503986	Disscussion on SRS IL imbalance for 6RX UE

Type: discussion		For: Discussion
					Source: Spreadtrum,UNISOC

Decision: 		The document was not treated.

R4-2503993	SRS imbalance solution considerations

Type: other		For: Approval
					Source: Nokia

Decision: 		The document was not treated.

R4-2504089	On SRS IL imbalance issue

Type: other		For: Approval
					Source: Huawei, HiSilicon

Decision: 		The document was not treated.

R4-2504100	Discussion on SRS IL imbalance for 6Rx UE

Type: discussion		For: Approval
					Source: Google

Decision: 		The document was not treated.

R4-2504121	SRS IL imbalance indication

Type: other		For: Approval
					Source: Qualcomm France

Abstract:

SRS IL imbalance indication considered in this contribution.

Decision: 		The document was not treated.

R4-2504163	Views on SRS insertion loss compensation and reporting enhancements

Type: discussion		For: Discussion
					Source: Intel Corporation

Decision: 		The document was not treated.

R4-2504389	SRS IL imbalance issue

Type: discussion		For: Discussion
					Source: Samsung

Decision: 		The document was not treated.

R4-2504562	SRS Power Imbalance Handling

Type: discussion		For: Approval
					Source: Lenovo

Decision: 		The document was not treated.

R4-2504580	On the SRS IL imbalance reporting for the SRS-AS

Type: other		For: Approval
					Source: Ericsson

Abstract:

Proposals on the details of the SRS IL imbalance reporting solution, including an analysis on the impact on specifications and the triggering conditions.

Decision: 		The document was not treated.

#### 7.1.2	RRM core and performance requirements for 6Rx

R4-2503891	Discussion on RLM/BFD test cases for 6Rx UE

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon

Decision: 		The document was not treated.

R4-2504374	Discussions on RRM performance requirements of Rel-19 WI on UE RF enhancements

Type: other		For: Discussion
					Source: Ericsson

Abstract:

In this paper we discuss the RRM performance impact of 6Rx UE.

Decision: 		The document was not treated.

#### 7.1.3	UE demodulation performance requirements for 6Rx

R4-2503142	Discussion on demodulation and CSI requirements for 6Rx UE

Type: discussion		For: Discussion
					Source: MediaTek inc.

Decision: 		The document was not treated.

R4-2503157	Discussion on 6 Layer MIMO performance requirements

Type: discussion		For: Discussion
					Source: Nokia

Decision: 		The document was not treated.

R4-2503208	Discussion on UE demodulation performance requirements for 6Rx

Type: discussion		For: Discussion
					Source: China Telecom

Decision: 		The document was not treated.

R4-2503248	Discussions on demodulation performance requirements for 6Rx devices

Type: discussion		For: Discussion
					Source: QUALCOMM Europe Inc. - Spain

Decision: 		The document was not treated.

R4-2503525	On UE Demodulation Performance Requirements for 6Rx

Type: discussion		For: Discussion
					Source: Apple

Decision: 		The document was not treated.

R4-2503656	Discussion on UE demodulation performance requirements for 6Rx

Type: discussion		For: Discussion
					Source: Samsung

Decision: 		The document was not treated.

R4-2503999	Discussion on UE demodulation performance requirements for 6Rx

Type: other		For: Approval
					Source: ZTE Corporation, Sanechips

Decision: 		The document was not treated.

R4-2504311	Overview on 6Rx performance requirements

Type: discussion		For: Discussion
					Source: Huawei,HiSilicon

Decision: 		The document was not treated.

R4-2504312	Work plan for 6Rx performance requirements

Type: Work Plan		For: Approval
					Source: Huawei,HiSilicon

Decision: 		The document was not treated.

R4-2504329	On UE demodulation and CSI reporting requirements for 6Rx

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

This contribution provided our views on how to define  demodulation, SDR, CSI requirements for 6Rx

Decision: 		The document was not treated.

#### 7.1.4	Moderator summary and conclusions

R4-2503611	Topic summary for [114bis][201] NR_ENDC_RF_Ph4

Type: other		For: Information
					Source: Moderator (Huawei)

Abstract:

Topic summary in RRM session

Decision: 		The document was not treated.

R4-2504650	Topic summary for [114bis][325] NR_ENDC_RF_Ph4_Demod_6Rx

Type: other		For: Information
					Source: Moderator (Huawei)

Abstract:

[114bis] BDaT Session AI 7.1.3

Decision: 		The document was not treated.

R4-2504670	Topic summary for [114bis][112] NR_ENDC_RF_Ph4_part1

Type: other		For: Information
					Source: Moderator(Huawei)

Abstract:

Summary for AI 7.1.1.2

Decision: 		The document was not treated.

R4-2504671	Topic summary for [114bis][113] NR_ENDC_RF_Ph4_part2

Type: other		For: Information
					Source: Moderator(Samsung)

Abstract:

Summary for AI 7.1.1.1

Decision: 		The document was not treated.

R4-2504672	Topic summary for [114bis][114] NR_ENDC_RF_Ph4_part3

Type: other		For: Information
					Source: Moderator(Intel)

Abstract:

Summary for AI 7.1.1.3

Decision: 		The document was not treated.

### 7.2	NR sidelink Intra-band Carrier Aggregation in ITS band

#### 7.2.1	UE RF requirements for intra-band non-contiguous CA

R4-2503595	Tx requirements for SL intra-band non-contiguous CA

Type: discussion		For: Discussion
					Source: LG Electronics

Abstract:

It is for discussion of A-MPR for SL intra-band NC CA.

Decision: 		The document was not treated.

R4-2503597	TP to TR 38.787 on SL intra-band NC CA A-MPR

Type: pCR		For: Approval
					38.787 v0.1.0	  CR-  rev  Cat:  (Rel-19)

					Source: LG Electronics

Abstract:

It is TP of A-MPR of PSFCH and S-SSB for SL intra-band NC CA.

Decision: 		The document was not treated.

R4-2504210	UE RF requirments for intra-band non-contiguous CA

Type: other		For: Approval
					Source: OPPO

Decision: 		The document was not treated.

#### 7.2.2	UE RF requirements for intra-band contiguous CA

#### 7.2.3	Moderator summary and conclusions

R4-2504673	Topic summary for [114bis][115] NR_SL_intraB_CA_ITS_part1

Type: other		For: Information
					Source: Moderator(OPPO)

Abstract:

Summary for AI 7.2, 7.2.1

Decision: 		The document was not treated.

R4-2504674	Topic summary for [114bis][116] NR_SL_intraB_CA_ITS_part2

Type: other		For: Information
					Source: Moderator(LGE)

Abstract:

Summary for AI 7.2.2. MCC: This is shown as withdrawn by Chair.

Decision: 		The document was withdrawn.

### 7.3	NR channel BW less than 5MHz for FR1 Phase 2

#### 7.3.1	UE RF requirements maintenance for inter-band NR CA/DC with 3MHz CBW

R4-2503165	Draft CR to TS 38.101-1 on addition of 3 MHz channel bandwidth into Annex D.1

Type: draftCR		For: Endorsement
					38.101-1 v19.1.0	  CR-  rev  Cat: F (Rel-19)

					Source: Nokia

Abstract:

Add 3 MHz channel bandwidth into Annex D.1.

Decision: 		The document was not treated.

R4-2503481	CR for less than 5MHz UE RF requirements in Rel-19

Type: draftCR		For: Endorsement
					38.101-1 v19.1.0	  CR-  rev  Cat: F (Rel-19)

					Source: CATT

Decision: 		The document was not treated.

#### 7.3.2	RRM core requirements maintenance

R4-2503857	draft CR to TS 38.133: NR channel BW less than 5MHz for FR1

Type: draftCR		For: Endorsement
					38.133 v19.0.0	  CR-  rev  Cat: B (Rel-19)

					Source: ZTE Corporation, Sanechips

Decision: 		The document was not treated.

R4-2503893	draftCR on RRM requirements for less than 5MHz Ph2

Type: draftCR		For: Endorsement
					38.133 v19.0.0	  CR-  rev  Cat: F (Rel-19)

					Source: Huawei, HiSilicon

Decision: 		The document was not treated.

R4-2503894	Discussion on RRM requirements for less than 5MHz Ph2

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon

Decision: 		The document was not treated.

R4-2504017	Core maintenance draftCR on less than 5MHz RRM requirements

Type: draftCR		For: Endorsement
					38.133 v19.0.0	  CR-  rev  Cat: F (Rel-19)

					Source: Intel Corporation

Decision: 		The document was not treated.

#### 7.3.3	RRM performance requirements

R4-2503858	Discussion on performance for less than 5MHz BW

Type: other		For: Approval
					Source: ZTE Corporation, Sanechips

Decision: 		The document was not treated.

R4-2503859	draft CR to TS 38.133: test cases for NR channel BW less than 5MHz for FR1

Type: draftCR		For: Endorsement
					38.133 v19.0.0	  CR-  rev  Cat: B (Rel-19)

					Source: ZTE Corporation, Sanechips

Decision: 		The document was not treated.

R4-2503895	draftCR on TC 3-2 and TC 4-1 for less than 5MHz PH2

Type: draftCR		For: Endorsement
					38.133 v19.0.0	  CR-  rev  Cat: B (Rel-19)

					Source: Huawei, HiSilicon

Decision: 		The document was not treated.

R4-2504015	Discussion on Performance part of less than 5MHz RRM requirements in Rel-19

Type: discussion		For: Discussion
					Source: Intel Corporation

Decision: 		The document was not treated.

R4-2504016	Test cases requirements for handover with PSCell delay and deactivated PSCell measurements

Type: draftCR		For: Endorsement
					38.133 v19.0.0	  CR-  rev  Cat: B (Rel-19)

					Source: Intel Corporation

Decision: 		The document was not treated.

R4-2504458	Discussion on RRM performance requirement for NR Less Than 5 MHz CA/DC

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

This  contribution discuss the  test cases scenarios for  PSCell addition change delay for less than 5 MHz  and SCell Activation and Deactivation Delay Requirement with less than 5 MHz.

Decision: 		The document was not treated.

R4-2504459	Draft CR for Rel-19 TC on PSCell addition change delay for less than 5 MHz

Type: draftCR		For: Endorsement
					38.133 v19.0.0	  CR-  rev  Cat: B (Rel-19)

					Source: Ericsson

Abstract:

This draftCR introduces test cases on PSCell addition change delay for less than 5 MHz

Decision: 		The document was not treated.

R4-2504460	Draft CR for Rel-19 TC on SCell Activation and Deactivation Delay Requirement with less than 5 MHz

Type: draftCR		For: Endorsement
					38.133 v19.0.0	  CR-  rev  Cat: B (Rel-19)

					Source: Ericsson

Abstract:

This draftCR introduces test cases on SCell Activation and Deactivation Delay Requirement with less than 5 MHz

Decision: 		The document was not treated.

R4-2504469	draftCR to 38.133 on SA event triggered reporting with PSCell 12 PRB SSB without

Type: discussion		For: Discussion
					38.133 v	  CR-  rev  Cat:  (Rel-19)

					Source: Nokia

Decision: 		The document was withdrawn.

R4-2504472	draftCR to 38.133 on SA event triggered reporting with PSCell 12 PRB SSB without DRX

Type: draftCR		For: Endorsement
					38.133 v19.0.0	  CR-  rev  Cat:  (Rel-19)

					Source: Nokia

Decision: 		The document was not treated.

#### 7.3.4	UE demodulation performance requirements

R4-2503209	Discussion on UE demodulation performance requirements for NR channel bandwidth less than 5MHz

Type: discussion		For: Discussion
					Source: China Telecom

Decision: 		The document was not treated.

R4-2503526	On UE Demodulation Performance Requirements for NR Channel BW less than 5MHz for FR1 Phase 2

Type: discussion		For: Discussion
					Source: Apple

Decision: 		The document was not treated.

R4-2503657	Discussion on UE demodulation performance requirements for less than 5MHz FR1 Phase 2

Type: discussion		For: Discussion
					Source: Samsung

Decision: 		The document was not treated.

R4-2503923	Discussion on UE CA Demodulation performance requirements for lessthan 5MHz

Type: discussion		For: Discussion
					Source: Nokia

Decision: 		The document was not treated.

R4-2504313	On CA performance requirements for less than 5MHz

Type: discussion		For: Discussion
					Source: Huawei,HiSilicon

Decision: 		The document was not treated.

R4-2504347	UE demodulation requirements for CA including 3MHz CBW

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

This contribution discusses the UE demodulation requirements consindering CA n100 and n101.

Decision: 		The document was not treated.

#### 7.3.5	Moderator summary and conclusions

R4-2503612	Topic summary for [114bis][202] NR_FR1_lessthan_5MHz_BW_Ph2

Type: other		For: Information
					Source: Moderator (Intel)

Abstract:

Topic summary in RRM session

Decision: 		The document was not treated.

R4-2504651	Topic summary for [114bis][326] NR_less_than_5MHz_Ph2_demod

Type: other		For: Information
					Source: Moderator (China Telecom)

Abstract:

[114bis] BDaT Session AI 7.3.4

Decision: 		The document was not treated.

R4-2504675	Topic summary for [114bis][117] NR_FR1_5MHz_BW_Ph2

Type: other		For: Information
					Source: Moderator(Intel)

Abstract:

Summary for AI 7.3, 7.3.1

Decision: 		The document was not treated.

### 7.4	Support of intra-band non-collocated EN-DC/NR-CA deployment Phase2: new receiver type(s)

#### 7.4.1	UE RF requirements

R4-2504119	Intra-band Non-collocated NR CA/EN-DC requirements

Type: other		For: Approval
					Source: Qualcomm France

Abstract:

Considerations for Non-collocated NR CA/EN-DC UE are provided in this contribution.

Decision: 		The document was not treated.

##### 7.4.1.1	UE RF requirements for Type 4a/4b capable FWA UE for EN-DC/NR-CA

R4-2503278	LS on Rel-19 signaling support for intra-band non-collocated NR-CA, EN-DC

Type: LS out		For: Approval
					to RAN2
					Source: Huawei Technologies France

Decision: 		The document was not treated.

R4-2503364	LS to RAN2 on Type-4 for non-collocated deployment

Type: LS out		For: Approval
					to RAN2
					Source: Samsung

Decision: 		The document was not treated.

R4-2504197	UE RF requirements for Type 4a/4b capable FWA UE for EN-DC/NR-CA

Type: other		For: Approval
					Source: Ericsson

Abstract:

Remaining requirements as per WF from last meeting

Decision: 		The document was not treated.

R4-2504203	draft CR to introduce Rel-19 intra-band non-collocated requirement

Type: draftCR		For: Endorsement
					38.101-1 v19.1.0	  CR-  rev  Cat:  (Rel-19)

					Source: OPPO

Decision: 		The document was not treated.

##### 7.4.1.2	UE Capability/UE behavior and network signaling for Type 4 EN-DC/NR-CA

R4-2503226	On non-collocated intra-band EN-DC and NR-CA open issues

Type: other		For: Approval
					Source: Nokia

Decision: 		The document was not treated.

R4-2503228	LS to RAN2 on Type-4 for non-collocated deployment

Type: LS out		For: Approval
					to RAN 2
					Source: Nokia

Decision: 		The document was not treated.

R4-2503277	Remaining discussions on UE capability for type 4 UEs

Type: discussion		For: (not specified)
					Source: Huawei Technologies France

Decision: 		The document was not treated.

R4-2503363	Discussion on UE and NW behavior for non-collocated deployment

Type: discussion		For: Discussion
					Source: Samsung

Decision: 		The document was not treated.

R4-2503583	On signalling support for type 4 UE

Type: discussion		For: Discussion
					Source: Apple

Decision: 		The document was not treated.

R4-2503714	Discussion on UE Capability and BS Signaling for non-collocated Type 4a4b

Type: discussion		For: Discussion
					Source: KDDI Corporation

Decision: 		The document was not treated.

R4-2503971	Discussion on UE Capability UE behavior and network signaling for  Type 4 NR CA/ENDC

Type: other		For: Approval
					Source: ZTE Corporation, Sanechips

Decision: 		The document was not treated.

R4-2504198	UE Capability/UE behavior and network signaling for Type 4 EN-DC/NR-CA

Type: other		For: Approval
					Source: Ericsson

Abstract:

Remaining requirements as per WF from last meeting

Decision: 		The document was not treated.

R4-2504209	UE capability and network signalling for type 4

Type: other		For: Approval
					Source: OPPO

Decision: 		The document was not treated.

##### 7.4.1.3	Other aspects (incl. clarification of contiguous LTE CCs)

R4-2503584	draft CR on scheduling availability for type 4 UE

Type: draftCR		For: Endorsement
					38.133 v19.0.0	  CR-  rev  Cat: B (Rel-19)

					Source: Apple

Decision: 		The document was not treated.

R4-2504208	other aspects

Type: other		For: Approval
					Source: OPPO

Decision: 		The document was not treated.

#### 7.4.2	RRM core requirements

R4-2503506	draftCR on MTTD/MRTD for intra-band non-collocated scenarios Phase 2

Type: draftCR		For: Endorsement
					38.133 v19.0.0	  CR-  rev  Cat:  (Rel-19)

					Source: Nokia, Nokia Shanghai Bell

Decision: 		The document was not treated.

R4-2503507	RRM requirements for intra-band non-collocated scenarios Phase2

Type: discussion		For: Discussion
					38.133 v	  CR-  rev  Cat:  (Rel-19)

					Source: Nokia, Nokia Shanghai Bell

Decision: 		The document was not treated.

R4-2503860	draftCR on SCell Activation Delay Requirement for NonCol_intraB

Type: draftCR		For: Endorsement
					38.133 v19.0.0	  CR-  rev  Cat: B (Rel-19)

					Source: ZTE Corporation, Sanechips

Decision: 		The document was not treated.

R4-2503881	Discussion on RRM requirements for supporting intra-band non-collocated EN-DC/NR-CA deployment Phase2: new receiver type(s)

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon

Decision: 		The document was not treated.

R4-2503882	Draft CR on RRM requirements for supporting intra-band non-collocated EN-DC/NR-CA deployment Phase2: new receiver type(s)

Type: draftCR		For: Endorsement
					38.133 v19.0.0	  CR-  rev  Cat: B (Rel-19)

					Source: Huawei, HiSilicon

Decision: 		The document was not treated.

R4-2504199	Remaining requirements for type 4 UEs in non-colocated scenario

Type: other		For: Approval
					Source: Ericsson

Abstract:

Remaining requirements as per WF from last meeting

Decision: 		The document was not treated.

R4-2504200	DraftCR: Scheduling availability of UE during radio link monitoring

Type: draftCR		For: Endorsement
					38.133 v19.0.0	  CR-  rev  Cat: B (Rel-19)

					Source: Ericsson

Abstract:

CR based on CR split.

The scheduling availibility rules have to be updated, since new capabilities and signals for intra-band non-contiguous carrier aggregation have been introduced, intraBandNR-CA-non-collocated-r19 and

nonCollocatedTypeNR-CA-r19.

Decision: 		The document was not treated.

R4-2504201	DraftCR: Scheduling availability of UE performing beam failure detection + L1-RSRP

Type: draftCR		For: Endorsement
					38.133 v19.0.0	  CR-  rev  Cat: B (Rel-19)

					Source: Ericsson

Abstract:

CR based on CR split.

The scheduling availibility rules have to be updated, since new capabilities and signals for intra-band non-contiguous carrier aggregation have been introduced, intraBandNR-CA-non-collocated-r19 and

nonCollocatedTypeNR-CA-r19.

Decision: 		The document was not treated.

R4-2504292	Draft CR on interruption requirements for Type 4 EN-DCNR-CA

Type: draftCR		For: Endorsement
					38.133 v19.0.0	  CR-  rev  Cat: B (Rel-19)

					Source: Samsung

Decision: 		The document was not treated.

#### 7.4.3	Moderator summary and conclusions

R4-2503613	Topic summary for [114bis][203] NonCol_intraB_ENDC_NR_CA_Ph2

Type: other		For: Information
					Source: Moderator (Huawei)

Abstract:

Topic summary in RRM session

Decision: 		The document was not treated.

R4-2504676	Topic summary for [114bis][118] NonCol_intraB_ENDC_NR_CA

Type: other		For: Information
					Source: Moderator(KDDI)

Abstract:

Summary for AI 7.4.1

Decision: 		The document was not treated.

### 7.5	Study on NR FR1 DL Fragmented Carriers

#### 7.5.1	General aspects

R4-2503479	TP to TR 38.755 on Clause 5

Type: discussion		For: Discussion
					Source: CATT

Decision: 		The document was not treated.

R4-2504105	draft update TR 38.755 FR1 Fragmented carriers_rm

Type: draft TR		For: Approval
					38.755 v0.1.0	  CR-  rev  Cat:  (Rel-19)

					Source: MediaTek Inc.

Abstract:

Draft update on TR38.755 to capture agreed TPs in RAN4#114

Decision: 		The document was not treated.

R4-2504129	General aspects of Fragmented CA

Type: other		For: Approval
					Source: Nokia

Decision: 		The document was not treated.

R4-2504211	draft update TR 38.755 FR1 Fragmented carriers

Type: draft TR		For: Agreement
					38.755 v0.2.0	  CR-  rev  Cat:  (Rel-19)

					Source: MediaTek Inc.

Abstract:

Reserved draft TR update to capture new approved TPs in RAN4#114bis

Decision: 		The document was not treated.

R4-2504452	TP on UE RF requirements impact for fragmented carriers

Type: pCR		For: Approval
					38.755 v0.0.1	  CR-  rev  Cat:  (Rel-19)

					Source: Apple

Decision: 		The document was not treated.

R4-2504453	Draft LS on signaling aspects for fragmented carriers

Type: LS out		For: Approval
					to RAN2
					Source: Apple

Decision: 		The document was not treated.

#### 7.5.2	Methods for reducing the number of UE Rx chains

R4-2503337	Methods for reducing the number of UE Rx chains

Type: discussion		For: Discussion
					Source: Xiaomi

Decision: 		The document was not treated.

R4-2503366	On fallback behavior and signalling aspect for Fragmented carriers

Type: discussion		For: Discussion
					Source: Samsung

Decision: 		The document was not treated.

R4-2503556	Procedures for fragmented carriers

Type: other		For: Approval
					Source: Qualcomm Incorporated

Abstract:

Vote for Gene!

Decision: 		The document was not treated.

R4-2503835	Discussion on methods for reducing the number of UE RX chains

Type: other		For: Approval
					Source: vivo

Decision: 		The document was not treated.

R4-2503963	Discussion the methods for reducing the number of UE Rx chain

Type: other		For: Approval
					Source: ZTE Corporation, Sanechips

Decision: 		The document was not treated.

R4-2504036	Discussion on UE capability indication for single Rx chain

Type: other		For: Approval
					Source: Ericsson

Abstract:

The paper will discuss some signaling-related issues of fragmented carriers.

Decision: 		The document was not treated.

R4-2504102	Discussion on methods for reducing the number of UE Rx chains for fragmented carriers

Type: discussion		For: Approval
					Source: Google

Decision: 		The document was not treated.

R4-2504153	Discussion on reducing the number of UE Rx chains

Type: discussion		For: Discussion
					Source: OPPO

Decision: 		The document was not treated.

R4-2504173	Discussion on methods for reducing the number of UE Rx chains

Type: other		For: Approval
					Source: Nokia

Decision: 		The document was not treated.

R4-2504179	Discussion on methods for reducing the number of UE RX chains

Type: discussion		For: Discussion
					Source: Spreadtrum,UNISOC

Decision: 		The document was not treated.

R4-2504213	Discussion on one Rx RF chain mode signalling aspects

Type: pCR		For: Approval
					38.755 v0.0.1	  CR-  rev  Cat:  (Rel-19)

					Source: MediaTek Inc.

Decision: 		The document was not treated.

R4-2504260	Views on the usage of the FR1 fragmented carriers

Type: discussion		For: Discussion
					Source: CHTTL

Decision: 		The document was not treated.

R4-2504283	On signalling aspect of fragmented carriers

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon

Decision: 		The document was not treated.

R4-2504450	On methods for reducing the number of UE Rx chains for fragmented carriers

Type: discussion		For: Discussion
					Source: Apple

Decision: 		The document was not treated.

#### 7.5.3	Impacts on UE RF requirements and DL performance

R4-2503338	Impacts on UE RF requirements and DL performance

Type: discussion		For: Discussion
					Source: Xiaomi

Decision: 		The document was not treated.

R4-2503557	UE performance in one RX mode

Type: other		For: Approval
					Source: Qualcomm Incorporated

Abstract:

Vote for Gene!

Decision: 		The document was not treated.

R4-2503558	TP for TR38.755: Company C evaluation results

Type: pCR		For: Approval
					38.755 v0.0.1	  CR-  rev  Cat:  (Rel-19)

					Source: Qualcomm Incorporated

Abstract:

Vote for Gene!

Decision: 		The document was not treated.

R4-2503836	Impacts on UE RF requirements and DL performance

Type: other		For: Approval
					Source: vivo

Decision: 		The document was not treated.

R4-2503964	Discussion on the RF requirement for fragmented carrier

Type: other		For: Approval
					Source: ZTE Corporation, Sanechips

Decision: 		The document was not treated.

R4-2504037	Discussion on the DL PCell REFSENS degradation of fragmented carriers

Type: other		For: Approval
					Source: Ericsson

Abstract:

The paper will discuss the impact on UE RF requirements of fragmented carriers.

Decision: 		The document was not treated.

R4-2504128	Discussion on RF requirements for Fragmented CA

Type: discussion		For: Approval
					Source: Nokia

Decision: 		The document was not treated.

R4-2504154	Discussion on impacts on UE RF requirements

Type: discussion		For: Discussion
					Source: OPPO

Decision: 		The document was not treated.

R4-2504214	Discussion of impacts on UE RF requirements and DL performance

Type: pCR		For: Approval
					38.755 v0.0.1	  CR-  rev  Cat:  (Rel-19)

					Source: MediaTek Inc.

Decision: 		The document was not treated.

R4-2504239	Discussion of impacts on UE RF requirements and DL performance

Type: discussion		For: Discussion
					Source: Spreadtrum,UNISOC

Decision: 		The document was not treated.

R4-2504284	On RF requirements of fragmented carriers

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon

Decision: 		The document was not treated.

R4-2504426	Link budget calculation results for fragmented carrier

Type: discussion		For: Discussion
					Source: Samsung

Decision: 		The document was not treated.

R4-2504451	On UE RF requirements for fragmented carriers

Type: discussion		For: Discussion
					Source: Apple

Decision: 		The document was not treated.

#### 7.5.4	Moderator summary and conclusions

R4-2504677	Topic summary for [114bis][119] FS_NR_DL_Frag_Carrier

Type: other		For: Information
					Source: Moderator(Mediatek)

Abstract:

Summary for AI 7.5

Decision: 		The document was not treated.

### 7.6	NR power class 2 RedCap (Reduced Capability) UE in FR1

#### 7.6.1	UE RF requirements

R4-2503198	On PC2 RedCap n1 and n3 1Rx REFSENS

Type: other		For: Approval
					Source: Apple

Decision: 		The document was not treated.

R4-2503318	Discussion on PC2 RedCap RSD

Type: discussion		For: Discussion
					Source: Xiaomi

Decision: 		The document was not treated.

R4-2503457	Discussion on UE RF requirements for RedCap PC2 FDD

Type: other		For: Approval
					Source: CATT

Decision: 		The document was not treated.

R4-2503458	Draft CR for 38.101-1: Introduction onpower class 2 RedCap for TDD bands and FDD bands

Type: draftCR		For: Endorsement
					38.101-1 v19.1.0	  CR-  rev  Cat: B (Rel-19)

					Source: CATT

Decision: 		The document was not treated.

R4-2503696	Discussion on the Rel-19 PC2 FDD RedCap UE

Type: other		For: Approval
					Source: Huawei, HiSilicon

Decision: 		The document was not treated.

R4-2503783	RF Rx requirements for PC2 RedCap in FDD bands

Type: discussion		For: Discussion
					Source: China Telecom

Decision: 		The document was not treated.

R4-2503784	Draft CR for 38.101-1 to introduce PC2 RedCap

Type: draftCR		For: Endorsement
					38.101-1 v19.1.0	  CR-  rev  Cat:  (Rel-19)

					Source: China Telecom

Decision: 		The document was not treated.

R4-2503961	On PC2 RedCap in FR1

Type: other		For: Approval
					Source: ZTE Corporation, Sanechips

Decision: 		The document was not treated.

R4-2504120	RedCap PC2 FDD REFSENS

Type: other		For: Approval
					Source: Qualcomm France

Abstract:

RedCap PC2 1Rx FD-FDD REFSENS for n1 and n3 is analyzed in this contribution.

Decision: 		The document was not treated.

R4-2504402	RF Impact for FDD bands for PC2 RedCap

Type: other		For: Approval
					Source: Ericsson

Abstract:

In this paper, we present our view on PC2 RedCap UE RF for FDD bands added in WID [1].

Decision: 		The document was not treated.

#### 7.6.2	Moderator summary and conclusions

R4-2504678	Topic summary for [114bis][120] NR_PC2_RedCap_UE

Type: other		For: Information
					Source: Moderator(China Telecom)

Abstract:

Summary for AI 7.6

Decision: 		The document was not treated.

### 7.7	Low band carrier aggregation via switching

#### 7.7.1	General aspects

R4-2503477	TP to TR 38.768 on switching periods

Type: pCR		For: Approval
					38.768 v0.0.1	  CR-  rev  Cat:  (Rel-19)

					Source: CATT

Decision: 		The document was not treated.

R4-2503550	TP to TR38.768 on operator provided scenarios for low band aggregation via switching

Type: pCR		For: Approval
					38.768 v0.0.2	  CR-  rev  Cat:  (Rel-19)

					Source: Apple

Decision: 		The document was not treated.

#### 7.7.2	UE RF requirements and UE capability

R4-2503152	Discussion on LB-LB switching

Type: other		For: Approval
					Source: Huawei, Hisilicon

Decision: 		The document was not treated.

R4-2503194	UE RF requirements for LB CA via switching

Type: discussion		For: Discussion
					Source: Apple

Decision: 		The document was not treated.

R4-2503244	PA transients for LB-LB switching

Type: other		For: Approval
					38.101-1 v	  CR-  rev  Cat:  (Rel-19)

					Source: Skyworks Solutions Inc.

Abstract:

This paper discusses ON-to-OFF and OFF-to-ON PA transient considerations for LB-LB switching.

Decision: 		The document was not treated.

R4-2503336	LB (FDD) – LB (SDL) CA with switching

Type: discussion		For: Discussion
					Source: Xiaomi

Decision: 		The document was not treated.

R4-2503478	On UE RF requirements and UE capability for low band CA via switching

Type: discussion		For: Discussion
					Source: CATT

Decision: 		The document was not treated.

R4-2503561	RX Switching time, frequency and location

Type: other		For: Approval
					Source: Qualcomm Incorporated

Abstract:

Vote for Gene!

Decision: 		The document was not treated.

R4-2503599	Discussion on UE RF requirements of LB carrier aggregation

Type: discussion		For: Discussion
					Source: LG Electronics

Abstract:

It is for discussion of UE RF requirements for LB-LB CA.

Decision: 		The document was not treated.

R4-2503731	Discussion on UE RF requirements for LB-LB CA switching

Type: discussion		For: Discussion
					Source: Spreadtrum,UNISOC

Decision: 		The document was not treated.

R4-2503845	Discussion of UE RF impact for LB-LB switching

Type: other		For: Approval
					Source: vivo

Decision: 		The document was not treated.

R4-2503962	View on low-low band CA via switching

Type: other		For: Approval
					Source: ZTE Corporation, Sanechips

Decision: 		The document was not treated.

R4-2503994	NR_LBCA_Sw Considerations

Type: other		For: Approval
					Source: Nokia

Decision: 		The document was not treated.

R4-2504101	Discussion on low-band CA switching

Type: discussion		For: Approval
					38.101-1 v	  CR-  rev  Cat:  (Rel-19)

					Source: MediaTek Inc.

Decision: 		The document was not treated.

R4-2504104	Discussion on low band carrier aggregation via switching

Type: discussion		For: Approval
					Source: Google

Decision: 		The document was not treated.

R4-2504190	RF requirements for low-band CA via switching

Type: other		For: Approval
					Source: Ericsson

Abstract:

In this contribution we discuss the switching time and propose time masks and a UE reference architectrure for LB-LB switching

Decision: 		The document was not treated.

R4-2504388	UE RF requirements for LB CA via switching

Type: discussion		For: Discussion
					Source: Samsung

Decision: 		The document was not treated.

#### 7.7.3	RRM core requirements

R4-2503256	RRM requirements for low band carrier aggregation via switching

Type: discussion		For: Discussion
					Source: Qualcomm Incorporated

Decision: 		The document was not treated.

R4-2503329	Discussion on LB-CA RRM requirements

Type: discussion		For: Discussion
					Source: Xiaomi

Decision: 		The document was not treated.

R4-2503359	Discussion on RRM requirements for low band carrier aggregation via switching

Type: other		For: Approval
					Source: OPPO

Decision: 		The document was not treated.

R4-2503421	Discussion on the impacts of low band CA via switching on RRM requirements

Type: discussion		For: Discussion
					Source: CATT

Decision: 		The document was not treated.

R4-2503532	On RRM requirement for R19 LB CA

Type: discussion		For: Discussion
					38.133 v	  CR-  rev  Cat:  (Rel-19)

					Source: Apple

Decision: 		The document was not treated.

R4-2503702	Discussion on RRM requirements for Low Band CA operation

Type: discussion		For: Discussion
					Source: LG Electronics Inc.

Decision: 		The document was not treated.

R4-2503890	Discussion on low band carrier aggregation via switching

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon

Decision: 		The document was not treated.

R4-2503933	Discussion on RRM core requirements for LB CA via switching

Type: discussion		For: Discussion
					Source: Ericsson

Decision: 		The document was not treated.

R4-2504028	Further discussion on RRM requirements for LB CA via switching

Type: discussion		For: Discussion
					Source: vivo

Decision: 		The document was not treated.

R4-2504253	Discussion on RRM aspects of R19 Low band carrier aggregation via switching

Type: discussion		For: Approval
					Source: ZTE Corporation, Sanechips

Decision: 		The document was not treated.

R4-2504285	General discussion on RRM requirements for LB CA switching

Type: discussion		For: Discussion
					38.133 v	  CR-  rev  Cat:  (Rel-19)

					Source: Samsung

Decision: 		The document was not treated.

R4-2504478	Discussion on low-band CA switching

Type: discussion		For: Discussion
					38.133 v	  CR-  rev  Cat:  (Rel-19)

					Source: MediaTek inc.

Decision: 		The document was not treated.

R4-2504499	On LBCA RRM requirement impact

Type: discussion		For: Discussion
					Source: Nokia

Decision: 		The document was not treated.

#### 7.7.4	Moderator summary and conclusions

R4-2503614	Topic summary for [114bis][204] NR_LBCA_Sw

Type: other		For: Information
					Source: Moderator (Apple)

Abstract:

Topic summary in RRM session

Decision: 		The document was not treated.

R4-2504679	Topic summary for [114bis][121] NR_LBCA_Sw

Type: other		For: Information
					Source: Moderator(Apple)

Abstract:

Summary for AI 7.7

Decision: 		The document was not treated.

### 7.8	NR FR1 7 MHz Channel Bandwidth

#### 7.8.1	General aspects

R4-2503473	Clarification on the usage of 7MHz channel bandwidth

Type: discussion		For: Discussion
					Source: CATT

Decision: 		The document was not treated.

R4-2503476	draftCR to TS 38.115-1 on 7MHz channel bandwidth

Type: draftCR		For: Endorsement
					38.115-1 v19.0.0	  CR-  rev  Cat: B (Rel-19)

					Source: CATT

Decision: 		The document was not treated.

#### 7.8.2	UE RF requirements and system parameters

R4-2503192	On the introduction of 7MHz CBW

Type: discussion		For: Discussion
					Source: Apple

Decision: 		The document was not treated.

R4-2503232	n26 7 MHz A-MPR simulations and proposal

Type: other		For: Approval
					Source: Nokia

Decision: 		The document was not treated.

R4-2503262	Discussion on initial BWP size restriction to 5 MHz for 7MHz Channel Bandwidth

Type: discussion		For: Discussion
					Source: China Telecom

Decision: 		The document was not treated.

R4-2503276	draftCR for Tx RF requirements for introduction of 7MHz

Type: draftCR		For: Endorsement
					38.101-1 v19.1.0	  CR-  rev  Cat: B (Rel-19)

					Source: Huawei Technologies France

Decision: 		The document was not treated.

R4-2503474	Further discussion on UE RF requirements for 7MHz channel bandwidth

Type: discussion		For: Discussion
					Source: CATT

Decision: 		The document was not treated.

R4-2503642	Discussion on A-MPR of 7MHz for band n26

Type: other		For: Approval
					Source: ZTE Corporation, Sanechips

Decision: 		The document was not treated.

R4-2503950	7 MHz channel BW: UE RF and system parameters

Type: other		For: Approval
					Source: Ericsson

Abstract:

This contribution discusses the UE RF and system parameters when introducing support for 7 MHz channel BW

Decision: 		The document was not treated.

R4-2504261	Discussion on UE RF requirements for 7MHz CBW

Type: discussion		For: Approval
					Source: MediaTek (Wuhan) Inc.

Decision: 		The document was not treated.

R4-2504475	draft CR to 38.101-1: Introduction of 7 MHz channel bandwidth for n26 and n5

Type: draftCR		For: Endorsement
					38.101-1 v19.1.0	  CR-  rev  Cat: B (Rel-19)

					Source: Nokia

Decision: 		The document was not treated.

R4-2504484	Band n26 A-MPR Analysis for 7 MHz Channel Bandwidth

Type: discussion		For: Discussion
					Source: Qualcomm Incorporated

Abstract:

Proposal for discussion on A-MPR for 7 MHz CHBW in band n26

Decision: 		The document was not treated.

R4-2504485	7 MHz Channel Bandwidth for n5 and n26

Type: draftCR		For: Endorsement
					38.101-1 v19.1.0	  CR-  rev  Cat:  (Rel-19)

					Source: Qualcomm Incorporated

Abstract:

Proposal to include 7 MHz channel bandwidth in bands n5 and n26

Decision: 		The document was not treated.

#### 7.8.3	BS RF core requirements

R4-2503234	On BS specification impact due to introduction of 7MHz channel bandwidth

Type: other		For: Approval
					Source: Nokia

Decision: 		The document was not treated.

R4-2503475	Further discussion on BS RF requirements for 7MHz channel bandwidth

Type: discussion		For: Discussion
					Source: CATT

Decision: 		The document was not treated.

R4-2503643	Discussion on BS RF requirements for supporting 7 MHz channel bandwidth

Type: other		For: Approval
					Source: ZTE Corporation, Sanechips

Decision: 		The document was not treated.

R4-2503644	Draft CR to TS37.104: Introduction of 7 MHz NR FR1 channel bandwidth

Type: draftCR		For: Endorsement
					37.104 v19.0.0	  CR-  rev  Cat:  (Rel-19)

					Source: ZTE Corporation, Sanechips

Decision: 		The document was not treated.

R4-2503951	7 MHz channel BW: BS RF impacts

Type: other		For: Approval
					Source: Ericsson

Abstract:

This contribution discusses the BS RF when introducing support for 7 MHz channel BW

Decision: 		The document was not treated.

R4-2503952	Draft running CR to TS 38.104 - Introduction of 7 MHz channel BW

Type: draftCR		For: Endorsement
					38.104 v19.0.0	  CR-  rev  Cat: B (Rel-19)

					Source: Ericsson

Abstract:

This draft CR is the running CR to TS 38.104 capturing all agreements made when introducing 7 MHz channel BW

Decision: 		The document was not treated.

R4-2504588	OTA requirements consideration for network-nodes specifications for 7MHz channel introduction

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon

Abstract:

In this contribution, we provide feedback on the consideration of OTA requirements for network-nodes specifications for the introduction of 7 MHz channel bandwidth.

Decision: 		The document was not treated.

R4-2504589	Draft CR to TS 37.105: 7MHz channel bandwidth introduction

Type: draftCR		For: Endorsement
					37.105 v19.0.0	  CR-  rev  Cat: B (Rel-19)

					Source: Huawei, HiSilicon

Abstract:

Introduction of 7MHz channel bandwidth for NR.

Decision: 		The document was not treated.

R4-2504590	Draft CR to TS 37.145-1: 7MHz channel bandwidth introduction

Type: draftCR		For: Endorsement
					37.145-1 v19.0.0	  CR-  rev  Cat: B (Rel-19)

					Source: Huawei, HiSilicon

Abstract:

Introduction of 7MHz channel bandwidth for NR.

Decision: 		The document was not treated.

R4-2504591	Draft CR to TS 37.145-2: 7MHz channel bandwidth introduction

Type: draftCR		For: Endorsement
					37.145-2 v19.0.0	  CR-  rev  Cat: B (Rel-19)

					Source: Huawei, HiSilicon

Abstract:

Introduction of 7MHz channel bandwidth for NR.

Decision: 		The document was not treated.

R4-2504592	Draft CR to TS 38.141-2: 7MHz channel bandwidth introduction

Type: draftCR		For: Endorsement
					38.141-2 v19.0.0	  CR-  rev  Cat: B (Rel-19)

					Source: Huawei, HiSilicon

Abstract:

Introduction of 7MHz channel bandwidth for NR.

Decision: 		The document was not treated.

R4-2504593	Draft CR to TS 38.176-2: restriction of 7MHz channel bandwidth introduction

Type: draftCR		For: Endorsement
					38.176-2 v19.0.0	  CR-  rev  Cat: B (Rel-19)

					Source: Huawei, HiSilicon

Abstract:

Introduction of restriction for 7MHz channel bandwidth for IAB, based on the agreement captured in the WF in R4-2502870.

Decision: 		The document was not treated.

#### 7.8.4	Moderator summary and conclusions

R4-2504680	Topic summary for [114bis][122] NR_FR1_7MHz_BW

Type: other		For: Information
					Source: Moderator(Ericsson)

Abstract:

Summary for AI 7.8

Decision: 		The document was not treated.

### 7.9	New LTE band for 5G broadcast for region 3 utilizing a geosynchronous satellite

#### 7.9.1	General aspects and work plan

R4-2504070	Work plan for WID: New LTE band for 5G broadcast for region 3 utilizing a geosynchronous satellite

Type: Work Plan		For: Approval
					Source: ASTRUM MOBILE PTE. LTD.

Decision: 		The document was not treated.

#### 7.9.2	Band definition and system parameters

R4-2503747	Discussions on band definition and system parameters for supporting new SDO band for LTE-based 5G Broadcast

Type: other		For: Approval
					Source: ZTE Corporation, Sanechips

Decision: 		The document was not treated.

R4-2504053	Discussion on band definition and system parameters

Type: other		For: Approval
					Source: Qualcomm Incorporated, Astrum Mobile

Decision: 		The document was not treated.

#### 7.9.3	UE RF core requirements

R4-2503549	On Rx requirements for new satellite broadcast band

Type: other		For: Approval
					Source: Apple

Decision: 		The document was not treated.

R4-2503748	Discussion on UE RF requirements for supporting new SDO band for LTE-based 5G Broadcast

Type: other		For: Approval
					Source: ZTE Corporation, Sanechips

Decision: 		The document was not treated.

R4-2504054	Discussion on UE RF requirements

Type: other		For: Approval
					Source: Qualcomm Incorporated

Decision: 		The document was not treated.

#### 7.9.4	SAN RF core requirements

R4-2503749	Discussion on SAN RF requirements for supporting new SDO band for LTE-based 5G Broadcast

Type: other		For: Approval
					Source: ZTE Corporation, Sanechips

Decision: 		The document was not treated.

R4-2504093	Discussion on SAN RF requirements

Type: discussion		For: Approval
					Source: ASTRUM MOBILE PTE. LTD.

Decision: 		The document was not treated.

#### 7.9.5	Moderator summary and conclusions

R4-2504643	Topic summary for [114bis][318] 5G_Broadcast_GSO_NTN_band

Type: other		For: Information
					Source: Moderator (Astrum)

Abstract:

[114bis] BDaT Session AI 7.9.1, 7.9.2, 7.9.3, 7.9.4

Decision: 		The document was not treated.

### 7.10	Enhanced requirements and conductive test methodology for NR NTN and IoT NTN

#### 7.10.1	General aspects

R4-2504330	Potential impacts on spec and TR from HPUE

Type: other		For: Approval
					Source: Samsung

Decision: 		The document was not treated.

#### 7.10.2	UE RF requirements for NTN HPUE

R4-2503708	On NR-NTN HPUE requirements

Type: discussion		For: Discussion
					Source: ViaSat Satellite Holdings Ltd, Inmarsat

Decision: 		The document was not treated.

##### 7.10.2.1	Coexistence study for example bands

R4-2503319	NTN and IoT-NTN HPUE co-existence study and calibration

Type: discussion		For: Approval
					Source: xiaomi

Decision: 		The document was not treated.

R4-2504331	Simulation assumption for co-existence study

Type: discussion		For: Information
					Source: Samsung

Decision: 		The document was not treated.

R4-2504332	Summary of NTN HPUE co-existence study

Type: other		For: Approval
					Source: Samsung

Decision: 		The document was not treated.

##### 7.10.2.2	NR-NTN HPUE RF requirements

###### 7.10.2.2.1	Tx requirements

R4-2503180	Further considerations on the high-power UE Tx requirements

Type: discussion		For: Discussion
					Source: Apple

Decision: 		The document was not treated.

R4-2503181	Initial power back-off results for PC2 on NTN L-/S-band

Type: discussion		For: Discussion
					Source: Apple, Globalstar

Decision: 		The document was not treated.

R4-2503182	Initial power back-off results for PC2 on NTN S-band

Type: discussion		For: Discussion
					Source: Apple

Decision: 		The document was not treated.

R4-2503183	On restricted set of power back-off simulations for the satellite band UE Tx requirements

Type: discussion		For: Discussion
					Source: Apple

Decision: 		The document was not treated.

R4-2503448	Discussion on UE Tx requirement for NTN support HPUE

Type: other		For: Approval
					Source: CATT

Decision: 		The document was not treated.

R4-2503699	Discussion on Tx requirements for NR NTN HPUE

Type: other		For: Approval
					Source: Huawei, HiSilicon

Decision: 		The document was not treated.

R4-2503719	Discussion for UE RF Tx requirements of NTN_NR_HPUE

Type: discussion		For: Discussion
					Source: LG Electronics France

Decision: 		The document was not treated.

R4-2503840	NR NTN HPUE Tx Requirements and feasibility analysis

Type: other		For: Approval
					Source: vivo

Decision: 		The document was not treated.

R4-2504155	Discussion on Tx requirements for NR-NTN HPUE

Type: discussion		For: Discussion
					Source: OPPO

Decision: 		The document was not treated.

R4-2504333	On NR-NTN HPUE Tx requirements

Type: other		For: Approval
					Source: Samsung

Decision: 		The document was not treated.

R4-2504376	Discussion on NR-NTN HPUE TX RF requirements

Type: discussion		For: Approval
					38.101-5 v	  CR-  rev  Cat:  (Rel-19)

					Source: MediaTek (Hefei) Inc.

Decision: 		The document was not treated.

R4-2504546	NR NTN HPUE Tx requirements

Type: other		For: Approval
					Source: Qualcomm Inc.

Decision: 		The document was not treated.

R4-2504584	On the UE RF Tx requirements for NR-NTN HPUE

Type: other		For: Approval
					Source: Ericsson

Abstract:

Proposals on the open issues for the UE RF Tx requirements for NR-NTN HPUE.

Decision: 		The document was not treated.

R4-2504610	Initial power boosting result for NR-NTN for a PC2 NR PA

Type: other		For: Approval
					Source: Skyworks Solutions Inc.

Abstract:

. In this contribution, we provide preliminary measurement power boosting results on a PC2 1Tx PA. Further investigations will be needed as the measurement was made with external current and thermal limitations based on NR waveforms which did not allow th

Decision: 		The document was not treated.

###### 7.10.2.2.2	Rx requirements

R4-2503322	RF requirements for NR-NTN

Type: discussion		For: Approval
					Source: xiaomi

Decision: 		The document was not treated.

R4-2503449	Discussion on UE Rx requirement for NTN support HPUE

Type: other		For: Approval
					Source: CATT

Decision: 		The document was not treated.

R4-2503700	Discussion on Rx requirements for NR NTN HPUE

Type: other		For: Approval
					Source: Huawei, HiSilicon

Decision: 		The document was not treated.

R4-2503722	Discussion for UE RF Rx requirements of NTN_NR_HPUE

Type: discussion		For: Discussion
					Source: LG Electronics

Decision: 		The document was not treated.

R4-2503841	NR NTN HPUE Rx Requirements

Type: other		For: Approval
					Source: vivo

Decision: 		The document was not treated.

R4-2504156	Discussion on Rx requirements for NR-NTN HPUE

Type: discussion		For: Discussion
					Source: OPPO

Decision: 		The document was not treated.

R4-2504334	On NR-NTN HPUE Rx requirements

Type: other		For: Approval
					Source: Samsung

Decision: 		The document was not treated.

R4-2504379	Discussion on NR-NTN HPUE RX RSD

Type: discussion		For: Approval
					38.101-5 v	  CR-  rev  Cat:  (Rel-19)

					Source: MediaTek (Hefei) Inc.

Decision: 		The document was not treated.

R4-2504547	NR NTN HPUE Rx requirements

Type: other		For: Approval
					Source: Qualcomm Inc.

Decision: 		The document was not treated.

R4-2504585	On the UE RF Rx requirements for NR-NTN HPUE

Type: other		For: Approval
					Source: Ericsson

Abstract:

Proposals on the open issues for the UE RF Rx requirements for NR-NTN HPUE.

Decision: 		The document was not treated.

##### 7.10.2.3	IoT-NTN HPUE RF requirements

###### 7.10.2.3.1	Tx requirements

R4-2503321	RF requirements for IoT-NTN

Type: discussion		For: Approval
					Source: xiaomi

Decision: 		The document was not treated.

R4-2503842	Iot NTN HPUE Tx Requirements and feasibility analysis

Type: other		For: Approval
					Source: vivo, SmarterMicro

Decision: 		The document was not treated.

R4-2504157	Discussion on Tx requirements for IoT-NTN HPUE

Type: discussion		For: Discussion
					Source: OPPO

Decision: 		The document was not treated.

R4-2504172	R19 on NTN SAR

Type: other		For: Approval
					Source: OPPO

Decision: 		The document was not treated.

R4-2504335	On IoT-NTN HPUE Tx requirements and handheld SAR

Type: other		For: Approval
					Source: Samsung

Decision: 		The document was not treated.

R4-2504378	Discussion on IoT-NTN HPUE TX RF requirements

Type: discussion		For: Approval
					Source: MediaTek (Hefei) Inc.

Decision: 		The document was not treated.

R4-2504548	IoT NTN HPUE Tx requirements

Type: other		For: Approval
					Source: Qualcomm Inc.

Decision: 		The document was not treated.

R4-2504586	On the UE RF Tx requirements for IoT-NTN HPUE

Type: other		For: Approval
					Source: Ericsson, Sony, Nokia

Abstract:

Proposals on the SAR related issues for the UE RF Tx requirements for IoT-NTN HPUE.

Decision: 		The document was not treated.

R4-2504604	Initial power boosting result for NB-IoT-NTN for a PC2 NR PA

Type: other		For: Approval
					Source: Skyworks Solutions Inc.

Abstract:

In this contribution, we provide preliminary measurement results on a PC2 1Tx PA. Further investigations will be needed as the measurement was made with external current and thermal limitations based on NR waveforms which did not allow the maximum boostin

Decision: 		The document was not treated.

###### 7.10.2.3.2	Rx requirements

R4-2503843	Iot NTN HPUE Rx Requirements

Type: other		For: Approval
					Source: vivo

Decision: 		The document was not treated.

R4-2504158	Discussion on Rx requirements for IoT-NTN HPUE

Type: discussion		For: Discussion
					Source: OPPO

Decision: 		The document was not treated.

R4-2504336	On IoT-NTN HPUE Rx requirements

Type: other		For: Approval
					Source: Samsung

Decision: 		The document was not treated.

R4-2504385	Discussion on IoT-NTN HPUE RX RSD

Type: discussion		For: Approval
					Source: MediaTek (Hefei) Inc.

Decision: 		The document was not treated.

R4-2504549	IoT NTN HPUE Rx requirements

Type: other		For: Approval
					Source: Qualcomm Inc.

Decision: 		The document was not treated.

R4-2504587	On the UE RF Rx requirements for IoT-NTN HPUE

Type: other		For: Approval
					Source: Ericsson

Abstract:

Proposals on the open issues for the UE RF Rx requirements for IoT-NTN HPUE.

Decision: 		The document was not treated.

#### 7.10.3	Less than 5MHz for NTN

##### 7.10.3.1	System parameters

R4-2503446	Discussion on system parameter for NTN support spectrum less than 5 MHz

Type: other		For: Approval
					Source: CATT

Decision: 		The document was not treated.

R4-2503739	LS on UE capability signalling for NTN less than 5MHz

Type: LS out		For: Approval
					to RAN2
					Source: ZTE Corporation, Sanechips

Decision: 		The document was not treated.

R4-2504582	On system parameters for less than 5 MHz CBW for NTN

Type: other		For: Approval
					Source: Ericsson

Abstract:

Proposals on the open issue for system parameters for less than 5 MHz CBW for FR1-NTN.

Decision: 		The document was not treated.

##### 7.10.3.2	UE RF requirements

R4-2503184	Power back-off values for the 3MHz channel in band n254

Type: discussion		For: Discussion
					Source: Apple, Globalstar

Decision: 		The document was not treated.

R4-2503185	Power back-off values for the 3MHz channel in band n256

Type: discussion		For: Discussion
					Source: Apple

Decision: 		The document was not treated.

R4-2503186	Addition of the 3MHz channel to TS 38.101-5 (sub-clause 6.2)

Type: draftCR		For: Endorsement
					38.101-5 v19.0.0	  CR-  rev  Cat: B (Rel-19)

					Source: Apple

Decision: 		The document was not treated.

R4-2503187	Addition of the 3MHz channel to TR 38.863

Type: draftCR		For: Endorsement
					38.863 v18.5.0	  CR-  rev  Cat: B (Rel-19)

					Source: Apple, Thales, Echostar, Aalyria Technologies

Decision: 		The document was not treated.

R4-2503188	Addition of the 3MHz channel to TR 38.741

Type: draftCR		For: Endorsement
					38.741 v18.4.0	  CR-  rev  Cat: B (Rel-19)

					Source: Apple, Globalstar

Decision: 		The document was not treated.

R4-2503265	draftCR for RF requirements for introduction of 3MHz on NTN bands

Type: draftCR		For: Endorsement
					38.101-5 v19.0.0	  CR-  rev  Cat: B (Rel-19)

					Source: Huawei Technologies France

Decision: 		The document was not treated.

R4-2503266	REFSENS for NTN bands supporting 3MHz CBW

Type: discussion		For: (not specified)
					Source: Huawei Technologies France

Decision: 		The document was not treated.

R4-2503330	Discussion on UE RF requirements for NTN less than 5MHz

Type: discussion		For: Discussion
					Source: Xiaomi

Decision: 		The document was not treated.

R4-2503331	Draft big CR on UE RF requirements for NTN less than 5MHz

Type: draftCR		For: Endorsement
					38.101-5 v19.0.0	  CR-  rev  Cat:  (Rel-19)

					Source: Xiaomi

Decision: 		The document was not treated.

R4-2503447	Discussion on UE RF requirements for NTN support spectrum less than 5 MHz

Type: other		For: Approval
					Source: CATT

Decision: 		The document was not treated.

R4-2504259	NTN n254 3 MHz A-MPR proposal + other CBWs

Type: other		For: Approval
					Source: Nokia

Decision: 		The document was not treated.

R4-2504337	On NTN Less than 5MHz UE RF requirements

Type: other		For: Approval
					Source: Samsung

Decision: 		The document was not treated.

R4-2504410	Discussion on NR-NTN 3MHz REFSENS requirements

Type: discussion		For: Approval
					38.101-5 v	  CR-  rev  Cat:  (Rel-19)

					Source: MediaTek (Hefei) Inc.

Decision: 		The document was not treated.

R4-2504554	UE RF requirements for NTN less than 5 MHz

Type: other		For: Approval
					Source: Qualcomm Inc.

Decision: 		The document was not treated.

R4-2504583	On the UE RF requirements for less than 5 MHz CBW for NTN

Type: other		For: Approval
					Source: Ericsson

Abstract:

Proposal on the open issues for the UE RF Rx requirements for less than 5 MHz for FR1-NTN.

Decision: 		The document was not treated.

##### 7.10.3.3	SAN RF core and conformance requirements

R4-2503166	Draft big CR to TS 38.181 to introduce less than 5MHz for NTN

Type: draftCR		For: Endorsement
					38.181 v18.5.0	  CR-  rev  Cat: B (Rel-19)

					Source: Nokia, ZTE Corporation, Sanechips, CATT, Samsung

Abstract:

Introduce less than 5MHz for NTN.

Decision: 		The document was not treated.

R4-2503201	Draft CR to TS 38.181 for section 4 and 6

Type: draftCR		For: Endorsement
					38.181 v18.5.0	  CR-  rev  Cat: B (Rel-19)

					Source: Samsung

Abstract:

To introduce 3MHz in TS 38.181 in section 4 and 6 for SAN conformance testing.

Decision: 		The document was not treated.

##### 7.10.3.4	RRM core requirements

R4-2503323	Discussion on RRM requirements for less than 5MHz NTN

Type: discussion		For: Discussion
					Source: Xiaomi

Decision: 		The document was not treated.

R4-2503325	draftCR on NTN less than 5MHz (9.2C NR intra-frequency measurements for SAN)

Type: draftCR		For: Endorsement
					38.133 v19.0.0	  CR-  rev  Cat: B (Rel-19)

					Source: Xiaomi

Decision: 		The document was not treated.

R4-2503326	Draft big CR on RRM requirements for NTN less than 5MHz in Rel-19

Type: draftCR		For: Endorsement
					38.133 v19.0.0	  CR-  rev  Cat: B (Rel-19)

					Source: Xiaomi

Decision: 		The document was not treated.

R4-2503423	Further discussion on less than 5MHz for NTN RRM requirement

Type: discussion		For: Discussion
					Source: CATT

Decision: 		The document was not treated.

R4-2503537	On less than 5MHz for NTN RRM

Type: discussion		For: Discussion
					38.133 v	  CR-  rev  Cat:  (Rel-19)

					Source: Apple

Decision: 		The document was not treated.

R4-2503662	Discussion on RRM core requirements of Rel-19 NR-NTN in less than 5MHz

Type: discussion		For: Discussion
					Source: Samsung

Decision: 		The document was not treated.

R4-2503664	Draft CR on NR SAN FR1 conditional handover for NTN less than 5MHz

Type: draftCR		For: Endorsement
					38.133 v19.0.0	  CR-  rev  Cat: B (Rel-19)

					Source: Samsung

Decision: 		The document was not treated.

R4-2503795	Discussion on RRM requirements for less than 5M in NTN

Type: other		For: Approval
					Source: ZTECorporation,Sanechips

Decision: 		The document was not treated.

R4-2503896	Discussion on RRM requirements for less than 5MHz for NTN

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon

Decision: 		The document was not treated.

R4-2503897	draftCR on requirments for less than 5MHz for NTN

Type: draftCR		For: Endorsement
					38.133 v19.0.0	  CR-  rev  Cat: B (Rel-19)

					Source: Huawei, HiSilicon

Decision: 		The document was not treated.

R4-2504109	Discussion on RRM requirements of NTN with channel BW less than 5MHz

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

Discussion on RRM requirements of NTN with channel BW less than 5MHz

Decision: 		The document was not treated.

R4-2504445	SSB index detection requirements in Less than 5 MHz operation in NTN.

Type: discussion		For: Discussion
					Source: Nokia

Decision: 		The document was not treated.

##### 7.10.3.5	RRM performance requirements

R4-2503324	Discussion on the performance part requirements for less than 5MHz NTN

Type: discussion		For: Discussion
					Source: Xiaomi

Decision: 		The document was not treated.

R4-2503424	Discussion on  less than 5MHz for NTN RRM performance requirements

Type: discussion		For: Discussion
					Source: CATT

Decision: 		The document was not treated.

R4-2503663	Discussion on RRM performance requirements of Rel-19 NR-NTN in less than 5MHz

Type: discussion		For: Discussion
					Source: Samsung

Decision: 		The document was not treated.

R4-2503797	Discussion on RRM performance requirements for less than 5MHz in NTN enhancement

Type: other		For: Approval
					Source: ZTECorporation,Sanechips

Decision: 		The document was not treated.

R4-2503898	Discussion on RRM test cases for less than 5MHz for NTN

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon

Decision: 		The document was not treated.

R4-2504446	RRM Performance Requirements for less than 5 MHz over NTN

Type: discussion		For: Discussion
					Source: Nokia

Decision: 		The document was not treated.

##### 7.10.3.6	Demodulation performance requirements

R4-2503245	Discussions on demodulation performance requirements for NR-NTN with bandwidth less than 5MHz

Type: discussion		For: Discussion
					Source: QUALCOMM Europe Inc. - Spain

Decision: 		The document was not treated.

R4-2503910	Discussion on Rel-19 NTN UE/SAN demodulation requirements for less than 5 MHz channel bandwidth

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

This contribution discusses a general view for UE and SAN demodulation requirements for less than 5 MHz channel bandwidth

Decision: 		The document was not treated.

R4-2504538	Discussion on NTN demodulation requirements for Less than 5MHz

Type: discussion		For: Discussion
					Source: Samsung

Decision: 		The document was not treated.

#### 7.10.4	NTN testing for NGSO

R4-2504277	Discussion on NGSO channel modelling

Type: discussion		For: Discussion
					Source: Nordic Semiconductor ASA

Decision: 		The document was not treated.

##### 7.10.4.1	Channel modeling

R4-2503159	Discussion on NTN NGSO Channel Model

Type: discussion		For: Discussion
					Source: Nokia

Decision: 		The document was not treated.

R4-2503247	Discussions on channel modeling for NTN testing for NGSO

Type: discussion		For: Discussion
					Source: QUALCOMM Europe Inc. - Spain

Decision: 		The document was not treated.

R4-2503710	Views on TE emulated channel model for satellite mobility

Type: other		For: Approval
					Source: Anritsu Corporation

Decision: 		The document was not treated.

R4-2504001	Discussion on NTN_testing_NGSO_channel_model

Type: other		For: Approval
					Source: ZTE Corporation, Sanechips

Decision: 		The document was not treated.

R4-2504174	Views on propagation delay correction

Type: other		For: Approval
					Source: ROHDE & SCHWARZ

Decision: 		The document was not treated.

R4-2504314	Discussion on NTN testing for NGSO channel model

Type: discussion		For: Discussion
					Source: Huawei,HiSilicon

Decision: 		The document was not treated.

R4-2504344	TE-emulated channel model for NTN UE tests

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

This contribution discusses TE-emulated Doppler shift and propagation delay models for NTN UE demodulation requirements.

Decision: 		The document was not treated.

R4-2504349	Discussion on NSGO channel model for NTN testing

Type: discussion		For: Discussion
					Source: MediaTek inc.

Decision: 		The document was not treated.

R4-2504536	Discussion on NTN testing NGSO channel model

Type: discussion		For: Discussion
					Source: Samsung

Decision: 		The document was not treated.

##### 7.10.4.2	Demodulation performance requirements

R4-2503160	Discussion on NTN NGSO Performance Requirements

Type: discussion		For: Discussion
					Source: Nokia

Decision: 		The document was not treated.

R4-2503246	Discussions on demodulation performance requirements for NTN testing for NGSO

Type: discussion		For: Discussion
					Source: QUALCOMM Europe Inc. - Spain

Decision: 		The document was not treated.

R4-2504315	Discussion on less than 5MHz for NR NTN

Type: discussion		For: Discussion
					Source: Huawei,HiSilicon

Decision: 		The document was not treated.

R4-2504345	UE demodulation requirements with time varying Doppler shift and propagation delay model

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

This contribution discusses UE demodulation requirements with the time-varying Doppler shift and propagation delay models for NTN

Decision: 		The document was not treated.

R4-2504350	Discussions on demodulation performance requirements for NGSO channel model

Type: discussion		For: Discussion
					Source: MediaTek inc.

Decision: 		The document was not treated.

R4-2504537	Discussion on demodulation requirement for NTN NGSO testing

Type: discussion		For: Discussion
					Source: Samsung

Decision: 		The document was not treated.

#### 7.10.5	Moderator summary and conclusions

R4-2503615	Topic summary for [114bis][205] NR_IoT_NTN_req_test_enh

Type: other		For: Information
					Source: Moderator (Xiaomi)

Abstract:

Topic summary in RRM session

Decision: 		The document was not treated.

R4-2504635	Topic summary for [114bis][310] NR_IoT_NTN_less_than_5MHz_UERF

Type: other		For: Information
					Source: Moderator (Xiaomi)

Abstract:

[114bis] BDaT Session AI 7.10.3.1, 7.10.3.2

Decision: 		The document was not treated.

R4-2504636	Topic summary for [114bis][311] NR_IoT_NTN_less_than_5MHz_BSRF

Type: other		For: Information
					Source: Moderator (Nokia)

Abstract:

[114bis] BDaT Session AI 7.10.3.3

Decision: 		The document was not treated.

R4-2504648	Topic summary for [114bis][323] NTN_testing_NGSO_channel_model_and_demod

Type: other		For: Information
					Source: Moderator (Samsung)

Abstract:

[114bis] BDaT Session AI 7.10.4.1, 7.10.4.2

Decision: 		The document was not treated.

R4-2504649	Topic summary for [114bis][324] NR_IoT_NTN_less_than_5MHz_demod

Type: other		For: Information
					Source: Moderator (Ericsson)

Abstract:

[114bis] BDaT Session AI 7.10.3.6

Decision: 		The document was not treated.

R4-2504681	Topic summary for [114bis][123] NR_IoT_NTN_HPUE_part1

Type: other		For: Information
					Source: Moderator(Samsung)

Abstract:

Summary for AI 7.10, 7.10.1, 7.10.2, 7.10.2.1, 7.10.2.2

Decision: 		The document was not treated.

R4-2504682	Topic summary for [114bis][124] NR_IoT_NTN_HPUE_part2

Type: other		For: Information
					Source: Moderator(Vivo)

Abstract:

Summary for AI 7.10.2.3

Decision: 		The document was not treated.

### 7.11	Introduction of Ku Band for NR NTN

#### 7.11.1	General aspects

R4-2503274	Draft CR for Operating bands including priority 2 cases

Type: other		For: Endorsement
					38.101-5 v	  CR-  rev  Cat:  (Rel-19)

					Source: Eutelsat Group

Decision: 		The document was not treated.

R4-2503940	NTN Ku-band - General aspects - bands

Type: other		For: Approval
					Source: Ericsson

Abstract:

This contribution further analyze the NTN Ku-bands applicability and initiate discussion on the 2nd priority frequency range listed in the WI

Decision: 		The document was not treated.

#### 7.11.2	Coexistence study based on ITU regulations

R4-2503320	Discussion the assumptions and parameters for NR NTN Ku Band coexistence study

Type: discussion		For: Approval
					Source: xiaomi

Decision: 		The document was not treated.

R4-2503454	Further discussion on coexistence evaluations for Ku-band for NR NTN

Type: other		For: Approval
					Source: CATT

Decision: 		The document was not treated.

R4-2504263	On co-existence simulation assumptions for NR NTN Ku band

Type: other		For: Approval
					Source: Ericsson

Abstract:

This contribution captures the simulation assumption agreements so far and triggers the discussion points for some missing assumptions.

Decision: 		The document was not treated.

R4-2504264	On co-existence simulation results for NR NTN Ku band

Type: other		For: Discussion
					Source: Ericsson

Abstract:

This contribution provides co-existence simulation results based on the agreements so far.

Decision: 		The document was not treated.

R4-2504338	On Ku-band coexsitence studies

Type: other		For: Approval
					Source: Samsung

Decision: 		The document was not treated.

R4-2504477	Discussion on Ku-band co-existence study

Type: discussion		For: Discussion
					Source: SKY Perfect JSAT Corporation

Decision: 		The document was not treated.

R4-2504559	Discussion on coexistence simulation parameters for GEO and LEO VSAT

Type: discussion		For: Decision
					Source: SHARP Corporation

Decision: 		The document was not treated.

R4-2504658	Considerations for reducing Ku-band coexistence configurations

Type: discussion		For: Discussion
					Source: THALES

Abstract:

MCC: I reserved this tdoc on behalf of Thales having issues.

Decision: 		The document was not treated.

#### 7.11.3	System parameters

R4-2503231	Ku Band: Further System Parameters incorporating 15 kHz Sub-carrier Spacing and lower Uplink A Band (Priority 2 Band Combination)

Type: discussion		For: Decision
					Source: Intelsat

Decision: 		The document was not treated.

R4-2503334	System parameters for Ku band(s)

Type: discussion		For: Discussion
					Source: Xiaomi

Decision: 		The document was not treated.

R4-2503455	Further discussion on system parameters for Ku-band for NR NTN

Type: other		For: Approval
					Source: CATT

Decision: 		The document was not treated.

R4-2503740	Discussion on system parameters for NTN Ku band

Type: other		For: Approval
					Source: ZTE Corporation, Sanechips

Decision: 		The document was not treated.

R4-2503941	NTN Ku-band - System parameters

Type: other		For: Approval
					Source: Ericsson

Abstract:

This contribution proposes the system parameters for the NTN Ku-bands following last WI updates

Decision: 		The document was not treated.

R4-2504183	On Ku-band general requirements

Type: discussion		For: Approval
					38.101-5 v	  CR-  rev  Cat:  (Rel-19)

					Source: LG Electronics Finland

Abstract:

On general requirements for Ku-band NTN VSAT

Decision: 		The document was not treated.

R4-2504278	Discussion on the asymmetric channel bandwidths support for NR NTN Ku band

Type: discussion		For: Approval
					Source: CHTTL, SKY Perfect JSAT Corporation, Intelsat

Decision: 		The document was not treated.

R4-2504280	Further discussion on the remaining system parameters for NR NTN Ku band with the inclusion of 15 kHz SCS

Type: discussion		For: Discussion
					Source: CHTTL

Decision: 		The document was not treated.

R4-2504427	Draft CR for channel arrangement

Type: other		For: Agreement
					38.101-5 v	  CR-  rev  Cat:  (Rel-19)

					Source: Eutelsat Group

Decision: 		The document was not treated.

R4-2504473	NR NTN system parameters for Ku band

Type: other		For: Approval
					Source: Nokia

Decision: 		The document was not treated.

#### 7.11.4	UE RF requirements

R4-2503335	Requirements for VSAT on Ku band(s)

Type: discussion		For: Discussion
					Source: Xiaomi

Decision: 		The document was not treated.

R4-2503698	Discussion on VSAT requirements for NR NTN Ku band

Type: other		For: Approval
					Source: Huawei, HiSilicon

Decision: 		The document was not treated.

R4-2503741	Discussion on UE RF requirements for NTN Ku band

Type: other		For: Approval
					Source: ZTE Corporation, Sanechips

Decision: 		The document was not treated.

R4-2503942	NTN Ku-band - NTN VSAT RF requirements

Type: other		For: Approval
					Source: Ericsson

Abstract:

This contribution discusses the impacts on NTN VSAT RF requirements when introducing the Ku-band(s) following last WI updates

Decision: 		The document was not treated.

R4-2503944	Draft CR to 38101-5 - Needed updates to support NTN VSAT in FR1-NTN

Type: draftCR		For: Endorsement
					38.101-5 v19.0.0	  CR-  rev  Cat: B (Rel-19)

					Source: Ericsson

Abstract:

This draft CR proposes the needed updates to TS 38.101-5 to support NTN VSAT in FR1-NTN

Decision: 		The document was not treated.

R4-2504212	On Ku-band NTN VSAT requirements

Type: discussion		For: Approval
					38.101-5 v	  CR-  rev  Cat:  (Rel-19)

					Source: LG Electronics Finland

Abstract:

Discussiion on requirements for Ku-band NTN VSAT.

Decision: 		The document was not treated.

R4-2504279	Discussion on NTN Ku band UE RF requirements

Type: discussion		For: Approval
					Source: MediaTek (Wuhan) Inc.

Decision: 		The document was not treated.

R4-2504281	Discussion on the minimum peak EIRP, EIS and the maximum input level requirement for the Rel.19 NTN Ku band VSAT

Type: discussion		For: Approval
					Source: CHTTL

Decision: 		The document was not treated.

#### 7.11.5	SAN RF core requirements

R4-2503456	Further discussion on SAN RF core requirements for Ku-band for NR NTN

Type: other		For: Approval
					Source: CATT

Decision: 		The document was not treated.

R4-2503742	Discussion on SAN RF requirements for NTN Ku band

Type: other		For: Approval
					Source: ZTE Corporation, Sanechips

Decision: 		The document was not treated.

R4-2503743	DraftCR to TS38.108 Add 10.4 and 10.5 requirements for NTN Ku bands

Type: draftCR		For: Endorsement
					38.108 v19.0.0	  CR-  rev  Cat: B (Rel-19)

					Source: ZTE Corporation, Sanechips

Decision: 		The document was not treated.

R4-2503943	NTN Ku-band - SAN requirements

Type: other		For: Approval
					Source: Ericsson

Abstract:

This contribution discusses the impacts on SAN RF requirements when introducing the Ku-band(s) following last WI updates

Decision: 		The document was not treated.

R4-2503945	Draft CR to 38108 - Needed updates to add a new SAN type in FR1-NTN

Type: draftCR		For: Endorsement
					38.108 v19.0.0	  CR-  rev  Cat: B (Rel-19)

					Source: Ericsson

Abstract:

This draft CR proposes the needed updates to TS 38.108 to introduce a new SAN type in FR1-NTN

Decision: 		The document was not treated.

R4-2504293	draft running CR to TS 38.108 the SAN requirement of the NR NTN Ku band

Type: draftCR		For: Endorsement
					38.108 v19.0.0	  CR-  rev  Cat: B (Rel-19)

					Source: CHTTL

Decision: 		The document was not treated.

R4-2504474	Discussion on NR NTN SAN requirement for Ku band

Type: other		For: Approval
					Source: Nokia

Decision: 		The document was not treated.

#### 7.11.6	RRM core requirements

R4-2503129	Discussion on RRM requirements for Ku bands for NR NTN

Type: discussion		For: Discussion
					Source: MediaTek inc.

Decision: 		The document was not treated.

R4-2503139	Draft CR to 38.133 to modify uplink timing requirements to enable mobile VSAT served by NGSO

Type: draftCR		For: Endorsement
					38.133 v19.0.0	  CR-  rev  Cat: C (Rel-19)

					Source: Eutelsat Group

Decision: 		The document was not treated.

R4-2503508	Ku Band RRM updates required to support FR1-NTN and FR2-NTN numerology

Type: discussion		For: Agreement
					Source: Eutelsat Group

Decision: 		The document was not treated.

R4-2503706	Discussion on RRM requirements for Ku-band NTN

Type: discussion		For: Discussion
					Source: LG Electronics Inc.

Decision: 		The document was not treated.

R4-2503794	Discussion on Ku bands for NR NTN

Type: other		For: Approval
					Source: ZTECorporation,Sanechips

Decision: 		The document was not treated.

R4-2503906	Discussion on RRM requirements for NTN in Ku band

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon

Decision: 		The document was not treated.

R4-2504449	Discussing on the adoption of VSAT Requirements for FR1-NTN

Type: discussion		For: Discussion
					Source: Nokia

Decision: 		The document was not treated.

R4-2504532	RRM requirements for VSAT UE in Ku band

Type: discussion		For: Discussion
					Source: Qualcomm Incorporated

Decision: 		The document was not treated.

#### 7.11.7	Moderator summary and conclusions

R4-2503616	Topic summary for [114bis][206] NR_NTN_Ku_bands

Type: other		For: Information
					Source: Moderator (Apple)

Abstract:

Topic summary in RRM session

Decision: 		The document was not treated.

R4-2504637	Topic summary for [114bis][312] NR_NTN_Ku_Band_General

Type: other		For: Information
					Source: Moderator (Eutelsat)

Abstract:

[114bis] BDaT Session AI 7.11.1,  7.11.3

Decision: 		The document was not treated.

R4-2504638	Topic summary for [114bis][313] NR_NTN_Ku_Band_Coexistence

Type: other		For: Information
					Source: Moderator (Intelsat)

Abstract:

[114bis] BDaT Session AI 7.11.2

Decision: 		The document was not treated.

R4-2504639	Topic summary for [114bis][314] NR_NTN_Ku_Band_UE_SAN_RF

Type: other		For: Information
					Source: Moderator (CHTTL)

Abstract:

[114bis] BDaT Session AI 7.11.4, 7.11.5

Decision: 		The document was not treated.

### 7.12	Enhancements for Air-to-ground network for NR

#### 7.12.1	UE RF requirements

R4-2503586	On remaining issue for ATG RF requirement

Type: discussion		For: Discussion
					Source: Apple

Decision: 		The document was not treated.

##### 7.12.1.1	Intra-band contiguous and inter-band CA

R4-2503396	Discussion on UE RF requirements for ATG with inter-band CA

Type: discussion		For: Discussion
					Source: CMCC

Decision: 		The document was not treated.

R4-2503694	Discussion on Rel-19 ATG UE RF with DL inter-band CA_n3-n39

Type: other		For: Approval
					Source: Huawei, HiSilicon

Decision: 		The document was not treated.

R4-2503723	Discussion for ATG UE RF requirements

Type: discussion		For: Discussion
					Source: LG Electronics France

Decision: 		The document was not treated.

R4-2504038	Discussion on RF requirements of ATG UE inter-band CA

Type: other		For: Approval
					Source: Ericsson

Abstract:

This paper will delve into the remaining RF requirement for ATG UE inter-band CA.

Decision: 		The document was not treated.

R4-2504057	Discussion on Rel-19 ATG UE RF requirements

Type: other		For: Approval
					Source: Qualcomm Incorporated

Decision: 		The document was not treated.

R4-2504356	Discussion on RF requirement for Inter-band CA for ATG UE in Rel-19

Type: other		For: Approval
					Source: ZTE Corporation, Sanechips

Decision: 		The document was not treated.

##### 7.12.1.2	UL MIMO

R4-2503395	Discussion on UE RF requirements for ATG with UL-MIMO

Type: discussion		For: Discussion
					Source: CMCC

Decision: 		The document was not treated.

R4-2503695	Discussion on Rel-19 ATG UE RF with UL MIMO

Type: other		For: Approval
					Source: Huawei, HiSilicon

Decision: 		The document was not treated.

R4-2504039	Discussion on RF requirements of ATG UE UL MIMO

Type: other		For: Approval
					Source: Ericsson

Abstract:

Discussion on RF requirement of ATG UE UL MIMO

Decision: 		The document was not treated.

R4-2504357	Discussion on RF requirement for UL-MIMO for ATG UE in Rel-19

Type: other		For: Approval
					Source: ZTE Corporation, Sanechips

Decision: 		The document was not treated.

##### 7.12.1.3	DL MIMO

##### 7.12.1.4	Others

#### 7.12.2	BS RF requirements

R4-2503218	Draft CR for TS 38.141-1 to add ATG enhancement BS RF requirements

Type: draftCR		For: Endorsement
					38.141-1 v19.0.0	  CR-  rev  Cat:  (Rel-19)

					Source: Huawei, HiSilicon

Decision: 		The document was not treated.

R4-2503219	Draft CR for TS 38.141-2 to add ATG enhancement BS RF requirements

Type: draftCR		For: Endorsement
					38.141-2 v19.0.0	  CR-  rev  Cat:  (Rel-19)

					Source: Huawei, HiSilicon

Decision: 		The document was not treated.

R4-2503394	Discussion on BS RF requirements for ATG

Type: discussion		For: Discussion
					Source: CMCC

Decision: 		The document was not treated.

R4-2503958	Clarification on the multi-band requirements for ATG BS CA

Type: other		For: Approval
					Source: ZTE Corporation, Sanechips

Decision: 		The document was not treated.

R4-2503959	draft CR to TS 38.104: the introduction of Rel-19 ATG BS supporting CA

Type: draftCR		For: Endorsement
					38.104 v19.0.0	  CR-  rev  Cat: B (Rel-19)

					Source: ZTE Corporation, Sanechips

Decision: 		The document was not treated.

#### 7.12.3	BS RF conformance requirements

R4-2503397	Draft CR for 38.141-2 to Introduce of R19 ATG enhancement BS RF requirements

Type: draftCR		For: Endorsement
					38.141-2 v19.0.0	  CR-  rev  Cat:  (Rel-19)

					Source: CMCC

Decision: 		The document was not treated.

R4-2503459	Draft CR for 38.141-1, Introduction of R19 ATG enhancement BS RF requirements

Type: draftCR		For: Endorsement
					38.141-1 v19.0.0	  CR-  rev  Cat: B (Rel-19)

					Source: CATT

Decision: 		The document was not treated.

R4-2503460	Draft CR for 38.141-2, Introduction of R19 ATG enhancement BS RF requirements

Type: draftCR		For: Endorsement
					38.141-2 v19.0.0	  CR-  rev  Cat: B (Rel-19)

					Source: CATT

Decision: 		The document was not treated.

R4-2503960	Discussion on the ATG BS CA conformance test

Type: other		For: Approval
					Source: ZTE Corporation, Sanechips

Decision: 		The document was not treated.

R4-2504040	Draft CR for 38.141-1  ATG BS test requirements

Type: draftCR		For: Endorsement
					38.141-1 v19.0.0	  CR-  rev  Cat:  (Rel-19)

					Source: Ericsson

Abstract:

Remove “There is no multi-band operation requirement for ATG BS.” “There is no TAE requirements for ATG BS”, add “Unless stated otherwise, the requirements for BS type 1-C and BS type 1-H are applied for ATG BS type 1-C and 1-H, respectively”

Decision: 		The document was not treated.

R4-2504041	Draft CR for 38.141-2  ATG BS test requirements

Type: draftCR		For: Endorsement
					38.141-2 v19.0.0	  CR-  rev  Cat:  (Rel-19)

					Source: Ericsson

Abstract:

Remove “There is no multi-band operation requirement for ATG BS.” “The TAE requirement does not apply to ATG BS.” Add “Unless stated otherwise, the requirements for BS type 1-H and BS type 1-O are applied for ATG BS type 1-H and 1-O, respectively”

Decision: 		The document was not treated.

R4-2504042	Discussion on ATG BS DL test requirement

Type: other		For: Approval
					Source: Ericsson

Abstract:

The following session will discuss the test requirement for ATG BS.

Decision: 		The document was not treated.

#### 7.12.4	RRM core requirements for CA

R4-2503372	draftCR Introduce requirement applicability and cell re-selection requirement for ATG CA

Type: draftCR		For: Endorsement
					38.133 v19.0.0	  CR-  rev  Cat:  (Rel-19)

					Source: CMCC

Decision: 		The document was not treated.

R4-2503373	Discussion on RRM requirements for R19 ATG

Type: discussion		For: Discussion
					Source: CMCC

Decision: 		The document was not treated.

R4-2503425	Draft CR to TS 38.133 on general measurement requirement for ATG

Type: draftCR		For: Endorsement
					38.133 v19.0.0	  CR-  rev  Cat: B (Rel-19)

					Source: CATT

Decision: 		The document was not treated.

R4-2503426	Draft CR to TS 38.133 on CSI-RS based L3 measurements for ATG

Type: draftCR		For: Endorsement
					38.133 v19.0.0	  CR-  rev  Cat: B (Rel-19)

					Source: CATT

Decision: 		The document was not treated.

R4-2503427	Discussion on enhancements for Rel-19 ATG RRM core requirements

Type: discussion		For: Discussion
					Source: CATT

Decision: 		The document was not treated.

R4-2503587	draft CR on ATG UE RRM requirement

Type: draftCR		For: Endorsement
					38.133 v19.0.0	  CR-  rev  Cat: B (Rel-19)

					Source: Apple

Decision: 		The document was not treated.

R4-2503703	Discussion on RRM requirements for ATG CA operation

Type: discussion		For: Discussion
					Source: LG Electronics Inc.

Decision: 		The document was not treated.

R4-2503704	Draft CR on L1-RSRP measurements for Reporting for ATG CA in Clause 9.5D

Type: draftCR		For: Endorsement
					38.133 v19.0.0	  CR-  rev  Cat: B (Rel-19)

					Source: LG Electronics Inc.

Decision: 		The document was not treated.

R4-2503878	Discussions on RRM core requirements for Rel-19 ATG

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon

Decision: 		The document was not treated.

R4-2503880	Draft CR on core requirements for Rel-19 ATG

Type: discussion		For: Discussion
					38.133 v	  CR-  rev  Cat:  (Rel-19)

					Source: Huawei, HiSilicon

Decision: 		The document was withdrawn.

R4-2504014	Draft CR on core requirements for Rel-19 ATG

Type: draftCR		For: Endorsement
					38.133 v19.0.0	  CR-  rev  Cat: B (Rel-19)

					Source: Huawei, HiSilicon

Decision: 		The document was not treated.

R4-2504018	draftCR for MRTD and interruption requirements for ATG UE in R19

Type: draftCR		For: Endorsement
					38.133 v19.0.0	  CR-  rev  Cat: B (Rel-19)

					Source: Intel Corporation

Decision: 		The document was not treated.

R4-2504244	Discussion on RRM aspects of R19 ATG

Type: discussion		For: Approval
					Source: ZTE Corporation, Sanechips

Decision: 		The document was not treated.

R4-2504254	Draft CR on intra-frequency measurement for R19 ATG

Type: draftCR		For: Endorsement
					38.133 v19.0.0	  CR-  rev  Cat: B (Rel-19)

					Source: ZTE Corporation, Sanechips

Decision: 		The document was not treated.

R4-2504255	Draft CR on inter-frequency measurement for R19 ATG

Type: draftCR		For: Endorsement
					38.133 v19.0.0	  CR-  rev  Cat: B (Rel-19)

					Source: ZTE Corporation, Sanechips

Decision: 		The document was not treated.

R4-2504461	Discussion on Remaining Issues Related to Rel-19 ATG CA in RRM Core

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

This contribution discusses the remaining open issues related to ATG CA enhancement.

Decision: 		The document was not treated.

R4-2504462	Draft CR for ATG Rel-19 Pre-configured measurement gap activation/deactivation delay

Type: draftCR		For: Endorsement
					38.133 v19.0.0	  CR-  rev  Cat: B (Rel-19)

					Source: Ericsson

Abstract:

This draftCR introduces ATG-specific requirements of Pre-configured measurement gap activation/deactivation delay upon SCell activation/deactivation

Decision: 		The document was not treated.

R4-2504463	Draft CR for L1-SINR measurements for Reporting with ATG

Type: draftCR		For: Endorsement
					38.133 v19.0.0	  CR-  rev  Cat: B (Rel-19)

					Source: Ericsson

Abstract:

This draftCr introduces explicit support for ATG UEs with potentially complex antenna and measurement configurations

Decision: 		The document was not treated.

#### 7.12.5	RRM performance requirements

R4-2503374	Discussion on measurement accuracy and test cases for R19 ATG

Type: discussion		For: Discussion
					Source: CMCC

Decision: 		The document was not treated.

R4-2503428	Discussion on enhancements for Rel-19 ATG RRM performance requirements

Type: discussion		For: Discussion
					Source: CATT

Decision: 		The document was not treated.

R4-2503879	Discussions on RRM performance requirements for Rel-19 ATG

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon

Decision: 		The document was not treated.

R4-2504245	Discussion on RRM performance of R19 ATG

Type: discussion		For: Approval
					Source: ZTE Corporation, Sanechips

Decision: 		The document was not treated.

R4-2504464	Discussions on RRM Performance Enhancement Requirements for Rel-19 ATG CA

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

This contribution discuss and provide our initial view on the performance requirements based on the core requirements agreements and open issues.

Decision: 		The document was not treated.

#### 7.12.6	Demodulation performance requirements

R4-2503375	Discussion on demodulation and CSI requirements for Rel-19 ATG

Type: discussion		For: Discussion
					Source: CMCC

Decision: 		The document was not treated.

R4-2504000	Discussion on Rel-19 ATG demodulation requirements

Type: other		For: Approval
					Source: ZTE Corporation, Sanechips

Decision: 		The document was not treated.

R4-2504384	Discussion on NR ATG enhancement for demodulation

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

General view on Rel-19 ATG demodulation requirements

Decision: 		The document was not treated.

R4-2504443	On Enhancements for Air-to-ground network for NR

Type: discussion		For: Discussion
					Source: Nokia

Decision: 		The document was not treated.

R4-2504540	View on BS demodulation requirement for ATG

Type: discussion		For: Discussion
					Source: Samsung

Decision: 		The document was not treated.

#### 7.12.7	Moderator summary and conclusions

R4-2503617	Topic summary for [114bis][207] NR_ATG_enh

Type: other		For: Information
					Source: Moderator (CMCC)

Abstract:

Topic summary in RRM session

Decision: 		The document was not treated.

R4-2504630	Topic summary for [114bis][305] NR_ATG_enh

Type: other		For: Information
					Source: Moderator (ZTE)

Abstract:

[114bis] BDaT Session AI 7.12.2, 7.12.3

Decision: 		The document was not treated.

R4-2504683	Topic summary for [114bis][125] NR_ATG_enh

Type: other		For: Information
					Source: Moderator(CMCC)

Abstract:

Summary for AI 7.12.1

Decision: 		The document was not treated.

R4-2504691	Topic summary for [114bis][332] NR_ATG_enh_demod

Type: other		For: Information
					Source: Moderator (CMCC)

Abstract:

[114bis] BDaT Session AI 7.12.6

Decision: 		The document was not treated.

### 7.13	NR base station (BS) RF requirement evolution for FR1/FR2 and testing

#### 7.13.1	General aspects

R4-2503726	Rel-19 BS RF evolution overview

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

In this contribution we provide a status overview of work packages included in BS RF evolution WI. For each work package we have collected the relevant findings in respective area to prepare for creating CRs for BS RF specifications and preparing work in

Decision: 		The document was not treated.

#### 7.13.2	Expected EIRP mask for upper 6GHz

R4-2503577	[37.941] Incorrect MU data for n104 band

Type: CR		For: Endorsement
					37.941 v17.5.0	  CR-0058  rev  Cat: F (Rel-17)

					Source: ROHDE & SCHWARZ

Abstract:

The 37.941 document MU values are missing for n104. Annex C2 does not list the MU values for n104 band. The  FR1 TX MU calculation table spreadsheet is also missing the same. 38.141-2 data also needs to be updated on this topic for which a separate CR has

Decision: 		The document was not treated.

R4-2503588	[37.941] Incorrect MU data for n104 band

Type: CR		For: Endorsement
					37.941 v18.2.0	  CR-0059  rev  Cat: A (Rel-18)

					Source: ROHDE & SCHWARZ

Abstract:

The 37.941 document MU values are missing for n104. Annex C2 does not list the MU values for n104 band. The  FR1 TX MU calculation table spreadsheet is also missing the same. 38.141-2 data also needs to be updated on this topic for which a separate CR has

Decision: 		The document was not treated.

R4-2503589	[38.141-2] Incorrect MU data for n104 band

Type: CR		For: Endorsement
					38.141-2 v17.17.0	  CR-0632  rev  Cat: F (Rel-17)

					Source: ROHDE & SCHWARZ

Abstract:

Section 4.1.2.2 has a misleading frequency limit, concerning n104 band, when declaring Max Test System Uncertainty for clause 6.2(Radiated Tx Power). MCC: This is a CR for Rel-17, yet the agenda 7.13.2 is under Rel-19 on-going non-spectrum related work/st

Decision: 		The document was not treated.

R4-2503593	[38.141-2] Incorrect MU data for n104 band

Type: CR		For: Endorsement
					38.141-2 v18.9.0	  CR-0633  rev  Cat: A (Rel-18)

					Source: ROHDE & SCHWARZ

Abstract:

Section 4.1.2.2 has a misleading frequency limit, concerning n104 band, when declaring Max Test System Uncertainty for clause 6.2(Radiated Tx Power). MCC: This is a CR for Rel-18, yet the agenda 7.13.2 is under Rel-19 on-going non-spectrum related work/st

Decision: 		The document was not treated.

R4-2503601	[Not inc C1-1] PWS specific MU data to be included for n104 band

Type: CR		For: Endorsement
					38.908 v19.0.0	  CR-0001  rev  Cat: F (Rel-19)

					Source: ROHDE & SCHWARZ

Abstract:

R4-2500548 was merged into other similar CRs in the RAN4 114 meeting(Athens). However on a following phone meeting the proposed MU for the Spec An(i.e. C1-1) was not approved. Hence listing all the other MU values (except C1-1) in this CR to keep track of

Decision: 		The document was withdrawn.

R4-2503602	[C1-1 focus] Spec An MU to be updated for n104 band

Type: CR		For: Endorsement
					38.908 v19.0.0	  CR-0002  rev  Cat: F (Rel-19)

					Source: ROHDE & SCHWARZ

Abstract:

R4-2500548 was merged into other similar CRs in the RAN4 114 meeting(Athens). However on a following phone meeting the proposed MU for the Spec An(i.e. C1-1) was not approved. Hence listed all the other MU values (except C1-1) in another CR(R4-2503601.zip

Decision: 		The document was not treated.

R4-2504433	Draft CR to TS 38.106 for introduction of expected EIRP mask core requirement for NCR

Type: draftCR		For: Endorsement
					38.106 v19.0.0	  CR-  rev  Cat: B (Rel-19)

					Source: Nokia, ZTE Corporation, Ericsson, Huawei, Samsung, Qualcomm, CATT

Decision: 		The document was not treated.

R4-2504436	Draft CR to TR 38.908 with updates to EEIRP sampling Error simulation results

Type: draftCR		For: Endorsement
					38.908 v19.0.0	  CR-  rev  Cat: F (Rel-19)

					Source: Nokia

Decision: 		The document was not treated.

R4-2504437	Draft CR to TR 38.908 with updates to test procedure

Type: draftCR		For: Endorsement
					38.908 v19.0.0	  CR-  rev  Cat: F (Rel-19)

					Source: Nokia

Decision: 		The document was not treated.

R4-2504513	Draft CR to TR 38-908: Corrections and Improvements related to EEIRP

Type: draftCR		For: Endorsement
					38.908 v19.0.0	  CR-  rev  Cat:  (Rel-19)

					Source: Ericsson

Abstract:

Corrections on the newly created TR

Decision: 		The document was not treated.

#### 7.13.3	OTA test enhancement

R4-2504013	Simulation of BS-to-BS out-of-band isolation for edge-to-edge co-location isolation between 3.5 and 7 GHz

Type: other		For: Approval
					Source: Ericsson

Abstract:

Discussion on OTA test enhancement

Decision: 		The document was not treated.

R4-2504191	On OTA test enhancements

Type: other		For: Approval
					Source: Nokia

Decision: 		The document was not treated.

R4-2504358	Further discussion on OTA test enhancement

Type: other		For: Approval
					Source: ZTE Corporation, Sanechips

Decision: 		The document was not treated.

R4-2504594	Further discussion on CLTA related matters

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon

Abstract:

In this contribution, we provide further inputs on the CLTA related matters, including BS-BS isolation, intra-system TX IMD, beam steering, etc.

Decision: 		The document was not treated.

R4-2504596	Draft CR to TS 37.941: CLTA agreements

Type: draftCR		For: Endorsement
					37.941 v18.2.0	  CR-  rev  Cat: B (Rel-19)

					Source: Huawei, HiSilicon

Abstract:

Based on the agreement captured in the WF in R4-2502279, this Draft CR provides placeholder to capture Rel-19 CLTA and co-location measurements related agreements.

Decision: 		The document was not treated.

#### 7.13.4	Transmitter co-existence spurious emission requirements

R4-2504192	On TX co-existence spurious emissions requirements

Type: other		For: Approval
					Source: Nokia

Decision: 		The document was not treated.

R4-2504267	On Co-existence parameters for BS Tx spurious emission co-existence requirement limits

Type: other		For: Approval
					Source: Ericsson

Abstract:

In this contribution we present some discussion points in relation to transmitter co-existence spurious emission requirements.

Decision: 		The document was not treated.

R4-2504298	Transmitter co-existence spurious emission requirements

Type: other		For: Approval
					Source: Huawei, HiSilicon

Decision: 		The document was not treated.

R4-2504343	Views on BS FR1 transmitter co-existence spurious emission requirements

Type: discussion		For: Approval
					Source: Qualcomm Germany

Decision: 		The document was not treated.

R4-2504359	Discussion on FR1 BS transmitter co-existence spurious emission requirements

Type: other		For: Discussion
					Source: ZTE Corporation, Sanechips

Decision: 		The document was not treated.

#### 7.13.5	BS conformance testing

##### 7.13.5.1	U6GHz BS EEIRP mask

R4-2504295	Further discussion on Expected EIRP test

Type: other		For: Approval
					Source: Huawei, HiSilicon

Decision: 		The document was not treated.

R4-2504296	Draft CR to TS 38.908 EEIRP measurement uncertainty

Type: draftCR		For: Endorsement
					38.908 v19.0.0	  CR-  rev  Cat: F (Rel-19)

					Source: Huawei, HiSilicon

Decision: 		The document was not treated.

R4-2504297	Draft CR to TS 38.141-2 EEIRP measurement uncertainty and coordinate alignment

Type: draftCR		For: Endorsement
					38.141-2 v19.0.0	  CR-  rev  Cat: F (Rel-19)

					Source: Huawei, HiSilicon

Decision: 		The document was not treated.

R4-2504360	Discussion on conformance testing for U6GHz BS EEIRP mask

Type: other		For: Approval
					Source: ZTE Corporation, Sanechips

Decision: 		The document was not treated.

R4-2504361	Draft CR to TS 38.141-2: MU value and angular transformation for U6GHz EEIRP mask

Type: draftCR		For: Endorsement
					38.141-2 v19.0.0	  CR-  rev  Cat: F (Rel-19)

					Source: ZTE Corporation, Sanechips

Decision: 		The document was not treated.

R4-2504362	Draft CR to TR 38.908 EEIRP measurement uncertainty and angular transformation

Type: draftCR		For: Endorsement
					38.908 v19.0.0	  CR-  rev  Cat: F (Rel-19)

					Source: ZTE Corporation, Sanechips

Decision: 		The document was not treated.

R4-2504363	Big Draft CR to TS 38.908: MU and others

Type: draftCR		For: Endorsement
					38.908 v19.0.0	  CR-  rev  Cat: F (Rel-19)

					Source: ZTE Corporation, Sanechips

Abstract:

to capture all endorsed draft CRs for TR38.908 and for e-mail Endorsement after the meeting

Decision: 		The document was not treated.

R4-2504434	Draft CR to TS 38.115-2 for introduction of expected EIRP mask conformance test for NCR

Type: draftCR		For: Endorsement
					38.115-2 v18.4.0	  CR-  rev  Cat: B (Rel-19)

					Source: Nokia, ZTE Corporation, Ericsson, Huawei, Samsung, Qualcomm, CATT

Decision: 		The document was not treated.

R4-2504435	Draft CR to TS 38.141-2 with updates for EEIRP requirement conformance testing

Type: draftCR		For: Endorsement
					38.141-2 v19.0.0	  CR-  rev  Cat: F (Rel-19)

					Source: Nokia

Decision: 		The document was not treated.

R4-2504514	Draft CR to TS 38.141-2: Improvements related to Expected EIRP (EEIRP)

Type: draftCR		For: Endorsement
					38.141-2 v19.0.0	  CR-  rev  Cat:  (Rel-19)

					Source: Ericsson

Abstract:

Corrections related to introduction of EEIRP

Decision: 		The document was not treated.

##### 7.13.5.2	Identification and reduction of BS OTA test scope

R4-2504012	OTA test scope reduction

Type: other		For: Approval
					Source: Ericsson

Abstract:

Discussion on OTA test scope reduction on TX IM and out-of-band blocking

Decision: 		The document was not treated.

R4-2504299	OTA test case reduction

Type: other		For: Approval
					Source: Huawei, HiSilicon

Decision: 		The document was not treated.

R4-2504364	Discussion on the OTA testing scope reduction

Type: other		For: Approval
					Source: ZTE Corporation, Sanechips

Decision: 		The document was not treated.

R4-2504438	Further discussion BS OTA test scope reduction

Type: other		For: Approval
					Source: Nokia

Decision: 		The document was not treated.

##### 7.13.5.3	Simplification of BS TRP test methods

R4-2503167	Simplification of BS TRP test methods

Type: other		For: Approval
					Source: Nokia

Abstract:

This contribution provides proposals to progress the objective of the WI for TRP simplification in RAN4.

Decision: 		The document was not treated.

R4-2504365	Discussion on the applicability improvement of TRP measurement

Type: other		For: Approval
					Source: ZTE Corporation, Sanechips

Decision: 		The document was not treated.

R4-2504515	TS 38.141-2 Annex I: Further discussion on improvement of TRP measurement methods

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

Discussion on improving the normative Annex I by removing part of the information and clarifying the normative part.

Decision: 		The document was not treated.

R4-2504597	Information on European Commission feedback wrt. alternative test methods

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon

Abstract:

In this contribution, we provide European Commission’s feedback on alternative test methods, which was received during ETSI meeting with the European Commission, and reported via ETSI OCG RED.

Decision: 		The document was not treated.

#### 7.13.6	Moderator summary and conclusions

R4-2504627	Topic summary for [114bis][302] NR_BS_RF_Part1_E_EIRP

Type: other		For: Information
					Source: Moderator (ZTE)

Abstract:

[114bis] BDaT Session AI 7.13.2, 7.13.5.1

Decision: 		The document was not treated.

R4-2504628	Topic summary for [114bis][303] NR_BS_RF_Part2_CLTA_SE

Type: other		For: Information
					Source: Moderator (Huawei)

Abstract:

[114bis] BDaT Session AI 7.13.3, 7.13.4, 7.13.5 (CLTA part)

Decision: 		The document was not treated.

R4-2504629	Topic summary for [114bis][304] NR_BS_RF_Part3_OTA_TRP

Type: other		For: Information
					Source: Moderator (ZTE)

Abstract:

[114bis] BDaT Session AI 7.13.5.2, 7.13.5.3

Decision: 		The document was not treated.

### 7.14	TRP (Total Radiated Power), TRS (Total Radiated Sensitivity) and MIMO OTA (Over the Air) testing enhancement Phase 3

#### 7.14.1	General aspects

R4-2504568	DraftCR to add min measurement distance for 50cm QZ

Type: draftCR		For: Endorsement
					38.870 v18.4.0	  CR-  rev  Cat: F (Rel-18)

					Source: Keysight Technologies UK Ltd

Decision: 		The document was not treated.

#### 7.14.2	Core requirements

##### 7.14.2.1	Test methodology for FR1 non-RedCap headworn XR devices

R4-2503136	Further discussion on XR test method

Type: other		For: Approval
					Source: Ericsson-LG Co., LTD

Decision: 		The document was not treated.

R4-2503498	on XR OTA tests

Type: other		For: Approval
					Source: Huawei, HiSilicon

Abstract:

on XR OTA tests

Decision: 		The document was not treated.

R4-2503810	CR to TS38.161 for an alternate TRS and TRP test procedure for XR devices

Type: CR		For: Endorsement
					38.161 v18.3.0	  CR-0024  rev  Cat: B (Rel-19)

					Source: Apple

Abstract:

All CRs are only for endorsement at RAN4#114-bis and need to be resubmitted at RAN4#115 for agreement.

Decision: 		The document was not treated.

R4-2504440	Draft CR for split measurement grids method for TRP/TRS measurements

Type: draftCR		For: Endorsement
					38.870 v18.4.0	  CR-  rev  Cat: B (Rel-18)

					Source: Nokia, Keysight Technologies UK Ltd

Decision: 		The document was not treated.

R4-2504621	Discussion on OTA Test for XR Devices

Type: discussion		For: Discussion
					Source: Meta

Decision: 		The document was not treated.

##### 7.14.2.2	Test methodology and radiated performance metric for FR1 NTN devices

R4-2503332	Discussion on OTA test for FR1 NTN devices

Type: discussion		For: Discussion
					Source: Xiaomi

Decision: 		The document was not treated.

R4-2503496	on NTN OTA tests

Type: other		For: Approval
					Source: Huawei, HiSilicon

Abstract:

on NTN OTA tests

Decision: 		The document was not treated.

R4-2503551	On NTN OTA test methodology

Type: discussion		For: Discussion
					Source: Apple

Decision: 		The document was withdrawn.

R4-2503552	draftCR to TR38.870 on NTN DUT usage scenario applicability

Type: draftCR		For: Endorsement
					38.870 v19.0.0	  CR-  rev  Cat: B (Rel-19)

					Source: Apple

Decision: 		The document was withdrawn.

R4-2503650	On OTA performance metric for handheld NTN UE

Type: discussion		For: Discussion
					Source: Samsung

Decision: 		The document was not treated.

R4-2503651	Draft CR to TR 38.870 on NTN usage scenario and performance metric

Type: draftCR		For: Endorsement
					38.870 v18.4.0	  CR-  rev  Cat: B (Rel-18)

					Source: Samsung

Decision: 		The document was not treated.

R4-2503828	draft CR to 38.870 on NTN OTA performance metric and test method

Type: draftCR		For: Endorsement
					38.870 v19.0.0	  CR-  rev  Cat: B (Rel-19)

					Source: vivo

Decision: 		The document was not treated.

R4-2504058	Discussion on FR1 NTN devices OTA

Type: other		For: Approval
					Source: Qualcomm Incorporated

Decision: 		The document was not treated.

R4-2504182	Draft CR for split measurement grids method for TRP/TRS measurements

Type: CR		For: Endorsement
					38.870 v18.4.0	  CR-0020  rev  Cat: B (Rel-18)

					Source: Nokia, Keysight Technologies UK Ltd

Abstract:

All CRs are only for endorsement at RAN4#114-bis and need to be resubmitted at RAN4#115 for agreement.

Decision: 		The document was withdrawn.

R4-2504425	Draft CR for split measurement grids method for TRP/TRS measurements

Type: draftCR		For: Endorsement
					38.870 v18.4.0	  CR-  rev  Cat: B (Rel-18)

					Source: Nokia, Keysight Technologies UK Ltd

Decision: 		The document was not treated.

R4-2504578	On NTN OTA test methodology

Type: discussion		For: Discussion
					Source: Apple

Decision: 		The document was not treated.

R4-2504579	draftCR to TR38.870 on NTN DUT usage scenario applicability

Type: draftCR		For: Endorsement
					38.870 v19.0.0	  CR-  rev  Cat: B (Rel-19)

					Source: Apple

Decision: 		The document was not treated.

##### 7.14.2.3	FR1 dynamic MIMO OTA test methodology

R4-2503495	on MIMO OTA tests

Type: other		For: Approval
					Source: Huawei, HiSilicon

Abstract:

on MIMO OTA tests

Decision: 		The document was not treated.

R4-2503652	On performance metric for FR1 dynamic MIMO

Type: discussion		For: Discussion
					Source: Samsung

Decision: 		The document was not treated.

R4-2503809	On FR1 MIMO OTA Dynamic Channel Model

Type: discussion		For: Discussion
					Source: Apple

Decision: 		The document was not treated.

R4-2503990	On Rel-19 MIMO OTA

Type: other		For: Approval
					Source: CAICT

Decision: 		The document was not treated.

R4-2503991	TR 38.762 V0.3.0 on MIMO OTA dynamic test methodology for FR1 UEs

Type: draft TR		For: Agreement
					38.762 v0.3.0	  CR-  rev  Cat:  (Rel-19)

					Source: CAICT

Decision: 		The document was not treated.

R4-2504081	On dynamic MIMO OTA channel models reference target values

Type: other		For: Approval
					Source: CMCC

Decision: 		The document was not treated.

R4-2504566	TP for TR 38.762: Clarifications of Channel Models and Applicability

Type: other		For: Approval
					Source: Keysight Technologies UK Ltd, Spirent Communications, ETS-Lindgren, CAICT

Decision: 		The document was not treated.

#### 7.14.3	Performance requirements

##### 7.14.3.1	TRP TRS requirements

R4-2503137	Discussion on Rel-19 TRP TRS requirements

Type: other		For: Approval
					Source: Ericsson-LG Co., LTD

Decision: 		The document was not treated.

R4-2503497	on TRP TRS OTA tests

Type: other		For: Approval
					Source: Huawei, HiSilicon

Abstract:

on size 1 UE requirements

Decision: 		The document was not treated.

R4-2503653	On TRP TRS spec for narrow width UE

Type: discussion		For: Discussion
					Source: Samsung

Decision: 		The document was not treated.

R4-2503811	Views on Rel-19 TRP TRS Performance Requirements

Type: discussion		For: Discussion
					Source: Apple

Decision: 		The document was not treated.

R4-2503829	Updated working procedure for Rel-19 TRP TRS performance part

Type: other		For: Approval
					Source: vivo

Decision: 		The document was not treated.

R4-2504150	Discussion on TRP TRS requirements

Type: other		For: Approval
					Source: CAICT

Decision: 		The document was not treated.

R4-2504577	Proposals for Rel-19 TRP TRS requirements topic

Type: other		For: Approval
					Source: Telecom Italia S.p.A., Vodafone

Decision: 		The document was not treated.

##### 7.14.3.2	MIMO OTA requirements

#### 7.14.4	Moderator summary and conclusions

R4-2504653	Topic summary for [114bis][328] TRP_TRS_MIMO_OTA_Ph3

Type: other		For: Information
					Source: Moderator (vivo)

Abstract:

[114bis] BDaT Session AI 7.14.1, 7.14.2.1, 7.14.2.2, 7.14.2.3,  7.14.3.1, 7.14.3.2

Decision: 		The document was not treated.

### 7.15	Study on NR FR2 OTA (Over the Air) testing enhancement Phase 3

#### 7.15.1	General aspects

R4-2503819	TP to TR 38.771 on the test procedure of sTxMP

Type: pCR		For: Approval
					38.771 v0.0.2	  CR-  rev  Cat:  (Rel-19)

					Source: vivo

Decision: 		The document was not treated.

R4-2504061	TR 38.771v0.3.0

Type: other		For: Approval
					38.771 v	  CR-  rev  Cat:  (Rel-19)

					Source: Qualcomm Incorporated

Decision: 		The document was not treated.

#### 7.15.2	RF testing methodology for FR2 non-handheld UE that can transmit simultaneously with multi-panel

R4-2503654	On remaining issues for STxMP test method

Type: discussion		For: Discussion
					Source: Samsung

Decision: 		The document was not treated.

R4-2503818	Discussion on the FR2 OTA test method

Type: other		For: Approval
					Source: vivo

Decision: 		The document was not treated.

R4-2504059	Remaining issues for FR2 OTA test method on STxMP

Type: other		For: Approval
					Source: Qualcomm Incorporated

Decision: 		The document was not treated.

R4-2504060	TP to TR 38.771 on MU

Type: other		For: Approval
					38.771 v	  CR-  rev  Cat:  (Rel-19)

					Source: Qualcomm Incorporated

Decision: 		The document was not treated.

R4-2504567	TP for 38.771 on system related aspects

Type: other		For: Approval
					Source: Keysight Technologies UK Ltd

Decision: 		The document was not treated.

#### 7.15.3	Moderator summary and conclusions

R4-2504654	Topic summary for [114bis][329] NR_FR2_OTA

Type: other		For: Information
					Source: Moderator (Qualcomm)

Abstract:

[114bis] BDaT Session AI 7.15.1, 7.15.2

Decision: 		The document was not treated.

### 7.16	Study on spatial channel model for demodulation performance requirements

#### 7.16.1	General aspects and work plan

R4-2503161	Discussion on SCM General Aspects

Type: discussion		For: Discussion
					Source: Nokia

Decision: 		The document was not treated.

R4-2503509	TP on TR 38.753 Conclusions Chapter

Type: pCR		For: Approval
					38.753 v0.0.1	  CR-  rev  Cat:  (Rel-19)

					Source: Nokia

Decision: 		The document was not treated.

R4-2503539	On general aspects of Spatial Channel Modeling study

Type: discussion		For: Discussion
					Source: Apple

Decision: 		The document was not treated.

R4-2504177	TP to TR38753 on Spatial Channel Modeling for Demod - Annex

Type: pCR		For: Approval
					38.753 v0.0.1	  CR-  rev  Cat:  (Rel-19)

					Source: Qualcomm Incorporated, Apple

Decision: 		The document was not treated.

R4-2504380	Discussion on NR SCM general issue

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

General issue on way of working and content of chapter 6 and 7.

Decision: 		The document was not treated.

R4-2504383	Draft TP for TR38.753 Chapter 7 on simulation results alignment

Type: draft TR		For: Endorsement
					38.753 v0.0.1	  CR-  rev  Cat:  (Rel-19)

					Source: Ericsson

Abstract:

Draft TP for TR38.753 on content of Chapter 7

Decision: 		The document was withdrawn.

R4-2504431	PMI statistics for measured MIMO channels in band n78

Type: discussion		For: Information
					38.753 v	  CR-  rev  Cat:  (Rel-19)

					Source: BT plc, Ericsson

Decision: 		The document was not treated.

R4-2504476	pCR for TR38.753 Chapter 7 on simulation results alignment

Type: pCR		For: Approval
					38.753 v0.0.1	  CR-  rev  Cat:  (Rel-19)

					Source: Ericsson

Abstract:

TP for TR38.753 on content of Chapter 7

Decision: 		The document was not treated.

#### 7.16.2	Spatial channel modelling methodology

R4-2503199	Big pCR for TR 38.753

Type: pCR		For: Approval
					38.753 v0.0.1	  CR-  rev  Cat:  (Rel-19)

					Source: Nokia

Decision: 		The document was not treated.

R4-2503206	TP to TR 38.753: Scope and reference

Type: pCR		For: Approval
					38.753 v0.0.1	  CR-  rev  Cat:  (Rel-19)

					Source: China Telecom

Decision: 		The document was not treated.

R4-2503267	Discussion on spatial channel modelling

Type: discussion		For: Discussion
					Source: MediaTek inc.

Decision: 		The document was not treated.

R4-2503268	Simulation results of spatial channel modelling

Type: discussion		For: Discussion
					Source: MediaTek inc.

Decision: 		The document was not treated.

R4-2503269	TP to TR38.753: TDL approaches and related Annex

Type: pCR		For: Approval
					38.753 v0.0.1	  CR-  rev  Cat:  (Rel-19)

					Source: MediaTek inc.

Decision: 		The document was not treated.

R4-2503461	TP for 38.753: on definitions of terms, symbols and abbreviations

Type: pCR		For: Approval
					38.753 v0.0.1	  CR-  rev  Cat:  (Rel-19)

					Source: CATT

Decision: 		The document was not treated.

R4-2503540	Discussion on Spatial Channel Modeling Study

Type: discussion		For: Discussion
					Source: Apple

Decision: 		The document was not treated.

R4-2503541	Simulation results for Spatial Channel Modeling study

Type: discussion		For: Discussion
					Source: Apple

Decision: 		The document was not treated.

R4-2503542	Draft TP to TR 38.753 on SCM study - 5.1 CDL approach

Type: pCR		For: Approval
					38.753 v0.0.1	  CR-  rev  Cat:  (Rel-19)

					Source: Apple, Qualcomm

Decision: 		The document was not treated.

R4-2503554	Discussion on Spatial Channel Model for Demodulation Performance Requirements

Type: discussion		For: Discussion
					Source: Nokia

Decision: 		The document was not treated.

R4-2503555	Simulation Results and CDL Implementation for Spatial Channel Model for Demodulation Performance Requirements

Type: discussion		For: Information
					Source: Nokia

Decision: 		The document was not treated.

R4-2503658	Discussion on spatial channel model for demodulation performance requirements

Type: discussion		For: Discussion
					Source: Samsung

Decision: 		The document was not treated.

R4-2503659	Simulation results on spatial channel model

Type: discussion		For: Discussion
					Source: Samsung

Decision: 		The document was not treated.

R4-2503660	TP for 38.753 on chapter 4 and chapter 2

Type: pCR		For: Approval
					38.753 v0.0.1	  CR-  rev  Cat:  (Rel-19)

					Source: Samsung

Decision: 		The document was not treated.

R4-2504002	Discussion on spatial channel model for demodulation performance requirements

Type: other		For: Approval
					Source: ZTE Corporation, Sanechips

Decision: 		The document was not treated.

R4-2504003	Simulation results on spatial channel model for demodulation performance requirements.

Type: discussion		For: Discussion
					Source: ZTE Corporation, Sanechips

Decision: 		The document was not treated.

R4-2504004	TP for 38.753 introduction of spatial channel models for SU-MIMO PMI cases

Type: pCR		For: Approval
					38.753 v0.0.1	  CR-  rev  Cat:  (Rel-19)

					Source: ZTE Corporation, Sanechips

Decision: 		The document was not treated.

R4-2504175	Discussion on CDL channel model

Type: other		For: Approval
					Source: ROHDE & SCHWARZ

Decision: 		The document was not treated.

R4-2504176	Views on SCM for demod

Type: discussion		For: Agreement
					Source: Qualcomm Incorporated

Decision: 		The document was not treated.

R4-2504181	Simulation Results for SCM Demod

Type: discussion		For: Information
					Source: Qualcomm Incorporated

Decision: 		The document was not treated.

R4-2504308	Discussion on spatial channel model for demodulation requirements

Type: discussion		For: Discussion
					Source: Huawei,HiSilicon

Decision: 		The document was not treated.

R4-2504309	Simulation results on spatial channel model

Type: other		For: Discussion
					Source: Huawei,HiSilicon

Decision: 		The document was not treated.

R4-2504310	Draft TP introduction of comparision of spatial channel models for SU-MIMO PDSCH cases

Type: pCR		For: Approval
					38.753 v0.0.1	  CR-  rev  Cat:  (Rel-19)

					Source: Huawei,HiSilicon

Decision: 		The document was not treated.

R4-2504381	Discussion on NR SCM methodology for demodulation

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

Open issue on SCM methodologies

Decision: 		The document was not treated.

R4-2504382	Simulation results for NR SCM methodology

Type: other		For: Discussion
					Source: Ericsson

Abstract:

Simulaiton results for type-I and eType-II comparison

Decision: 		The document was not treated.

R4-2504429	Results collection table for NR SCM

Type: discussion		For: Discussion
					38.753 v	  CR-  rev  Cat:  (Rel-19)

					Source: BT plc

Decision: 		The document was not treated.

R4-2504430	Simulation Results for SCM Comparison and Alignment

Type: discussion		For: Discussion
					38.753 v	  CR-  rev  Cat:  (Rel-19)

					Source: BT plc

Decision: 		The document was not treated.

R4-2504432	Channel Properties Comparison Methodologies

Type: discussion		For: Information
					38.753 v	  CR-  rev  Cat:  (Rel-19)

					Source: BT plc

Decision: 		The document was not treated.

R4-2504616	TP for TR 38.753 Section 6.2 MU-MIMO

Type: pCR		For: Approval
					38.753 v0.0.1	  CR-  rev  Cat:  (Rel-19)

					Source: BT plc

Decision: 		The document was not treated.

R4-2504617	TP for TR 38.753 Annex A Channel Measurements

Type: pCR		For: Approval
					38.753 v0.0.1	  CR-  rev  Cat:  (Rel-19)

					Source: BT plc

Decision: 		The document was not treated.

#### 7.16.3	Moderator summary and conclusions

R4-2504645	Topic summary for [114bis][320] NR_SCM

Type: other		For: Information
					Source: Moderator (Nokia)

Abstract:

[114bis] BDaT Session AI 7.16.1, 7.16.2

Decision: 		The document was not treated.

### 7.17	NR Radio Resource Management (RRM) Phase 5

#### 7.17.1	General aspects

R4-2503530	Test case (performance part) plan for R19 RRM Phase 5 WI

Type: Work Plan		For: Approval
					Source: Apple, CATT

Decision: 		The document was not treated.

#### 7.17.2	RRM core requirements of FR2-1 SSB based L3 measurement delay reduction for connected mode

##### 7.17.2.1	FR2-1 L3 measurement delay by optimizing Rx beam sweeping factor

R4-2503251	FR2-1 L3 measurement delay reduction by optimizing RX beam sweeping factor

Type: discussion		For: Discussion
					Source: Qualcomm Incorporated

Decision: 		The document was not treated.

R4-2503289	Remaining issues on FR2-1 L3 measurement delay by optimizing Rx beam sweeping factor

Type: discussion		For: Discussion
					Source: vivo

Decision: 		The document was not treated.

R4-2503356	Discussion on FR2-1 L3 measurement delay by optimizing Rx beam sweeping factor

Type: other		For: Approval
					Source: OPPO

Decision: 		The document was not treated.

R4-2503357	DraftCR on Multi-Rx based RRC Connection Release with Redirection requirements

Type: draftCR		For: Endorsement
					38.133 v19.0.0	  CR-  rev  Cat:  (Rel-19)

					Source: OPPO

Decision: 		The document was not treated.

R4-2503416	Discussion on FR2-1 L3 measurement delay reduction by optimizing Rx beam sweeping factor

Type: discussion		For: Discussion
					Source: CATT

Decision: 		The document was not treated.

R4-2503420	DraftCR on SSB-based intra-frequency measurement without gap delay reduction by optimizing Rx BSF

Type: draftCR		For: Endorsement
					38.133 v19.0.0	  CR-  rev  Cat: B (Rel-19)

					Source: CATT

Decision: 		The document was not treated.

R4-2503707	Discussion on RRM requirements for FR2-1 L3 measurement delay reduction

Type: discussion		For: Discussion
					Source: LG Electronics Inc.

Decision: 		The document was not treated.

R4-2503763	Discussion on L3 measurement delay reduction by optimizing Rx beam sweeping factor

Type: discussion		For: Discussion
					Source: CMCC

Decision: 		The document was not treated.

R4-2503779	DraftCR for optimizing Rx beam sweeping factor on SSB based Inter-frequency measurement with MG

Type: draftCR		For: Endorsement
					38.133 v18.9.0	  CR-  rev  Cat: B (Rel-18)

					Source: CMCC

Decision: 		The document was not treated.

R4-2503785	Discussions on FR2-1 SSB based L3 measurement delay reduction by optimizing Rx beam sweeping factor

Type: discussion		For: Discussion
					Source: NTT DOCOMO, INC.

Decision: 		The document was not treated.

R4-2503862	Discussion on FR2-1 L3 measurement delay by optimizing Rx beam sweeping factor

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon

Decision: 		The document was not treated.

R4-2503911	Discussion on FR2-1 L3 measurement delay by optimizing Rx beam sweeping factor

Type: discussion		For: Discussion
					Source: China Telecom

Decision: 		The document was not treated.

R4-2504020	Discussion on delay reduction by optimizing Rx beam sweeping factors

Type: discussion		For: Discussion
					Source: Intel Corporation

Decision: 		The document was not treated.

R4-2504075	Draft CR HO requirements and FBS requirement applicability

Type: draftCR		For: Endorsement
					38.133 v19.0.0	  CR-  rev  Cat: B (Rel-19)

					Source: Nokia

Decision: 		The document was not treated.

R4-2504113	Discussion on FR2-1 L3 measurement delay by optimizing Rx beam sweeping factor

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

Discussion on FR2-1 L3 measurement delay by optimizing Rx beam sweeping factor

Decision: 		The document was not treated.

R4-2504243	Discussion on Rx beam factor optimization of R19 RRM enhancements

Type: discussion		For: Approval
					Source: ZTE Corporation, Sanechips

Decision: 		The document was not treated.

R4-2504256	Draft CR on FBS based RRC Re-establishment for R19 RRM enh

Type: draftCR		For: Endorsement
					38.133 v19.0.0	  CR-  rev  Cat: B (Rel-19)

					Source: ZTE Corporation, Sanechips

Decision: 		The document was not treated.

R4-2504456	On FR2-1 L3 measurement delay by optimizing Rx beam sweeping factor

Type: discussion		For: Discussion
					Source: Apple

Decision: 		The document was not treated.

R4-2504457	Draft CR on SSB based Intra-frequency measurement with MG

Type: draftCR		For: Endorsement
					38.133 v19.0.0	  CR-  rev  Cat: B (Rel-19)

					Source: Apple

Decision: 		The document was not treated.

R4-2504500	On L3 FBS requirements

Type: discussion		For: Discussion
					Source: Nokia, Nokia Shanghai Bell

Decision: 		The document was not treated.

R4-2504565	Discussion on L3 measurement enhancement by optimizing Rx BSF

Type: discussion		For: Discussion
					38.133 v	  CR-  rev  Cat:  (Rel-19)

					Source: Samsung

Decision: 		The document was not treated.

R4-2504600	Discussion on FR2-1 SSB based L3 measurement delay reduction by optimizing Rx beam sweeping factor

Type: discussion		For: Discussion
					Source: MediaTek inc.

Decision: 		The document was not treated.

R4-2504657	draft CR for Rx beam sweeping optimization for SSB based Intra-frequency measurement with NCSG

Type: draftCR		For: Endorsement
					38.133 v19.0.0	  CR-  rev  Cat: B (Rel-19)

					Source: MediaTek Germany GmbH

Decision: 		The document was not treated.

##### 7.17.2.2	FR2-1 L3 measurement delay by optimizing CSSF outside gap in CA/DC

R4-2503250	Enhancement on FR2 CSSF

Type: discussion		For: Discussion
					Source: Qualcomm Incorporated

Decision: 		The document was not treated.

R4-2503288	Remaining issues on FR 2-1 L3 measurement delay by optimizing CSSF outside gap in CA/DC

Type: discussion		For: Discussion
					Source: vivo

Decision: 		The document was not treated.

R4-2503340	On optimizing CSSF for FR2-1 L3 measurement delay

Type: other		For: Approval
					Source: OPPO

Decision: 		The document was not treated.

R4-2503417	Discussion on FR2-1 L3 measurement delay reduction by optimizing CSSF outside gap in CADC

Type: discussion		For: Discussion
					Source: CATT

Decision: 		The document was not treated.

R4-2503503	FR2-1 SSB based L3 measurement delay reduction with CSSF optimization

Type: discussion		For: Discussion
					38.133 v	  CR-  rev  Cat:  (Rel-19)

					Source: Nokia, Nokia Shanghai Bell

Decision: 		The document was not treated.

R4-2503504	draftCR on CSSF optimization solutions

Type: draftCR		For: Endorsement
					38.133 v19.0.0	  CR-  rev  Cat:  (Rel-19)

					Source: Nokia, Nokia Shanghai Bell

Decision: 		The document was not treated.

R4-2503529	draft CR for Rel-19 CSSF enhancement solution 1 (one CC measurement per band)

Type: draftCR		For: Endorsement
					38.133 v19.0.0	  CR-  rev  Cat: B (Rel-19)

					Source: Apple, CATT, Huawei, HiSilicon, Qualcomm, Ericsson, ZTE, MediaTek, Samsung

Decision: 		The document was not treated.

R4-2503531	On FR2-1 L3 measurement delay by optimizing CSSF outside gap in CA/DC

Type: discussion		For: Discussion
					38.133 v	  CR-  rev  Cat:  (Rel-19)

					Source: Apple

Decision: 		The document was not treated.

R4-2503764	Discussion on L3 measurement delay reduction by optimizing CSSF outside gap in CA/DC

Type: discussion		For: Discussion
					Source: CMCC

Decision: 		The document was not treated.

R4-2503874	Discussion on FR2-1 L3 measurement delay by optimising CSSF outside gap in CA/DC

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon

Decision: 		The document was not treated.

R4-2503912	Discussion on FR2-1 L3 measurement delay by optimizing CSSF outside gap in CA/DC

Type: discussion		For: Discussion
					Source: China Telecom

Decision: 		The document was not treated.

R4-2504021	Discussion on delay reduction by optimizing CSSF outside gap

Type: discussion		For: Discussion
					Source: Intel Corporation

Decision: 		The document was not treated.

R4-2504114	Discussion on FR2-1 L3 measurement delay by optimizing CSSF outside gap in CA/DC

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

Discussion on FR2-1 L3 measurement delay by optimizing CSSF outside gap in CA/DC

Decision: 		The document was not treated.

R4-2504242	Discussion on CSSF optimization of R19 RRM enhancements

Type: discussion		For: Approval
					Source: ZTE Corporation, Sanechips

Decision: 		The document was not treated.

R4-2504601	Discussion on FR2-1 SSB based L3 measurement delay reduction by optimizing CSSF

Type: discussion		For: Discussion
					Source: MediaTek inc.

Decision: 		The document was not treated.

#### 7.17.3	RRM core requirements of Fast SCell activation for UE supporting Rel-18 EMR

R4-2503568	draft CR for fast SCell activation for UE supporting Rel-18 EMR

Type: draftCR		For: Endorsement
					38.133 v19.0.0	  CR-  rev  Cat: B (Rel-19)

					Source: Apple

Decision: 		The document was not treated.

R4-2503753	Discusssion for fast scell activation for UE supporting Rel-18 EMR

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

This contribution discuss the remaining issues on fast Scell activation for UE supporting Rel-18 EMR

Decision: 		The document was not treated.

R4-2503765	Discussion on fast SCell activation for UE supporting Rel-18 EMR

Type: discussion		For: Discussion
					Source: CMCC

Decision: 		The document was not treated.

R4-2503848	Discussion on the fast SCell activation for UE supporting Rel-18 EMR

Type: other		For: Approval
					Source: ZTE Corporation, Sanechips

Decision: 		The document was not treated.

R4-2503863	Discussion on maintenance for fast SCell activation based on EMR

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon

Decision: 		The document was not treated.

R4-2503864	Draft CR on fast SCell activation for UE supporting Rel-18 EMR

Type: draftCR		For: Endorsement
					38.133 v19.0.0	  CR-  rev  Cat: B (Rel-19)

					Source: Huawei, HiSilicon

Decision: 		The document was not treated.

#### 7.17.4	RRM performance requirements of FR2-1 SSB based L3 measurement delay reduction for connected mode

R4-2503418	Discussion on performance requirements for FR2-1 L3 measurement delay reduction

Type: discussion		For: Discussion
					Source: CATT

Decision: 		The document was not treated.

R4-2503505	draftCR on test case for intra-frequency measurement with CSSF solution 1

Type: draftCR		For: Endorsement
					38.133 v19.0.0	  CR-  rev  Cat:  (Rel-19)

					Source: Nokia, Nokia Shanghai Bell

Decision: 		The document was not treated.

R4-2503646	Discussion on performance requirements of FR2-1 SSB based L3 measurement delay reduction for connected mode

Type: other		For: Approval
					Source: OPPO

Decision: 		The document was not treated.

R4-2503766	Discussion on RRM performance requirements of FR2-1 SSB based L3 measurement delay reduction for connected mode

Type: discussion		For: Discussion
					Source: CMCC

Decision: 		The document was not treated.

R4-2503875	Discussion Discussion on performance part for FR2-1 SSB based L3 measurement delay reduction for connected mode

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon

Decision: 		The document was not treated.

R4-2503913	Discussion on RRM performance requirements of FR2-1 SSB based L3 measurement delay reduction for connected mode

Type: discussion		For: Discussion
					Source: China Telecom

Decision: 		The document was not treated.

R4-2504076	On performance part of L3 measurement delay reduction

Type: discussion		For: Discussion
					Source: Nokia, Nokia Shanghai Bell

Decision: 		The document was not treated.

R4-2504115	Discussion on performance requirements for FR2-1 L3 measurement enhancement

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

Discussion on performance requirements for FR2-1 L3 measurement enhancement

Decision: 		The document was not treated.

#### 7.17.5	RRM performance requirements of Fast SCell activation for UE supporting Rel-18 EMR

R4-2503358	Discussion on performance requirements for fast SCell activation for UE supporting Rel-18 EMR

Type: other		For: Approval
					Source: OPPO

Decision: 		The document was not treated.

R4-2503419	Discussion on performance requirements of fast SCell activation for UE supporting R18 EMR

Type: discussion		For: Discussion
					Source: CATT

Decision: 		The document was not treated.

R4-2503754	Discussion on performance part of the fast Scell activation for UE supporting Rel-18 EMR

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

This contribution share the initial thoughts on the performance part of the fast Scell activaiton for UE supporting Rel-18 EMR

Decision: 		The document was not treated.

R4-2503767	Discussion on RRM performance requirements for fast SCell activation for UE supporting Rel-18 EMR

Type: discussion		For: Discussion
					Source: CMCC

Decision: 		The document was not treated.

R4-2503849	Discussion on RRM performance requirements of Fast SCell activation for UE supporting Rel-18 EMR

Type: other		For: Approval
					Source: ZTE Corporation, Sanechips

Decision: 		The document was not treated.

R4-2503865	Discussion on performance part for Fast SCell activation for UE supporting Rel-18 EMR

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon

Decision: 		The document was not treated.

R4-2503914	Discussion on RRM performance requirements of Fast SCell activation for UE supporting Rel-18 EMR

Type: discussion		For: Discussion
					Source: China Telecom

Decision: 		The document was not treated.

R4-2504518	Discussion on RRM performance requirements of Fast SCell activation for UE supporting Rel-18 EMR

Type: discussion		For: Discussion
					38.133 v	  CR-  rev  Cat:  (Rel-19)

					Source: Nokia

Decision: 		The document was not treated.

#### 7.17.6	Moderator summary and conclusions

R4-2503618	Topic summary for [114bis][208] NR_RRM_Ph5_Part1

Type: other		For: Information
					Source: Moderator (Apple)

Abstract:

Topic summary in RRM session

Decision: 		The document was not treated.

R4-2503619	Topic summary for [114bis][209] NR_RRM_Ph5_Part2

Type: other		For: Information
					Source: Moderator (CATT)

Abstract:

Topic summary in RRM session

Decision: 		The document was not treated.

### 7.18	NR demodulation performance Phase 5

R4-2504519	Views on 8Rx UE demodulation and CSI requirements with inter-cell and intra-cell inter-user interference

Type: discussion		For: Discussion
					Source: Qualcomm

Abstract:

Views on 8Rx demodulation and CQI requirements with intercell interference along with demod requirements wiith MU-MIMO

Decision: 		The document was not treated.

#### 7.18.1	General aspects

R4-2503202	Updated work plan for NR demodulation performance Phase 5

Type: Work Plan		For: Approval
					Source: China Telecom

Abstract:

Addition of CR work split.

Decision: 		The document was not treated.

R4-2503909	On general issues for Rel-19 MMSE-IRC receiver under interference

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

This contribution discusses the general issues related to UE and BS requirements for MMSE-IRC under interference

Decision: 		The document was not treated.

R4-2503924	Discussion on general topics for 8Rx UE with MMSE-IRC

Type: discussion		For: Discussion
					Source: Nokia

Decision: 		The document was not treated.

#### 7.18.2	UE demodulation performance requirements for 8Rx with MMSE-IRC

R4-2503140	Discussion on UE demodulation requirements with 8Rx MMSE-IRC receiver

Type: discussion		For: Discussion
					Source: MediaTek inc.

Decision: 		The document was not treated.

R4-2503141	Simulation results for UE demodulation requirements with 8Rx MMSE-IRC receiver

Type: discussion		For: Information
					Source: MediaTek inc.

Decision: 		The document was not treated.

R4-2503203	Discussion on UE demodulation performance requirements for 8Rx with MMSE-IRC

Type: discussion		For: Discussion
					Source: China Telecom

Decision: 		The document was not treated.

R4-2503376	Simulation results for 8Rx UE interference suppressing

Type: discussion		For: Discussion
					Source: CMCC

Decision: 		The document was not treated.

R4-2503377	Discussion on interference suppressing performance for 8Rx UE

Type: discussion		For: Discussion
					Source: CMCC

Decision: 		The document was not treated.

R4-2503543	On UE demodulation performance requirements with 8RX

Type: discussion		For: Discussion
					Source: Apple

Decision: 		The document was not treated.

R4-2503544	Simulation results for ICI and MU-MIMO with 8RX

Type: discussion		For: Information
					Source: Apple

Decision: 		The document was not treated.

R4-2503545	Summary of simulation results for ICI and MU-MIMO with 8RX

Type: discussion		For: Information
					Source: Apple

Decision: 		The document was not treated.

R4-2503661	Discussion on UE demodulation and CSI performance requirements for 8Rx with MMSE-IRC

Type: discussion		For: Discussion
					Source: Samsung

Decision: 		The document was not treated.

R4-2503925	Discussion on UE demodulation performance requirements for 8Rx with MMSE-IRC

Type: discussion		For: Discussion
					Source: Nokia

Decision: 		The document was not treated.

R4-2503926	Simulations for UE demodulation performance requirements for 8Rx with MMSE-IRC

Type: discussion		For: Information
					Source: Nokia

Decision: 		The document was not treated.

R4-2503995	Discussion on UE demodulation performance requirements for 8Rx with MMSE-IRC

Type: other		For: Approval
					Source: ZTE Corporation, Sanechips

Decision: 		The document was not treated.

R4-2503996	Simulation results on UE demodulation performance requirements for 8Rx with MMSE-IRC

Type: discussion		For: Discussion
					Source: ZTE Corporation, Sanechips

Decision: 		The document was not treated.

R4-2504304	Discussion on 8Rx MMSE-IRC requirements

Type: discussion		For: Discussion
					Source: Huawei,HiSilicon

Decision: 		The document was not treated.

R4-2504305	Simulation results on 8Rx MMSE-IRC requirements

Type: other		For: Discussion
					Source: Huawei,HiSilicon

Decision: 		The document was not treated.

R4-2504327	On MMSE-IRC receiver for interference mitigation with 8Rx

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

This contribution provided our views on the left open issues for demodulation and CQI reporting requirements with 8Rx plus interference scenarios

Decision: 		The document was not treated.

R4-2504328	Simulation results for MMSE-IRC receiver for 8Rx reception

Type: other		For: Discussion
					Source: Ericsson

Abstract:

This contribution provided our simulation results

Decision: 		The document was not treated.

#### 7.18.3	BS demodulation performance requirements for MMSE-IRC

R4-2503204	Simulation results on BS demodulation performance requirements for MMSE-IRC

Type: discussion		For: Discussion
					Source: China Telecom

Decision: 		The document was not treated.

R4-2503205	Discussion on BS demodulation performance requirements for MMSE-IRC

Type: discussion		For: Discussion
					Source: China Telecom

Decision: 		The document was not treated.

R4-2503275	Discussion on BS demodulation performance requirement using MMSE-IRC

Type: discussion		For: (not specified)
					Source: Tejas Network Limited

Decision: 		The document was not treated.

R4-2503378	Simulation results for BS interference suppressing with MMSE-IRC

Type: discussion		For: Discussion
					Source: CMCC

Decision: 		The document was not treated.

R4-2503379	Discussion on BS demodulation requirements for interference suppressing with MMSE-IRC receiver

Type: discussion		For: Discussion
					Source: CMCC

Decision: 		The document was not treated.

R4-2503452	Further discussion on BS demodulation performance for MMSE-IRC

Type: other		For: Approval
					Source: CATT

Decision: 		The document was not treated.

R4-2503453	Simulation results for BS requirement with MMSE_IRC receiver

Type: other		For: Discussion
					Source: CATT

Decision: 		The document was not treated.

R4-2503712	Simulation Results – NR BS Demodulation Performance Requirements for MMSE-IRC

Type: discussion		For: Discussion
					Source: Tejas Network Limited

Decision: 		The document was not treated.

R4-2503907	Discussion on Rel-19 MMSE-IRC BS demodulation requirements

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

This contribution discusses open issues on PUSCH MMSE-IRC requirements

Decision: 		The document was not treated.

R4-2503908	Simulation results for Rel-19 MMSE-IRC BS demodulation requirements

Type: other		For: Discussion
					Source: Ericsson

Abstract:

This contribution presents the simulation results for PUSCH MMSE-IRC

Decision: 		The document was not treated.

R4-2503997	Discussion on BS demodulation performance for MMSE-IRC

Type: other		For: Approval
					Source: ZTE Corporation, Sanechips

Decision: 		The document was not treated.

R4-2503998	Simulation results on BS demodulation performance for MMSE-IRC

Type: discussion		For: Discussion
					Source: ZTE Corporation, Sanechips

Decision: 		The document was not treated.

R4-2504306	Discussion on BS MMSE-IRC requirements with inter-cell interference

Type: discussion		For: Discussion
					Source: Huawei,HiSilicon

Decision: 		The document was not treated.

R4-2504307	Simulation results on BS MMSE-IRC requirements with inter-cell interference

Type: other		For: Discussion
					Source: Huawei,HiSilicon

Decision: 		The document was not treated.

R4-2504342	Views on BS demodulation requirements for MMSE-IRC

Type: discussion		For: Approval
					Source: Qualcomm Germany

Decision: 		The document was not treated.

R4-2504441	NR Demodulation Performance Phase 5: BS demodulation performance requirements for MMSE-IRC

Type: discussion		For: Discussion
					Source: Nokia

Decision: 		The document was not treated.

R4-2504442	Simulation Results – NR BS Demodulation Performance Requirements for MMSE-IRC

Type: other		For: Discussion
					Source: Nokia

Decision: 		The document was not treated.

R4-2504521	Discussion on BS demodulation for NR MMSE-IRC.

Type: discussion		For: Discussion
					Source: NTT DOCOMO, INC.

Decision: 		The document was not treated.

R4-2504533	Simulation results summary for BS requirement with MMSE-IRC receiver

Type: other		For: Discussion
					Source: Samsung

Decision: 		The document was not treated.

R4-2504534	View on the remaining issue for BS demodulation requirement with MMSE-IRC receiver

Type: discussion		For: Discussion
					Source: Samsung

Decision: 		The document was not treated.

R4-2504535	Simulation results on BS demodulation requirement with MMSE-IRC receiver

Type: discussion		For: Discussion
					Source: Samsung

Decision: 		The document was not treated.

#### 7.18.4	Moderator summary and conclusions

R4-2504646	Topic summary for [114bis][321] NR_demod_Ph5_Part1_General_BS

Type: other		For: Information
					Source: Moderator (China Telecom)

Abstract:

[114bis] BDaT Session AI 7.18.1, 7.18.3

Decision: 		The document was not treated.

R4-2504647	Topic summary for [114bis][322] NR_demod_Ph5_Part2_UE

Type: other		For: Information
					Source: Moderator (Nokia)

Abstract:

[114bis] BDaT Session AI 7.18.2

Decision: 		The document was not treated.

### 7.19	Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface

#### 7.19.1	General aspects

R4-2503227	Discussion on General Aspects for AI/ML Air Interface

Type: discussion		For: Discussion
					Source: Korea Testing Laboratory

Decision: 		The document was not treated.

R4-2503229	Discussion on general aspects of AIML for NR air interface

Type: discussion		For: Discussion
					Source: CAICT

Decision: 		The document was not treated.

R4-2503253	Generalization aspects of AI-ML

Type: discussion		For: Discussion
					Source: Qualcomm Incorporated

Decision: 		The document was not treated.

R4-2503306	Discussion on general aspect for AI

Type: discussion		For: Discussion
					Source: Xiaomi

Decision: 		The document was not treated.

R4-2503514	Discussion on General Aspects on AI/ML for NR Air Interface

Type: discussion		For: (not specified)
					Source: Apple

Decision: 		The document was not treated.

R4-2503770	Discussion on general aspects

Type: discussion		For: Discussion
					Source: CMCC

Decision: 		The document was not treated.

R4-2503920	On general issues for AIML

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

On general issues for AIML

Decision: 		The document was not treated.

R4-2504229	General aspects of AI/ML for NR Air interface

Type: discussion		For: Discussion
					Source: Nokia

Decision: 		The document was not treated.

R4-2504316	Discussion on general aspects on AIML for NR air interface

Type: discussion		For: Discussion
					Source: Huawei,HiSilicon

Decision: 		The document was not treated.

#### 7.19.2	CSI reporting requirement and testing framework for CSI prediction

R4-2503252	CSI Prediction: Simulation results and core requirements

Type: discussion		For: Discussion
					Source: Qualcomm Incorporated

Decision: 		The document was not treated.

R4-2503270	Discussion on CSI reporting requirement framework for CSI prediction

Type: discussion		For: Discussion
					Source: MediaTek inc.

Decision: 		The document was not treated.

R4-2503406	Discussion on CSI reporting requirement framework for CSI prediction

Type: discussion		For: Discussion
					Source: CATT

Decision: 		The document was not treated.

R4-2503517	Discussion on CSI Prediction: CSI reporting requirement framework for CSI prediction

Type: discussion		For: (not specified)
					Source: Apple

Decision: 		The document was not treated.

R4-2503769	Discussion on CSI prediction

Type: discussion		For: Discussion
					Source: CMCC

Decision: 		The document was not treated.

R4-2504005	Discussion on CSI reporting requirement and testing framework for CSI prediction.

Type: other		For: Approval
					Source: ZTE Corporation, Sanechips

Decision: 		The document was not treated.

R4-2504022	Discussion on CSI reporting requirement and testing framework for CSI prediction

Type: discussion		For: Discussion
					Source: vivo

Decision: 		The document was not treated.

R4-2504235	CSI reporting requirement and testing framework for CSI prediction

Type: other		For: Approval
					Source: OPPO

Decision: 		The document was not treated.

R4-2504317	Discussion on CSI reporting requirement framework for AIML CSI prediction

Type: discussion		For: Discussion
					Source: Huawei,HiSilicon

Decision: 		The document was not treated.

R4-2504346	Discussion on CSI prediction

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

This contribution discusses the AI/ML-based CSI prediction.

Decision: 		The document was not treated.

R4-2504539	View on CSI reporting requirements framework for CSI prediction

Type: discussion		For: Discussion
					Source: Samsung

Decision: 		The document was not treated.

R4-2504541	CSI reporting requirement and testing framework for CSI prediction

Type: discussion		For: Discussion
					Source: Nokia

Decision: 		The document was not treated.

#### 7.19.3	RRM core requirement and testing framework for beam management

R4-2503230	Discussion on RRM core requirements for beam management

Type: discussion		For: Discussion
					Source: CAICT

Decision: 		The document was not treated.

R4-2503257	On DL Tx Hierarchy Beamforming Codebook Design and the Prediction Impact

Type: discussion		For: Approval
					Source: NTU

Decision: 		The document was not treated.

R4-2503307	Discussion on core part requirement and testing framework for AI beam management

Type: discussion		For: Discussion
					Source: Xiaomi

Decision: 		The document was not treated.

R4-2503407	Discussion on RRM core requirements testing framework for BM

Type: discussion		For: Discussion
					Source: CATT

Decision: 		The document was not treated.

R4-2503515	Discussion on Beam Management: RRM core requirement and testing framework for beam management

Type: discussion		For: (not specified)
					Source: Apple

Decision: 		The document was not treated.

R4-2503553	RRM Core Requirements for AI/ML Beam Management

Type: discussion		For: Discussion
					Source: Qualcomm Incorporated

Decision: 		The document was not treated.

R4-2503606	Discussion on RRM core requirement and testing framework for beam management

Type: discussion		For: Discussion
					Source: MediaTek Inc.

Decision: 		The document was not treated.

R4-2503701	RRM core requirement and testing framework for beam management

Type: discussion		For: Discussion
					Source: Tejas Network Limited

Decision: 		The document was not treated.

R4-2503711	FR2 test setup for AI/ML beam management evaluation

Type: other		For: Approval
					Source: Anritsu Corporation

Decision: 		The document was not treated.

R4-2503768	Discussion on RRM requirements for beam management

Type: discussion		For: Discussion
					Source: CMCC

Decision: 		The document was not treated.

R4-2503786	Discussions on RRM core requirement for beam management

Type: discussion		For: Discussion
					Source: NTT DOCOMO, INC.

Decision: 		The document was not treated.

R4-2503789	Discussion on the RRM requirements of AI/ML Beam management

Type: other		For: Approval
					Source: ZTECorporation,Sanechips

Decision: 		The document was not treated.

R4-2504023	Discussion on RRM core requirement and testing framework for beam management

Type: discussion		For: Discussion
					Source: vivo

Decision: 		The document was not treated.

R4-2504055	Testability for AI/ML beam management

Type: other		For: Approval
					Source: Qualcomm Incorporated

Decision: 		The document was not treated.

R4-2504107	Discussion on RRM core requirements and test framework for beam management

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

Discussion on RRM core requirements and test framework for beam management

Decision: 		The document was not treated.

R4-2504216	Discussion on RRM core requirements and testing framework for AI-BM

Type: other		For: Approval
					Source: Samsung

Decision: 		The document was not treated.

R4-2504230	RRM core requirement and testing framework for beam management

Type: discussion		For: Discussion
					Source: Nokia

Decision: 		The document was not treated.

R4-2504238	RRM core requirement and testing framework for beam management

Type: other		For: Approval
					Source: OPPO

Decision: 		The document was not treated.

R4-2504268	Discussion on test setup for AI/ML based beam management

Type: discussion		For: Discussion
					Source: Rohde & Schwarz

Decision: 		The document was not treated.

R4-2504318	Discussion on RRM core requirement and testing framework for AIML Beam management

Type: discussion		For: Discussion
					Source: Huawei,HiSilicon

Decision: 		The document was not treated.

#### 7.19.4	RRM core requirement and testing framework for Positioning accuracy enhancement

R4-2503254	RRM Core Requirements for Positioning Accuracy

Type: discussion		For: Discussion
					Source: Qualcomm Incorporated

Decision: 		The document was not treated.

R4-2503408	Discussion on RRM core requirements testing framework for positioning

Type: discussion		For: Discussion
					Source: CATT

Decision: 		The document was not treated.

R4-2503518	Discussion on RRM Core Requirements for positioning accuracy enhancement

Type: discussion		For: (not specified)
					Source: Apple

Decision: 		The document was not treated.

R4-2503771	Discussion on RRM requirements for positioning accuracy enhancement

Type: discussion		For: Discussion
					Source: CMCC

Decision: 		The document was not treated.

R4-2503790	Discussion on the RRM requirements of AI/ML positioning

Type: other		For: Approval
					Source: ZTECorporation,Sanechips

Decision: 		The document was not treated.

R4-2503927	On requirements for AI/ML based positioning

Type: other		For: Approval
					Source: Ericsson

Abstract:

This paper addresses the issues related to RAN4 requirements for AI/ML based positioning.

Decision: 		The document was not treated.

R4-2504024	Discussion on RRM core requirement and testing framework for Positioning accuracy enhancement

Type: discussion		For: Discussion
					Source: vivo

Decision: 		The document was not treated.

R4-2504237	RRM core requirement and testing framework for Positioning

Type: other		For: Approval
					Source: OPPO

Decision: 		The document was not treated.

R4-2504319	Discussion on RRM core requirement and testing framework for AIML Positioning

Type: discussion		For: Discussion
					Source: Huawei,HiSilicon

Decision: 		The document was not treated.

R4-2504542	RRM core requirements and testing framework for AI/ML positioning accuracy enhancement

Type: discussion		For: Discussion
					Source: Nokia

Decision: 		The document was not treated.

#### 7.19.5	Moderator summary and conclusions

R4-2504684	Topic summary for [114bis][126] NR_AIML_air_part1

Type: other		For: Information
					Source: Moderator(Qualcomm)

Abstract:

Summary for AI 7.19

Decision: 		The document was not treated.

### 7.20	Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface Phase 2

#### 7.20.1	General aspects

R4-2503409	Discussion on general aspects for AIML for NR air

Type: discussion		For: Discussion
					Source: CATT

Decision: 		The document was not treated.

R4-2503519	General aspects for NR Air Interface

Type: discussion		For: (not specified)
					Source: Apple

Decision: 		The document was not treated.

R4-2503772	Discussion on general aspects for study on AI/ML

Type: discussion		For: Discussion
					Source: CMCC

Decision: 		The document was not treated.

R4-2504025	Discussion on general aspects for AI/ML for NR air interface Phase 2

Type: discussion		For: Discussion
					Source: vivo

Decision: 		The document was not treated.

R4-2504215	On General Aspects of AI/ML for NR Air Interface Phase 2

Type: discussion		For: Discussion
					Source: Nokia

Decision: 		The document was not treated.

R4-2504320	Discussion on general aspects on AIML for NR air interface phase 2

Type: discussion		For: Discussion
					Source: Huawei,HiSilicon

Decision: 		The document was not treated.

#### 7.20.2	Testing issues for CSI compression two-sided models

R4-2503255	CSI Compression: Testability and Interoperability

Type: discussion		For: Discussion
					Source: Qualcomm Incorporated

Decision: 		The document was not treated.

R4-2503258	On AI/ML Interoperability and Standardization of Testing and Verification for CSI Use Case – A PCA based approach

Type: discussion		For: Approval
					Source: NTU

Decision: 		The document was not treated.

R4-2503271	Discussion on Testing issues for CSI compression two-sided models

Type: discussion		For: Discussion
					Source: MediaTek inc.

Decision: 		The document was not treated.

R4-2503410	Discussion on testing issues for CSI compression two-sided models

Type: discussion		For: Discussion
					Source: CATT

Decision: 		The document was not treated.

R4-2503516	Study on CSI Compression: Testability and Interoperability issues

Type: discussion		For: (not specified)
					Source: Apple

Decision: 		The document was not treated.

R4-2503773	Discussion on testing issues for CSI compression two-sided models

Type: discussion		For: Discussion
					Source: CMCC

Decision: 		The document was not treated.

R4-2504006	Discussion on Testing issues for CSI compression two-sided models

Type: other		For: Approval
					Source: ZTE Corporation, Sanechips

Decision: 		The document was not treated.

R4-2504026	Discussion on testing issues for CSI compression two-sided models

Type: discussion		For: Discussion
					Source: vivo

Decision: 		The document was not treated.

R4-2504217	Study on testing issues for CSI compression two-sided models

Type: other		For: Approval
					Source: Samsung

Decision: 		The document was not treated.

R4-2504231	Testing issues for CSI compression two-sided models

Type: discussion		For: Discussion
					Source: Nokia

Decision: 		The document was not treated.

R4-2504236	Testing issues for CSI compression two-sided models

Type: other		For: Approval
					Source: OPPO

Decision: 		The document was not treated.

R4-2504321	Discussion on testing issues for CSI compression two-sided models

Type: discussion		For: Discussion
					Source: Huawei,HiSilicon

Decision: 		The document was not treated.

R4-2504439	Two sided CSI compression

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

Simulation results and further proposals for 2-sided CSI

Decision: 		The document was not treated.

#### 7.20.3	Moderator summary and conclusions

R4-2504685	Topic summary for [114bis][127] NR_AIML_air_part2

Type: other		For: Information
					Source: Moderator(Ericsson)

Abstract:

Summary for AI 7.20

Decision: 		The document was not treated.

### 7.21	Study on AI (Artificial Intelligence)/ML (Machine Learning) for mobility in NR

#### 7.21.1	General aspects

R4-2503350	Discussion on general aspects for AI mobility

Type: other		For: Approval
					Source: OPPO

Decision: 		The document was not treated.

R4-2503521	General aspects on AI/ML for mobility in NR

Type: discussion		For: (not specified)
					Source: Apple

Decision: 		The document was not treated.

R4-2503645	TP on RAN4 aspects for TR 38.744

Type: pCR		For: Approval
					38.744 v0.0.4	  CR-  rev  Cat:  (Rel-19)

					Source: OPPO

Decision: 		The document was not treated.

R4-2503774	Discussion on general part for AI/ML for mobility

Type: discussion		For: Discussion
					Source: CMCC

Decision: 		The document was not treated.

R4-2503850	Discussion on General Aspects of AIML Mobility

Type: other		For: Approval
					Source: ZTE Corporation, Sanechips

Decision: 		The document was not treated.

R4-2503886	Discussion on genereal aspects in AIML mobility

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon

Decision: 		The document was not treated.

R4-2503928	General discussion on AI/ML for mobility

Type: other		For: Approval
					Source: Ericsson

Abstract:

This paper discusses issues that have an impact on different prediction types considered for AI/ML based mobility.

Decision: 		The document was not treated.

R4-2504046	Discussion on general aspects for AI mobility

Type: discussion		For: Discussion
					Source: vivo

Decision: 		The document was not treated.

R4-2504288	Discussion on AI mobility general and scope

Type: discussion		For: Discussion
					38.133 v	  CR-  rev  Cat:  (Rel-19)

					Source: Samsung

Decision: 		The document was not treated.

#### 7.21.2	Study of impacts on RAN4 requirements

R4-2503705	Study of impacts on RAN4 requirements for AIML mobility in NR

Type: discussion		For: Discussion
					Source: Tejas Network Limited

Decision: 		The document was not treated.

##### 7.21.2.1	Study of RAN4 impacts for RRM measurement prediction

R4-2503308	Discussion on impacts on RAN4 requirement for RRM measurement prediction in AI mobility

Type: discussion		For: Discussion
					Source: Xiaomi

Decision: 		The document was not treated.

R4-2503351	Study of impacts on RAN4 requirements for RRM measurement prediction

Type: other		For: Approval
					Source: OPPO

Decision: 		The document was not treated.

R4-2503411	Discussion on impacts of AIML RRM measurement prediction on RRM requirements

Type: discussion		For: Discussion
					Source: CATT

Decision: 		The document was not treated.

R4-2503522	Study of RAN4 impacts for RRM measurement prediction for AIML Mobility in NR

Type: discussion		For: (not specified)
					Source: Apple

Decision: 		The document was not treated.

R4-2503582	Discussion of RAN4 impacts for RRM measurement prediction

Type: discussion		For: Discussion
					38.133 v	  CR-  rev  Cat:  (Rel-19)

					Source: LG Electronics Inc.

Decision: 		The document was not treated.

R4-2503607	Discussion on RAN4 impacts for RRM measurement prediction

Type: discussion		For: Discussion
					Source: MediaTek Inc.

Decision: 		The document was not treated.

R4-2503775	Discussion on RAN4 impacts for RRM measurement prediction

Type: discussion		For: Discussion
					Source: CMCC

Decision: 		The document was not treated.

R4-2503787	Discussions on RRM measurement impacts of AIML for mobility

Type: discussion		For: Discussion
					Source: NTT DOCOMO, INC.

Decision: 		The document was not treated.

R4-2503851	Discussion impacts on requirements of AI/ML mobility

Type: other		For: Approval
					Source: ZTE Corporation, Sanechips

Decision: 		The document was not treated.

R4-2503887	Discussion on impacts for RRM measurement prediction

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon

Decision: 		The document was not treated.

R4-2503929	On requirements for RRM measurement prediction for AI/ML based mobility

Type: other		For: Approval
					Source: Ericsson

Abstract:

This paper discusses impact of RRM measurement prediction for AI/ML based mobility on RAN4 requirements.

Decision: 		The document was not treated.

R4-2504047	Discussion on RAN4 impacts for RRM measurement prediction

Type: discussion		For: Discussion
					Source: vivo

Decision: 		The document was not treated.

R4-2504232	Study of RAN4 impacts for RRM measurement prediction

Type: discussion		For: Discussion
					Source: Nokia

Decision: 		The document was not treated.

R4-2504287	General discussion on AI mobility regarding RAN4 requirements impact

Type: discussion		For: Discussion
					38.133 v	  CR-  rev  Cat:  (Rel-19)

					Source: Samsung

Decision: 		The document was not treated.

R4-2504524	Impact of AI based mobility on RRM measurement prediction

Type: discussion		For: Discussion
					Source: Qualcomm Incorporated

Decision: 		The document was not treated.

##### 7.21.2.2	Study of RAN4 impacts for measurement event prediction

R4-2503309	Discussion on impacts on RAN4 requirement for measurement event prediction in AI mobility

Type: discussion		For: Discussion
					Source: Xiaomi

Decision: 		The document was not treated.

R4-2503352	Study of impacts on RAN4 requirements for measurement event prediction

Type: other		For: Approval
					Source: OPPO

Decision: 		The document was not treated.

R4-2503412	Discussion on impacts of AIML measurement event prediction on RRM requirements

Type: discussion		For: Discussion
					Source: CATT

Decision: 		The document was not treated.

R4-2503523	Study of RAN4 impacts for measurement event prediction for AIML Mobility in NR

Type: discussion		For: (not specified)
					Source: Apple

Decision: 		The document was not treated.

R4-2503608	Discussion on RAN4 impacts for measurement event prediction

Type: discussion		For: Discussion
					Source: MediaTek Inc.

Decision: 		The document was not treated.

R4-2503776	Discussion on RAN4 impacts for measurement event prediction

Type: discussion		For: Discussion
					Source: CMCC

Decision: 		The document was not treated.

R4-2503852	Discussion on impacts for measurement event prediction

Type: other		For: Approval
					Source: ZTE Corporation, Sanechips

Decision: 		The document was not treated.

R4-2503888	Discussion on impacts for measurement event prediction

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon

Decision: 		The document was not treated.

R4-2503930	On requirements for Event prediction for AI/ML based mobility

Type: other		For: Approval
					Source: Ericsson

Abstract:

This paper discusses impact of event prediction for AI/ML based mobility on RAN4 requirements.

Decision: 		The document was not treated.

R4-2504048	Discussion on RAN4 impacts for measurement event prediction

Type: discussion		For: Discussion
					Source: vivo

Decision: 		The document was not treated.

R4-2504233	Study of RAN4 impacts for measurement event prediction

Type: discussion		For: Discussion
					Source: Nokia

Decision: 		The document was not treated.

R4-2504525	Impact of AI based mobility on RRM event prediction

Type: discussion		For: Discussion
					Source: Qualcomm Incorporated

Decision: 		The document was not treated.

#### 7.21.3	Study of testability and interoperability

R4-2503300	Discussion on study of testability and interoperability for AIML mobility

Type: discussion		For: Discussion
					Source: Xiaomi

Decision: 		The document was not treated.

R4-2503353	Study of testability and interoperability for AI mobility

Type: other		For: Approval
					Source: OPPO

Decision: 		The document was not treated.

R4-2503413	Discussion on testability and interoperability issues for AIML mobility

Type: discussion		For: Discussion
					Source: CATT

Decision: 		The document was not treated.

R4-2503520	On Testability and Interoperability Issues for AIML Mobility in NR

Type: discussion		For: (not specified)
					Source: Apple

Decision: 		The document was not treated.

R4-2503609	Discussion on testability and interoperability of AI mobility

Type: discussion		For: Discussion
					Source: MediaTek Inc.

Decision: 		The document was not treated.

R4-2503777	Discussion on testability and interoperability for AI/ML for mobility

Type: discussion		For: Discussion
					Source: CMCC

Decision: 		The document was not treated.

R4-2503853	Discussion on the interoperability and testability aspects

Type: other		For: Approval
					Source: ZTE Corporation, Sanechips

Decision: 		The document was not treated.

R4-2503889	Discussion on testability and interoperability issues in AIML mobility

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon

Decision: 		The document was not treated.

R4-2503931	On testing of AI/ML based mobility

Type: other		For: Approval
					Source: Ericsson

Abstract:

This paper discusses issues related to the testing of AI/ML based predictions for mobility purposes.

Decision: 		The document was not treated.

R4-2504049	Discussion on testability and interoperability for AI mobility

Type: discussion		For: Discussion
					Source: vivo

Decision: 		The document was not treated.

R4-2504234	Study of testability and interoperability on AI/ML Mobility

Type: discussion		For: Discussion
					Source: Nokia

Decision: 		The document was not treated.

R4-2504286	General discussion on AI mobility regarding testability and interoperability

Type: discussion		For: Discussion
					38.133 v	  CR-  rev  Cat:  (Rel-19)

					Source: Samsung

Decision: 		The document was not treated.

R4-2504526	Testability and data consistency

Type: discussion		For: Discussion
					Source: Qualcomm Incorporated

Decision: 		The document was not treated.

#### 7.21.4	Moderator summary and conclusions

R4-2503620	Topic summary for [114bis][210] FS_NR_AIML_Mob

Type: other		For: Information
					Source: Moderator (Nokia)

Abstract:

Topic summary in RRM session

Decision: 		The document was not treated.

### 7.22	NR MIMO Phase 5

#### 7.22.1	General aspects

R4-2504169	R19 on power class for 3T

Type: other		For: Approval
					Source: OPPO

Decision: 		The document was not treated.

#### 7.22.2	UE RF requirements

R4-2503172	On the UE RF requirement for the FR2 asymmetric connection scenario

Type: other		For: Approval
					Source: Qualcomm, Sony, Nokia

Abstract:

Discussion paper of UE RF proposal

Decision: 		The document was not treated.

R4-2503173	dCR on RF requirements for a UE with different TCI states for UL and DL

Type: draftCR		For: Endorsement
					38.101-2 v19.0.0	  CR-  rev  Cat:  (Rel-19)

					Source: Qualcomm, Sony, Nokia

Abstract:

Proposal for the UE RF requirement for the asymmetric FR2 connection feature in CR form

Decision: 		The document was not treated.

R4-2503510	3Tx NR specifications

Type: other		For: Approval
					38.101-1 v	  CR-  rev  Cat:  (Rel-19)

					Source: Qualcomm Technologies

Abstract:

In this paper we present our views on the remaining work that needs to be done to introduce 3 Tx into the NR specifications

Decision: 		The document was not treated.

R4-2503844	Further discussion of UE RF impact for NR MIMO phase 5

Type: other		For: Approval
					Source: vivo

Decision: 		The document was not treated.

R4-2504090	On RF requirements for Rel-19 NR-MIMO

Type: other		For: Approval
					Source: Huawei, HiSilicon

Decision: 		The document was not treated.

R4-2504168	R19 MPR for 3Tx

Type: other		For: Approval
					Source: OPPO

Decision: 		The document was not treated.

R4-2504390	UE RF requirements of NR MIMO Phase 5

Type: discussion		For: Discussion
					Source: Samsung

Decision: 		The document was not treated.

R4-2504581	On the UE RF requirements for NR MIMO Phase 5

Type: other		For: Approval
					Source: Ericsson

Abstract:

Proposals on the open issues related to "3 Tx" and "Asymmetric DL sTRP/UL mTRP" topics.

Decision: 		The document was not treated.

#### 7.22.3	RRM core requirements

##### 7.22.3.1	Enhancement for UE-initiated/event-driven beam management

R4-2503127	Views for enhanced UE-initiated/event-driven beam management

Type: discussion		For: Discussion
					Source: Qualcomm Technologies Ireland

Decision: 		The document was not treated.

R4-2503312	Discussion on UE-initiated/event-driven beam management  on Rel-19 MIMO phase 5

Type: discussion		For: Discussion
					Source: Xiaomi

Decision: 		The document was not treated.

R4-2503546	On RRM Requirements for UE initiated beam management

Type: discussion		For: Discussion
					Source: Apple

Decision: 		The document was not treated.

R4-2503598	Discussion on RRM requirements of UE-initiated/event-driven beam management

Type: discussion		For: Discussion
					Source: LG Electronics

Decision: 		The document was not treated.

R4-2503665	Discussion on RRM requirements of UE-initiated beam management in Rel-19 MIMO phase 5

Type: discussion		For: Discussion
					Source: Samsung

Decision: 		The document was not treated.

R4-2503755	Discussion on RRM requirements for event-driven beam management for NR MIMO Phase 5

Type: discussion		For: Discussion
					Source: CMCC

Decision: 		The document was not treated.

R4-2503869	Discussion on RRM impact for Rel-19 MIMO on UE initiated BM

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon

Decision: 		The document was not treated.

R4-2503934	Discussion on UE-initiated/event-driven beam management

Type: discussion		For: Discussion
					Source: Ericsson

Decision: 		The document was not treated.

R4-2504073	On NR MIMO Phase 5 UEIBM requirements

Type: discussion		For: Discussion
					Source: Nokia

Decision: 		The document was not treated.

R4-2504074	DraftCR for UEIBM reporting requirements

Type: draftCR		For: Endorsement
					38.133 v19.0.0	  CR-  rev  Cat: B (Rel-19)

					Source: Nokia

Decision: 		The document was not treated.

R4-2504248	Discussion on RRM aspects of UE-initiated beam management of R19 NR MIMO Phase 5

Type: discussion		For: Approval
					Source: ZTE Corporation, Sanechips

Decision: 		The document was not treated.

R4-2504602	Discussion on R19 MIMO for UE-initiated/event-driven beam management

Type: discussion		For: Discussion
					Source: MediaTek inc.

Decision: 		The document was not treated.

##### 7.22.3.2	Other RRM requirements

R4-2503128	Views on additional RRM requirements for NR MIMO Phase 5

Type: discussion		For: Discussion
					Source: Qualcomm Technologies Ireland

Decision: 		The document was not treated.

R4-2503313	Discussion on other RRM requirement on Rel-19 MIMO phase 5

Type: discussion		For: Discussion
					Source: Xiaomi

Decision: 		The document was not treated.

R4-2503547	On RRM Requirements for MIMO Phase 5

Type: discussion		For: Discussion
					Source: Apple

Decision: 		The document was not treated.

R4-2503666	Discussion on RRM requirements of other aspects in Rel-19 MIMO phase 5

Type: discussion		For: Discussion
					Source: Samsung

Decision: 		The document was not treated.

R4-2503756	Discussion on other RRM requirements for NR MIMO Phase 5

Type: discussion		For: Discussion
					Source: CMCC

Decision: 		The document was not treated.

R4-2503876	Discussion on RRM impact for Rel-19 MIMO on other aspect

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon

Decision: 		The document was not treated.

R4-2503877	Discussion on simulation results of CJT calibration

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon

Decision: 		The document was not treated.

R4-2503935	Discussion on CJTC reporting and asymmteric UL TRP requirements

Type: discussion		For: Discussion
					Source: Ericsson

Decision: 		The document was not treated.

R4-2504249	Discussion on other RRM aspects of R19 NR MIMO Phase 5

Type: discussion		For: Approval
					Source: ZTE Corporation, Sanechips

Decision: 		The document was not treated.

R4-2504501	Other RRM requirements for MIMO ph5

Type: discussion		For: Discussion
					Source: Nokia

Decision: 		The document was not treated.

R4-2504502	Draft CR for UL only TRP TCI state switching

Type: draftCR		For: Endorsement
					38.133 v19.0.0	  CR-  rev  Cat:  (Rel-19)

					Source: Nokia

Decision: 		The document was not treated.

R4-2504503	Simulation results for CJT RRM requirements

Type: other		For: Discussion
					Source: Nokia

Decision: 		The document was not treated.

R4-2504603	Discussion on R19 MIMO for other RRM requirements

Type: discussion		For: Discussion
					Source: MediaTek inc.

Decision: 		The document was not treated.

#### 7.22.4	Moderator summary and conclusions

R4-2503621	Topic summary for [114bis][211] NR_MIMO_Ph5_Part1

Type: other		For: Information
					Source: Moderator (Samsung)

Abstract:

Topic summary in RRM session

Decision: 		The document was not treated.

R4-2503622	Topic summary for [114bis][212] NR_MIMO_Ph5_Part2

Type: other		For: Information
					Source: Moderator (MediaTek)

Abstract:

Topic summary in RRM session

Decision: 		The document was not treated.

R4-2504686	Topic summary for [114bis][128] NR_MIMO_Ph5_UE

Type: other		For: Information
					Source: Moderator(Samsung)

Abstract:

Summary for AI 7.22.1, 7.22.2

Decision: 		The document was not treated.

### 7.23	Evolution of NR duplex operation: Sub-band full duplex (SBFD)

#### 7.23.1	General aspects (including RAN4 aspects for SBFD system parameters)

R4-2503235	Fair coexistence between SBFD and TDD networks

Type: discussion		For: Agreement
					Source: Charter Communications, Inc

Abstract:

In Rel 18 study, SBFD performance was evaluated, and it was concluded in TR 38.858 [1] that SBFD provides benefits of reduced latency and increased UL throughput over legacy TDD. It was concluded in the study phase that SBFD may cause UE-to-UE interferenc

Decision: 		The document was not treated.

R4-2503285	Discussion on SBFD General aspects

Type: discussion		For: (not specified)
					Source: Tejas Network Limited

Decision: 		The document was not treated.

R4-2503405	Discussion on SBFD general part

Type: discussion		For: Discussion
					Source: CMCC

Decision: 		The document was not treated.

R4-2503466	Discussion on SBFD general issues

Type: discussion		For: Approval
					Source: CATT

Decision: 		The document was not treated.

R4-2503823	Discussion on the subband configurations and guardbands for gNB SBFD

Type: other		For: Approval
					Source: vivo

Decision: 		The document was not treated.

R4-2504007	SBFD general aspects

Type: other		For: Approval
					Source: Ericsson

Abstract:

Discussion on SBFD general aspects

Decision: 		The document was not treated.

R4-2504011	Draft CR to 38.104 for introduction of SBFD

Type: draftCR		For: Endorsement
					38.104 v19.0.0	  CR-  rev  Cat: B (Rel-19)

					Source: Ericsson

Abstract:

This draft CR introduce the new feature SBFD to TS 38.104

Decision: 		The document was not treated.

R4-2504043	Discussion on MPR impact on SBFD UL resource muting

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

MPR impact on SBFD UL resource muting

Decision: 		The document was not treated.

R4-2504091	On general aspects for SBFD

Type: other		For: Approval
					Source: Huawei, HiSilicon

Decision: 		The document was not treated.

R4-2504193	On SBFD system parameters

Type: other		For: Approval
					Source: Nokia

Decision: 		The document was not treated.

R4-2504218	Further discussion on general aspects for SBFD system

Type: other		For: Approval
					Source: Samsung

Decision: 		The document was not treated.

R4-2504339	Views on general aspects for SBFD

Type: discussion		For: Approval
					Source: Qualcomm Germany

Decision: 		The document was not treated.

R4-2504368	Discussion on system parameters for SBFD BS

Type: other		For: Approval
					Source: ZTE Corporation, Sanechips

Decision: 		The document was not treated.

R4-2504455	RF impact due to UL resource muting

Type: discussion		For: Discussion
					Source: Apple

Decision: 		The document was not treated.

R4-2504483	Cross Link Interference (CLI) handling impact on MPR for SBFD operation

Type: discussion		For: Discussion
					Source: Qualcomm Incorporated

Abstract:

Proposal for discussion on CLI handling impact on MPR in SBFD operation

Decision: 		The document was not treated.

#### 7.23.2	BS RF requirements

R4-2504300	Deployment scenarios for FR1 SBFD macro BS

Type: other		For: Approval
					Source: Huawei, HiSilicon, Samsung, Qualcomm, Verizon, Vodafone, China Telecom, China Unicom

Abstract:

MCC: Treat in email thread [306].

Decision: 		The document was not treated.

##### 7.23.2.1	Potentially new requirements for SBFD operation for FR1 and FR2-1

R4-2503286	Discussion on Potentially new requirements for SBFD operation

Type: discussion		For: (not specified)
					Source: Tejas Network Limited

Decision: 		The document was not treated.

R4-2503404	Discussion on new RF requirement for SBFD

Type: discussion		For: Discussion
					Source: CMCC

Decision: 		The document was not treated.

R4-2503469	Discussion on potentially new requirements for SBFD operation

Type: discussion		For: Discussion
					Source: CATT

Decision: 		The document was not treated.

R4-2504008	New SBFD BS RF requirements

Type: other		For: Approval
					Source: Ericsson

Abstract:

Discussion on potentially new requirements for SBFD

Decision: 		The document was not treated.

R4-2504092	On potentially new requirements for SBFD

Type: other		For: Approval
					Source: Huawei, HiSilicon

Decision: 		The document was not treated.

R4-2504194	On potentially new BS RF requirements for SBFD operation

Type: other		For: Approval
					Source: Nokia

Decision: 		The document was not treated.

R4-2504219	On the new requirements for SBFD operation for FR1 and FR2-1

Type: other		For: Approval
					Source: Samsung

Decision: 		The document was not treated.

R4-2504341	Views on new BS RF requirements for SBFD

Type: discussion		For: Approval
					Source: Qualcomm Germany

Decision: 		The document was not treated.

R4-2504369	Discussion on potentially new requirements for SBFD operation

Type: other		For: Approval
					Source: ZTE Corporation, Sanechips

Decision: 		The document was not treated.

##### 7.23.2.2	Modification of existing Tx requirements for FR1 and FR2-1

R4-2503287	Discussion on existing Tx requirements for SBFD operation

Type: discussion		For: (not specified)
					Source: Tejas Network Limited

Decision: 		The document was not treated.

R4-2503403	Discussion on existing Tx requirements for SBFD

Type: discussion		For: Discussion
					Source: CMCC

Decision: 		The document was not treated.

R4-2503467	Discussion on modification of existing Tx requirements for SBFD operation

Type: discussion		For: Discussion
					Source: CATT

Decision: 		The document was not treated.

R4-2504009	Impact on SBFD BS RF TX requirements

Type: other		For: Approval
					Source: Ericsson

Abstract:

Discussion on impact on existing BS RF TX requirements for SBFD

Decision: 		The document was not treated.

R4-2504195	On existing BS RF TX requirements for SBFD operation

Type: other		For: Approval
					Source: Nokia

Decision: 		The document was not treated.

R4-2504220	On the modification of existing TX requirements for SBFD-capable BS

Type: other		For: Approval
					Source: Samsung

Decision: 		The document was not treated.

R4-2504301	On modification of existing TX RF requirements for SBFD

Type: other		For: Approval
					Source: Huawei, HiSilicon

Decision: 		The document was not treated.

R4-2504340	Views on existing BS RF requirements for SBFD

Type: discussion		For: Approval
					Source: Qualcomm Germany

Decision: 		The document was not treated.

R4-2504370	Discussion on modification of existing Tx requirements for FR1 and FR2-1 for SBFD BS

Type: other		For: Approval
					Source: ZTE Corporation, Sanechips

Decision: 		The document was not treated.

##### 7.23.2.3	Modification of existing Rx requirements for FR1 and FR2-1

R4-2503299	Views on modification of existing Rx requirements for SBFD

Type: discussion		For: (not specified)
					Source: Tejas Network Limited

Decision: 		The document was not treated.

R4-2503402	Discussion on existing Rx requirements for SBFD

Type: discussion		For: Discussion
					Source: CMCC

Decision: 		The document was not treated.

R4-2503468	Discussion on modification of existing Rx requirements for SBFD operation

Type: discussion		For: Discussion
					Source: CATT

Decision: 		The document was not treated.

R4-2504010	Impact on SBFD BS RF RX requirements

Type: other		For: Approval
					Source: Ericsson

Abstract:

Discussion on impact on existing BS RF RX requirements for SBFD

Decision: 		The document was not treated.

R4-2504196	On existing BS RF RX requirements for SBFD operation

Type: other		For: Approval
					Source: Nokia

Decision: 		The document was not treated.

R4-2504221	On the modification of existing RX requirements for SBFD-capable BS

Type: other		For: Approval
					Source: Samsung

Decision: 		The document was not treated.

R4-2504302	On modification of existing RX RF requirements for SBFD

Type: other		For: Approval
					Source: Huawei, HiSilicon

Decision: 		The document was not treated.

R4-2504371	Discussion on modification of existing Rx requirements for FR1 and FR2-1 for SBFD BS

Type: other		For: Approval
					Source: ZTE Corporation, Sanechips

Decision: 		The document was not treated.

#### 7.23.3	RRM core requirements

##### 7.23.3.1	RRM requirements for UE-to-UE CLI handling

R4-2503290	On RRM requirements for UE-to-UE CLI handling

Type: discussion		For: Discussion
					Source: vivo

Decision: 		The document was not treated.

R4-2503341	On RRM requirements for UE-to-UE CLI handling

Type: other		For: Approval
					Source: OPPO

Decision: 		The document was not treated.

R4-2503414	Discussion on UE to UE CLI handling

Type: discussion		For: Discussion
					Source: CATT

Decision: 		The document was not treated.

R4-2503500	RRM requirements for UE-to-UE CLI handling

Type: discussion		For: Discussion
					38.133 v	  CR-  rev  Cat:  (Rel-19)

					Source: Nokia, Nokia Shanghai Bell

Decision: 		The document was not treated.

R4-2503502	Simulation result for L1-CLI-SRS-RSRP measurement

Type: discussion		For: Discussion
					38.133 v	  CR-  rev  Cat:  (Rel-19)

					Source: Nokia, Nokia Shanghai Bell

Decision: 		The document was not treated.

R4-2503578	Discussion on RRM impacts of UE-to-UE CLI handling

Type: discussion		For: Discussion
					38.133 v	  CR-  rev  Cat:  (Rel-19)

					Source: LG Electronics Inc.

Decision: 		The document was not treated.

R4-2503667	Views on L1 CLI measurement requirements

Type: discussion		For: Discussion
					Source: Samsung

Decision: 		The document was not treated.

R4-2503750	Discussion for RRM requirements for SBFD UE to UE CLI handling

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

This contribuion disucss the SBFD UE-UE CLI remaining issues

Decision: 		The document was not treated.

R4-2503762	Discussion on RRM requirements for UE-to-UE CLI handling

Type: discussion		For: Discussion
					Source: CMCC

Decision: 		The document was not treated.

R4-2503901	Discussion on UE CLI measurement for SBFD

Type: LS out		For: Approval
					to RAN1
					Source: Huawei, HiSilicon

Decision: 		The document was not treated.

R4-2503902	Additional simulation results for UE CLI measurement

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon

Decision: 		The document was not treated.

R4-2503915	Discussion on RRM requirements for UE-to-UE CLI handling for evolution of NR duplex operation

Type: discussion		For: Discussion
					Source: China Telecom

Decision: 		The document was not treated.

R4-2504250	Discussion on UE-to-UE CLI of R19 SBFD

Type: discussion		For: Approval
					Source: ZTE Corporation, Sanechips

Decision: 		The document was not treated.

R4-2504454	On UE RRM requirements for UE-to-UE CLI handling

Type: discussion		For: Discussion
					Source: Apple

Decision: 		The document was not treated.

R4-2504479	Discussion on SBFD UE-to-UE CLI

Type: discussion		For: Discussion
					38.133 v	  CR-  rev  Cat:  (Rel-19)

					Source: MediaTek inc.

Decision: 		The document was not treated.

R4-2504608	RRM requirements for UE-to-UE CLI handling

Type: discussion		For: Discussion
					Source: Qualcomm Incorporated

Decision: 		The document was not treated.

##### 7.23.3.2	RRM impacts for SBFD operation

R4-2503291	Further consideration on RRM impacts for SBFD operation

Type: discussion		For: Discussion
					Source: vivo

Decision: 		The document was not treated.

R4-2503342	On RRM impacts for SBFD operation

Type: other		For: Approval
					Source: OPPO

Decision: 		The document was not treated.

R4-2503415	Discussion on impacts of SBFD on RRM requirements

Type: discussion		For: Discussion
					Source: CATT

Decision: 		The document was not treated.

R4-2503501	RRM requirements for SBFD operation

Type: discussion		For: Discussion
					38.133 v	  CR-  rev  Cat:  (Rel-19)

					Source: Nokia, Nokia Shanghai Bell

Decision: 		The document was not treated.

R4-2503579	Discussion on RRM impacts of SBFD operation

Type: discussion		For: Discussion
					38.133 v	  CR-  rev  Cat:  (Rel-19)

					Source: LG Electronics Inc.

Decision: 		The document was not treated.

R4-2503668	Views on RRM requirements for Rel-19 SBFD operation

Type: discussion		For: Discussion
					Source: Samsung

Decision: 		The document was not treated.

R4-2503751	Discussion on RRM impacts for SBFD operations

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

This contribution discuss the RRM requirement impacts for SBFD operations

Decision: 		The document was not treated.

R4-2503761	Discussion on RRM impacts for SBFD operation

Type: discussion		For: Discussion
					Source: CMCC

Decision: 		The document was not treated.

R4-2503903	Discussion on RRM impacts for SBFD operation

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon

Decision: 		The document was not treated.

R4-2503916	Discussion on RRM impacts for SBFD operation for evolution of NR duplex operation

Type: discussion		For: Discussion
					Source: China Telecom

Decision: 		The document was not treated.

R4-2504251	Discussion on other RRM impacts of R19 SBFD

Type: discussion		For: Approval
					Source: ZTE Corporation, Sanechips

Decision: 		The document was not treated.

R4-2504480	Discussion on SBFD Operation

Type: discussion		For: Discussion
					38.133 v	  CR-  rev  Cat:  (Rel-19)

					Source: MediaTek inc.

Decision: 		The document was not treated.

R4-2504607	RRM requirements for SBFD operation

Type: discussion		For: Discussion
					Source: Qualcomm Incorporated

Decision: 		The document was not treated.

#### 7.23.4	Moderator summary and conclusions

R4-2503623	Topic summary for [114bis][213] NR_duplex_evo

Type: other		For: Information
					Source: Moderator (Huawei)

Abstract:

Topic summary in RRM session

Decision: 		The document was not treated.

R4-2504631	Topic summary for [114bis][306] NR_duplex_evo_General

Type: other		For: Information
					Source: Moderator (Samsung)

Abstract:

[114bis] BDaT Session AI 7.23.1, 7.23.2.1

Decision: 		The document was not treated.

R4-2504632	Topic summary for [114bis][307] NR_duplex_evo_BSRF

Type: other		For: Information
					Source: Moderator (Huawei)

Abstract:

[114bis] BDaT Session AI 7.23.2.2, 7.23.2.3

Decision: 		The document was not treated.

### 7.24	Solutions for Ambient IoT (Internet of Things) in NR

#### 7.24.1	General aspects

R4-2503216	A-IoT general aspects

Type: other		For: Approval
					38.194 v	  CR-  rev  Cat:  (Rel-19)

					Source: Huawei, HiSilicon

Decision: 		The document was not treated.

R4-2503314	Discussion on AIoT system parameters

Type: discussion		For: Discussion
					Source: Xiaomi

Decision: 		The document was not treated.

R4-2503392	Discussion on A-IoT general requirements

Type: discussion		For: Discussion
					Source: CMCC

Decision: 		The document was not treated.

R4-2503472	Discussion on system parameters for A-IoT

Type: discussion		For: Discussion
					Source: CATT

Decision: 		The document was not treated.

R4-2503673	Discussion on system parameters for ambient IoT

Type: discussion		For: Discussion
					Source: Spreadtrum,UNISOC

Decision: 		The document was not treated.

R4-2503813	Discussion on the systeme parameter of AIoT

Type: other		For: Approval
					Source: vivo

Decision: 		The document was not treated.

R4-2503932	(new) TS 38.191 Ambient IoT device radio transmission and reception

Type: draft TS		For: Agreement
					38.191 v0.0.1	  CR-  rev  Cat:  (Rel-19)

					Source: CMCC

Abstract:

TS skeleton of TS 38.191, used for online discussion. MCC reserved on  behalf of Chunxia Guo, CMCC.

Decision: 		The document was not treated.

R4-2504069	3GPP_TS 38.194 skeleton

Type: other		For: Endorsement
					38.194 v	  CR-  rev  Cat:  (Rel-19)

					Source: Huawei, HiSilicon

Decision: 		The document was withdrawn.

R4-2504096	3GPP_TS 38.194 skeleton

Type: draft TS		For: Agreement
					38.194 v0.0.1	  CR-  rev  Cat:  (Rel-19)

					Source: Huawei, HiSilicon

Decision: 		The document was not treated.

R4-2504207	on system parameter

Type: other		For: Approval
					Source: OPPO

Decision: 		The document was not treated.

R4-2504352	Discussions on General aspect for A-IoT

Type: other		For: Approval
					Source: ZTE Corporation, Sanechips

Decision: 		The document was not treated.

R4-2504399	A-IoT general overview

Type: other		For: Approval
					Source: Ericsson

Abstract:

In this paper, we present our thoughts on general aspects for A-IoT WI [1].

Decision: 		The document was not treated.

#### 7.24.2	RF requirements for A-IoT

##### 7.24.2.1	RF requirements for Type 1-C Ambient-IoT BS

R4-2503217	RF requirements for A-IoT BS

Type: other		For: Approval
					38.194 v	  CR-  rev  Cat:  (Rel-19)

					Source: Huawei, HiSilicon

Decision: 		The document was not treated.

R4-2503315	Initial D2R LLS simulation result

Type: discussion		For: Discussion
					Source: Xiaomi

Decision: 		The document was not treated.

R4-2503390	Discussion on A-IoT BS RF requirements

Type: discussion		For: Discussion
					Source: CMCC

Decision: 		The document was not treated.

R4-2503470	Discussion on A-IoT BS requirements

Type: discussion		For: Discussion
					Source: CATT

Decision: 		The document was not treated.

R4-2503560	Relevance of phase noise and LLS for SNR for REFSENS

Type: other		For: Approval
					Source: Qualcomm Incorporated

Abstract:

Vote for Gene!

Decision: 		The document was not treated.

R4-2503814	Discussion on the RF requirement for AIoT BS

Type: other		For: Approval
					Source: vivo

Decision: 		The document was not treated.

R4-2504353	Discussions on RF requirements for A-IoT BS

Type: other		For: Approval
					Source: ZTE Corporation, Sanechips

Decision: 		The document was not treated.

R4-2504398	A-IoT BS RF impact overview

Type: other		For: Approval
					Source: Ericsson

Abstract:

In this paper, we present our overview for A-IoT BS RF requirements.

Decision: 		The document was not treated.

##### 7.24.2.2	RF requirements for device 1

R4-2503316	Discussion on AIoT device 1 RF requirement

Type: discussion		For: Discussion
					Source: Xiaomi

Decision: 		The document was not treated.

R4-2503391	Discussion on A-IoT device RF requirements

Type: discussion		For: Discussion
					Source: CMCC

Decision: 		The document was not treated.

R4-2503438	Discussion on Ambient IoT device 1 RF requirements

Type: other		For: Approval
					Source: CATT

Decision: 		The document was not treated.

R4-2503559	Ambient IoT device backscattering and SFO error

Type: other		For: Approval
					Source: Qualcomm Incorporated

Abstract:

Vote for Gene!

Decision: 		The document was not treated.

R4-2503671	Discussion on RF requirements for ambient IoT device 1

Type: discussion		For: Discussion
					Source: Spreadtrum,UNISOC

Decision: 		The document was not treated.

R4-2503815	Discussion on the RF requirement for device 1

Type: other		For: Approval
					Source: vivo

Decision: 		The document was not treated.

R4-2504063	RF requirements of ambient IoT device 1

Type: discussion		For: Approval
					Source: Sony

Decision: 		The document was not treated.

R4-2504206	on RF requirements for device 1

Type: other		For: Approval
					Source: OPPO

Decision: 		The document was not treated.

R4-2504354	Discussion on RF requirement of Ambient IoT device

Type: other		For: Approval
					Source: ZTE Corporation, Sanechips

Decision: 		The document was not treated.

R4-2504377	Discussion on RF requirement for A-IoT device 1

Type: discussion		For: Discussion
					Source: LG Electronics UK

Decision: 		The document was withdrawn.

R4-2504400	A-IoT device requirement overview

Type: other		For: Approval
					Source: Ericsson

Abstract:

In this paper, we present our view on the A-IoT device RF requirements

Decision: 		The document was not treated.

R4-2504516	Discussion on RF requirement for A-IoT device 1

Type: discussion		For: Discussion
					Source: LG Electronics UK

Decision: 		The document was not treated.

R4-2504557	On the RF requirements for Ambient IoT Device

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon

Decision: 		The document was not treated.

##### 7.24.2.3	RF requirements for CW

R4-2503317	Discussion on AIoT CW requirements

Type: discussion		For: Discussion
					Source: Xiaomi

Decision: 		The document was not treated.

R4-2503389	Discussion on A-IoT CW RF requirements

Type: discussion		For: Discussion
					Source: CMCC

Decision: 		The document was not treated.

R4-2503471	Discussion on RF requirements for CW for D1T1

Type: discussion		For: Discussion
					Source: CATT

Decision: 		The document was not treated.

R4-2503672	Discussion on RF requirements for CW

Type: discussion		For: Discussion
					Source: Spreadtrum,UNISOC

Decision: 		The document was not treated.

R4-2503816	Discussion on the RF requirement for CW

Type: other		For: Approval
					Source: vivo

Decision: 		The document was not treated.

R4-2504205	on RF requirements for CW

Type: other		For: Approval
					Source: OPPO

Decision: 		The document was not treated.

R4-2504303	RF requirements for CW

Type: other		For: Approval
					Source: Huawei, HiSilicon

Decision: 		The document was not treated.

R4-2504355	Discussion on RF requirement for CW node

Type: other		For: Approval
					Source: ZTE Corporation, Sanechips

Decision: 		The document was not treated.

R4-2504401	CW node RF impact overview

Type: other		For: Approval
					Source: Ericsson

Abstract:

In this paper, we present our overview for RF requirements for a CW node.

Decision: 		The document was not treated.

R4-2504409	Discussion on RF requirement for CW

Type: discussion		For: Discussion
					Source: LG Electronics UK

Decision: 		The document was withdrawn.

R4-2504517	Discussion on RF requirement for CW

Type: discussion		For: Discussion
					Source: LG Electronics UK

Decision: 		The document was not treated.

#### 7.24.3	RRM core requirements

R4-2503292	Initial discussion on RRM impacts for R19 A-IoT device

Type: discussion		For: Discussion
					Source: vivo

Decision: 		The document was not treated.

R4-2503422	Discussion on the impacts of A-IoT on RRM requirements

Type: discussion		For: Discussion
					Source: CATT

Decision: 		The document was not treated.

R4-2503778	Discussion on RRM requirements for A-IoT

Type: discussion		For: Discussion
					Source: CMCC

Decision: 		The document was not treated.

R4-2503788	Discussions on RRM requirements for A-IoT

Type: discussion		For: Discussion
					Source: NTT DOCOMO, INC.

Decision: 		The document was not treated.

R4-2503791	Discussion on RRM requirements for Ambient-IoT

Type: other		For: Approval
					Source: ZTECorporation,Sanechips

Decision: 		The document was not treated.

R4-2503873	Discussion on RRM impacts for R19 A-IoT device

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon

Decision: 		The document was not treated.

R4-2503954	Discussion on RRM requirements for ambient IoT device 1

Type: discussion		For: Discussion
					Source: Spreadtrum,UNISOC

Decision: 		The document was not treated.

R4-2504108	Initial discussion on RRM requirements for Ambient-IoT

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

Initial discussion on RRM requirements for Ambient-IoT

Decision: 		The document was not treated.

#### 7.24.4	OTA test method for A-IoT device 1

R4-2503138	Discussion on OTA testing for A-IoT device 1

Type: other		For: Approval
					Source: Ericsson-LG Co., LTD

Decision: 		The document was not treated.

R4-2503393	Discussion on A-IoT OTA requirements

Type: discussion		For: Discussion
					Source: CMCC

Decision: 		The document was not treated.

R4-2503494	on OTA tests for ambient IoT devices

Type: other		For: Approval
					Source: Huawei, HiSilicon

Abstract:

A-IoT OTA test discussion

Decision: 		The document was not treated.

R4-2503817	Discussion on the OTA test method for device

Type: other		For: Approval
					Source: vivo

Decision: 		The document was not treated.

R4-2504056	Views on Ambient IoT Testability

Type: other		For: Approval
					Source: Qualcomm Incorporated

Decision: 		The document was not treated.

R4-2504064	Consideration on the OTA test of ambient IoT device 1

Type: discussion		For: Approval
					Source: Sony

Decision: 		The document was not treated.

R4-2504151	Discussion on OTA test method for A-IoT device

Type: other		For: Approval
					Source: CAICT

Decision: 		The document was not treated.

R4-2504204	on OTA test aspect

Type: other		For: Approval
					Source: OPPO

Decision: 		The document was not treated.

#### 7.24.5	Moderator summary and conclusions

R4-2503624	Topic summary for [114bis][214] Ambient_IoT_Solutions

Type: other		For: Information
					Source: Moderator (CMCC)

Abstract:

Topic summary in RRM session

Decision: 		The document was not treated.

R4-2504687	Topic summary for [114bis][129] A-IoT_device

Type: other		For: Information
					Source: Moderator(CMCC)

Abstract:

Summary for AI 7.24.1, 7.24.2.2, 7.24.4

Decision: 		The document was not treated.

R4-2504688	Topic summary for [114bis][130] A-IoT_BSCW

Type: other		For: Information
					Source: Moderator(Huawei)

Abstract:

Summary for AI 7.24.2.1, 7.24.2.3

Decision: 		The document was not treated.

### 7.25	Enhancements of network energy savings for NR

#### 7.25.1	General aspects

R4-2504225	Discussion on OD-SSB general part in Rel-19 NES

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

This contribution discusses the general work scope and CR split

Decision: 		The document was not treated.

#### 7.25.2	RRM core requirements

##### 7.25.2.1	On-demand SSB SCell operation

R4-2503124	Views on on-demand SSB SCell operation

Type: discussion		For: Discussion
					Source: Qualcomm Technologies Ireland

Decision: 		The document was not treated.

R4-2503130	Discussion on RRM requirements for OD-SSB for network energy saving

Type: discussion		For: Discussion
					Source: MediaTek inc.

Decision: 		The document was not treated.

R4-2503293	Discussion on RRM requirements for on-demand SSB SCell operation in R19 NES

Type: discussion		For: Discussion
					Source: vivo

Decision: 		The document was not treated.

R4-2503327	Discussion on OD-SSB requirements

Type: discussion		For: Discussion
					Source: Xiaomi

Decision: 		The document was not treated.

R4-2503343	On RRM requirements of on-demand SSB SCell operation

Type: other		For: Approval
					Source: OPPO

Decision: 		The document was not treated.

R4-2503380	Discussion on RRM requirement of OD-SSB for NES enhancement

Type: discussion		For: Discussion
					Source: CMCC

Decision: 		The document was not treated.

R4-2503429	Discussion on RRM requirements for on-demand SSB SCell operation

Type: discussion		For: Discussion
					Source: CATT

Decision: 		The document was not treated.

R4-2503499	RRM requirements for OD-SSB SCell operation

Type: discussion		For: Discussion
					38.133 v	  CR-  rev  Cat:  (Rel-19)

					Source: Nokia, Nokia Shanghai Bell

Decision: 		The document was not treated.

R4-2503569	Discussion on on-demand SSB SCell operation

Type: discussion		For: Discussion
					Source: Apple

Decision: 		The document was not treated.

R4-2503591	Discussion on RRM requirements of on-demand SSB Scell operation for Rel-19 NES

Type: discussion		For: Discussion
					Source: LG Electronics

Decision: 		The document was not treated.

R4-2503866	Discussion on RRM impact for Rel-19 NES OD-SSB

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon

Decision: 		The document was not treated.

R4-2503917	Discussion on On-demand SSB SCell operation for enhancements of network energy savings for NR

Type: discussion		For: Discussion
					Source: China Telecom

Decision: 		The document was not treated.

R4-2504019	Discussion on RRM requirements for On-demand SSB SCell operations

Type: discussion		For: Discussion
					Source: Intel Corporation

Decision: 		The document was not treated.

R4-2504226	Discussion on OD-SSB in Rel-19 NES

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

This contribution discusses the Rel-19 NES Obj.1 requirement

Decision: 		The document was not treated.

R4-2504246	Discussion on On-demand SSB SCell operation of R19 NES

Type: discussion		For: Approval
					Source: ZTE Corporation, Sanechips

Decision: 		The document was not treated.

R4-2504289	Discussion on RRM requirements for NES OD-SSB

Type: discussion		For: Discussion
					38.133 v	  CR-  rev  Cat:  (Rel-19)

					Source: Samsung

Decision: 		The document was not treated.

##### 7.25.2.2	Adaptation of common signal/channel transmission

R4-2503125	About RRM requirements for adaptation of common signal/channel transmission

Type: discussion		For: Discussion
					Source: Qualcomm Technologies Ireland

Decision: 		The document was not treated.

R4-2503131	Discussion on RRM requirements for adaptation of common signal/channel transmission

Type: discussion		For: Discussion
					Source: MediaTek inc.

Decision: 		The document was not treated.

R4-2503294	On RRM requirements for common signal or channel adaptation in R19 NES

Type: discussion		For: Discussion
					Source: vivo

Decision: 		The document was not treated.

R4-2503328	Discussion on RRM requirements for adaptation of common signal/channel in NES

Type: discussion		For: Discussion
					Source: Xiaomi

Decision: 		The document was not treated.

R4-2503344	On RRM requirements of SSB adaptation

Type: other		For: Approval
					Source: OPPO

Decision: 		The document was not treated.

R4-2503381	Discussion on RRM requirement of signal adapatation for NES enhancement

Type: discussion		For: Discussion
					Source: CMCC

Decision: 		The document was not treated.

R4-2503430	Discussion on RRM requirements for adaptation of common signalchannel transmission

Type: discussion		For: Discussion
					Source: CATT

Decision: 		The document was not treated.

R4-2503570	Discussion on adaptation of common signal/channel transmission

Type: discussion		For: Discussion
					Source: Apple

Decision: 		The document was not treated.

R4-2503594	Discussion on RRM requirements of SSB adaptation for Rel-19 NES

Type: discussion		For: Discussion
					Source: LG Electronics

Decision: 		The document was not treated.

R4-2503867	Discussion on RRM impact for Rel-19 NES on adaptation

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon

Decision: 		The document was not treated.

R4-2503918	Discussion on adaptation of common signal/channel transmission for enhancements of network energy savings for NR

Type: discussion		For: Discussion
					Source: China Telecom

Decision: 		The document was not treated.

R4-2504227	Discussion on RS and channel adaptation in Rel-19 NES

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

This contribution discusses the Rel-19 NES Obj.3 requirement

Decision: 		The document was not treated.

R4-2504247	Discussion on adaptation of common signal and channel of R19 NES

Type: discussion		For: Approval
					Source: ZTE Corporation, Sanechips

Decision: 		The document was not treated.

R4-2504290	General discussion on the NES common signal adaptation

Type: discussion		For: Discussion
					38.133 v	  CR-  rev  Cat:  (Rel-19)

					Source: Samsung

Decision: 		The document was not treated.

R4-2504506	RRM requirements for SSB adaptation

Type: discussion		For: Discussion
					Source: Nokia, Nokia Shanghai Bell

Decision: 		The document was not treated.

##### 7.25.2.3	On-demand SIB1

R4-2503126	On RRM requirements for on-demand SIB1

Type: discussion		For: Discussion
					Source: Qualcomm Technologies Ireland

Decision: 		The document was not treated.

R4-2503295	Discussion on RRM requirement impacts from on-demand SIB1 in R19 NES

Type: discussion		For: Discussion
					Source: vivo

Decision: 		The document was not treated.

R4-2503345	On RRM requirement for on-demand SIB1

Type: other		For: Approval
					Source: OPPO

Decision: 		The document was not treated.

R4-2503382	Discussion on RRM requirement of OD-SIB1 for NES enhancement

Type: discussion		For: Discussion
					Source: CMCC

Decision: 		The document was not treated.

R4-2503431	Discussion on RRM requirements for On-demand SIB1

Type: discussion		For: Discussion
					Source: CATT

Decision: 		The document was not treated.

R4-2503571	Discussion on on-demand SIB1

Type: discussion		For: Discussion
					Source: Apple

Decision: 		The document was not treated.

R4-2503596	Discussion on RRM requirements of on-demand SIB1 for Rel-19 NES

Type: discussion		For: Discussion
					Source: LG Electronics

Decision: 		The document was not treated.

R4-2503868	Discussion on RRM impact for Rel-19 OD-SIB1

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon

Decision: 		The document was not treated.

R4-2504228	Discussion on OD-SIB1 in Rel-19 NES

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

This contribution discusses the Rel-19 NES Obj.2 requirement

Decision: 		The document was not treated.

R4-2504507	RRM requirements for On-demand-SIB1

Type: discussion		For: Discussion
					Source: Nokia, Nokia Shanghai Bell

Decision: 		The document was not treated.

#### 7.25.3	Moderator summary and conclusions

R4-2503625	Topic summary for [114bis][215] Netw_Energy_NR_enh_Part1

Type: other		For: Information
					Source: Moderator (Ericsson)

Abstract:

Topic summary in RRM session

Decision: 		The document was not treated.

R4-2503626	Topic summary for [114bis][216] Netw_Energy_NR_enh_Part2

Type: other		For: Information
					Source: Moderator (Huawei)

Abstract:

Topic summary in RRM session

Decision: 		The document was not treated.

### 7.26	Low-power wake-up signal and receiver for NR (LP-WUS/WUR)

#### 7.26.1	General aspects and RF TR (for email approval)

R4-2503804	Further consideration on general aspects of LP-WUS

Type: other		For: Approval
					Source: Huawei, HiSilicon

Decision: 		The document was not treated.

R4-2503830	TR 38.774 v0.3.0

Type: draft TR		For: Agreement
					38.774 v0.3.0	  CR-  rev  Cat:  (Rel-19)

					Source: vivo

Decision: 		The document was not treated.

R4-2503831	Draft Big CR for LP-WUS UE RF requirements in Rel-19

Type: draftCR		For: Endorsement
					38.101-1 v19.1.0	  CR-  rev  Cat: B (Rel-19)

					Source: vivo

Decision: 		The document was not treated.

R4-2504393	On general issues for WUR

Type: other		For: Approval
					Source: Ericsson

Abstract:

In this paper, we present our overview for WUR.

Decision: 		The document was not treated.

R4-2504417	TP for TR 38.774 on LP-WUS

Type: pCR		For: Approval
					38.774 v0.2.0	  CR-  rev  Cat:  (Rel-19)

					Source: Nokia

Decision: 		The document was not treated.

#### 7.26.2	UE RF requirements for LP-WUS/WUR

R4-2504065	views on requirements of the low-power wake-up receiver

Type: discussion		For: Approval
					Source: Sony

Decision: 		The document was not treated.

##### 7.26.2.1	System parameters

R4-2503400	Discussion on LP-WUS UE system parameters requirements

Type: discussion		For: Discussion
					Source: CMCC

Decision: 		The document was not treated.

R4-2503483	TP to TR 38.774 on Clause 5 (System parameters)

Type: pCR		For: Approval
					38.774 v0.2.0	  CR-  rev  Cat:  (Rel-19)

					Source: CATT

Abstract:

Correction on operating bands according to RAN4 agreements

Decision: 		The document was not treated.

R4-2503832	TP to TR 38.774 on LP-WUS

Type: pCR		For: Approval
					38.774 v0.2.0	  CR-  rev  Cat:  (Rel-19)

					Source: vivo

Decision: 		The document was not treated.

R4-2504394	On system parameter for WUR

Type: other		For: Approval
					Source: Ericsson

Abstract:

in this paper, we present our view on system parameter for WUR.

Decision: 		The document was not treated.

R4-2504413	System parameters for LP-WUR

Type: other		For: Approval
					Source: Nokia

Decision: 		The document was not treated.

##### 7.26.2.2	Rx requirements of REFSENS, ASCS and ACS

R4-2503170	On UE RF requirements for the LPWUR

Type: other		For: Approval
					Source: Qualcomm Incorporated

Abstract:

Proposals for UE RF core requirement

Decision: 		The document was not treated.

R4-2503191	On LP-WUR Rx requirements

Type: discussion		For: Discussion
					Source: Apple

Decision: 		The document was not treated.

R4-2503399	Discussion on LP-WUS UE RF Rx requirements of REFSENS, ASCS and ACS

Type: discussion		For: Discussion
					Source: CMCC

Decision: 		The document was not treated.

R4-2503484	Further discussion on REFSENS for LP-WUS

Type: other		For: Discussion
					Source: CATT

Decision: 		The document was not treated.

R4-2503638	Discussion on REFSENS, ASCS, ACS for LP-WUR

Type: other		For: Approval
					Source: ZTE Corporation, Sanechips

Decision: 		The document was not treated.

R4-2503648	Discussion on UE RF requirements for FR2 LP-WUR

Type: discussion		For: Discussion
					Source: Samsung

Decision: 		The document was not treated.

R4-2503805	Further consideration on UE REFSENS, ACS requirements for Rel-19 LP-WUS

Type: other		For: Approval
					Source: Huawei, HiSilicon

Decision: 		The document was not treated.

R4-2503833	Updates on LP-WUS REFSENS, ASCS and ACS

Type: other		For: Approval
					Source: vivo

Decision: 		The document was not treated.

R4-2504397	WUR RF requirement REFSESN ACS ASCS

Type: other		For: Approval
					Source: Ericsson

Abstract:

In this paper, we present our view on the WUR RF requirement of REFSENS, ASC, ASCS

Decision: 		The document was not treated.

R4-2504414	Fundamental RX requirements for LP-WUR

Type: other		For: Approval
					Source: Nokia

Decision: 		The document was not treated.

##### 7.26.2.3	Rx requirements of IBB, OBB, intermodulation, spurious emissions and others

R4-2503401	Discussion on LP-WUS UE RF Rx requirements of IBB, OBB, intermodulation, spurious emissions and others

Type: discussion		For: Discussion
					Source: CMCC

Decision: 		The document was not treated.

R4-2503485	Further discussion on other RF requirements for LP-WUS

Type: other		For: Discussion
					Source: CATT

Decision: 		The document was not treated.

R4-2503639	Discussion on receiver characteristics for LP-WUR

Type: other		For: Approval
					Source: ZTE Corporation, Sanechips

Decision: 		The document was not treated.

R4-2503806	Further consideration on UE RF other Rx requirements for Rel-19 LP-WUS

Type: other		For: Approval
					Source: Huawei, HiSilicon

Decision: 		The document was not treated.

R4-2503834	draft CR to 38.101-1 on LP-WUS UE RF

Type: other		For: Approval
					38.101-1 v	  CR-  rev  Cat:  (Rel-19)

					Source: vivo

Decision: 		The document was not treated.

R4-2504396	WUR RF requirement other than REFSENS

Type: other		For: Approval
					Source: Ericsson

Abstract:

In this paper, we present our view on the WUR RF requirement of IBB, OBB, intermodulation, spurious emissions and others .

Decision: 		The document was not treated.

R4-2504415	Other RX requirements for LP-WUR

Type: other		For: Approval
					Source: Nokia

Decision: 		The document was not treated.

##### 7.26.2.4	Testability for UE RF requirements

R4-2503171	On test method for UE RF requirements for the LPWUR

Type: other		For: Approval
					Source: Qualcomm Incorporated

Abstract:

A test method without specifying a test mode is preferred

Decision: 		The document was not treated.

R4-2503486	Further discussion on Testability for UE RF requirements for LP-WUS

Type: other		For: Discussion
					Source: CATT

Decision: 		The document was not treated.

R4-2503640	Discussion on testability of LP-WUR

Type: other		For: Approval
					Source: ZTE Corporation, Sanechips

Decision: 		The document was not treated.

R4-2503649	Discussion on testability for LP-WUR

Type: discussion		For: Discussion
					Source: Samsung

Decision: 		The document was not treated.

R4-2503807	Further consideration on UE RF testability issues for Rel-19 LP-WUS

Type: other		For: Approval
					Source: Huawei, HiSilicon

Decision: 		The document was not treated.

R4-2504395	On WUR RF requirement testability

Type: other		For: Approval
					Source: Ericsson

Abstract:

In this paper, we present our overview on the WUR RF requirement testability issue

Decision: 		The document was not treated.

R4-2504416	Testability aspects of LP-WUR

Type: other		For: Approval
					Source: Nokia

Decision: 		The document was not treated.

#### 7.26.3	BS RF requirements for LP-WUS/WUR

R4-2503168	BS RF requirements for low-power wake-up signal for NR

Type: other		For: Approval
					Source: Nokia

Abstract:

This contribution provides proposals to progress the issues that were FFS in the agreed WF.

Decision: 		The document was not treated.

R4-2503398	Discussion on LP-WUS BS RF requirements

Type: discussion		For: Discussion
					Source: CMCC

Decision: 		The document was not treated.

R4-2503487	Further discussion on BS RF requirements for LP-WUS

Type: other		For: Discussion
					Source: CATT

Decision: 		The document was not treated.

R4-2503488	draftCR to TS 38.141-1 on declaration of support for LP-WUS

Type: draftCR		For: Endorsement
					38.141-1 v19.0.0	  CR-  rev  Cat: B (Rel-19)

					Source: CATT

Decision: 		The document was not treated.

R4-2503489	draftCR to TS 38.141-2 on declaration of support for LP-WUS

Type: draftCR		For: Endorsement
					38.141-2 v19.0.0	  CR-  rev  Cat:  (Rel-19)

					Source: CATT

Decision: 		The document was not treated.

R4-2503636	Discussion on BS RF requirements for LP-WUS/WUR

Type: other		For: Approval
					Source: ZTE Corporation, Sanechips

Decision: 		The document was not treated.

R4-2503637	TP to TR 38.774 on LP-WUS power boosting (Clause 7.2.2)

Type: pCR		For: Approval
					38.774 v0.2.0	  CR-  rev  Cat:  (Rel-19)

					Source: ZTE Corporation, Sanechips

Decision: 		The document was not treated.

R4-2503808	Further consideration on BS RF for Rel-19 LP-WUS

Type: other		For: Approval
					Source: Huawei, HiSilicon

Decision: 		The document was not treated.

R4-2504392	TP to TR38.774: BS RF requirement overview for LP-WUS

Type: pCR		For: Approval
					38.774 v0.2.0	  CR-  rev  Cat:  (Rel-19)

					Source: Ericsson

Abstract:

In this paper, we provide our BS RF requirement impact.

Decision: 		The document was not treated.

#### 7.26.4	RRM core requirements for LP-WUS/WUR

##### 7.26.4.1	Simulation assumptions and results

R4-2503296	Further discussion on simulation assumptions for LP-WUR

Type: discussion		For: Discussion
					Source: vivo

Decision: 		The document was not treated.

R4-2503297	Simulation results for LP-WUR

Type: discussion		For: Discussion
					Source: vivo

Decision: 		The document was not treated.

R4-2503301	Discussion on simulation assumptions and results for LP-WUS/WUR

Type: discussion		For: Discussion
					Source: Xiaomi

Decision: 		The document was not treated.

R4-2503354	Simulation assumption and results for LP-WUSWUR

Type: other		For: Approval
					Source: OPPO

Decision: 		The document was not treated.

R4-2503383	Simulation assumption and results for LP-WUR

Type: discussion		For: Discussion
					Source: CMCC

Decision: 		The document was not treated.

R4-2503527	Discussion of Simulation Assumptions for LP-WUR RRM Requirements

Type: discussion		For: Discussion
					Source: Apple

Decision: 		The document was not treated.

R4-2503528	Summary of Simulation Results for LP-WUR RRM Requirements

Type: discussion		For: Discussion
					Source: Apple

Decision: 		The document was not treated.

R4-2503580	Discussion on simulation assumptions and results for LP-WUS/WUR

Type: discussion		For: Discussion
					38.133 v	  CR-  rev  Cat:  (Rel-19)

					Source: LG Electronics Inc.

Decision: 		The document was not treated.

R4-2503796	Discussion on OOK based LP-WUR simulation

Type: other		For: Approval
					Source: ZTECorporation,Sanechips

Decision: 		The document was not treated.

R4-2503899	Simulation assumption and additional simulation results for LP-WUR measurement

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon

Decision: 		The document was not treated.

R4-2504223	Simulation assumption on LP-WUS RRM measurement

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

This contribution discusses the LP-WUS RRM simulation assumption

Decision: 		The document was not treated.

R4-2504224	Simulation on LP-WUS RRM measurement

Type: other		For: Discussion
					Source: Ericsson

Abstract:

This contribution shares the LP-WUS RRM simulation results

Decision: 		The document was not treated.

R4-2504466	Simulation results for RRM LP-SS simulation campaign

Type: discussion		For: Discussion
					38.133 v	  CR-  rev  Cat:  (Rel-19)

					Source: Nokia

Decision: 		The document was not treated.

R4-2504467	Discussion on LP-WUS RRM Simulations

Type: discussion		For: Discussion
					38.133 v	  CR-  rev  Cat:  (Rel-19)

					Source: Nokia

Decision: 		The document was not treated.

R4-2504598	Simulation assumptions for R19 LP-WUS

Type: discussion		For: Discussion
					Source: MediaTek inc.

Decision: 		The document was not treated.

##### 7.26.4.2	RRM core requirements

R4-2503298	Consideration on remaining RRM issues for LP-WUR

Type: discussion		For: Discussion
					Source: vivo

Decision: 		The document was not treated.

R4-2503302	Discussion on RRM core requirements for LP-WUS/WUR

Type: discussion		For: Discussion
					Source: Xiaomi

Decision: 		The document was not treated.

R4-2503355	Discussion on RRM requirements for LP-WUSWUR

Type: other		For: Approval
					Source: OPPO

Decision: 		The document was not treated.

R4-2503384	Discussion on RRM impact of LP-WUR

Type: discussion		For: Discussion
					Source: CMCC

Decision: 		The document was not treated.

R4-2503432	Discussion on RRM requirements for LP-WUS/WUR

Type: discussion		For: Discussion
					Source: CATT

Decision: 		The document was not treated.

R4-2503533	On RRM core requirements for LP-WUR

Type: discussion		For: Discussion
					38.133 v	  CR-  rev  Cat:  (Rel-19)

					Source: Apple

Decision: 		The document was not treated.

R4-2503534	Reply LS on the wake-up delay for LP-WUS operation in IDLE/INACTIVE mode

Type: LS out		For: Approval
					to RAN1, cc RAN2
					Source: Apple

Decision: 		The document was not treated.

R4-2503581	Discussion on RRM core requirements for LP-WUS/WUR

Type: discussion		For: Discussion
					38.133 v	  CR-  rev  Cat:  (Rel-19)

					Source: LG Electronics Inc.

Decision: 		The document was not treated.

R4-2503792	Discussion on LP-WUS for core part

Type: other		For: Approval
					Source: ZTECorporation,Sanechips

Decision: 		The document was not treated.

R4-2503803	Draft reply LS to RAN1 on wake-up delay for LP-WUS operation in IDLE/INACTIVE mode

Type: LS out		For: Approval
					to RAN1
					Source: ZTECorporation,Sanechips

Decision: 		The document was not treated.

R4-2503900	Discussion on RRM requirements for LP-WUR

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon

Decision: 		The document was not treated.

R4-2503919	Discussion on RRM core requirements for LP-WUS/WUR

Type: discussion		For: Discussion
					Source: China Telecom

Decision: 		The document was not treated.

R4-2504222	Discussion on LP-WUS RRM requirement

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

This contribution discusses the LP-WUS requirement

Decision: 		The document was not treated.

R4-2504468	Discussion on LP-WUS core requirements

Type: discussion		For: Discussion
					38.133 v	  CR-  rev  Cat:  (Rel-19)

					Source: Nokia

Decision: 		The document was not treated.

R4-2504599	Discussion on the RRM core requirements for LP-WUS

Type: discussion		For: Discussion
					Source: MediaTek inc.

Decision: 		The document was not treated.

R4-2504605	RRM requirements for LP-WUR

Type: discussion		For: Discussion
					Source: Qualcomm Incorporated

Decision: 		The document was not treated.

#### 7.26.5	Moderator summary and conclusions

R4-2503627	Topic summary for [114bis][217] NR_LPWUS

Type: other		For: Information
					Source: Moderator (vivo)

Abstract:

Topic summary in RRM session

Decision: 		The document was not treated.

R4-2504633	Topic summary for [114bis][308] NR_LPWUS

Type: other		For: Information
					Source: Moderator (Huawei)

Abstract:

[114bis] BDaT Session AI 7.26.3

Decision: 		The document was not treated.

R4-2504689	Topic summary for [114bis][131] NR_LPWUS_UERF

Type: other		For: Information
					Source: Moderator(Vivo)

Abstract:

Summary for AI 7.26, 7.26.1, 7.26.2

Decision: 		The document was not treated.

### 7.27	NR mobility enhancements Phase 4

#### 7.27.1	General aspects

#### 7.27.2	RRM core requirements

##### 7.27.2.1	Event triggered L1 measurement reporting

R4-2503310	Discussion on event triggered L1 measurement reporting for mobility

Type: discussion		For: Discussion
					Source: Xiaomi

Decision: 		The document was not treated.

R4-2503346	On event triggered L1 measurement reporting

Type: other		For: Approval
					Source: OPPO

Decision: 		The document was not treated.

R4-2503433	Discussion on Event triggered L1 measurement reporting for Rel-19 LTM enhancements

Type: discussion		For: Discussion
					Source: CATT

Decision: 		The document was not treated.

R4-2503572	Discussion on event triggered L1 measurement reporting

Type: discussion		For: Discussion
					Source: Apple

Decision: 		The document was not treated.

R4-2503603	Discussion on event triggered L1 report for LTM

Type: discussion		For: Discussion
					Source: MediaTek Inc.

Decision: 		The document was not treated.

R4-2503757	Discussion on event triggered L1 measurement reporting

Type: discussion		For: Discussion
					Source: CMCC

Decision: 		The document was not treated.

R4-2503854	Discussion on event triggered L1 measurement reporting

Type: other		For: Approval
					Source: ZTE Corporation, Sanechips

Decision: 		The document was not treated.

R4-2503883	Discussion on Event triggered L1 measurement reporting

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon

Decision: 		The document was not treated.

R4-2503936	Discussion on Event triggered L1 measurement reporting

Type: discussion		For: Discussion
					Source: Ericsson

Decision: 		The document was not treated.

R4-2503987	RRM Core requirements on event triggered L1 measurement reporting

Type: discussion		For: Discussion
					Source: China Telecom

Decision: 		The document was not treated.

R4-2504030	On RRM requirements for LTM event-triggered L1-RSRP reporting

Type: discussion		For: Discussion
					Source: vivo

Decision: 		The document was not treated.

R4-2504071	On event-triggered L1 reporting for LTM

Type: discussion		For: Discussion
					Source: Nokia

Decision: 		The document was not treated.

R4-2504072	DraftCR for LTM event-triggered L1 requirements

Type: draftCR		For: Endorsement
					38.133 v19.0.0	  CR-  rev  Cat: B (Rel-19)

					Source: Nokia

Decision: 		The document was not treated.

##### 7.27.2.2	CSI-RS based L1 measurement

R4-2503311	Discussion on CSI-RS based L1 for mobility

Type: discussion		For: Discussion
					Source: Xiaomi

Decision: 		The document was not treated.

R4-2503347	On CSI-RS based L1 measurement

Type: other		For: Approval
					Source: OPPO

Decision: 		The document was not treated.

R4-2503434	Discussion on CSI-RS based L1 measurement  for Rel-19 LTM enhancements

Type: discussion		For: Discussion
					Source: CATT

Decision: 		The document was not treated.

R4-2503585	CSI-RS based L1-RSRP measurement

Type: discussion		For: Discussion
					Source: Apple

Decision: 		The document was not treated.

R4-2503604	Discussion on CSI-RS L1 measurement for LTM

Type: discussion		For: Discussion
					Source: MediaTek Inc.

Decision: 		The document was not treated.

R4-2503758	Discussion on CSI-RS based L1 measurement

Type: discussion		For: Discussion
					Source: CMCC

Decision: 		The document was not treated.

R4-2503855	Discussion on CSI-RS based L1-RSRP measurement

Type: other		For: Approval
					Source: ZTE Corporation, Sanechips

Decision: 		The document was not treated.

R4-2503884	Discussion on CSI-RS based L1 measurement

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon

Decision: 		The document was not treated.

R4-2503937	Discussion on CSI-RS based L1 measurement

Type: discussion		For: Discussion
					Source: Ericsson

Decision: 		The document was not treated.

R4-2503988	RRM Core requirements on CSI-RS based L1 measurement

Type: discussion		For: Discussion
					Source: China Telecom

Decision: 		The document was not treated.

R4-2504031	On RRM requirements for LTM CSI-RS based L1 measurement

Type: discussion		For: Discussion
					Source: vivo

Decision: 		The document was not treated.

R4-2504470	Discussion on CSI-RS based L1 measurements

Type: discussion		For: Discussion
					38.133 v	  CR-  rev  Cat:  (Rel-19)

					Source: Nokia

Decision: 		The document was not treated.

R4-2504527	CSI-RS based L1 measurement for LTM

Type: discussion		For: Discussion
					Source: Qualcomm Incorporated

Decision: 		The document was not treated.

##### 7.27.2.3	Conditional Intra-CU LTM

R4-2503303	Discussion on Conditional Intra-CU LTM for NR mobility enhancements Phase 4

Type: discussion		For: Discussion
					Source: Xiaomi

Decision: 		The document was not treated.

R4-2503435	Discussion on conditional Intra-CU LTM for Rel-19 mobility enhancements

Type: discussion		For: Discussion
					Source: CATT

Decision: 		The document was not treated.

R4-2503573	Discussion on conditional Intra-CU LTM

Type: discussion		For: Discussion
					Source: Apple

Decision: 		The document was not treated.

R4-2503605	Discussion on Conditional LTM

Type: discussion		For: Discussion
					Source: MediaTek Inc.

Decision: 		The document was not treated.

R4-2503752	Discussion on intra-CU conditional LTM

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

This contribution discuss the RRM requirement for intra-CU conditional LTM

Decision: 		The document was not treated.

R4-2503759	Discussion on conditional intra-CU LTM

Type: discussion		For: Discussion
					Source: CMCC

Decision: 		The document was not treated.

R4-2503856	Discussion on Conditional Intra-CU LTM

Type: other		For: Approval
					Source: ZTE Corporation, Sanechips

Decision: 		The document was not treated.

R4-2503885	Discussion on conditional Intra-CU LTM

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon

Decision: 		The document was not treated.

R4-2503989	RRM Core requirements on conditional Intra-CU LTM

Type: discussion		For: Discussion
					Source: China Telecom

Decision: 		The document was not treated.

R4-2504052	Further discussion on Conditional Intra-CU LTM

Type: discussion		For: Discussion
					Source: vivo

Decision: 		The document was not treated.

R4-2504202	On CLTM RRM requirements

Type: discussion		For: Discussion
					Source: Nokia

Decision: 		The document was not treated.

R4-2504528	Conditional LTM

Type: discussion		For: Discussion
					Source: Qualcomm Incorporated

Decision: 		The document was not treated.

#### 7.27.3	Moderator summary and conclusions

R4-2503628	Topic summary for [114bis][218] NR_Mob_Ph4_Part1

Type: other		For: Information
					Source: Moderator (Apple)

Abstract:

Topic summary in RRM session

Decision: 		The document was not treated.

R4-2503629	Topic summary for [114bis][219] NR_Mob_Ph4_Part2

Type: other		For: Information
					Source: Moderator (China Telecom)

Abstract:

Topic summary in RRM session

Decision: 		The document was not treated.

### 7.28	XR for NR Phase 3

#### 7.28.1	General aspects

R4-2503921	draftCR38133 Inter-frequency measurement requirements with XR

Type: draftCR		For: Endorsement
					38.133 v19.0.0	  CR-  rev  Cat:  (Rel-19)

					Source: Ericsson

Abstract:

draftCR38133 Inter-frequency measurement requirements with XR

Decision: 		The document was not treated.

R4-2504504	Collection of XR_Ph3 agreements for information

Type: other		For: Information
					Source: Nokia

Decision: 		The document was not treated.

#### 7.28.2	RRM core requirements

R4-2503236	NR_XR_Ph3 remaining issues

Type: other		For: Agreement
					Source: InterDigital, Inc.

Decision: 		The document was not treated.

R4-2503574	Discussion of RRM impact of XR for NR Phase 3

Type: discussion		For: Discussion
					Source: Apple

Decision: 		The document was not treated.

R4-2503575	Inter-RAT measurements E-UTRAN TDD measurements

Type: draftCR		For: Endorsement
					38.133 v19.0.0	  CR-  rev  Cat: B (Rel-19)

					Source: Apple

Decision: 		The document was not treated.

R4-2503760	Discussion on XR (eXtended Reality) for NR Phase 3

Type: discussion		For: Discussion
					Source: CMCC

Decision: 		The document was not treated.

R4-2503870	Discussion on RRM requirements for Rel-19 XR

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon

Decision: 		The document was not treated.

R4-2503922	On RRM requirements for XR

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

On RRM requirements for XR

Decision: 		The document was not treated.

R4-2504027	Further discussion on RRM requirements for XR

Type: discussion		For: Discussion
					Source: vivo

Decision: 		The document was not treated.

R4-2504029	Draft CR on NR inter-frequency L1 measurement for XR

Type: draftCR		For: Endorsement
					38.133 v19.0.0	  CR-  rev  Cat: B (Rel-19)

					Source: vivo

Decision: 		The document was not treated.

R4-2504252	Discussion on RRM aspects of R19 XR

Type: discussion		For: Approval
					Source: ZTE Corporation, Sanechips

Decision: 		The document was not treated.

R4-2504257	Draft CR on measurement gap skipping in intra-frequency measurement with gaps for R19 XR

Type: draftCR		For: Endorsement
					38.133 v19.0.0	  CR-  rev  Cat: B (Rel-19)

					Source: ZTE Corporation, Sanechips

Decision: 		The document was not treated.

R4-2504291	General discussion on XR requirement

Type: discussion		For: Discussion
					38.133 v	  CR-  rev  Cat:  (Rel-19)

					Source: Samsung

Decision: 		The document was not treated.

R4-2504481	Discussion on RRM requirements for XR

Type: discussion		For: Discussion
					38.133 v	  CR-  rev  Cat:  (Rel-19)

					Source: MediaTek inc.

Decision: 		The document was not treated.

R4-2504505	Analysis of skip ratio on XR capacity and mobility performance

Type: other		For: Information
					Source: Nokia

Decision: 		The document was not treated.

R4-2504508	Discussion on XR ph3 RRM requirements

Type: discussion		For: Discussion
					Source: Nokia

Decision: 		The document was not treated.

R4-2504509	Draft CR38.133 Introduction of measurement gap cancellation

Type: draftCR		For: Endorsement
					38.133 v19.0.0	  CR-  rev  Cat: B (Rel-19)

					Source: Nokia

Decision: 		The document was not treated.

R4-2504606	RRM requirements for XR enhancements

Type: discussion		For: Discussion
					Source: Qualcomm Incorporated

Decision: 		The document was not treated.

R4-2504622	Discussion on RRM Requirements for XR

Type: discussion		For: Discussion
					Source: Meta

Decision: 		The document was not treated.

#### 7.28.3	Moderator summary and conclusions

R4-2503630	Topic summary for [114bis][220] NR_XR_Ph3

Type: other		For: Information
					Source: Moderator (Nokia)

Abstract:

Topic summary in RRM session

Decision: 		The document was not treated.

### 7.29	Non-Terrestrial Networks (NTN) for NR Phase 3

#### 7.29.1	General aspects

R4-2503451	DraftCR for TS 38.181, Introduction on SAN diagram for SAN supporting regenerative payload

Type: draftCR		For: Endorsement
					38.181 v18.5.0	  CR-  rev  Cat: B (Rel-19)

					Source: CATT

Decision: 		The document was not treated.

R4-2503713	Draft CR to TS 38.108: Correction of Regenerative Payload Figures

Type: draftCR		For: Endorsement
					38.108 v19.0.0	  CR-  rev  Cat: B (Rel-19)

					Source: NEC Europe Ltd, THALES

Abstract:

Updated clause 4.3.1 and 4.3.2 to capture correctly the support of regenerative payload with the satellite payload.

Decision: 		The document was not treated.

R4-2503824	Discussion on simultaneous operation between GNSS and UL transmission in NR NTN

Type: other		For: Approval
					Source: vivo

Decision: 		The document was not treated.

R4-2503825	Draft reply LS on simultaneous operation between GNSS and UL transmission in NR NTN

Type: LS out		For: Approval
					to RAN2, cc RAN1
					Source: vivo

Decision: 		The document was not treated.

R4-2504406	LS reply on IDC of GNSS

Type: other		For: Approval
					Source: Ericsson

Abstract:

In this paper we present our views on the questions brought by the LS.

Decision: 		The document was not treated.

#### 7.29.2	UE RF requirements

##### 7.29.2.1	RedCap UE RF requirements

R4-2503444	Discussion on RF requirements for NTN RedCap UE

Type: other		For: Approval
					Source: CATT

Decision: 		The document was not treated.

R4-2503693	Discussion on the simultaneous operation between GNSS and NR NTN

Type: other		For: Approval
					Source: Huawei, HiSilicon

Decision: 		The document was not treated.

R4-2503732	Discussion on NTN (e)Redcap UE RF requirements

Type: discussion		For: Discussion
					Source: Spreadtrum,UNISOC

Decision: 		The document was not treated.

R4-2504403	RedCap UE RF impact on HD-FDD

Type: other		For: Approval
					Source: Ericsson

Abstract:

In this paper, we present our general view on the RedCap UE RF remaining issue.

Decision: 		The document was not treated.

R4-2504418	RedCap NTN UEs

Type: other		For: Approval
					Source: Nokia

Decision: 		The document was not treated.

R4-2504428	Discussion on Rel-19 NR-NTN RedCap UE RF requirements

Type: discussion		For: Approval
					Source: MediaTek (Hefei) Inc.

Decision: 		The document was not treated.

R4-2504550	On simultaneous operation between GNSS and NR NTN

Type: other		For: Approval
					Source: Qualcomm Inc.

Decision: 		The document was not treated.

R4-2504551	Draft CR to TS 38.101-5: Introduction of RedCap and eRedCap

Type: draftCR		For: Endorsement
					38.101-5 v19.0.0	  CR-  rev  Cat: B (Rel-19)

					Source: Qualcomm Inc.

Decision: 		The document was not treated.

##### 7.29.2.2	Other requirements

R4-2504066	GNSS simultaneously operation with NR NTN

Type: discussion		For: Approval
					Source: Sony

Decision: 		The document was not treated.

R4-2504404	Other NTN UE RF impact

Type: other		For: Approval
					Source: Ericsson

Abstract:

In this paper, we present our general overview on NTN UE RF impact other than Redcap.

Decision: 		The document was not treated.

R4-2504552	NR NTN UL Capacity Enhancements

Type: other		For: Approval
					Source: Qualcomm Inc.

Decision: 		The document was not treated.

#### 7.29.3	SAN RF requirements

R4-2503445	Discussion on SAN diagram for NTN SAN

Type: other		For: Approval
					Source: CATT

Decision: 		The document was not treated.

R4-2503450	DraftCR for TS 38.108, Introduction on SAN diagram for SAN supporting regenerative payload

Type: draftCR		For: Endorsement
					38.108 v19.0.0	  CR-  rev  Cat: B (Rel-19)

					Source: CATT

Decision: 		The document was not treated.

#### 7.29.4	RRM core requirements

##### 7.29.4.1	(e)RedCap RRM requirements

R4-2503304	Discussion on (e)RedCap RRM requirements in NTN for NR Phase 3

Type: discussion		For: Discussion
					Source: Xiaomi

Decision: 		The document was not treated.

R4-2503348	Discussion on (e)RedCap RRM requirements of R19 NR NTN

Type: other		For: Approval
					Source: OPPO

Decision: 		The document was not treated.

R4-2503385	Discussion on the RRM requirement for Redcap over NTN

Type: discussion		For: Discussion
					Source: CMCC

Decision: 		The document was not treated.

R4-2503436	Discussion on (e)RedCap RRM requirements for Rel-19 NTN phase3

Type: discussion		For: Discussion
					Source: CATT

Decision: 		The document was not treated.

R4-2503535	On R19 NTN (e)RedCap RRM requirements

Type: discussion		For: Discussion
					38.133 v	  CR-  rev  Cat:  (Rel-19)

					Source: Apple

Decision: 		The document was not treated.

R4-2503669	Discussion on RRM requirements of RedCap in Rel-19 NTN phase 3

Type: discussion		For: Discussion
					Source: Samsung

Decision: 		The document was not treated.

R4-2503793	Discussion on RRM requirements for RedCap NTN enhancement

Type: other		For: Approval
					Source: ZTECorporation,Sanechips

Decision: 		The document was not treated.

R4-2503904	Discussion on RRM requirements for RedCap UE in NTN

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon

Decision: 		The document was not treated.

R4-2504050	Discussion on RRM requirements on (e)RedCap for R19 NR NTN Phase 3

Type: discussion		For: Discussion
					Source: vivo

Decision: 		The document was not treated.

R4-2504112	Discussion on RedCap RRM requirements for NTN for NR Phase 3

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

Discussion on RedCap RRM requirements for NTN for NR Phase 3

Decision: 		The document was not treated.

R4-2504510	Discussion on requirements for RedCap support in NR NTN

Type: discussion		For: Discussion
					Source: Nokia

Decision: 		The document was not treated.

R4-2504511	Draft CR38.133 Inter-RAT E-UTRAN in TN cell reselection for (e)RedCap over NR NTN

Type: draftCR		For: Endorsement
					38.133 v19.0.0	  CR-  rev  Cat: B (Rel-19)

					Source: Nokia

Decision: 		The document was not treated.

R4-2504530	RedCap for NTN

Type: discussion		For: Discussion
					Source: Qualcomm Incorporated

Decision: 		The document was not treated.

##### 7.29.4.2	Other RRM requirements

R4-2503305	Discussion on other RRM core requirements in NTN for NR Phase 3

Type: discussion		For: Discussion
					Source: Xiaomi

Decision: 		The document was not treated.

R4-2503349	Discussion on other RRM requirements of R19 NR NTN

Type: other		For: Approval
					Source: OPPO

Decision: 		The document was not treated.

R4-2503386	Discussion on the RRM requirement for NTN phase3

Type: discussion		For: Discussion
					Source: CMCC

Decision: 		The document was not treated.

R4-2503437	Discussion on other RRM requirements for Rel-19 NTN phase3

Type: discussion		For: Discussion
					Source: CATT

Decision: 		The document was not treated.

R4-2503536	On R19 other NTN RRM requirements

Type: discussion		For: Discussion
					38.133 v	  CR-  rev  Cat:  (Rel-19)

					Source: Apple

Decision: 		The document was not treated.

R4-2503670	Discussion on RRM requirements of other aspects in Rel-19 NTN phase 3

Type: discussion		For: Discussion
					Source: Samsung

Decision: 		The document was not treated.

R4-2503798	Discussion on RRM requirements for downlink coverage enhancement

Type: other		For: Approval
					Source: ZTECorporation,Sanechips

Decision: 		The document was not treated.

R4-2503905	Discussion on other RRM requirements for Rel-19 NTN

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon

Decision: 		The document was not treated.

R4-2504051	Discussion on RRM impacts on DL coverage for R19 NR NTN Phase 3

Type: discussion		For: Discussion
					Source: vivo

Decision: 		The document was not treated.

R4-2504111	Discussion on other RRM requirements for NTN for NR Phase 3

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

Discussion on other RRM requirements for NTN for NR Phase 3

Decision: 		The document was not treated.

R4-2504447	Mobility aspects related to extended SSB periodicity in NR over NTN

Type: discussion		For: Discussion
					Source: Nokia

Decision: 		The document was not treated.

R4-2504531	Downlink coverage enhancement for NTN

Type: discussion		For: Discussion
					Source: Qualcomm Incorporated

Decision: 		The document was not treated.

#### 7.29.5	Moderator summary and conclusions

R4-2503631	Topic summary for [114bis][221] NR_NTN_Ph3_Part1

Type: other		For: Information
					Source: Moderator (CATT)

Abstract:

Topic summary in RRM session

Decision: 		The document was not treated.

R4-2503632	Topic summary for [114bis][222] NR_NTN_Ph3_Part2

Type: other		For: Information
					Source: Moderator (Qualcomm)

Abstract:

Topic summary in RRM session

Decision: 		The document was not treated.

R4-2504634	Topic summary for [114bis][309] NR_NTN_Ph3_General_UE_SAN_RF

Type: other		For: Information
					Source: Moderator (Qualcomm)

Abstract:

[114bis] BDaT Session AI 7.29.1, 7.29.2.1, 7.29.2.2, 7.29.3

Decision: 		The document was not treated.

### 7.30	Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3

#### 7.30.1	General aspects

R4-2503716	Draft CR to TS 36.108: Correction of Regenerative Payload Figures

Type: draftCR		For: Endorsement
					36.108 v18.8.0	  CR-  rev  Cat: B (Rel-19)

					Source: NEC Europe Ltd, THALES

Abstract:

Updated clause 4.3.1 and 4.3.2 to capture correctly the support of regenerative payload with the satellite payload.

Decision: 		The document was not treated.

#### 7.30.2	RF core requirements

R4-2503387	Discussion on the LS on TDM DMRS for 3.75kHz SCS OCC

Type: discussion		For: Discussion
					Source: CMCC

Decision: 		The document was not treated.

R4-2503715	Discussion on feasibility of TDM DMRS for 3.75kHz SCS OCC

Type: discussion		For: Discussion
					Source: LG Electronics Inc.

Decision: 		The document was not treated.

R4-2504067	OCC scheme on the RF requirements for IoT NTN

Type: discussion		For: Approval
					Source: Sony

Decision: 		The document was not treated.

R4-2504405	LS reply for IoT NTN

Type: other		For: Approval
					Source: Ericsson

Abstract:

In this paper, we present our reply over the LS from RAN1[1].

Decision: 		The document was not treated.

R4-2504423	Discussion on the LS Reply regarding DMRS puncturing in NB-IoT over NTN

Type: other		For: Approval
					Source: Nokia

Decision: 		The document was not treated.

R4-2504465	Discussion on RF requirement impact for IoT NTN phase 3

Type: discussion		For: Approval
					Source: MediaTek (Hefei) Inc.

Decision: 		The document was not treated.

R4-2504553	IoT NTN UL Capacity Enhancements

Type: other		For: Approval
					Source: Qualcomm Inc.

Decision: 		The document was not treated.

R4-2504556	On TDM DMRS for 3.75kHz SCS OCC for IoT NTN

Type: other		For: Approval
					Source: Huawei, HiSilicon

Decision: 		The document was not treated.

#### 7.30.3	RRM core requirements

R4-2503132	Discussion on RRM requirements for IoT NTN phase 3

Type: discussion		For: Discussion
					Source: MediaTek inc.

Decision: 		The document was not treated.

R4-2503133	DraftCR on timing requirement for IoT NTN phase 3

Type: draftCR		For: Endorsement
					36.133 v19.0.0	  CR-  rev  Cat: B (Rel-19)

					Source: MediaTek inc.

Decision: 		The document was not treated.

R4-2503388	Discussion on the RRM requirement for IOT NTN phase3

Type: discussion		For: Discussion
					Source: CMCC

Decision: 		The document was not treated.

R4-2503871	Discussion on RRM impact for IoT NTN

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon

Decision: 		The document was not treated.

R4-2504110	Discussion on RRM requirements of Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

Discussion on RRM requirements of Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3

Decision: 		The document was not treated.

#### 7.30.4	Moderator summary and conclusions

R4-2503633	Topic summary for [114bis][223] IoT_NTN_Ph3

Type: other		For: Information
					Source: Moderator (MediaTek)

Abstract:

Topic summary in RRM session

Decision: 		The document was not treated.

R4-2504640	Topic summary for [114bis][315] IoT_NTN_Ph3

Type: other		For: Information
					Source: Moderator (MediaTek)

Abstract:

[114bis] BDaT Session AI 7.30.1, 7.30.2

Decision: 		The document was not treated.

### 7.31	Introduction of IoT-NTN TDD mode

R4-2503200	Work plan for WID: introduction of IoT-NTN TDD mode

Type: Work Plan		For: Approval
					Source: Iridium Satellite LLC

Decision: 		The document was not treated.

#### 7.31.1	General aspects

#### 7.31.2	Band and system parameters (DL and UL channelization, channel bandwidth)

R4-2503744	draftCR to TS36.102 Introduction of IoT-NTN TDD band

Type: draftCR		For: Endorsement
					36.102 v18.8.0	  CR-  rev  Cat: B (Rel-19)

					Source: ZTE Corporation, Sanechips

Decision: 		The document was not treated.

#### 7.31.3	SAN RF requirements

R4-2503745	Discussion on SAN RF requirements for IoT-NTN TDD band

Type: other		For: Approval
					Source: ZTE Corporation, Sanechips

Decision: 		The document was not treated.

R4-2503746	draftCR to TS36.108 Introduction of IoT-NTN TDD band

Type: draftCR		For: Endorsement
					36.108 v18.8.0	  CR-  rev  Cat: B (Rel-19)

					Source: ZTE Corporation, Sanechips

Decision: 		The document was not treated.

R4-2504618	SAN RF requirements for NTN NB-IoT TDD

Type: discussion		For: Discussion
					Source: THALES, Iridium Satellite LLC

Abstract:

Discussion on SAN RF requirements for TS 36.108 - for TDD NTN NB-IoT

Decision: 		The document was not treated.

#### 7.31.4	UE RF requirements

R4-2503564	Frequency error and UE regulatory requirements

Type: other		For: Approval
					Source: Qualcomm Incorporated

Abstract:

Vote for Gene!

Decision: 		The document was not treated.

R4-2504619	UE RF requirements for NTN NB-IoT TDD

Type: discussion		For: Discussion
					Source: THALES, Iridium Satellite LLC

Abstract:

Discussion on UE RF requirements from TS 36.102 - for TDD NTN NB-IoT UE.

Decision: 		The document was not treated.

#### 7.31.5	RRM core requirements

R4-2503134	Discussion on RRM requirements for IoT-NTN TDD mode

Type: discussion		For: Discussion
					Source: MediaTek inc.

Decision: 		The document was not treated.

R4-2503799	Discussion on IoT NTN TDD mode

Type: other		For: Approval
					Source: ZTECorporation,Sanechips

Decision: 		The document was not treated.

R4-2503872	Discussion on RRM impact due to periodic pattern for IoT NTN

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon

Decision: 		The document was not treated.

R4-2504448	Discussion on IoT NTN TDD requirements

Type: discussion		For: Discussion
					Source: Nokia

Decision: 		The document was not treated.

R4-2504529	RRM requirements for IoT NTN in TDD mode

Type: discussion		For: Discussion
					Source: Qualcomm Incorporated

Decision: 		The document was not treated.

R4-2504615	RRM requirements for NTN NB-IoT TDD

Type: discussion		For: Discussion
					Source: THALES, Iridium Satellite LLC

Abstract:

Proposals on some RRM requirements from previous meeting RAN4#114.

Decision: 		The document was not treated.

#### 7.31.6	Moderator summary and conclusions

R4-2503634	Topic summary for [114bis][224] IoT_NTN_TDD

Type: other		For: Information
					Source: Moderator (Qualcomm)

Abstract:

Topic summary in RRM session

Decision: 		The document was not treated.

R4-2504641	Topic summary for [114bis][316] IoT_NTN_TDD_UE_SAN_RF

Type: other		For: Information
					Source: Moderator (Thales)

Abstract:

[114bis] BDaT Session AI 7.31.1, 7.31.2, 7.31.3, 7.31.4

Decision: 		The document was not treated.

## 8	Liaison output to other groups and related issues

### 8.1	R19 related (LS which cannot be submitted to any dedicated agenda under AI 7)

### 8.2	R18 related

R4-2503360	Reply LS on AdditionalSpectrumEmission

Type: LS out		For: Approval
					to RAN2
					Source: MediaTek Korea Inc.

Abstract:

RAN4 kindly reply on the RAN2 question and ask to take above feedback information into account for RAN2 work to support the new NS value (“AdditionalSpectrumEmission”) signaling. MCC: R4-2503360 is moved from AI 7.2.1 to AI 8.2 and treat it in [132].

Decision: 		The document was not treated.

R4-2503576	Reply LS on granularity definition of pdcch-RACH-AffectedBandsList-r18

Type: discussion		For: Discussion
					Source: Apple

Decision: 		The document was not treated.

R4-2503600	Reply LS on AdditionalSpectrumEmission

Type: LS out		For: Approval
					to RAN2
					Source: LG Electronics

Abstract:

It is reply LS on RAN2 for SL AdditionalSpectrumEmission.

Decision: 		The document was not treated.

R4-2503610	Discussion on the question from RAN2 on R18 LTM

Type: discussion		For: Discussion
					Source: MediaTek Inc.

Decision: 		The document was not treated.

R4-2503641	Reply LS to RAN2 on additionalSpectrumEmission in NR SL

Type: LS out		For: Approval
					to RAN2, cc RAN1
					Source: ZTE Corporation, Sanechips

Abstract:

In this contribution,  we provide the reply LS to RAN2 LS(R2-2501383) on additionalSpectrumEmission in NR SL.

Decision: 		The document was not treated.

R4-2503827	Discussion and reply LS on additionalSpectrumEmission on Sidelink

Type: other		For: Approval
					Source: vivo

Decision: 		The document was not treated.

R4-2503861	Discussion and draft Reply LS to RAN2 on granularity definition of pdcch-RACH-AffectedBandsList-r18

Type: other		For: Approval
					Source: ZTE Corporation, Sanechips

Decision: 		The document was not treated.

R4-2503892	Reply LS on granularity definition of pdcch-RACH-AffectedBandsList-r18

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon

Decision: 		The document was not treated.

R4-2504032	Discussion on measurement quantities for IoT NTN

Type: discussion		For: Discussion
					Source: vivo

Abstract:

MCC: Treat in email thread [315].

Decision: 		The document was not treated.

R4-2504033	Reply LS on measurement quantities for IoT NTN

Type: LS out		For: Approval
					to RAN1
					Source: vivo

Abstract:

MCC: Treat in email thread [315].

Decision: 		The document was not treated.

R4-2504351	Reply LS on granularity definition of pdcch-RACH-AffectedBandsList-r18

Type: discussion		For: Discussion
					Source: vivo

Decision: 		The document was not treated.

R4-2504424	Discussion on reply LS on measurement quantities for IoT NTN

Type: LS out		For: Approval
					to RAN1
					Source: Ericsson

Abstract:

In this paper we present our views on the questions in LS. MCC: Treat in email thread [315].

Decision: 		The document was not treated.

R4-2504444	(IoT_NTN_enh) Discussion on the LS Reply regarding measurement quantities in NB-IoT over NTN (R1-2501512)

Type: discussion		For: Discussion
					Source: Nokia

Abstract:

MCC: Treat in email thread [315].

Decision: 		The document was not treated.

R4-2504471	LS reply to RAN2 on granularity definition of pdcch-RACH-AffectedBandsList-r18

Type: discussion		For: Discussion
					Source: Nokia

Decision: 		The document was not treated.

R4-2504523	Reply LS to RAN2 on granularity definition of pdcch-RACH-AffectedBandsList-r18

Type: discussion		For: Discussion
					Source: Qualcomm Incorporated

Decision: 		The document was not treated.

### 8.3	R17, R15 and R16 related

R4-2504407	Discussion on reply LS on measurement quantities for IoT NTN

Type: LS out		For: Approval
					to RAN1
					Source: Ericsson

Abstract:

In this paper we present our views on the questions in LS.

Decision: 		The document was withdrawn.

### 8.4	Moderator summary and conclusions

R4-2503635	Topic summary for [114bis][225] Reply_LS

Type: other		For: Information
					Source: Moderator (Apple)

Abstract:

Topic summary in RRM session

Decision: 		The document was not treated.

R4-2504655	Topic summary for [114bis][330] LS_BDaT

Type: other		For: Information
					Source: Moderator (Qualcomm)

Abstract:

[114bis] BDaT Session AI 8.1, 8.2, 8.3 (BDaT related). MCC: The session chair marked this as withdrawn.

Decision: 		The document was withdrawn.

R4-2504690	Topic summary for [114bis][132] NR_reply_LS_UE_RF

Type: other		For: Information
					Source: Moderator(Apple)

Abstract:

Summary for AI 8 UE RF topics

Decision: 		The document was not treated.

## 9	RAN task and other topics

R4-2504125	Progress update and next steps for the CA database

Type: discussion		For: Approval
					Source: Nokia

Decision: 		The document was not treated.

### 9.1	Framework simplification for co-location/co-existence requirements (RP-243288)

R4-2503233	On RAN task to work on framework simplification for co-location/co-existence requirements

Type: other		For: Approval
					Source: Nokia

Decision: 		The document was not treated.

R4-2503462	Further discussion on framework simplification for co-location/co-existence requirements

Type: other		For: Approval
					Source: CATT

Decision: 		The document was not treated.

R4-2503463	Draft CR for 38.104, on framework simplification for co-location/co-existence requirement

Type: draftCR		For: Endorsement
					38.104 v19.0.0	  CR-  rev  Cat: F (Rel-19)

					Source: CATT

Decision: 		The document was not treated.

R4-2503464	Draft CR for 38.106, on framework simplification for co-location/co-existence requirement

Type: draftCR		For: Endorsement
					38.106 v19.0.0	  CR-  rev  Cat: F (Rel-19)

					Source: CATT

Decision: 		The document was not treated.

R4-2503465	Draft CR for 38.174, on framework simplification for co-location/co-existence requirement

Type: draftCR		For: Endorsement
					38.174 v19.0.0	  CR-  rev  Cat: F (Rel-19)

					Source: CATT

Decision: 		The document was not treated.

R4-2503727	Draft CR to TS 38.104: Introduction of new requirement structure for transmitter spurious emission in subclause 6.6.5.2.3 and subclause 6.6.5.2.4

Type: draftCR		For: Endorsement
					38.104 v18.9.0	  CR-  rev  Cat: D (Rel-18)

					Source: Ericsson

Abstract:

Based on the outcome of the RAN task to improve the BS RF specification structure for co-existence and co-location requirements this CR was created to highlight the change needed to improve the specification. The improvement is based on a new approach whe

Decision: 		The document was not treated.

R4-2503728	Draft CR to TS 38.141-2: Introduction of new requirement structure for transmitter spurious emission in sub-clause 6.7.5.4.5 and sub-clause 6.7.5.5.5

Type: draftCR		For: Endorsement
					38.141-2 v18.9.0	  CR-  rev  Cat: D (Rel-18)

					Source: Ericsson

Abstract:

Based on the outcome of the RAN task to improve the BS RF specification structure for co-existence and co-location requirements this CR was created to highlight the change needed to improve the specification. The improvement is based on a new approach whe

Decision: 		The document was not treated.

R4-2503729	On the topic of BS RF specification structure improvements

Type: other		For: Approval
					Source: Ericsson

Abstract:

In this contribution we present a solution to improve the BS RF specification structure with respect to transmitter spurious emission requirements for co-existence and co-location.

Decision: 		The document was not treated.

R4-2503730	Draft CR to TS 38.141-1: Introduction of new requirement structure for transmitter spurious emission in sub-clause 6.6.5.5.1.3 and sub-clause 6.6.5.5.1.4

Type: draftCR		For: Endorsement
					38.141-1 v18.9.0	  CR-  rev  Cat: D (Rel-18)

					Source: Ericsson

Abstract:

Based on the outcome of the RAN task to improve the BS RF specification structure for co-existence and co-location requirements this CR was created to highlight the change needed to improve the specification. The improvement is based on a new approach whe

Decision: 		The document was not treated.

R4-2504366	Discussion on the simplification for BS spec

Type: other		For: Approval
					Source: ZTE Corporation, Sanechips

Decision: 		The document was not treated.

R4-2504498	Some considerations on CA framework database

Type: other		For: Approval
					Source: ZTE Corporation, Sanechips

Decision: 		The document was not treated.

R4-2504572	draftCR to 38.104: on framework simplification for co-location/co-existence requirements

Type: draftCR		For: Endorsement
					38.104 v19.0.0	  CR-  rev  Cat:  (Rel-19)

					Source: Nokia

Decision: 		The document was not treated.

R4-2504573	draftCR to 38.141-1: on framework simplification for co-location/co-existence requirements

Type: draftCR		For: Endorsement
					38.141-1 v19.0.0	  CR-  rev  Cat:  (Rel-19)

					Source: Nokia

Decision: 		The document was not treated.

R4-2504595	Further discussion on the framework simplification for co-location/co-existence requirements

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon

Abstract:

In this contribution, we provide further discussion on the framework simplification for co-location/co-existence requirements.

Decision: 		The document was not treated.

R4-2504656	Topic summary for [114bis][331] BSRF_Simplify_CoLo_Coex

Type: other		For: Information
					Source: Moderator (Ericsson)

Abstract:

[114bis] BDaT Session AI 9.1

Decision: 		The document was not treated.

### 9.2	Principle of selected specification series in case of spectrum sharing by multiple RATs (related to approval of RP-250611)

R4-2504408	Specification impact on spectrum sharing by multiple RATs

Type: other		For: Approval
					Source: Ericsson

Abstract:

RAN task RAN4 to study the principle of selected specification series in case of spectrum sharing by multiple RATs (related to approval of RP-250611), this paper present our view on it. MCC: Treat in email thread [315].

Decision: 		The document was not treated.

## 10	New or revised WID/SID

R4-2503718	WF on 200 MHz channel bandwidth

Type: WID new		For: Information
					Source: KDDI, China Unicom, China Telecom, BT, Bouygues Telecom, Deutsche Telekom, KPN, Vodafone, Orange, Odido, Telefonica, Telenor, TIM, Telia Company, Spark NZ, Jio Platforms(JPL), Telstra, NTT Docomo, SoftBank, CKH IOD UK LTD, Ericsson, Huawei

Discussion:

China has identified the band 6425-7125 MHz for IMT, this came into effect 1st July 2023, see Chinese table of allocations and has given certainty that the band will be for 5G/6G.

UAE has added the band 6425-7125 MHz to their IMT holdings expects actual operation to begin between the years 2025 and 2026 by 2025/2026.

RCC Recommendation 1/21  to harmonize the technical conditions for 6425-7125 MHz, including channel used by 5G/IMT-2020 systems can consist of one or more adjacent frequency blocks but should not exceed 400 MHz.

The Indian Government decided to allocate the entire Upper 6 GHz band for IMT (3GPP n104), with different parts of the band being made available through to 2030, Cabinet Approves Spectrum Refarming for 5G and Future 6G Services: Report.

Brazil's National Telecommunications Agency (Anatel) identified the upper 6 GHz for IMT in Jan, 2025 and has now announced a draft schedule for frequency auctions over the next decade (2026–2036) , including 6 GHz in 2026,   SEI/ANATEL - 13277590 - Draft Resolution.

The amount of spectrum for each operator in these markets is expected to be at least 200 MHz.

200 MHz of spectrum per operator is also expected to be relevant for other markets.

Certainty needs to be given to operators and regulators that 200 MHz channel bandwidth will be supported by 3GPP.

Way Forward: 200 MHz channel bandwidth will be developed by 3GPP in Rel-20 considering n104.

Decision: 		The document was not treated.

R4-2503720	Motivation on supporting Intra-band non-collocated EN-DC/NR-CA deployment in Release 20 (Type 3 UE for handheld)

Type: WID new		For: Information
					Source: KDDI, NTT Docomo, SoftBank, KT, LG Uplus

Decision: 		The document was not treated.

R4-2503721	Observation on UE simulation assumption for MPR, A-MPR

Type: WID new		For: Information
					Source: KDDI Corporation

Decision: 		The document was not treated.

R4-2503725	Considerations on Rel-20 BS RF package objectives

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

In this contribution we focus only on BS RF centric evolution in Rel-20 with the intention to clarify technical justification and objectives for work to be included in BS RF work package.  It is essential to find relevant scope of each work package category to be able to fit it within RAN4 Rel-20 Time Unit (TU) budget. The information provided in this contribution can be used when new Work Item(s) (WI) are defined for RAN4 Rel-20 time period.

Decision: 		The document was not treated.

R4-2503847	New WID on New Power Class and enhanced requirements for NR and IoT NTN

Type: WID new		For: Information
					Source: vivo

Decision: 		The document was not treated.

R4-2504077	MediaTek Views on Rel-20 RAN4 5G-A scope

Type: other		For: Information
					Source: MediaTek inc.

Decision: 		The document was not treated.

R4-2504367	Discussion on Rel-20 BS RF enhancement

Type: other		For: Information
					Source: ZTE Corporation, Sanechips

Decision: 		The document was not treated.

R4-2504375	Views on Rel-20 RRM topics

Type: other		For: Information
					Source: Ericsson

Abstract:

In this contriution we present out views on Rel-20 RRM package.

Decision: 		The document was not treated.

## 11	Any other business

R4-2503567	6G specification format modernization: towards Automation-Native

Type: other		For: Information
					Source: Nokia

Discussion:

In this paper, Nokia discusses a new approach for 6G specifications from Rel-21.

MS Word is well established and has some highly appreciated strengths, most notably “what you see is what you get” (WYSIWYG) editing and highly functional change tracking.

However, several limitations have become increasingly evident throughout 3GPP, leading to a growing awareness that the time may be coming to re-evaluate whether Word is still the best tool for the job in the 21st century after four generations. These limitations include issues with stability and latency, inconsistent use of styles and inconsistent file formatting, as elaborated in [1] and [2]. These issues present challenges in particular for CR implementation into the specifications, which is a slow and error-prone process, and for applying other automation to the extraction and processing of specification information.

With the advent of 6G in Rel-21, a once-in-a-decade opportunity presents itself to make a clean start, taking advantage of state-of-the-art 21st century tools to produce world-class, beautiful specifications with flawless quality and accuracy in the most efficient way possible, natively supporting advanced automated processing.

An attractive vision for the 6G specifications could comprise:

-	A single body of specification information, with linking and searching enabled throughout;

-	Automatic, error-free, instantaneous CR implementation, with new spec versions instantly available after each 3GPP TSG Plenary;

-	Support for automated processing with consistent formatting and full version control.

This could be summarised with the following two fundamental goals:

Various text-based formats could be considered; among them, Markdown in particular stands out, which is already used by oneM2M and Wikipedia.

GoToWebinar sessions have recently been held at the instigation of TSG RAN#107, introducing some possibilities for Markdown and Git-based specifications. A recording of the GTW sessions, and the associated slides, are available at https://www.3gpp.org/ftp/tsg_ran/TSG_RAN/Tool_Automation_6G . There seems to be widespread interest in considering such an approach, although many details would still need to be addressed and pros/cons carefully considered (e.g. in relation to formatting, Git repository structure, CR process and procedure). Further investigations will take place to look into these matters.

It is not intended to start parallel investigations in individual WGs, but further investigation in 3GPP overall is expected, and information will be provided as soon as possible to enable interested delegates to participate therein and to contribute their expertise and perspectives.

Decision: 		The document was not treated.

## 12	Close of the meeting

The RAN4 Chair Xizeng Dai (Huawei) formally closed the RAN4#114-biss meeting on Friday, April 11, 2025, at 16h00.

Report prepared by: MCC