3GPP TSG-RAN WG4 #115	R4-2505350

St. Julians, Malta, May 19-23, 2025

Agenda item:	7.24.4.1		Enhancement for UE-initiated/Event-driven Beam Management

Source:	Qualcomm Incorporated

Title:	On core requirements for enhanced UE-initiated/event-driven beam management

Document for:	Discussion

# Introduction

In RAN4 #112, RAN4 work on Rel-19 MIMO Phase 5 was started. Several agreements have been made for UI-initiated/event-driven beam management, and additional open issues have been identified in [1]

In this paper we provide our view for the open issues identified in the way forward in [1] in RAN4 #114bis.

# Discussion

In RAN4 #114bis good progress was made on the definition of the measurement period in event triggered measurement reporting. It was agreed that T_RS of the current beam and the new beam are the same, i.e., requirements will only be defined for the same periodicities of the RS. Regarding the timeRestrictionForChannelMeasurement, it has been agreed that requirements at least will be defined for M = 1. Whether requirements will also be defined for M = 3 is for further discussion. Also, parameter P is still open after RAN4 #114bis. P ≥ 1 is a value for the SSB resources to be measured in case some of the SSB are not available due to collisions with measurement gaps.

#### Issue 1-1: How to define measurement period in Event triggered measurement reporting delay for the basic feature (M instance = 1, Event-2)?

Agreement:

  - For T_RS (TSSB or TCSI-RS) in L1_measurement periods

    - T_RS (TSSB or TCSI-RS):

      - the value of current beam and new beams are the same.

  - For M in L1_measurement periods:

    - If timeRestrictionForChannelMeasurement configured, M= 1, event evaluation should be based on the most recent result from both new beam and current beam

    - If timeRestrictionForChannelMeasurement is not configured,

      - Further discuss on:

        - Option1: Reuse the same values of legacy, measurement period is based on 3 sample. No further restriction of UE implantation of which one is used

        - Option 2: No requirements even the NW can configure it.

        - Option 3: FFS

  - For P (sharing factor) in L1_measurement periods:

    - For SSB based or CSI-RS based,

      - The definition is the same as legacy

      - The value can be the same or different for the current beam and new beam

In our view both the legacy values can be reused. In Figure 2-1 the circles in red indicate reference signals that are not available for measurements due to collisions with gaps causing P to become larger than one. In this approach the measurement period for the detecting beam is simply extended by a factor of P while for the non-detecting beam potentially more samples are taken before the event happened OTA. This is the same approach that we already agreed to for the beam sweeping factor. It can simply be extended to P > 1 and M > 1.

Figure 2-1: Measurement Period for Event-2 for P > 1

Proposal 1: For the case M > 1 and P > 1, the measurement period can be defined based on N*P*M*TSSB where M and P are the corresponding scaling factors for the detecting beam.

Proposal 2: RAN4 should define delay requirements for the measurement period of UE-initiated/event-driven beam management taking both possible values of M into account.

Issue 1-2 addresses the question whether TL1_measurement period should be defined based on the maximum of the L1 measurements periods for current and new beam or whether TL1_measurement period should be defined based on the beam that detects the event. It has been agreed that the triggered measurement reporting delay includes the Tfirst UL channel. Tfirst UL channel is from the time point at RS which triggered the L1 reporting to the time point at next first PUCCH

#### Issue 1-2: Components of Event triggered measurement reporting delay?

Agreement:

- The event triggered measurement reporting delay includes the Tfirst UL channel

  - Tfirst UL channel is from the time point at RS which triggered the L1 reporting to the time point at next first PUCCH

Further discuss the two options and select one option in the next meeting:

- The event triggered measurement reporting delay is TL1_measurement period + Tfirst UL channel

  - Option 1: if TL1_measurement period is different for current beam and new beam, the TL1_measurement period is maximum of the L1 measurement periods to measure both current beam and new beams. (Samsung, Xiaomi, Apple, MTK)

  - Option 2: If the event is triggered by the current beam measurement, the event triggered measurement delay is TL1_measurement periods_currentbeam. If the event is triggered by the new beam measurement, the event triggered measurement delay is TL1_measurement periods_newbeam.. (E///, Nokia, QC)

In Figure 2-1 it is seen that the measurements on current and new beam happen in a sliding window. The event is always triggered by one beam and the measurement of the other beam serves as the reference for event-2. The way the measurements are done in the UE leads very naturally to option 2.

Proposal 3: For the event triggered measurement reporting delay TL1_measurement period, Option 2 in Issue 1-2 should be adopted. The measurement reporting delay should be defined based on the beam that detects the event.

Figure 2-2 illustrates the meaning of the agreement that Tfirst UL channel is included in the measurement reporting delay. In the figure, Tfirst UL channel is the time after the L1 measurement. In discussions in RAN4 #114bis the opinion was shared that including Tfirst UL channel in the reporting delay means that after the event has been detected, no additional measurements are conducted even if SSB or CSI-RS fall into the time interval Tfirst UL channel. It is worthwhile capturing this point in an explicit agreement.

Observation 1: In discussions in RAN4 #114bis the opinion was shared that including Tfirst UL channel in the reporting delay means that after the event has been detected, no additional measurements are conducted even if SSB or CSI-RS fall into the time interval Tfirst UL channel.

Proposal 4: It should be explicitly captured in an agreement that the UE is not required to continue evaluating the event in the time interval Tfirst UL channel once the event has already been detected.

Figure 2-2: Illustration of Contributions to Measurement Reporting Delay

Another open topic in Issue 1-3 is whether RAN4 should define requirements both for the cases that no time window and that a time window is defined.

#### Issue 1-3: Whether RAN4 to specify requirements for no time window based or cover both no time window based and with time window based with eventInstanceCount-r19?

<Way forward >

- Option 1: (Qualcomm, Xiaomi, Apple, LGE)

  - Do not define requirements for eventInstanceCount-r19 > 1

- Option 1a: (Apple): To extend the requirements for case when eventInstanceCount-r19 > 1 is configured, add note longer delay is expected if eventInstanceCount-r19 > 1 is configured.

- Option 2: (Samsung, CMCC, Huawei, ZTE, MediaTek)

  - Specify requirements to cover both cases of no time window based and with time window based with configurable eventInstanceCount-r19

- Option 3: (Nokia, Ericsson)

  - Wait for further RAN1 conclusion

- For how to specify the requirements in option 2 above:

  - Option 2a:

    - Measurement reporting delay is TL1-RSRP_event triggered_measurement_Period * C + Tfirst UL channel given that measurement reporting delay ≤ Ttime_window, where,

    - TL1-RSRP_event triggered_measurement_Period: L1-RSRP measurement period in event triggered measurement reporting for either SSB or CSI-RS.

    - C: event instances threshold is configured by NW.

    - Ttime_window: time window is configured by NW.

    - Tfirst UL channel: the uncertainty time to send beam reporting notification on 1st PUCCH.

    - Other options are not precluded

Several proposals have been made in RAN4 #114bis how to specify requirements, however, all of them were premature since RAN1 or RAN2 has not yet defined how the measurement window should be defined. In RAN1 #120bis, RAN1 made the following agreement that the UE starts a timer for the new beam when event-2 is obtained in case the timer for the new beam is not yet running.

RAN1 #120bis Agreement

Regarding triggering event determination for Event 2, on the measurement window for initiating the UE-initiated/event-driven beam reporting procedure, support Option-4.

- Option-4: If an Event-2 instance for a new beam is obtained at the time t and the timer for the new beam is not running, UE starts the timer for the new beam, where the expiry time of the timer is equal to the NW-configured length of the time window (T_window)

  - Note: Timer is new beam specific.

- Note: T_window is the agreed time window parameter for measurement.

- Introduce separate UE capability to limit the number of timers. There is only 1 timer per new beam.

Above agreement is captured in RAN1 specifications.

Since RAN4 has agreed that the existing beam sweeping factor N is reused for UE-initiated/Event-driven beam management, the window-based approach needs to take the beam sweeping factor N, the factor P and the factor M  {1, 3} into account in case it is decided that requirements in RAN4 are defined. It is still unclear from this agreement in Option 4 how the RAN4 measurement procedure fits together with the agreement in RAN1. Based on this unclarity how existing RAN4 agreements fit together with the new RAN1 agreement, RAN4 should discuss whether a LS is sent to RAN1 seeking further clarity. In our opinion the existing measurement procedure in RAN4, mainly due to Rx beam refinement, is a sliding window-based approach while the RAN1 agreement seems rather to define a fixed window-based approach.

Proposal 5: RAN4 should discuss whether an LS should be sent to RAN1 summarizing the RAN4 agreements for the measurement periods and asking for further clarity of the RAN1 agreement.

However, it has been agreed in RAN4 #113 that the UE does not do any filtering of L1-RSRP measurements if a configurable time window is applied. If no filtering is applied, it is not obvious whether additional reporting delay requirements for the configurable time window needs to be defined. The measurement would simply be extended by a factor based on the configured value M. Hence, we do not see that defining additional requirements tests much more functionality of the UE.

Observation 2: In case no filtering is applied in the UE, the measurement period for the time window approach would simply be extended by an additional scaling. No major UE implementation would be tested.

Proposal 6: RAN4 should not define requirements for the window-based approach whether the event has occurred M times.

In Issue 1-5 it was discussed whether core requirements for CSI-RS based L1-RSRP measurement reports should be introduced for inter-cell beam management. Section 9.13 defines L1-RSRP measurement reporting requirements for inter-cell beam management for SSB only. However, section 9.5 defines L1-RSRP measurement reporting requirements for CSI-RS without clearly mentioning whether this is for serving cell or neighbor cells. Hence, it is left in a somewhat vague status whether there are requirements for CSI-RS based L1-RSRP for inter-cell beam management.

#### Issue 1-5: CSI-RS based L1-RSRP for inter-cell BM

<Way forward >

- Option 1:

  - Not introduce in Rel-19 CSI-RS based L1-RSRP measurements for inter-cell beam management

- Option 2:

  - Reuse the same requirements structure as that in Rel-17 9.5. No need to add sharing factor for serving cell CSI-RS and NSC CSI-RS. From the configuration of CSI-RS perspective, there is only one configuration. If this CSI-RS is QCLed to the SSB of serving cell or SSB of additionPCI cell, they use the same set of requirements.

Our understanding of option 2 is that section 9.5 is used as a baseline for defining similarly vague requirements for CSI-RS based L1-RSRP for UE-initiated/event-triggered beam management but section 9.13 is not updated for introducing CSI-RS based requirements. Since almost all companies share the same that this is the best way forward, we agree with that approach as well.

Proposal 7: RAN4 could reuse the same requirement structure as in section 9.5 for CSI-RS L1-RSRP for inter-cell beam management. Section 9.13 of TS 38.133 is not impacted.

Another discussion in RAN4 #114bis is captured in Issue 1-6 whether new TCI switching delay requirements should be introduced.

#### Issue 1-6: Whether to specify new unified TCI switching delay requirements?

<Way forward >

- For MAC-CE based DL/UL TCI state switch delay

  - Option 1: (Qualcomm, Apple, Samsung (for known case), Ericsson)

    - The existing requirements for TCI state switch delay should be reused for UE-initiated/even-triggered beam management

  - Option 2: (ZTE)

    - Remove components of TL1-RSRP and Tfirst-SSB+TSSB-proc

  - Option 3: (MediaTek)

    - FFS on whether to remove Tfirst-SSB+TSSB-proc

- For DCI based DL/UL TCI state switch delay

  - Option 1:

    - The existing requirements for TCI state switch delay should be reused for UE-initiated/even-triggered beam management

  - Option 2:

    - Whether to further reduce the delay, wait for further RAN1 progress

In RAN4 #114bis, there were proposals to remove TL1-RSRP and Tfirst-SSB + TSSB-proc from the existing requirement. The existing requirement for MAC CE based TCI state switch for a known TCI state says that the switch occurs in slot n + THARQ + 3Nslot TOk * (Tfirst-SSB + TSSB-proc) with TOk = 1 if the target state is not in the active TCI state list and TOk = 0 otherwise. Removal of Tfirst-SSB + TSSB-proc from the delay requirement means essentially that the new beam is always treated as if it has been in the active TCI state list even if it has not yet been activated.

This discussion in RAN4 is related to the discussion in RAN1 #120bis that could not be concluded in RAN1 but has been moved to RAN4 for further discussion.

Proposal 5.1’:  Send an LS to RAN4 for informing the following potential enhancement on the reduction of beam application latency after a UEI/ED beam report is sent:

  - Alt-3: After sending a UE-initiated beam report [on Event-7], the UE should store the QCL properties of the SSB associated with the reference signal reported in the beam report, subject to UE capability.

    - In such case, at the reception of a subsequent reception of Unified TCI States Activation/Deactivation MAC CE activating a TCI state associated with the report RS, the UE activates the TCI state without additional SSB reception if all activated TCI states are associated with the SSB(s) corresponding to the reference signal reported in the beam report.

    - [At the reception of a subsequent reception of RRC signaling to reconfigure the reference signal in TCI-State or TCI-UL-State, the UE activates the TCI state without additional SSB reception if all reconfigured TCI states are associated with the SSB(s) associated with the reference signal reported in the beam report.]

Whether/how to support the potential enhancement is up to RAN4.

The RAN1 proposal says that the QCL properties of the SSB associated with the reference signal reported in the beam report should be stored by the UE such that the UE can activate the TCI state without additional SSB reception. Like the discussion in RAN4, this proposal demands the UE to treat the new beam the same way as the beams in the active TCI state list. One more problem with the RAN1 proposal is that it remains unclear how long the UE should store the beam. Since it is not known to the UE when the network triggers a beam switch, the UE must potentially store the QCL properties for a very long time.

Observation 3: RAN1 proposal 5.1’ demands the UE to treat the new beam the same way as the beams in the active TCI state list.

Observation 4: Since it is not known to the UE when the network triggers a beam switch, the UE must potentially store the QCL properties for a very long time.

We do not see the benefits of this RAN1 proposal over swapping the beams in the active TCI state list or by enlarging the size of the active TCI state list. There is already a UE capability to support up to eight active TCI states. We do not see why this size should not be enough to avoid the additional switching delay and why an additional beam should be considered that is not yet in the active TCI state list. Based on current RAN1 agreements, we believe that existing TCI state delay switches should be reused for UE-initiated/event-triggered beam management

Observation 5: Existing UE capabilities for the size of active TCI state list should be sufficient to avoid additional switching delays.

Proposal 8: The existing requirements for TCI state switch delay should be reused for UE-initiated/even-triggered beam management.

Under Issue 1-10 it was discussed whether capabilities for event triggering and reporting criteria should be introduced. Obviously, the number of parallel measurements a UE can support is limited and for example given by the number of CSI-RS resources a UE can support. The evaluation of the various events adds additional processing load. In this sense it can well be that the number of simultaneous events should be limited, either in RAN1 or RAN4 spec. But for doing a proper analysis, first the definition of the core requirements and UE capabilities should be finalized and then RAN1/RAN4 working groups can investigate potential limitations.

#### Issue 1-10: Capabilities for Support of Event Triggering and Reporting Criteria (similar Capabilities in 38.133 9.1.4)?

<Way forward >

Further discuss the two options. Proponents of option 2 to bring proposals on the exact numbers for the capabilities.

- Option 1:

  - For event trigger L1 reporting, do not define such capability in parallel per category like L3.

- Option 2:

  - Define capabilities for support of UE-initiated/event-driven beam reporting similar as L3.

Observation 6: The number of parallel measurements a UE can support is limited and for example given by the number of CSI-RS resources a UE can support. The evaluation of the various events adds additional processing load.

Proposal 9: First the definition of the core requirements and UE capabilities should be finalized and then RAN1/RAN4 working groups can investigate potential limitations of the number of event triggering and reporting criteria.

# Conclusions

In this paper we provided our views on several of the open issues identified in RAN4 #114. Our observations and proposals are summarized as follows:

Proposal 1: For the case M > 1 and P > 1, the measurement period can be defined based on N*P*M*TSSB where M and P are the corresponding scaling factors for the detecting beam.

Proposal 2: RAN4 should define delay requirements for the measurement period of UE-initiated/event-driven beam management taking both possible values of M into account.

Proposal 3: For the event triggered measurement reporting delay TL1_measurement period, Option 2 in Issue 1-2 should be adopted. The measurement reporting delay should be defined based on the beam that detects the event.

Observation 1: In discussions in RAN4 #114bis the opinion was shared that including Tfirst UL channel in the reporting delay means that after the event has been detected, no additional measurements are conducted even if SSB or CSI-RS fall into the time interval Tfirst UL channel.

Proposal 4: It should be explicitly captured in an agreement that the UE is not required to continue evaluating the event in the time interval Tfirst UL channel once the event has already been detected.

Proposal 5: RAN4 should discuss whether an LS should be sent to RAN1 summarizing the RAN4 agreements for the measurement periods and asking for further clarity of the RAN1 agreement.

Observation 2: In case no filtering is applied in the UE, the measurement period for the time window approach would simply be extended by an additional scaling. No major UE implementation would be tested.

Proposal 6: RAN4 should not define requirements for the window-based approach whether the event has occurred M times.

Proposal 7: RAN4 could reuse the same requirement structure as in section 9.5 for CSI-RS L1-RSRP for inter-cell beam management. Section 9.13 of TS 38.133 is not impacted.

Observation 3: RAN1 proposal 5.1’ demands the UE to treat the new beam the same way as the beams in the active TCI state list.

Observation 4: Since it is not known to the UE when the network triggers a beam switch, the UE must potentially store the QCL properties for a very long time.

Observation 5: Existing UE capabilities for the size of active TCI state list should be sufficient to avoid additional switching delays.

Proposal 8: The existing requirements for TCI state switch delay should be reused for UE-initiated/even-triggered beam management.

Observation 6: The number of parallel measurements a UE can support is limited and for example given by the number of CSI-RS resources a UE can support. The evaluation of the various events adds additional processing load.

Proposal 9: First the definition of the core requirements and UE capabilities should be finalized and then RAN1/RAN4 working groups can investigate potential limitations of the number of event triggering and reporting criteria.

# References

- R4-2504902, WF on RRM requirements for NR_MIMO_Ph5_Part1, RAN4 #114bis, April 2025

- R1-2503092, Moderator Summary #4 on UE-initiated/event-driven beam management, RAN1 #120bis, April 2025