3GPP TSG-RAN WG4 #115	R4-2505352

St. Julians, Malta, May 19-23, 2025

Agenda item:	7.24.5		RRM performance requirements

Source:	Qualcomm Incorporated

Title: 						On RRM performance requirements for NR MIMO Phase 5

Document for:	Discussion

# Introduction

In RAN4 #112, RAN4 work on Rel-19 MIMO Phase 5 was started, the WID is provided in [1]. According to the work plan, the work on RRM performance requirements will start in RAN4 #115.

In this paper, we provide our very preliminary thoughts on the RRM performance test cases for Rel-19 MIMO Phase 5.

# Discussion

We provide a first list of scenarios that should be considered in the Rel-19 RRM performance requirements and in the design of the corresponding test cases.

- UE-initiated/event-triggered beam management

  - Event-2 and event-7 measurement reporting delay:

    - Current and new beam in list of active beams

    - Current beam is active, new beam is not active

  - Event-1 measurement reporting delay

  - Beam switching delay for known and unknown TCI state for all three events

  - Event-triggering for intra-cell beam management for SSB and CSI-RS

  - Event-triggering for inter-cell beam management for SSB only

- UE reporting enhancements for CJT deployments

  - Focus should be TDD 3.5 GHz, SCS = 30 kHz

  - The test should define absolute phase, frequency and delay offset accuracy requirements subject to the number of samples

    - For example, 19 ns for delay offset and 3 Hz for frequency offset reporting

    - The SNR and the number of samples should be defined accordingly to achieve the absolute accuracy requirement.

- Asymmetric DL sTRP / UL mTRP

  - Focus is on FR1 and FR2 scenarios

  - For FR2, tests should only cover scenario #2 (UL TRP transmits SSB)

  - For FR1, both scenario #1 (UL TRP does not transmit SSB) and scenario #2 (UL TRP transmits SSB) should be tested

  - Uplink TCI switching both for Rel-17 and Rel-18 unified TCI state.

    - Separate UL TCI  separate UL TCI

    - Joint TCI  separate UL TCI and separate UL TCI  separate UL TCI

# Conclusions

In this paper we provided our very preliminary thoughts on the RRM performance test cases for the Rel-19 MIMO Phase 5. RAN4 should consider these proposals for the definition of the performance tests.

# References

- RP-242394, WID revision, NR MIMO Phase 5, RAN #105, September 2025