3GPP TSG-RAN WG4 #115	R4-2505355

St. Julians, Malta, May 19-23, 2025

Agenda item:	7.27.3.3		On-demand SIB1

Source:	Qualcomm Incorporated

Title:	On core requirements for on-demand SIB1

Document for:	Discussion

# Introduction

In RAN4 #114bis the work on RRM core requirements for OD-SIB1 has been started and first agreements have been made in [1].

In this paper we provide our view for the open issues identified for the way forward in [1] in RAN4 #114bis

# Discussion

The scenario under discussion in Rel-19 for OD-SIB1 is shown in Figure 2-1. The UE receives UL WUS config from cell A, transmits UL WUS to the NES cell and receives the on-demand Sib1 from the NES cell as well. OD-SIB1 only applies to UEs in idle/inactive mode. Hence, the potential impact for RRM requirements is only in these modes but not for RRC_CONNECTED.

Figure 2-1: OD-SIB1 Design Case 2

In RAN4 #114bis the following agreement has been made:

Issue 2-1-2: RRM requirements impacts for OD-SIB1 – timing

Agreement

- Clarify in transmit timing requirements to include UL WUS transmission (PRACH) for OD-SIB1:

  - reference cell is the NES cell

  - NTA for UL-WUS is defined as 0

  - NTA offset is the [n-TimingAdvanceOffset] from UL-WUS configuration if configured, if not, follow the default value defined in Table 7.1.2-2 TS 38.133

  - Note: The IE can be updated based on other WG’s progress.

In our understanding the agreement for UL WUS transmission timing means that PRACH as the UL-WUS signal is transmitted to the NES cell and follows legacy timing requirements. No changes to PRACH timing are made.

Observation 1: The agreement in Issue 2-1-2 for the UL WUS timing follows legacy timing requirements. No changes to PRACH timing are made.

Paging interruption was identified in RAN4 #114bis as a topic with impact on RRM requirements (Issue 2-1-1).

Issue 2-1-1: RRM requirements impacts for OD-SIB1 – paging interruption

Way forward:

    - Option A: Clarify that paging interruption includes UL WUS transmission and RAR reception for OD-SIB1 acquisition. (HW, OPPO, Apple, Nokia, Xiaomi, CATT, MTK, vivo)

    - Option B: No impacts on core requirements. Discuss whether to extend TSI-NR in test case. (CMCC, QC, LGE)

    - Option C: No RRM impacts on the requirements, i.e., UL-WUS can be postponed during the paing time. (E///, LGE, MTK)

Sending UL WUS, receiving the RAR and monitoring / decoding OD-SIB1 are steps before the UE camps on the NES cell. This may have an impact of the maximum interruption in paging reception. In TS 38.133, the maximum allowed interruption time is defined as TSI-NR + 2*Ttarget_cell_SMTC_period. Ttarget_cell_SMTC_period is the periodicity of the configured SMTC of the target cell and TSI-NR is the time required for receiving all relevant system information. In some tests, TSI-NR is assumed to be 1280 ms.

Since the core requirement for the maximum interruption time is written in a very generic manner, the core requirement is not impacted by OD-SIB1. Hence no clarification is required that paging interruption includes UL WUS transmission and RAR reception for OD-SIB1 acquisition. It should be further investigated whether TSI-NR = 1280 ms can cover UL-WUS transmission and OD-SIB1 reception as well. If this is not the case, in the performance requirements a larger TSI-NR could be defined for OD-SIB1. Regarding Option C in Issue 2-1-1, it is not clear what it means that UL-WUS can be postponed during the paging time. It requires further clarification, what the paging time is and how often the UL-WUS can be shifted in case the allowed interruption time is larger than the paging cycle. It needs further discussion whether in this case the impact to paging could be fully avoided.

Observation 2: RAN4 should further clarify Option C, e.g., how often the UL-WUS can be shifted in case the allowed interruption time is larger than the paging cycle.

Observation 3: Since the core requirement for the maximum interruption time is very generic, the core requirement for paging interruption is not impacted by OD-SIB1.

Proposal 1: RAN4 should further investigate whether TSI-NR = 1280 ms can cover UL-WUS transmission and OD-SIB1 reception as well or whether in the performance tests a larger TSI-NR should be defined.

In RAN1 #120, also the scenario was discussed that the NES cell is already transmitting OD-SIB1, and the UE may not need to send UL WUS to request OD-SIB1. In RAN4 #120bis it has been decided that it is up to UE implementation whether to monitor Type-0 PDCCH for SIB1 transmission before UL WUS is transmitted. Hence, also no requirements should be defined for the UE.

Observation 4: It is up to UE implementation whether the UE monitors type-0 PDCCH for OD-SIB1 in case that OD-SIB1 is already transmitted by the NES cell.

Proposal 2: RAN4 should not define interruption requirements for the case that the UE monitors OD-SIB1 before UL WUS is transmitted.

# Conclusions

In this paper we provided our views on several of the open issues identified in RAN4 #114bis. Our observations and proposals are summarized as follows:

Observation 1: The agreement in Issue 2-1-2 for the UL WUS timing follows legacy timing requirements. No changes to PRACH timing are made.

Observation 2: RAN4 should further clarify Option C, e.g., how often the UL-WUS can be shifted in case the allowed interruption time is larger than the paging cycle.

Observation 3: Since the core requirement for the maximum interruption time is very generic, the core requirement for paging interruption is not impacted by OD-SIB1.

Proposal 1: RAN4 should further investigate whether TSI-NR = 1280 ms can cover UL-WUS transmission and OD-SIB1 reception as well or whether in the performance tests a larger TSI-NR should be defined.

Observation 3: It is up to UE implementation whether the UE monitors type-0 PDCCH for OD-SIB1 in case that OD-SIB1 is already transmitted by the NES cell.

Proposal 2: RAN4 should not define interruption requirements for the case that the UE monitors OD-SIB1 before UL WUS is transmitted.

# References

- R4-2504907, WF on RRM requirements for Netw_Energy_NR_enh_Part2, RAN4 #114bis, April 2025