3GPP TSG-RAN WG4 #115	R4-2505357

St. Julians, Malta, May 19-23, 2025

Agenda item:	7.27.5		UE and BS demodulation performance requirements

Source:	Qualcomm Incorporated

Title:	On demodulation performance requirements for Rel-19 NES

Document for:	Discussion

# Introduction

The work item description of Rel-19 network energy saving is provided below [1]:

- Specify procedures and signaling method(s) to support on-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA. [RAN1/2/3/4]

  - Specify triggering method(s) (select from UE uplink wake-up-signal using an existing signal/channel, cell on/off indication via backhaul, Scell activation/deactivation signaling)

  - Note1: On-demand SSB transmission can be used by UE for at least SCell time/frequency synchronization, L1/L3 measurements and SCell activation, and is supported for FR1 and FR2 in non-shared spectrum.

- Specify support for on-demand SIB1 for UEs in idle/inactive mode [RAN1/2/3]

  - Specify procedures and signaling method(s) for Case 2 [RAN1/2]

    - Case 2: UE obtains UL WUS configuration from Cell A, UE transmits UL WUS on NES Cell, UE receives on-demand SIB1 from NES Cell

    - Triggering method by UL WUS using PRACH

  - Specify inter NG-RAN node signalling at least for the configuration of UL WUS [RAN3]

  - Note 1: No modification of SSB will be discussed under this objective

  - Note 2:

    - UL WUS: Uplink wake-up signal

    - Cell A: A cell that is periodically transmitting at least its own SIB1

    - NES Cell: A cell that may transmit SIB1 transmission in response to UL WUS from a UE

  - Note 3:

    - RAN1 strives to minimize impact to legacy UE

    - RAN1 specification impact to support this feature should be minimized

- Specify adaptation of common signal/channel transmissions. [RAN1/2/3/4]

  - Adaptation of SSB in time domain, e.g. adapting periodicity

  - Adaptation of PRACH in time domain

  - Adaptation of paging occasions including confining the paging occasions in the time domain

    - Note: there shall be no paging latency increase

  - Note: there shall be no negative impact to legacy UEs, unless significant benefits are shown

- Specify the corresponding core requirements, for the above features [RAN4].

The RAN4 work on the RRM core requirements started in RAN4 #114 and the work of the RRM performance requirements is about to start in RAN4 #115. The work plan for the RRM requirements has been defined in [1].

In this paper we provide our view for the UE and BS demodulation performance requirements.

# Discussion

Topics 1 and 3 deal with on-demand SSB (OD-SSB) and SSB adaptation. On-demand SSB can be activated and deactivated by a MAC CE command and is a periodic regular SSB as long as it is active. The range of supported periodicities of OD-SSB, in case it is active, is identical to the supported periodicities of a regular SSB. SSB adaptation refers to the change of the SSB burst periodicity based on DCI 2-9 and potentially also based on MAC CE in case RAN2 still defines a corresponding MAC CE. The SSB signal itself is unchanged by SSB adaptation. Like OD-SSB, the new periodicity after adaptation is within the supported periodicities of legacy SSB bursts. Since neither the signal structure nor the supported range of SSB burst periodicities change in Rel-19 NES work item, we do not think that UE demod requirements are needed.

PRACH

Proposal 1: RAN4 should not introduce UE demod requirements for OD-SSB and SSB adaptation since neither the signal structure nor the supported range of SSB burst periodicities change in Rel-19 NES work item.

Topic 2 deals with on-demand SIB1 (OD-SIB1) based on a UL WUS sent by the UE. OD-SIB1 does not have any differences compared to legacy SIB1. Hence, no additional demod requirements need to be introduced for OD-SIB1.

Proposal 2: RAN4 should not introduce UE demod requirements for OD-SIB1.

The OD-SIB1 is triggered by a UL WUS signal that is sent to the network. The UL WUS signal is a legacy PRACH. Since RAN1 did not agree on supporting UL WUS repetition in Rel-19, detection probability and false alarm rate are unchanged and no requirements in 38.104 need to be introduced.

Observation 1: Detection probability and false alarm rate are unchanged in case of OD-SIB1.

Proposal 3: No additional requirements for PRACH in case of OD-SIB1 need to be introduced in 38.104.

Topic 3 also introduces PRACH adaptation in time domain. Additional resources for PRACH transmission can be enabled if triggered by the network. However, the PRACH itself is unchanged. Hence, detection probability and false alarm rate are unchanged and no requirements in 38.104 need to be introduced.

Observation 2: Detection probability and false alarm rate are unchanged in case of PRACH adaptation.

Proposal 4: No additional requirements for PRACH adaptation need to be introduced in 38.104.

# Conclusions

In this paper we provided our views on several of the open issues identified in RAN4 #114bis. Our observations and proposals are summarized as follows:

Proposal 1: RAN4 should not introduce UE demod requirements for OD-SSB and SSB adaptation since neither the signal structure nor the supported range of SSB burst periodicities change in Rel-19 NES work item.

Proposal 2: RAN4 should not introduce UE demod requirements for OD-SIB1.

Observation 1: Detection probability and false alarm rate are unchanged in case of OD-SIB1.

Proposal 3: No additional requirements for PRACH in case of OD-SIB1 need to be introduced in 38.104.

Observation 2: Detection probability and false alarm rate are unchanged in case of PRACH adaptation.

Proposal 4: No additional requirements for PRACH adaptation need to be introduced in 38.104.

# References

- RP-250343, Revised WID on enhancements of network energy savings for NR, RAN4 #107, March 2025

- R4-2412508, Work plan for R19 NES, RAN4 #112, August 2024