# 3GPP TSG-RAN WG4 Meeting #115		R4-2505378

Malta, May 19 - 23 2025

Source:	China Telecom

Title:	Discussion on UE demodulation performance requirements for 8Rx with MMSE-IRC

Agenda Item:	7.17.3

Document for:	discussion

Introduction

In RAN#104 meeting, the Rel-19 new WID on NR demodulation performance: Phase 5 is approved.

The following objectives are approved for UE with 8Rx:

- Define the following UE performance requirements for 8Rx CPE/FWA/vehicle/industrial devices:

  - PDSCH demodulation and CQI reporting requirements with inter-cell interference with MMSE-IRC receiver

    - Reuse the Rel-17 interference model for 2/4Rx UE as a baseline

  - PDSCH demodulation requirements with intra-cell inter-user interference with MMSE-IRC receiver

    - Reuse the existing interference model for 2/4Rx UE as a baseline

The WF was approved in [1]. In this contribution, our views on the open issues are given.

Discussion

General

Definition of MMSE-IRC receiver capability for 8Rx UE

Status in the WF [1]: Agreement: - Redefine the simplified and baseline receiver definitions in 38.101-4 section 3.1 and 38.306 section 5.2 to the following: - Baseline 8Rx MMSE-IRC Receiver: 8Rx MMSE-IRC receivers for MIMO transmissions [with support of] up to 8 layers with joint 8Rx MIMO detector. - Simplified 8Rx MMSE-IRC Receiver: 8Rx MMSE-IRC receivers for MIMO transmissions [with support of] only up to 4 layers with two joint 4Rx MIMO detectors.

In the last meeting, the re-definition of 8Rx UE receiver is almost agreed with remaining square brackets to ‘with support of’ as shown above. And there were proposals on using ‘detection’.

As per the MMSE-IRC receiver implementation assumption as captured in TR38.833 (copied below), the interference covariance matrix is derived simply by extracting the target UE’s DMRS symbols from the total received signal, when co-scheduled UE is in the same CDM group with target UE. Therefore, the MMSE-IRC receiver does not need to ‘detect’ data layers from the co-scheduled UE.

Observation 1: As per the MMSE-IRC receiver implementation assumption as captured in TR38.833, interference covariance matrix is derived simply by extracting the target UE’s DMRS symbols from the total received signal.

We understand there could be different implementations for MMSE-IRC in different chipset products, but we believe the original wording ‘with support of up to X layers’ has been a very general expression and it is also aligned with the MMSE-IRC receiver implementation assumption as captured in TR38.833.

- 4.2.2 MMSE IRC receiver To suppress the co-scheduled UE’s interference, the candidate MMSE IRC receiver type is captured in this subclause. The MMSE IRC receiver weight matrix is expressed as follow: WRX,1k,l=H1H(k,l)R-1 R= P1H1k,lH1Hk,l+1Nrek,l∈DMRS of NUEr(k,l)r(k,l)H, rk,l=rk, l-H1(k,l)d1k,l if co-scheduled UE is in the same CDM group with target UE, and rk,l=rk, l, if co-scheduled UE is in the different CDM group with target UE. Where, HjHk,l and d1(k,l) denote the estimated channel matrix and the transmit signal of target UE’s DMRS symbols, respectively, where the estimated channel matrix is also based on DMRS. Nre is the number of sampling REs of intra-user’s DMRS. P1 is the transmission power of the serving cell and is equal to Ed1k,l2.

Proposal 1: Redefine the simplified and baseline receiver definitions in 38.101-4 section 3.1 and 38.306 section 5.2 to the following:

-   Baseline 8Rx MMSE-IRC Receiver: 8Rx MMSE-IRC receivers for MIMO transmissions with support of up to 8 layers with joint 8Rx MIMO detector.

-   Simplified 8Rx MMSE-IRC Receiver: 8Rx MMSE-IRC receivers for MIMO transmissions with support of only up to 4 layers with two joint 4Rx MIMO detectors.

Applicability rule and release independent

Test applicability

Status in the WF [1]: Test applicability to skip 2/4Rx requirements Proposals - Option 1: For UEs supporting 8Rx, 2Rx and/or 4Rx, if UE passes 8Rx tests for inter-cell interference (PDSCH and CQI) and intra-cell inter-user interference (PDSCH), the 2 and/or 4Rx tests for inter-cell interference (PDSCH and CQI) and intra-cell inter-user interference (PDSCH) can be skipped, no matter same test configurations as 2RX/4RX are defined or not. - Option 2: To consider if UEs supporting both 8Rx and 2/4Rx, the 2/4Rx tests under inter cell interference and under intra cell inter user interference can be skipped if the corresponding 8Rx requirements are passed. - Option 3: For UEs supporting both 8Rx and 2/4Rx, the 2/4Rx tests under inter cell interference and under intra cell inter user interference can be skipped if the corresponding 8Rx requirements are passed. - Option 4: Introduce test applicability under inter-cell interference and under intra-cell inter user interference to skip a 2/4Rx requirement if at least the following applies for each considered 8Rx test: - UE support both 8Rx and 2/4Rx - The UE passes an 8Rx requirement with same rank and interference configuration as the 2/4Rx requirement. - Option 5: RAN4 shall have the following applicability rules: - UE that passes Rank2 ICI test case with 8Rx shall skip Rank1 ICI requirements with 4Rx. - UE that passes Rank2 MU-MIMO test case with 8Rx shall skip Rank1/2 MU-MIMO requirements with 4Rx.

For UEs supporting both 8Rx and 2/4Rx, in Rel-18, we have already introduced test applicability rules to skip the 2/4Rx requirements if the corresponding 8Rx requirements are passed as shown below.

We believe the same approach can be reused for the Rel-19 new requirement to be defined.

Table 5.1.1.2-1: Requirements applicability Supported RX antenna ports; Test type; Test list. UE supports 8Rx, 4Rx and 2Rx; PDSCH; All tests in Clause 5.2.3 (Note 2, 3) All tests in Clause 5.2.4 (Note 2); If UE passes tests in Clause 5.2.4, UE can skip Test 2-1 and Test 2-2 in Clause 5.2.3 Table 5.2.3.1.1-4, Table 5.2.3.2.1-4 and Test 4-1 in Clause 5.2.3 Table 5.2.3.1.1-6, Table 5.2.3.2.1-6. or; PDCCH; All tests in Clause 5.3.3 (Note 2,3). UE supports both 8Rx and 4Rx; PBCH; All tests in Clause 5.4.3 (Note 1). UE supports both 8Rx and 2Rx; PDSCH; All tests in Clause 5.2.2 (Note 2, 4) All tests in Clause 5.2.4 (Note 2); If UE passes tests in Clause 5.2.4, UE can skip Test 2-1 and Test 2-2 in Clause 5.2.2 Table 5.2.2.1.1-4, Table 5.2.2.2.1-4. PDCCH; All tests in Clause 5.3.2 (Note 2,4). PBCH; All tests in Clause 5.4.2.

Regarding the meaning of ‘corresponding’, we think if 8Rx UE has passed test for the one scenario, it can skip test for 2/4Rx with the same scenario, i.e., option 1.

Proposal 2: For UEs supporting 8Rx, 2Rx and/or 4Rx, if UE passes 8Rx tests for inter-cell interference (PDSCH and CQI) and intra-cell inter-user interference (PDSCH), the 2 and/or 4Rx tests for inter-cell interference (PDSCH and CQI) and intra-cell inter-user interference (PDSCH) can be skipped, no matter same test configurations as 2RX/4RX are defined or not.

Release independent

Status in the WF [1]: Agreement: - The new Rel-19 requirements shall at least be release independent from Rel-18. Remaining proposals: - Option 1: The new Rel-19 requirements shall also be release independent from Rel-17. - Option 2: The new Rel-19 requirements shall also be release independent from Rel-17. The requirements shall be mandatory for Rel-18 and optional from Rel-17.

For release independent, the 8Rx UE feature is introduced in Rel-18 with MMSE-IRC as the receiver assumption. The corresponding 8Rx UE performance requirements are release independent from Rel-17 as specified in TS38.307. In addition, the UE MMSE-IRC is a mandatory feature for all Rel-17 and forward 2/4Rx UEs:

Table 5.4-1: Additional requirements of other release independent features Feature; Release independent from; Requirements to be fulfilled (see 38.307 of the REL when the feature was introduced); Further information. UE demodulation and CSI requirements for MMSE-IRC receiver for scenarios with inter cell and intra cell inter user interference; Rel-15; Table B.3.3-1; Rel-17 WI NR_demod_enh2-Perf: see Table B.3.3-1. These requirements are optional for Rel-15 and Rel-16 UEs and can be executed based on UE declaration.. Demodulation requirements for 8Rx UE; Rel-17; Table B.3.5-1. Table B.3.5-1: UE PDSCH demodulation requirements with 8Rx Section / Clause; Description. 5.2.4.1; PDSCH demodulation requirements for 8Rx FDD. 5.2.4.2; PDSCH demodulation requirements for 8Rx TDD. 5.2A.4; PDSCH CA demodulation requirements for 8Rx. 6.2.4; CQI requirements with 8Rx.

We have already discussed the possibility to cover MMSE-IRC requirements during the performance part discussion under WI ‘NR_ENDC_ RF_FR1_enh2’. Such requirements are not introduced in Rel-18 only because of the limited time and huge workload. Therefore, we think the new requirements for 8Rx should also be release independent from Rel-17.

We do not believe the 8Rx UE MMSE-IRC receiver implementation could be different in Rel-17, therefore we do not see the reason to make the Rel-17 test requirements optional for 8Rx UE particularly.

Proposal 3: The new Rel-19 requirements for 8Rx should be release independent from Rel-17.

Proposal 4: There is no need to make the Rel-17 test requirements optional for 8Rx UE particularly.

Conclusion

Proposal 1: Redefine the simplified and baseline receiver definitions in 38.101-4 section 3.1 and 38.306 section 5.2 to the following:

-   Baseline 8Rx MMSE-IRC Receiver: 8Rx MMSE-IRC receivers for MIMO transmissions with support of up to 8 layers with joint 8Rx MIMO detector.

-   Simplified 8Rx MMSE-IRC Receiver: 8Rx MMSE-IRC receivers for MIMO transmissions with support of only up to 4 layers with two joint 4Rx MIMO detectors.

Proposal 2: For UEs supporting 8Rx, 2Rx and/or 4Rx, if UE passes 8Rx tests for inter-cell interference (PDSCH and CQI) and intra-cell inter-user interference (PDSCH), the 2 and/or 4Rx tests for inter-cell interference (PDSCH and CQI) and intra-cell inter-user interference (PDSCH) can be skipped, no matter same test configurations as 2RX/4RX are defined or not.

Proposal 3: The new Rel-19 requirements for 8Rx should be release independent from Rel-17.

Proposal 4: There is no need to make the Rel-17 test requirements optional for 8Rx UE particularly.

Reference

R4-2504694 Way forward for [114bis][322] NR_demod_Ph5_Part2_UE, Nokia, TSG-RAN WG4 Meeting #114bis