3GPP TSG RAN WG4 Meeting #115                  	R4-2505388

Malta, MT, 19th - 23rd May, 2025

Title:	RRM requirements on UE-initiated/event-driven beam management

Source:	China Telecom

Agenda item:			7.24.4.1

Document for:	Discussion

- Introduction

In the RAN4 #114bis meeting, the RRM requirements for NR_MIMO_Ph5_Part1 were discussed and WF [1] has been approved. In this paper, we present our views on some open issues.

- Discussion

Issue 1-2: Components of Event triggered measurement reporting delay?

The following agreement and options are captured in the WF:

RAN4 #113 Agreement:

- Start point of Event triggered measurement reporting delay: reuse the same definition in L3 event trigger report which is the event is met over the air.

- ……

RAN4 #113 Agreement: For specifying the RAN4 delay requirement for event 2

- Measure both the current beam and new beam after the starting point, FFS the details.

- Note: this does not imply to define the delay requirement based on the sum of the current beam measurement period and new beam measurement period, which is pending on further RAN1 discussion.

- ……

RAN4 #114bis Agreement:

- The event triggered measurement reporting delay includes the Tfirst UL channel

  - Tfirst UL channel is from the time point at RS which triggered the L1 reporting to the time point at next first PUCCH

Further discuss the two options and select one option in the next meeting:

- The event triggered measurement reporting delay is TL1_measurement period + Tfirst UL channel

  - Option 1: if TL1_measurement period is different for current beam and new beam, the TL1_measurement period is maximum of the L1 measurement periods to measure both current beam and new beams. (Samsung, Xiaomi, Apple, MTK)

  - Option 2: If the event is triggered by the current beam measurement, the event triggered measurement delay is TL1_measurement periods_currentbeam. If the event is triggered by the new beam measurement, the event triggered measurement delay is TL1_measurement periods_newbeam.. (E///, Nokia, QC)

For the measurement reporting delay, based on the previous agreements, the starting point is the time when the event is met over the air, and both the current beam and new beam after the starting point will be measured. For option 2 of L1 measurement period, if in some cases, the L1 measurement period is shorter than the period in option 1, it means that either the current beam or the new beam is actually measured before the starting point.

Therefore, we support option 1 for L1 measurement period which aligns with the previous agreements.

Proposal 1: If TL1_measurement period is different for current beam and new beam, the TL1_measurement period is maximum of the L1 measurement periods to measure both current beam and new beams.

#### Issue 1-3: Whether RAN4 to specify requirements for no time window based or cover both no time window based and with time window based with eventInstanceCount-r19?

The following options are captured in the WF:

- Option 1: (Qualcomm, Xiaomi, Apple, LGE)

  - Do not define requirements for eventInstanceCount-r19 > 1

- Option 1a: (Apple): To extend the requirements for case when eventInstanceCount-r19 > 1 is configured, add note longer delay is expected if eventInstanceCount-r19 > 1 is configured.

- Option 2: (Samsung, CMCC, Huawei, ZTE, MediaTek)

  - Specify requirements to cover both cases of no time window based and with time window based with configurable eventInstanceCount-r19

- Option 3: (Nokia, Ericsson)

  - Wait for further RAN1 conclusion

In option 2, it specifies requirement for time window based case as well. Although the requirement for time window based case is more like functionality verification, it is preferred to introduce the requirement given it is a useful function to avoid ping-pong.

Regarding the test case design and test applicability rule for UE supporting both case 1 and case 2, we can further discuss in the performance part.

Proposal 2: Specify requirements to cover both cases of no time window based and with time window based with configurable eventInstanceCount-r19.

Issue 1-5: CSI-RS based L1-RSRP for inter-cell BM

The following options are captured in the WF:

- Option 1:

  - Not introduce in Rel-19 CSI-RS based L1-RSRP measurements for inter-cell beam management

- Option 2:

  - Reuse the same requirements structure as that in Rel-17 9.5. No need to add sharing factor for serving cell CSI-RS and NSC CSI-RS. From the configuration of CSI-RS perspective, there is only one configuration. If this CSI-RS is QCLed to the SSB of serving cell or SSB of additionPCI cell, they use the same set of requirements.

Based on the previous discussion, proponents of option 1 think that inter-cell scenario is not supported in RAN1 spec. If this is common understanding in RAN1, option 2 for RAN4 requirements also means that inter-cell scenario is not supported.

Given that option 2 does not explicitly include or preclude the inter-cell scenario from RAN4 perspective, and it follows the legacy RAN4 requirement approach, we support to use option 2.

Proposal 3: Reuse the same requirements structure as that in Rel-17 9.5. No need to add sharing factor for serving cell CSI-RS and NSC CSI-RS. From the configuration of CSI-RS perspective, there is only one configuration. If this CSI-RS is QCLed to the SSB of serving cell or SSB of additionPCI cell, they use the same set of requirements.

- Summary

In this paper, we provide our views on RRM requirements for UE-initiated/event-driven beam management. From this discussion we have the following proposals:

Proposal 1: If TL1_measurement period is different for current beam and new beam, the TL1_measurement period is maximum of the L1 measurement periods to measure both current beam and new beams.

Proposal 2: Specify requirements to cover both cases of no time window based and with time window based with configurable eventInstanceCount-r19.

Proposal 3: Reuse the same requirements structure as that in Rel-17 9.5. No need to add sharing factor for serving cell CSI-RS and NSC CSI-RS. From the configuration of CSI-RS perspective, there is only one configuration. If this CSI-RS is QCLed to the SSB of serving cell or SSB of additionPCI cell, they use the same set of requirements.

- References

[1] R4- R4-2504902, WF on RRM requirements for NR_MIMO_Ph5_Part1, Samsung