3GPP TSG-RAN WG4 Meeting #115												R4-2505398

Malta, 19 – 23 May, 2025

Agenda item:			7.14.4.1

Source:	Huawei, HiSilicon

Title:	on TRP TRS OTA tests

Document for:	approval

# Introduction

The issue of performance for devices narrower than 72mm has been discussed in the last meeting. This contribution recaps the related discussion.

# Discussion

The trend from last few meetings on the issue of performance for devices narrower than 72mm is to use offset method between size 1 and size 2. The proposal in the last meeting to “use offset=0 for TRP and TRS performances between size 1 and size 2 devices for all releases and all corresponding bands from and including release 19 onwards” has gained some traction albeit not agreed by all. The benefits of such a proposal are its simplicity and meeting/lab time saving. Because it means that all results for devices wider than 72mm automatically applies to devices narrower than 72mm for corresponding bands and no additional meeting discussion and lab measurement are needed.

MIMO OTA test with phantoms [1] has been proposed for Release 20 work. The most likely scenario for MIMO OTA tests is to use hand phantoms as the head phantom scenario for MIMO is not a practical application. This means that the issue of size 1 and size 2 performances could arise in Release 20 MIMO OTA work. If the above proposal of zero offset between size 1 and size 2 is agreed for TRP and TRS work, it can also be agreed for Release 20 MIMO OTA tests with phantoms. In other words, additional savings in terms of meeting discussion and lab time on MIMO OTA tests for future releases.

Observation 1: If the zero offset between size 1 and size 2 is agreed for TRP and TRS work, it can also be agreed for Release 20 MIMO OTA tests with hand phantoms.

Proposal 1: use offset=0 between size 1 and size 2 devices for all releases and all corresponding bands from and including release 19 onwards.

# Conclusion

This contribution makes the following proposal and observation.

Observation 1: If the zero offset between size 1 and size 2 is agreed for TRP and TRS work, it can also be agreed for Release 20 MIMO OTA tests with hand phantoms.

Proposal 1: use offset=0 for TRP and TRS performances between size 1 and size 2 devices for all releases and all corresponding bands from and including release 19 onwards.

# Reference

- RP-250423 views on RAN4 Rel-20 OTA topics