3GPP RAN WG4 Meeting #115																R4-2505415

Malta, Malta, 19th – 23rd May 2025

Agenda item:	6.15.1

Source:	Apple

Title:	Further considerations on the emission requirements for the UE operating in 1626.5-1660.5MHz and 1668-1675MHz UL frequency ranges

WI/SI:	NR_NTN_combinedLband-Core

Release:	Rel-19

Document for:	Decision

# 1	Introduction

After the RAN#86 meeting, the Rel-17 WI was approved that aimed at enabling the 5G/NR radio access technology for non-terrestrial satellite deployments [1]. One of the WI objectives was to define and introduce the corresponding bands into the 3GPP specifications. The outcome of these discussions was two NTN bands – L-band and S-band – which were added as bands n255 and n256, respectively, to the Rel-17 core specifications. However, already at that time it was noted that there exist other satellite deployments. To limit the overall workload in Rel-17, it was decided by RAN WG4 to focus on L- and S-bands with the common understanding that other potential NTN bands should follow the "normal" process of submitting a new spectrum WI. For instance, after the RAN#98 meeting a new Rel-18 NTN spectrum WI was agreed with the objective of adding support for a new NTN band (band n254) where the UL is on the L-band and the DL is on the S-band [2].

After the RAN#104 meeting another spectrum WI was agreed to introduce new bands operating in the L-band frequency range [3]. As discussed during the RAN4#114, there exist ETSI requirements that govern UE operation in the frequency ranges corresponding to the considered L-bands. Based on that RAN4#114 meeting concluded to capture the corresponding ETSI requirements followed by the RAN4#114bis decision to introduce new NS flags to introduce these requirements into the normative specifications.

# 2	Regulatory requirements for L-bands

## 2.1	Background and ETSI L-band frequency arrangements

The satellite L-bands are regulated by a number of documents including ETSI requirements, such as ETSI EN 301441 and EN 301681. It should be noted that ETSI EN 301441 covers the case when the L-band has only UL allocation at 1.6GHz, while the DL part is in the S-band at 2.5GHz; this combination has been introduced as 3GPP NTN band n254. On the contrary to it, ETSI EN 301681 covers the case when both DL and UL are in the L-band frequency range (more precisely 1.5GHz for the DL block and 1.6GHz for the UL block).  The table below from the corresponding ETSI document presents frequency block arrangements, for which additional ETSI specific requirements exist. As can be seen from the table these frequency ranges, sub-bands 1 and 2, corresponding to the considered 3GPP NTN bands.

Table 2.1-1: ETSI EN 301681 frequency block arrangements.

## 2.2	Emission requirements and NS flags

Referring to sub-clause 4.2.2 in ETSI EN 301681, there are additional in-band emission requirements defined for a device operating in either sub-band 1 or sub-band 2. In addition to the in-band emission requirements, ETSI EN 301681 also defines out-of-band emission requirements as (refer to sub-clause 4.2.1 in the ETSI EN 301681 for a complete technical description). For the sake of clarity, the corresponding ETSI requirements are presented in Annex.

As agreed during the RAN4#114bis meeting, two new NS flags will be defined, whereupon each NS flag will have associated ETSI in-band requirements (common for sub-band 1 and 2) and ETSI out-of-band emission requirements. The corresponding tables can be found in Annex B.

## 2.3	Power back-off analysis for ETSI L-band sub-ranges

### 2.3.1	ETSI sub-range 1 (1626.5-1660.5MHz)

In this section we provide the power back-off results for a UE operating in the ETSI sub-band 1. We consider the following three major test points:

-	a channel in the middle of the band (Fc=1645MHz)

-	a channel at the lower edge of the band (Fc=1629MHz)

-	a channel at the upper edge of the band (Fc=1658MHz)

#### 2.3.1.1	5MHz channel bandwidth

As can be seen from the Figure 2.3.1.1-1 below, when the NR channel is in the middle of the band, no additional AMPR is anticipated: the out-of-band emission requirements are not relevant because of the size of the band and the in-band ETSI emission mask does not create any additional restrictions either.

Figure 2.3.1.1-1: Power back-off for CP-OFDM and DFT-s-OFDM (Fc=1645MHz).

As for the channels either at the lower or the upper edge of the band, extra A-MPR is needed for larger allocations with DFT-s-OFDM and the right-most channel edge allocations as presented in Figure 2.3.1.1-2 below. And the same observations can be done for the 5MHz channel at the upper edge of the band: left-most channel allocations will need extra A-MPR to comply with the out-of-band emission requirements.

Figure 2.3.1.1-2: Power back-off for CP-OFDM and DFT-s-OFDM (Fc=1629MHz).

Figure 2.3.1.1-3: Power back-off for CP-OFDM and DFT-s-OFDM (Fc=1658MHz).

One of the reasons why the left- and right-most channel power back-off look identical despite the fact that out-of-band emission requirements are not the same is the fact that the limiting factor is -30dBm/30kHz. Two figures below present the spectrum plot for the 5MHz channels at the left and right edges of the band, from which one can see that the most limiting factor is the -30dBm/30kHz emission limit at the offset of 2MHz from the band edge.

Figure 2.3.1.1-4: Spectrum plot for the 5MHz channel at the left edge (top figure) and right edge (bottom figure) of the band.

#### 2.3.1.2	10MHz channel bandwidth

In this sub-section we present power back-off simulation results for the 10MHz channel residing at the left and the right edges of the band. General observations are similar to the 5MHz channel, the only noticeable difference is that the 10MHz channel at the upper edge of the band requires slightly higher power back-off due to more stringent out-of-band emission requirements.

Figure 2.3.1.3-1: Power back-off for CP-OFDM and DFT-s-OFDM (Fc=1631.5MHz).

Figure 2.3.1.3-2: Power back-off for CP-OFDM and DFT-s-OFDM (Fc=1655.5MHz).

#### 2.3.1.3	15MHz channel bandwidth

Power back-off simulation results for the 15MHz are logically identical to the 10MHz results presented in the previous section. The 15MHz channel at the upper edge of the band requires even higher power back-off for large allocations and the for the right-most allocations within the band.

Figure 2.3.1.3-1: Power back-off for CP-OFDM and DFT-s-OFDM (Fc=1634MHz).

Figure 2.3.1.3-2: Power back-off for CP-OFDM and DFT-s-OFDM (Fc=1653MHz).

#### 2.3.1.4	20MHz channel bandwidth

The 20MHz channel in the ETSI sub-range 1 will require even higher power back-off for the edge channel allocations. For the channel at the lower edge of the band, the left-most allocations will need the highest power back-off reaching 5-6.5dB. And similarly, the right-most allocations within the channel at the upper edge of the band will require power back-off in the order of  5.5dB.

Figure 2.3.1.4-1: Power back-off for CP-OFDM and DFT-s-OFDM (Fc=1636.5MHz).

Figure 2.3.1.4-2: Power back-off for CP-OFDM and DFT-s-OFDM (Fc=1650.5MHz).

#### 2.3.1.5		Summary of the power back-off results for ETSI sub-range 1

Based on the presented power back-off results in the previous sub-section we suggest adopting the following preliminary A-MPR values (subject for further checking and revisions).

Table 2.3.1.5-1: A-MPR regions for the ETSI sub-range 1

| Channel Bandwidth (MHz) | Carrier Centre Frequency, Fc (MHz) | Region |  |  |
| --- | --- | --- | --- | --- |
|  |  | RBstart*12*SCS | LCRB*12*SCS | A-MPR |
| 5MHz | 1629 <= Fc < 1630.5 | <= 0.72 |  | A1 |
|  | 1656.5 < Fc <= 1658 | >= 3.6 |  | A1 |
| 10MHz | 1631.5 <= Fc < 1634 | <= 2.16 | < 7.2 | A1 |
|  |  |  | >= 7.2 | A2 |
|  | 1653 < Fc <= 1655.5 | >= 6.84 | < 7.2 | A3 |
|  |  |  | >= 7.2 | A2 |
| 15MHz | 1634 <= Fc < 1639 | <= 2.16 |  | A2 |
|  | 1648 < Fc <= 1653 | >= 10.08 | < 10.8 | A4 |
|  |  |  | >= 10.8 | A3 |
| 20MHz | 1636.5 <= Fc < 1641.5 | <= 1.8 | <= 7.2 | A5 |
|  |  | >1.8, <=5.4 | <=0.72 | A1 |
|  |  |  | > 7.2 | A4 |
|  |  | >= 18.0 |  | A1 |
|  | 1645.5 < Fc <= 1650.5 | >=9.72 |  | A4 |
|  |  | <=1.08 |  | A1 |

Table 2.3.1.5-2: A-MPR values for the ETSI sub-range 1

|  | Modulation | A1 | A2 | A3 | A4 | A5 |
| --- | --- | --- | --- | --- | --- | --- |
| DFT-s-OFDM | Pi/2 BPSK |  |  |  |  |  |
|  | QPSK | 1.5 | 2.0 | 2.5 | 3.5 | 5 |
|  | 16QAM |  |  |  |  |  |
|  | 64QAM |  |  |  |  |  |
| CP-OFDM | QPSK | 3.0 | 3.5 | 4.0 | 5.5 | 6.5 |
|  | 16QAM |  |  |  |  |  |
|  | 64QAM |  |  |  |  |  |

### 2.3.2	ETSI sub-range 2 (1668-1675MHz)

In this section we provide the power back-off results for a UE operating in the ETSI sub-band 2. Since the total sub-band size is 7MHz, we consider only the 5MHz channel at the following test points:

-	a 5MHz channel at the lower edge of the band (Fc=1670.5MHz)

-	a 5MHz channel in the middle of the band (Fc=1671.5MHz)

-	a 5MHz channel at the upper edge of the band (Fc=1672.5MHz)

#### 2.3.2.1	5MHz channel

Figure 2.3.2.1-1 presents power back-off values for the 5MHz channel at the lower edge of the band. As can be seen most allocations do not need A-MPR, except the case(s) with a single RB allocated at the right edge of the 5MHz channel. The reason for that is the upper edge out-of-band emission requirements which would require some A-MPR. As presented in Figure 2.3.2.1-2 below, CP-OFDM slightly exceeds the limit of -30dBm/30kHz and DFT-s-OFDM is also hitting that limit.

Figure 2.3.2.1-1: Power back-off for CP-OFDM and DFT-s-OFDM (Fc=1670.5MHz).

Figure 2.3.2.1-2: Spectrum plot for the 5MHz channel and 1RB allocation (Fc=1670.5MHz).

Once the 5MHz channel is shifted farther towards the upper edge of the band, even higher A-MPR might be needed, which is however mostly concerns single RB allocations at the upper edge of the configured channel.

Figure 2.3.2.1-3: Power back-off for CP-OFDM and DFT-s-OFDM (Fc=1671.5MHz).

Figure 2.3.2.1-4: Power back-off for CP-OFDM and DFT-s-OFDM (Fc=1672.5MHz).

#### 2.3.2.2		Summary of the power back-off results for ETSI sub-range 2

Based on the presented power back-off results in the previous sub-section we suggest adopting the following preliminary A-MPR values (subject for further checking and revisions).

Table 2.3.2.2-1: A-MPR regions for the ETSI sub-range 2

| Channel Bandwidth (MHz) | Carrier Centre Frequency, Fc (MHz) | Region |  |  |
| --- | --- | --- | --- | --- |
|  |  | RBstart*12*SCS | LCRB*12*SCS | A-MPR |
| 5MHz | 1670.5 <= Fc <= 1671.5 | >= 3.6 |  | A1 |
|  | 1671.5 < Fc <= 1672.5 | >= 3.6 |  | A1 |
|  |  |  | >= 3.6 | A1 |

Table 2.3.2.2-2: A-MPR values for the ETSI sub-range 2

|  | Modulation | A1 | A2 | A3 | A4 | A5 |
| --- | --- | --- | --- | --- | --- | --- |
| DFT-s-OFDM | Pi/2 BPSK |  |  |  |  |  |
|  | QPSK | 1.5 |  |  |  |  |
|  | 16QAM |  |  |  |  |  |
|  | 64QAM |  |  |  |  |  |
| CP-OFDM | QPSK | 3.0 |  |  |  |  |
|  | 16QAM |  |  |  |  |  |
|  | 64QAM |  |  |  |  |  |

# 3	Conclusions

In this discussion paper we have presented our further technical analysis on the existing ETSI regulations for a device operating UL in the 1.6GHz frequency range. Since there are additional ETSI in- band out-of-band emission requirements for the 1626.5-1660.5MHz and 1668-1675MHz sub-ranges, 3GPP already agreed to introduce two NS flags that will be associated with the corresponding ETSI requirements and A-MPR values presented in this discussion paper.

Proposal 1:	Define AMPR values for ETSI sub-range 1 as presented in Table 2.3.1.5-1 and 2.3.1.5-2.

Proposal 2:	Define AMPR values for ETSI sub-range 2 as presented in Table 2.3.2.2-1 and 2.3.2.2-2.

# 4	References

- RP-193234, "Solutions for NR to support non-terrestrial networks (NTN)", Thales

- RP-223431, "New WID on Introduction of the satellite L-/S-band", Globalstar, Apple

RP-241689, "Revised WID on New NR NTN bands to support Extended L-band and combined MSS L-band and Extended L-band ranges ", Inmarsat, Viasat

Annex A: ETSI requirements for L-bands

Figure A-1: ETSI in-band emission masks for a device operating in 1626.5-1660.5MHz (left table) and 1668-1675MHz (left and right tables)

Figure A-2: ETSI out-of-band emission requirements for a device operating in 1626.5-1660.5MHz (left table) and 1668-1675MHz (right table)

Annex B: 3GPP requirements based on ETSI requirements

Table B-1: Additional requirements for ETSI sub-band 1 and 2

| ΔfOOB (kHz) | Spectrum emission limit (dBm) | Measurement bandwidth | Note (measurement method) |
| --- | --- | --- | --- |
| ± 0-25 | 30 to 15 | 3 kHz | Average |
| ± 25-125 | 15 to -20 |  |  |
| ± 125-425 | -20 |  |  |
| ± 425-1500 | -20 to -35 |  |  |
| ± 1500-36000 | -25 | 30 kHz |  |
| NOTE 1: Spectrum emissions are linearly interpolated versus frequency offset. NOTE 2: The EIRP requirement in regulation is converted to conducted requirement using a 0dBi antenna. |  |  |  |

Table B-2: Additional out-of-band requirements for ETSI sub-band 1 (1626.5-1660.5MHz)

| Frequency range (MHz) | Spectrum emission limit1 (dBm) | Measurement bandwidth | NOTE (measurement method) |
| --- | --- | --- | --- |
| 30 to 1000 | -36 | 100 kHz | Peak hold |
| 1000 to 1559 | -31 | 1 MHz | Average |
| 1559 to 1605 | -40 | 1 MHz | Average |
| 1605 to 1612.5 | -40 to -28.5 | 1 MHz | Average |
| 1612.5 to 1616.5 | -25 to -20 | 1 MHz | Average |
| 1616.5 to 1621.5 | -20 to -16 | 1 MHz | Average |
| 1621.5 to 1624.5 | -30 to -27.5 | 30 kHz | Average |
| 1624.5 to 1625.125 | -27.5 to -27.2 | 30 kHz | Average |
| 1625.125 to 1625.8 | -27.2 to -20 | 30 kHz | Average |
| 1625.8 to 1626 | -20 to -17 | 30 kHz | Average |
| 1626 to 1626.2 | -17 to -10 | 30 kHz | Average |
| 1626.2 to 1626.5 | -10 | 30 kHz | Average |
| 1626.5 to 1660.5 | ETSI sub-band 1 |  |  |
| 1660.5 to 1660.525 | 30 to 15 | 3 kHz | Average |
| 1660.525 to 1660.625 | 15 to - 20 | 3 kHz | Average |
| 1660.625 to 1660.925 | -20 | 3 kHz | Average |
| 1660.925 to 1662 | -20 to -35 | 3 kHz | Average |
| 1662 to 1662.5 | -25 | 30 kHz | Average |
| 1662.5 to 1665.5 | -30 | 30 kHz | Average |
| 1665.5 to 1670.5 | -30 | 100 kHz | Average |
| 1670.5 to 1680.5 | -30 | 300 kHz | Average |
| 1680.5 to 1690.5 | -30 | 1 MHz | Average |
| 1690.5 to 2250 | -30 | 3 MHz | Average |
| 2250 to 12750 | -30 | 3 MHz | Peak hold |
| NOTE 1: Spectrum emissions are linearly interpolated versus frequency offset. NOTE 2: The EIRP requirement in regulation is converted to conducted requirement using a 0dBi antenna. NOTE 3: The average measurement method shall apply except that an averaging period of 20ms shall be used in the sub-band 1573.42 MHz to 1580.42 MHz |  |  |  |

Table B-3: Additional out-of-band requirements for ETSI sub-band 2 (1668-1675MHz)

| Frequency range (MHz) | Spectrum emission limit1 (dBm) | Measurement bandwidth | NOTE (measurement method) |
| --- | --- | --- | --- |
| 30 to 1000 | -36 | 100 kHz | Peak hold |
| 1000 to 1559 | -31 | 1 MHz | Average |
| 1559 to 1605 | -40 | 1 MHz | Average |
| 1605 to 1612.5 | -40 to -28.5 | 1 MHz | Average |
| 1612.5 to 1616.5 | -25 to -20 | 1 MHz | Average |
| 1616.5 to 1621.5 | -20 to -16 | 1 MHz | Average |
| 1621.5 to 1624.5 | -30 to -27.5 | 30 kHz | Average |
| 1624.5 to 1625.125 | -27.5 to -27.2 | 30 kHz | Average |
| 1625.125 to 1625.8 | -27.2 to -20 | 30 kHz | Average |
| 1625.8 to 1626 | -20 to -17 | 30 kHz | Average |
| 1626 to 1626.2 | -17 to -10 | 30 kHz | Average |
| 1626.2 to 1626.5 | -10 | 30 kHz | Average |
| 1660.5 to 1666.5 | -25 | 30 kHz | Average |
| 1666.5 to 1667.575 | -20 to -35 | 3 kHz | Average |
| 1667.575 to 1667.875 | -20 | 3 kHz | Average |
| 1667.875 to 1667.975 | 15 to - 20 | 3 kHz | Average |
| 1667.975 to 1668 | 30 to 15 | 3 kHz | Average |
| 1668 to 1675 | ETSI sub-band 2 |  |  |
| 1675 to 1675.025 | 30 to 15 | 3 kHz | Average |
| 1675.025 to 1675.125 | 15 to - 20 | 3 kHz | Average |
| 1675.125 to 1675.425 | -20 | 3 kHz | Average |
| 1675.425 to 1676.5 | -20 to -35 | 3 kHz | Average |
| 1676.5 to 1677 | -25 | 30 kHz | Average |
| 1677 to 1680 | -30 | 30 kHz | Average |
| 1680 to 1685 | -30 | 100 kHz | Average |
| 1685 to 1695 | -30 | 300 kHz | Average |
| 1695 to 1705 | -30 | 1 MHz | Average |
| 1705 to 2250 | -30 | 3 MHz | Average |
| 2250 to 12750 | -30 | 3 MHz | Peak hold |
| NOTE 1: Spectrum emissions are linearly interpolated versus frequency offset. NOTE 2: The EIRP requirement in regulation is converted to conducted requirement using a 0dBi antenna. NOTE 3: The average measurement method shall apply except that an averaging period of 20ms shall be used in the sub-band 1573.42 MHz to 1580.42 MHz |  |  |  |