3GPP TSG-RAN WG4 Meeting # 115		R4-2505439

Malta, MT, May 19 – 23, 2025

Agenda item:	7.7.3

Source:	Apple

Title:	UE RF requirements for LB CA via switching

WI/SI:	NR_LBCA_Sw-Core

Release:	Rel-19

Document for:	Discussion

# 1	Introduction and Scope

In the previous RAN4 meeting (Meeting #114Bis, Wuhan, China, April 07 – 11, 2025), discussions were held on several UE RF aspects of LB CA via switching (NR_LBCA_Sw) work item. The way forward for the main topics that were debated is shown WF below [1] with the remaining issues highlighted.

Way Forward

Issue 1: Switching period capability

-   The switching period capability can be defined per band pair per band combination.

Issue 2: Switching period location

-   Await outcome of RAN1 design

Issue 3: Time mask for switching

-   Define the Tx on-off time mask requirements

-   FFS on how to define the Rx switching requirements to reflect where the switching period occurs

-   RAN4 has understanding that RAN1 will take switching period and timing advance into account for RAN1 scheduling pattern for low band CA via switching

Issue 4: Considerations related to the Tx on-off time mask

-   Option 1: Specify transmit on/off time mask for FDD UL carrier/Pcell for switching between LB(FDD)-LB(SDL)

-   Option 2: Specify on->off time mask for switching from FDD to SDL carrier and specify off->on time mask for switching from SDL to FDD carrier

-   Other options are not precluded

Issue 5: Test methodology

-   Companies are encouraged to contribute proposals related to the testability of the requirement on the time mask for switching

In this discussion paper, we will share our views on some of the issues highlighted above.

# Discussion

  - Reference channel for receiver sensitivity measurement

As defined in Clause 7 of the TS38.101-1, the reference sensitivity power level REFSENS is the minimum mean power at which the receiver throughput shall meet  or exceed 95% of maximum throughput of  the reference measurement channel as specified in Annexes A.2.2.2, A3.2 and A.3.3. The maximum throughput depends on the information bit payload per slot, which will be affected when symbols are blanked to accommodate the switching period when switching from FDD carrier to SDL carrier and from SDL carrier to FDD carrier.

Observation 1: The maximum throughput of the reference measurement channel will change when switching between FDD and SDL carriers as symbols with be blanked to accommodate the switching period.

To calculate the maximum throughput of the reference channel, we need to know how many symbols within a frame will be affected by the switching period. In RAN1#120bis meeting, the agreement made on the switching period locations and on how many switching transitions can occur within one FDD/SDL slot are highlighted below in gree:

- For the switching period location, RAN1 agreed that the switching period is always on the “switch from carrier” when switching from SDL to FDD and for FDDSDL, no agreement was made. For our analysis, we will assume that the period will be located on the “switch from” carrier for FDDSDL switching as well.

- RAN1 also agreed that only one switching event will occur within one slot.

- However, RAN1 still has not specified how many switching events can occur within a frame. We will assume up to 2 switching events per NR frame for the analysis.

Using the RAN1 agreements and the assumptions made above on the switching period location and the number of switching events within one frame, we will use the frame structure below to derive the calculations of the maximum throughput doe the reference channel.

Figure 2.1-1: Frame structure for FDD-SDL switching

We assumed two switching events within a frame, one to switch from the FDD carrier to the SDL carrier, and one from SDL carrier to FDD.

    - Reference channel for the FDD carrier

To calculate the maximum throughput for the reference channel, we need to determine the information bit payload per slot by first calculating the transport block size (TBS) based on the configurations for the reference channel shown in Tables A.3.2.2-1 and in Table A.3.1-1 of TS38.101-1, as well as the procedure highlighted in clause 5.1.3.2 of the TS38.214 specifications.

The algorithm on calculating the TBS is highlighted below, taken from clause 5.1.3.2 of the TS38.214 specifications:

1)	The UE shall first determine the number of REs (NRE) NRE)within the slot.

-	A UE first determines the number of REs allocated for PDSCH within a PRB () by, where is the number of subcarriers in a physical resource block, Nsymbslot is the number of symbols of the PDSCH allocation within the slot, NDMRSPRB is the number of REs for DM-RS per PRB in the scheduled duration including the overhead of the DM-RS CDM groups without data, as indicated by DCI format 1_1, 1_2 or 1_3 or as described for format 1_0 in Clause 5.1.6.2, and

-	A UE determines the total number of REs allocated for PDSCH ()NRE) by NRE= NRE'* nPRB, where nPRB is the total number of allocated PRBs for the UE.

2)	Unquantized intermediate variable (Ninfo) TBStemp) is obtained by TBStemp= NRE*R*Qm*ʋ.

If

Use step 3 as the next step of the TBS determination

else

Use step 4 as the next step of the TBS determination

end if

3)	When , TBS is determined as follows

-	quantized intermediate number of information bits , where .

-	use Table 5.1.3.2-1 find the closest TBS that is not less than .

Using the above algorithm and assuming a CBW of 10MHz and a switching period of 140uSec ( 2 OFDM symbols with 15kHz SCS), we can calculate the maximum throughput as follows:

- For slots 2, 3, 9, 12, 13, 14, and 19, the information bit payload remains the same because those slots are not affected by the switching period: 3368 Bits

- For slot 4 which has a switching period of 140uSec (2 OFDM symbols) the payload is calculated to be 2600 Bits when only 12 OFDM symbols are assumed instead of 14 symbols.

- The total payload for the whole frame will be: 7 x 3368 + 2600 = 26176 Bits

- This will result into a maximum throughput of 26176/10mSec = 2.6176 Mbps for one frame

It can be observed that there is a throughput loss of (2.6944-2.6176) = 0.0768 Mbps or 2.8% when LB CA via switching is implemented compared to the case without switching.

Observation 2: With 140uSec of switching time for LB CA via switching, there is about 2.8% loss for the reference channel maximum throughput compared to the throughput with no switching.

Proposal 1: We would like to propose the maximum DL throughput for  reference channel shown in Table 2.1.1-1 for the FDD channel with 10MHz CBW and 15KHz SCS when LB-LB CA view switching is considered between the FDD carrier and the SDL carrier  with 140uSec of switching period.

| Parameter | Unit | Value |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel bandwidth | MHz | 3 | 5, 10, 15, 20 (Note 5) | 10 | 15 | 20 | 25 | 30 | 40 | 50 |
| Subcarrier spacing | kHz | 15 | 15 | 15 | 15 | 15 | 15 | 15 | 15 | 15 |
| Subcarrier spacing configuration |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Allocated resource blocks |  | 15 | 25 | 52 | 79 | 106 | 133 | 160 | 216 | 270 |
| Subcarriers per resource block |  | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 |
| Allocated slots per Frame |  | 8 | 8 | 8 | 8 | 8 | 8 | 8 | 8 | 8 |
| MCS Index |  | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 |
| MCS Table for TBS determination |  | 64QAM |  |  |  |  |  |  |  |  |
| Modulation |  | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK |
| Target Coding Rate |  | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 |
| Maximum number of HARQ transmissions |  | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| Information Bit Payload per Slot |  |  |  |  |  |  |  |  |  |  |
| For Slots 0,1 | Bits | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| For Slots 2,3,4,5,6,7,8,9 | Bits | 984 | 1672 | 3368 (for slot 2,3,9,12,13,14,19) 2600 (for slot 4 with switching) | 5120 | 6912 | 8712 | 10504 | 14088 | 17424 |
| Transport block CRC | Bits | 16 | 16 | 16 | 24 | 24 | 24 | 24 | 24 | 24 |
| LDPC base graph |  | 2 | 2 | 2 | 1 | 1 | 1 | 1 | 1 | 1 |
| Number of Code Blocks per Slot |  |  |  |  |  |  |  |  |  |  |
| For Slots 0,1 | CBs | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| For Slots 2,3,4,5,6,7,8,9 | CBs | 1 | 1 | 1 | 1 | 1 | 2 | 2 | 2 | 3 |
| Binary Channel Bits per Slot |  |  |  |  |  |  |  |  |  |  |
| For Slots 0,1 | Bits | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| For Slots 2,3,4,5,6,7,8,9 | Bits | 3240 | 5400 | 11232 | 17064 | 22896 | 28728 | 34560 | 46656 | 58320 |
| Max. Throughput averaged over 1 frame | Mbps | 0.787 | 1.338 | 2.694 (no switching) 2.6176 with switching | 4.096 | 5.530 | 6.970 | 8.403 | 11.270 | 13.9392 |
| NOTE 1: Additional parameters are specified in Table A.3.1-1 and Table A.3.2.1-1. NOTE 2: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). NOTE 3: SS/PBCH block is transmitted in slot #0 of each frame NOTE 4: Slot i is slot index per frame NOTE 5: Channel bandwidths 10, 15, and 20 MHz in this column only apply to UEs supporting IE supportOfERedCap-r18 but not eRedCapNotReducedBB-BW-r18. |  |  |  |  |  |  |  |  |  |  |

Table 2.1.1-1: Reference channel maximum throughput for the FDD carrier when switching between FDD and SDL carriers

    - Reference channel for the SDL carrier

Using the frame structure in Figure 2.1-1, we can calculate the maximum throughput for the reference channel for the SDL carrier the same way we did it for the FDD carrier. The victim slot (slot where the switching period is located) is now slot #8, instead of slot #4 as was the case for the FDD carrier.

Proposal 2: We would like to propose the maximum DL throughput for  reference channel shown in Table 2.1.2-1 for the SDL channel with 10MHz CBW and 15KHz SCS when LB-LB CA view switching is considered between the FDD carrier and the SDL carrier  with 140uSec of switching period.

| Parameter | Unit | Value |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel bandwidth | MHz | 3 | 5, 10, 15, 20 (Note 5) | 10 | 15 | 20 | 25 | 30 | 40 | 50 |
| Subcarrier spacing | kHz | 15 | 15 | 15 | 15 | 15 | 15 | 15 | 15 | 15 |
| Subcarrier spacing configuration |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Allocated resource blocks |  | 15 | 25 | 52 | 79 | 106 | 133 | 160 | 216 | 270 |
| Subcarriers per resource block |  | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 |
| Allocated slots per Frame |  | 8 | 8 | 8 | 8 | 8 | 8 | 8 | 8 | 8 |
| MCS Index |  | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 |
| MCS Table for TBS determination |  | 64QAM |  |  |  |  |  |  |  |  |
| Modulation |  | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK |
| Target Coding Rate |  | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 |
| Maximum number of HARQ transmissions |  | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| Information Bit Payload per Slot |  |  |  |  |  |  |  |  |  |  |
| For Slots 0,1 | Bits | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| For Slots 2,3,4,5,6,7,8,9 | Bits | 984 | 1672 | 3368 (for slot 5,6,7,15,16,17,18) 2600 (for slot 8 with switching) | 5120 | 6912 | 8712 | 10504 | 14088 | 17424 |
| Transport block CRC | Bits | 16 | 16 | 16 | 24 | 24 | 24 | 24 | 24 | 24 |
| LDPC base graph |  | 2 | 2 | 2 | 1 | 1 | 1 | 1 | 1 | 1 |
| Number of Code Blocks per Slot |  |  |  |  |  |  |  |  |  |  |
| For Slots 0,1 | CBs | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| For Slots 2,3,4,5,6,7,8,9 | CBs | 1 | 1 | 1 | 1 | 1 | 2 | 2 | 2 | 3 |
| Binary Channel Bits per Slot |  |  |  |  |  |  |  |  |  |  |
| For Slots 0,1 | Bits | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| For Slots 2,3,4,5,6,7,8,9 | Bits | 3240 | 5400 | 11232 | 17064 | 22896 | 28728 | 34560 | 46656 | 58320 |
| Max. Throughput averaged over 1 frame | Mbps | 0.787 | 1.338 | 2.694 (no switching) 2.6176 with switching | 4.096 | 5.530 | 6.970 | 8.403 | 11.270 | 13.9392 |
| NOTE 1: Additional parameters are specified in Table A.3.1-1 and Table A.3.2.1-1. NOTE 2: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). NOTE 3: SS/PBCH block is transmitted in slot #0 of each frame NOTE 4: Slot i is slot index per frame NOTE 5: Channel bandwidths 10, 15, and 20 MHz in this column only apply to UEs supporting IE supportOfERedCap-r18 but not eRedCapNotReducedBB-BW-r18. |  |  |  |  |  |  |  |  |  |  |

Table 2.1.2-1: Reference channel maximum throughput for the SDL carrier when switching between FDD and SDL carriers

  - Switching period capability

Proposal 3: We would like to propose the switching period as a UE capability as described in Table 2.2-1

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 59 NR_LBCA_Sw | 59.1 | Switching period for switching between a LB FDD band and an SDL band | UE to indicate support of LB-LB carrier aggregation via switching between a LB FDD band and an SDL band. Switching_Period_For_FDD-SDL indicates the length the switching time between the FDD band and the SDL band: 35us represents 35 us of switching time, 140us represents 140us of switching time, as specified in TS 38.101-1. | N/A | Yes | N/A | Operation of Carrier aggregation between the FDD and the SDL via switching is not supported | per band pair per band combination |  | Applicable to FR1 only | Yes | N/A | Optional with capability signaling |

Table 2.2-1: Switching period capability for FDD-SDL CA via switching

# 3	Conclusions

Observation 1: The maximum throughput of the reference measurement channel will change when switching between FDD and SDL carriers as symbols with be blanked to accommodate the switching period.

Observation 2: With 140uSec of switching time for LB CA via switching, there is about 2.8% loss for the reference channel maximum throughput compared to the throughput with no switching.

Proposal 1: We would like to propose the maximum DL throughput for  reference channel shown in Table 2.1.1-1 for the FDD channel with 10MHz CBW and 15KHz SCS when LB-LB CA view switching is considered between the FDD carrier and the SDL carrier  with 140uSec of switching period.

Proposal 2: We would like to propose the maximum DL throughput for  reference channel shown in Table 2.1.2-1 for the SDL channel with 10MHz CBW and 15KHz SCS when LB-LB CA view switching is considered between the FDD carrier and the SDL carrier  with 140uSec of switching period.

Proposal 3: We would like to propose the switching period as a UE capability as described in Table 2.2-1

# 4	References

R4-2505223, “WF on UE RF requirements for LB CA via switching”, by Apple,  3GPP TSG-RAN WG4 Meeting #114bis, Athens,  Greece, Athens, Greece, February 17-21, 2025