3GPP TSG-RAN4 Meeting #115	R4-2505449

# Malta, May 19th – 23rd, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | 2745 | rev |  | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to 38.101-1 on introducing UL Tx switching for intra-band NC UL CA |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19 |  |  |  |  |  | Date: |  |  | 2025-04-21 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | To introduce UL Tx switching feature for intra-band non-contiguous UL CA |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a new sub-clause 6.3A.3.2.2 Time mask for switching between two uplink carriers under clause 6.3A.3.2 Transmit ON/OFF time mask for intra-band non-contiguous CA. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UL Tx switching for intra-band non-contiguous UL CA is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3A.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<< Start of changed sections >>>

#### 6.3A.3.2	Transmit ON/OFF time mask for intra-band non-contiguous CA

##### 6.3A.3.2.1	General

For intra-band non-contiguous carrier aggregation, the general output power ON/OFF time mask specified in clause 6.3.3.1 is applicable for each component carrier during the ON power period and the transient periods. The OFF period as specified in clause 6.3.3.1 shall only be applicable for each component carrier when all the component carriers are OFF.

##### 6.3A.3.2.2	Time mask for switching between two uplink carriers

In addition to the requirements in 6.3A.3.2.1 and the maximum output power requirement specified in Table 6.2A.1.2-1 with uplink assigned to two carriers, the switching time mask specified in this clause is applicable for an intra-band non-contiguous UL CA configuration when the capability [uplinkTxSwitchingPeriod2T2T] is present, and is only applicable for uplink switching mechanisms specified in clause 6.1.6 of TS 38.214 [10], where both NR UL carriers are capable of two transmit antenna connectors. The UE shall support switching between the two uplink carriers both with two-layer transmission following the scheduling commands and rank adaptation.

The switching periods described in Figure 6.3A.3.2.2-1a and Figure 6.3A.3.2.2-1b are located in either NR carrier 1 or carrier 2 as indicated in RRC signalling uplinkTxSwitchingPeriodLocation [7], and the length of uplink switching period X is less than the value indicated by UE capability [uplinkTxSwitchingPeriod2T2T].

When switching from one carrier to another, if there is no uplink transmission scheduled or configured on the switch-from carrier for at least the duration of the switching period (X µs) before the point in time the UE is scheduled or configured to start the transmission on the switch-to carrier, the switching period is fully contained in the time period between the end of the transmission on the switch-from carrier and the start of the transmission on the switch-to carrier. In addition, the RRC signalling uplinkTxSwitchingPeriodLocation is ignored by the UE and does not take effect in this case.

Figure 6.3A.3.2.2-1a: Time mask for switching between UL carrier 1 and UL carrier 2, where the switching period is located in carrier 1

Figure 6.3A.3.2.2-1b: Time mask for switching between UL carrier 1 and UL carrier 2, where the switching period is located in carrier 2

<<< End of changed sections >>>