3GPP TSG-RAN WG4 Meeting #115        				            R4-2505473

Malta, MT, 19th - 23rd May 2025

Title:	RRM Core requirements on event triggered L1 measurement reporting

Source:	China Telecom

Agenda item:			7.29.3.1

Document for:	Discussion

- Introduction

In the RAN4 #114 meeting, the measurements related enhancements were discussed and WF [1] has been approved. In this paper, we will continue to discuss the RRM Core requirements on event triggered L1 measurement reporting for LTM.

- Discussion

Issue 2-3: whether to introduce/update requirements for reporting criteria per measurement category

Candidate options: - Option 1: Yes (Xiaomi, CMCC, ZTE, CTC, vivo) - Option 2: assuming new UE capabilities regarding support of event triggering needs to be introduced, RAN4 shall discuss whether it is still necessary to update/introduce new Event Triggering and Reporting Criteria for LTM. (Apple)

Issue 2-4: whether to define a separate capability for supported number of events triggered L1 measurement reporting

Candidate options: - Option 1: RAN4 to define a new UE capability for Event Triggering and Reporting Criteria for LTM. (Xiaomi, CMCC, ZTE, E///, CTC) - Option 1a: Yes. RAN4 to support a common LTM Ecat capability to be shared between intra- and inter-frequency candidate cells. Ecat capability for LTM L1 measurements includes both PCell event, and candidate cells’ events. The detail number of Ecat is FFS. (E///) - Option 2: RAN4 to update intra-frequency and inter-frequency measurement reporting criteria requirements for SSB and CSI-RS based L1-RSRP with the same Ecat. (OPPO)

In legacy, requirements for reporting criteria per measurement category for L3 measurement and reporting had been specified in section 9.1.4. Similarly, for event triggered L1 measurement reporting, the requirements for reporting criteria per measurement category shall be defined too.

Both L3 measurement reporting and L1 measurement reporting could trigger LTM, no need for UE to be able to support different events of L3 and L1 in parallel per category. So we think a separate capability is better.

Proposal 1: Introduce requirements for reporting criteria per measurement category.

Proposal 2: Define a separate capability for supported number of events triggered L1 measurement reporting.

- Summary

In this paper, we provide our views on measurements related enhancements for LTM. From this discussion we have derived the following proposals:

Proposal 1: Introduce requirements for reporting criteria per measurement category.

Proposal 2: Define a separate capability for supported number of events triggered L1 measurement reporting.

- References

[1] R4-2504965, WF on RRM requirements for NR_Mob_Ph4_Part1, Apple Inc.